# RSP Test Specification for Servers

# SGP.23-3 Version 3.1

# 1 December 2023

Security Classification: Non-confidential

Access to and distribution of this document is restricted to the persons permitted by the security classification. This document is subject to copyright protection. This document is to be used only for the purposes for which it has been supplied and information contained in it must not be disclosed or in any other way made available, in whole or in part, to persons other than those permitted under the security classification without the prior written approval of the Association.

Copyright Notice

Copyright © 2023 GSM Association

Disclaimer

The GSM Association (“Association”) makes no representation, warranty or undertaking (express or implied) with respect to and does not accept any responsibility for, and hereby disclaims liability for the accuracy or completeness or timeliness of the information contained in this document. The information contained in this document may be subject to change without prior notice.

Compliance Notice

The information contain herein is in full compliance with the GSM Association’s antitrust compliance policy.

This Permanent Reference Document is classified by GSMA as an Industry Specification, as such it has been developed and is maintained by GSMA in accordance with the provisions set out in GSMA AA.35 - Procedures for Industry Specifications.

Table of Contents

1	Introduction	6

1.1	Overview	6

1.2	Scope	6

1.3	Definition of Terms	6

1.4	Abbreviations	7

1.5	Document Cross-references	7

1.6	Conventions	9

2	Testing Rules	9

2.1	Applicability	9

2.1.1	Format of the Optional Features Table	9

2.1.2	Format of the Applicability Table	9

2.1.3	Applicability and Notations	10

2.1.4	Optional Features Table	10

2.1.5	Applicability Table	10

2.2	General Consideration	16

2.2.1	Test Case Definition	16

2.2.2	Test Cases Format	16

2.2.3	VOID	21

2.2.4	VOID	21

2.2.5	Pass Criteria	21

2.2.6	Future Study	21

2.2.7	General Rules for SM-DP+ Testing	21

3	Testing Architecture	22

3.1	Testing Scope	22

3.2	Testing Execution	23

3.2.1	VOID	24

3.2.2	SM-DP+ and SM-DS - Test Environment	24

3.2.3	VOID	26

3.2.4	VOID	26

3.2.5	VOID	26

4	Interface Compliance Testing	26

4.1	General Overview	26

4.2	VOID	27

4.3	SM-DP+ Interfaces	27

4.3.1	ES2+ (Operator -- SM-DP+): DownloadOrder	27

4.3.2	ES2+ (Operator -- SM-DP+): ConfirmOrder	32

4.3.3	ES2+ (Operator -- SM-DP+): CancelOrder	44

4.3.4	ES2+ (Operator -- SM-DP+): ReleaseProfile	50

4.3.5	ES2+ (Operator -- SM-DP+): HandleNotification	50

4.3.6	ES2+ (Operator -- SM-DP+): TLS, Mutual Authentication, Server, Session Establishment	50

4.3.7	ES8+ (SM-DP+ -- eUICC): InitialiseSecureChannel	50

4.3.8	ES8+ (SM-DP+ -- eUICC): ConfigureISDP	50

4.3.9	ES8+ (SM-DP+ -- eUICC): StoreMetadata	50

4.3.10	ES8+ (SM-DP+ -- eUICC): ReplaceSessionKeys	51

4.3.11	ES8+ (SM-DP+ -- eUICC): LoadProfileElements	51

4.3.12	ES9+ (LPA -- SM-DP+): InitiateAuthentication	51

4.3.13	ES9+ (LPA -- SM-DP+): GetBoundProfilePackage	55

4.3.14	ES9+ (LPA -- SM-DP+): AuthenticateClient	74

4.3.15	ES9+ (LPA -- SM-DP+): HandleNotification	105

4.3.16	ES9+ (LPA -- SM-DP+): CancelSession	115

4.3.17	ES9+ (LPA -- SM-DP+): TLS, Server Authentication, Session Establishment	129

4.3.18	ES12 (SM-DP+ -- SM-DS): RegisterEvent	130

4.3.19	ES12 (SM-DP+ -- SM-DS): DeleteEvent	130

4.3.20	ES12 (SM-DP+ -- SM-DS): TLS, Mutual Authentication, Client, Session Establishment	130

4.4	VOID	131

4.5	SM-DS Interfaces	131

4.5.1	ES12 (SM-DP+ -- SM-DS): RegisterEvent	131

4.5.2	ES12 (SM-DS -- SM-DP+): DeleteEvent	139

4.5.3	ES15 (SM-DS -- SM-DS): RegisterEvent	145

4.5.4	ES15 (SM-DS -- SM-DS): DeleteEvent	147

4.5.5	ES11 (LPA -- SM-DS): InitiateAuthentication	149

4.5.6	ES11 (LPA -- SM-DS): AuthenticateClient	150

4.5.7	ES15 (SM-DS -- SM-DS): TLS, Mutual Authentication, Client, Session Establishment	165

4.5.8	ES12 (SM-DS -- SM-DP+): TLS, Mutual Authentication, Server, Session Establishment	166

4.5.9	ES15 (SM-DS -- SM-DS): TLS, Mutual Authentication, Server, Session Establishment	166

4.5.10	ES11 (LPA -- SM-DS): TLS, Server Authentication, Session Establishment	167

4.6	TLS Interface	167

4.6.1	TLS, Mutual Authentication, Client, TLS Establishment	167

4.6.2	TLS, Mutual Authentication, Server, TLS Establishment	178

4.6.3	TLS, Server Authentication, TLS Establishment	186

4.7	LPAe Interfaces	189

5	Procedure - Behaviour Testing	189

5.1	General Overview	189

5.2	VOID	189

5.3	Platform Procedures	189

5.3.1	Profile Download and Installation Procedure	189

5.3.2	Common Mutual Authentication Process	189

5.3.3	Profile Download and Installation Process	190

5.4	VOID	193

6	VOID	194

7	External Test Specifications	194

7.1	VOID	194

Annex A	Constants	195

A.1	Generic Constants	195

A.2	Test Certificates and Test Keys	208

Annex B	Dynamic Content	217

Annex C	Methods And Procedures	227

C.1	Methods	227

C.2	Procedures	244

Annex D	Commands And Responses	262

D.1	ES8+ Requests And Responses	262

D.1.1 ES8+ Requests	262

D.2	ES9+ Requests And Responses	279

D.2.1	ES9+ Requests	279

D.2.2	ES9+ Responses	315

D.3	ES10x Requests And Responses	330

D.3.1	ES10x Requests	330

D.3.2	ES10x Responses	342

D.4	APDU	368

D.4.1	APDU Commands	368

D.4.2	R-APDU Chaining	369

D.5	ES6 Requests And Responses	370

D.5.1	ES6 Requests	370

D.6	ES11 Requests And Responses	371

D.6.1	ES11 Requests	371

D.6.2	ES11 Responses	378

D.7	ES12 Requests And Responses	381

D.8	ES15 Requests And Responses	381

D.9	Common Server Responses	381

D.10	ES2+ Requests And Responses	391

Annex E	Profiles	392

Annex F	IUT Settings	407

F.1	VOID	407

F.2	Platforms Settings	407

F.3	VOID	407

F.4	Common Settings	408

Annex G	Initial States	409

G.1	VOID	409

G.2	VOID	409

G.3	SM-DP+ and SM-DS	409

Annex H	Icons and QR Codes	410

Annex I	Requirements	411

Annex J	VOID	412

Annex K	Document Management	413

K.1	Document History	413

K.2	Other Information	417

# Introduction

## 1.1	Overview

The main aim of the GSMA Remote SIM Provisioning specifications [2] & [3] is to provide solution for the Remote SIM Provisioning of Consumer Devices. The adoption of this technical solution will provide the basis for global interoperability between different Operator deployment scenarios, for example network equipment (e.g. Subscription Manager Data Preparation (SM-DP+)) and various eUICC platforms.

This Test Plan provides a set of test cases to be used for testing the implementations of the provisioning system specifications documents [2] & [3]. This document offers to the involved entities an unified test strategy and ensures interoperability between different implementations.

## 1.2	Scope

This document is intended for:

Parties which develop test tools and platforms

Vendors (SM-DP+ and SM-DS Providers)

Operators

The Test Plan consists of a set of relevant test cases for testing all entities involved in the eUICC remote provisioning system. The Implementations Under Test (IUT) are:

the SM-DP+

the SM-DS

The testing scopes developed in this document are:

Interface compliance testing: Test cases to verify the compliance of the interfaces within the system.

System behaviour testing: Test cases to verify the functional behaviour of the system.

Each test case specified within this Test Plan refers to one or more requirements.

The Test Plan contains test cases for the following versions of SGP.22:

GSMA RSP Technical Specification V3.1 [2]

This document includes an applicability table providing an indication whether test cases are relevant for a specific entity.

## 1.3	Definition of Terms

In addition to the terms which are defined below, the terms defined in SGP.22 [2] also apply

| Term | Description |
| --- | --- |
| End User | The person using the Device. |
| Integrated eUICC Test Interface | An external interface provided by its manufacturer for the purpose of testing eUICC functionality. |
| Standalone Device | A Device which provides all the capabilities to be able to be used in an RSP environment and needs no other Device for the purpose of Remote SIM Provisioning. |
| Test Plan | Current document describing the test cases that allow the RSP ecosystem to be tested. |

## 1.4	Abbreviations

In addition to the abbreviations which are defined below, the abbreviations defined in SGP.22 [2] also apply.

| Abbreviation | Description |
| --- | --- |
| APDU | Application Protocol Data Unit |
| ATR | Answer To Reset |
| C-APDU | Command APDU |
| CCID | (USB) Chip Card Interface Device |
| DER TLV | Distinguished Encoding Rules - Tag Length Value |
| FCP | File Control Parameters |
| HW | Hardware |
| IUT | Implementation Under Test |
| KVN | Key Version Number |
| OCE | Off-Card Entity |
| OS | Operating System |
| PIR | Profile Installation Result |
| POR | Proof Of Receipt |
| R-APDU | Response APDU |
| SoC | System on a Chip |
| SP | Service Provider |
| SSD | Supplemental Security Domain |
| USB | Universal Serial Bus |

## 1.5	Document Cross-references

| Ref | Document Number | Title |
| --- | --- | --- |
| [1] | SGP.02 | GSMA "Remote Provisioning of Embedded UICC Technical specification" V3.1 |
| [2] | SGP.22 | RSP Technical Specification V3.1 |
| [3] | SGP.21 | RSP Architecture V3.1 |
| [4] | eUICC Profile Package | Trusted Connectivity Alliance (formerly SIMalliance) eUICC Profile Package: Interoperable Format Technical Specification V2.1 or later |
| [5] | ETSI TS 102 221 | Smart Cards; UICC-Terminal interface |
| [6] | GPC_SPE_034 | GlobalPlatform Card Specification v.2.3 |
| [7] | ISO/IEC 7816-4:2013 | Identification cards – Integrated circuit cards - Part 4: Organization, security and commands for interchange |
| [8] | RFC 5639 | Elliptic Curve Cryptography (ECC) Brainpool Standard Curves and Curve Generation |
| [9] | ANSSI ECC FRP256V1 | Avis relatif aux paramètres de courbes elliptiques définis par l'Etat français. JORF n°0241 du 16 octobre 2011 page 17533. texte n° 30. 2011 |
| [10] | ITU E.118 | The international telecommunication charge card |
| [11] | NIST SP 800-56A | NIST Special Publication SP 800-56A: Recommendation for Pair-Wise Key Establishment Schemes Using Discrete Logarithm Cryptography (Revision 2), May 2013 |
| [12] | 3GPP TS 23.003 | Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); Numbering, addressing and identification |
| [13] | ETSI TS 102 225 | Secured packet structure for UICC based applications; Release 12 |
| [14] | ETSI TS 102 226 | Remote APDU structure for UICC based applications; Release 9 |
| [15] | TS.26 | GSMA NFC Handset Requirements V9.0 |
| [16] | ITU-T X.690 (11/2008) | ASN.1 Encoding Rules: Specification of Basic Encoding Rules (BER), Canonical Encoding Rules (CER) and Distinguished Encoding Rules (DER) including Corrigendum 1 and 2 |
| [17] | ETSI TS 102 241 | Smart cards; UICC Application Programming Interface (UICC API) for Java Card™ |
| [18] | 3GPP TS 31.102 | Characteristics of the Universal Subscriber Identity Module (USIM) application |
| [19] | GPC_SPE_095 | GlobalPlatform Card - Digital Letter of Approval - Version 1.0 |
| [20] | RFC 2119 | Key words for use in RFCs to Indicate Requirement Levels, S. Bradner http://www.ietf.org/rfc/rfc2119.txt |
| [21] | Void |  |
| [22] | 3GPP TS 23.040 | Technical realization of the Short Message Service (SMS) |
| [23] | TCA Test | Trusted Connectivity Alliance (TCA) eUICC Profile Package: Interoperable Format Test Specification Version 3.1 |
| [24] | RFC 4492 | Elliptic Curve Cryptography (ECC) Cipher Suites for Transport Layer Security (TLS) |
| [25] | SGP.26 | RSP Test Certificates Definition v1.5 |
| [26] | 3GPP TS 29.002 | Mobile Application Part (MAP) specification |
| [27] | RFC 5246 | The Transport Layer Security (TLS) Protocol Version 1.2 |
| [28] | GSMA PRD AA.35 | Procedures for Industry Specifications Product |
| [29] | CCID Rev 1.1 | CCID Specification for Integrated Circuit(s) Cards Interface Devices |

## 1.6	Conventions

The key words "SHALL", "SHALL NOT", "SHOULD", "SHOULD NOT", and "MAY" in this document SHALL be interpreted as described in RFC 2119 [20].

# Testing Rules

## Applicability

### Format of the Optional Features Table

The columns in Table 4 have the following meaning:

| Column | Meaning |
| --- | --- |
| Option | The optional feature supported or not by the implementation. |
| Mnemonic | The mnemonic column contains mnemonic identifiers for each item. |

- Table 1: Format of the Optional Features Table

### Format of the Applicability Table

The applicability of every test in Table 5 is formally expressed by the use of a Boolean expression defined in the following clause.

The columns in Table 5 have the following meaning:

| Column | Meaning |
| --- | --- |
| Test case | The "Test case" column gives a reference to the test case number detailed in the present document and is required to validate the implementation of the corresponding item in the "Name" column. |
| Name | In the "Name" column, a short non-exhaustive description of the test is found. |
| Roles | SM-DP+, SM-DS, Device, LPAd, LPAe or eUICC Entities under test that take in charge the functions used in the test case. |
| Version | This column indicates which test cases are applicable for the given SGP.22 version. See clause 2.1.3 'Applicability and Notations'. |
| Test Env. | Test environment used for executing the test case. |

- Table 2: Format of the Applicability Table

### Applicability and Notations

The following notations are used for the Applicability column:

| Applicability code | Meaning |
| --- | --- |
| M | mandatory - the capability is required to be supported. |
| N/A | not applicable - in the given context, it is impossible to use the capability. |
| Ci | conditional - the requirement on the capability depends on the support of other items. "i" is an integer identifying an unique conditional status expression which is defined immediately following the table. For nested conditional expressions, the syntax "IF ... THEN (IF ... THEN ... ELSE...) ELSE ..." is to be used to avoid ambiguities. |

- Table 3: Applicability and Notations

### Optional Features Table

The supplier of the implementation SHALL state the support of possible options in Table 5.

| SM-DP+ Options | Mnemonic |
| --- | --- |
| SM-DP+ reuses otPK.EUICC.AKA from previous unsuccessful download attempt | O_P_REUSE_OTPK |
| SM-DP+ supports usage of session keys (S-ENC, S-MAC) for profile protection | O_P_SESSION_KEYS |
| SM-DP+ accepts receiving two identical function call successively via ES2+ | O_P_ES2+_RETRY |
| SM-DP+ supports brainpoolP256r1 for TLS handshake | O_P_TLS_BRP |
| SM-DS Options | Mnemonic |
| SM-DS is an Alternative SM-DS. NOTE: If an SM-DS is not an Alternative SM-DS then it is a Root SM-DS. | O_S_ALT |
| SM-DS supports brainpoolP256r1 for TLS handshake | O_S_TLS_BRP |

- Table 4: Options

### Applicability Table

Table 5 specifies the applicability of each test case. See clause 2.1.2 for the format of this table.

| Test case | Name | Role |  |  |  |  | V3.0 | Test Env. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SM-DP+ Interfaces Compliance Testing |  |  |  |  |  |  |  |  |
| 4.3.1.2.1 | TC_SM-DP+_ES2+.DownloadOrder | SM-DP+ |  |  |  |  | M | TE_P3 |
| 4.3.1.2.2 | TC_SM-DP+_ES2+.DownloadOrder_RetryCases | SM-DP+ |  |  |  |  | C042 | TE_P3 |
| 4.3.1.2.3 | TC_SM-DP+_ES2+.DownloadOrder_ErrorCases | SM-DP+ |  |  |  |  | M | TE_P3 |
| 4.3.2.2.1 | TC_SM-DP+_ES2+.ConfirmOrder | SM-DP+ |  |  |  |  | M | TE_P3 |
| 4.3.2.2.2 | TC_SM-DP+_ES2+.ConfirmOrder_RetryCases | SM-DP+ |  |  |  |  | C042 | TE_P3 |
| 4.3.2.2.3 | TC_SM-DP+_ES2+.ConfirmOrder_ErrorCases | SM-DP+ |  |  |  |  | M | TE_P3 |
| 4.3.3.2.1 | TC_SM-DP+_ES2+.CancelOrder | SM-DP+ |  |  |  |  | M | TE_P3 |
| 4.3.3.2.2 | TC_SM-DP+_ES2+.CancelOrder_ErrorCases | SM-DP+ |  |  |  |  | M | TE_P3 |
| 4.3.12.2.1 | TC_SM-DP+_ES9+.InitiateAuthenticationNIST | SM-DP+ |  |  |  |  | M | TE_P2 |
| 4.3.12.2.2 | TC_SM-DP+_ES9+.InitiateAuthenticationFRP | SM-DP+ |  |  |  |  | M | TE_P2 |
| 4.3.12.2.3 | TC_SM-DP+_ES9+.InitiateAuthenticationBRP | SM-DP+ |  |  |  |  | M | TE_P2 |
| 4.3.13.2.1 | TC_SM-DP+_ES9+.GetBoundProfilePackageNIST Test sequences #1, #2 and #5 | SM-DP+ |  |  |  |  | C028 | TE_P2 |
| 4.3.13.2.1 | TC_SM-DP+_ES9+.GetBoundProfilePackageNIST Test sequences #3, #4 and #6 | SM-DP+ |  |  |  |  | M | TE_P2 |
| 4.3.13.2.2 | TC_SM-DP+_ES9+.GetBoundProfilePackageFRP | SM-DP+ |  |  |  |  | M | TE_P2 |
| 4.3.13.2.3 | TC_SM-DP+_ES9+.GetBoundProfilePackageBRP Test sequence #1 | SM-DP+ |  |  |  |  | C028 | TE_P2 |
| 4.3.13.2.3 | TC_SM-DP+_ES9+.GetBoundProfilePackageBRP Test sequence #2 | SM-DP+ |  |  |  |  | M | TE_P2 |
| 4.3.13.2.4 | TC_SM-DP+_ES9+.GetBoundProfilePackage_RetryCases_ReuseOTPK_NIST Test sequences #1, #2, #5 and #6 | SM-DP+ |  |  |  |  | C029 | TE_P2 |
| 4.3.13.2.4 | TC_SM-DP+_ES9+.GetBoundProfilePackage_RetryCases_ReuseOTPK_NIST Test sequences #3, #4, #7, #8 and #9 | SM-DP+ |  |  |  |  | C015 | TE_P2 |
| 4.3.13.2.7 | TC_SM-DP+_ES9+.GetBoundProfilePackage_RetryCases_DifferentOTPK_NIST Test sequences #1 and #2 | SM-DP+ |  |  |  |  | C030 | TE_P2 |
| 4.3.13.2.7 | TC_SM-DP+_ES9+.GetBoundProfilePackage_RetryCases_DifferentOTPK_NIST Test sequences #3 and #4 | SM-DP+ |  |  |  |  | C016 | TE_P2 |
| 4.3.13.2.10 | TC_SM-DP+_ES9+.GetBoundProfilePackage_ErrorCasesNIST | SM-DP+ |  |  |  |  | M | TE_P2 |
| 4.3.14.2.1 | TC_SM-DP+_ES9+.AuthenticateClientNIST | SM-DP+ |  |  |  |  | M | TE_P2 |
| 4.3.14.2.2 | TC_SM-DP+_ES9+.AuthenticateClientNIST_ErrorCases | SM-DP+ |  |  |  |  | M | TE_P2 |
| 4.3.14.2.3 | TC_SM-DP+_ES9+.AuthenticateClientFRP | SM-DP+ |  |  |  |  | M | TE_P2 |
| 4.3.14.2.5 | TC_SM-DP+_ES9+.AuthenticateClientBRP | SM-DP+ |  |  |  |  | M | TE_P2 |
| 4.3.14.2.6 | TC_SM-DP+_ES9+.AuthenticateClient_RetryCases_Reuse_OTPK | SM-DP+ |  |  |  |  | C015 | TE_P2 |
| 4.3.15.2.1 | TC_SM-DP+_ES9+_HandleNotificationNIST | SM-DP+ |  |  |  |  | M | TE_P2 |
| 4.3.15.2.2 | TC_SM_DP+_ES9+_HandleNotificationFRP | SM-DP+ |  |  |  |  | M | TE_P2 |
| 4.3.15.2.3 | TC_SM-DP+_ES9+_HandleNotificationBRP | SM-DP+ |  |  |  |  | M | TE_P2 |
| 4.3.16.2.1 | TC_SM-DP+_ES9+.CancelSession_After_AuthenticateClientNIST | SM-DP+ |  |  |  |  | M | TE_P2 |
| 4.3.16.2.2 | TC_SM-DP+_ES9+.CancelSession_After_GetBoundProfilePackageNIST | SM-DP+ |  |  |  |  | M | TE_P2 |
| 4.3.16.2.3 | TC_SM_DP+_ES9+.CancelSession_After_AuthenticateClientFRP | SM-DP+ |  |  |  |  | M | TE_P2 |
| 4.3.16.2.4 | 4.3.16.2.4 TC_SM_DP+_ES9+.CancelSession_After_GetBoundProfilePackageFRP | SM-DP+ |  |  |  |  | M | TE_P2 |
| 4.3.16.2.5 | TC_SM-DP+_ES9+.CancelSession_After_AuthenticateClientBRP | SM-DP+ |  |  |  |  | M | TE_P2 |
| 4.3.16.2.6 | TC_SM-DP+_ES9+.CancelSession_After_GetBoundProfilePackageBRP | SM-DP+ |  |  |  |  | M | TE_P2 |
| 4.3.17.1 | TC_SM-DP+_ES9+_Server_Authentication_for_HTTPS_EstablishmentNIST | SM-DP+ |  |  |  |  | M | TE_P2 |
| 4.3.17.2 | TC_SM-DP+_ES9+_Server_Authentication_for_HTTPS_EstablishmentBRP | SM-DP+ |  |  |  |  | C053 | TE_P2 |
| 4.3.20.1 | TC_SM-DP+_ES12_Client_Mutual_Authentication_for_HTTPS_EstablishmentNIST | SM-DP+ |  |  |  |  | M | TE_P1 |
| 4.3.20.2 | TC_SM-DP+_ES12_Client_Mutual_Authentication_for_HTTPS_EstablishmentBRP | SM-DP+ |  |  |  |  | C053 | TE_P1 |
| SM-DS Interfaces Compliance Testing |  |  |  |  |  |  |  |  |
| 4.5.1.2.1 | TC_ROOT_SM_DS_ES12.RegisterEvent | SM-DS |  |  |  |  | C024 | TE_S3 |
| 4.5.1.2.2 | TC_ALT_SM_DS_ES12.RegisterEvent | SM-DS |  |  |  |  | C021 | TE_SA2 |
| 4.5.2.2.1 | TC_ROOT_SM_DS_ES12.DeleteEvent | SM-DS |  |  |  |  | C024 | TE_S3 |
| 4.5.2.2.2 | TC_ALT_SM_DS_ES12.DeleteEvent | SM-DS |  |  |  |  | C021 | TE_SA2 |
| 4.5.2.2.3 | TC_ALT_SM_DS_ES12.DeleteEvent_Error_Nonexistant_EventID | SM-DS |  |  |  |  | C021 | TE_S2 |
| 4.5.3.2.1 | TC_ROOT_SM_DS_ES15.RegisterEvent | SM-DS |  |  |  |  | C024 | TE_SR2 |
| 4.5.4.2.1 | TC_ROOT_SM_DS_ES15.DeleteEvent | SM-DS |  |  |  |  | C024 | TE_SR2 |
| 4.5.5.2.1 | TC_SM_DS_ES11.InitiateAuthenticationNIST | SM-DS |  |  |  |  | M | TE_S1 |
| 4.5.5.2.2 | TC_SM_DS_ES11.InitiateAuthenticationBRP | SM-DS |  |  |  |  | M | TE_S1 |
| 4.5.6.2.1 | TC_SM_DS_ES11.AuthenticateClientNIST All test sequences except sequences #07 and #08 | SM-DS |  |  |  |  | M | TE_S1 |
| 4.5.6.2.1 | TC_SM_DS_ES11.AuthenticateClientNIST Only test sequences #07 and #08 | SM-DS |  |  |  |  | C021 | TE_S1 |
| 4.5.6.2.2 | TC_SM_DS_ES11.AuthenticateClientBRP All test sequences except sequences #07 and #08 | SM-DS |  |  |  |  | M | TE_S1 |
| 4.5.6.2.2 | TC_SM_DS_ES11.AuthenticateClientBRP Only test sequences #07 and #08 | SM-DS |  |  |  |  | C021 | TE_S1 |
| 4.5.7.1 | TC_ALT_SM_DS_ES15_Client_Mutual_Authentication_for_HTTPS_EstablishmentNIST | SM-DS |  |  |  |  | C021 | TE_SA1 |
| 4.5.7.2 | TC_ALT_SM_DS_ES15_Client_Mutual_Authentication_for_HTTPS_EstablishmentBRP | SM-DS |  |  |  |  | C055 | TE_SA1 |
| 4.5.8.1 | TC_SM_DS_ES12_Server_Mutual_Authentication_for_HTTPS_EstablishmentNIST | SM-DS |  |  |  |  | M | TE_S2 |
| 4.5.8.2 | TC_SM_DS_ES12_Server_Mutual_Authentication_for_HTTPS_EstablishmentBRP | SM-DS |  |  |  |  | C054 | TE_S2 |
| 4.5.9.1 | TC_ROOT_SM_DS_ES15_Server_Mutual_Authentication_for_HTTPS_EstablishmentNIST | SM-DS |  |  |  |  | C024 | TE_SR1 |
| 4.5.9.2 | TC_ROOT_SM_DS_ES15_Server_Mutual_Authentication_for_HTTPS_EstablishmentBRP | SM-DS |  |  |  |  | C056 | TE_SR1 |
| 4.5.10.1 | TC_SM_DS_ES11_Server_Authentication_for_HTTPS_EstablishmentNIST | SM-DS |  |  |  |  | M | TE_S1 |
| 4.5.10.2 | TC_SM_DS_ES11_Server_Authentication_for_HTTPS_EstablishmentBRP | SM-DS |  |  |  |  | C054 | TE_S1 |
| 4.6.1.2.1 | TC_Client_Mutual_Authentication_for_HTTPS_EstablishmentNIST |  |  |  |  |  |  |  |
| 4.6.1.2.2 | TC_Client_Mutual_Authentication_for_HTTPS_EstablishmentBRP |  |  |  |  |  |  |  |
| 4.6.2.2.1 | TC_Server_Mutual_Authentication_for_HTTPS_EstablishmentNIST |  |  |  |  |  |  |  |
| 4.6.2.2.2 | TC_Server_Mutual_Authentication_for_HTTPS_EstablishmentBRP |  |  |  |  |  |  |  |
| 4.6.3.2.1 | TC_Server_Authentication_for_HTTPS_EstablishmentNIST |  |  |  |  |  |  |  |
| 4.6.3.2.2 | TC_Server_Authentication_for_HTTPS_EstablishmentBRP |  |  |  |  |  |  |  |
| Procedure - Behaviour Testing |  |  |  |  |  |  |  |  |
| 5.3.3.2.1 | TC_SM-DP+_ProfileMetadata | SM-DP+ |  |  |  |  | M |  |

- Table 5: Applicability of Tests

| Conditional item | Condition |
| --- | --- |
| C015 | IF (O_P_REUSE_OTPK) THEN M ELSE N/A |
| C016 | IF (NOT O_P_REUSE_OTPK) THEN M ELSE N/A |
| C021 | IF (O_S_ALT) THEN M ELSE N/A |
| C024 | IF (NOT O_S_ALT) THEN M ELSE N/A |
| C028 | IF (O_P_SESSION_KEYS) THEN M ELSE N/A |
| C029 | IF (O_P_SESSION_KEYS AND O_P_REUSE_OTPK) THEN M ELSE N/A |
| C030 | IF (O_P_SESSION_KEYS AND NOT O_P_REUSE_OTPK) THEN M ELSE N/A |
| C042 | IF (O_P_ES2+_RETRY) THEN M ELSE N/A |
| C053 | IF (O_P_TLS BRP) THEN M ELSE N/A |
| C054 | IF (O_S_TLS BRP) THEN M ELSE N/A |
| C055 | IF (O_S_ALT AND O_S_TLS_BRP) THEN M ELSE N/A |
| C056 | IF (NOT O_S_ALT AND O_S_TLS_BRP) THEN M ELSE N/A |
| Conditions applicable to SGP.23 v3.1 only |  |

- Table 6: Conditional Items Referenced by Table 5

Note: Conditions C0XX which are missing in Table 6 are present in an earlier version of SGP.23 but are not used in the current version.

## General Consideration

This section contains some general considerations about the test cases defined in this document. Note that some external test specifications are referred to in chapter 7. Consequently, the following sub sections SHALL only apply for test cases defined in sections 4 and 5 and 6.

### Test Case Definition

Test descriptions are independent.

For each test described in this document, a chapter provides a general description of the initial conditions applicable for the whole test. This description is completed by specific configurations to each individual sub-case.

It is implicitly assumed that all entities under test SHALL be compliant with the initial states described in Annex G. An initial state SHALL be considered as a pre-requisite to execute all the test cases described in this Test Plan.

After completing the test, the configuration is reset before the execution of the following test.

### Test Cases Format

Here is an explanation of the way to define the test cases in chapters 4, 5 and 6.

4.X.Y.Z Test Cases 4.X.Y.Z.1 TC_IUT_TestName1 General Initial Conditions. Entity; Description of the general initial condition. Entity1; Test case - general condition 1. Entity2; Test case - general condition 2. Test Sequence #01: Short Description Description of the aim of the test sequence N°1 Initial Conditions. Entity; Description of the initial condition. Entity1; Test sequence N°1 - initial condition 1. Entity2; Test sequence N°1 - initial condition 2. Step; Direction; Sequence / Description; Expected result; REQ. IC1; Entity1 → Entity2; Command or Message to send from Entity1 to Entity2; Expected result N°1.1. 1; Entity1 → Entity2; Command or Message to send from Entity1 to Entity2; 1- expected result N°1.2 2- expected result N°1.3; REQ1. 2; Entity2 → Entity3; Command or Message to send from Entity2 to Entity3. Test Sequence #02 Description of the aim of the test sequence N°2 Step; Direction; Sequence / Description; Expected result; REQ. 1; Entity1 → Entity2; Command or Message to send from Entity1 to Entity2. 2; Entity2 → Entity3; Command or Message to send from Entity2 to Entity3; 1- expected result N°2.1 2- expected result N°2.2; REQ2. 4.X.Y.Z.2 TC_IUT_TestName2 …

The test cases TC_IUT_TestName1 and TC_IUT_TestName2 are referenced in Table 5 that allows indicating the applicability of the tests.

In the test case TC_IUT_TestName1, the requirements REQ1 and REQ2 are respectively covered by the test sequences #01 and #02.

Note: For some test cases, requirements to be covered are not listed in the test sequences. In that case, references to sections in GSMA RSP Technical Specification [2] covered by the test sequences are indicated in the Conformance Requirements References section of the test case.

The test sequence #01 SHALL be executed if and only if these conditions are met:

Test case - general condition 1

Test case - general condition 2

Test sequence N°1 - initial condition 1

Test sequence N°1 - initial condition 2

The test sequence #02 SHALL be executed if and only if these conditions are met:

Test case - general condition 1

Test case - general condition 2

The tables defining the different initial conditions are optional.

Initial Conditions are intended to be reached dynamically using the Test Tool when possible.

No additional operation SHALL be done prior to the test sequence besides those indicated in the Initial Conditions (e.g. no other Profiles SHALL be present on the eUICC besides those defined in the Initial Conditions).

In the test sequence #01:

the step IC1 corresponds to an additional Initial Condition

in the step N°1, if the expected results N°1 and N°2 are validated, the requirement REQ1 (or a part of the REQ1) SHALL be considered as implemented

Note that all initial states (described in Annex G) SHALL be implemented by the entity under test whatever the test cases to execute.

In addition, following 2.2.1 sub sections present all information (e.g. Methods, Constants…) that MAY be referenced in test sequences.

After execution of each test sequence a clean-up procedure (CU) SHALL be executed to restore the IUT to the Common Initial State as defined in Annex G.

#### Methods and Procedures

A method is referenced as follow:

- 	MTD_NAME_OF_THE_METHOD(PARAM1, PARAM2…)

The key word “NO_PARAM” SHALL be set in method call if the related optional parameter is not used.

All methods and their related parameters are described in Annex C.1.

A procedure is a generic sub-sequence and is referenced as follow:

PROC_NAME_OF_THE_PROCEDURE

All procedures are described in Annex C.2.

The implementation of these methods and procedures is under the responsibility of the test tool providers.

#### Constants and Dynamic Content

A constant (e.g. text, ASN.1 structure, hexadecimal string, icon, URI, integer, EID, AID…) is referenced as follow:

- 	#NAME_OF_THE_CONSTANT

All constants are defined in Annex A.

When provided as an ASN.1 value notation, a constant SHALL be encoded in DER TLV (as specified in ITU-T X.690 [16]) by the test tool.

A dynamic content (e.g. TLV, ASN.1 structure, signature, integer, AID, one-time key pair…) is referenced as follow:

- 	<NAME_OF_THE_VARIABLE>

All dynamic contents are defined in Annex B.

A dynamic content is either generated by an IUT or by a test tool provider.

#### Requests and Responses

An ASN.1 or a JSON request is referenced as follow:

- 	#NAME_OF_THE_REQUEST

An ASN.1 or a JSON response is referenced as follows:

- 	#R_NAME_OF_THE_RESPONSE

Each ASN.1 or JSON request and response MAY refer to a constant or a dynamic content. All these structures are defined in Annex D.

When provided as an ASN.1 value notation, a request or a response SHALL be encoded in DER TLV (as specified in ITU-T X.690 [16]) by the test tool.

When an ASN.1 element definition contains three points (i.e. “…”), it means that fields MAY be present but SHALL not be checked by the test tool.

In the following example, several fields MAY be part of the ProfileInfoListResponse but only the profileNickname SHALL be verified.

resp ProfileInfoListResponse ::=

profileInfoListOk :{

{ 
       ...
       profileNickname #NICKNAME
       ...

}

}

This rule applies also for Constants definition.

Some ASN.1 SEQUENCE components have a DEFAULT value (for example, profileClass in StoreMetadataRequest). In this specification, when values are specified in ASN.1 syntax and the DEFAULT value is intended, two different formulations (both of which are valid) may be used:

the relevant component is specified with the DEFAULT value;

the relevant component is missing entirely.

These are logically equivalent and lead to the same DER encoding. In both cases, the following rules apply:

When the test tool is sending the DER value, it SHALL NOT include the component (as per DER rules).

When the test tool is checking a received DER value from the entity under test, it SHALL check that the component is NOT present.

Test tools SHALL consider two BIT STRINGs to be equivalent if the BIT STRINGs have the same DER encoding. For example, '0101'B shall be considered to be equivalent to '010100'B.

NOTE:	this is equivalent to removing any trailing zero bits from the BIT STRINGs in "bstring" notation (e.g. '010100'B  '0101'B) and then comparing the strings textually.

NOTE:	according to the DER format, the encoding of transmitted values will remove the trailing zeroes. The definition above allows for values which are specified using ASN.1 value notation and are not transmitted, such as values specified in the Annexes of the current document, including IUT settings which might be specified by a user of the current document and may contain trailing zeroes in the ASN.1 value notation.

#### APDUs

A C-APDU is referenced as follow:

- 	[NAME_OF_THE_CAPDU]

All C-APDUs are defined in Annex D.4.

An R-APDU is referenced as follow:

- 	[R_NAME_OF_THE_RAPDU]

All R-APDUs are defined in Annex D.4.

Each APDU MAY refer to a constant or a dynamic content.

The APDU TERMINAL RESPONSE SHALL be dynamically generated by the test tool according to the related proactive command. Therefore, this particular command is not referenced with brackets in this specification. If not explicitly defined in the step, the general result SHALL be set by default to “Command performed successfully” (i.e. 0x83 01 00).

#### Profiles

In order to execute the test cases described in this document, Operational, Test and Provisioning Profiles are necessary. All these Profiles are defined in Annex E with the Profile Metadata content and the corresponding Profile Package as defined in the eUICC Profile Package Specification [4].

A Profile is referenced as follow:

- 	PROFILE_OPERATIONALx with x the identifier of the Operational Profile

or

- 	PROFILE_TESTx with x the identifier of the Test Profile

or

- 	PROFILE_PROVISIONINGx with x the identifier of the Provisioning Profile

NOTE:	Test Profiles and Provisioning Profiles are out of the scope of this version of test specification.

#### IUT Settings

For the purpose of some test cases, Device and eUICC manufacturers and Platforms (i.e. SM-DP+, SM-DS) providers need to give some information related to their products to the test tools providers (e.g. supported Java Card version).

An IUT setting is referenced as follow:

- 	#IUT_NAME_OF_SETTING

All these settings are defined in Annex F.

#### Referenced Requirements

All requirements referenced in this document by their identifiers are present and described in Annex I. These requirements have been extracted from the specifications:

- 	GSMA RSP Technical Specification [2]

- 	GSMA RSP Architecture [3]

### VOID

### VOID

### Pass Criteria

A test execution is considered as successful only if the test procedure was fully carried out successfully.

A test execution is considered as failed if the tested feature provides an unexpected behaviour.

A test execution is considered as inconclusive when the pass criteria cannot be evaluated due to issues during the setup of the initial conditions (including the ICx steps) or during the execution of steps in which no requirement is referenced.

### Future Study

Some of the test cases or test sequences described in this Test Plan are FFS (For Future Study). This MAY mean that some clarifications are expected at the requirement level to conclude on a test method. As consequence, the corresponding test SHALL not be executed.

### General Rules for SM-DP+ Testing

#### Default Profile Processing

By default, for ES2+ testing, the SM-DP+ SHALL use random keys to protect profiles.

# 3	Testing Architecture

## 3.1	Testing Scope

All the interfaces, intended to be tested in the scope of this document, are presented hereafter:

| Interface | Between |  | Description |
| --- | --- | --- | --- |
| ES2+ | Operator | SM-DP+ | Used by the Operator to order Profiles for specific eUICCs as well as other administrative functions. NOTE: this interface is out of scope of this specification. |
| ES6 | Operator | eUICC | Used by the Operator for the management of Operator services via OTA services. |
| ES8+ | SM-DP+ | eUICC | Provides a secure end-to-end channel between the SM-DP+ and the eUICC for the administration of the ISD-P and the associated Profile during download and installation. It provides Perfect Forward Secrecy. |
| ES9+ | SM-DP+ | LPD | Used to provide a secure transport between the SM-DP+ and the LPA (LPD) for the delivery of the Bound Profile Package and the delivery of Remote Profile Management Commands. |
| ES10a | LDSd | eUICC | Used between the LDSd and the LPA Services to handle a Profile discovery. |
| ES10b | LPDd | eUICC | Used between the LPDd and the LPA services to transfer a Bound Profile Package to the eUICC. This interface plays no role in the decryption of Profile Packages. |
| ES10c | LUId | eUICC | Used between the LUId and the LPA services for Local Profile Management by the End User. |
| ES11 | LDS | SM-DS | Used by the LDS to retrieve Event Records for the respective eUICC. |
| ES12 | SM-DP+ | SM-DS | Used by the SM-DP+ to issue or remove Event Registrations on the SM-DS. |
| ES15 | SM-DS | SM-DS | Used in the case of deployments of cascaded SM-DSs to connect those SM-DSs. |

- Table 7: Interfaces Descriptions

## 3.2	Testing Execution

This chapter aims to describe the different testing environments and equipments to allow the test cases to be executed.

To permit the execution of the different test cases described in this Test Plan, specifics simulators SHALL be used. The simulators that have been defined are listed hereafter:

- 	S_Device: the Device Simulator used to send some commands to the eUICC under test using ISO/IEC 7816-4 [7] on the contact interface

- 	S_SM-DP+: the SM-DP+ Simulator

- 	S_SM-DS: the SM-DS Simulator

- 	S_MNO: the MNO Simulator

- 	S_LPAd: the LPAd Simulator

- 	S_LPAe: the LPAe Simulator

- 	S_EndUser: the End User Simulator that acts as an End User. This simulator MAY be either a person (i.e. a Tester) or a software that simulates the End User interactions.

- 	S_CLIENT: the HTTPs client Simulator for the purpose of TLS testing. The S_CLIENT MAY be S_SM-DP+, S_SM-DS depending on the component under test.

- 	S_SERVER: the HTTPs server Simulator for the purpose of TLS testing. The S_SERVER MAY be S_SM-DP+ or S_SM-DS depending on the component under test.

- 	Implementation of these simulators remains under the responsibility of the test tool providers.

- 	The aim of all the test cases is to verify the compliance of an Actor/Component (i.e. eUICC, SM-DP+, Alternative SM-DS, Root SM-DS, LPAe, LPAd, Device).

Following notations are used:

- 	S_ComponentName for a simulated component

- 	ComponentName for the Implementation Under Test (IUT)

- 	Where ComponentName is indicated by CLIENT, SERVER

- 	Depending on the component under test, the CLIENT MAY be the SM-DP+ or the SM-DS. The Operator component is currently out of scope.

- Depending on the component under test, the SERVER MAY be the SM-DP+ or the SM-DS. The Operator component is currently out of scope.

- The use of "-- optional" in any ASN.1 elements defined within this document indicate that the test tool SHALL allow for the value either being present with that value, or being absent.

### 3.2.1	VOID

### 3.2.2	SM-DP+ and SM-DS - Test Environment

The following test environment is used for all SM-DP+ and SM-DS Interfaces related test cases as defined in chapter 4.3 and 4.5 (unless it is specified differently in the specific test case). Following conditions apply:

- 	SM-DS / SM-DP+ / LPA Simulators SHALL be implemented by the test tools

- 	Simulators act as a RSP server or a RSP client

- 	Definition of the TLS parameters/configuration is provided

- 	JSON (and ASN.1) input data are used (NOTE: ASN.1 format is out of scope of this specification)

#### 3.2.2.1	Test environment for SM-DP+ under test

Test Environment reference:

- 	TE_P1 (SM-DP+ on ES12)

SM-DP+

S_SM-DS

ES12

Test Environment reference:

- 	TE_P2 (SM-DP+ on ES9+)

S_LPAd

ES9+

SM-DP+

#### Test Environment reference:

- 	TE_P3 (SM-DP+ on ES2+)

SM-DP+

S_MNO

ES2+

S_LPAd

ES9+

S_SM-DS

ES12

#### 3.2.2.2	Test environment for SM-DS under test

Test Environment reference:

- 	TE_S1 (SM-DS on ES11)

ES11

S_LPAd

SM-DS

Test Environment reference:

- 	TE_S2 (SM-DS on ES12)

ES12

SM-DS

S_SM-DP+

Test Environment reference:

- 	TE_S3 (SM-DS on ES12 and ES11)

ES11

ES12

S_LPAd

SM-DS

S_SM-DP+

Test Environment reference:

- 	TE_SA1 (Alternative SM-DS on ES12 and ES15)

SM-DS
(alt)

(

S_SM-DP+

ES12

S_SM-DS
(root)

ES15

Test Environment reference:

TE_SA2 (Alternative SM-DS on ES12, ES15 and ES11)

SM-DS
(alt)

(

S_SM-DP+

ES12

S_SM-DS
(root)

ES15

S_LPAd

ES11

Test Environment reference:

- 	TE_SR1 (Root SM-DS on ES15)

ES15

S_SM-DS
(alt)

SM-DS
(root)

(

Test Environment reference:

- 	TE_SR2 (Root SM-DS on ES15 and ES11)

ES11

ES15

S_SM-DS
(alt)

S_LPAd

SM-DS
(root)

(

### 3.2.3	VOID

### 3.2.4	VOID

### 3.2.5	VOID

# 4	Interface Compliance Testing

## 4.1	General Overview

This section focuses on the implementation of the different interfaces according to the GSMA RSP Technical Specification [2]. The aim is to verify the compliance of all interfaces within the system.

## 4.2	VOID

## 4.3	SM-DP+ Interfaces

### 4.3.1	ES2+ (Operator -- SM-DP+): DownloadOrder

#### 4.3.1.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Section 5.3.1

#### 4.3.1.2	Test Cases

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in ‘Available’ state for S_MNO |
| S_MNO | For the TLS connection, CERT_CLIENT_TLS = #CERT_S_OPERATOR_TLS |

##### 4.3.1.2.1	TC_SM-DP+_ES2+.DownloadOrder

Test Sequence #01 Nominal: EID and ICCID

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO →SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_DOWNLOAD_ORDER, MTD_DOWNLOAD_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM)) | MTD_HTTP_RESP( #R_SUCCESS_ICCID1) |
| 2 | S_LPAd →SM-DP+ | PROC_ES9+_VERIFY_PROFILE_NOT_RELEASED_EMPTY_MID |  |

##### 4.3.1.2.2	TC_SM-DP+_ES2+.DownloadOrder_RetryCases

Test Sequence #01 Nominal

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_DOWNLOAD_ORDER, MTD_DOWNLOAD_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM)) | MTD_HTTP_RESP( #R_SUCCESS_ICCID1) |
| 2 | S_MNO → SM-DP+ | Close TLS session (unless SM-DP+ has already closed TLS session) |  |
| 3 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 4 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_DOWNLOAD_ORDER, MTD_DOWNLOAD_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM)) | MTD_HTTP_RESP( #R_SUCCESS_ICCID1) |

##### 4.3.1.2.3	TC_SM-DP+_ES2+.DownloadOrder_ErrorCases

Test Sequence #01 Error: ICCID already in use (8.2.1/3.3)

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| IC2 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_DOWNLOAD_ORDER, MTD_DOWNLOAD_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, NO_PARAM, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM)) | MTD_HTTP_RESP( #R_SUCCESS_ICCID1) |
| 1 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_DOWNLOAD_ORDER, MTD_DOWNLOAD_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM)) | MTD_HTTP_RESP( #R_ERROR_8_2_1_3_3) |

Test Sequence #02 Error: unknown profile (8.2.1/3.9)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | No Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in the SM-DP+ for S_MNO |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_DOWNLOAD_ORDER, MTD_DOWNLOAD_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM)) | MTD_HTTP_RESP( #R_ERROR_8_2_1_3_9) |

#### 4.3.1.3	Test Cases

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in ‘Available’ state for S_MNO |
| S_MNO | For the TLS connection, CERT_CLIENT_TLS = #CERT_S_OPERATOR_TLS |

##### 4.3.1.3.1	TC_SM-DP+_ES2+.DownloadOrder

Test Sequence #01 Nominal: EID and ICCID

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO →SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_DOWNLOAD_ORDER, MTD_DOWNLOAD_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM)) | MTD_HTTP_RESP( #R_SUCCESS_ICCID1) |
| 2 | S_LPAd →SM-DP+ | PROC_ES9+_VERIFY_PROFILE _NOT_RELEASED_EMPTY_MID |  |

##### 4.3.1.3.2	TC_SM-DP+_ES2+.DownloadOrder_RetryCases

Test Sequence #01 Nominal

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_DOWNLOAD_ORDER, MTD_DOWNLOAD_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM)) | MTD_HTTP_RESP( #R_SUCCESS_ICCID1) |
| 2 | S_MNO → SM-DP+ | Close TLS session (unless SM-DP+ has already closed TLS session) |  |
| 3 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 4 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_DOWNLOAD_ORDER, MTD_DOWNLOAD_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM)) | MTD_HTTP_RESP( #R_SUCCESS_ICCID1) |

##### 4.3.1.3.3	TC_SM-DP+_ES2+.DownloadOrder_ErrorCases

Test Sequence #01 Error: ICCID already in use (8.2.1/3.3)

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| IC2 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_DOWNLOAD_ORDER, MTD_DOWNLOAD_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, NO_PARAM, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM)) | MTD_HTTP_RESP( #R_SUCCESS_ICCID1) |
| 1 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_DOWNLOAD_ORDER, MTD_DOWNLOAD_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM)) | MTD_HTTP_RESP( #R_ERROR_8_2_1_3_3) |

Test Sequence #02 Error: unknown profile (8.2.1/3.9)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | No Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in the SM-DP+ for S_MNO |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_DOWNLOAD_ORDER, MTD_DOWNLOAD_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM)) | MTD_HTTP_RESP( #R_ERROR_8_2_1_3_9) |

### 4.3.2	ES2+ (Operator -- SM-DP+): ConfirmOrder

#### 4.3.2.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

- Section 5.3.2

#### 4.3.2.2	Test Cases

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| S_MNO | For the TLS connection, CERT_CLIENT_TLS #CERT_S_OPERATOR_TLS |

##### 4.3.2.2.1	TC_SM-DP+_ES2+.ConfirmOrder

Test Sequence #01 Nominal: using ‘Allocated’ state / ICCID / matching ID / confirmation code / releaseFlag=”true”

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in ‘Allocated’ state by S_MNO |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CONFIRM_ORDER, MTD_CONFIRM_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM, #MATCHING_ID_1, #CONFIRMATION_CODE1, NO_PARAM, NO_PARAM, TRUE)) | MTD_HTTP_RESP( #R_SUCCESS_MATCHING_ID) with <MATCHING_ID>= #MATCHING_ID_1 |
| 2 | S_LPAd →SM-DP+ | PROC_ES9+_VERIFY_PROFILE_RELEASED_WITH_MID_WITH_CC |  |

Test Sequence #02 Nominal: using ‘Allocated’ state / ICCID / empty matching ID / EID / confirmation code / releaseFlag=”true”

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in ‘Allocated’ state by S_MNO |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO →SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CONFIRM_ORDER, MTD_CONFIRM_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, #EID1, #MATCHING_ID_EMPTY, #CONFIRMATION_CODE1, NO_PARAM, NO_PARAM, TRUE)) | MTD_HTTP_RESP( #R_SUCCESS_MATCHING_ID_EID with <MATCHING_ID>= #MATCHING_ID_EMPTY) |
| 2 | S_LPAd →SM-DP+ | PROC_ES9+_VERIFY_PROFILE_RELEASED_EMPTY_MID_WITH_CC |  |

Test Sequence #03 Nominal: using ‘Allocated’ state / ICCID / matching ID / EID / confirmation code / releaseFlag=”true” / rootSmdsAddress

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in ‘Allocated’ state by S_MNO |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | S_MNO →SM-DP+ | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES2+ |  |
| 1 | S_MNO →SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CONFIRM_ORDER, MTD_CONFIRM_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, #EID1, #MATCHING_ID_1, #CONFIRMATION_CODE1, NO_PARAM, #TEST_ROOT_DS_ADDRESS, TRUE )) |  |
| 2 | SM-DP+ → S_SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES12 |  |
| 3 | SM-DP+ → S_SM-DS | Call ES12.RegisterEvent | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT_V3( <FUNCTION_REQ_ID>, <FUNCTION_CALL_ID>, #EID1, #IUT_SM_DP_ADDRESS, #MATCHING_ID_1, <FORWARDING_INDICATOR_ANY>, NO_PARAM, #EVENT_TYPE_PROFILE_DOWNLOAD, NO_PARAM, NO_PARAM, NO_PARAM, NO_PARAM )) |
| 4 | S_SM-DS → SM-DP+ | MTD_HTTP_RESP(#R_SUCCESS) on ES12 | No Error |
| 5 | SM-DP+ →S_MNO | Return final result | MTD_HTTP_RESP( #R_SUCCESS_MATCHING_ID_EID) on ES2+ with <MATCHING_ID>= #MATCHING_ID_1 |
| 6 | S_LPAd →SM-DP+ | PROC_ES9+_VERIFY_PROFILE_RELEASED_EMPTY_MID_WITH_CC |  |

Test Sequence #04 Nominal: using ‘Linked’ state / ICCID / matching ID / confirmation code / releaseFlag=”true”

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in ‘Linked’ state for S_MNO, and bound to #EID1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO →SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CONFIRM_ORDER, MTD_CONFIRM_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM, #MATCHING_ID_1, #CONFIRMATION_CODE1, NO_PARAM, NO_PARAM, TRUE)) | MTD_HTTP_RESP( #R_SUCCESS_MATCHING_ID_EID) with <MATCHING_ID>= #MATCHING_ID_1 |
| 2 | S_LPAd →SM-DP+ | PROC_ES9+_VERIFY_PROFILE_RELEASED_EMPTY_MID_WITH_CC |  |

Test Sequence #05 Nominal: using ‘Linked’ state / ICCID / empty matching ID / EID / confirmation code / releaseFlag=”true”

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in ‘Linked’ state for S_MNO, and bound to #EID1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CONFIRM_ORDER, MTD_CONFIRM_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, #EID1, #MATCHING_ID_EMPTY, #CONFIRMATION_CODE1, NO_PARAM, NO_PARAM, TRUE)) | MTD_HTTP_RESP( #R_SUCCESS_MATCHING_ID_EID with <MATCHING_ID>= #MATCHING_ID_EMPTY) |
| 2 | S_LPAd → SM-DP+ | PROC_ES9+_VERIFY_PROFILE_RELEASED_EMPTY_MID_WITH_CC |  |

Test Sequence #06 Nominal: using ‘Linked’ state / ICCID / matching ID / EID / confirmation code / releaseFlag=”true” / rootSmdsAddress

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in ‘Linked’ state for S_MNO, and bound to #EID1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | S_MNO→ SM-DP+ | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES2+ |  |
| 1 | S_MNO→ SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CONFIRM_ORDER, MTD_CONFIRM_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, #EID1, #MATCHING_ID_1, #CONFIRMATION_CODE1, NO_PARAM, #TEST_ROOT_DS_ADDRESS, TRUE )) |  |
| 2 | SM-DP+→ S_SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES12 |  |
| 3 | SM-DP+→ S_SM-DS | Call ES12.RegisterEvent | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT_V3( <FUNCTION_REQ_ID>, <FUNCTION_CALL_ID>, #EID1, #IUT_SM_DP_ADDRESS, #MATCHING_ID_1, <FORWARDING_INDICATOR_ANY>, NO_PARAM, #EVENT_TYPE_PROFILE_DOWNLOAD, NO_PARAM, NO_PARAM, NO_PARAM, NO_PARAM )) |
| 4 | S_SM-DS→ SM-DP+ | Return ES12.RegisterEvent result | MTD_HTTP_RESP(#R_SUCCESS) on ES12 |
| 5 | SM-DP+→ S_MNO | Return final result | MTD_HTTP_RESP( #R_SUCCESS_MATCHING_ID_EID) on ES2+ with <MATCHING_ID>= #MATCHING_ID_1 |
| 6 | S_LPAd→ SM-DP+ | PROC_ES9+_VERIFY_PROFILE_RELEASED_EMPTY_MID_WITH_CC |  |

##### 4.3.2.2.2	TC_SM-DP+_ES2+.ConfirmOrder_RetryCases

Test Sequence #01 Nominal: using ‘Allocated’ state

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in ‘Allocated’ state by S_MNO |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CONFIRM_ORDER, MTD_CONFIRM_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM, #MATCHING_ID_1, #CONFIRMATION_CODE1, NO_PARAM, NO_PARAM, TRUE)) | MTD_HTTP_RESP( #R_SUCCESS_MATCHING_ID) with <MATCHING_ID>= #MATCHING_ID_1 |
| 2 | S_MNO → SM-DP+ | Close TLS session (unless SM-DP+ has already closed TLS session) |  |
| 3 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 4 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CONFIRM_ORDER, MTD_CONFIRM_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM, #MATCHING_ID_1, #CONFIRMATION_CODE1, NO_PARAM, NO_PARAM, TRUE)) | MTD_HTTP_RESP( #R_SUCCESS_MATCHING_ID) with <MATCHING_ID>= #MATCHING_ID_1 |

Test Sequence #02 Nominal: using ‘Linked’ state

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in ‘Linked’ state by S_MNO and bound to #EID1 |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CONFIRM_ORDER, MTD_CONFIRM_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM, #MATCHING_ID_1, #CONFIRMATION_CODE1, NO_PARAM, NO_PARAM, TRUE)) | MTD_HTTP_RESP( #R_SUCCESS_MATCHING_ID_EID) with <MATCHING_ID>= #MATCHING_ID_1 |
| 2 | S_MNO → SM-DP+ | Close TLS session (unless SM-DP+ has already closed TLS session) |  |
| 3 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 4 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CONFIRM_ORDER, MTD_CONFIRM_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM, #MATCHING_ID_1, #CONFIRMATION_CODE1, NO_PARAM, NO_PARAM, TRUE)) | MTD_HTTP_RESP( #R_SUCCESS_MATCHING_ID_EID) with <MATCHING_ID>= #MATCHING_ID_1 |

Test Sequence #03 Error: different matchingID (unspecified Error Code)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in ‘Linked’ state by S_MNO and bound to #EID1 |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CONFIRM_ORDER, MTD_CONFIRM_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM, #MATCHING_ID_1, #CONFIRMATION_CODE1, NO_PARAM, NO_PARAM, TRUE)) | MTD_HTTP_RESP( #R_SUCCESS_MATCHING_ID_EID) with <MATCHING_ID>= #MATCHING_ID_1 |
| 2 | S_MNO → SM-DP+ | Close TLS session (unless SM-DP+ has already closed TLS session) |  |
| 3 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 4 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CONFIRM_ORDER, MTD_CONFIRM_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM, #MATCHING_ID_2, #CONFIRMATION_CODE1, NO_PARAM, NO_PARAM, TRUE)) | MTD_HTTP_RESP( #R_ERROR_ANY) |

Test Sequence #04 Error: different Confirmation Code (unspecified Error Code)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in ‘Linked’ state by S_MNO and bound to #EID1 |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CONFIRM_ORDER, MTD_CONFIRM_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM, #MATCHING_ID_1, #CONFIRMATION_CODE1, NO_PARAM, NO_PARAM, TRUE)) | MTD_HTTP_RESP( #R_SUCCESS_MATCHING_ID_EID) with <MATCHING_ID>= #MATCHING_ID_1 |
| 2 | S_MNO → SM-DP+ | Close TLS session (unless SM-DP+ has already closed TLS session) |  |
| 3 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 4 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CONFIRM_ORDER, MTD_CONFIRM_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM, #MATCHING_ID_1, #CONFIRMATION_CODE2, NO_PARAM, NO_PARAM, TRUE)) | MTD_HTTP_RESP( #R_ERROR_ANY) |

##### 4.3.2.2.3 TC_SM-DP+_ES2+.ConfirmOrder_ErrorCases

Test Sequence #01 Error: unknown Profile (8.2.1/3.9)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | No Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in the server for S_MNO |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CONFIRM_ORDER, MTD_CONFIRM_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM, #MATCHING_ID_1, #CONFIRMATION_CODE1, NO_PARAM, NO_PARAM, TRUE)) | MTD_HTTP_RESP( #R_ERROR_8_2_1_3_9) |

Test Sequence #02 Error: Profile in ‘Available’ state (unspecified Error Code)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in ‘Available’ state for S_MNO |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO →SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CONFIRM_ORDER, MTD_CONFIRM_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM, #MATCHING_ID_1, #CONFIRMATION_CODE1, NO_PARAM, NO_PARAM, TRUE)) | MTD_HTTP_RESP( #R_ERROR_ANY) |

Test Sequence #03 Error: conflicting matching ID (8.2.6/3.3)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in ‘Allocated’ state for S_MNO. The matchingID #MATCHING_ID_1 is already stored for S_MNO and associated to another Profile identified by ICCID_OP_PROF2_NON_SWAP. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO →SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CONFIRM_ORDER, MTD_CONFIRM_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM, #MATCHING_ID_1, #CONFIRMATION_CODE1, NO_PARAM, NO_PARAM, TRUE)) | MTD_HTTP_RESP( #R_ERROR_8_2_6_3_3) |

Test Sequence #04 Error: incorrect smdsAddress (8.9/5.1)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in ‘Linked’ state for S_MNO. |
| S_SM-DS | The S_SM-DS is not reachable through #TEST_DS_ADDRESS1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO →SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CONFIRM_ORDER, MTD_CONFIRM_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM, #MATCHING_ID_1, #CONFIRMATION_CODE1, NO_PARAM, #TEST_DS_ADDRESS1, TRUE)) | MTD_HTTP_RESP( #R_ERROR_8_9_5_1) |

Test Sequence #05 Error: missing EID (8.1.1/2.2)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in ‘Allocated’ state for S_MNO. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO →SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CONFIRM_ORDER, MTD_CONFIRM_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM, #MATCHING_ID_EMPTY , #CONFIRMATION_CODE1, NO_PARAM, NO_PARAM, TRUE)) | MTD_HTTP_RESP( #R_ERROR_8_1_1_2_2) |

Test Sequence #06 Error: different EID (8.1.1/3.10)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in ‘Linked’ state for S_MNO, associated to #EID1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO →SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CONFIRM_ORDER, MTD_CONFIRM_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, #EID2, #MATCHING_ID_1, #CONFIRMATION_CODE1, NO_PARAM, NO_PARAM, TRUE)) | MTD_HTTP_RESP( #R_ERROR_8_1_1_3_10) |

Test Sequence #07 Error: Matching ID invalid format (8.2.6/2.1)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in ‘Allocated’ state by S_MNO |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CONFIRM_ORDER, MTD_CONFIRM_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM, #INVALID_FORMAT_MATCHING_ID, #CONFIRMATION_CODE1, NO_PARAM, NO_PARAM, TRUE)) | MTD_HTTP_RESP( #R_ERROR_8_2_6_2_1) |

### 4.3.3	ES2+ (Operator -- SM-DP+): CancelOrder

#### 4.3.3.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

- Section 5.3.3

#### 4.3.3.2	Test Cases

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| S_MNO | For the TLS connection, CERT_CLIENT_TLS = #CERT_S_OPERATOR_TLS |

##### 4.3.3.2.1	TC_SM-DP+_ES2+.CancelOrder

Test Sequence #01 Nominal: ‘Linked’ state, using EID, final status = Available

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in ‘Linked’ state for S_MNO and bound to #EID1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CANCEL_ORDER, MTD_CANCEL_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, #EID1, NO_PARAM, #PROFILE_STATUS_AVAILABLE)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 2 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_DOWNLOAD_ORDER, MTD_DOWNLOAD_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, NO_PARAM, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM)) | MTD_HTTP_RESP( #R_SUCCESS_ICCID1) |

Test Sequence #02 Nominal: ‘Confirmed’ state, using EID, final status = Available, SM-DS Use Case

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in ‘Confirmed’ state by S_MNO and bound to #EID1. |
| SM-DP+ | The SM-DP+ has executed a confirm order procedure for the SM-DS use case with smdsAddress #TEST_ROOT_DS_ADDRESS used for Event Registration, and with MatchingID set to #MATCHING_ID_1 in input parameters. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO →SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CANCEL_ORDER, MTD_CANCEL_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, #EID1, #MATCHING_ID_1, #PROFILE_STATUS_AVAILABLE)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 2 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_DOWNLOAD_ORDER, MTD_DOWNLOAD_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, NO_PARAM, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM)) | MTD_HTTP_RESP( #R_SUCCESS_ICCID1) |

Test Sequence #03 Nominal: ‘Error’ state, using MatchingID, final status = Available

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by ICCID_OP_PROF1_NON_SWAP is configured in ‘Error’ state for S_MNO, associated with #MATCHING_ID_1 and not bound to any EID. The ‘Error’ state has been entered by ES9+.CancelSession sent with reason = endUserRejection after the ES9+.AuthenticateClient response of a previous Profile Download attempt. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO →SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CANCEL_ORDER, MTD_CANCEL_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM, #MATCHING_ID_1, #PROFILE_STATUS_AVAILABLE)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 2 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_DOWNLOAD_ORDER, MTD_DOWNLOAD_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, NO_PARAM, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM)) | MTD_HTTP_RESP( #R_SUCCESS_ICCID1) |

##### 4.3.3.2.2	TC_SM-DP+_ES2+.CancelOrder_ErrorCases

Test Sequence #01 Error: unknown ICCID (8.2.1/3.9)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | No Profile identified by #ICCID_OP_PROF1_NON_SWAP is configured in the server for S_MNO |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CANCEL_ORDER, MTD_CANCEL_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, #EID1, NO_PARAM, #PROFILE_STATUS_AVAILABLE)) | MTD_HTTP_RESP( #R_ERROR_8_2_1_3_9) |

Test Sequence #02 Error: missing EID (8.1.1/2.2)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by #ICCID_OP_PROF1_NON_SWAP is configured in ‘Confirmed’ state by S_MNO and bound to #EID1 |
| SM-DP+ | The SM-DP+ has previously executed a confirm order with MatchingID set to #MATCHING_ID_1 in input parameters |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO →SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CANCEL_ORDER, MTD_CANCEL_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM, #MATCHING_ID_1 , #PROFILE_STATUS_AVAILABLE)) | MTD_HTTP_RESP( #R_ERROR_8_1_1_2_2) |

Test Sequence #03 Error: incorrect matchingID (8.2.6/3.10)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by #ICCID_OP_PROF1_NON_SWAP is configured in ‘Released’ state by S_MNO, is bound to #EID1 and is associated with matchingID #MATCHING_ID_1 |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO →SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CANCEL_ORDER, MTD_CANCEL_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, #EID1, #MATCHING_ID_2, #PROFILE_STATUS_AVAILABLE)) | MTD_HTTP_RESP( #R_ERROR_8_2_6_3_10) |

Test Sequence #04 Error: profile in Available state (unspecified Error Code)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by #ICCID_OP_PROF1_NON_SWAP is configured in ‘Available’ state for S_MNO |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO →SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CANCEL_ORDER, MTD_CANCEL_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, NO_PARAM, NO_PARAM, #PROFILE_STATUS_AVAILABLE)) | MTD_HTTP_RESP( #R_ERROR_ANY) |

Test Sequence #05 Error: profile in Installed state (8.2.1/3.3)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by #ICCID_OP_PROF1_NON_SWAP is configured in ‘Installed’ state by S_MNO and is bound to #EID1 |
| SM-DP+ | The SM-DP+ has previously executed a confirm order with MatchingID set to #MATCHING_ID_1 in input parameters |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO →SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CANCEL_ORDER, MTD_CANCEL_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, #EID1, #MATCHING_ID_1, #PROFILE_STATUS_AVAILABLE)) | MTD_HTTP_RESP( #R_ERROR_8_2_1_3_3) |

Test Sequence #06 Error: different EID (8.2.1/3.10)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | The Profile identified by #ICCID_OP_PROF1_NON_SWAP is configured in ‘Released’ state by S_MNO, is bound to #EID1 and is associated with matchingID #MATCHING_ID_1 |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |  |
| 1 | S_MNO →SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS_ES2_PLUS, #PATH_CANCEL_ORDER, MTD_CANCEL_ORDER( #S_MNO_F_REQ_ID, #FUNCTION_CALL_ID_1, #ICCID_OP_PROF1_NON_SWAP, #EID2, #MATCHING_ID_1, #PROFILE_STATUS_AVAILABLE)) | MTD_HTTP_RESP( #R_ERROR_8_2_1_3_10) |

### 4.3.4	ES2+ (Operator -- SM-DP+): ReleaseProfile

This test case is defined as FFS and not applicable for this version of test specification.

### 4.3.5	ES2+ (Operator -- SM-DP+): HandleNotification

This test case is defined as FFS and not applicable for this version of test specification.

### 4.3.6	ES2+ (Operator -- SM-DP+): TLS, Mutual Authentication, Server, Session Establishment

This test case is defined as FFS and not applicable for this version of test specification.

### 4.3.7	ES8+ (SM-DP+ -- eUICC): InitialiseSecureChannel

#### 4.3.7.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Requirements

#### 4.3.7.2	Test Cases

All testing for ES8+ functions is performed in section 4.3.13 ES9+ (LPA -- SM-DP+): GetBoundProfilePackage.

### 4.3.8	ES8+ (SM-DP+ -- eUICC): ConfigureISDP

#### 4.3.8.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Requirements

#### 4.3.8.2	Test Cases

All testing for ES8+ functions is performed in section 4.3.13 ES9+ (LPA -- SM-DP+): GetBoundProfilePackage.

### 4.3.9	ES8+ (SM-DP+ -- eUICC): StoreMetadata

#### 4.3.9.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Requirements

#### 4.3.9.2	Test Cases

All testing for ES8+ functions is performed in section 4.3.13 ES9+ (LPA -- SM-DP+): GetBoundProfilePackage.

### 4.3.10	ES8+ (SM-DP+ -- eUICC): ReplaceSessionKeys

#### 4.3.10.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Requirements

#### 4.3.10.2	Test Cases

All testing for ES8+ functions is performed in section 4.3.13 ES9+ (LPA -- SM-DP+): GetBoundProfilePackage.

### 4.3.11	ES8+ (SM-DP+ -- eUICC): LoadProfileElements

#### 4.3.11.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Requirements

#### 4.3.11.2	Test Cases

All testing for ES8+ functions is performed in section 4.3.13 ES9+ (LPA -- SM-DP+): GetBoundProfilePackage.

### 4.3.12	ES9+ (LPA -- SM-DP+): InitiateAuthentication

The test sequences defined in this section are intended for testing on both the SM-DP+ and the SM-DS.

#### 4.3.12.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

Section 2.6.16.2

Section 3.0.1, 3.1.3

Section 4.5.2.1, 4.5.2.2

Section 5.6.1

Section 5.7.13

Section 6.2

Section 6.5.1, 6.5.1.1, 6.5.1.2, 6.5.1.3, 6.5.1.4, 6.5.2, 6.5.2.6

#### 4.3.12.2	Test Cases

| General Initial Conditions for SM-DP + testing |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for NIST. |

##### 4.3.12.2.1	TC_SM-DP+_ES9+.InitiateAuthenticationNIST

Test Sequence #01 Nominal

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 1 | S_LPAd → SERVER | MTD_HTTP_REQ( #SERVER_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #SERVER_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) • Verify that <TRANSACTION_ID_IA> matches <TRANSACTION_ID_SIGNED_IA> • Verify the validity of <SERVER_SIGNATURE1> using the public key #PK_SM_XXauth_SIG contained in #CERT_SM_XXauth_SIG |

Test Sequence #02 Nominal: Uniqueness of Transaction ID and Server Challenge

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 1 | S_LPAd → SERVER | MTD_HTTP_REQ( #SERVER_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #SERVER_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 3 | S_LPAd → SERVER | MTD_HTTP_REQ( #SERVER_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #SERVER_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK_2) Verify that: -  <TRANSACTION_ID_2> received in this step is different to the <TRANSACTION_ID_IA> in Step 1 - <TRANSACTION_ID_SIGNED_2> received in this step is different to the <TRANSACTION_ID_SIGNED_IA> in Step 1  <SERVER_CHALLENGE_2> received in this step is different to the <SERVER_CHALLENGE> in Step 1. |

Test Sequence #03 Error: Failed due to Invalid Server Address (Subject Code 8.8.1 Reason Code 3.8)

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 1 | S_LPAd → SERVER | MTD_HTTP_REQ ( #SERVER_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #UNKNOWN_SERVER_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_ERROR_SMXX_1_3_8) |

Test Sequence #04 Error: Failed due to Unsupported Public Key Identifiers (Subject Code 8.8.2 Reason Code 3.1)

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 1 | S_LPAd → SERVER | MTD_HTTP_REQ( #SERVER_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #EUICC_INFO1_8_8_2_3_1, #SERVER_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_ERROR_SMXX_2_3_1) |

Test Sequence #05 Error: Failed due to Unsupported Specification Version Number (Subject Code 8.8.3 Reason Code 3.1)

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 1 | S_LPAd → SERVER | MTD_HTTP_REQ( #SERVER_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #EUICC_INFO1_8_8_3_3_1_LOWER, #SERVER_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_ERROR_SMXX_3_3_1) |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 3 | S_LPAd → SERVER | MTD_HTTP_REQ( #SERVER_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #EUICC_INFO1_8_8_3_3_1_HIGHER, #SERVER_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_ERROR_SMXX_3_3_1) |

Test Sequence #06 Error: Failed due to Unavailable Server Auth Certificate (Subject Code 8.8.4 Reason Code 3.7)

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 1 | S_LPAd → SERVER | MTD_HTTP_REQ( #SERVER_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #EUICC_INFO1_8_8_4_3_7, #SERVER_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_ERROR_SMXX_4_3_7) |

Test Sequence #07 Nominal: VOID

Test Sequence #08 Nominal: VOID

Test Sequence #09 Nominal: VOID

##### 4.3.12.2.2	TC_SM-DP+_ES9+.InitiateAuthenticationFRP

This test case is defined as FFS and not applicable for this version of test specification.

##### 4.3.12.2.3	TC_SM-DP+_ES9+.InitiateAuthenticationBRP

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for BRP. |

Test Sequence #01 Nominal

This test sequence SHALL be the same as the Test Sequence #01 defined in section 4.3.12.2.1 TC_SM-DP+_ES9+.InitiateAuthenticationNIST except that all auth/pb keys and certificates SHALL be based on BrainpoolP256r1.

### 4.3.13	ES9+ (LPA -- SM-DP+): GetBoundProfilePackage

#### 4.3.13.1	Conformance Requirements

References

GSMA RSP Technical Specification [2] :

Section 2.4.5.1

Section 2.5.1, 2.5.3, 2.5.4, 2.5.4.1, 2.5.4.2, 2.5.4.3, 2.5.4.4, 2.5.5

Section 2.6.4, 2.6.5, 2.6.7.1, 2.6.7.2

Section 3.0.1, 3.1.1.2, 3.1.1.4

Section 3.2.5

Section 4.4

Section 4.7

Section 5.5, 5.5.1, 5.5.2, 5.5.3, 5.5.4

Section 5.6.2

Section 5.7.5

Section 6.2

Section 6.5.1, 6.5.1.1, 6.5.1.2, 6.5.1.3, 6.5.1.4, 6.5.2, 6.5.2.7

Annex G

#### 4.3.13.2	Test Cases

##### 4.3.13.2.1	TC_SM-DP+_ES9+.GetBoundProfilePackageNIST

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for NIST following variant O PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with an empty MatchingID. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. There have been no previous attempts to download the pending profile. |

Test Sequence #01 Nominal: Using S-ENC and S-MAC without Confirmation Code

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is loaded as an Unprotected Profile Package. Confirmation Code is not provided by the Operator to the SM-DP+.Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP(#R_GET_BPP_RESP_OP1_SK) • Verify that <TRANSACTION_ID_GBPP> matches <S_TRANSACTION_ID> MTD_TEST_ES8+_GET_BPP_SK (#R_GET_BPP_RESP_OP1_SK, <S_MAC>, <S_ENC>, #SMDP_METADATA_OP_PROF1) |

Test Sequence #02 Nominal: Using S-ENC and S-MAC with Confirmation Code

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is loaded as an Unprotected Profile Package. Confirmation Code #CONFIRMATION_CODE1 associated to PROFILE_OPERATIONAL1 is provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_CC)) | MTD_HTTP_RESP(#R_GET_BPP_RESP_OP1_SK) • Verify that <TRANSACTION_ID_GBPP> matches <S_TRANSACTION_ID> MTD_TEST_ES8+_GET_BPP_SK (#R_GET_BPP_RESP_OP1_SK, <S_MAC>, <S_ENC>, #SMDP_METADATA_OP_PROF1) |

Test Sequence #03 Nominal: Using PPK-ENC and PPK-MAC without Confirmation Code

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |  |
| --- | --- | --- | --- | --- |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP(#R_GET_BPP_RESP_OP1_PPK) • Verify that <TRANSACTION_ID_GBPP> matches <S_TRANSACTION_ID> MTD_TEST_ES8+_GET_BPP_PPK (#R_GET_BPP_RESP_OP1_PPK, <S_MAC>, <S_ENC>, <PPK_MAC>, <PPK_ENC>, #SMDP_METADATA_OP_PROF1) |  |

Test Sequence #04 Nominal: Using PPK-ENC and PPK-MAC with Confirmation Code

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code #CONFIRMATION_CODE1 associated to PROFILE_OPERATIONAL1 is provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_CC)) | MTD_HTTP_RESP(#R_GET_BPP_RESP_OP1_PPK) • Verify that <TRANSACTION_ID_GBPP> matches <S_TRANSACTION_ID> MTD_TEST_ES8+_GET_BPP_PPK (#R_GET_BPP_RESP_OP1_PPK, <S_MAC>, <S_ENC>, <PPK_MAC>, <PPK_ENC>, #SMDP_METADATA_OP_PROF1) |

Test Sequence #05 Nominal: Using S-ENC and S-MAC with Metadata split over 2 segments without Confirmation Code

The purpose of this test is to test that the LPAd can request the delivery and the binding of a Profile Package using the S-ENC and S-MAC with the metadata split over two sequenceOf88 segments without a Confirmation Code.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1_2_SEG is loaded as an Unprotected Profile Package. Confirmation Code is not provided by the Operator to the SM-DP+. |

This test sequence SHALL be the same as the Test Sequence #01 defined in this section except that #SMDP_METADATA_OP_PROF1_2_SEG replaces #SMDP_METADATA_OP_PROF1.

NOTE:	There is no testing required in addition to Test Sequence #01 as the R_GET_BPP_RESP_OP1_SK constants allow for 1 or 2 segments and for the SM-DP+ to successfully pass this test sequence it SHALL use 2 segments to deliver the metadata.

Test Sequence #06 Nominal: Using PPK-ENC and PPK-MAC with Metadata split over 2 segments without Confirmation Code

The purpose of this test is to test that the LPAd can request the delivery and the binding of a Profile Package using the PPK-ENC and PPK-MAC with the metadata split over two sequenceOf88 segments without a Confirmation Code.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1_2_SEG is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

This test sequence SHALL be the same as the Test Sequence #03 defined in this section except that #SMDP_METADATA_OP_PROF1_2_SEG replaces #SMDP_METADATA_OP_PROF1.

NOTE:	There is no testing required in addition to Test Sequence #03 as the R_GET_BPP_RESP_OP1_PPK constants allow for 1 or 2 segments and for the SM-DP+ to successfully pass this test sequence it SHALL use 2 segments to deliver the metadata.

##### 4.3.13.2.2	TC_SM-DP+_ES9+.GetBoundProfilePackageFRP

This test case is defined as FFS and not applicable for this version of test specification.

##### 4.3.13.2.3	TC_SM-DP+_ES9+.GetBoundProfilePackageBRP

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for BRP following variant O. PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1. Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with an empty MatchingID. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. There have been no previous attempts to download the pending profile. |

Test Sequence #01 Nominal: Using S-ENC and S-MAC without Confirmation Code

This test sequence SHALL be the same as the Test Sequence #01 defined in section 4.3.13.2.1 TC_SM-DP+_ES9+.GetBoundProfilePackageNIST except that all auth/pb keys and certificates SHALL be based on BrainpoolP256r1.

Test Sequence #02 Nominal: Using PPK-ENC and PPK-MAC without Confirmation Code

This test sequence SHALL be the same as the Test Sequence #03 defined in section 4.3.13.2.1 TC_SM-DP+_ES9+.GetBoundProfilePackageNIST except that all auth/pb keys and certificates SHALL be based on BrainpoolP256r1.

##### 4.3.13.2.4	TC_SM-DP+_ES9+.GetBoundProfilePackage_RetryCases_ReuseOTPK_NIST

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for NIST following variant O. PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1. Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with an empty MatchingID. The EID is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. |

Test Sequence #01 Nominal: Retry with same otPK.EUICC.AKA using S-ENC and S-MAC without Confirmation Code

The purpose of this test is to test that the LPAd can request the delivery and the binding of a Profile Package for a retry attempt for the same otPK.EUICC.AKA using S-ENC and S-MAC for Profile protection without a Confirmation Code.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 is loaded as an Unprotected Profile Package. Confirmation Code is not provided by the Operator to the SM-DP+. There have been no previous attempts to download the pending profile. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROF_DOWNLOAD_DEF_DP_USE_CASE_CANCEL_SESSION_SK Extract <OTPK_SM_DP+_AKA> from #INIT_SC_PROF1 in the GetBoundProfilePackage Response in Step 4. |  |  |
| IC2 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC_RETRY |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP(#R_GET_BPP_RESP_OP1_SK) • Verify that <TRANSACTION_ID_GBPP> matches <S_TRANSACTION_ID> MTD_TEST_ES8+_GET_BPP_SK (#R_GET_BPP_RESP_OP1_SK, <S_MAC>, <S_ENC>, #SMDP_METADATA_OP_PROF1) • Verify that <OTPK_SM_DP+_AKA> in #INIT_SC_PROF1 matches the value previously received in the GetBoundProfilePackage response in step 4 of the procedure in IC1 |

Test Sequence #02 Nominal: Retry with same otPK.EUICC.AKA using S-ENC and S-MAC with Confirmation Code

The purpose of this test is to test that the LPAd can request the delivery and the binding of a Profile Package for a retry attempt for the same otPK.EUICC.AKA using the S-ENC and S-MAC for Profile protection with a Confirmation Code.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 is loaded as an Unprotected Profile Package. Confirmation Code #CONFIRMATION_CODE1 associated to PROFILE_OPERATIONAL1 is provided by the Operator to the SM-DP+. There have been no previous attempts to download the pending profile. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROF_DOWNLOAD_DEF_DP_USE_CASE_CC_CANCEL_SESSION_SK Extract <OTPK_SM_DP+_AKA> from #INIT_SC_PROF1 in the GetBoundProfilePackage Response in Step 4. |  |  |
| IC2 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_CC_RETRY |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_CC)) | MTD_HTTP_RESP(#R_GET_BPP_RESP_OP1_SK) • Verify that <TRANSACTION_ID_GBPP> matches <S_TRANSACTION_ID> MTD_TEST_ES8+_GET_BPP_SK (#R_GET_BPP_RESP_OP1_SK, <S_MAC>, <S_ENC>, #SMDP_METADATA_OP_PROF1) • Verify that <OTPK_SM_DP+_AKA> in #INIT_SC_PROF1 matches the value previously received in the GetBoundProfilePackage response in step 4 of the procedure in IC1 |

Test Sequence #03 Nominal: Retry with same otPK.EUICC.AKA using PPK-ENC and PPK-MAC without Confirmation Code

The purpose of this test is to test that the LPAd can request the delivery and the binding of a Profile Package for a retry attempt for the same otPK.EUICC.AKA using the PPK-ENC and PPK-MAC for Profile protection without a Confirmation Code.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. There has been no previous attempts to download the pending profile. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROF_DOWNLOAD_DEF_DP_USE_CASE_CANCEL_SESSION_PPK Extract <OTPK_SM_DP+_AKA> from #INIT_SC_PROF1 in the GetBoundProfilePackage Response in Step 4. |  |  |
| IC2 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC_RETRY |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_PPK) • Verify that <TRANSACTION_ID_GBPP> matches <S_TRANSACTION_ID> MTD_TEST_ES8+_GET_BPP_PPK (#R_GET_BPP_RESP_OP1_PPK, <S_MAC>, <S_ENC>, <PPK_MAC>, <PPK_ENC>, #SMDP_METADATA_OP_PROF1) • Verify that <OTPK_SM_DP+_AKA> in #INIT_SC_PROF1 matches the value previously received in the GetBoundProfilePackage response in step 4 of the procedure in IC1 |

Test Sequence #04 Nominal: Retry with same otPK.EUICC.AKA using PPK-ENC and PPK-MAC with Confirmation Code

The purpose of this test is to test that the LPAd can request the delivery and the binding of a Profile Package with a retry attempt for the same otPK.EUICC.AKA using the PPK-ENC and PPK-MAC for Profile protection with a Confirmation Code.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code #CONFIRMATION_CODE1 associated to PROFILE_OPERATIONAL1 is provided by the Operator to the SM-DP+. There has been no previous attempts to download the pending profile. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROF_DOWNLOAD_DEF_DP_USE_CASE_CC_CANCEL_SESSION_PPK Extract <OTPK_SM_DP+_AKA> from #INIT_SC_PROF1 in the GetBoundProfilePackage Response in Step 4. |  |  |
| IC2 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_CC_RETRY |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_CC)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_PPK) • Verify that <TRANSACTION_ID_GBPP> matches <S_TRANSACTION_ID> MTD_TEST_ES8+_GET_BPP_PPK (#R_GET_BPP_RESP_OP1_PPK, <S_MAC>, <S_ENC>, <PPK_MAC>, <PPK_ENC>, #SMDP_METADATA_OP_PROF1) • Verify that <OTPK_SM_DP+_AKA> in #INIT_SC_PROF1 matches the value previously received in the GetBoundProfilePackage response in step 4 of the procedure in IC1 |

Test Sequence #05 Nominal: Retry with same otPK.EUICC.AKA rejected by eUICC using S-ENC and S-MAC without Confirmation Code

The purpose of this test is to test that the LPAd can request the delivery and the binding of a Profile Package for a retry attempt with the same otPK.EUICC.AKA rejected by the eUICC using the S-ENC and S-MAC without a Confirmation Code.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 is loaded as an Unprotected Profile Package. Confirmation Code is not provided by the Operator to the SM-DP+. There have been no previous attempt to download the pending profile. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROF_DOWNLOAD_DEF_DP_USE_CASE_CANCEL_SESSION_SK Extract <OTPK_SM_DP+_AKA> from #INIT_SC_PROF1 in the GetBoundProfilePackage Response in Step 4. |  |  |
| IC2 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC_RETRY |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_NEW_OTPK)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_SK) • Verify that <TRANSACTION_ID_GBPP> matches <S_TRANSACTION_ID> MTD_TEST_ES8+_GET_BPP_SK (#R_GET_BPP_RESP_OP1_SK, <S_MAC>, <S_ENC>, #SMDP_METADATA_OP_PROF1) • Verify that <OTPK_SM_DP+_AKA> in #INIT_SC_PROF1 is different from the value previously received in the GetBoundProfilePackage response in step 4 of the procedure in IC1 |

Test Sequence #06 Nominal: Retry with same otPK.EUICC.AKA rejected by eUICC using S-ENC and S-MAC with Confirmation Code

The purpose of this test is to test that the LPAd can request the delivery and the binding of a Profile Package for a retry attempt with the same otPK.EUICC.AKA rejected by the eUICC using the S-ENC and S-MAC with a Confirmation Code.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 is loaded as an Unprotected Profile Package. Confirmation Code #CONFIRMATION_CODE1 associated to PROFILE_OPERATIONAL1 is provided by the Operator to the SM-DP+. There have been no previous attempt to download the pending profile. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROF_DOWNLOAD_DEF_DP_USE_CASE_CC_CANCEL_SESSION_SK Extract <OTPK_SM_DP+_AKA> from #INIT_SC_PROF1 in the GetBoundProfilePackage Response in Step 4 |  |  |
| IC2 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_CC_RETRY |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_NEW_OTPK_CC)) | MTD_HTTP_RESP(#R_GET_BPP_RESP_OP1_SK) • Verify that <TRANSACTION_ID_GBPP> matches <S_TRANSACTION_ID> MTD_TEST_ES8+_GET_BPP_SK (#R_GET_BPP_RESP_OP1_SK, <S_MAC>, <S_ENC>, #SMDP_METADATA_OP_PROF1) • Verify that <OTPK_SM_DP+_AKA> in #INIT_SC_PROF1 is different from the value previously received in the GetBoundProfilePackage response in step 4 of the procedure in IC1 |

Test Sequence #07 Nominal: Retry with same otPK.EUICC.AKA rejected by eUICC using PPK-ENC and PPK-MAC without Confirmation Code

The purpose of this test is to test that the LPAd can request the delivery and the binding of a Profile Package for a retry attempt with the same otPK.EUICC.AKA rejected by the eUICC using the PPK-ENC and PPK-MAC without a Confirmation Code.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. There have been no previous attempt to download the pending profile. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROF_DOWNLOAD_DEF_DP_USE_CASE_CANCEL_SESSION_PPK Extract <OTPK_SM_DP+_AKA> from #INIT_SC_PROF1 in the GetBoundProfilePackage Response in Step 4. |  |  |
| IC2 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC_RETRY |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_NEW_OTPK)) | MTD_HTTP_RESP(#R_GET_BPP_RESP_OP1_PPK) • Verify that <TRANSACTION_ID_GBPP> matches <S_TRANSACTION_ID> MTD_TEST_ES8+_GET_BPP_PPK (#R_GET_BPP_RESP_OP1_PPK, <S_MAC>, <S_ENC>, <PPK_MAC>, <PPK_ENC>, #SMDP_METADATA_OP_PROF1) • Verify that <OTPK_SM_DP+_AKA> in #INIT_SC_PROF1 is different from the value previously received in the GetBoundProfilePackage response in step 4 of the procedure in IC1 |

Test Sequence #08 Nominal: Retry with same otPK.EUICC.AKA rejected by eUICC using PPK-ENC and PPK-MAC with Confirmation Code

The purpose of this test is to test that the LPAd can request the delivery and the binding of a Profile Package for a retry attempt with the same otPK.EUICC.AKA rejected by the eUICC using the PPK-ENC and PPK-MAC.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code #CONFIRMATION_CODE1 associated to PROFILE_OPERATIONAL1 is provided by the Operator to the SM-DP+. There have been no previous attempt to download the pending profile. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROF_DOWNLOAD_DEF_DP_USE_CASE_CC_CANCEL_SESSION_PPK Extract <OTPK_SM_DP+_AKA> from #INIT_SC_PROF1 in the GetBoundProfilePackage Response in Step 4. |  |  |
| IC2 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_CC_RETRY |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_NEW_OTPK_CC)) | MTD_HTTP_RESP(#R_GET_BPP_RESP_OP1_PPK) • Verify that <TRANSACTION_ID_GBPP> matches <S_TRANSACTION_ID> MTD_TEST_ES8+_GET_BPP_PPK (#R_GET_BPP_RESP_OP1_PPK, <S_MAC>, <S_ENC>, <PPK_MAC>, <PPK_ENC>, #SMDP_METADATA_OP_PROF1) • Verify that <OTPK_SM_DP+_AKA> in #INIT_SC_PROF1 is different from the value previously received in the GetBoundProfilePackage response in step 4 of the procedure in IC1 |

Test Sequence #09 Nominal: Confirmation Code retry

The purpose of this test is to test that the SM-DP+ accepts a subsequent correct Confirmation Code after the initial Confirmation Code supplied in the GetBoundProfilePackageRequest ASN.1 euiccSigned2 element is unknown.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 is securely loaded as a Protected Profile Package using <PPK_ENC> and PPK_MAC>. Confirmation Code #CONFIRMATION_CODE1 associated to PROFILE_OPERATIONAL1 is provided by the Operator to the SM-DP+. The SM-DP+ is configured with two retries allowed for the receipt of a valid Confirmation Code. There have been no previous attempts to download the pending profile. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_INVALID_CC |  |  |
| IC2 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_CC)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_PPK) • Verify that <TRANSACTION_ID_GBPP> matches <S_TRANSACTION_ID> MTD_TEST_ES8+_GET_BPP_PPK (#R_GET_BPP_RESP_OP1_PPK, <S_MAC>, <S_ENC>, <PPK_MAC>, <PPK_ENC>, #SMDP_METADATA_OP_PROF1) |

##### 4.3.13.2.5	VOID

##### 4.3.13.2.6	VOID

##### 4.3.13.2.7	TC_SM-DP+_ES9+.GetBoundProfilePackage_RetryCases_DifferentOTPK_NIST

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for NIST following variant O PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1. Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with an empty MatchingID. The EID is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. |

Test Sequence #01 Nominal: Retry without otPK.EUICC.AKA using S-ENC and S-MAC without Confirmation Code

The purpose of this test is to test that the LPAd can request the delivery and the binding of a Profile Package for a retry attempt without otPK.EUICC.AKA using the S-ENC and S-MAC without a Confirmation Code.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 is loaded as an Unprotected Profile Package. Confirmation Code is not provided by the Operator to the SM-DP+. There have been no previous attempt to download the pending profile. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROF_DOWNLOAD_DEF_DP_USE_CASE_CANCEL_SESSION_SK Extract <OTPK_SM_DP+_AKA> from the GetBoundProfilePackage Response in Step 4. |  |  |
| IC2 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_NEW_OTPK)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_SK) • Verify that <TRANSACTION_ID_GBPP> matches <S_TRANSACTION_ID> MTD_TEST_ES8+_GET_BPP_SK (#R_GET_BPP_RESP_OP1_SK, <S_MAC>, <S_ENC>, #SMDP_METADATA_OP_PROF1) • Verify that <OTPK_SM_DP+_AKA> in #INIT_SC_PROF1 is different from the value previously received in the GetBoundProfilePackage response in step 4 of the procedure in IC1 |

Test Sequence #02 Nominal: Retry without otPK.EUICC.AKA using S-ENC and S-MAC with Confirmation Code

The purpose of this test is to test that the LPAd can request the delivery and the binding of a Profile Package for a retry attempt without otPK.EUICC.AKA using the S-ENC and S-MAC with a Confirmation Code.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 is loaded as an Unprotected Profile Package. Confirmation Code #CONFIRMATION_CODE1 associated to PROFILE_OPERATIONAL1 is provided by the Operator to the SM-DP+. There have been no previous attempt to download the pending profile. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROF_DOWNLOAD_DEF_DP_USE_CASE_CC_CANCEL_SESSION_SK Extract <OTPK_SM_DP+_AKA> from #INIT_SC_PROF1 in the GetBoundProfilePackage Response in Step 4. |  |  |
| IC2 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_NEW_OTPK_CC)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_SK) • Verify that <TRANSACTION_ID_GBPP> matches <S_TRANSACTION_ID> MTD_TEST_ES8+_GET_BPP_SK (#R_GET_BPP_RESP_OP1_SK, <S_MAC>, <S_ENC>, #SMDP_METADATA_OP_PROF1) • Verify that <OTPK_SM_DP+_AKA> in #INIT_SC_PROF1 is different from the value previously received in the GetBoundProfilePackage response in step 4 of the procedure in IC1 |

Test Sequence #03 Nominal: Retry without otPK.EUICC.AKA using PPK-ENC and PPK-MAC without Confirmation Code

The purpose of this test is to test that the LPAd can request the delivery and the binding of a Profile Package for a retry attempt without otPK.EUICC.AKA using the PPK-ENC and PPK-MAC without a Confirmation Code.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. There have been no previous attempt to download the pending profile. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROF_DOWNLOAD_DEF_DP_USE_CASE_CANCEL_SESSION_PPK Extract <OTPK_SM_DP+_AKA> from #INIT_SC_PROF1 in the GetBoundProfilePackage Response in Step 4. |  |  |
| IC2 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_NEW_OTPK)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_PPK) • Verify that <TRANSACTION_ID_GBPP> matches <S_TRANSACTION_ID> MTD_TEST_ES8+_GET_BPP_PPK (#R_GET_BPP_RESP_OP1_PPK, <S_MAC>, <S_ENC>, <PPK_MAC>, <PPK_ENC>, #SMDP_METADATA_OP_PROF1) • Verify that <OTPK_SM_DP+_AKA> in #INIT_SC_PROF1 is different from the value previously received in the GetBoundProfilePackage response in step 4 of the procedure in IC1 |

Test Sequence #04 Nominal: Retry without otPK.EUICC.AKA using PPK-ENC and PPK-MAC with Confirmation Code

The purpose of this test is to test that the LPAd can request the delivery and the binding of a Profile Package for a retry attempt without otPK.EUICC.AKA using the PPK-ENC and PPK-MAC with a Confirmation Code.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code #CONFIRMATION_CODE1 associated to PROFILE_OPERATIONAL1 is provided by the Operator to the SM-DP+. There have been no previous attempt to download the pending profile. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROF_DOWNLOAD_DEF_DP_USE_CASE_CC_CANCEL_SESSION_PPK Extract <OTPK_SM_DP+_AKA> from #INIT_SC_PROF1 in the GetBoundProfilePackage Response in Step 4. |  |  |
| IC2 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_NEW_OTPK_CC)) | MTD_HTTP_RESP(#R_GET_BPP_RESP_OP1_PPK) • Verify that <TRANSACTION_ID_GBPP> matches <S_TRANSACTION_ID> MTD_TEST_ES8+_GET_BPP_PPK (#R_GET_BPP_RESP_OP1_PPK, <S_MAC>, <S_ENC>, <PPK_MAC>, <PPK_ENC>, #SMDP_METADATA_OP_PROF1) • Verify that <OTPK_SM_DP+_AKA> in #INIT_SC_PROF1 is different from the value previously received in the GetBoundProfilePackage response in step 4 of the procedure in IC1. |

##### 4.3.13.2.8	VOID

##### 4.3.13.2.9	VOID

##### 4.3.13.2.10	TC_SM-DP+_ES9+.GetBoundProfilePackage_ErrorCasesNIST

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for NIST following variant O. PROFILE_OPERATIONAL1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with an empty MatchingID. The EID is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. There have been no previous attempts to download the pending profile. |

Test Sequence #01 Error: Invalid eUICC Signature (Subject Code 8.1 Reason Code 6.1)

The purpose of this test is to test that the SM-DP+ returns the correct error code for an invalid eUICC signature supplied in GetBoundProfilePackageRequest.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_8_1_6_1)) | MTD_HTTP_RESP( #R_ERROR_8_1_6_1) |

Test Sequence #02 Error: Unknown TransactionID in JSON transport layer (Subject Code 8.10.1 Reason Code 3.9)

The purpose of this test is to test that the SM-DP+ returns the correct error code when the TransactionID supplied in GetBoundProfilePackageRequest JSON transport layer is unknown.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <INVALID_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP( #R_ERROR_8_10_1_3_9) |

Test Sequence #03 Error: Unknown TransactionID in ASN.1 euiccSigned2 element (Subject Code 8.10.1 Reason Code 3.9)

The purpose of this test is to test that the SM DP+ returns the correct error code when the TransactionID supplied in the GetBoundProfilePackageRequest ASN.1 euiccSigned2 element is unknown.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_8_10_1_3_9)) | MTD_HTTP_RESP( #R_ERROR_8_10_1_3_9) |

Test Sequence #04 Error: Missing Confirmation Code (Subject Code 8.2.7 Reason Code 2.2)

The purpose of this test is to test that the SM-DP+ returns the correct error code when the Confirmation Code is missing in the PrepareDownloadResponse request ASN.1 euiccSigned2 element.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Confirmation Code #CONFIRMATION_CODE1 associated to PROFILE_OPERATIONAL1 is provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP( #R_ERROR_8_2_7_2_2) |

Test Sequence #05 Error: Refused Confirmation Code (Subject Code 8.2.7 Reason Code 3.8)

The purpose of this test is to test that the SM-DP+ returns the correct error code when the Confirmation Code supplied in the GetBoundProfilePackageRequest ASN.1 euiccSigned2 element is unknown.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Confirmation Code #CONFIRMATION_CODE2 associated to PROFILE_OPERATIONAL1 is provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_CC |  |  |
| IC2 | <S_HASHED_CC> = MTD_GENERATE_HASHED_CC(#CONFIRMATION_CODE1, <S_TRANSACTION_ID>) |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_CC)) | MTD_HTTP_RESP( #R_ERROR_8_2_7_3_8) |

Test Sequence #06 VOID

Test Sequence #07 Error: Refused Confirmation Code Retry Exceeded (Subject Code 8.2.7 Reason Code 6.4)

The purpose of this test is to test that the SM-DP+ returns the correct error code when the Confirmation Code supplied in the GetBoundProfilePackageRequest ASN.1 euiccSigned2 element is unknown and the maximum number of retry is exceeded.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Confirmation Code #CONFIRMATION_CODE2 associated to PROFILE_OPERATIONAL1 is provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_CC |  |  |
| IC2 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_INVALID_CC |  |  |
| IC3 | <S_HASHED_CC> = MTD_GENERATE_HASHED_CC(#CONFIRMATION_CODE1, <S_TRANSACTION_ID>) |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_CC)) | MTD_HTTP_RESP(#R_ERROR_8_2_7_6_4) |

##### 4.3.13.2.11	VOID

##### 4.3.13.2.12	VOID

##### 4.3.13.2.13	TC_SM-DP+_ES9+.GetBoundProfilePackageNISTVariantA

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for NIST following variant A. PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1. Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with an empty MatchingID. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. There have been no previous attempts to download the pending profile. |

Test Sequence #01 Nominal: Using S-ENC and S-MAC without Confirmation Code

This test sequence SHALL be the same as the Test Sequence #01 defined in section 4.3.13.2.1 TC_SM-DP+_ES9+.GetBoundProfilePackageNIST except that #CERT_SM_DPauth_SIG certificate is following variant A.

##### 4.3.13.2.14	TC_SM-DP+_ES9+.GetBoundProfilePackageNISTVariantB

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for NIST following variant B. PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1. Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with an empty MatchingID. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. There have been no previous attempts to download the pending profile. |

Test Sequence #01 Nominal: Using S-ENC and S-MAC without Confirmation Code

This test sequence SHALL be the same as the Test Sequence #01 defined in section 4.3.13.2.1 TC_SM-DP+_ES9+.GetBoundProfilePackageNIST except that #CERT_SM_DPauth_SIG certificate is following variant B.

##### 4.3.13.2.15	TC_SM-DP+_ES9+.GetBoundProfilePackageNISTVariantC

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for NIST following variant C. PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1. Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with an empty MatchingID. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. There have been no previous attempts to download the pending profile. |

Test Sequence #01 Nominal: Using S-ENC and S-MAC without Confirmation Code

This test sequence SHALL be the same as the Test Sequence #01 defined in section 4.3.13.2.1 TC_SM-DP+_ES9+.GetBoundProfilePackageNIST except that #CERT_SM_DPauth_SIG certificate is following variant C.

### 4.3.14	ES9+ (LPA -- SM-DP+): AuthenticateClient

#### 4.3.14.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Requirements

- 	RQ26_033

- RQ31_025, RQ31_058RQ31_058, RQ31_059, RQ31_060, RQ31_061, RQ31_067, RQ31_080, RQ31_081, RQ31_082, RQ31_083, RQ31_085, RQ31_086, RQ31_089, RQ31_090, RQ31_091, RQ31_092, RQ31_093, RQ31_094, RQ31_095

- 	RQ41_001, RQ41_006, RQ41_007, RQ41_008

- 	RQ42_001

- 	RQ45_006, RQ45_017, RQ45_026, RQ45_026_1, RQ45_027, RQ45_028, RQ45_029

- 	RQ47_001

- 	RQ56_029, RQ56_030, RQ56_031, RQ56_032, RQ56_033, RQ56_034, RQ56_035, RQ56_036, RQ56_036_1, RQ56_037, RQ56_038, RQ56_039, RQ56_040, RQ56_041, RQ56_041_1, RQ56_041_2

- 	RQ57_037, RQ57_057, RQ57_057_1, RQ57_108

- 	RQ62_001, RQ62_002, RQ62_004, RQ62_005, RQ62_006, RQ62_007

- 	RQ65_001, RQ65_002, RQ65_003, RQ65_004, RQ65_005, RQ65_007, RQ65_008 RQ65_009, RQ65_022, RQ65_023

#### 4.3.14.2	Test Cases

##### 4.3.14.2.1	TC_SM-DP+_ES9+.AuthenticateClientNIST

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for NIST and #CERT_SM_DPpb_SIG for NIST. PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. There have been no previous attempts to download the pending profile. |

Test Sequence #01 Nominal for Default SM-DP+ Address Use Case without Confirmation Code

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_EMPTY. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP(#R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP(#R_AUTH_CLIENT_OK) • Verify that <TRANSACTION_ID_AC> matches <S_TRANSACTION_ID> • Verify the validity of the smdpSignature2 <SMDP_SIGNATURE2> using the #PK_SM_DPpb_SIG • Verify that <TRANSACTION_ID_SIGNED_AC> matches <S_TRANSACTION_ID> |

Test Sequence #02 Nominal for Default SM-DP+ Address Use Case with Confirmation Code

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_EMPTY. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. Confirmation Code #CONFIRMATION_CODE1 is provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE , #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP(#R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK_CC) • Verify that <TRANSACTION_ID_AC> matches <S_TRANSACTION_ID> • Verify the validity of the smdpSignature2 <SMDP_SIGNATURE2> using the #PK_SM_DPpb_SIG • Verify that <TRANSACTION_ID_SIGNED_AC> matches <S_TRANSACTION_ID> |

Test Sequence #03 Nominal for Default SM-DP+ Use Case Second Attempt without Confirmation Code

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_EMPTY. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_AUTH_CLIENT_FAIL_DEF_DP_USE_CASE_INVALID_MATCHING_ID |  |  |
| IC2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK) • Verify that <TRANSACTION_ID_AC> matches <S_TRANSACTION_ID> • Verify the validity of the smdpSignature2 <SMDP_SIGNATURE2> using the #PK_SM_DPpb_SIG • Verify that <TRANSACTION_ID_SIGNED_AC> matches <S_TRANSACTION_ID> |

Test Sequence #04 VOID

Test Sequence #05 Nominal for SM-DS Use Case without Confirmation Code

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 in the 'Released' state with a MatchingID equal to <MATCHING_ID_EVENT>. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_SMDS_UC_OK)) | MTD_HTTP_RESP(#R_AUTH_CLIENT_OK) • Verify that <TRANSACTION_ID_AC> matches <S_TRANSACTION_ID> • Verify the validity of the smdpSignature2 <SMDP_SIGNATURE2> using the #PK_SM_DPpb_SIG • Verify that <TRANSACTION_ID_SIGNED_AC> matches <S_TRANSACTION_ID> |

Test Sequence #06 Nominal for SM-DS Use Case with Confirmation Code

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 in the 'Released' state with a MatchingID equal to <MATCHING_ID_EVENT>. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. Confirmation Code #CONFIRMATION_CODE1 is provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP(#R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_SMDS_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK_CC) • Verify that <TRANSACTION_ID_AC> matches <S_TRANSACTION_ID> • Verify the validity of the smdpSignature2 <SMDP_SIGNATURE2> using the #PK_SM_DPpb_SIG • Verify that <TRANSACTION_ID_SIGNED_AC> matches <S_TRANSACTION_ID> |

Test Sequence #07 VOID

Test Sequence #08 Nominal for Activation Code Use Case with Matching ID without Confirmation Code

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with the MatchingID set as an Activation Code Token with the value #MATCHING_ID_1. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_ACT_CODE_UC_OK)) | MTD_HTTP_RESP(#R_AUTH_CLIENT_OK) • Verify that <TRANSACTION_ID_AC> matches <S_TRANSACTION_ID> • Verify the validity of the smdpSignature2 <SMDP_SIGNATURE2> using the #PK_SM_DPpb_SIG • Verify that <TRANSACTION_ID_SIGNED_AC> matches <S_TRANSACTION_ID> |

Test Sequence #09 Nominal for Activation Code Use Case with Matching ID with Confirmation Code

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with the MatchingID set as an Activation Code Token with the value #MATCHING_ID_1. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. Confirmation Code #CONFIRMATION_CODE1 is provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_ACT_CODE_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK_CC) • Verify that <TRANSACTION_ID_AC> matches <S_TRANSACTION_ID> • Verify the validity of the smdpSignature2 <SMDP_SIGNATURE2> using the #PK_SM_DPpb_SIG • Verify that <TRANSACTION_ID_SIGNED_AC> matches <S_TRANSACTION_ID> |

Test Sequence #10 VOID

Test Sequence #11 Nominal for Activation Code Use Case with Matching ID without Confirmation Code not associated to EID

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with the MatchingID set as an Activation Code Token with the value #MATCHING_ID_1. EID #EID1 is not known to the SM-DP+ and is not associated to PROFILE_OPERATIONAL1. Confirmation Code is not provided by the Operator to the SM-DP+. |

This test sequence SHALL be the same as the Test Sequence #08 defined in this section.

Test Sequence #12 Nominal for Activation Code Use Case with Matching ID and Confirmation Code not associated to EID

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with the MatchingID set as an Activation Code Token with the value #MATCHING_ID_1. EID #EID1 is not known to the SM-DP+ and is not associated to PROFILE_OPERATIONAL1. Confirmation Code #CONFIRMATION_CODE1 is provided by the Operator to the SM-DP+. |

This test sequence SHALL be the same as the Test Sequence #9 defined in this section.

Test Sequence #13 VOID

Test Sequence #14 Nominal for Default SM-DP+ Address Use Case with MatchingId omitted

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_EMPTY. EID #EID1 is known to the SM-DP+ and is associated to PROFILE_OPERATIONAL1. Confirmation Code is not provided by the Operator to the SM-DP+. |

This test sequence SHALL be the same as the Test Sequence #01 defined in this section except that #AUTH_SERVER_RESP_SMDP_MATCHING_ID_OMITTED shall be used in MTD_AUTHENTICATE_CLIENT instead of #AUTH_SERVER_RESP_DEF_DP_UC_OK.

Test Sequence #15 Nominal for SM-DS Use Case with MatchingId omitted

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 in the 'Released' state with a MatchingID equal to <MATCHING_ID_EVENT>. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. Confirmation Code is not provided by the Operator to the SM-DP+. |

This test sequence SHALL be the same as the Test Sequence #05 defined in this section except that #AUTH_SERVER_RESP_SMDP_MATCHING_ID_OMITTED shall be used in MTD_AUTHENTICATE_CLIENT instead of #AUTH_SERVER_RESP_SMDS_UC_OK.

Test Sequence #16 Nominal for SM-DS Use Case with empty MatchingId

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 in the 'Released' state with a MatchingID equal to <MATCHING_ID_EVENT>. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. Confirmation Code is not provided by the Operator to the SM-DP+. |

This test sequence SHALL be the same as the Test Sequence #05 defined in this section except that #AUTH_SERVER_RESP_SMDP_MATCHING_ID_EMPTY shall be used in MTD_AUTHENTICATE_CLIENT instead of #AUTH_SERVER_RESP_SMDS_UC_OK.

Test Sequence #17 Nominal for Activation Code Use Case with MatchingId omitted

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with the MatchingID set as an Activation Code Token with the value #MATCHING_ID_1. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. Confirmation Code is not provided by the Operator to the SM-DP+. |

This test sequence SHALL be the same as the Test Sequence #08 defined in this section except that #AUTH_SERVER_RESP_SMDP_MATCHING_ID_OMITTED shall be used in MTD_AUTHENTICATE_CLIENT instead of #AUTH_SERVER_RESP_ACT_CODE_UC_OK.

Test Sequence #18 Nominal for Activation Code Use Case with empty MatchingId

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with the MatchingID set as an Activation Code Token with the value #MATCHING_ID_1. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. Confirmation Code is not provided by the Operator to the SM-DP+. |

This test sequence SHALL be the same as the Test Sequence #08 defined in this section except that #AUTH_SERVER_RESP_SMDP_MATCHING_ID_EMPTY shall be used in MTD_AUTHENTICATE_CLIENT instead of #AUTH_SERVER_RESP_ACT_CODE_UC_OK.

Test Sequence #19 Nominal with extended UICC Capability in eUICCInfo2

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_EMPTY. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP(#R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_OK_UICC_EXT)) | MTD_HTTP_RESP(#R_AUTH_CLIENT_OK) • Verify that <TRANSACTION_ID_AC> matches <S_TRANSACTION_ID> • Verify that <TRANSACTION_ID_SIGNED_AC> matches <S_TRANSACTION_ID> |

Test Sequence #20 Nominal with extended DeviceInfo

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_EMPTY. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP(#R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_OK_DEVICE_EXT)) | MTD_HTTP_RESP(#R_AUTH_CLIENT_OK) • Verify that <TRANSACTION_ID_AC> matches <S_TRANSACTION_ID> • Verify that <TRANSACTION_ID_SIGNED_AC> matches <S_TRANSACTION_ID> |

Test Sequence #21 Nominal with extended eUICCInfo2

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_EMPTY. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP(#R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_OK_eUICC_EXT)) | MTD_HTTP_RESP(#R_AUTH_CLIENT_OK) • Verify that <TRANSACTION_ID_AC> matches <S_TRANSACTION_ID> • Verify that <TRANSACTION_ID_SIGNED_AC> matches <S_TRANSACTION_ID> |

##### 4.3.14.2.2	TC_SM-DP+_ES9+.AuthenticateClientNIST_ErrorCases

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for NIST and #CERT_SM_DPpb_SIG for NIST. Confirmation Code is not provided by the Operator to the SM-DP+ for the pending profile. |

Test Sequence #1 Error: Invalid EUM Certificate (Subject Code 8.1.2 Reason Code 6.1)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_EMPTY. There have been no previous attempts to download the pending profile. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_8_1_2_6_1_SIG)) | MTD_HTTP_RESP( #R_ERROR_8_1_2_6_1) |
| 2 | S_LPAd → SM-DP+ | Close TLS session (unless SM-DP+ has already closed TLS session) |  |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 5 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_8_1_2_6_1_EX_KU)) | MTD_HTTP_RESP( #R_ERROR_8_1_2_6_1) |
| 6 | S_LPAd → SM-DP+ | Close TLS session (unless SM-DP+ has already closed TLS session) |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 8 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 9 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_8_1_2_6_1_EX_CP)) | MTD_HTTP_RESP( #R_ERROR_8_1_2_6_1) |
| 10 | S_LPAd → SM-DP+ | Close TLS session (unless SM-DP+ has already closed TLS session) |  |
| 11 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 12 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 13 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_8_1_2_6_1_EX_BC_cA)) | MTD_HTTP_RESP( #R_ERROR_8_1_2_6_1) |
| 14 | S_LPAd → SM-DP+ | Close TLS session (unless SM-DP+ has already closed TLS session) |  |
| 15 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 16 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 17 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_8_1_2_6_1_EX_BC_PLC)) | MTD_HTTP_RESP( #R_ERROR_8_1_2_6_1) |

Test Sequence #2 Error: Expired EUM Certificate (Subject Code 8.1.2 Reason Code 6.3)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial state |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the ‘Released’ state with #MATCHING_ID_EMPTY. Pending Profile PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. There have been no previous attempts to download the pending profile. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_8_1_2_6_3)) | MTD_HTTP_RESP( #R_ERROR_8_1_2_6_3) |

Test Sequence #3 Error: Invalid eUICC Certificate (Subject Code 8.1.3 Reason Code 6.1)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_EMPTY. Pending Profile PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. There have been no previous attempts to download the pending profile. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_8_1_3_6_1_SIG)) | MTD_HTTP_RESP( #R_ERROR_8_1_3_6_1) |
| 2 | S_LPAd → SM-DP+ | Close TLS session (unless SM-DP+ has already closed TLS session) |  |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 5 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_8_1_3_6_1_EX_KU)) | MTD_HTTP_RESP( #R_ERROR_8_1_3_6_1) |
| 6 | S_LPAd → SM-DP+ | Close TLS session (unless SM-DP+ has already closed TLS session) |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 8 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 9 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_8_1_3_6_1_EX_CP)) | MTD_HTTP_RESP( #R_ERROR_8_1_3_6_1) |
| 10 | S_LPAd → SM-DP+ | Close TLS session (unless SM-DP+ has already closed TLS session) |  |
| 11 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 12 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 13 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_8_1_3_6_1_SUB_ORG)) | MTD_HTTP_RESP( #R_ERROR_8_1_3_6_1) |
| 14 | S_LPAd → SM-DP+ | Close TLS session (unless SM-DP+ has already closed TLS session) |  |
| 15 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 16 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 17 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_8_1_3_6_1_SUB_SN)) | MTD_HTTP_RESP( #R_ERROR_8_1_3_6_1) |

Test Sequence #4 Error: Expired eUICC Certificate (Subject Code 8.1.3 Reason Code 6.3)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_EMPTY. Pending Profile PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. There have been no previous attempts to download the pending profile. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_8_1_3_6_3)) | MTD_HTTP_RESP( #R_ERROR_8_1_3_6_3) |

Test Sequence #5 Error: Invalid eUICC Signature (Subject Code 8.1 Reason Code 6.1)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_EMPTY. Pending Profile PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. There have been no previous attempts to download the pending profile. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_8_1_6_1_SIG)) | MTD_HTTP_RESP( #R_ERROR_8_1_6_1) |

Test Sequence #6 Error: Invalid Server Challenge (Subject Code 8.1 Reason Code 6.1)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_EMPTY. Pending Profile PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. There have been no previous attempts to download the pending profile. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_8_1_6_1_CHA)) | MTD_HTTP_RESP( #R_ERROR_8_1_6_1) |

Test Sequence #7 Error: Unknown CI Public Key (Subject Code 8.11.1 Reason Code 3.9)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_EMPTY. Pending Profile PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. There have been no previous attempts to download the pending profile. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_8_11_1_3_9)) | MTD_HTTP_RESP( #R_ERROR_8_11_1_3_9) |

Test Sequence #8 Error: Profile not released (Subject Code 8.2 Reason Code 1.2)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is not in the ‘Released’ state with #MATCHING_ID_EMPTY. Pending Profile PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. There have been no previous attempts to download the pending profile. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_ERROR_8_2_1_2) |

Test Sequence #9 Error: Unknown Transaction ID in JSON transport layer (Subject Code 8.10.1 Reason Code 3.9)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_EMPTY. Pending Profile PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. There have been no previous attempts to download the pending profile. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <INVALID_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_ERROR_8_10_1_3_9) |

Test Sequence #10 Error: Unknown Transaction ID in ASN.1 euiccSigned1 payload (Subject Code 8.10.1 Reason Code 3.9)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_EMPTY. Pending Profile PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. There have been no previous attempts to download the pending profile. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_8_10_1_3_9)) | MTD_HTTP_RESP( #R_ERROR_8_10_1_3_9) |

Test Sequence #11 Error: Invalid Matching ID for Profile Download Default DP+ Address Use Case (Subject Code 8.2.6 Reason Code 3.8)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_EMPTY. Pending Profile PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. There have been no previous attempts to download the pending profile. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_ACT_CODE_UC_OK)) | MTD_HTTP_RESP( #R_ERROR_8_2_6_3_8) |

Test Sequence #12 Error: Invalid Matching ID for Profile Download Activation Code Use Case (Subject Code 8.2.6 Reason Code 3.8)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_1. Pending Profile PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. There have been no previous attempts to download the pending profile. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_ACT_CODE_2_UC_OK)) | MTD_HTTP_RESP( #R_ERROR_8_2_6_3_8) |

Test Sequence #13 Error: Invalid Matching ID for Profile Download SM-DS Use Case (Subject Code 8.2.6 Reason Code 3.8)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 in the 'Released' state with a MatchingID equal to <MATCHING_ID_EVENT>. Pending Profile PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. There have been no previous attempts to download the pending profile. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_ACT_CODE_UC_OK)) | MTD_HTTP_RESP( #R_ERROR_8_2_6_3_8) |

Test Sequence #14 Error: Un-matched EID for Default SM-DP+ Address Use Case (Subject Code 8.1.1 Reason Code 3.8)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_EMPTY. Pending Profile PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. There have been no previous attempts to download the pending profile. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. EID #EID2 is not known to the SM-DP+ and is not associated to PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_8_1_1_3_8)) | MTD_HTTP_RESP( #R_ERROR_8_1_1_3_8) |

Test Sequence #15 Error: No Eligible Profile (Subject Code 8.2.5 Reason Code 4.3)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL3 configured with #SMDP_METADATA_OP_PROF3 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Pending Profile PROFILE_OPERATIONAL3 is in the ‘Released’ state, with an empty MatchingID. There have been no previous attempts to download the pending profile. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL3. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_8_2_5_4_3)) | MTD_HTTP_RESP( #R_ERROR_8_2_5_4_3) |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #16 Error: Download Order Expired (Subject Code 8.8.5 Reason Code 4.10)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_EMPTY. Pending Profile PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. There have been no previous attempts to download the pending profile. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. The SM-DP+ has expired Profile download order. NOTE: This is expected to be done through proprietary means. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_ERROR_8_8_5_4_10) |

Test Sequence #17 Error: Maximum number of retries for Profile download order exceeded (Subject Code 8.8.5 Reason Code 6.4)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_EMPTY. Pending Profile PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC TC_SM-DP+_ES9+.AuthenticateClientBRP. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. All previous attempts to download the pending Profile have been unsuccessful. The SM-DP+'s maximum number of attempts as defined in #IUT_SM-DP+_MAX_NUMBER_DOWNLOAD_ATTEMPTS for the Profile download order has been reached. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_ERROR_8_8_5_6_4) |

Test Sequence #18 VOID

Test Sequence #19 Un-matched EID for SM-DS Use Case (Subject Code 8.1.1 Reason Code 3.8)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 in the 'Released' state with a MatchingID equal to <MATCHING_ID_EVENT>. Pending Profile PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. There have been no previous attempts to download the pending profile. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_SMDS_UC_OK_EID2)) | MTD_HTTP_RESP( #R_ERROR_8_1_1_3_8) |

Test Sequence #20 Un-matched EID for Activation Code Use Case (Subject Code 8.1.1 Reason Code 3.8)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_1. Pending Profile PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. There have been no previous attempts to download the pending profile. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_ACT_CODE_UC_OK_EID2)) | MTD_HTTP_RESP( #R_ERROR_8_1_1_3_8) |

Test Sequence #21 Invalid MatchingId for Activation Code Use Case not associated to EID (Subject Code 8.2.6 Reason Code 3.8)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_1. Pending Profile PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. There have been no previous attempts to download the pending profile. EID #EID1 is not known to the SM-DP+ and is not associated to PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_ACT_CODE_2_UC_OK)) | MTD_HTTP_RESP( #R_ERROR_8_2_6_3_8) |

##### 4.3.14.2.3	TC_SM-DP+_ES9+.AuthenticateClientFRP

This test case is defined as FFS and not applicable for this version of test specification.

##### 4.3.14.2.4	VOID

##### 4.3.14.2.5	TC_SM-DP+_ES9+.AuthenticateClientBRP

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for BRP. PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. There have been no previous attempts to download the pending profile. |

Test Sequence #01 Nominal for Default SM-DP+ Address Use Case without Confirmation Code

This test sequence SHALL be the same as the Test Sequence #01 defined in section 4.3.14.2.1 TC_SM-DP+_ES9+.AuthenticateClientNIST except that all auth/pb keys and certificates SHALL be based on BrainpoolP256r1.

##### 4.3.14.2.6	TC_SM-DP+_ES9+.AuthenticateClient_RetryCases_Reuse_OTPK

Test Sequence #01 Nominal Default SM-DP+ Use Case Retry Attempt without Confirmation Code for reuse of OTPK

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for NIST PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with #MATCHING_ID_EMPTY. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROF_DOWNLOAD_DEF_DP_USE_CASE_CANCEL_SESSION_PPK |  |  |
| IC2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_RETRY_OK) • Verify that <TRANSACTION_ID_AC> matches <S_TRANSACTION_ID> • Verify the validity of the smdpSignature2 <SMDP_SIGNATURE2> using the #PK_SM_DPauth_SIG • Verify that <TRANSACTION_ID_SIGNED_AC> matches <S_TRANSACTION_ID> |

Test Sequence #02 Nominal SM-DS Use Case Retry Attempt without Confirmation Code for reuse of OTPK

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for NIST PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Pending Profile PROFILE_OPERATIONAL1 in the 'Released' state with a MatchingID equal to <MATCHING_ID_EVENT>. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROF_DOWNLOAD_SM_DS_USE_CASE_CANCEL_SESSION |  |  |
| IC2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_SMDS_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_RETRY_OK) • Verify that <TRANSACTION_ID_AC> matches <S_TRANSACTION_ID> • Verify the validity of the smdpSignature2 <SMDP_SIGNATURE2> using the #PK_SM_DPauth_SIG • Verify that <TRANSACTION_ID_SIGNED_AC> matches <S_TRANSACTION_ID> |

Test Sequence #03 Nominal Activation Code Use Case with Matching ID Retry Attempt without Confirmation Code for reuse of OTPK

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for NIST PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with the MatchingID set as an Activation Code Token with the value #MATCHING_ID_1. EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROF_DOWNLOAD_ACT_CODE_USE_CASE_CANCEL_SESSION |  |  |
| IC2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_ACT_CODE_UC_OK)) | MTD_HTTP_RESP(#R_AUTH_CLIENT_RETRY_OK) • Verify that <TRANSACTION_ID_AC> matches <S_TRANSACTION_ID> • Verify the validity of the smdpSignature2 <SMDP_SIGNATURE2> using the #PK_SM_DPauth_SIG • Verify that <TRANSACTION_ID_SIGNED_AC> matches <S_TRANSACTION_ID> |

Test Sequence #04 Nominal Activation Code Use Case with Matching ID for Retry Attempt without Confirmation Code not associated to EID for reuse of OTPK

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for NIST PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with the MatchingID set as an Activation Code Token with the value #MATCHING_ID_1. EID #EID1 is not known to the SM-DP+ and not associated to PROFILE_OPERATIONAL1. Confirmation Code is not provided by the Operator to the SM-DP+. |

This test sequence SHALL be the same as the Test Sequence #03 defined in this section.

### 4.3.15	ES9+ (LPA -- SM-DP+): HandleNotification

#### 4.3.15.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Requirements

- 	RQ25_016, RQ25_018, RQ25_023

- 	RQ25_024, RQ25_025, RQ25_026

- 	RQ31_171, RQ31_176, RQ31_177, RQ31_177_1, RQ31_178, RQ31_181

- 	RQ35_017, RQ35_019, RQ35_022

- 	RQ45_006, RQ45_026, RQ45_026_1

- 	RQ55_048_1

- 	RQ56_042, RQ56_042_1, RQ56_042_2

- 	RQ57_075

- 	RQ62_001, RQ62_002, RQ62_003, RQ62_004, RQ62_005, RQ62_006, RQ62_007, RQ62_009

- 	RQ63_005

- 	RQ65_001, RQ65_002, RQ65_003, RQ65_004, RQ65_005, RQ65_006, RQ65_007, RQ65_008, RQ65_009, RQ65_024

#### 4.3.15.2	Test Cases

##### 4.3.15.2.1	TC_SM-DP+_ES9+_HandleNotificationNIST

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for NIST. Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with an empty MatchingID. The EID is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. There have been no previous attempts to download pending Profile PROFILE_OPERATIONAL1. Confirmation Code is not provided by the Operator to the SM-DP+. |

Test Sequence #01 Nominal: All Notifications

The purpose of this test is to verify that the SM-DP+ acknowledges the incoming ProfileInstallationResult and OtherSignedNotification for all types of Profile notifications.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_ALL_NOTIF is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC using #R_AUTH_CLIENT_OK_ALL_NOTIF instead of #R_AUTH_CLIENT_OK |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PN_PIR_OK1)) | #R_HTTP_204_OK |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PENDING_NOTIF_OTHER_INST1)) | #R_HTTP_204_OK |
| 5 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 6 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PENDING_NOTIF_EN1)) | #R_HTTP_204_OK |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 8 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PENDING_NOTIF_DIS1)) | #R_HTTP_204_OK |
| 9 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 10 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PENDING_NOTIF_DE1)) | #R_HTTP_204_OK |
| 11 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 12 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PENDING_NOTIF_RPM_EN1)) | #R_HTTP_204_OK |
| 13 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 14 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PENDING_NOTIF_RPM_DIS1)) | #R_HTTP_204_OK |
| 15 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 16 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PENDING_NOTIF_RPM_DE1)) | #R_HTTP_204_OK |
| 17 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 18 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PENDING_NOTIF_RPM_UM1)) | #R_HTTP_204_OK |

Test Sequence #02 Nominal: Successful PIR, no install OtherSignedNotification and then Enable OtherSignedNotification Notifications

The purpose of this test is to verify that the SM-DP+ acknowledges the incoming ProfileInstallationResult and OtherSignedNotification for Profile enable.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1_EN is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC_EN |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PN_PIR_OK1)) | #R_HTTP_204_OK |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PENDING_NOTIF_EN1)) | #R_HTTP_204_OK |

Test Sequence #03 Error: Invalid Transaction ID

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PN_PIR_INVALID_TRANS_ID)) | #R_HTTP_204_OK |
| 2 | PROC_ES9+_VERIFY_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC |  |  |

Test Sequence #04 Error: PIR Error Reason - incorrect Input Values

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PN_PIR_INCORRECT_INPUT_VALUES)) | #R_HTTP_204_OK |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #05 Error: PIR Error Reason – invalid signature

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PN_PIR_INVALID_SIGN)) | #R_HTTP_204_OK |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #06 Error: PIR Error Reason – unsupported Crt Values

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>.. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PN_PIR_UNSUPPORTED_CRT)) | #R_HTTP_204_OK |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #07 Error: PIR Error Reason – unsupported Remote Operation Type

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PN_PIR_UNSUP_REMOTE_OP_TYPE)) | #R_HTTP_204_OK |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #08 Error: PIR Error Reason – unsupported Profile Class

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PN_PIR_UNSUP_PROFILE_CLASS)) | #R_HTTP_204_OK |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #09 Error: PIR Error Reason – SCP03t Structure Error

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PN_PIR_SCP03T_STRUCTURE_ERROR)) | #R_HTTP_204_OK |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #10 Error: PIR Error Reason – SCP03t Security Error

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PN_PIR_SCP03T_SECURITY_ERROR)) | #R_HTTP_204_OK |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #11 Error: PIR Error Reason – install Failed Due To Iccid Already Exists On eUICC

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PN_PIR_ICCID_ALREADY_EXISTS)) | #R_HTTP_204_OK |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #12 Error: PIR Error Reason – install Failed Due To Insufficient Memory For Profile

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | - PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PN_PIR_INSUFFICIENT_MEMORY)) | #R_HTTP_204_OK |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #13 Error: PIR Error Reason – install Failed Due To Interruption

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PN_PIR_INSTALL_INTERRUPTION)) | #R_HTTP_204_OK |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #14 Error: PIR Error Reason – install Failed Due To PE Processing Error

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | - PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PN_PIR_PE_PROCESSING_ERROR)) | #R_HTTP_204_OK |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #15 Error: PIR Error Reason – install Failed Due To Data Mismatch

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PN_PIR_DATA_MISMATCH)) | #R_HTTP_204_OK |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #16 Error: PIR Error Reason – test Profile Install Failed Due To Invalid Naa Key

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PN_PIR_TEST_PROFILE_INVALID_NAA_KEY)) | #R_HTTP_204_OK |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #17 Error: PIR Error Reason – PPR Not Allowed

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PN_PIR_PPR_NOT_ALLOWED)) | #R_HTTP_204_OK |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #18 Error: PIR Error Reason – install Failed Due To Unknown Error

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #S_PN_PIR_UNKNOWN_ERROR)) | #R_HTTP_204_OK |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

##### 4.3.15.2.2	TC_SM-DP+_ES9+_HandleNotificationFRP

This test case is defined as FFS and not applicable for this version of test specification.

##### 4.3.15.2.3	TC_SM-DP+_ES9+_HandleNotificationBRP

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for BRP. Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with an empty MatchingID. The EID is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. There have been no previous attempts to download pending Profile PROFILE_OPERATIONAL1. Confirmation Code is not provided by the Operator to the SM-DP+. |

Test Sequence #01 Nominal: All Notifications

This test sequence SHALL be the same as the Test Sequence #01 defined in section 4.3.15.2.1 TC_SM-DP+_ES9+_HandleNotificationNIST except that all auth/pb keys and certificates SHALL be based on BrainpoolP256r1.

### 4.3.16	ES9+ (LPA -- SM-DP+): CancelSession

#### 4.3.16.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Requirements

- 	RQ31_118, RQ31_119, RQ31_120, RQ31_121, RQ31_122, RQ31_123, RQ31_123_1, RQ31_124, RQ31_125, RQ31_126, RQ31_129, RQ31_160

- 	RQ45_006, RQ45_026, RQ45_026_1

- 	RQ55_048

- 	RQ56_043, RQ56_044, RQ56_045, RQ56_046, RQ56_047, RQ56_048, RQ56_049

- 	RQ57_114_1, RQ57_116

- 	RQ62_001, RQ62_002, RQ62_003, RQ62_004, RQ62_005, RQ62_006, RQ62_007, RQ62_009

- 	RQ63_004

- 	RQ65_001, RQ65_002, RQ65_003, RQ65_004, RQ65_005, RQ65_007, RQ65_008, RQ65_009, RQ65_025

#### 4.3.16.2	Test Cases

##### 4.3.16.2.1	TC_SM-DP+_ES9+.CancelSession_After_AuthenticateClientNIST

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for NIST. PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1. Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with an empty MatchingID. The EID is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. There have been no previous attempts to download the pending profile. |

Test Sequence #01 Nominal: End User Rejection after Authenticate Client

The purpose of this test is to verify that the LPAd can request the cancellation of an on-going RSP session using the 'End User Rejection' reason after Authenticate Client, and that the RSP session is terminated by the SM-DP+.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_EU_REJ)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #02 Nominal: End User Postponed after Authenticate Client

The purpose of this test is to verify that the LPAd can request the cancellation of an on-going RSP session using the 'End User postponed' reason after Authenticate Client, and the SM-DP+ keeps the RSP session’s corresponding Profile download order in the 'Released' state available for a further retry.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_POSTPONED)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 2 | PROC_ES9+_VERIFY_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC |  |  |

Test Sequence #03 Nominal: Timeout after Authenticate Client

The purpose of this test is to verify that the LPAd can request the cancellation of an on-going RSP session using the 'Timeout' reason after Authenticate Client, and the SM-DP+ keeps the RSP session's corresponding Profile download order in the 'Released' state available for a further retry.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_TIMEOUT)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 2 | PROC_ES9+_VERIFY_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC |  |  |

Test Sequence #04 Nominal: PPR Not Allowed after Authenticate Client

The purpose of this test is to verify that the LPAd can request the cancellation of an on-going RSP session using the 'PPR Not Allowed' reason after Authenticate Client, and that the RSP session is terminated by the SM-DP+.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 is configured with #SMDP_METADATA_OP_PROF1_PPR2. PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1_PPR2 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC using #R_AUTH_CLIENT_OK_PPR2 instead of #R_AUTH_CLIENT_OK |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_PPR_NOT_ALLOWED)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #05 Nominal: Undefined Reason after Authenticate Client

The purpose of this test is to verify that the LPAd can request the cancellation of an on-going RSP session using the 'Undefined Reason' reason after Authenticate Client, and that the RSP session is terminated by the SM-DP+.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_UNDEFINED)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #06 Error: Unknown Transaction ID in JSON transport layer (Subject Code 8.10.1, Reason Code 3.9) after Authenticate Client

The purpose of this test is to verify that if the LPAd requests the cancellation of an on-going RSP session using an Invalid Transaction ID after Authenticate Client, that the SM-DP+ returns a function execution status 'Failed' Subject Code 8.10.1, Reason Code 3.9, and keeps the RSP session's corresponding Profile download order in the 'Released' state available for a further retry.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <INVALID_TRANSACTION_ID>, #CS_RESP_OK_POSTPONED)) | MTD_HTTP_RESP( #R_ERROR_8_10_1_3_9) |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_POSTPONED)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 3 | PROC_ES9+_VERIFY_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC |  |  |

Test Sequence #07 Error: Unknown Transaction ID in ASN.1 CancelSessionResponse Element (Subject Code 8.10.1, Reason Code 3.9) after Authenticate Client

The purpose of this test is to verify that if the LPAd requests the cancellation of an on-going RSP session using an Invalid Transaction ID in the ASN.1 CancelSessionResponse element after Authenticate Client, that the SM-DP+ returns a function execution status 'Failed' with Subject Code 8.10.1, Reason Code 3.9, and keeps the RSP session's corresponding Profile download order in the 'Released' state available for a further retry.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_ERROR_8_10_1_3_9)) | MTD_HTTP_RESP( #R_ERROR_8_10_1_3_9) |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_POSTPONED)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 3 | PROC_ES9+_VERIFY_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC |  |  |

Test Sequence #08 Error: Invalid eUICC Signature (Subject Code 8.1 Reason Code 6.1) after Authenticate Client

The purpose of this test is to verify that if the LPAd requests the cancellation of an on-going RSP session using an Invalid Signature after Authenticate Client that the SM-DP+ returns a function execution status 'Failed' with Subject Code 8.1 Reason Code 6.1 and that the RSP session is stopped by the SM-DP+ and keeps the RSP session's corresponding Profile download order in the 'Released' state available for a further retry.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_ERROR_8_1_6_1)) | MTD_HTTP_RESP( #R_ERROR_8_1_6_1) |
| 2 | PROC_ES9+_VERIFY_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC |  |  |

Test Sequence #09 Error: Invalid OID (Subject Code 8.8 Reason Code 3.10) after Authenticate Client

The purpose of this test is to verify that if the LPAd requests the cancellation of an on-going RSP session using an Invalid OID after Authenticate Client that the SM-DP+ returns a function execution status 'Failed' with Subject Code 8.8 Reason Code 3.10 and that the RSP session is stopped by the SM-DP+ and keeps the RSP session's corresponding Profile download order in the 'Released' state available for a further retry.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_ERROR_8_8_3_10)) | MTD_HTTP_RESP( #R_ERROR_8_8_3_10) |
| 2 | PROC_ES9+_VERIFY_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC |  |  |

##### 4.3.16.2.2	TC_SM-DP+_ES9+.CancelSession_After_GetBoundProfilePackageNIST

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for NIST. Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with an empty MatchingID. The EID is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. There have been no previous attempts to download the pending profile. |

Test Sequence #01 Nominal: End User Rejection after GetBoundProfilePackage

The purpose of this test is to verify that the LPAd can request the cancellation of an on-going RSP session using the 'End User Rejection' reason after GetBoundProfilePackage, and that the RSP session is terminated by the SM-DP+.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_EU_REJ)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #02 Nominal: End User Postponed after GetBoundProfilePackage

The purpose of this test is to verify that the LPAd can request the cancellation of an on-going RSP session using the 'End User postponed' reason after GetBoundProfilePackage, and the SM-DP+ keeps the RSP session's corresponding Profile download order in the 'Released' state available for a further retry.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_POSTPONED)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 2 | PROC_ES9+_VERIFY_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC |  |  |

Test Sequence #03 Nominal: Timeout after GetBoundProfilePackage

The purpose of this test is to verify that the LPAd can request the cancellation of an on-going RSP session using the 'Timeout' reason after GetBoundProfilePackage , and the SM-DP+ keeps the RSP session's corresponding Profile download order in the 'Released' state available for a further retry.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_TIMEOUT)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 2 | PROC_ES9+_VERIFY_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC |  |  |

Test Sequence #04 Nominal: PPR Not Allowed after GetBoundProfilePackage

The purpose of this test is to verify that the LPAd can request the cancellation of an on-going RSP session using the 'PPR Not Allowed' reason after GetBoundProfilePackage, and that the RSP session is terminated by the SM-DP+.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 is configured with #SMDP_METADATA_OP_PROF1_PPR2. PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1_PPR2 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC using #R_AUTH_CLIENT_OK_PPR2 instead of #R_AUTH_CLIENT_OK |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_PPR_NOT_ALLOWED)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #05 Nominal: Metadata Mismatch after GetBoundProfilePackage

The purpose of this test is to verify that the LPAd can request the cancellation of an on-going RSP session using the 'Metadata Mismatch' reason after GetBoundProfilePackage, and that the RSP session is terminated by the SM-DP+.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_M_DATA_MISMATCH)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #06 Nominal: Load BPP Execution Error after GetBoundProfilePackage

The purpose of this test is to verify that if the LPAd requests the cancellation of an on-going RSP session using that the 'loadBppExecutionError' reason after GetBoundProfilePackage, that the RSP session is terminated by the SM-DP+.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_L_BPP_EXE_ERROR)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #07 Nominal: Undefined Reason after GetBoundProfilePackage

The purpose of this test is to verify that if the LPAd requests the cancellation of an on-going RSP session using the 'Undefined Reason' reason after GetBoundProfilePackage, and that the RSP session is terminated by the SM-DP+.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_UNDEFINED)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 2 | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |  |

Test Sequence #08 Error: Unknown Transaction ID in JSON transport layer (Subject Code 8.10.1, Reason Code 3.9) after GetBoundProfilePackage

The purpose of this test is to verify that if the LPAd requests the cancellation of an on-going RSP session using an Invalid Transaction ID after GetBoundProfilePackage that the SM-DP+ returns a function execution status 'Failed' Subject Code 8.10.1, Reason Code 3.9 and keeps the RSP session's corresponding Profile download order in the 'Released' state available for a further retry.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <INVALID_TRANSACTION_ID>, #CS_RESP_OK_POSTPONED)) | MTD_HTTP_RESP( #R_ERROR_8_10_1_3_9) |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_POSTPONED)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 3 | PROC_ES9+_VERIFY_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC |  |  |

Test Sequence #09 Error: Unknown Transaction ID in ASN.1 CancelSessionResponse Element (Subject Code 8.10.1, Reason Code 3.9) after GetBoundProfilePackage

The purpose of this test is to verify that if the LPAd requests the cancellation of an on-going RSP session using an Invalid Transaction ID in the ASN.1 CancelSessionResponse element after GetBoundProfilePackage that the SM-DP+ returns a function execution status 'Failed' with Subject Code 8.10.1, Reason Code 3.9 and keeps the RSP session's corresponding Profile download order in the 'Released' state available for a further retry.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_ERROR_8_10_1_3_9)) | MTD_HTTP_RESP( #R_ERROR_8_10_1_3_9) |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_POSTPONED)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 3 | PROC_ES9+_VERIFY_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC |  |  |

Test Sequence #10 Error: Invalid eUICC Signature (Subject Code 8.1 Reason Code 6.1) after GetBoundProfilePackage

The purpose of this test is to verify that if the LPAd can request the cancellation of an on-going RSP session using an Invalid Signature after GetBoundProfilePackage using S-ENC and S-MAC. But the SM-DP+ returns a function execution status 'Failed' with Subject Code 8.1 Reason Code 6.1 and that the RSP session is stopped by the SM-DP+ and keeps the RSP session's corresponding Profile download order in the 'Downloaded' state available for a further retry.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_ERROR_8_1_6_1)) | MTD_HTTP_RESP( #R_ERROR_8_1_6_1) |
| 2 | PROC_ES9+_VERIFY_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC |  |  |

Test Sequence #11 Error: Invalid OID (Subject Code 8.8 Reason Code 3.10) after GetBoundProfilePackage

The purpose of this test is to verify that if the LPAd requests the cancellation of an on-going RSP session using an Invalid OID after GetBoundProfilePackage that the SM-DP+ returns a function execution status 'Failed' with Subject Code 8.8 Reason Code 3.10 and that the RSP session is stopped by the SM-DP+ and keeps the RSP session's corresponding Profile download order in the 'Downloaded' state available for a further retry.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1 is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. Confirmation Code is not provided by the Operator to the SM-DP+. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_ERROR_8_8_3_10)) | MTD_HTTP_RESP( #R_ERROR_8_8_3_10) |
| 2 | PROC_ES9+_VERIFY_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC |  |  |

##### 4.3.16.2.3	TC_SM-DP+_ES9+.CancelSession_After_AuthenticateClientFRP

This test case is defined as FFS and not applicable for this version of test specification.

##### 4.3.16.2.4	TC_SM-DP+_ES9+.CancelSession_After_GetBoundProfilePackageFRP

This test case is defined as FFS and not applicable for this version of test specification.

##### 4.3.16.2.5	TC_SM-DP+_ES9+.CancelSession_After_AuthenticateClientBRP

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for BRP. PROFILE_OPERATIONAL1 configured with #SMDP_METADATA_OP_PROF1. Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with an empty MatchingID. The EID is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. There have been no previous attempts to download the pending profile. |

Test Sequence #01 Nominal: End User Rejection after Authenticate Client

This test sequence SHALL be the same as the Test Sequence #01 defined in section 4.3.16.2.1 TC_SM-DP+_ES9+.CancelSession_After_AuthenticateClientNIST except that all auth/pb keys and certificates SHALL be based on BrainpoolP256r1.

Test Sequence #02 Nominal: End User Postponed after Authenticate Client

This test sequence SHALL be the same as the Test Sequence #02 defined in section 4.3.16.2.1 TC_SM-DP+_ES9+.CancelSession_After_AuthenticateClientNIST except that all auth/pb keys and certificates SHALL be based on BrainpoolP256r1.

##### 4.3.16.2.6	TC_SM-DP+_ES9+.CancelSession_After_GetBoundProfilePackageBRP

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | SM-DP+ is configured with the #CERT_SM_DPauth_SIG for BRP. |

Test Sequence #01 Nominal: End User Rejection after GetBoundProfilePackage

This test sequence SHALL be the same as the Test Sequence #01 defined in section 4.3.16.2.2 TC_SM-DP+_ES9+.CancelSession_After_GetBoundProfilePackageNIST except that all auth/pb keys and certificates SHALL be based on BrainpoolP256r1.

Test Sequence #02 Nominal: End User Postponed after GetBoundProfilePackage

This test sequence SHALL be the same as the Test Sequence #02 defined in section 4.3.16.2.2 TC_SM-DP+_ES9+.CancelSession_After_GetBoundProfilePackageNIST except that all auth/pb keys and certificates SHALL be based on BrainpoolP256r1.

### 4.3.17	ES9+ (LPA -- SM-DP+): TLS, Server Authentication, Session Establishment

#### 4.3.17.1	TC_SM-DP+_ES9+_Server_Authentication_for_HTTPS_EstablishmentNIST

Perform all test sequences defined in section 4.6.3.2.1 with the following variables set as follows:

- 	SERVER = SM-DP+ under test

- o	CERT_SERVER_TLS = #CERT_SM_DP_TLS

#### 4.3.17.2	TC_SM-DP+_ES9+_Server_Authentication_for_HTTPS_EstablishmentBRP

Perform all test sequences defined in section 4.6.3.2.2 with the following variables set as follows:

- 	SERVER = SM-DP+ under test

- o	CERT_SERVER_TLS = #CERT_SM_DP_TLS

### 4.3.18	ES12 (SM-DP+ -- SM-DS): RegisterEvent

This test case is defined as FFS and not applicable for this version of test specification.

### 4.3.19	ES12 (SM-DP+ -- SM-DS): DeleteEvent

This test case is defined as FFS and not applicable for this version of test specification.

### 4.3.20	ES12 (SM-DP+ -- SM-DS): TLS, Mutual Authentication, Client, Session Establishment

#### 4.3.20.1	TC_SM-DP+_ES12_Client_Mutual_Authentication_for_HTTPS_EstablishmentNIST

Perform all test sequences defined in section 4.6.1.2.1 with the following variables set as follows:

- 	CLIENT = SM-DP+ under test

- o	CERT_CLIENT_TLS = #CERT_SM_DP_TLS for NIST

- 	SERVER = S_SM-DS

- o	CERT_S_SERVER_TLS = #CERT_S_SM_DS_TLS for NIST

#### 4.3.20.2 TC_SM-DP+_ES12_Client_Mutual_Authentication_for_HTTPS_EstablishmentBRP

Perform all test sequences defined in section 4.6.1.2.2 with the following variables set as follows:

- 	CLIENT = SM-DP+ under test

- o	CERT_CLIENT_TLS = #CERT_SM_DP_TLS for BRP

- 	SERVER = S_SM-DS

- o	CERT_S_SERVER_TLS = #CERT_S_SM_DS_TLS for BRP

## 4.4	VOID

## 4.5	SM-DS Interfaces

### 4.5.1	ES12 (SM-DP+ -- SM-DS): RegisterEvent

#### 4.5.1.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Requirements

- 	RQ36_004, RQ36_005, RQ36_006, RQ36_007, RQ36_008, RQ36_009, RQ36_010, RQ36_011, RQ36_012, RQ36_013

- 	RQ59_003, RQ59_004, RQ59_005, RQ59_006, RQ59_007, RQ59_009, RQ59_010, RQ59_011, RQ59_012, RQ59_013, RQ59_014, RQ59_015

- 	RQ62_001, RQ62_002, RQ62_004, RQ62_005, RQ62_006, RQ62_007

- 	RQ65_001, RQ65_002, RQ65_003, RQ65_005, RQ65_007, RQ65_008, RQ65_009, RQ65_030

#### 4.5.1.2	Test Cases

##### 4.5.1.2.1	TC_ROOT_SM_DS_ES12.RegisterEvent

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Root SM-DS | No TLS connections are established between the Root SM-DS and any of the simulator test tools. |

Test Sequence #01 Nominal: EventID Registration to SM-DS without Event forwarding

The purpose of this test is to verify that the SM-DS can perform Event Registration without Event forwarding set.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Root SM-DS | #EVENT_ID_1 is not already used by the Root SM-DS. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES12 |  |  |
| 1 | S_SM-DP+ → Root SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( #S_SM_DP+_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #TEST_DP_ADDRESS1, #EVENT_ID_1, FALSE)) | MTD_HTTP_RESP(#R_SUCCESS) |
| 2 | S_LPAd → Root SM-DS | PROC_ES11_VERIFY_EVENT_RETRIEVAL_NO_EVENT_ID |  |

Test Sequence #02 Nominal: EventID Registration to SM-DS with Event forwarding

The purpose of this test is to verify that the SM-DS ignores the ForwardingIndicator and successfully performs Event Registration with Event forwarding set.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Root SM-DS | #EVENT_ID_1 is not already used by the Root SM-DS |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES12 |  |  |
| 1 | S_SM-DP+ → Root SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( #S_SM_DP+_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #TEST_DP_ADDRESS1, #EVENT_ID_1, TRUE)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 2 | S_LPAd → Root SM-DS | PROC_ES11_VERIFY_EVENT_RETRIEVAL_NO_EVENT_ID |  |

Test Sequence #03 Nominal: EventID Registration to SM-DS with Hashed ICCID and without Event forwarding

The purpose of this test is to verify that the SM-DS can perform Event Registration without Event forwarding set where Hashed ICCID is present.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Root SM-DS | #EVENT_ID_1 is not already used by the Root SM-DS. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES12 |  |  |
| 1 | S_SM-DP+ → Root SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT_HASHED_ICCID( #S_SM_DP+_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #TEST_DP_ADDRESS1, #EVENT_ID_1, FALSE, #HASHED_ICCID_OP_PROF1)) | MTD_HTTP_RESP(#R_SUCCESS) |
| 2 | S_LPAd → Root SM-DS | PROC_ES11_VERIFY_EVENT_RETRIEVAL_NO_EVENT_ID |  |

Test Sequence #04 Nominal: EventID Registration to SM-DS with Hashed Salted ICCID and without Event forwarding

The purpose of this test is to verify that the SM-DS can perform Event Registration without Event forwarding set where Hashed ICCID and Salt are present.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Root SM-DS | #EVENT_ID_1 is not already used by the Root SM-DS. |

| Step | Direction | Sequence / Description | Expected result |  |
| --- | --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES12 |  |  |  |
| 1 | S_SM-DP+ → Root SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT_HASHED_SALTED_ICCID( #S_SM_DP+_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #TEST_DP_ADDRESS1, #EVENT_ID_1, FALSE, #HASHED_SALTED_ICCID_OP_PROF1, #SALT)) |  | MTD_HTTP_RESP(#R_SUCCESS) |
| 2 | S_LPAd → Root SM-DS | PROC_ES11_VERIFY_EVENT_RETRIEVAL_NO_EVENT_ID |  |  |

Test Sequence #05 Error: Event Record Already Exists without Event Forwarding (Subject Code 8.9.5 Reason Code 3.3)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Root SM-DS | #EVENT_ID_1 is already used by the Root SM-DS for #EID2, registered with S_SM_DP+_OID.. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES12 |  |  |
| 1 | S_SM-DP+ → Root SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( #S_SM_DP+_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #TEST_DP_ADDRESS1, #EVENT_ID_1, FALSE)) | MTD_HTTP_RESP( #R_ERROR_8_9_5_3_3) |
| 2 | S_LPAd → Root SM-DS | PROC_ES11_VERIFY_EVENT_RETRIEVAL_NO_EVENT_ID_ERROR |  |

##### 4.5.1.2.2	TC_ALT_SM_DS_ES12.RegisterEvent

The test sequences in this section test the Alternative SM-DS acting as a Server on ES12 and a Client on ES15.

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Alt. SM-DS | No TLS connections are established between the Alternative SM-DS and any of the simulator test tools. |

Test Sequence #01 Nominal: EventID Registration on Alternative SM-DS with Event forwarding

The purpose of this test is to verify that Alternative SM-DS can perform Event Registration with Event forwarding set.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Alt. SM-DS | #EVENT_ID_1 is not already used by the Alternative SM-DS. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | S_SM-DP+ → Alt. SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES12 |  |
| 1 | S_SM-DP+ → Alt. SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( #S_SM_DP+_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #TEST_DP_ADDRESS1, #EVENT_ID_1, TRUE)) |  |
| 2 | Alt. SM-DS → S_SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES15 |  |
| 3 | Alt. SM-DS → S_SM-DS | Call ES15.RegisterEvent | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( <FUNCTION_REQ_ID>, <FUNCTION_CALL_ID>, #EID1, #IUT_SM_DS_ADDRESS_ES11, <EVENT_ID_R>, FALSE)) |
| 4 | S_SM-DS → Alt. SM-DS | MTD_HTTP_RESP(#R_SUCCESS) on ES15 | No Error |
| 5 | Alt. SM-DS → S_SM-DP+ | Successful result is sent to the S_SM-DP+ | MTD_HTTP_RESP(#R_SUCCESS) on ES12 |
| 6 | S_LPAd → Alt. SM-DS | PROC_ES11_VERIFY_EVENT_RETRIEVAL_EVENT_ID |  |

Test Sequence #02 Nominal: Uniqueness of EventID Registration by Alternative SM-DS with Event forwarding

The purpose of this test is to verify that Alternative SM-DS can perform Event Registration using a unique EventID2 value with Event forwarding set.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Alt. SM-DS | #EVENT_ID_1 is not already used by the Alternative SM-DS. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | S_SM-DP+ → Alt. SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES12 |  |
| 1 | S_SM-DP+ → Alt. SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( #S_SM_DP+_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #TEST_DP_ADDRESS1, #EVENT_ID_1, TRUE)) |  |
| 2 | Alt. SM-DS → S_SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES15 |  |
| 3 | Alt. SM-DS → S_SM-DS | Call ES15.RegisterEvent | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( <FUNCTION_REQ_ID>, <FUNCTION_CALL_ID>, #EID1, #IUT_SM_DS_ADDRESS_ES11, <EVENT_ID_R>, FALSE)) Extract the value of <EVENT_ID_R> |
| 4 | S_SM-DS → Alt. SM-DS | MTD_HTTP_RESP(#R_SUCCESS) on ES15 | No Error |
| 5 | Alt. SM-DS → S_SM-DP+ | Successful result is sent to the S_SM-DP+ | MTD_HTTP_RESP(#R_SUCCESS) on ES12 |
| 6 | S_SM-DP+ → Alt. SM-DS | Close TLS session on ES12 (unless Alternative SM-DS has already closed TLS session) |  |
| 7 | S_SM-DP+ → Alt. SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES12 |  |
| 8 | S_SM-DP+ → Alt. SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( #S_SM_DP+_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #TEST_DP_ADDRESS1, #EVENT_ID_2, TRUE)) |  |
| 9 | Alt. SM-DS → S_SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES15 |  |
| 10 | Alt. SM-DS → S_SM-DS | Call ES15.RegisterEvent | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( <FUNCTION_REQ_ID>, <FUNCTION_CALL_ID>, #EID1, #IUT_SM_DS_ADDRESS_ES11, <EVENT_ID_R>, FALSE)) Verify that <EVENT_ID_R> in step 3 is not equal to <EVENT_ID_R> |

Test Sequence #03 Error: SM-DS registration failed, Root SM-DS unavailable (Subject Code 8.9 Reason Code 5.1)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Alt. SM-DS | #EVENT_ID_1 is not already used by the Alternative SM-DS. S_SM_DS (Root SM-DS simulator) is not available – it will not respond to any client attempts to connect. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | S_SM-DP+ → Alt. SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES12 |  |
| 1 | S_SM-DP+ → Alt. SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( #S_SM_DP+_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #TEST_DP_ADDRESS1, #EVENT_ID_1, TRUE)) | MTD_HTTP_RESP( #R_ERROR_8_9_5_1) |
| 2 | S_LPAd → Alt. SM-DS | PROC_ES11_VERIFY_EVENT_RETRIEVAL_NO_EVENT_ID_ERROR |  |

Test Sequence #04 Error: SM-DS registration failed, Root SM-DS error (Subject Code 8.9 Reason Code 4.2)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Alt. SM-DS | #EVENT_ID_1 is not already used by the Alternative SM-DS for #EID1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | S_SM-DP+ → Alt. SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES12 |  |
| 1 | S_SM-DP+ → Alt. SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( #S_SM_DP+_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #TEST_DP_ADDRESS1, #EVENT_ID_1, TRUE)) |  |
| 2 | Alt. SM-DS → S_SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES15 |  |
| 3 | Alt. SM-DS → S_SM-DS | Call ES15.RegisterEvent | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( <FUNCTION_REQ_ID>, <FUNCTION_CALL_ID>, #EID1, #IUT_SM_DS_ADDRESS_ES11, <EVENT_ID_R>, FALSE)) |
| 4 | S_SM-DS → Alt. SM-DS | MTD_HTTP_RESP( #R_ERROR_1_2_4_2) | No Error |
| 5 | Alt. SM-DS → S_SM-DP+ | SM-DS forwards error response back to S_SM-DP+ | MTD_HTTP_RESP( #R_ERROR_8_9_4_2) |
| 6 | S_LPAd → Alt. SM-DS | PROC_ES11_VERIFY_EVENT_RETRIEVAL_EVENT_ID_ERROR |  |

Test Sequence #05 Error: Event Record Already Exists on Alternative SM-DS (Subject Code 8.9.5 Reason Code 3.3)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Alt. SM-DS | #EVENT_ID_1 is already used by the Alternative SM-DS, registered with S_SM_DP+_OID for #EID2. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | S_SM-DP+ → Alt. SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES12 |  |
| 1 | S_SM-DP+ → Alt. SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( #S_SM_DP+_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #TEST_DP_ADDRESS1, #EVENT_ID_1, TRUE)) | MTD_HTTP_RESP( #R_ERROR_8_9_5_3_3) |
| 2 | S_LPAd → Alt. SM-DS | PROC_ES11_VERIFY_EVENT_RETRIEVAL_NO_EVENT_ID_ERROR |  |

### 4.5.2	ES12 (SM-DS -- SM-DP+): DeleteEvent

#### 4.5.2.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Requirements

- 	RQ36_024, RQ36_025, RQ36_025_1, RQ36_027, RQ36_028, RQ36_029, RQ36_030, RQ36_031, RQ36_032

- RQ510_019, RQ510_020

- 	RQ59_016, RQ59_016_1, RQ59_017, RQ59_017_1, RQ59_017_2, RQ59_018, RQ59_019, RQ59_021, RQ59_022, RQ59_023, RQ59_024, RQ59_025

- 	RQ62_001, RQ62_002, RQ62_004, RQ62_005, RQ62_006, RQ62_007

- 	RQ65_001, RQ65_002, RQ65_003, RQ65_005, RQ65_007, RQ65_008, RQ65_009, RQ65_031

#### 4.5.2.2	Test Cases

##### 4.5.2.2.1	TC_ROOT_SM_DS_ES12.DeleteEvent

Test Sequence #01 Nominal: Event Deletion

The purpose of this test is to verify that the Root SM-DS can perform Event Deletion.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Root SM-DS | #EVENT_ID_1 was registered for #EID1 and #TEST_DP_ADDRESS1, registered with S_SM_DP+_OID |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES12 |  |  |
| 1 | S_SM-DP+ → Root SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_DELETE_EVENT, MTD_DELETE_EVENT( #S_SM_DP+_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #EVENT_ID_1)) | MTD_HTTP_RESP(#R_SUCCESS) |
| 2 | S_LPAd → Root SM-DS | PROC_ES11_VERIFY_EVENT_RETRIEVAL_NO_EVENT_ID_ERROR |  |

Test Sequence #02 Error: Event Record Does Not Exist (Subject Code 8.9.5 Reason Code 3.9)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Root SM-DS | #EVENT_ID_1 is not registered. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES12 |  |  |
| 1 | S_SM-DP+ → Root SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_DELETE_EVENT, MTD_DELETE_EVENT( #S_SM_DP+_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #EVENT_ID_1)) | MTD_HTTP_RESP( #R_ERROR_8_9_5_3_9) |

Test Sequence #03 Error: Event Record Does Not Match OID (Subject Code 8.9.5 Reason Code 3.9)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Root SM-DS | #EVENT_ID_1 was registered for #EID1 and #TEST_DP_ADDRESS1, registered with S_SM_DP+_OID. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | S_SM-DP+ → Root SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH_INV_OID on ES12 |  |
| 1 | S_SM-DP+ → Root SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_DELETE_EVENT, MTD_DELETE_EVENT( #S_SM_DP+_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #EVENT_ID_1)) | MTD_HTTP_RESP( #R_ERROR_8_9_5_3_9) |
| 2 | S_LPAd → Root SM-DS | PROC_ES11_VERIFY_EVENT_RETRIEVAL_NO_EVENT_ID |  |

##### 4.5.2.2.2	TC_ALT_SM_DS_ES12.DeleteEvent

The test sequences in this section test the Alternative SM-DS acting as a Server on ES12 and a Client on ES15.

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Alt. SM-DS | No TLS connections are established between the Alternative SM-DS and any of the simulator test tools. |

Test Sequence #01 Nominal: Cascaded Event Deletion on Alternative SM-DS

The purpose of this test is to verify that Alternative SM-DS can perform cascaded Event Deletion.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Alt. SM-DS | #EVENT_ID_1 registration for #EID1 and #TEST_DP_ADDRESS1 (registered with S_SM_DP+_OID) was cascaded using <EVENT_ID_R> to the Root SM-DS. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | S_SM-DP+ → Alt. SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES12 |  |
| 1 | S_SM-DP+ → Alt. SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_DELETE_EVENT, MTD_DELETE_EVENT( #S_SM_DP+_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #EVENT_ID_1)) |  |
| 2 | Alt. SM-DS → S_SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES15 |  |
| 3 | Alt. SM-DS → S_SM-DS | Call ES15.DeleteEvent | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_DELETE_EVENT, MTD_DELETE_EVENT( <FUNCTION_REQ_ID>, <FUNCTION_CALL_ID>, #EID1, <EVENT_ID_D>)) Verify that <EVENT_ID_D> is equal to <EVENT_ID_R> |
| 4 | S_SM-DS → Alt. SM-DS | MTD_HTTP_RESP( #R_SUCCESS) on ES15 | No Error |
| 5 | Alt. SM-DS → S_SM-DP+ | SM-DS sends response back to S_SM-DP+ | MTD_HTTP_RESP( #R_SUCCESS) on ES12 |
| 6 | S_LPAd → Alt. SM-DS | PROC_ES11_VERIFY_EVENT_RETRIEVAL_EVENT_ID_ERROR |  |

Test Sequence #02 Nominal: Cascaded Event Deletion, Event Record not found on Root SM-DS

The purpose of this test is to verify that if cascaded deletion fails because the Event Record was not found in the Root SM-DS the Alternative SM-DS can ignore this error case and continue.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Alt. SM-DS | #EVENT_ID_1 registration for #EID1 and #TEST_DP_ADDRESS1 (registered with S_SM_DP+_OID) was cascaded using <EVENT_ID_R> to the Root SM-DS. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | S_SM-DP+ → Alt. SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES12 |  |
| 1 | S_SM-DP+ → Alt. SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_DELETE_EVENT, MTD_DELETE_EVENT( #S_SM_DP+_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #EVENT_ID_1)) |  |
| 2 | Alt. SM-DS → S_SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES15 |  |
| 3 | Alt. SM-DS → S_SM-DS | Call ES15.DeleteEvent | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_DELETE_EVENT, MTD_DELETE_EVENT( <FUNCTION_REQ_ID>, <FUNCTION_CALL_ID>, #EID1, <EVENT_ID_D>)) Verify that <EVENT_ID_D> is equal to <EVENT_ID_R> |
| 4 | S_SM-DS → Alt. SM-DS | MTD_HTTP_RESP( #R_ERROR_8_9_5_3_9) | No Error |
| 5 | Alt. SM-DS → S_SM-DP+ | SM-DS sends response back to S_SM-DP+ | MTD_HTTP_RESP( #R_SUCCESS) on ES12 |
| 6 | S_LPAd → Alt. SM-DS | PROC_ES11_VERIFY_EVENT_RETRIEVAL_EVENT_ID_ERROR |  |

Test Sequence #03 Error: Cascaded Event Deletion failed, Root SM-DS Unavailable (Subject Code 8.9 Reason Code 5.1)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Alt. SM-DS | #EVENT_ID_1 registration for #EID1 and #TEST_DP_ADDRESS1 (registered with S_SM_DP+_OID) was cascaded using <EVENT_ID_R> to the Root SM-DS. S_SM_DS (Root SM-DS simulator) is not available – it will not respond to any client attempts to connect. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | S_SM-DP+ → Alt. SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES12 |  |
| 1 | S_SM-DP+ → Alt. SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_DELETE_EVENT, MTD_DELETE_EVENT( #S_SM_DP+_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #EVENT_ID_1)) | MTD_HTTP_RESP( #R_ERROR_8_9_5_1) |
| 2 | S_LPAd → Alt. SM-DS | PROC_ES11_VERIFY_EVENT_RETRIEVAL_EVENT_ID |  |

Test Sequence #04 Error: Cascaded Event Deletion failed, Root SM-DS execution error (Subject Code 8.9 Reason Code 4.2)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Alt. SM-DS | #EVENT_ID_1 registration for #EID1 and #TEST_DP_ADDRESS1 (registered with S_SM_DP+_OID) was cascaded using <EVENT_ID_R> to the Root SM-DS. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | S_SM-DP+ → Alt. SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES12 |  |
| 1 | S_SM-DP+ → Alt. SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_DELETE_EVENT, MTD_DELETE_EVENT( #S_SM_DP+_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #EVENT_ID_1)) |  |
| 2 | Alt. SM-DS → S_SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES15 |  |
| 3 | Alt. SM-DS → S_SM-DS | Call ES15.DeleteEvent | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_DELETE_EVENT, MTD_DELETE_EVENT( <FUNCTION_REQ_ID>, <FUNCTION_CALL_ID>, #EID1, <EVENT_ID_D>)) |
| 4 | S_SM-DS → Alt. SM-DS | MTD_HTTP_RESP( #R_ERROR_1_2_4_2) | No Error |
| 5 | Alt. SM-DS → S_SM-DP+ | SM-DS sends response back to S_SM-DP+ | MTD_HTTP_RESP( #R_ERROR_8_9_4_2) |
| 6 | S_LPAd → Alt. SM-DS | PROC_ES11_VERIFY_EVENT_RETRIEVAL_EVENT_ID |  |

Test Sequence #05 Error: Event Record Does Not Match OID (Subject Code 8.9.5 Reason Code 3.9)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Alt. SM-DS | #EVENT_ID_1 registration for #EID1 and #TEST_DP_ADDRESS1 (registered with S_SM_DP+_OID) was cascaded using <EVENT_ID_R> to the Root SM-DS. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | S_SM-DP+ → Alt. SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH_INV_OID on ES12 |  |
| 1 | S_SM-DP+ → Alt. SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_DELETE_EVENT, MTD_DELETE_EVENT( #S_SM_DP+_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #EVENT_ID_1)) | MTD_HTTP_RESP( #R_ERROR_8_9_5_3_9) |
| 2 | S_LPAd → Alt. SM-DS | PROC_ES11_VERIFY_EVENT_RETRIEVAL_EVENT_ID |  |

##### 4.5.2.2.3	TC_ALT_SM_DS_ES12.DeleteEvent_Error_Nonexistant_EventID

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Alt. SM-DS | No TLS connections are established between the Alternative SM-DS and any of the simulator test tools. |

Test Sequence #01 Error: Event Record Does Not Exist (Subject Code 8.9.5 Reason Code 3.9)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Alt. SM-DS | #EVENT_ID_1 is not registered. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | S_SM-DP+ → Alt. SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES12 |  |
| 1 | S_SM-DP+ → Alt. SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_DELETE_EVENT, MTD_DELETE_EVENT( #S_SM_DP+_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #EVENT_ID_1)) | MTD_HTTP_RESP( #R_ERROR_8_9_5_3_9) |

### 4.5.3	ES15 (SM-DS -- SM-DS): RegisterEvent

#### 4.5.3.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Requirements

- 	RQ36_005, RQ36_010, RQ36_011, RQ36_012

- 	RQ62_001, RQ62_002, RQ62_005, RQ62_006

- 	RQ65_001, RQ65_002, RQ65_003, RQ65_005, RQ65_007, RQ65_008, RQ65_009, RQ65_030

- 	RQ510_003, RQ510_004, RQ510_005, RQ510_006, RQ510_009, RQ510_010, RQ510_011, RQ510_012, RQ510_013, RQ510_014, RQ510_015

#### 4.5.3.2	Test Cases

##### 4.5.3.2.1	TC_ROOT_SM_DS_ES15.RegisterEvent

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Root SM-DS | No TLS connections are established between the Root SM-DS and any of the simulator test tools. |

Test Sequence #01 Nominal: EventID Registration to SM-DS with Event forwarding

The purpose of this test is to verify that the Root SM-DS ignores the ForwardingIndicator and successfully performs Event Registration with Event forwarding set.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Root SM-DS | #EVENT_ID_1 is not already used by the Root SM-DS. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES15 |  |  |
| 1 | S_SM-DS → Root SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES15, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( #S_SM_DS_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #TEST_ALT_DS_ADDRESS, #EVENT_ID_1, TRUE)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 2 | S_LPAd → Root SM-DS | PROC_ES11_VERIFY_EVENT_RETRIEVAL_NO_EVENT_ID using R_AUTH_CLIENT_DS_EVENT_ENTRY_1_ALT_DS_OK instead of R_AUTH_CLIENT_DS_EVENT_ENTRY_1_OK |  |

Test Sequence #02 Nominal: EventID Registration to SM-DS without Event forwarding

The purpose of this test is to verify that the Root SM-DS successfully performs Event Registration with Event without Event forwarding set.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Root SM-DS | #EVENT_ID_1 is not already used by the Root SM-DS. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES15 |  |  |
| 1 | S_SM-DS → Root SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES15, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( #S_SM_DS_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #TEST_ALT_DS_ADDRESS, #EVENT_ID_1, FALSE)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 2 | S_LPAd → Root SM-DS | PROC_ES11_VERIFY_EVENT_RETRIEVAL_NO_EVENT_ID using R_AUTH_CLIENT_DS_EVENT_ENTRY_1_ALT_DS_OK instead of R_AUTH_CLIENT_DS_EVENT_ENTRY_1_OK |  |

Test Sequence #03 Error: Event Record Already Exists without Event Forwarding (Subject Code 8.9.5 Reason Code 3.3)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Root SM-DS | #EVENT_ID_1 is already used by the Root SM-DS for #EID2, registered with S_SM_DS_OID. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES15 |  |  |
| 1 | S_SM-DS → Root SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES15, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( #S_SM_DS_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #TEST_ALT_DS_ADDRESS, #EVENT_ID_1, FALSE)) | MTD_HTTP_RESP( #R_ERROR_8_9_5_3_3) |
| 2 | S_LPAd → Root SM-DS | PROC_ES11_VERIFY_EVENT_RETRIEVAL_NO_EVENT_ID_ERROR |  |

### 4.5.4	ES15 (SM-DS -- SM-DS): DeleteEvent

#### 4.5.4.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Requirements

- 	RQ36_028, RQ36_029, RQ36_030, RQ36_031

- 	RQ62_001, RQ62_002, RQ62_005, RQ62_006

- 	RQ65_001, RQ65_002, RQ65_003, RQ65_005, RQ65_007, RQ65_008, RQ65_009, RQ65_031

- 	RQ510_016, RQ510_016_1, RQ510_021, RQ510_022, RQ510_023, RQ510_024, RQ510_025

#### 4.5.4.2	Test Cases

##### 4.5.4.2.1	TC_ROOT_SM_DS_ES15.DeleteEvent

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Root SM-DS | No TLS connections are established between the Alternative SM-DS and any of the simulator test tools. |

Test Sequence #01 Nominal: Event Deletion

The purpose of this test is to verify that the Root SM-DS can perform Event Deletion.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Root SM-DS | #EVENT_ID_1 was registered for #EID1 and #TEST_ALT_DS_ADDRESS, registered with S_SM_DS_OID. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES15 |  |  |
| 1 | S_SM-DS → Root SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES15, #PATH_DELETE_EVENT, MTD_DELETE_EVENT( #S_SM_DS_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #EVENT_ID_1)) | MTD_HTTP_RESP( #R_SUCCESS) |
| 2 | S_LPAd → Root SM-DS | PROC_ES11_VERIFY_EVENT_RETRIEVAL_NO_EVENT_ID_ERROR |  |

Test Sequence #02 Error: Event Record Does Not Exist (Subject Code 8.9.5 Reason Code 3.9)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Root SM-DS | #EVENT_ID_1 is not registered. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH on ES15 |  |  |
| 1 | S_SM-DS → Root SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES15, #PATH_DELETE_EVENT, MTD_DELETE_EVENT( #S_SM_DS_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #EVENT_ID_1)) | MTD_HTTP_RESP( #R_ERROR_8_9_5_3_9) |

Test Sequence #03 Error: Event Record Does Not Match OID (Subject Code 8.9.5 Reason Code 3.9)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| Root SM-DS | #EVENT_ID_1 was registered for #EID1 and #TEST_ALT_DS_ADDRESS, registered with S_SM_DS_OID. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | S_SM-DS → Root SM-DS | PROC_TLS_INITIALIZATION_MUTUAL_AUTH_INV_OID on ES15 |  |
| 1 | S_SM-DS → Root SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES15, #PATH_DELETE_EVENT, MTD_DELETE_EVENT( #S_SM_DS_F_REQ_ID, #FUNCTION_CALL_ID_1, #EID1, #EVENT_ID_1)) | MTD_HTTP_RESP( #R_ERROR_8_9_5_3_9) |
| 2 | S_LPAd → Root SM-DS | PROC_ES11_VERIFY_EVENT_RETRIEVAL_NO_EVENT_ID using #R_AUTH_CLIENT_DS_EVENT_ENTRY_1_ALT_DS_OK instead of #R_AUTH_CLIENT_DS_EVENT_ENTRY_1_OK |  |

### 4.5.5	ES11 (LPA -- SM-DS): InitiateAuthentication

#### 4.5.5.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Requirements

Section 2.6.6.2

Section 3.0.1, 3.1.3

Section 4.5.2.1, 4.5.2.2

Section 5.7.13

Section 5.8.1

Section 6.2

Section 6.5.1, 6.5.1.1, 6.5.1.2, 6.5.1.3, 6.5.1.4, 6.5.2, 6.5.2.6

#### 4.5.5.2	Test Cases

##### 4.5.5.2.1	TC_SM_DS_ES11.InitiateAuthenticationNIST

| General Initial Conditions for SM-DS testing |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DS | SM-DS is configured with the #CERT_SM_DSauth_SIG for NIST. |

Perform all test sequences defined in 4.3.12.2.1 with the following variables:

- 	Test Environment  = TE_S1

- 	SERVER = SM-DS

- o	CERT_SM_XXauth_SIG = CERT_SM_DSauth_SIG

- o	PK_SM_XXauth_SIG = PK_SM_DSauth_SIG

##### 4.5.5.2.2	TC_SM_DS_ES11.InitiateAuthenticationBRP

| General Initial Conditions for SM-DS testing |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DS | SM-DS is configured with the #CERT_SM_DSauth_SIG for BRP. |

Perform all test sequences defined in 4.3.12.2.3 with the following variables:

- 	Test Environment  = TE_S1

- 	SERVER = SM-DS

- o	CERT_SM_XXauth_SIG = CERT_SM_DSauth_SIG

- o	PK_SM_XXauth_SIG = PK_SM_DSauth_SIG

### 4.5.6	ES11 (LPA -- SM-DS): AuthenticateClient

#### 4.5.6.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Requirements

- 	RQ26_005, RQ26_006, RQ26_012, RQ26_014

- 	RQ31_058, RQ31_059, RQ31_060

- 	RQ36_017, RQ36_021, RQ36_022

- 	RQ45_006, RQ45_026, RQ45_026_1, RQ45_027, RQ45_028, RQ45_029

- 	RQ57_037, RQ57_108

- 	RQ58_025, RQ58_026, RQ58_027, RQ58_028, RQ58_029, RQ58_031, RQ58_036, RQ58_036_1, RQ58_037, RQ58_038, RQ58_039

- 	RQ62_001, RQ62_002

- RQ65_27, RQ65_028, RQ65_029

#### 4.5.6.2	Test Cases

##### 4.5.6.2.1	TC_SM-DS_ES11.AuthenticateClientNIST

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DS | SM-DS is configured with the #CERT_SM_DSauth_SIG for NIST. |

Test Sequence #01 Nominal Matching ID Empty for one pending Event

The purpose of this test is to verify that common mutual authentication between the SM-DS and the S_LPAd is performed successfully with an empty Matching ID, and that Event Retrieval occurs for one pending Event.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DS | #EVENT_ID_1 has been registered in the SM-DS with #EID1 and #TEST_DP_ADDRESS1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| IC2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_MATCHING_ID_EMPTY)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_DS_EVENT_ENTRY_1_OK) |

Test Sequence #02 Nominal Matching ID Empty for two pending Events

The purpose of this test is to verify that common mutual authentication between the SM-DS and the S_LPAd is performed successfully with an empty Matching ID, and that Event Retrieval occurs for any pending Events.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DS | #EVENT_ID_1 has been registered in the SM-DS with #EID1 and #TEST_DP_ADDRESS1. #EVENT_ID_2 has been registered in the SM-DS with #EID1 and #TEST_DP_ADDRESS2. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| IC2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_MATCHING_ID_EMPTY)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_DS_EVENT_ENTRY_MULTI_OK) |

Test Sequence #03 Nominal Matching ID Empty for no pending Events

The purpose of this test is to verify that common mutual authentication between the SM-DS and the S_LPAd is performed successfully with an empty Matching ID, and that Event Retrieval returns no pending Events.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DS | No Events have been registered in the SM-DS for #EID1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| IC2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_MATCHING_ID_EMPTY)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_DS_EVENT_ENTRY_EMPTY_OK) |

Test Sequence #04 Nominal Matching ID Omitted for one pending Event

The purpose of this test is to verify that common mutual authentication between the SM-DS and the S_LPAd is performed successfully with the Matching ID omitted, and that Event Retrieval occurs for one pending Event.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DS | #EVENT_ID_1 has been registered in the SM-DS with #EID1 and #TEST_DP_ADDRESS1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| IC2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_MATCHING_ID_OMITTED)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_DS_EVENT_ENTRY_1_OK) |

Test Sequence #05 Nominal Matching ID Omitted for two pending Events

The purpose of this test is to verify that common mutual authentication between the SM-DS and the S_LPAd is performed successfully with the Matching ID omitted, and that Event Retrieval occurs for any pending Events.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DS | #EVENT_ID_1 has been registered in the SM-DS with #EID1 and #TEST_DP_ADDRESS1. #EVENT_ID_2 has been registered in the SM-DS with #EID1 and #TEST_DP_ADDRESS2. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| IC2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_MATCHING_ID_OMITTED)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_DS_EVENT_ENTRY_MULTI_OK) |

Test Sequence #06 Nominal Matching ID Omitted for no pending Events

The purpose of this test is to verify that common mutual authentication between the SM-DS and the S_LPAd is performed successfully with the Matching ID omitted, and that Event Retrieval returns no pending Events.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DS | No Events have been registered in the SM-DS for #EID1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| IC2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_MATCHING_ID_OMITTED)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_DS_EVENT_ENTRY_EMPTY_OK) |

Test Sequence #07 Alt. Nominal Matching ID containing EventID with one pending Event

The purpose of this test is to verify that common mutual authentication between the Alternative SM-DS and the S_LPAd is performed successfully with a Matching ID containing an EventID, and that Event Retrieval occurs for the requested pending Event.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DS | #EVENT_ID_1 has been registered in the Alternative SM-DS with #EID1 and #TEST_DP_ADDRESS1 and was cascaded using <EVENT_ID_R> to the Root SM-DS. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| IC2 | S_LPAd → Alt. SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → Alt. SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_MATCHING_ID_EVENT_ID_R)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_DS_EVENT_ENTRY_1_OK) |

Test Sequence #08 Alt Nominal Matching ID containing EventID with two pending Events

The purpose of this test is to verify that common mutual authentication between the Alternative SM-DS and the S_LPAd is performed successfully with a Matching ID containing an EventID, and that Event Retrieval occurs for only the requested pending Event.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DS | #EVENT_ID_1 has been registered in the Alternative SM-DS with #EID1 and #TEST_DP_ADDRESS1 and was cascaded using <EVENT_ID_R> to the Root SM-DS. #EVENT_ID_2 has been registered in the Alternative SM-DS with #EID1 and #TEST_DP_ADDRESS2 and was cascaded, using an EventID which is different from <EVENT_ID_R>, to the Root SM-DS. |

Repeat Test Sequence #07 Nominal Matching ID containing one Event with one pending Event.

Test Sequence #09 Error: Invalid EUM Certificate (Subject Code 8.1.2 Reason Code 6.1)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DS | #EVENT_ID_1 has been registered in the SM-DS with #EID1 and #TEST_DP_ADDRESS1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| IC2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_SMDS_8_1_2_6_1_SIG)) | MTD_HTTP_RESP( #R_ERROR_8_1_2_6_1) |
| 2 | S_LPAd → SM-DS | Close TLS session (unless SM-DS has already closed TLS session) |  |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| 4 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 5 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_SMDS_8_1_2_6_1_EX_KU)) | MTD_HTTP_RESP( #R_ERROR_8_1_2_6_1) |
| 6 | S_LPAd → SM-DS | Close TLS session (unless SM-DS has already closed TLS session) |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| 8 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 9 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_SMDS_8_1_2_6_1_EX_CP)) | MTD_HTTP_RESP( #R_ERROR_8_1_2_6_1) |
| 10 | S_LPAd → SM-DS | Close TLS session (unless SM-DS has already closed TLS session) |  |
| 11 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| 12 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 13 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_SMDS_8_1_2_6_1_EX_BC_cA)) | MTD_HTTP_RESP( #R_ERROR_8_1_2_6_1) |
| 14 | S_LPAd → SM-DS | Close TLS session (unless SM-DS has already closed TLS session) |  |
| 15 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| 16 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 17 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_SMDS_8_1_2_6_1_EX_BC_PLC)) | MTD_HTTP_RESP( #R_ERROR_8_1_2_6_1) |

Test Sequence #10 Error: Expired EUM Certificate (Subject Code 8.1.2 Reason Code 6.3)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DS | #EVENT_ID_1 has been registered in the SM-DS with #EID1 and #TEST_DP_ADDRESS1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| IC2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_SMDS_8_1_2_6_3)) | MTD_HTTP_RESP( #R_ERROR_8_1_2_6_3) |

Test Sequence #11 Error: Invalid eUICC Certificate (Subject Code 8.1.3 Reason Code 6.1)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DS | #EVENT_ID_1 has been registered in the SM-DS with #EID1 and #TEST_DP_ADDRESS1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| IC2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_SMDS_8_1_3_6_1_SIG)) | MTD_HTTP_RESP( #R_ERROR_8_1_3_6_1) |
| 2 | S_LPAd → SM-DS | Close TLS session (unless SM-DS has already closed TLS session) |  |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| 4 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 5 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_SMDS_8_1_3_6_1_EX_KU)) | MTD_HTTP_RESP( #R_ERROR_8_1_3_6_1) |
| 6 | S_LPAd → SM-DS | Close TLS session (unless SM-DS has already closed TLS session) |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| 8 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 9 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_SMDS_8_1_3_6_1_EX_CP)) | MTD_HTTP_RESP( #R_ERROR_8_1_3_6_1) |
| 10 | S_LPAd → SM-DS | Close TLS session (unless SM-DS has already closed TLS session) |  |
| 11 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| 12 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 13 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_SMDS_8_1_3_6_1_SUB_ORG)) | MTD_HTTP_RESP( #R_ERROR_8_1_3_6_1) |
| 14 | S_LPAd → SM-DS | Close TLS session (unless SM-DS has already closed TLS session) |  |
| 15 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| 16 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 17 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_SMDS_8_1_3_6_1_SUB_SN)) | MTD_HTTP_RESP( #R_ERROR_8_1_3_6_1) |

Test Sequence #12 Error: Expired eUICC Certificate (Subject Code 8.1.3 Reason Code 6.3)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DS | #EVENT_ID_1 has been registered in the SM-DS with #EID1 and #TEST_DP_ADDRESS1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| IC2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_SMDS_8_1_3_6_3)) | MTD_HTTP_RESP( #R_ERROR_8_1_3_6_3) |

Test Sequence #13 Error: Invalid eUICC Signature (Subject Code 8.1 Reason Code 6.1)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DS | #EVENT_ID_1 has been registered in the SM-DS with #EID1 and #TEST_DP_ADDRESS1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| IC2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_SMDS_8_1_6_1_SIG)) | MTD_HTTP_RESP( #R_ERROR_8_1_6_1) |

Test Sequence #14 Error: Invalid Server Challenge (Subject Code 8.1 Reason Code 6.1)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DS | #EVENT_ID_1 has been registered in the SM-DS with #EID1 and #TEST_DP_ADDRESS1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| IC2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_SMDS_8_1_6_1_CHA)) | MTD_HTTP_RESP( #R_ERROR_8_1_6_1) |

Test Sequence #15 Error: Unknown Transaction ID in JSON transport layer (Subject Code 8.10.1 Reason Code 3.9)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DS | #EVENT_ID_1 has been registered in the SM-DS with #EID1 and #TEST_DP_ADDRESS1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| IC2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <INVALID_TRANSACTION_ID>, #AUTH_SERVER_RESP_MATCHING_ID_EMPTY)) | MTD_HTTP_RESP( #R_ERROR_8_10_1_3_9) |

Test Sequence #16 Error: Unknown Transaction ID in ASN.1 euiccSigned1 payload (Subject Code 8.10.1 Reason Code 3.9)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DS | #EVENT_ID_1 has been registered in the SM-DS with #EID1 and #TEST_DP_ADDRESS1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| IC2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_SMDS_8_10_1_3_9)) | MTD_HTTP_RESP( #R_ERROR_8_10_1_3_9) |

Test Sequence #17 Error: Matching ID containing EventID with no pending Event

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DS | No Events have been registered in the SM-DS for #EID1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| IC2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11, #S_LPA_RSP_CAPABILITY)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_MATCHING_ID_EVENT_ID)) | MTD_HTTP_RESP( #R_ERROR_8_9_5_3_9) |

##### 4.5.6.2.2	TC_SM-DS_ES11.AuthenticateClientBRP

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DS | SM-DS is configured with the #CERT_SM_DSauth_SIG for BrainpoolP256r1. |

Test Sequence #01 Nominal Matching ID Empty for one pending Event

This test sequence SHALL be the same as the Test Sequence #01 defined in section 4.5.6.2.1 TC_SM-DS_ES11.AuthenticateClientNIST except that all auth keys and certificates SHALL be based on BrainpoolP256r1.

Test Sequence #02 Nominal Matching ID Empty for two pending Events

This test sequence SHALL be the same as the Test Sequence #02 defined in section 4.5.6.2.1 TC_SM-DS_ES11.AuthenticateClientNIST except that all auth keys and certificates SHALL be based on BrainpoolP256r1.

Test Sequence #03 Nominal Matching ID Empty for no pending Events

This test sequence SHALL be the same as the Test Sequence #03 defined in section 4.5.6.2.1 TC_SM-DS_ES11.AuthenticateClientNIST except that all auth keys and certificates SHALL be based on BrainpoolP256r1.

Test Sequence #04 Nominal Matching ID Omitted for one pending Event

This test sequence SHALL be the same as the Test Sequence #04 defined in section 4.5.6.2.1 TC_SM-DS_ES11.AuthenticateClientNIST except that all auth keys and certificates SHALL be based on BrainpoolP256r1.

Test Sequence #05 Nominal Matching ID Omitted for two pending Events

This test sequence SHALL be the same as the Test Sequence #05 defined in section 4.5.6.2.1 TC_SM-DS_ES11.AuthenticateClientNIST except that all auth keys and certificates SHALL be based on BrainpoolP256r1.

Test Sequence #06 Nominal Matching ID Omitted for no pending Events

This test sequence SHALL be the same as the Test Sequence #06 defined in section 4.5.6.2.1 TC_SM-DS_ES11.AuthenticateClientNIST except that all auth keys and certificates SHALL be based on BrainpoolP256r1.

Test Sequence #07 Alt. Nominal Matching ID containing EventID with one pending Event

This test sequence SHALL be the same as the Test Sequence #07 defined in section 4.5.6.2.1 TC_SM-DS_ES11.AuthenticateClientNIST except that all auth keys and certificates SHALL be based on BrainpoolP256r1.

Test Sequence #08 Alt. Nominal Matching ID containing EventID with two pending Events

This test sequence SHALL be the same as the Test Sequence #08 defined in section 4.5.6.2.1 TC_SM-DS_ES11.AuthenticateClientNIST except that all auth keys and certificates SHALL be based on BrainpoolP256r1.

### 4.5.7	ES15 (SM-DS -- SM-DS): TLS, Mutual Authentication, Client, Session Establishment

#### 4.5.7.1	TC_ALT_SM-DS_ES15_Client_Mutual_Authentication_for_HTTPS_EstablishmentNIST

Perform all test sequences defined in section 4.6.1.2.1 with the following variables set as follows:

- 	CLIENT = Alternative SM-DS under test

- o	CERT_CLIENT_TLS = #CERT_SM_DS_TLS for NIST

- 	SERVER = Root S_SM-DS

- o	CERT_S_SERVER_TLS = #CERT_S_SM_DS_TLS for NIST

#### 4.5.7.2	TC_ALT_SM-DS_ES15_Client_Mutual_Authentication_for_HTTPS_EstablishmentBRP

Perform all test sequences defined in section 4.6.1.2.2 with the following variables set as follows:

- 	CLIENT = Alternative SM-DS under test

- o	CERT_CLIENT_TLS = #CERT_SM_DS_TLS for BRP

- 	SERVER = Root S_SM-DS

- o	CERT_S_SERVER_TLS = #CERT_S_SM_DS_TLS for BRP

### 4.5.8	ES12 (SM-DS -- SM-DP+): TLS, Mutual Authentication, Server, Session Establishment

#### 4.5.8.1	TC_SM-DS_ES12_Server_Mutual_Authentication_for_HTTPS_EstablishmentNIST

Perform all test sequences defined in section 4.6.2.2.1 with the following variables set as follows:

- 	CLIENT = S_SM-DP+

- o	CERT_S_CLIENT_TLS = CERT_S_SM_DP_TLS for NIST

- 	SERVER = Alternative or Root SM-DS under test.

- o	CERT_SERVER_TLS = CERT_SM_DS_TLS for NIST

#### 4.5.8.2	TC_SM-DS_ES12_Server_Mutual_Authentication_for_HTTPS_EstablishmentBRP

Perform all test sequences defined in section 4.6.2.2.2 with the following variables set as follows:

- 	CLIENT = S_SM-DP+

- o	CERT_S_CLIENT_TLS = CERT_S_SM_DP_TLS for BRP

- 	SERVER = Alternative or Root SM-DS under test.

- o	CERT_SERVER_TLS = CERT_SM_DS_TLS for BRP

### 4.5.9	ES15 (SM-DS -- SM-DS): TLS, Mutual Authentication, Server, Session Establishment

#### 4.5.9.1	TC_ROOT_SM-DS_ES15_Server_Mutual_Authentication_for_HTTPS_EstablishmentNIST

Perform all test sequences defined in section 4.6.2.2.1 with the following variables set as follows:

- 	CLIENT = Alternative S_SM-DS

- o	CERT_S_CLIENT_TLS = CERT_S_SM_DS_TLS for NIST

- 	SERVER = Root SM-DS under test.

- o	CERT_SERVER_TLS = CERT_SM_DS_TLS for NIST

#### 4.5.9.2	TC_ROOT_SM-DS_ES15_Server_Mutual_Authentication_for_HTTPS_EstablishmentBRP

Perform all test sequences defined in section 4.6.2.2.2 with the following variables set as follows:

- 	CLIENT = Alternative S_SM-DS

- o	CERT_S_CLIENT_TLS = CERT_S_SM_DS_TLS for BRP

- 	SERVER = Root SM-DS under test.

- o	CERT_SERVER_TLS = CERT_SM_DS_TLS for BRP

### 4.5.10	ES11 (LPA -- SM-DS): TLS, Server Authentication, Session Establishment

#### 4.5.10.1	TC_SM-DS_ES11_Server_Authentication_for_HTTPS_EstablishmentNIST

Perform all test sequences defined in section 4.6.3.2.1 with the following variables set as follows:

- 	CLIENT = S_LPAd

- 	SERVER = SM-DS under test.

- o	CERT_SERVER_TLS = #CERT_SM_DS_TLS for NIST

#### 4.5.10.2	TC_SM-DS_ES11_Server_Authentication_for_HTTPS_EstablishmentBRP

Perform all test sequences defined in section 4.6.3.2.2 with the following variables set as follows:

- 	CLIENT = S_LPAd

- 	SERVER = SM-DS under test.

- o	CERT_SERVER_TLS = #CERT_SM_DS_TLS for BRP

## 4.6	TLS Interface

### 4.6.1	TLS, Mutual Authentication, Client, TLS Establishment

#### 4.6.1.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Requirements

- 	RQ26_023, RQ26_024, RQ26_025, RQ26_025_1, RQ26_026, RQ26_027, RQ26_028

- 	RQ31_032

- 	RQ45_006, RQ45_026, RQ45_026_1

- 	RQ56_001, RQ56_002, RQ56_003

- 	RQ58_001, RQ58_002

- 	RQ59_001

- RQ60_002, RQ60_003

- 	RQ61_001

- 	RQ63_006

- 	RQ510_001

#### 4.6.1.2	Test Cases

##### 4.6.1.2.1	TC_Client_Mutual_Authentication_for_HTTPS_EstablishmentNIST

| General Initial Conditions for SM-DP+ as Client under test |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | The SM-DP+ is ready to execute a download order procedure (see SGP.22 [2] section 3.1.1) for the SM-DS use case with smdsAddress #TEST_ROOT_DS_ADDRESS to be used for Event Registration (for example, a suitable profile is available).There is currently no TLS connection established to the S_SM-DS. |

| General Initial Conditions for SM-DS as Client under test |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DS | EventID to be used by the S_SM-DP+ is not already used in the SM-DS. There is currently no TLS connection established to the S_SM-DS. |

Test Sequence #01 Nominal: HTTPS Session Establishment

The purpose of this test is to verify that the Client correctly establishes an HTTPS Session with the Server using Mutual Authentication.

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | When the Client under test is the SM-DP+, initiate the download order procedure (see SGP.22 [2] section 3.1.1) for the SM-DS use case with smdsAddress #TEST_ROOT_DS_ADDRESS to be used for Event Registration. When the Client under test is the SM-DS, the S_SM-DP+ calls ES12.RegisterEvent configured as follows: MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( #EID1, #TEST_DP_ADDRESS1, <EVENT_ID>, TRUE) |  |  |
| 1 | CLIENT → S_SERVER | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_CLIENT_TLS_VER, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) Verify that: • <TLS_CIPHER_SUITES> SHALL contain at least one of TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 or TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 (see note 1) • <EXT_SHA256_ECDSA> SHALL have at least the 'supported_signature_algorithms' extension set with HashAlgorithm sha256 (04) and SignatureAlgorithm ecdsa (03). |
| 2 | S_SERVER → CLIENT | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <S_SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_S_SERVER_TLS, <SERVER_TLS_EPHEM_KEY>, #CLIENT_CERT_TYPE, #S_SAH_SHA256_ECDSA, #DIST_NAME_CI) | MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH( #CERT_CLIENT_TLS, <CLIENT_TLS_EPHEM_KEY>) |
| 3 | S_SERVER → CLIENT | MTD_TLS_SERVER_END( #CHANGE_CIPHER_SPEC, <SERVER_FINISHED>) | HTTPS connection established |
| Note 1: if the verification fails, the test tool cannot continue execution while remaining compliant with both SGP.22 [2] and RFC 5246 [27]. |  |  |  |

Test Sequence #02 Nominal: Non-reuse of session keys

The purpose of this test sequence is to verify that the Client is not reusing ephemeral keys from the previous session.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | The SM-DP+ is ready to execute a further download order procedure (see SGP.22 [2] section 3.1.1) for the SM-DS use case with smdsAddress #TEST_ROOT_DS_ADDRESS to be used for Event Registration (for example, a further suitable profile is available). |

| Step | Direction | Sequence / Description |  | Expected result |
| --- | --- | --- | --- | --- |
| IC1 | When the Client under test is the SM-DP+, initiate the download order procedure (see SGP.22 [2] section 3.1.1) for the SM-DS use case with smdsAddress #TEST_ROOT_DS_ADDRESS to be used for Event Registration. When the Client under test is the SM-DS, the S_SM-DP+ calls ES12.RegisterEvent configured as follows: MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( #EID1, #TEST_DP_ADDRESS1, <EVENT_ID>, TRUE) |  |  |  |
| IC2 | CLIENT → S_SERVER |  | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_CLIENT_TLS_VER, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) Extract <SESSION_ID_CLIENT>. |
| IC3 | S_SERVER → CLIENT |  | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <S_SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_S_SERVER_TLS, <SERVER_TLS_EPHEM_KEY>, #CLIENT_CERT_TYPE, #S_SAH_SHA256_ECDSA, #DIST_NAME_CI) | MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH( #CERT_CLIENT_TLS, <CLIENT_TLS_EPHEM_KEY>) Extract <SESSION_ID_RANDOM>. Extract <CLIENT_TLS_EPHEM_KEY> from the ClientKeyExchange message. |
| IC4 | S_SERVER → CLIENT |  | MTD_TLS_SERVER_END( #CHANGE_CIPHER_SPEC, <SERVER_FINISHED>) | HTTPS connection established |
| IC5 | S_SERVER → CLIENT | Close TLS session (unless CLIENT has already closed TLS session) |  |  |
| IC6 | When the Client under test is the SM-DP+, initiate a further download order procedure (see SGP.22 [2] section 3.1.1) for the SM-DS use case with smdsAddress #TEST_ROOT_DS_ADDRESS to be used for Event Registration. When the Client under test is the SM-DS, repeat IC1 |  |  |  |
| 1 | CLIENT → S_SERVER | Send TLS Client Hello |  | MTD_TLS_CLIENT_HELLO( #IUT_CLIENT_TLS_VER, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) <SESSION_ID_CLIENT> SHALL be different from any non-empty value of session_id (in ClientHello or ServerHello) used in IC2. Verify that: • <TLS_CIPHER_SUITES> SHALL contain at least one of TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 or TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 (see Note 1) |
| 2 | S_SERVER → CLIENT | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <S_SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_S_SERVER_TLS, <SERVER_TLS_EPHEM_KEY>, #CLIENT_CERT_TYPE, #S_SAH_SHA256_ECDSA, #DIST_NAME_CI) |  | MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH( #CERT_CLIENT_TLS, <CLIENT_TLS_EPHEM_KEY>) Verify that in the ClientKeyExchange message: • <CLIENT_TLS_EPHEM_KEY> is different from the one used by the CLIENT in IC1 |
| 3 | S_SERVER → CLIENT | MTD_TLS_SERVER_END( #CHANGE_CIPHER_SPEC, <SERVER_FINISHED>) |  | HTTPS connection established |
| Note 1: if the verification fails, the test tool cannot continue execution while remaining compliant with both SGP.22 [2] and RFC 5246 [27]. |  |  |  |  |

Test Sequence #03 Error: Invalid Server TLS Version

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | When the Client under test is the SM-DP+, initiate the download order procedure (see SGP.22 [2] section 3.1.1) for the SM-DS use case with smdsAddress #TEST_ROOT_DS_ADDRESS to be used for Event Registration. When the Client under test is the SM-DS, the S_SM-DP+ calls ES12.RegisterEvent configured as follows: MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( #EID1, #TEST_DP_ADDRESS1, <EVENT_ID>, TRUE) |  |  |
| 1 | CLIENT → S_SERVER | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_CLIENT_TLS_VER, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) Verify that: • <TLS_CIPHER_SUITES> SHALL contain at least one of TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 or TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 (see note 1) |
| 2 | S_SERVER → CLIENT | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_1, <S_SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_S_SERVER_TLS, <SERVER_TLS_EPHEM_KEY>, #CLIENT_CERT_TYPE, #S_SAH_SHA256_ECDSA, #DIST_NAME_CI) Note: if the Client sends an Alert during or after any of the messages sent by the S_SERVER in MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC, then the S_SERVER might not send the messages specified in MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC which occur after the Alert. | Client sends a TLS Fatal-alert during or after any of the messages sent by the S_SERVER in MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC |
| Note 1: if the verification fails, the test tool cannot continue execution while remaining compliant with both SGP.22 [2] and RFC 5246 [27]. |  |  |  |

Test Sequence #04 Error: Invalid Server TLS Certificate Signature

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | When the Client under test is the SM-DP+, initiate the download order procedure (see SGP.22 [2] section 3.1.1) for the SM-DS use case with smdsAddress #TEST_ROOT_DS_ADDRESS to be used for Event Registration. When the Client under test is the SM-DS, the S_SM-DP+ calls ES12.RegisterEvent configured as follows: MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( #EID1, #TEST_DP_ADDRESS1, <EVENT_ID>, TRUE) |  |  |
| 1 | CLIENT → S_SERVER | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_CLIENT_TLS_VER, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) Verify that: • <TLS_CIPHER_SUITES> SHALL contain at least one of TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 or TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 (see note 1) |
| 2 | S_SERVER → CLIENT | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <S_SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_S_SERVER_TLS_INV_SIG, <SERVER_TLS_EPHEM_KEY>, #CLIENT_CERT_TYPE, #S_SAH_SHA256_ECDSA, #DIST_NAME_CI) Note: if the Client sends an Alert during or after any of the messages sent by the S_SERVER in MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC, then the S_SERVER might not send the messages specified in MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC which occur after the Alert. | Client sends a TLS Fatal-alert during or after any of the messages sent by the S_SERVER in MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC |
| Note 1: if the verification fails, the test tool cannot continue execution while remaining compliant with both SGP.22 [2] and RFC 5246 [27]. |  |  |  |

Test Sequence #05 Error: Expired Server TLS Certificate

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | When the Client under test is the SM-DP+, initiate the download order procedure (see SGP.22 [2] section 3.1.1) for the SM-DS use case with smdsAddress #TEST_ROOT_DS_ADDRESS to be used for Event Registration. When the Client under test is the SM-DS, the S_SM-DP+ calls ES12.RegisterEvent configured as follows: MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( #EID1, #TEST_DP_ADDRESS1, <EVENT_ID>, TRUE) |  |  |
| 1 | CLIENT → S_SERVER | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_CLIENT_TLS_VER, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) Verify that: • <TLS_CIPHER_SUITES> SHALL contain at least one of TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 or TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 (see note 1) |
| 2 | S_SERVER → CLIENT | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <S_SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_S_SERVER_TLS_EXPIRED, <SERVER_TLS_EPHEM_KEY>, #CLIENT_CERT_TYPE, #S_SAH_SHA256_ECDSA, #DIST_NAME_CI) Note: if the Client sends an Alert during or after any of the messages sent by the S_SERVER in MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC, then the S_SERVER might not send the messages specified in MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC which occur after the Alert. | Client sends a TLS Fatal-alert during or after any of the messages sent by the S_SERVER in MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC |
| Note 1: if the verification fails, the test tool cannot continue execution while remaining compliant with both SGP.22 [2] and RFC 5246 [27]. |  |  |  |

Test Sequence #06 Error: Invalid Server TLS Certificate with critical extension not set

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | When the Client under test is the SM-DP+, initiate the download order procedure (see SGP.22 [2] section 3.1.1) for the SM-DS use case with smdsAddress #TEST_ROOT_DS_ADDRESS to be used for Event Registration. When the Client under test is the SM-DS, the S_SM-DP+ calls ES12.RegisterEvent configured as follows: MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( #EID1, #TEST_DP_ADDRESS1, <EVENT_ID>, TRUE) |  |  |
| 1 | CLIENT → S_SERVER | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_CLIENT_TLS_VER, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) Verify that: • <TLS_CIPHER_SUITES> SHALL contain at least one of TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 or TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 (see note 1) |
| 2 | S_SERVER → CLIENT | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <S_SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_S_SERVER_TLS_INV_CRITICAL_EXT, <SERVER_TLS_EPHEM_KEY>, #CLIENT_CERT_TYPE, #S_SAH_SHA256_ECDSA, #DIST_NAME_CI) Note: if the Client sends an Alert during or after any of the messages sent by the S_SERVER in MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC, then the S_SERVER might not send the messages specified in MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC which occur after the Alert. | Client sends a TLS Fatal-alert during or after any of the messages sent by the S_SERVER in MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC |
| Note 1: if the verification fails, the test tool cannot continue execution while remaining compliant with both SGP.22 [2] and RFC 5246 [27]. |  |  |  |

Test Sequence #07 Error: Invalid Server TLS Certificate with invalid 'key usage' extension

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | When the Client under test is the SM-DP+, initiate the download order procedure (see SGP.22 [2] section 3.1.1) for the SM-DS use case with smdsAddress #TEST_ROOT_DS_ADDRESS to be used for Event Registration. When the Client under test is the SM-DS, the S_SM-DP+ calls ES12.RegisterEvent configured as follows: MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( #EID1, #TEST_DP_ADDRESS1, <EVENT_ID>, TRUE) |  |  |
| 1 | CLIENT → S_SERVER | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_CLIENT_TLS_VER, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) Verify that: • <TLS_CIPHER_SUITES> SHALL contain at least one of TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 or TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 (see note 1) |
| 2 | S_SERVER → CLIENT | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <S_SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_S_SERVER_TLS_INV_KEY_USAGE, <SERVER_TLS_EPHEM_KEY>, #CLIENT_CERT_TYPE, #S_SAH_SHA256_ECDSA, #DIST_NAME_CI) Note: if the Client sends an Alert during or after any of the messages sent by the S_SERVER in MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC, then the S_SERVER might not send the messages specified in MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC which occur after the Alert. | Client sends a TLS Fatal-alert during or after any of the messages sent by the S_SERVER in MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC |
| Note 1: if the verification fails, the test tool cannot continue execution while remaining compliant with both SGP.22 [2] and RFC 5246 [27]. |  |  |  |

Test Sequence #08 Error: Invalid TLS Certificate with invalid 'extended key usage' extension

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | When the Client under test is the SM-DP+, initiate the download order procedure (see SGP.22 [2] section 3.1.1) for the SM-DS use case with smdsAddress #TEST_ROOT_DS_ADDRESS to be used for Event Registration. When the Client under test is the SM-DS, the S_SM-DP+ calls ES12.RegisterEvent configured as follows: MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( #EID1, #TEST_DP_ADDRESS1, <EVENT_ID>, TRUE) |  |  |
| 1 | CLIENT → S_SERVER | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_CLIENT_TLS_VER, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) Verify that: • <TLS_CIPHER_SUITES> SHALL contain at least one of TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 or TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 (see note 1) |
| 2 | S_SERVER → CLIENT | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <S_SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_S_SERVER_TLS_INV_EXT_KEY_USAGE, <SERVER_TLS_EPHEM_KEY>, #CLIENT_CERT_TYPE, #S_SAH_SHA256_ECDSA, #DIST_NAME_CI) Note: if the Client sends an Alert during or after any of the messages sent by the S_SERVER in MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC, then the S_SERVER might not send the messages specified in MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC which occur after the Alert. | Client sends a TLS Fatal-alert during or after any of the messages sent by the S_SERVER in MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC |
| Note 1: if the verification fails, the test tool cannot continue execution while remaining compliant with both SGP.22 [2] and RFC 5246 [27]. |  |  |  |

Test Sequence #09 Error: Invalid Client TLS Certificate with invalid 'Certificate Policies' extensions

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | When the Client under test is the SM-DP+, initiate the download order procedure (see SGP.22 [2] section 3.1.1) for the SM-DS use case with smdsAddress #TEST_ROOT_DS_ADDRESS to be used for Event Registration. When the Client under test is the SM-DS, the S_SM-DP+ calls ES12.RegisterEvent configured as follows: MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES12, #PATH_REGISTER_EVENT, MTD_REGISTER_EVENT( #EID1, #TEST_DP_ADDRESS1, <EVENT_ID>, TRUE) |  |  |
| 1 | CLIENT → S_SERVER | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_CLIENT_TLS_VER, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) Verify that: • <TLS_CIPHER_SUITES> SHALL contain at least one of TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 or TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 (see note 1) |
| 2 | S_SERVER → CLIENT | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <S_SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_S_SERVER_TLS_INV_CERT_POL, <SERVER_TLS_EPHEM_KEY>, #CLIENT_CERT_TYPE, #S_SAH_SHA256_ECDSA, #DIST_NAME_CI) Note: if the Client sends an Alert during or after any of the messages sent by the S_SERVER in MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC, then the S_SERVER might not send the messages specified in MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC which occur after the Alert. | Client sends a TLS Fatal-alert during or after any of the messages sent by the S_SERVER in MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC |
| Note 1: if the verification fails, the test tool cannot continue execution while remaining compliant with both SGP.22 [2] and RFC 5246 [27]. |  |  |  |

##### 4.6.1.2.2	TC_Client_Mutual_Authentication_for_HTTPS_EstablishmentBRP

Test Sequence #01 Nominal: HTTPS Session Establishment

This test sequence SHALL be the same as the Test Sequence #01 defined in section 4.6.1.2.1 TC_Client_Mutual_Authentication_for_HTTPS_EstablishmentNIST, except that the brainpoolP256r1 curve is used.

Test Sequence #02 Nominal: Non-reuse of session keys

This test sequence SHALL be the same as the Test Sequence #02 defined in section 4.6.1.2.1 TC_Client_Mutual_Authentication_for_HTTPS_EstablishmentNIST, except that the brainpoolP256r1 curve is used.

### 4.6.2	TLS, Mutual Authentication, Server, TLS Establishment

#### 4.6.2.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Requirements

- 	RQ26_023, RQ26_024, RQ26_025, RQ26_026, RQ26_027, RQ26_028

- 	RQ45_006, RQ45_026, RQ45_026_1

- 	RQ56_002

- 	RQ59_001

- 	RQ60_003

- 	RQ61_001

#### 4.6.2.2	Test Cases

##### 4.6.2.2.1	TC_Server_Mutual_Authentication_for_HTTPS_EstablishmentNIST

Test Sequence #01 Nominal: HTTPS Session Establishment

The purpose of this test is to verify that the Server correctly establishes an HTTPS Session with the Client using Mutual Authentication.

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_CLIENT → SERVER | MTD_TLS_CLIENT_HELLO( #TLS_VERSION_1_2, #MIN_TLS_1_2_CIPHER_SUITES, #S_SESSION_ID_EMPTY, #S_EXT_SHA256_ECDSA) | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_SERVER_TLS, <SERVER_TLS_EPHEM_KEY>, #CLIENT_CERT_TYPE, <SAH_SHA256_ECDSA>, #DIST_NAME_CI) Verify that in the Server Hello message: •<SEL_TLS_CIPHER_SUITE> SHALL contain only TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 |
| 2 | S_CLIENT → SERVER | MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH( #CERT_S_CLIENT_TLS, <CLIENT_TLS_EPHEM_KEY>) | MTD_TLS_SERVER_END( #CHANGE_CIPHER_SPEC, <SERVER_FINISHED>) |

Test Sequence #02 Nominal: Non-reuse of session keys

The purpose of this test sequence is to verify that the Server is not reusing ephemeral keys from the previous session.

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_MUTUAL_AUTH Extract <SERVER_TLS_EPHEM_KEY> from the ServerKeyExchange message |  |  |
| IC2 | Terminate the TLS session |  |  |
| 1 | S_CLIENT → SERVER | MTD_TLS_CLIENT_HELLO( #TLS_VERSION_1_2, #MIN_TLS_1_2_CIPHER_SUITES, #S_SESSION_ID_EMPTY, #S_EXT_SHA256_ECDSA) | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_SERVER_TLS, <SERVER_TLS_EPHEM_KEY>, #CLIENT_CERT_TYPE, <SAH_SHA256_ECDSA>, #DIST_NAME_CI) Verify that in the ServerKeyExchange message: •<SERVER_TLS_EPHEM_KEY> is different from the <SERVER_TLS_EPHEM_KEY> value used in IC1. |
| 2 | S_CLIENT → SERVER | MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH( #CERT_S_CLIENT_TLS, <CLIENT_TLS_EPHEM_KEY>) | MTD_TLS_SERVER_END( #CHANGE_CIPHER_SPEC, <SERVER_FINISHED>) |

Test Sequence #03 Nominal: HTTPS Session Establishment with supported and unsupported Cipher Suites

The purpose of this test is to verify that the Server correctly establishes an HTTPS Session with the Client when supported and unsupported Cipher Suites are offered by the Client.

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_CLIENT → SERVER | MTD_TLS_CLIENT_HELLO( #TLS_VERSION_1_2, #PROP_TLS_1_2_CIPHER_SUITES, #S_SESSION_ID_EMPTY, #S_EXT_SHA256_ECDSA) | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_SERVER_TLS, <SERVER_TLS_EPHEM_KEY>, #CLIENT_CERT_TYPE, <SAH_SHA256_ECDSA>, #DIST_NAME_CI) Verify that in the Server Hello message: •<SEL_TLS_CIPHER_SUITE> SHALL contain either TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 OR TLS_ECDHE_ECDSA_WITH_AES_256_GCM_SHA384 |
| 2 | S_CLIENT → SERVER | MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH( #CERT_S_CLIENT_TLS, <CLIENT_TLS_EPHEM_KEY>) | MTD_TLS_SERVER_END( #CHANGE_CIPHER_SPEC, <SERVER_FINISHED>) |

Test Sequence #04 Error: Invalid TLS Version

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_CLIENT → SERVER | MTD_TLS_CLIENT_HELLO( #TLS_VERSION_1_1, #MIN_TLS_1_2_CIPHER_SUITES, #S_SESSION_ID_EMPTY, NO_PARAM) | Server sends a TLS Fatal-alert |

Test Sequence #05 Error: Unsupported Cipher Suites and Extensions

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_CLIENT → SERVER | MTD_TLS_CLIENT_HELLO( #TLS_VERSION_1_2, #UNSUP_TLS_CIPHER_SUITES, #S_SESSION_ID_EMPTY, #EXT_SHA256_RSA) | Server sends a TLS Fatal-alert |

Test Sequence #06 Error: Invalid Client TLS Certificate Signature

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_CLIENT → SERVER | MTD_TLS_CLIENT_HELLO( #TLS_VERSION_1_2, #MIN_TLS_1_2_CIPHER_SUITES, #S_SESSION_ID_EMPTY, #S_EXT_SHA256_ECDSA) | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_SERVER_TLS, <SERVER_TLS_EPHEM_KEY>, #CLIENT_CERT_TYPE, <SAH_SHA256_ECDSA>, #DIST_NAME_CI) Verify that in the Server Hello message: •<SEL_TLS_CIPHER_SUITE> SHALL contain only TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 |
| 2 | S_CLIENT → SERVER | MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH( #CERT_S_CLIENT_TLS_INV_SIG, <CLIENT_TLS_EPHEM_KEY>) Note: if the Server sends an Alert during or after any of the messages sent by the S_CLIENT in MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH, then the S_CLIENT might not send the messages specified in MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH which occur after the Alert. | Server sends a TLS Fatal-alert during or after any of the messages sent by the S_CLIENT in MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH |

Test Sequence #07 Error: Expired Client TLS Certificate

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_CLIENT → SERVER | MTD_TLS_CLIENT_HELLO( #TLS_VERSION_1_2, #MIN_TLS_1_2_CIPHER_SUITES, #S_SESSION_ID_EMPTY, #S_EXT_SHA256_ECDSA) | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_SERVER_TLS, <SERVER_TLS_EPHEM_KEY>, #CLIENT_CERT_TYPE, <SAH_SHA256_ECDSA>, #DIST_NAME_CI) Verify that in the Server Hello message: •<SEL_TLS_CIPHER_SUITE> SHALL contain only TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 |
| 2 | S_CLIENT → SERVER | MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH( #CERT_S_CLIENT_TLS_EXPIRED, <CLIENT_TLS_EPHEM_KEY>) Note: if the Server sends an Alert during or after any of the messages sent by the S_CLIENT in MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH, then the S_CLIENT might not send the messages specified in MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH which occur after the Alert. | Server sends a TLS Fatal-alert during or after any of the messages sent by the S_CLIENT in MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH |

Test Sequence #08 Error: Invalid Client TLS Certificate with critical extension not set

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_CLIENT → SERVER | MTD_TLS_CLIENT_HELLO( #TLS_VERSION_1_2, #MIN_TLS_1_2_CIPHER_SUITES, #S_SESSION_ID_EMPTY, #S_EXT_SHA256_ECDSA) | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_SERVER_TLS, <SERVER_TLS_EPHEM_KEY>, #CLIENT_CERT_TYPE, <SAH_SHA256_ECDSA>, #DIST_NAME_CI) Verify that in the Server Hello message: •<SEL_TLS_CIPHER_SUITE> SHALL contain only TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 |
| 2 | S_CLIENT → SERVER | MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH( #CERT_S_CLIENT_TLS_INV_CRITICAL_EXT, <CLIENT_TLS_EPHEM_KEY>) Note: if the Server sends an Alert during or after any of the messages sent by the S_CLIENT in MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH, then the S_CLIENT might not send the messages specified in MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH which occur after the Alert. | Server sends a TLS Fatal-alert during or after any of the messages sent by the S_CLIENT in MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH |

Test Sequence #09 Error: Invalid Client TLS Certificate with invalid 'key usage' extension

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_CLIENT → SERVER | MTD_TLS_CLIENT_HELLO( #TLS_VERSION_1_2, #MIN_TLS_1_2_CIPHER_SUITES, #S_SESSION_ID_EMPTY, #S_EXT_SHA256_ECDSA) | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_SERVER_TLS, <SERVER_TLS_EPHEM_KEY>, #CLIENT_CERT_TYPE, <SAH_SHA256_ECDSA>, #DIST_NAME_CI) Verify that in the Server Hello message: •<SEL_TLS_CIPHER_SUITE> SHALL contain only TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 |
| 2 | S_CLIENT → SERVER | MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH( #CERT_S_CLIENT_TLS_INV_KEY_USAGE, <CLIENT_TLS_EPHEM_KEY>) Note: if the Server sends an Alert during or after any of the messages sent by the S_CLIENT in MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH, then the S_CLIENT might not send the messages specified in MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH which occur after the Alert. | Server sends a TLS Fatal-alert during or after any of the messages sent by the S_CLIENT in MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH |

Test Sequence #10 Error: Invalid TLS Certificate with invalid 'extended key usage' extension

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_CLIENT → SERVER | MTD_TLS_CLIENT_HELLO( #TLS_VERSION_1_2, #MIN_TLS_1_2_CIPHER_SUITES, #S_SESSION_ID_EMPTY, #S_EXT_SHA256_ECDSA) | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_SERVER_TLS, <SERVER_TLS_EPHEM_KEY>, #CLIENT_CERT_TYPE, <SAH_SHA256_ECDSA>, #DIST_NAME_CI) Verify that in the Server Hello message: •<SEL_TLS_CIPHER_SUITE> SHALL contain only TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 |
| 2 | S_CLIENT → SERVER | MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH( #CERT_S_CLIENT_TLS_INV_EXT_KEY_USAGE, <CLIENT_TLS_EPHEM_KEY>) Note: if the Server sends an Alert during or after any of the messages sent by the S_CLIENT in MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH, then the S_CLIENT might not send the messages specified in MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH which occur after the Alert. | Server sends a TLS Fatal-alert during or after any of the messages sent by the S_CLIENT in MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH |

Test Sequence #11 Error: Invalid Client TLS Certificate with invalid 'Certificate Policies' extensions

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_CLIENT → SERVER | MTD_TLS_CLIENT_HELLO( #TLS_VERSION_1_2, #MIN_TLS_1_2_CIPHER_SUITES, #S_SESSION_ID_EMPTY, #S_EXT_SHA256_ECDSA) | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_SERVER_TLS, <SERVER_TLS_EPHEM_KEY>, #CLIENT_CERT_TYPE, <SAH_SHA256_ECDSA>, #DIST_NAME_CI) Verify that in the Server Hello message: •<SEL_TLS_CIPHER_SUITE> SHALL contain only TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 |
| 2 | S_CLIENT → SERVER | MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH( #CERT_S_CLIENT_TLS_INV_CERT_POL, <CLIENT_TLS_EPHEM_KEY>) Note: if the Server sends an Alert during or after any of the messages sent by the S_CLIENT in MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH, then the S_CLIENT might not send the messages specified in MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH which occur after the Alert. | Server sends a TLS Fatal-alert during or after any of the messages sent by the S_CLIENT in MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH |

Test Sequence #12 Error: No suitable Client certificate available

The purpose of this test is to verify that the Server does not establish an HTTPS Session with the Client using Mutual Authentication when the CERT.CLIENT.TLS certificate of the S_CLIENT certificate message contains no certificates (the certificate_list structure has a length of zero).

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_CLIENT → SERVER | MTD_TLS_CLIENT_HELLO( #TLS_VERSION_1_2, #MIN_TLS_1_2_CIPHER_SUITES, #S_SESSION_ID_EMPTY, #S_EXT_SHA256_ECDSA) | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_SERVER_TLS, <SERVER_TLS_EPHEM_KEY>, #CLIENT_CERT_TYPE, <SAH_SHA256_ECDSA>, #DIST_NAME_CI) Verify that in the Server Hello message: •<SEL_TLS_CIPHER_SUITE> SHALL contain only TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 |
| 2 | S_CLIENT → SERVER | MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH( NO_PARAM, <CLIENT_TLS_EPHEM_KEY>) Note: if the Server sends an Alert during or after any of the messages sent by the S_CLIENT in MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH, then the S_CLIENT might not send the messages specified in MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH which occur after the Alert. | Server sends a TLS Fatal-alert during or after any of the messages sent by the S_CLIENT in MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH |

##### 4.6.2.2.2	TC_Server_Mutual_Authentication_for_HTTPS_EstablishmentBRP

Test Sequence #01 Nominal: HTTPS Session Establishment

This test sequence SHALL be the same as the Test Sequence #01 defined in section 4.6.2.2.1 TC_Server_Mutual_Authentication_for_HTTPS_EstablishmentNIST, except that the brainpoolP256r1 curve is used.

Test Sequence #02 Nominal: Non-reuse of session keys

This test sequence SHALL be the same as the Test Sequence #02 defined in section 4.6.2.2.1 TC_Server_Mutual_Authentication_for_HTTPS_EstablishmentNIST, except that the brainpoolP256r1 curve is used.

Test Sequence #03 Nominal: HTTPS Session Establishment with supported and unsupported Cipher Suites

This test sequence SHALL be the same as the Test Sequence #03 defined in section 4.6.2.2.1 TC_Server_Mutual_Authentication_for_HTTPS_EstablishmentNIST, except that the brainpoolP256r1 curve is used.

### 4.6.3	TLS, Server Authentication, TLS Establishment

#### 4.6.3.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Requirements

- 	RQ26_023, RQ26_024, RQ26_025, RQ26_025_1, RQ26_026, RQ26_027, RQ26_028

- 	RQ31_032

- 	RQ45_026, RQ45_026_1

- 	RQ56_001, RQ56_002, RQ56_003

- 	RQ58_001, RQ58_002

- 	RQ60_002

- 	RQ61_001

- 	RQ63_006

#### 4.6.3.2	Test Cases

##### 4.6.3.2.1	TC_Server_Authentication_for_HTTPS_EstablishmentNIST

Test Sequence #01 Nominal: HTTPS Session Establishment

The purpose of this test is to verify that the Server correctly establishes an HTTPS Session with the Client.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | The server needs to have an address starting with “rsp3-“ and S_LPAd shall contact the server using this address. The S_LPAd shall contact the server using this address. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_LPAd → SERVER | MTD_TLS_CLIENT_HELLO( #TLS_VERSION_1_2, #MIN_TLS_1_2_CIPHER_SUITES, #S_SESSION_ID_EMPTY, #S_EXT_SHA256_ECDSA, #SERVER_ADDRESS_V3) | MTD_TLS_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_SERVER_TLS, <SERVER_TLS_EPHEM_KEY>) Verify that in the Server Hello message: •<SEL_TLS_CIPHER_SUITE> SHALL contain only TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 |
| 2 | S_LPAd → SERVER | MTD_TLS_CLIENT_KEY_EXCH_ETC(<CLIENT_TLS_EPHEM_KEY>) | MTD_TLS_SERVER_END( #CHANGE_CIPHER_SPEC, <SERVER_FINISHED>) |

Test Sequence #02 Nominal: Non-reuse of session keys

The purpose of this test sequence is to verify that the Server is not reusing ephemeral keys from the previous session.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | The server needs to have an address starting with “rsp3-“ and S_LPAd shall contact the server using this address. The S_LPAd shall contact the server using this address. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH Extract <SERVER_TLS_EPHEM_KEY> from the ServerKeyExchange message |  |  |
| IC2 | Terminate the TLS session |  |  |
| 1 | S_LPAd → SERVER | MTD_TLS_CLIENT_HELLO( #TLS_VERSION_1_2, #MIN_TLS_CIPHER_SUITES, #S_SESSION_ID_EMPTY, #S_EXT_SHA256_ECDSA, #SERVER_ADDRESS_V3) | MTD_TLS_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_SERVER_TLS, <SERVER_TLS_EPHEM_KEY>) Verify that in the ServerKeyExchange message: •<SERVER_TLS_EPHEM_KEY> is different from the <SERVER_TLS_EPHEM_KEY> value used in IC1. |
| 2 | S_LPAd → SERVER | MTD_TLS_CLIENT_KEY_EXCH_ETC(<CLIENT_TLS_EPHEM_KEY>) | MTD_TLS_SERVER_END( #CHANGE_CIPHER_SPEC, <SERVER_FINISHED>) |

Test Sequence #03 Nominal: HTTPS Session Establishment with supported and unsupported Cipher Suites

The purpose of this test is to verify that the Server correctly establishes an HTTPS Session with the Client when supported and unsupported Cipher Suites are offered by the Client.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | The server needs to have an address starting with “rsp3-“ and S_LPAd shall contact the server using this address. The S_LPAd shall contact the server using this address. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_LPAd → SERVER | MTD_TLS_CLIENT_HELLO( #TLS_VERSION_1_2, #PROP_TLS_1_2_CIPHER_SUITES, #S_SESSION_ID_EMPTY, #S_EXT_SHA256_ECDSA, #SERVER_ADDRESS_V3) | MTD_TLS_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_SERVER_TLS, <SERVER_TLS_EPHEM_KEY>) Verify that in the Server Hello message: •<SEL_TLS_CIPHER_SUITE> SHALL contain only TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 |
| 2 | S_LPAd → SERVER | MTD_TLS_CLIENT_KEY_EXCH_ETC(<CLIENT_TLS_EPHEM_KEY>) | MTD_TLS_SERVER_END( #CHANGE_CIPHER_SPEC, <SERVER_FINISHED>) |

Test Sequence #04 Error: Invalid TLS Version

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | The server needs to have an address starting with “rsp3-“ and S_LPAd shall contact the server using this address. The S_LPAd shall contact the server using this address. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_LPAd → SERVER | MTD_TLS_CLIENT_HELLO( #TLS_VERSION_1_1, #MIN_TLS_CIPHER_SUITES, #S_SESSION_ID_EMPTY, #SERVER_ADDRESS_V3) | Server sends a TLS Fatal-alert |

Test Sequence #05 Error: Unsupported Cipher Suites and Extensions

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | The server needs to have an address starting with “rsp3-“ and S_LPAd shall contact the server using this address. The S_LPAd shall contact the server using this address. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_LPAd → SERVER | MTD_TLS_CLIENT_HELLO( #TLS_VERSION_1_2, #UNSUP_TLS_CIPHER_SUITES, #S_SESSION_ID_EMPTY, #EXT_SHA256_RSA, #SERVER_ADDRESS_V3) | Server sends a TLS Fatal-alert |

##### 4.6.3.2.2	TC_Server_Authentication_for_HTTPS_EstablishmentBRP

Test Sequence #01 Nominal: HTTPS Session Establishment

This test sequence SHALL be the same as the Test Sequence #01 defined in section 4.6.3.2.1 TC_Server_Authentication_for_HTTPS_EstablishmentNIST, except that the brainpoolP256r1 curve is used.

Test Sequence #02 Nominal: Non-reuse of session keys

This test sequence SHALL be the same as the Test Sequence #02 defined in section 4.6.3.2.1 TC_Server_Authentication_for_HTTPS_EstablishmentNIST, except that the brainpoolP256r1 curve is used.

Test Sequence #03 Nominal: HTTPS Session Establishment with supported and unsupported Cipher Suites

This test sequence SHALL be the same as the Test Sequence #03 defined in section 4.6.3.2.1 TC_Server_Authentication_for_HTTPS_EstablishmentNIST, except that the brainpoolP256r1 curve is used.

## 4.7	LPAe Interfaces

This section is defined as FFS.

# 5	Procedure - Behaviour Testing

## 5.1	General Overview

## 5.2	VOID

## 5.3	Platform Procedures

### 5.3.1	Profile Download and Installation Procedure

This section is defined as FFS and not applicable for this version of test specification.

### 5.3.2	Common Mutual Authentication Process

This section is defined as FFS and not applicable for this version of test specification.

### 5.3.3	Profile Download and Installation Process

#### 5.3.3.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Requirements

- 	RQ44_002

- 	RQ55_033_1

#### 5.3.3.2	Test Cases

##### 5.3.3.2.1	TC_SM_DP+_ProfileMetadata

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| SM-DP+ | - SM-DP+ is configured with the #CERT_SM_DPauth_SIG for NIST. - PROFILE_OPERATIONAL1 (configured with metadata as specified in each sequence) is securely loaded as a Protected Profile Package using <PPK_ENC> and <PPK_MAC>. - Pending Profile PROFILE_OPERATIONAL1 is in the 'Released' state with an empty MatchingID. - EID #EID1 is known to the SM-DP+ and associated to PROFILE_OPERATIONAL1. - Confirmation Code is not provided by the Operator to the SM-DP+. NOTE: the Profile Metadata for PROFILE_OPERATIONAL1 SHALL be specified in the Initial Conditions for each individual sequence. |

Test Sequence #01 Nominal: all elements present

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | SM-DP+ is configured with #SMDP_METADATA_ALL for the pending Profile PROFILE_OPERATIONAL1. |

Run the sequence below with the following parameter assignments:

- 	PARAM_R_AUTH_CLIENT = #R_AUTH_CLIENT_META_ALL

- 	PARAM_METADATA = #SMDP_METADATA_ALL

The sequence below has the following parameters:

- 	PARAM_R_AUTH_CLIENT

- 	PARAM_METADATA

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP(#R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP(PARAM_R_AUTH_CLIENT) |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP(#R_GET_BPP_RESP_OP1_PPK) Construct the complete metadata element from the <SMDP_METADATA_SEG_MAC> segment(s) and verify that it matches PARAM_METADATA |

Test Sequence #02 Nominal: optional elements missing

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | SM-DP+ is configured with #SMDP_METADATA_ABS for the pending Profile PROFILE_OPERATIONAL1. |

This test sequence SHALL be the same as the Test Sequence #01 defined in the current section, with the following parameter assignments:

- 	PARAM_R_AUTH_CLIENT = #R_AUTH_CLIENT_META_ABS

- 	PARAM_METADATA = #SMDP_METADATA_ABS

Test Sequence #03 Nominal: large icon

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | SM-DP+ is configured with #SMDP_METADATA_OP_PROF1_2_SEG for the pending Profile PROFILE_OPERATIONAL1. |

This test sequence SHALL be the same as the Test Sequence #01 defined in the current section, with the following parameter assignments:

- 	PARAM_R_AUTH_CLIENT = #R_AUTH_CLIENT_META_LARGE_ICON

- 	PARAM_METADATA = #SMDP_METADATA_OP_PROF1_2_SEG

Test Sequence #04 Nominal: long Service Provider name

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | SM-DP+ is configured with #SMDP_METADATA_SPN_LONG for the pending Profile PROFILE_OPERATIONAL1. |

This test sequence SHALL be the same as the Test Sequence #01 defined in the current section, with the following parameter assignments:

- 	PARAM_R_AUTH_CLIENT = #R_AUTH_CLIENT_META_SPN_LONG

- 	PARAM_METADATA = #SMDP_METADATA_SPN_LONG

Test Sequence #05 Nominal: long Profile name

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | SM-DP+ is configured with #SMDP_METADATA_PN_LONG for the pending Profile PROFILE_OPERATIONAL1. |

This test sequence SHALL be the same as the Test Sequence #01 defined in the current section, with the following parameter assignments:

- 	PARAM_R_AUTH_CLIENT = #R_AUTH_CLIENT_META_PN_LONG

- 	PARAM_METADATA = #SMDP_METADATA_PN_LONG

Test Sequence #06 Nominal: non-ASCII characters

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | SM-DP+ is configured with #SMDP_METADATA_NON_ASCII for the pending Profile PROFILE_OPERATIONAL1. |

This test sequence SHALL be the same as the Test Sequence #01 defined in the current section, with the following parameter assignments:

- 	PARAM_R_AUTH_CLIENT = #R_AUTH_CLIENT_META_NON_ASCII

- 	PARAM_METADATA = #SMDP_METADATA_NON_ASCII

Test Sequence #07 Nominal: multiple notificationConfigurationInfo elements

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| SM-DP+ | SM-DP+ is configured with #SMDP_METADATA_NOTIF_MULTI for the pending Profile PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP(#R_INITIATE_AUTH_OK) |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP(#R_AUTH_CLIENT_META_NOTIF_MULTI) |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP(#R_GET_BPP_RESP_OP1_PPK) Construct the complete metadata element from the response and verify that it matches #SMDP_METADATA_NOTIF_MULTI |

## 5.4	VOID

# 6	VOID

# 7	External Test Specifications

Some test specifications related to the RSP ecosystem have been developed by external organisations (e.g. TCA (formerly SIMalliance)). These organisations defined their own requirements for test benches, test applicability and pass criteria.

This section lists the test specifications that relate to SGP.21 [3] and SGP.22 [2] requirements.

## 7.1	VOID

- Annex A	Constants

- A.1	Generic Constants

| Name | Content |  |
| --- | --- | --- |
| ACTIVATION_CODE_1 | 1$#TEST_DP_ADDRESS1$#MATCHING_ID_1 ACTIVATION_CODE_1.png as defined in Annex H |  |
| ACTIVATION_CODE_2 | 1$#TEST_DP_ADDRESS1$#MATCHING_ID_2$#S_SM_DP+_OID ACTIVATION_CODE_2.png as defined in Annex H |  |
| ACTIVATION_CODE_3 | 1$#TEST_DP_ADDRESS1$#MATCHING_ID_3$$1 ACTIVATION_CODE_3.png as defined in Annex H |  |
| ACTIVATION_CODE_3_NO_CC | 1$#TEST_DP_ADDRESS1$#MATCHING_ID_3 ACTIVATION_CODE_3_NO_CC.png as defined in Annex H |  |
| ACTIVATION_CODE_4 | 1$#TEST_DP_ADDRESS1$#MATCHING_ID_4 ACTIVATION_CODE_4.png as defined in Annex H |  |
| ACTIVATION_CODE_5 | 1$#TEST_DP_ADDRESS1$#MATCHING_ID_EMPTY ACTIVATION_CODE_5.png as defined in Annex H |  |
| ACTIVATION_CODE_INVALID_FORMAT | 1#TEST_DP_ADDRESS1$#MATCHING_ID_1 ACTIVATION_CODE_INVALID_FORMAT.png as defined in Annex H |  |
| ADDITIONAL_SMDP_DATA_EXCEEDED_MAX | 0x01 02 03…76 77 78 -- additional data objects defined by the S_SM-DP+ depending on the length of the SM-DP+ OID, to ensure that total length of dpProprietaryData is 129 bytes |  |
| ADDITIONAL_SMDP_DATA_MAX_LENGTH | 0x01 02 03…75 76 77 -- additional data objects defined by the S_SM-DP+ depending on the length of the SM-DP+ OID, to ensure that total length of dpProprietaryData is 128 bytes |  |
| CHANGE_CIPHER_SPEC | 1 |  |
| CLIENT_CERT_TYPE | 64. The Certificate Type requested from the client by the server in the Certificate Request message as ecdsa_sign(64). |  |
| CONFIRMATION_CODE1 | 0102030405 |  |
| CONFIRMATION_CODE2 | ABCDEFGHIJ |  |
| CTX_PARAMS1 (CtxParams1) | ctxParamsForCommonAuthentication : { #S_DEVICE_INFO } |  |
| CTX_PARAMS1_DEVICE_INFO_EXT | ctx CtxParams1 ::= ctxParamsForCommonAuthentication : { matchingId #MATCHING_ID_EMPTY, deviceInfo #S_DEVICE_INFO_EXT } |  |
| CTX_PARAMS1_EVENT_ID (CtxParams1) | ctxParamsForCommonAuthentication : { matchingId #EVENT_ID_1, #S_DEVICE_INFO } |  |
| CTX_PARAMS1_EVENT_ID_IMEI (CtxParams1) | ctxParamsForCommonAuthentication : { matchingId #EVENT_ID_1, #S_DEVICE_INFO_IMEI } |  |
| CTX_PARAMS1_IMEI (CtxParams1) | ctxParamsForCommonAuthentication : { #S_DEVICE_INFO_IMEI } |  |
| CTX_PARAMS1_MATCH_ID (CtxParams1) | ctxParamsForCommonAuthentication : { matchingId #MATCHING_ID_1, #S_DEVICE_INFO } |  |
| CTX_PARAMS1_MATCH_ID_DEV_INFO (CtxParams1) | ctxParamsForCommonAuthentication : { matchingId <MATCHING_ID>, -- OPTIONAL - see NOTE #DEVICE_INFO } NOTE: the matchingId field may be present (with value <MATCHING_ID>) or may be absent. The presence or absence of matchingId may be checked in individual test cases. |  |
| DEVICE_INFO | deviceInfo { tac ..., deviceCapabilities { ... }, imei ... -- Optional }-- Check only that the field is present and has a valid TLV asn.1 structure NOTE: The content of deviceInfo is verified in individual test cases. |  |
| DIST_NAME_CI'C=IT,O=RSPTEST,OU=TESTCERT,CN=GSMA Test CI' |  |
| EF_UST1 | 0x0A 2E 14 8C E7 32 04 00 00 00 00 00 00 -- NOTE: Service n°17 (GID1) and n°18 (GID2) not available |  |
| EF_UST2 | 0x0A 2E 17 8C E7 32 04 00 00 00 00 00 00 -- NOTE: Service n°17 (GID1) and n°18 (GID2) available |  |
| EID1 | 0x89 04 90 32 12 34 51 23 45 12 34 56 78 90 12 35 |  |
| EID1_QR_CODE1 | QR code which decodes as: EID:89049032123451234512345678901235 |  |
| EID1_QR_CODE2 | QR code which decodes as: EID:89 04 90 32 12 34 51 23 45 12 34 56 78 90 12 35 |  |
| EID2 | 0x89 04 90 32 11 23 41 23 40 12 34 56 78 90 13 75 |  |
| EUICC_CI_PK_ID_LIST_FOR_SIGNING_1 | #CI_PKI_ID1, #CI_PKI_ID2 |  |
| EUICC_CI_PK_ID_LIST_FOR_SIGNING_2 | #CI_PKI_ID3, #CI_PKI_ID4 |  |
| EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1 | #CI_PKI_ID1, #CI_PKI_ID2 |  |
| EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_2 | #CI_PKI_ID3, #CI_PKI_ID4 |  |
| EUICC_INFO1_8_8_2_3_1 | euiccInfo1_8_8_2_3_1 EUICCInfo1 ::= { svn #RSP_SVN, euiccCiPKIdListForVerification { #EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1 }, euiccCiPKIdListForSigning { #EUICC_CI_PK_ID_LIST_FOR_SIGNING_2 } } |  |
| EUICC_INFO1_8_8_3_3_1_HIGHER | euiccInfo1_8_8_3_3_1 EUICCInfo1 ::= { svn #RSP_SVN_HIGHER, euiccCiPKIdListForVerification { #EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1 }, euiccCiPKIdListForSigning { #EUICC_CI_PK_ID_LIST_FOR_SIGNING_1 } } |  |
| EUICC_INFO1_8_8_3_3_1_LOWER | euiccInfo1_8_8_3_3_1 EUICCInfo1 ::= { svn #RSP_SVN_LOWER, euiccCiPKIdListForVerification { #EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1 }, euiccCiPKIdListForSigning { #EUICC_CI_PK_ID_LIST_FOR_SIGNING_1 } } |  |
| EUICC_INFO1_8_8_4_3_7 | euiccInfo1_8_8_4_3_7 EUICCInfo1 ::= { svn #RSP_SVN, euiccCiPKIdListForVerification { #EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_2 }, euiccCiPKIdListForSigning { #EUICC_CI_PK_ID_LIST_FOR_SIGNING_1 } } |  |
| EUICC_SIGNED1 | { transactionId <S_TRANSACTION_ID>, serverAddress #TEST_DP_ADDRESS1, serverChallenge <S_SMDP_CHALLENGE>, euiccInfo2 #R_EUICC_INFO2, -- check only that the field is present and has a valid TLV asn.1 structure ctxParams1 #CTX_PARAMS1 } |  |
| EVENT_ID_1 | 07399-BGH7E-T8779 |  |
| EVENT_ID_2 | 07399-BGH7E-T8778 |  |
| EVENT_TYPE_DOWNLOAD | 1 |  |
| EVENT_TYPE_PROFILE_DOWNLOAD | 1 |  |
| EVENT_TYPE_RPM | 2 |  |
| EXT_SHA256_RSA | TLS extension data for "supported_signature_algorithms" set as: o HashAlgorithm sha256 (04) and o SignatureAlgorithm rsa (01). |  |
| FUNCTION_CALL_ID_1 | 0000-0000-0000-0001 |  |
| FUNCTION_CALL_ID_2 | 0000-0000-0000-0002 |  |
| GID1 | 0x47 53 4D 41 |  |
| GID2 | 0x52 53 50 FF |  |
| HASHED_ICCID_OP_PROF1 | 0x6F 3E 4F E1 0E 22 08 C2 5D 86 91 85 19 78 9D 69 AE 89 B7 E4 5E 71 3D F0 53 76 BE 46 F2 1F 22 FD |  |
| HASHED_SALTED_ICCID_OP_PROF1 | 0x5C EC BF D1 F1 D9 4D 44 72 B3 29 01 37 CA D1 60 AB EE B1 B2 1F F4 A7 F7 09 72 73 27 F5 41 65 29 |  |
| HOST_ID | 0x47 53 4D 41 20 53 4D 2D 58 58 -- NOTE: 'GSMA SM-XX' in ASCII |  |
| ICCID_OP_PROF1 | 0x98 92 09 01 21 43 65 87 09 F5 |  |
| ICCID_OP_PROF1_NON_SWAP | Value of #ICCID_OP_PROF1 as an ICCID type as specified in SGP.22 [2] section 5.2.1 – i.e. a String in non-swapped format consisting of 20 characters (i.e. with the padding character): 89…905F. |  |
| ICCID_OP_PROF2 | 0x98 92 09 01 32 54 76 98 10 F9 |  |
| ICCID_OP_PROF2_NON_SWAP | Value of #ICCID_OP_PROF2 as an ICCID type as specified in SGP.22 [2] section 5.2.1 – i.e. a String in non-swapped format consisting of 20 characters (i.e. with the padding character): 89…019F. |  |
| ICCID_OP_PROF3 | 0x98 92 09 01 43 65 87 09 21 F5 |  |
| ICCID_OP_PROF4 | 0x98 92 09 01 54 76 98 10 32 F9 |  |
| ICCID_OP_PROF5 | 0x98 92 09 01 65 87 09 21 43 F5 |  |
| ICCID_OP_PROF6 | 0x98 92 09 01 76 98 10 32 54 F9 |  |
| ICCID_OP_PROF7 | 0x98 92 09 01 87 09 21 43 65 F5 |  |
| ICCID_OP_PROF8 | 0x98 92 09 01 98 10 32 54 76 F9 |  |
| ICCID_OP_PROF9 | 0x98 92 09 01 21 43 65 87 76 F5 |  |
| ICCID_OP_PROFX | 0x98 92 09 01 43 65 87 09 FF FF |  |
| ICCID_UNKNOWN | 0x98 92 01 0A 21 43 65 87 09 F8 |  |
| ICON_JPG | ICON_JPG.jpg as defined in Annex H |  |
| ICON_OP_PROF1 | profile_O1.png as defined in Annex H |  |
| ICON_OP_PROF1_2_SEG | profile_O1_2_SEG.png as defined in Annex H |  |
| ICON_OP_PROF2 | profile_O2.png as defined in Annex H |  |
| ICON_OP_PROF3 | profile_O3.png as defined in Annex H |  |
| ICON_OP_PROF4 | profile_O4.png as defined in Annex H |  |
| ICON_OP_PROF5 | profile_O5.png as defined in Annex H |  |
| ICON_OP_PROF6 | profile_O6.png as defined in Annex H |  |
| ICON_OP_PROF7 | profile_O7.png as defined in Annex H |  |
| ICON_OP_PROF8 | profile_O8.png as defined in Annex H |  |
| IMSI_OP_PROF1 | 0x08 29 99 18 11 32 54 76 98 |  |
| IMSI_OP_PROF2 | 0x08 29 99 28 11 32 54 76 97 |  |
| IMSI_OP_PROF3 | 0x08 29 99 28 11 32 54 76 96 |  |
| IMSI_OP_PROF4 | 0x08 29 99 48 43 65 87 09 21 |  |
| IMSI_OP_PROF5 | 0x08 29 99 18 11 32 54 76 98 |  |
| IMSI_OP_PROF6 | 0x08 29 99 28 11 32 54 76 97 |  |
| IMSI_OP_PROF7 | 0x08 29 99 88 43 65 87 09 21 |  |
| IMSI_OP_PROF8 | 0x08 29 99 88 43 65 87 09 21 |  |
| IMSI_OP_PROF9 | 0x08 29 99 98 43 65 87 09 21 |  |
| INSTALLED_PROFILES | 0x00 |  |
| INVALID_FORMAT_MATCHING_ID | 04386-agyft-A74Y8-3F815 |  |
| INVALID_KEY_TYPE | 0x80 |  |
| INVALID_REMOTE_OP_ID | 8 |  |
| ISD_R_AID | 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 00 01 00 |  |
| KEY_LENGTH | 0x10 |  |
| KEY_TYPE | 0x88 |  |
| MATCHING_ID_1 | 04386-AGYFT-A74Y8-3F815 |  |
| MATCHING_ID_2 | 04386-AGYFT-A74Y8-3F816 |  |
| MATCHING_ID_3 | 04386-AGYFT-A74Y8-3F817 |  |
| MATCHING_ID_4 | 04386-AGYFT-A74Y8-3F818 |  |
| MCC_MNC_WILDCARD | 0x92 F9 EE |  |
| MCC_MNC1 | 0x92 F9 18 |  |
| MCC_MNC2 | 0x92 F9 28 |  |
| MCC_MNC4 | 0x92 F9 48 |  |
| MCC_MNC8 | 0x92 F9 88 |  |
| MCC_MNC9 | 0x92 F9 98 |  |
| MIN_TLS_1_2_CIPHER_SUITES | The minimum TLS cipher suites proposed by the Client: o TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 |  |
| MNO_SCP80_AUTH_KEY | 0x11 22 33 44 55 66 77 88 99 AA BB CC DD EE FF 10 |  |
| MNO_SCP80_DATA_ENC_KEY | 0x99 AA BB CC DD EE FF 10 11 22 33 44 55 66 77 88 |  |
| MNO_SCP80_ENC_KEY | 0x66 77 88 99 AA BB CC DD 11 22 33 44 55 EE FF 10 |  |
| NAME_OP_PROF_LONG | Operational Profile Name with long name of sixty four characters NOTE: the exact text above SHOULD be used, as it is exactly 64 characters long. |  |
| NAME_OP_PROF1 | Operational Profile Name 1 |  |
| NAME_OP_PROF1_NON_ASCII | Operational Profile Name UTF-8 encoding: 0x4F 70 65 72 61 74 69 6F 6E 61 6C 20 50 72 6F 66 69 6C 65 20 4E 61 6D 65 20 E4 BD A0 E5 A5 BD |  |
| NAME_OP_PROF2 | Operational Profile Name 2 |  |
| NAME_OP_PROF3 | Operational Profile Name 3 |  |
| NAME_OP_PROF4 | Operational Profile Name 4 |  |
| NAME_OP_PROF5 | Operational Profile Name 5 |  |
| NAME_OP_PROF6 | Operational Profile Name 6 |  |
| NAME_OP_PROF7 | Operational Profile Name 7 |  |
| NAME_OP_PROF8 | Operational Profile Name 8 |  |
| NAME_OP_PROF9 | Operational Profile Name 9 |  |
| NICKNAME1 | Nickname 1 |  |
| NICKNAME2 | Nickname 2 |  |
| NICKNAME3 | Nickname 3 |  |
| NICKNAME4 | Nickname 4 |  |
| OWNER_OP_PROF1 | { mccMnc #MCC_MNC1 } |  |
| OWNER_OP_PROF2 | { mccMnc #MCC_MNC2 } |  |
| PATH_AUTH_CLIENT | /gsma/rsp2/es9plus/authenticateClient |  |
| PATH_CANCEL_ORDER | /gsma/rsp2/es2plus/cancelOrder |  |
| PATH_CANCEL_SESSION | /gsma/rsp2/es9plus/cancelSession |  |
| PATH_CONFIRM_ORDER | /gsma/rsp2/es2plus/confirmOrder |  |
| PATH_DELETE_EVENT | /gsma/rsp2/es12/deleteEvent |  |
| PATH_DOWNLOAD_ORDER | /gsma/rsp2/es2plus/downloadOrder |  |
| PATH_GET_BPP | /gsma/rsp2/es9plus/getBoundProfilePackage |  |
| PATH_HANDLE_NOTIF | /gsma/rsp2/es9plus/handleNotification |  |
| PATH_INITIATE_AUTH | /gsma/rsp2/es9plus/initiateAuthentication |  |
| PATH_REGISTER_EVENT | /gsma/rsp2/es12/registerEvent |  |
| PO1_PIN1 | 0x32 34 36 38 FF FF FF FF |  |
| PO2_PIN1 | 0x33 35 37 39 FF FF FF FF |  |
| PPK_ENC_INV_SIZE | 0x01 02 03 04 05 06 07 08 09 0A 0B 0C 0D 0E 0F 10 0D 0E 0F 10 0D 0E 0F 10 |  |
| PPK_INIT_MAC_INV_SIZE | 0x05 0A 04 0B 03 0C 02 0D 01 0E 00 0F 09 01 08 02 09 01 08 02 09 01 08 02 |  |
| PPK_MAC_INV_SIZE | 0x01 0E 00 0F 09 01 08 02 05 0A 04 0B 03 0C 02 0D 03 0C 02 0D 03 0C 02 0D |  |
| PROFILE_STATUS_AVAILABLE | Available |  |
| PROP_TLS_1_2_CIPHER_SUITES | The TLS cipher suites proposed by the Client: o TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 o TLS_ECDHE_ECDSA_WITH_AES_256_GCM_SHA384 o TLS_RSA_WITH_AES_128_CBC_SHA o TLS_RSA_WITH_AES_256_CBC_SHA256 o TLS_DHE_RSA_WITH_AES_256_CBC_SHA256 |  |
| REMOTE_OP_ID_INSTALL | 1 |  |
| RSP_SVN | This field is set to #IUT_RSP_VERSION (e.g. 2.1.0) |  |
| RSP_SVN_H | This field is set to #IUT_RSP_VERSION encoded as the value part of an ASN.1 VersionType (e.g. 0x02 01 00) |  |
| RSP_SVN_HIGHER | 100.0.0 |  |
| RSP_SVN_LOWER | 0.0.0 |  |
| RSP_SVN_V2_2_1 | 2.2.1 |  |
| RSP_SVN_V2_2_2 | 2.2.2 |  |
| RSP_SVN_V2_3 | 2.3.0 |  |
| S_DEVICE_CAP_EXT | deviceCapExt DeviceCapExt ::= { unknownServiceSupport2 } using the following definition of DeviceCapExt: DeviceCapExt ::= INTEGER { unknownServiceSupport1 (0), unknownServiceSupport2 (1) } |  |
| S_DEVICE_INFO | deviceInfo { tac #S_TAC, deviceCapabilities { gsmSupportedRelease '050000'H, utranSupportedRelease '080000'H, cdma2000onexSupportedRelease '010000'H, cdma2000hrpdSupportedRelease '010000'H, cdma2000ehrpdSupportedRelease '020000'H, eutranSupportedRelease '020000'H, contactlessSupportedRelease '090000'H, rspCrlSupportedVersion #RSP_SVN_H }, lpaRspCapability #S_LPA_RSP_CAPABILITY } |  |
| S_DEVICE_INFO_EXT | deviceInfo DeviceInfo { tac #S_TAC, deviceCapabilities { gsmSupportedRelease '050000'H, utranSupportedRelease '080000'H, cdma2000onexSupportedRelease '010000'H, cdma2000hrpdSupportedRelease '010000'H, cdma2000ehrpdSupportedRelease '020000'H, eutranEpcSupportedRelease '020000'H, contactlessSupportedRelease '090000'H, rspCrlSupportedVersion, #RSP_SVN_H nrEpcSupportedRelease '0F0000'H, nr5gcSupportedRelease '0F0000'H, eutran5gcSupportedRelease '0F0000'H, lpaSvn '030100'H, -- No catSupportedClasses field euiccFormFactorType 0, -- removable deviceAdditionalFeatureSupport { naiSupport '0F0000'H }, unknownServiceSupport #S_DEVICE_CAP_EXT }, lpaRspCapability #S_LPA_RSP_CAPABILITY } Note: the definition of DeviceInfo used above is equivalent to the definition in SGP.22 v3.1 (specific version of [2]) with the addition of a further field called “unknownServiceSupport” of type DeviceCapExt (see #S_DEVICE_CAP_EXT) after the “ deviceAdditionalFeatureSupport” field. |  |
| S_DEVICE_INFO_IMEI | deviceInfo { tac #S_TAC, deviceCapabilities { gsmSupportedRelease '050000'H, utranSupportedRelease '080000'H, cdma2000onexSupportedRelease '01000'H, eutranSupportedRelease '020000'H }, imei #S_IMEI, lpaRspCapability #S_LPA_RSP_CAPABILITY } |  |
| S_EUICC_CHALLENGE | 0x01 02 03 04 05 06 07 08 01 02 03 04 05 06 07 08 |  |
| S_EUICC_CHALLENGE_2 | 0x21 22 23 24 25 26 27 28 21 22 23 24 25 26 27 28 |  |
| S_EUICC_INFO1 | euiccInfo1 EUICCInfo1 ::= { svn #RSP_SVN, euiccCiPKIdListForVerification { #EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1 }, euiccCiPKIdListForSigning { #EUICC_CI_PK_ID_LIST_FOR_SIGNING_1 } euiccRspCapability { #RSP_CAPABILITY } } |  |
| S_EUICC_INFO1_V2_2_1 | euiccInfo1 EUICCInfo1 ::= { svn #RSP_SVN_V2_2_1, euiccCiPKIdListForVerification { #EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1 }, euiccCiPKIdListForSigning { #EUICC_CI_PK_ID_LIST_FOR_SIGNING_1 } } |  |
| S_EUICC_INFO1_V2_2_2 | euiccInfo1 EUICCInfo1 ::= { svn #RSP_SVN_V2_2_2, euiccCiPKIdListForVerification { #EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1 }, euiccCiPKIdListForSigning { #EUICC_CI_PK_ID_LIST_FOR_SIGNING_1 } } |  |
| S_EUICC_INFO1_V2_3 | euiccInfo1 EUICCInfo1 ::= { svn #RSP_SVN_V2_3, euiccCiPKIdListForVerification { #EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1 }, euiccCiPKIdListForSigning { #EUICC_CI_PK_ID_LIST_FOR_SIGNING_1 } } |  |
| S_EUICC_INFO2 | euiccInfo2 EUICCInfo2 ::= { profileVersion #PROFILE_VERSION, svn #RSP_SVN_H, euiccFirmwareVer #EUICC_FIRMWARE_VER, extCardResource #S_EXT_CARD_RESOURCE, uiccCapability #UICC_CAPABILITY, rspCapability #RSP_CAPABILITY, euiccCiPKIdListForVerification {#EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1}, euiccCiPKIdListForSigning {#EUICC_CI_PK_ID_LIST_FOR_SIGNING_1}, ppVersion #PP_VERSION, sasAcreditationNumber #SAS_ACREDITATION_NUMBER } |  |
| S_EUICC_INFO2_EXT | euiccInfo2 EUICCInfo2 ::= { profileVersion #PROFILE_VERSION, svn #RSP_SVN_H, euiccFirmwareVer #EUICC_FIRMWARE_VER, extCardResource #S_EXT_CARD_RESOURCE, uiccCapability #UICC_CAPABILITY, rspCapability #RSP_CAPABILITY, euiccCiPKIdListForVerification {#EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1}, euiccCiPKIdListForSigning {#EUICC_CI_PK_ID_LIST_FOR_SIGNING_1}, ppVersion #PP_VERSION, sasAcreditationNumber #SAS_ACREDITATION_NUMBER, unknownEuiccInfo2Ext #S_EUICC_INFO2_UNKNOWN_EXT } |  |
| S_EUICC_INFO2_UICC_EXT | euiccInfo2 EUICCInfo2 ::= { profileVersion #PROFILE_VERSION, svn #RSP_SVN_H, euiccFirmwareVer #EUICC_FIRMWARE_VER, extCardResource #S_EXT_CARD_RESOURCE, uiccCapability #UICC_CAPABILITY_EXT, rspCapability #RSP_CAPABILITY, euiccCiPKIdListForVerification {#EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1}, euiccCiPKIdListForSigning {#EUICC_CI_PK_ID_LIST_FOR_SIGNING_1}, ppVersion #PP_VERSION, sasAcreditationNumber #SAS_ACREDITATION_NUMBER } |  |
| S_EUICC_INFO2_UNKNOWN_EXT | euiccInfo2Ext EuiccInfo2Ext ::= { unknownServiceSupport2 } using the following definition of EuiccInfo2Ext: EuiccInfo2Ext ::= [120] INTEGER { -- Tag '9F78' unknownServiceSupport1 (0), unknownServiceSupport2 (1) } |  |
| S_EUICC_INFO2_DEV_EXT | euiccInfo2 EUICCInfo2 ::= { profileVersion #PROFILE_VERSION, svn #RSP_SVN_H, euiccFirmwareVer #EUICC_FIRMWARE_VER, extCardResource #S_EXT_CARD_RESOURCE, uiccCapability #UICC_CAPABILITY, rspCapability #RSP_CAPABILITY_EXT, euiccCiPKIdListForVerification {#EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1}, euiccCiPKIdListForSigning {#EUICC_CI_PK_ID_LIST_FOR_SIGNING_1}, ppVersion #PP_VERSION, sasAcreditationNumber #SAS_ACREDITATION_NUMBER } |  |
| S_EXT_SHA256_ECDSA | TLS extension data for "supported_signature_algorithms" set as: o HashAlgorithm sha256 (04) and o SignatureAlgorithm ecdsa (03). |  |
| S_IMEI | 0x00 00 00 00 11 11 11 11 |  |
| S_LPA_RSP_CAPABILITY | lpaRspCapability LpaRspCapability ::= { crlStaplingV3Support, certChainV3Support } |  |
| S_MNO_F_REQ_ID | “S_MNO” |  |
| S_SAH_SHA256_ECDSA | Signature And Hash Algorithm extension sent in the CertificateRequest message set as: o HashAlgorithm sha256 (04) and o SignatureAlgorithm ecsda(3). |  |
| S_SESSION_ID_EMPTY | Empty TLS session ID to identify a new session, with the Length set as ‘zero’. |  |
| S_SM_DP+_F_REQ_ID | “S_SM_DP_PLUS” |  |
| S_SM_DP+_OID | 2.999.10 |  |
| S_SM_DP+_OID2 | 2.999.12 |  |
| S_SM_DP+_OID4 | 2.999.14 |  |
| S_SM_DP+_OID8 | 2.999.18 |  |
| S_SM_DS_F_REQ_ID | “S_SM_DS” |  |
| S_SM_DS_OID | 2.999.15 |  |
| S_TAC | 0x00 00 00 00 |  |
| S_TLS_CIPHER_SUITE | TLS cipher suite selected as follows: o TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 if present in <TLS_CIPHER_SUITES>, otherwise o TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 |  |
| SALT | 0x00 11 22 33 44 55 66 77 |  |
| SERVER_ADDRESS | FQDN of the SERVER Under Test which can be one of the following depending on the entity under test: -  #IUT_SM_DP_ADDRESS -  #IUT_SM_DS_ADDRESS_ES11 |  |
| SERVER_ADDRESS_V3 | V3-specific FQDN of the SERVER Under Test which can be one of the following depending on the entity under test:  "rsp3-“ + #IUT_SM_DP_ADDRESS  "rsp3-“ + #IUT_SM_DS_ADDRESS_ES11 |  |
| SIMA_RESULT_OK | simaresp EUICCResponse ::= { peStatus { {status ok} } } |  |
| SP_NAME_LONG | SP Name as thirty two characters NOTE: the exact text above SHOULD be used, as it is exactly 32 characters long. |  |
| SP_NAME_NON_ASCII | SP Name UTF-8 encoding: 0x53 50 20 4E 61 6D 65 20 E3 83 AB |  |
| SP_NAME1 | SP Name 1 |  |
| SP_NAME2 | SP Name 2 |  |
| SP_NAME3 | SP Name 3 |  |
| SP_NAME4 | SP Name 4 |  |
| SP_NAME8 | SP Name 8 |  |
| SP_NAME9 | SP Name 9 |  |
| SSD_AID | 0xA0 00 00 05 59 10 10 01 02 73 64 56 61 6C 75 65 |  |
| TEST_ALT_DS_ADDRESS | testaltsmds.example.com |  |
| TEST_DEFAULT_DP_ADDRESS_1 | testdefaultsmdpplus1.example.com |  |
| TEST_DP_ADDRESS1 | testsmdpplus1.example.com |  |
| TEST_DP_ADDRESS2 | testsmdpplus2.example.com |  |
| TEST_DP_ADDRESS3 | testsmdpplus3.example.com |  |
| TEST_DP_ADDRESS4 | testsmdpplus4.example.com |  |
| TEST_DP_ADDRESS8 | testsmdpplus8.example.com |  |
| TEST_DS_ADDRESS1 | testsmds1.example.com |  |
| TEST_ROOT_DS_ADDRESS | testrootsmds.example.com |  |
| TLS_VERSION_1_1 | 1.1 |  |
| TLS_VERSION_1_2 | 1.2 The minimum TLS Version supported by the Server. |  |
| UNKNOWN_BPP_SEGMENT | 0xC9 05 01 02 03 04 05 |  |
| UNKNOWN_SERVER_ADDRESS | unknownserver.example.com |  |
| UNSUP_TLS_CIPHER_SUITES | The TLS cipher suites proposed by the Client: o TLS_RSA_WITH_AES_128_CBC_SHA o TLS_RSA_WITH_AES_256_CBC_SHA256 |  |
| UPP_OP_PROF1 | The Unprotected Profile Package related to the PROFILE_OPERATIONAL1 (see Annex E). |  |
| UPP_OP_PROF2 | The Unprotected Profile Package related to the PROFILE_OPERATIONAL2 (see Annex E). |  |
| UPP_OP_PROF3 | The Unprotected Profile Package related to the PROFILE_OPERATIONAL3 (see Annex E). |  |
| UPP_OP_PROF4 | The Unprotected Profile Package related to the PROFILE_OPERATIONAL4 (see Annex E). |  |
| UPP_OP_PROF9 | The Unprotected Profile Package related to the PROFILE_OPERATIONAL9 (see Annex E). |  |
| USIM_AID | 0xA0 00 00 00 87 10 02 FF 33 FF 01 89 00 00 01 00 |  |

- A.2	Test Certificates and Test Keys

All ECC certificates and keys described below are based on either:

- 	NIST P-256 curve, defined in Digital Signature Standard [11]

- 	brainpoolP256r1 curve, defined in RFC 5639 [8]

- 	FRP256V1 curve, defined in ANSSI ECC [9]

NOTE: 	SGP.26 [25] contains test keys, valid test certificates and instructions for how to generate invalid certificates. Unless specified differently, the test keys and test certificates used in the present document are bundled with SGP.26 [25].

| Name | Description |
| --- | --- |
| CERT_CI_SIG | Certificate of the CI for its Public ECDSA Key |
| CERT_CLIENT_TLS | CERT.CLIENT.TLS certificate of the Client under test, based on NIST or Brainpool for this version of the specification, where the Certificate MAY be one of the following depending on the type of Server and whether it is a Client under test or a Client Simulator: -  #CERT_SM_DP_TLS -  #CERT_SM_DS_TLS -  #CERT_S_SM_DP_TLS -  #CERT_S_SM_DS_TLS - • #CERT_S_OPERATOR_TLS |
| CERT_EUICC_SIG | Certificate of the eUICC for its Public ECDSA key CERT.EUICC.SIG in the X.509 format signed by the EUM with SK.EUM.ECDSA |
| CERT_EUICC_SIG_EID2 | Certificate of the eUICC for its Public ECDSA key (CERT.EUICC.SIG) in the X509 format signed by the EUM with SK.EUM.ECDSA with the subject field value serialNumber set as #EID2. Depending on the eUICC configuration, this certificate is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| CERT_EUICC_SIG_EXPIRED | RSP Certificate of the eUICC (CERT.EUICC.SIG) set as a fixed test CERT with 13th January 2016 set in the validity field. Depending on the eUICC configuration, this certificate is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| CERT_EUICC_SIG_INVALID_EX_CP | RSP Certificate of the eUICC (CERT.EUICC.SIG) set as a fixed test CERT with an invalid Certificate Policies extension field OID extnValue set as “id-rspRole-ci”. Depending on the eUICC configuration, this certificate is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| CERT_EUICC_SIG_INVALID_EX_KU | RSP Certificate of the eUICC (CERT.EUICC.SIG) set as a fixed test CERT with an invalid Key Usage extension field extnValue set as “dataEncipherment”. Depending on the eUICC configuration, this certificate is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| CERT_EUICC_SIG_INVALID_SIG | RSP Certificate of the eUICC (CERT.EUICC.SIG) set as a fixed test CERT with an invalid signature in the signatureValue field. Depending on the eUICC configuration, this certificate is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| CERT_EUICC_SIG_INVALID_SUB_ORG | RSP Certificate of the eUICC (CERT.EUICC.SIG) set as a fixed test CERT with an invalid 'organization' attribute value in the subject field set as “ERRORNAME”. Depending on the eUICC configuration, this certificate is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| CERT_EUICC_SIG_INVALID_SUB_SN | RSP Certificate of the eUICC (CERT.EUICC.SIG) set as a fixed test CERT with an invalid 'serialNumber' attribute value (starting with incorrect IIN) in the subject field set as “89299000112341234012345678901353”. Depending on the eUICC configuration, this certificate is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| CERT_EUM_SIG | Certificate of the EUM for its Public ECDSA key CERT.EUM.SIG in the X.509 format signed by the requested CI with SK.CI.SIG. |
| CERT_EUM_SIG_EXPIRED | RSP Certificate of the eUICC (CERT.EUM.SIG) set as a fixed test CERT with 13th January 2016 set in the validity field. Depending on the eUICC configuration, this certificate is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| CERT_EUM_SIG_INVALID_EX_BC_cA | RSP Certificate of the EUM (CERT.EUM.SIG) set as a fixed test CERT with an invalid Basic Constraints extension field set as “cA = false”. Depending on the eUICC configuration, this certificate is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| CERT_EUM_SIG_INVALID_EX_BC_PLC | RSP Certificate of the EUM (CERT.EUM.SIG) set as a fixed test CERT with an invalid Basic Constraints extension field set as “pathLenConstraint = 1”. Depending on the eUICC configuration, this certificate is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| CERT_EUM_SIG_INVALID_EX_CP | RSP Certificate of the EUM (CERT.EUM.SIG) set as a fixed test CERT with an invalid Certificate Policies extension field OID extnValue set as “id-rspRole-ci”. Depending on the eUICC configuration, this certificate is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| CERT_EUM_SIG_INVALID_EX_KU | RSP Certificate of the EUM (CERT.EUM.SIG) set as a fixed test CERT with an invalid Key Usage extension field extnValue set as “dataEncipherment”. Depending on the eUICC configuration, this certificate is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| CERT_EUM_SIG_INVALID_SIG | RSP Certificate of the EUM (CERT.EUM.SIG) set as a fixed test CERT with an invalid signature in the signatureValue field. Depending on the eUICC configuration, this certificate is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| CERT_EUM_SIG_UNKNOWN | RSP Certificate of the EUM (CERT.EUM.SIG) set as a fixed test CERT with the Authority Key Identity not trusted by the SM-DP+ as it is not found in #EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1 or #EUICC_CI_PK_ID_LIST_FOR_SIGNING_1. Depending on the eUICC configuration, this certificate is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| CERT_S_CLIENT_TLS | CERT.CLIENT.TLS certificate of the S_CLIENT, based on NIST or Brainpool for this version of the specification, where the Certificate MAY be one of the following depending on the role of the simulator: -  #CERT_S_SM_DP_TLS -  #CERT_S_SM_DS_TLS |
| CERT_S_CLIENT_TLS_EXPIRED | CERT.CLIENT.TLS certificate of the S_CLIENT, where the Certificate MAY be one of the following depending on the role of the simulator: -  #CERT_S_SM_DP_TLS_EXPIRED -  #CERT_S_SM_DS_TLS_EXPIRED |
| CERT_S_CLIENT_TLS_INV_CERT_POL | CERT.CLIENT.TLS certificate of the S_CLIENT, where the Certificate MAY be one of the following depending on the role of the simulator: -  #CERT_S_SM_DP_TLS_INV_CERT_POL -  #CERT_S_SM_DS_TLS_INV_CERT_POL |
| CERT_S_CLIENT_TLS_INV_CRITICAL_EXT | CERT.CLIENT.TLS certificate of the S_CLIENT, where the Certificate MAY be one of the following depending on the role of the simulator: -  #CERT_S_SM_DP_TLS_INV_CRITICAL_EXT -  #CERT_S_SM_DS_TLS_INV_CRITICAL_EXT |
| CERT_S_CLIENT_TLS_INV_EXT_KEY_USAGE | CERT.CLIENT.TLS certificate of the S_CLIENT, where the Certificate MAY be one of the following depending on the role of the simulator: -  #CERT_S_SM_DP_TLS_INV_EXT_KEY_USAGE -  #CERT_S_SM_DS_TLS_INV_EXT_KEY_USAGE |
| CERT_S_CLIENT_TLS_INV_KEY_USAGE | CERT.CLIENT.TLS certificate of the S_CLIENT, where the Certificate MAY be one of the following depending on the role of the simulator: -  #CERT_S_SM_DP_TLS_INV_KEY_USAGE -  #CERT_S_SM_DS_TLS_INV_KEY_USAGE |
| CERT_S_CLIENT_TLS_INV_OID | CERT.CLIENT.TLS certificate of the S_CLIENT, where the Certificate MAY be one of the following depending on the role of the simulator: -  #CERT_S_SM_DP_TLS_INV_OID -  #CERT_S_SM_DS_TLS_INV_OID |
| CERT_S_CLIENT_TLS_INV_SIG | CERT.CLIENT.TLS certificate of the S_CLIENT, where the Certificate MAY be one of the following depending on the role of the simulator: -  #CERT_S_SM_DP_TLS_INV_SIG -  #CERT_S_SM_DS_TLS_INV_SIG |
| CERT_S_OPERATOR_TLS | Certificate of the S_MNO in X.509 format and based on NIST or Brainpool for this version of the specification The CERT_S_OPERATOR_TLS Test certificate is not defined in SGP.26. The content of CERT_S_OPERATOR_TLS is expected to be provided either by the test tool or by the SM-DP+ vendor. |
| CERT_S_SERVER_TLS | CERT.SERVER.TLS certificate of the S_SERVER, based on NIST or Brainpool for this version of the specification, where the Certificate MAY be one of the following depending on the role of the simulator: -  #CERT_S_SM_DP_TLS on ES9+ -  #CERT_S_SM_DS_TLS on ES11 or ES12 |
| CERT_S_SERVER_TLS_EXPIRED | CERT.SERVER.TLS certificate of the S_SERVER, where the Certificate MAY be one of the following depending on the role of the simulator: -  #CERT_S_SM_DP_TLS_EXPIRED -  #CERT_S_SM_DS_TLS_EXPIRED |
| CERT_S_SERVER_TLS_INV_CERT_POL | CERT.SERVER.TLS certificate of the S_SERVER, where the Certificate MAY be one of the following depending on the role of the simulator: -  #CERT_S_SM_DP_TLS_INV_CERT_POL -  #CERT_S_SM_DS_TLS_INV_CERT_POL |
| CERT_S_SERVER_TLS_INV_CRITICAL_EXT | CERT.SERVER.TLS certificate of the S_SERVER, where the Certificate MAY be one of the following depending on the role of the simulator: -  #CERT_S_SM_DP_TLS_INV_CRITICAL_EXT -  #CERT_S_SM_DS_TLS_INV_CRITICAL_EXT |
| CERT_S_SERVER_TLS_INV_EXT_KEY_USAGE | CERT.SERVER.TLS certificate of the S_SERVER, where the Certificate MAY be one of the following depending on the role of the simulator: -  #CERT_S_SM_DP_TLS_INV_EXT_KEY_USAGE -  #CERT_S_SM_DS_TLS_INV_EXT_KEY_USAGE |
| CERT_S_SERVER_TLS_INV_KEY_USAGE | CERT.SERVER.TLS certificate of the S_SERVER, where the Certificate MAY be one of the following depending on the role of the simulator: -  #CERT_S_SM_DP_TLS_INV_KEY_USAGE -  #CERT_S_SM_DS_TLS_INV_KEY_USAGE |
| CERT_S_SERVER_TLS_INV_SIG | CERT.SERVER.TLS certificate of the S_SERVER, where the Certificate MAY be one of the following depending on the role of the simulator: -  #CERT_S_SM_DP_TLS_INV_SIG -  #CERT_S_SM_DS_TLS_INV_SIG |
| CERT_S_SM_DP_TLS | CERT.DP.TLS certificate of the S_SM-DP+, based on the same CI as defined in #IUT_LPAd_CI based on NIST for this version of the specification |
| CERT_S_SM_DP2_TLS | CERT.DP.TLS certificate of the S_SM-DP+, based on the same CI as defined in #IUT_LPAd_CI based on NIST for this version of the specification. Contains different SM-DP+ hostname (FQDN) as #CERT_S_SM_DP2_TLS. |
| CERT_S_SM_DP4_TLS | CERT.DP.TLS certificate of the S_SM-DP+, based on the same CI as defined in #IUT_LPAd_CI based on NIST for this version of the specification. Contains the SM-DP+ hostname (FQDN) #TEST_DP_ADDRESS4 and OID value #S_SM_DP+_OID4. |
| CERT_S_SM_DP8_TLS | CERT.DP.TLS certificate of the S_SM-DP+, based on the same CI as defined in #IUT_LPAd_CI based on NIST for this version of the specification. Contains the SM-DP+ hostname (FQDN) #TEST_DP_ADDRESS8 and OID value #S_SM_DP+_OID8. |
| CERT_S_SM_DP_TLS_EXPIRED | Expired CERT.DP.TLS certificate of the S_SM-DP+ with a valid signature, correctly formatted as X.509 certificate. |
| CERT_S_SM_DP_TLS_INV_CERT_POL | CERT.DP.TLS certificate of the S_SM-DP+ with invalid 'Certificate Policies' extension (OID not set to ‘id-rspRole-dp-tls' or 'id-rspRole-ds-tls'), formatted as X.509 certificate. |
| CERT_S_SM_DP_TLS_INV_CRITICAL_EXT | CERT.DP.TLS certificate of the S_SM-DP+ with one of the critical extensions not present, formatted as X.509 certificate. |
| CERT_S_SM_DP_TLS_INV_CURVE | CERT.DP.TLS certificate of the S_SM-DP+, based on the different CI as defined in #IUT_LPAd_CI, not based on  NIST P-256 curve, defined in Digital Signature Standard [11]  brainpoolP256r1 curve, defined in RFC 5639 [8]  FRP256V1 curve, defined in ANSSI ECC [9] |
| CERT_S_SM_DP_TLS_INV_EXT_KEY_USAGE | CERT.DP.TLS certificate of the S_SM-DP+ with invalid 'extended key usage' extension (not set to any combination of 'id-kp-serverAuth' or 'id-kp-clientAuth'), formatted as X.509 certificate. |
| CERT_S_SM_DP_TLS_INV_KEY_USAGE | CERT.DP.TLS certificate of the S_SM-DP+ with invalid 'key usage' extension (not set to 'digitalSignature'), formatted as X.509 certificate. |
| CERT_S_SM_DP_TLS_INV_OID | CERT.DP.TLS certificate of the S_SM-DP+ containing an invalid SM-DP+OID, different to #S_SM_DP+_OID, correctly formatted as X.509 certificate. |
| CERT_S_SM_DP_TLS_INV_SIG | Invalid CERT.DP.TLS certificate of the S_SM-DP+ with an invalid signature with the same tag and length as a valid signature, correctly formatted as X.509 certificate. |
| CERT_S_SM_DPauth_SIG | Certificate of the S_SM-DP+ for its Public ECDSA key used for SM-DP+ authentication. This certificate contains the OID #S_SM_DP+_OID. |
| CERT_S_SM_DP2auth_SIG | Certificate of the S_SM-DP+ for its Public ECDSA key used for SM-DP+ authentication. This certificate contains the OID #S_SM_DP+_OID2. |
| CERT_S_SM_DPauth_INV_SIGN | Invalid certificate of the S_SM-DP+ for its Public ECDSA key used for authentication. This certificate contains the OID #S_SM_DP+_OID and contains an invalid signature (i.e. not generated with the #SK_CI_SIG but with the same tag and length as a valid signature) |
| CERT_S_SM_DPauth_INV_CURVE | Certificate of the S_SM-DP+ for its Public ECDSA key used for Authentication. This certificate contains the OID #S_SM_DP+_OID and a public key based on a curve different from the following ones: -  NIST P-256 curve, defined in Digital Signature Standard [11] -  brainpoolP256r1 curve, defined in RFC 5639 [8] -  FRP256V1 curve, defined in ANSSI ECC [9] |
| CERT_S_SM_DSauth_INV_CURVE | Certificate of the S_SM-DS for its Public ECDSA key used for Authentication. This certificate contains the OID #S_SM_DS_OID and a public key based on a curve different from the following ones:  NIST P-256 curve, defined in Digital Signature Standard [11]  brainpoolP256r1 curve, defined in RFC 5639 [8]  FRP256V1 curve, defined in ANSSI ECC [9] |
| CERT_S_SM_DPpb_SIG | Certificate of the S_SM-DP+ for its Public ECDSA key used for Profile Package Binding. This certificate contains the OID #S_SM_DP+_OID. |
| CERT_S_SM_DPpb_INV_SIGN | Invalid certificate of the S_SM-DP+ for its Public ECDSA key used for Profile Package Binding. This certificate contains the OID #S_SM_DP+_OID and contains an invalid signature (i.e. not generated with the #SK_CI_SIG but with the same tag and length as a valid signature) |
| CERT_S_SM_DPpb_INV_CURVE | Certificate of the S_SM-DP+ for its Public ECDSA key used for Profile Package Binding. This certificate contains the OID #S_SM_DP+_OID and a public key based on a curve different from the following ones: -  NIST P-256 curve, defined in Digital Signature Standard [11] -  brainpoolP256r1 curve, defined in RFC 5639 [8] -  FRP256V1 curve, defined in ANSSI ECC [9] |
| CERT_S_SM_DP2pb_SIG | Certificate of the S_SM-DP+ for its Public ECDSA key used for Profile Package Binding. This certificate contains the OID #S_SM_DP+_OID2. |
| CERT_S_SM_DS_TLS | CERT.DS.TLS certificate of the S_SM-DS based on the same CI as defined in #IUT_LPAd_CI based on NIST or Brainpool for this version of the specification |
| CERT_S_SM_DS2_TLS | CERT.DS.TLS certificate of the S_SM-DS based on the same CI as defined in #IUT_LPAd_CI based on NIST or Brainpool for this version of the specification. Contains different SM-DS hostname (FQDN) as #CERT_S_SM_DS2_TLS. |
| CERT_S_SM_DS_TLS_EXPIRED | Expired CERT.DS.TLS certificate of the S_SM-DS with a valid signature, correctly formatted as X.509 certificate. |
| CERT_S_SM_DS_TLS_INV_CERT_POL | CERT.DS.TLS certificate of the S_SM-DS with invalid ‘Certificate Policies’ extension (OID not set to 'id-rspRole-ds-tls'), formatted as X.509 certificate. |
| CERT_S_SM_DS_TLS_INV_CRITICAL_EXT | CERT.DS.TLS certificate of the S_SM-DS with one of the critical extensions not present, formatted as X.509 certificate. |
| CERT_S_SM_DS_TLS_INV_CURVE | CERT.DP.TLS certificate of the S_SM-DP+, based on the different CI as defined in #IUT_LPAd_CI, not based on  NIST P-256 curve, defined in Digital Signature Standard [11]  brainpoolP256r1 curve, defined in RFC 5639 [8]  FRP256V1 curve, defined in ANSSI ECC [9] |
| CERT_S_SM_DS_TLS_INV_EXT_KEY_USAGE | CERT.DS.TLS certificate of the S_SM-DS with invalid 'extended key usage' extension (not set to any combination of 'id-kp-serverAuth' or 'id-kp-clientAuth'), formatted as X.509 certificate. |
| CERT_S_SM_DS_TLS_INV_KEY_USAGE | CERT.DP.TLS certificate of the S_SM-DS with invalid 'key usage' extension (not set to 'digitalSignature'), formatted as X.509 certificate. |
| CERT_S_SM_DS_TLS_INV_OID | CERT.DS.TLS certificate of the S_SM-DS containing an invalid SM-DS OID, different to #S_SM_DS_OID, correctly formatted as X.509 certificate. |
| CERT_S_SM_DS_TLS_INV_SIG | Invalid CERT.DS.TLS certificate of the S_SM_DS with an invalid signature with the same tag and length as a valid signature, correctly formatted as X.509 certificate. |
| CERT_S_SM_DSauth_SIG | Certificate of the S_SM-DS for its Public ECDSA key used for SM-DS authentication. This certificate contains the OID #S_SM_DS_OID. |
| CERT_S_SM_DSauth_INV_SIGN | Invalid certificate of the S_SM-DS for its Public ECDSA key used for SM-DS authentication. This certificate contains an invalid signature, (i.e. not generated with the #_SIGSK_CI_SIG but with the same tag and length as a valid signature) |
| CERT_SERVER_TLS | CERT.SERVER.TLS certificate of the Server under test, based on NIST or Brainpool for this version of the specification, where the Certificate MAY be one of the following depending on the type of Server and whether it is a Server under test or a Server simulator: -  #CERT_SM_DP_TLS -  #CERT_SM_DS_TLS -  #CERT_S_SM_DP_TLS -  #CERT_S_SM_DS_TLS |
| CERT_SM_DP_TLS | Certificate of the SM-DP+ for securing TLS, based on NIST or Brainpool for this version of the specification. CERT.DP.TLS in X.509 format. |
| CERT_SM_DPauth_SIG | Certificate of the SM-DP+ for its Public ECDSA key used for SM-DP+ authentication (CERT.DPauth.SIG) set as a fixed test CERT. Depending on the SM-DP+ configuration, this certificate is based on NIST P-256, brainpoolP256r1 or FRP256V1. -  The Authority Key Identifier is set as #CI_PKI_ID1 |
| CERT_SM_DPpb_SIG | Certificate of the SM-DP+ for its Public ECDSA key used for Profile Package Binding (CERT.DPpb.ECDSA) set as a fixed test CERT. Depending on the SM-DP+ configuration, this certificate is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| CERT_SM_DS_TLS | Certificate of the SM-DS for securing TLS, based on NIST or Brainpool for this version of the specification. CERT.DS.TLS in X.509 format. |
| CERT_SM_DSauth_SIG | Certificate of the SM-DS for its Public ECDSA key used for SM-DS authentication (CERT.DSauth.SIG) set as a fixed test CERT. Depending on the SM-DS configuration, this certificate is based on NIST P-256, brainpoolP256r1 or FRP256V1. -  The Authority Key Identifier is set as #CI_PKI_ID1 |
| CERT_SM_XXauth_SIG | CERT_SM_XXauth_SIG of the server under test, where XX = DP or XX = DS depending on the entity under test: -  #CERT_SM_DPauth_SIG -  #CERT_SM_DSauth_SIG |
| CI_PKI_ID1 | The CI Subject Key Identifier as defined in SGP.26 [25]. |
| CI_PKI_ID2 | 0x21 22 23 24 25 26 27 28 29 2A 2B 2C 2D 2E 2F 30 31 32 33 |
| CI_PKI_ID3 | 0x31 32 33 34 35 36 37 38 39 3A 3B 3C 3D 3E 3F 40 41 42 43 |
| CI_PKI_ID4 | 0x41 42 43 44 45 46 47 48 49 4A 4B 4C 4D 4E 4F 50 51 52 53 |
| CI_PK_ID_INV | 0x00 01 02 03 04 05 06 07 08 09 0A 0B 0C 0D 0E 0F 10 11 12 |
| PK_CI_SIG | Public Key of the CI, contained within #CERT_CI_SIG |
| PK_EUICC_SIG | Public Key of the eUICC, contained within #CERT_EUICC_SIG |
| PK_S_CLIENT_TLS | Public key of CERT_S_CLIENT_TLS of the S_CLIENT, where the key MAY be one of the following depending on the role of the simulator: -  #PK_S_SM_DP_TLS -  #PK_S_SM_DS_TLS |
| PK_S_SERVER_TLS | Public key of CERT_S_SERVER_TLS of the S_SERVER, where the Certificate MAY be one of the following depending on the role of the simulator: -  #PK_S_SM_DP_TLS on ES9+ -  #PK_S_SM_DS_TLS on ES11 |
| PK_S_SM_DP_TLS | Public key of CERT.DP.TLS of the S_SM-DP+. |
| PK_S_SM_DPauth_SIG | Public Key of the S_SM-DP+, contained within #CERT_S_SM_DPauth_SIG |
| PK_S_SM_DPpb_SIG | Public Key of the S_SM-DP+, contained within #CERT_S_SM_DPpb_SIG |
| PK_S_SM_DS_TLS | Public key of CERT_S_DS_TLS of the S_SM-DS. |
| PK_SM_DPauth_SIG | Public Key of the SM-DP+, contained within #CERT_SM_DPauth_SIG |
| PK_SM_DPpb_SIG | Public Key of the SM-DP+, contained within #CERT_SM_DPpb_SIG |
| PK_SM_DSauth_SIG | Public Key of the SM-DS, contained within #CERT_SM_DSauth_SIG |
| PK_SM_XXauth_SIG | PK_SM_XXauth_SIG of the server under test, where XX = DP or XX = DS depending on the entity under test: -  #PK_SM_DPauth_SIG -  #PK_SM_DSauth_SIG |
| SK_CI_SIG | Private Key of the CI |
| SK_EUICC_SIG | Private key of the eUICC for creating signatures |
| SK_S_SM_DPauth_SIG | Private Key of the of S_SM-DP+ for creating signatures for SM-DP+ authentication |
| SK_S_SM_DSauth_SIG | Private Key of the of S_SM-DS for creating signatures for SM-DS authentication |
| SK_S_SM_DPpb_SIG | Private key of the S_SM-DP+ used to provide signatures for Profile binding |

- Annex B	Dynamic Content

| Variable | Description |
| --- | --- |
| ANY_SW_IN_ERROR | Any Status Word in error (different from 0x9000) |
| BPP | Content of a Bound Profile Package to download within the eUICC. |
| BPP_OTPK_EUICC_AKA | One-time Public Key of the eUICC for ECKA used for the BPP |
| BPP_SEG_A0 | Bound Profile Package TLV segment containing the tag and length fields of the firstSequenceOf87 TLV plus the first 0x87 TLV containing the ConfigureISDP command |
| BPP_SEG_A1 | Bound Profile Package following TLV segment array, as defined in SGP.22 [2] – section 2.5.5: - array first element containing the Tag and length fields of the sequenceOf88 TLV - array following elements containing each of the ‘88’ TLVs containing the StoreMetadata command |
| BPP_SEG_A2 | Bound Profile Package TLV segment containing the Tag and length fields of the secondSequenceOf87 TLV plus the first '87' TLV, containing the ReplaceSessionKeys command |
| BPP_SEG_A3 | Bound Profile Package following TLV segment array, as defined in SGP.22 [2] – section 2.5.5: - array first element containing the tag and length fields of the sequenceOf86 TLV - array following elements containing each of the '86' TLVs containing the Protected Profile Package (PPP) |
| BPP_SEG_INIT | Bound Profile Package TLV segment containing the tag and length fields of the BoundProfilePackage TLV plus the initialiseSecureChannelRequest command |
| C_APDUS_SCRIPT | List of Command APDUs formatted as an expanded structure with definite length coding as defined in ETSI TS 102 226 [14]. |
| CC | SCP80 cryptographic checksum as defined in ETSI TS 102 225 [13] (8 bytes long). |
| CHANNEL_NUMBER | The logical channel number newly opened in the eUICC. If no logical channel is opened, the value is set to 0x00 (i.e. Basic Channel). |
| CLIENT_TLS_EPHEM_KEY | Client's ephemeral key and associated information. |
| CONF_ISDP_PROF1_ENC | An element of firstSequenceOf87, consisting of #CONF_ISDP_PROF1_SMDP protected with <S_ENC> and <S_MAC> and encapsulated in a TLV with tag 0x87, length <L> to a maximum size of 1020 bytes including the tag and length fields. |
| EUICC_CANCEL_SESSION_SIGNATURE | euiccCancelSessionSignature is created using the SK.EUICC.SIG signed over euiccCancelSessionSigned coded as ASN.1 OCTET STRING. |
| EUICC_CANCEL_SESSION_SIGNATURE_INVALID | eUICC signature randomly generated and coded as an ASN.1 OCTET STRING not equal to <EUICC_CANCEL_SESSION_SIGNATURE> but with the same length as a valid signature |
| EUICC_CHALLENGE | Random eUICC challenge, coded as asn.1 OCTET STRING, 16 bytes. |
| EUICC_CI_PK_ID_LIST_FOR_SIGNING | List of CI Public Key Identifiers supported on the eUICC for signature creation, coded as ASN.1 sequence of SubjectKeyIdentifier. The CI Public Key Identifier is from the list of possible CI Public Key Identifier. This possible CI Public Key Identifiers as supported by the eUICC will be defined later on. |
| EUICC_CI_PK_ID_LIST_FOR_VERIFICATION | List of CI Public Key Identifiers supported on the eUICC for signature verification, coded as ASN.1 sequence of SubjectKeyIdentifier. The CI Public Key Identifier is from the list of possible CI Public Key Identifier. This possible CI Public Key Identifiers as supported by the eUICC will be defined later on. |
| EUICC_CI_PK_ID_TO_BE_USED | CI Public Key Identifier to be used by the eUICC for signature, coded as ASN.1 sequence of SubjectKeyIdentifier. |
| EUICC_CS_SIGNATURE | The eUICC cancel session signature computed using the #SK_EUICC_SIG across the EuiccCancelSessionSigned present in the CancelSessionResponse structure |
| EUICC_RSP_CAPABILITY | RspCapability of the eUICC, coded as ASN.1 BIT STRING |
| EUICC_SIGN_PIR | The eUICC signature of the Profile Installation Result (PIR). The input data used to generate the <EUICC_SIGN_PIR> is the profileInstallationResultData TLV. |
| EUICC_SIGNATURE1 | The eUICC signature 1 (euiccSignature1) computed using #SK_EUICC_SIG across the euiccSigned1 present in the AuthenticateServerResponse structure, coded as ASN.1 OCTET STRING. |
| EUICC_SIGNATURE1_INVALID | eUICC signature randomly generated and coded as an ASN.1 OCTET STRING not equal to <EUICC_SIGNATURE1> |
| EUICC_SIGNATURE2 | The eUICC signature 2 (euiccSignature2) computed using the #SK_EUICC_SIG across the following data objects: - euiccSigned2 - smdpSignature2 present in the PrepareDownloadRequest structure |
| EUICC_SIGNATURE2_INVALID | eUICC signature randomly generated and coded as an ASN.1 OCTET STRING not equal to <EUICC_SIGNATURE2> |
| EVENT_ID | An EventID value in String format, generated by the SM-DP+ during Event Record registration. |
| EVENT_ID_D | An EventID value in String format, generated by the Alternative SM-DS during cascaded Event Record deletion on ES15 |
| EVENT_ID_R | The EventID value in String format generated by the Alternative SM-DS during cascaded Event Record registration on ES15. |
| EXT_CARD_RESOURCE | Extended Card Resource Information according to ETSI TS 102 226 [14], coded as ASN.1 OCTET STRING. 'Number of installed application' value field is '00'. |
| EXT_SHA256_ECDSA | TLS extension data for "supported_signature_algorithms" set as a minimum of HashAlgorithm sha256 (04) and SignatureAlgorithm ecdsa (03). |
| FORWARDING_INDICATOR_ANY | Any boolean value (TRUE/FALSE) |
| FREE_MEM_OP_PROF_INSTALLED | Non-volatile memory (tag 0x82) available in the eUICC when two or more PROFILE_OPERATIONAL are installed |
| FREE_MEM_OP_PROF1_DELETED | Non-volatile memory (tag 0x82) available in the eUICC after PROFILE_OPERATIONAL1 deletion |
| FREE_MEM_OP_PROF1_INSTALLED | Non-volatile memory (tag 0x82) available in the eUICC when only PROFILE_OPERATIONAL1 is installed |
| FREE_MEMORY_NO_PROFILE | Non-volatile memory (tag 0x82) available in the eUICC when there is no Profile installed |
| FUNCTION_CALL_ID | The function call ID generated by the entity that calls the function |
| FUNCTION_REQ_ID | The function requester ID |
| INVALID_SM_DP_OID | SM-DP+ OID (as defined in section 1.3) not equal to #IUT_SM_DP_OID |
| INVALID_TRANSACTION_ID | A Transaction Identifier generated by the S_SM-DP+ or the S_SM-DS that SHALL be different from <S_TRANSACTION_ID> if exists. Otherwise, a random value is generated. |
| ISD_P_AID | The ISD-P AID newly created in the eUICC. This AID value is in the range from 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 00 10 00 to 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 FF FF 00. Last byte is set to '00' as defined in SGP.02[1]. |
| ISD_P_AID1 | The ISD-P AID created in the eUICC for the PROFILE_OPERATIONAL1. This AID value belongs to the range from 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 00 10 00 to 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 FF FF 00. Last byte is set to '00' as defined in SGP.02[1]. |
| ISD_P_AID2 | The ISD-P AID created in the eUICC for the PROFILE_OPERATIONAL2. This AID value belongs to the range from 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 00 10 00 to 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 FF FF 00. Last byte is set to '00' as defined in SGP.02[1]. |
| ISD_P_AID3 | The ISD-P AID created in the eUICC for the PROFILE_OPERATIONAL3. This AID value belongs to the range from 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 00 10 00 to 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 FF FF 00. Last byte is set to '00' as defined in SGP.02[1]. |
| ISD_P_AID4 | The ISD-P AID created in the eUICC for the PROFILE_OPERATIONAL4. This AID value belongs to the range from 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 00 10 00 to 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 FF FF 00. Last byte is set to '00' as defined in SGP.02[1]. |
| ISD_P_AIDX | An invalid ISD-P AID not present on the eUICC. This AID value is in the range from 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 00 10 00 to 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 FF FF 00. |
| L | Exact length of the corresponding tag or of the remaining data. |
| LPA_RSP_CAPABILITY | LpaRspCapability of the LPA, coded as ASN.1 BIT STRING |
| MATCHING_ID | Unique identifier as defined in [2]. The content can be either empty, or the value of the EventID, or the value of the Activation Code token. |
| MATCHING_ID_EVENT | A Unique identifier of an Event for a specific EID generated by the SM-DP+ / SM-DS. |
| METADATA_OP_PROF1_SEG | The #METADATA_OP_PROF1 is mac-ed with <S_MAC> and split as necessary into segments of a maximum size of 1020 bytes (including the tag, length field, and MAC), |
| MNO_SCP80_COUNTER | SCP80 counter of the MNO-SD related to the KVN 0x01 (5 bytes long). Initial value is set to 0x00 00 00 00 01 and is incremented by one each time a secured packet is sent. |
| NB_EXECUTED_C_APDUS | Number of executed Command TLV objects as defined in ETSI TS 102 226 [14]. |
| NOTIF_SEQ_NO_DE1 | The Sequence Number of the Delete Notification related to the PROFILE_OPERATIONAL1. |
| NOTIF_SEQ_NO_DI1 | The Sequence Number of the Disable Notification related to the PROFILE_OPERATIONAL1. |
| NOTIF_SEQ_NO_EN1 | The Sequence Number of the Enable Notification related to the PROFILE_OPERATIONAL1. |
| NOTIF_SEQ_NO_EN2 | The Sequence Number of the Enable Notification related to the PROFILE_OPERATIONAL2. |
| NOTIF_SEQ_NO_IN1 | The Sequence Number of the Install Notification related to the PROFILE_OPERATIONAL1. |
| NOTIF_SEQ_NO_IN1_PIR | The Sequence Number of the Install Notification (PIR) related to the PROFILE_OPERATIONAL1. |
| NOTIF_SEQ_NO_IN2 | The Sequence Number of the Install Notification related to the PROFILE_OPERATIONAL2. |
| NOTIF_SEQ_NO_IN2_PIR | The Sequence Number of the Install Notification (PIR) related to the PROFILE_OPERATIONAL2. |
| NOTIF_SEQ_NO2_DE1 | The Sequence Number of the second Delete Notification related to the PROFILE_OPERATIONAL1. |
| NOTIF_SEQ_NO2_DI1 | The Sequence Number of the second Disable Notification related to the PROFILE_OPERATIONAL1. |
| NOTIF_SEQ_NO2_EN1 | The Sequence Number of the second Enable Notification related to the PROFILE_OPERATIONAL1. |
| OT_SK_S_SM_DP+_AKA | One-time Private Key generated by the S_SM-DP+ for ECKA. Depending on the eUICC configuration, this key is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| OTPK_EUICC_AKA | One-time Public Key generated by the eUICC for ECKA. Depending on the eUICC configuration, this key is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| OTPK_EUICC_AKA_NEW | One-time Public Key of the eUICC for ECKA used for the BPP which is a new generated value different from <OTPK_EUICC_AKA> |
| OTPK_S_SM_DP+_AKA | One-time Public Key generated by the S_SM-DP+ for ECKA. Depending on the eUICC configuration, this key is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| OTPK_SM_DP+_AKA | One-time Public Key generated by the SM-DP+ for ECKA. Depending on the eUICC configuration, this key is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| PPK_ENC | Random PPK-ENC value (16 bytes key length). This value is different from <S_ENC> value. |
| PPK_INIT_MAC | Random initial MAC chaining value (16 bytes). This value is different from the <S_MAC_CHAIN> value. |
| PPK_MAC | Random PPK-MAC value (16 bytes key length). This value is different from <S_MAC> value. |
| PPP_OP_PROF1_SEG_PPK | An element of sequenceOf86, consisting of a <UPP_OP_PROF1_SEG> protected with <PPK_ENC> and <PPK_MAC> and encapsulated in a TLV with tag 0x86 length <L>, up to a maximum size of 1020 bytes including the tag and length field. |
| PPP_OP_PROF1_SEG_SK | An element of sequenceOf86, consisting of a <UPP_OP_PROF1_SEG> segment protected with <S_ENC> and <S_MAC> and encapsulated in a TLV with tag 0x86, length <L>, up to a maximum size of 1020 bytes including the tag and length field. |
| PPP_OP_PROF1_SEG_SK_INV | <PPP_OP_PROF1_SEG_SK> modified (wrong encryption) |
| PPR_IDS | Forbidden Profile Policy Rules. This PPR list MAY be empty or MAY contain either PPR1 or PPR2 or both. |
| PROPRIETARY_DATA | Proprietary Data returned by the eUICC as part of FCI template |
| R_APDU_PARTx | Sub-part of a R-APDU (see Annex D.4.2) |
| RANDOM_SM_DP+_SIGN | Random SM-DP+ signature (i.e. content of the tag 0x5F37) with a size corresponding to a valid one. |
| RANDOM_SM_DS_SIGN | Random SM-DS signature (i.e. content of the tag 0x5F37) with a size corresponding to a valid one. |
| REPLACE_S_KEYS_REQ_ENC | An element of secondSequenceOf87, consisting of #REPLACE_S_KEYS_REQ protected with <S_ENC> and <S_MAC> and encapsulated in a TLV with tag 0x87, up to a maximum size of 1020 bytes including the tag and length field. |
| REASON_CODE_ANY | Any Reason Code, as defined in SGP.22 [2] – section 5.2.6.2 |
| RSP_SERVER_ADDRESS | RSP Server address in FQDN format where the operation corresponding to the Event can be processed. |
| S_ENC | SCP03T Encryption Session key (128 bits length) resulting from the key agreement with eUICC. |
| S_HASHED_CC | Hashed Confirmation Code generated by the LPA. When generated by the S_LPAd, the S_LPAd SHALL use #CONFIRMATION_CODE1 in the calculation unless otherwise specified. |
| S_HASHED_CC_ERROR | A random generated hash value of the Confirmation Code not equal to S_HASHED_CC. |
| S_INIT_MAC | SCP03T Initial MAC chaining value (128 bits length) resulting from the key agreement with eUICC. |
| S_MAC | SCP03T MACing Session key (128 bits length) resulting from the key agreement with eUICC. |
| S_MAC_CHAIN | Current MAC chaining value used for SCP03t BPP protection. |
| S_SEL_TLS_CIPHER_SUITE | TLS cipher suite selected by the Server set as follows: o TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 if present in <TLS_CIPHER_SUITES>, otherwise o TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256. |
| S_SESSION_ID_SERVER | Random value of the TLS session_id in ServerHello which is different from <SESSION_ID_CLIENT>. This value is non-empty. |
| S_SM_DP+_SIGN | The S_SM-DP+ signature (smdpSign), computed using the #SK_S_SM_DPpb_SIG across the following data objects: - remoteOpId - transactionId - controlRefTemplate - smdpOtpk - euiccOtpk, as provided earlier in the prepareDownloadResponse data object |
| S_SM_DP+_SIGNATURE2 | The ASN.1 OCTET STRING encoded SM-DP+ signature 2 (field smdpSignature2) computed using the private key related to the server certificate (field smdpCertificate) present in the PrepareDownloadRequest structure. This signature SHALL be generated across the following data objects: - smdpSignature2 - euiccSignature1 present in the AuthenticateServerResponse structure |
| S_SMDP_CHALLENGE | The SM-DP+ Challenge (serverChallenge) randomly chosen by the simulated SM-DP+ to be signed later by the eUICC for the eUICC authentication, coded as ASN.1 OCTET STRING of 16 bytes. |
| S_SMDP_SIGNATURE_INV | <S_SMDP_SIGNATURE1> NOT computed with the #SK_S_SM_DPauth_SIG but with the same length as a valid signature |
| S_SMDP_SIGNATURE1 | The ASN.1 OCTET STRING encoded SM-DP+ signature (field serverSignature1) computed using the private key related to the server certificate (field serverCertificate) present in the AuthenticateServerRequest structure. |
| S_SMDP_SIGNED_INV_ADDR | <S_SMDP_SIGNED1> with a different SM-DP+ address (#TEST_DP_ADDRESS2 instead of #TEST_DP_ADDRESS1) |
| S_SMDP_SIGNED1 (ServerSigned1) | { transactionId <S_TRANSACTION_ID>, euiccChallenge <EUICC_CHALLENGE>, serverAddress #TEST_DP_ADDRESS1, serverChallenge <S_SMDP_CHALLENGE> } |
| S_SMDS_CHALLENGE | The SM-DS Challenge (serverChallenge) randomly chosen by the simulated SM-DS to be signed later by the eUICC for the eUICC authentication, coded as ASN.1 OCTET STRING of 16 bytes. |
| S_SMDS_SIGNATURE_INV | <S_SMDS_SIGNATURE1> NOT computed with the #SK_S_SM_DSauth_SIG but with the same length as a valid signature |
| S_SMDS_SIGNATURE1 | The SM-DS signature 1 (serverSignature1) computed using #SK_S_SM_DSauth_SIG across the serverSigned1 present in the AuthenticateServerRequest structure, coded as ASN.1 OCTET STRING |
| S_SMDS_SIGNED_ADDR1 (ServerSigned1) | { transactionId <S_TRANSACTION_ID>, euiccChallenge <EUICC_CHALLENGE>, serverAddress #TEST_DS_ADDRESS1, serverChallenge <S_SMDS_CHALLENGE> } |
| S_SMDS_SIGNED_INV_ADDR | <S_SMDS_SIGNED1> with a different SM-DS address (#TEST_DP_ADDRESS1 instead of #TEST_ROOT_DS_ADDRESS) |
| S_SMDS_SIGNED1 (ServerSigned1) | { transactionId <S_TRANSACTION_ID>, euiccChallenge <EUICC_CHALLENGE>, serverAddress #TEST_ROOT_DS_ADDRESS, serverChallenge <S_SMDS_CHALLENGE> } |
| S_TRANSACTION_ID | The TransactionID (Unique Transaction Identifier) generated by the (S_)SM-DP+, or (S_)SM-DS which is used to uniquely identify the RSP session and to correlate the multiple ESXX request messages that belong to the same RSP session. This value (binary value) can start from 0x01 and can be increased by 1 each time a Profile is downloaded in the eUICC. 1-16 bytes (ASN.1 OCTET STRING). |
| SAH_SHA256_ECDSA | Signature And Hash Algorithm extension sent in the CertificateRequest message set as a minimum of: HashAlgorithm sha256 (04) and SignatureAlgorithm ecdsa (03). |
| SEL_TLS_CIPHER_SUITE | TLS cipher suite selected by the Server |
| SEQ_NUMBER | Sequence Number related to a Notification Metadata generated by the eUICC. |
| SERVER_CHALLENGE | Random value generated by the SM-XX server under test coded as ASN.1 OCTET STRING of 16 bytes which can be one of the following depending on the entity under test: - <SMDP_CHALLENGE> - <SMDS_CHALLENGE> |
| SERVER_CHALLENGE_2 | Random value generated by the SM-XX server under test coded as ASN.1 OCTET STRING of 16 bytes which can be one of the following depending on the entity under test: - <SMDP_CHALLENGE_2> - <SMDS_CHALLENGE_2> |
| SERVER_FINISHED | The first protected message with the negotiated algorithms, keys, and secrets. It is the Hash of the concatenation of all the data from all messages in this handshake up to, but not including, this message i.e. all handshake messages starting at ClientHello up to, but not including, this Finished message itself. NOTE: ChangeCipherSpec messages, alerts, and any other record type are not handshake messages and are not included in the hash computations. Also, HelloRequest messages are omitted from handshake hashes. |
| SERVER_SIGNATURE1 | Server signature (serverSignature1) which can be one of the following depending on the entity under test: - SM-DP+ signature (serverSignature1) generated over #SERVER_SIGNED1 using SK.DPauth.SIG, coded as ASN.1 OCTET STRING - SM-DS signature (serverSignature1) generated over #SERVER_SIGNED1 using SK.DPauth.SIG, coded as ASN.1 OCTET STRING |
| SERVER_SIGNATURE1_2 | SERVER signature (serverSignature1) which can be one of the following depending on the entity under test: - SM-DP signature (serverSignature1) generated over #SERVER_SIGNED1_2 using SK.DPauth.SIG, coded as ASN.1 OCTET STRING - SM-DS signature (serverSignature1) generated over #SERVER_SIGNED1_2 using SK.DPauth.SIG, coded as ASN.1 OCTET STRING |
| SERVER_TLS_EPHEM_KEY | Server's ephemeral key and associated information. |
| SESSION_ID_CLIENT | Random or empty value of the TLS session_id in ClientHello. |
| SESSION_ID_RANDOM | Random value of the TLS session. |
| SHS | Shared Secret resulting from the key agreement with eUICC. |
| SM_DP+_SIGN | The SM-DP+ signature in ES8+/InitialiseSecureChannelRequest/smdpSign. |
| SMDP_CHALLENGE | Random value generated by the SM-DP+ coded as ASN.1 OCTET STRING of 16 bytes. |
| SMDP_CHALLENGE_2 | Random value generated by the SM-DP+ coded as ASN.1 OCTET STRING of 16 bytes. |
| SMDP_CHALLENGE_INVALID | SM-DP+ Challenge randomly generated by the simulated SM-DP+ coded as ASN.1 OCTET STRING of 16 bytes not equal to <SMDP_CHALLENGE>. |
| SMDP_METADATA_SEG_MAC | An element of sequenceOf88, consisting of a segment of maximum size 1008 bytes protected with <S_MAC> and encapsulated in a TLV with tag 0x88, length <L>, up to a maximum size of 1020 bytes including the tag and length field. |
| SMDP_SIGNATURE2 | SM-DP+ signature (smdpSignature2) generated over smdpSigned2 using SK.DPauth.SIG, coded as ASN.1 OCTET STRING |
| SMDS_CHALLENGE | Random value generated by the SM-DS coded as ASN.1 OCTET STRING of 16 bytes. |
| SMDS_CHALLENGE_2 | Random value generated by the SM-DS coded as ASN.1 OCTET STRING of 16 bytes. |
| SMDS_CHALLENGE_INVALID | SM-DS Challenge randomly generated by the simulated SM-DS coded as ASN.1 OCTET STRING of 16 bytes not equal to <SMDS_CHALLENGE>. |
| STORE_DATA_BLOCK_NUM | The STORE DATA block number coded sequentially from 0x00 to 0xFF. If the value 0xFF has been reached and more STORE DATA commands are needed to complete the transfer, the numbering restarts and the next STORE DATA block number is set to 0x00. |
| SUBJECT_CODE_ANY | Any Subject Code, as defined in SGP.22 [2] – section 5.2.6.1 |
| TBS_EUICC_NOTIF_SIG | The eUICC signature generated over tbsOtherNotification. NotificationMetadata, coded as ASN.1 OCTET STRING. |
| TLS_CIPHER_SUITES | TLS cipher suite list supported by LPAd or the Client (SM-DP+ or SM-DS) under test. |
| TRANSACTION_ID_2 | A unique Transaction ID generated by an SM-DP+ or an SM-DS within the scope and lifetime of each SM-DP+ or SM-DS to uniquely identify the ongoing RSP session as OCTET STRING of up to 16 bytes. |
| TRANSACTION_ID_AC | A unique Transaction ID generated by an SM-DP+ or an SM-DS within the scope and lifetime of each SM-DP+ or SM-DS to uniquely identify the ongoing RSP session used by the AuthenticateClient function as OCTET STRING of up to 16 bytes. |
| TRANSACTION_ID_GBPP | A unique Transaction ID generated by an SM-DP+ within the scope and lifetime of each SM-DP+ to uniquely identify the ongoing RSP session used by the GetBoundProfilePackage function as OCTET STRING of up to 16 bytes. |
| TRANSACTION_ID_IA | A unique Transaction ID generated by an SM-DP+ or an SM-DS within the scope and lifetime of each SM-DP+ or an SM-DS to uniquely identify the ongoing RSP session used by the InitiateAuthentication function as OCTET STRING of up to 16 bytes. |
| TRANSACTION_ID_ISC | A unique Transaction ID generated by an SM-DP+ within the scope and lifetime of each SM-DP+ to uniquely identify the ongoing RSP session used by the InitialiseSecureChannelRequest function as OCTET STRING of up to 16 bytes. |
| TRANSACTION_ID_SIGNED | A unique Transaction ID generated by an SM-DP+ or an SM-DS within the scope and lifetime of each SM-DP+ or SM-DS to uniquely identify the ongoing RSP session as OCTET STRING of up to 16 bytes signed as part of #SERVER_SIGNED1 |
| TRANSACTION_ID_SIGNED_2 | A unique Transaction ID generated by an SM-DP+ or an SM-DS within the scope and lifetime of each SM-DP+ or SM-DS to uniquely identify the ongoing RSP session as OCTET STRING of up to 16 bytes signed as part of #SERVER_SIGNED1 |
| TRANSACTION_ID_SIGNED_AC | A unique Transaction ID generated by an SM-DP+ or an SM-DS within the scope and lifetime of each SM-DP+ or SM-DS to uniquely identify the ongoing RSP session used by the AuthenticateClient function as OCTET STRING of up to 16 bytes. |
| TRANSACTION_ID_SIGNED_IA | A unique Transaction ID generated by an SM-DP+ or an SM-SD within the scope and lifetime of each SM-DP+ or SM-DS to uniquely identify the ongoing RSP session used by the InitiateAuthentication function as OCTET STRING of up to 16 bytes. |
| TRE_PROPERTIES | The value of the treProperties field in EUICCInfo2. |
| TRE_REFERENCE | The value of the treProductReference field in EUICCInfo2. |
| UPP_OP_PROF1_SEG | A segment of the #UPP_OP_PROF1, with a maximum size of 1007 bytes. |
| UPP_OP_PROF2_SEG | A segment of the #UPP_OP_PROF2, with a maximum size of 1007 bytes. |

- Annex C	Methods And Procedures

This section describes methods and procedures used in the interfaces compliance test cases. They are part of test cases and SHALL not be executed in standalone mode.

- C.1	Methods

If the method is used in the “expected result” column, all parameters SHALL be verified by the simulated entity (test tool). If the method is used in the “Sequence / Description” column, the command SHALL be generated by the simulated entity.

| Method | MTD_AUTHENTICATE_CLIENT |
| --- | --- |
| Description | Generates or verifies the JSON formatted AuthenticateClient request |
| Parameter(s) | - paramTransactionId: random 16 byte identifier encoded as String Hexadecimal. - paramAuthenticateServerResponse: server authentication response structured as ASN.1 encoded as base 64. |
| Details | JSON body { "transactionId" : paramTransactionId, "authenticateServerResponse" : paramAuthenticateServerResponse } |

| Method | MTD_CANCEL_ORDER |
| --- | --- |
| Description | Sends and checks the JSON formatted CancelOrder request |
| Parameter(s) | -  paramFunctionRequesterId -  paramFunctionCallId -  paramIccid: identification of the targeted profile (mandatory) -  paramEID: EID of the targeted eUICC (conditional) -  paramMatchingId: matching ID generated by the Operator (conditional) -  paramProfileStatus: final Profile status indicator (mandatory) |
| Details | JSON requestHeader { "header" : { "functionRequesterIdentifier" : "paramFunctionRequesterId", "functionCallIdentifier" : "paramFunctionCallId" } JSON body { "iccid" : paramIccid "eid" : paramEID, "matchingId" : paramMatchingId "profileStatus" : paramProfileStatus } } Note: if some of the value of the parameters above are not provided, those parameters are not included as part of JSON body |

| Method | MTD_CANCEL_SESSION |
| --- | --- |
| Description | Sends or verifies the JSON formatted CancelSession request |
| Parameter(s) |  paramTransactionId: random 16 byte identifier.  paramCancelSessionResponse: eUICC information structured as ASN.1 encoded as base 64. |
| Details | JSON body { "transactionId" : paramTransactionId, "cancelSessionResponse" : paramCancelSessionResponse } |

| Method | MTD_CHECK_SMS_POR |
| --- | --- |
| Description | Check the content of the SMS POR containing the response of the ES6.UpdateMetadata request |
| Parameter(s) | paramExpectedSW: the expected Status Word of the last STORE DATA command |
| Details | Parse and retrieve the SCP80 response packet from the SMS. SCP80 response status code SHALL be equal to 0x00 – POR OK. The additional data from the response packet SHALL be formatted as an expanded structure with definite length as defined in ETSI TS 102 226 [14] and contains the following TLV: AB <L> 80 <L> <NB_EXECUTED_C_APDUS> -- Number of executed C-APDUs 23 <L> 00 90 00 –- R-APDU of the INSTALL FOR PERSONALIZATION command 23 <L> paramExpectedSW –- SW of the last STORE DATA command executed <NB_EXECUTED_C_APDUS> SHALL be equal to the number of executed C-APDUs (i.e. one INSTALL FOR PERSONALIZATION + n STORE DATA command(s)) |

| Method | MTD_ CONFIRM_ORDER |
| --- | --- |
| Description | Sends and checks the JSON formatted ConfirmOrder request |
| Parameter(s) | -  paramFunctionRequesterId -  paramFunctionCallId -  paramIccid: identification of the targeted profile (mandatory) -  paramEID: EID of the targeted eUICC (conditional) -  paramMatchingId: matching ID generated by the Operator (optional) -  paramConfirmationCode: confirmation code provided by the Operator (optional) -  paramSmdsAddress: Alternative SM-DS to be used for event registration (conditional) -  paramRootSmdsAddress: Root SM-DS to be used for event registration (conditional) -  paramReleaseFlag: boolean indicating if the profile shall be released (mandatory) |
| Details | JSON requestHeader { "header" : { "functionRequesterIdentifier" : "paramFunctionRequesterId", "functionCallIdentifier" : "paramFunctionCallId" } JSON body { "iccid" : paramIccid "eid" : paramEID, "matchingId" : paramMatchingId "confirmationCode" : paramConfirmationCode "smdsAddress" : paramSmdsAddress "releaseFlag" : paramReleaseFlag } } Note: if some of the value of the parameters above are not provided, those parameters are not included as part of JSON body |

| Method | MTD_DELETE_EVENT |
| --- | --- |
| Description | Sends and checks the JSON formatted DeleteEvent request |
| Parameter(s) | -  paramFunctionRequesterId: identification of the function requester. -  paramFunctionCallId: identification of the function call. -  paramEID: EID of the targeted eUICC -  paramEventId: unique Identification of the Event to be registered |
| Details | JSON requestHeader { "header" : { "functionRequesterIdentifier" : "paramFunctionRequesterId", "functionCallIdentifier" : "paramFunctionCallId" } JSON body { "eid" : paramEID, "eventId" : paramEventId } } |

| Method | MTD_DISABLE_PROFILE |
| --- | --- |
| Description | Generate the ASN.1 DisableProfileRequest structure according to the input parameters. |
| Parameter(s) | -  paramIccidValue: The ICCID of the Profile to Disable (optional) -  paramIsdpAidValue: The ISD-P AID of the Profile to Disable (optional) -  paramRefreshFlag: Boolean, TRUE if refreshFlag SHALL be set, FALSE otherwise Either paramIccidValue or paramIsdpAidValue is passed as a parameter. |
| Details | IF paramIccidValue is provided Then req DisableProfileRequest::= { profileIdentifier iccid : paramIccidValue, refreshFlag paramRefreshFlag } Else req DisableProfileRequest::= { profileIdentifier isdpAid : paramIsdpAidValue, refreshFlag paramRefreshFlag } End if |

| Method | MTD_ DOWNLOAD_ORDER |
| --- | --- |
| Description | Sends and checks the JSON formatted DownloadOrder request |
| Parameter(s) | -  paramFunctionRequesterId -  paramFunctionCallId -  paramEID: EID of the targeted eUICC (optional) -  paramIccid: identification of the targeted profile (conditional) -  paramProfileType: identification of the targeted profile type (conditional) |
| Details | JSON requestHeader { "header" : { "functionRequesterIdentifier" :"paramFunctionRequesterId", "functionCallIdentifier" :"paramFunctionCallId" } JSON body { "eid" : paramEID, "iccid" : paramIccid "profileType" : paramProfileType } } Note: if some of the value of the parameters above are not provided, those parameters are not included as part of JSON body |

| Method | MTD_ENABLE_PROFILE |
| --- | --- |
| Description | Generate the ASN.1 EnableProfileRequest structure according to the input parameters. |
| Parameter(s) | -  paramIccidValue: The ICCID of the Profile to Disable (optional) -  paramIsdpAidValue: The ISD-P AID of the Profile to Disable (optional) -  paramRefreshFlag: Boolean, TRUE if refreshFlag SHALL be set, FALSE otherwise Either paramIccidValue or paramIsdpAidValue is passed as a parameter. |
| Details | IF paramIccidValue is provided Then req EnableProfileRequest ::= { profileIdentifier iccid : paramIccidValue, refreshFlag paramRefreshFlag } Else req EnableProfileRequest ::= { profileIdentifier isdpAid : paramIsdpAidValue, refreshFlag paramRefreshFlag } End if |

| Method | MTD_DELETE_PROFILE |
| --- | --- |
| Description | Generate the ASN.1 DeleteProfileRequest structure according to the input parameters. |
| Parameter(s) | -  paramIccidValue: The ICCID of the Profile to Delete (optional) -  paramIsdpAidValue: The ISD-P AID of the Profile to Delete (optional) Either paramIccidValue or paramIsdpAidValue is passed as a parameter. |
| Details | IF paramIccidValue is provided Then req DeleteProfileRequest ::= iccid : paramIccidValue Else req DeleteProfileRequest ::= isdpAid : paramIsdpAidValue End if |

| Method | MTD_GET_PROFILE_INFO |
| --- | --- |
| Description | Generate the ASN.1 ProfileInfoListRequest according to the input parameters. |
| Parameter(s) |  paramIccidValue: The ICCID of the Profile  paramIsdpAidValue: The ISD-P AID of the Profile Either paramIccidValue or paramIsdpAidValue is passed as a parameter. |
| Details | IF paramIccidValue is provided Then req ProfileInfoListRequest::= { searchCriteria iccid: paramIccidValue } Else req ProfileInfoListRequest::= { searchCriteria isdpAid: paramIsdpAidValue } End If |

| Method | MTD_GENERATE_BPP |
| --- | --- |
| Description | Generate a BPP according to the input parameters. |
| Parameter(s) |  paramInitSC: The InitialiseSecureChannel request  paramConfISDP: The ConfigureISDP request (plain)  paramStoreMetadata: The StoreMetadata request (plain)  paramReplaceSessionKeys: The ReplaceSessionKeys request (plain) – Optional parameter  paramUPP: The Unprotected Profile Package to download |
| Details | Split the paramStoreMetadata in several segments of maximum 1008 bytes. Each Metadata segment is named <METADATA_SEG> here after. Split the paramUPP in several segments of maximum 1007 bytes. Each UPP segment named <UPP_SEG> here after. Create the following structure of data: req BoundProfilePackage ::= { paramInitSC, firstSequenceOf87 { 0x87 <L> paramConfISDP }, sequenceOf88 { 0x88 <L> <METADATA_SEG>, … 0x88 <L> <METADATA_SEG> }, -- secondSequenceOf87 SHALL be set only if paramReplaceSessionKeys is -- provided secondSequenceOf87 { 0x87 <L> paramReplaceSessionKeys }, sequenceOf86 { 0x86 <L> <UPP_SEG>, … 0x86 <L> <UPP_SEG> } } Use <OT_SK_S_SM_DP+_AKA> and <OTPK_EUICC_AKA> in order to generate the <SHS>. Concatenate #KEY_TYPE, #KEY_LENGTH, <L> #HOST_ID and <L> #EID1 as SharedInfo. Retrieve <S_ENC>, <S_MAC> and <S_INIT_MAC> across SHA-256 calculated from <SHS> and SharedInfo. Encrypt paramConfISDP with <S_ENC>. Calculate and add a MAC to the tag 0x87 of firstSequenceOf87 by using <S_MAC>. Calculate and add a MAC to all tags 0x88 of sequenceOf88 by using <S_MAC>. If paramReplaceSessionKeys is provided Then Encrypt paramReplaceSessionKeys with <S_ENC> Calculate and add a MAC to the tag 0x87 of secondSequenceOf87 by using <S_MAC>. End If Encrypt all <UPP_SEG> with <S_ENC>, or <PPK_ENC> if paramReplaceSessionKeys is provided. Calculate and add a MAC to all tags 0x86 of sequenceOf86 by using <S_MAC>, or <PPK_MAC> (and <PPK_INIT_MAC> for the first tag) if paramReplaceSessionKeys is provided. |

| Method | MTD_GENERATE_HASHED_CC |
| --- | --- |
| Description | Generate an Hashed Confirmation Code based on the Confirmation Code and the Transaction ID given in parameter. |
| Parameter(s) |  paramConfirmationCode: The Confirmation Code (plain)  paramTransactionId: The Transaction ID (plain) |
| Details | Generate a SHA-256 of the paramConfirmationCode. Concatenate the obtained hash value with the paramTransactionId. Generate and return a SHA-256 of these two concatenated elements. |

| Method | MTD_GET_BPP |
| --- | --- |
| Description | Generates or verifies the JSON formatted GetBoundProfilePackage request |
| Parameter(s) |  paramTransactionId: random 16 byte identifier.  paramPrepareDownloadResponse structured as ASN.1 encoded as base 64. |
| Details | JSON body { "transactionId" : paramTransactionId, "prepareDownloadResponse" : paramPrepareDownloadResponse } |

| Method | MTD_HANDLE_NOTIF |
| --- | --- |
| Description | Generates or verifies the JSON formatted HandleNotification request |
| Parameter(s) | paramPendingNotification: PendingNotification data object |
| Details | JSON body { "pendingNotification" : paramPendingNotification } |

| Method | MTD_HTTP_REQ |
| --- | --- |
| Description | Sends or verifies a secured HTTP request message delivering a JSON object payload using a network to an off-card entity. |
| Parameter(s) | - paramServerAddress: Target Server address - paramFunctionPath: Function path - paramRequestMessage: JSON Request message |
| Details | HTTP POST paramFunctionPath HTTP/1.1 Host: paramServerAddress User-Agent: See NOTE 1 X-Admin-Protocol:gsma/rsp/v#RSP_SVN Content-Type:application/json OR application/json;charset=UTF-8 (see NOTE 2) Content-Length: <L> paramRequestMessage NOTE 1: If the request is sent by the LPAd, the User-Agent SHALL be gsma-rsp-lpad. The "User-Agent" field may contain additional information after a semicolon. Otherwise the value of User-Agent is not specified by the current document. The additional information shall not be checked. NOTE 2: the Content-Type checking is relaxed in this specification, in order to allow for common internet usage of “charset=UTF-8” and for compatibility with SGP.22 v3.0. If the request is sent by the entity under test, both values are acceptable (where linear white space as specified in RFC 2616 is allowed after the semi-colon). Further, all parts of these allowed Content-Type value SHALL be checked in a case-insensitive manner, as per RFC 2616. If the request is sent by a simulator, application/json shall be used.The HTTP POST request may contain additional header fields. These shall not be checked. |

| Method | MTD_HTTP_RESP |
| --- | --- |
| Description | Sends or verifies a secured HTTP response message delivering a JSON object payload using a network to an off-card entity. |
| Parameter(s) |  paramResponseMessage: JSON Response message |
| Details | HTTP/1.1 200 (OK) X-Admin-Protocol: gsma/rsp/v#RSP_SVN Content-Type: application/json OR application/json;charset=UTF-8 (see NOTE) Content-Length: <L> paramResponseMessage NOTE: the Content-Type checking is relaxed in this specification, in order to allow for common internet usage of “charset=UTF-8” and for compatibility with SGP.22 v3.0 If the response is sent by the entity under test, both values are acceptable (where linear white space as specified in RFC 2616 is allowed after the semi-colon). Further, all parts of these allowed Content-Type value SHALL be checked in a case-insensitive manner, as per RFC 2616. If the response is sent by a simulator, application/json shall be used. The HTTP response may contain additional header fields. These shall not be checked. |

| Method | MTD_INITIATE_AUTHENTICATION |
| --- | --- |
| Description | Generates or verifies the JSON formatted Initiate Authentication request on ES9+ or ES11 as applicable. |
| Parameter(s) | - paramEUICCChallenge: random 16 byte challenge coded as base 64 - paramEUICCInfo1: eUICC information structured coded as base 64 - paramServerAddress: FQDN of the Server - paramLpaRspCapability: LPA_RSP_CAPABILITY coded as base 64 |
| Details | JSON body { "euiccChallenge" : paramEUICCChallenge, "euiccInfo1" : paramEUICCInfo1, "smdpAddress" : paramServerAddress, "lpaRspCapability " : paramLpaRspCapability } |

| Method | MTD_REGISTER_EVENT |
| --- | --- |
| Description | Send or checks the JSON formatted RegisterEvent request |
| Parameter(s) | -  paramFunctionRequesterId: identification of the function requester. -  paramFunctionCallId: identification of the function call. -  paramEID: EID of the targeted eUICC -  paramRspServerAddress: Address of the Server sending the RegisterEvent formatted as FQDN -  paramEventId: unique Identification of the Event to be registered -  paramForwardingIndicator: TRUE if registration has to be made to the Root SM-DS; FALSE if this is not to be made to the Root SM-DS |
| Details | JSON requestHeader { "header" : { "functionRequesterIdentifier" : "paramFunctionRequesterId", "functionCallIdentifier" : "paramFunctionCallId" } JSON body { "eid" : paramEID, "rspServerAddress" : paramRspServerAddress, "eventId" : paramEventId, "forwardingIndicator" : paramForwardingIndicator } } |

| Method | MTD_REGISTER_EVENT_HASHED_ICCID |
| --- | --- |
| Description | Send or checks the JSON formatted RegisterEvent request. |
| Parameter(s) | -  paramFunctionRequesterId: identification of the function requester. -  paramFunctionCallId: identification of the function call. -  paramEID: EID of the targeted eUICC. -  paramRspServerAddress: Address of the Server sending the RegisterEvent formatted as FQDN. -  paramEventId: unique Identification of the Event to be registered. -  paramForwardingIndicator: TRUE if registration has to be made to the Root SM-DS; FALSE if this is not to be made to the Root SM-DS. -  paramHashedIccid: SHA256 hash of ICCID of the Profile associated with this Event Record. |
| Details | JSON requestHeader { "header" : { "functionRequesterIdentifier" : "paramFunctionRequesterId", "functionCallIdentifier" : "paramFunctionCallId" } JSON body { "eid" : paramEID, "rspServerAddress" : paramRspServerAddress, "eventId" : paramEventId, "forwardingIndicator" : paramForwardingIndicator, "hashedIccids" : paramHashedIccid } } |

| Method | MTD_REGISTER_EVENT_HASHED_SALTED_ICCID |
| --- | --- |
| Description | Send or checks the JSON formatted RegisterEvent request. |
| Parameter(s) | -  paramFunctionRequesterId: identification of the function requester. -  paramFunctionCallId: identification of the function call. -  paramEID: EID of the targeted eUICC. -  paramRspServerAddress: Address of the Server sending the RegisterEvent formatted as FQDN. -  paramEventId: unique Identification of the Event to be registered. -  paramForwardingIndicator: TRUE if registration has to be made to the Root SM-DS; FALSE if this is not to be made to the Root SM-DS. -  paramHashedSaltedIccid: SHA256 hash of ICCID of the Profile and Salt associated with this Event Record. -  paramSalt: Salt associated with this Event Record. |
| Details | JSON requestHeader { "header" : { "functionRequesterIdentifier" : "paramFunctionRequesterId", "functionCallIdentifier" : "paramFunctionCallId" } JSON body { "eid" : paramEID, "rspServerAddress" : paramRspServerAddress, "eventId" : paramEventId, "forwardingIndicator" : paramForwardingIndicator, "hashedSaltedIccids" : paramHashedSaltedIccid, "salt" : paramSalt } } |

| Method | MTD_REGISTER_EVENT_V3 |
| --- | --- |
| Description | Send or checks the JSON formatted RegisterEvent request/ |
| Parameter(s) | paramFunctionRequesterId: identification of the function requester. paramFunctionCallId: identification of the function call. paramEID: EID of the targeted eUICC. paramRspServerAddress: Address of the Server sending the RegisterEvent formatted as FQDN. paramEventId: unique Identification of the Event to be registered. paramForwardingIndicator: TRUE if registration has to be made to the Root SM-DS; FALSE if this is not to be made to the Root SM-DS. paramRootSmdsAddress: to be used when paramForwardingIndicator is TRUE (conditional) paramEventType: Type of the RSP Operation associated with the Event. paramHashedIccids: Identification of the target Profile(s) (optional). paramSalt: Random string to be concatenated with ICCID(s) for hashing (conditional). paramServiceProviderName: Name of the Service Provider that requested the RSP Operation (optional). paramOperatorId: Identification of the Operator that requested the RSP Operation (optional). |
| Details | JSON requestHeader { "header" : { "functionRequesterIdentifier" : "paramFunctionRequesterId", "functionCallIdentifier" : "paramFunctionCallId" } JSON body { "eid" : paramEID, "rspServerAddress" : paramRspServerAddress, "eventId" : paramEventId, "forwardingIndicator" : paramForwardingIndicator "rootSmdsAddress” : paramRootSmdsAddress "eventType” : paramEventType "hashedIccids” : paramHashedIccids "salt” : paramSalt "serviceProviderName” : paramServiceProviderName "operatorId” : paramOperatorId } } |

| Method | MTD_REMOVE_NOTIF |
| --- | --- |
| Description | Constructs the command data for RemoveNotificationFromList |
| Parameter(s) | -  paramSeqNumber: the sequence number to be removed |
| Details | request NotificationSentRequest ::= { seqNumber paramSeqNumber } |

| Method | MTD_RETRIEVE_NOTIF_SEQ_NUM |
| --- | --- |
| Description | Constructs the command data for RetrieveNotificationsList filtered by sequence number |
| Parameter(s) | -  paramSeqNumber: the sequence number to be retrieved |
| Details | request RetrieveNotificationsListRequest ::= { searchCriteria seqNumber paramSeqNumber } |

| Method | MTD_SELECT |
| --- | --- |
| Description | Generates the SELECT command as defined in GlobalPlatform Card Specification [6]. |
| Parameter(s) | -  paramAID: the AID to select |
| Details | - CLA = 0x or 4x (x = <CHANNEL_NUMBER>) - INS = A4 - P1 = 04 - P2 = 00 - LC = <L> - paramAID - LE = 00 |

| Method | MTD_SEND_SMS_PP |
| --- | --- |
| Description | Generate and send an envelope SMS-PP download to the MNO-SD |
| Parameter(s) |  paramApdusList: the list of APDUs (plain) to send |
| Details | Generate and send the following envelope: 80 C2 00 00 <L> D1 <L> 02 02 83 81 -- Device identity Tag 06 07 91 33 86 09 40 00 F0 -- Address Tag (TON/NPI/..) 0B <L> -- SMS TPDU 44 -- SMS-DELIVER 05 85 02 13 F2 -- TP-Originating-Address 7F -- TP-Protocol-Identifier F6 -- TP-Data-Coding-Scheme 71 30 12 41 55 74 40 -- TP-Service-Centre-Time-Stamp <L> -- TP-User-Data-Length 02 -- User-Data-Header-Length 70 -- IEIa 00 -- IEIDLa <L> -- Command Packet Length (2 bytes) <L> -- Command Header Length (1 byte) 12 21 -- SPI 00 -- KIC 15 –- KID (SCP80 Keyset version 0x01 in Triple DES) B2 01 00 –- MNO-SD TAR <MNO_SCP80_COUNTER> 00 -- Padding Counter <CC> -- Cryptographic checksum <C_APDUS_SCRIPT> -- Command APDUs script <C_APDUS_SCRIPT> SHALL contain the paramApdusList (i.e. each APDU is named <APDU1>; <APDU2>; …; <APDUn> here after) formatted as an expanded structure with definite length as defined in ETSI TS 102 226 [14]: AA <L> 22 <L> <APDU1> 22 <L> <APDU2> … 22 <L> <APDUn> The Cryptographic checksum <CC> SHALL be generated in Triple DES (outer-CBC mode using two different keys) with the #MNO_SCP80_AUTH_KEY as defined in ETSI TS 102 225 [13]. If the command packet length is higher than 140 bytes, it SHALL be sent over several envelopes: SMS concatenation as defined in 3GPP TS 23.040 [22] SHALL be used. |

| Method | MTD_STORE_DATA |
| --- | --- |
| Description | Generates the STORE DATA command (Case 4) as defined in GlobalPlatform Card Specification [6]. |
| Parameter(s) | -  paramCommandData: the command data |
| Details | - CLA = 8x or Cx (x = <CHANNEL_NUMBER>) - INS = E2 - P1 = 91 - P2 = 00 - LC = <L> - paramCommandData - LE = 00 |

| Method | MTD_STORE_DATA_Case3 |
| --- | --- |
| Description | Generates the STORE DATA command (Case3) as defined in GlobalPlatform Card Specification [6]. |
| Parameter(s) | -  paramCommandData: the command data |
| Details | - CLA = 8x or Cx (x = <CHANNEL_NUMBER>) - INS = E2 - P1 = 90 - P2 = 00 - LC = <L> - paramCommandData |

| Method | MTD_STORE_DATA_SCRIPT |
| --- | --- |
| Description | Generate (multiple) STORE DATA command(s) by breaking the data into smaller components (if needed) for transmission. |
| Parameter(s) | -  paramTLVDataToTransmit: TLVs array or single TLV to transfer to the eUICC -  paramCase4Command (optional parameter, default value = TRUE): TRUE if the APDU is a Case 4 command, FALSE if the APDU is a Case 3 command |
| Details | For each element of paramTLVDataToTransmit If the size of the element is greater than 255 bytes, split the element in several blocks of 255 bytes. The last block MAY be shorter. Each block is named <DATA_SUB_PART> here after. If the element is up to 255 bytes, <DATA_SUB_PART> contains the value of the element. The bit b1 of P1 in the STORE DATA commands is named <B1_P1> here after and is defined as below: If paramCase4Command = TRUE Then <B1_P1> = 1 Else <B1_P1> = 0 End If Set <STORE_DATA_BLOCK_NUM> to 0 For each <DATA_SUB_PART> If <DATA_SUB_PART> is an intermediate part, generate the following STORE DATA: - CLA = 8x or Cx (x = <CHANNEL_NUMBER>) - INS = E2 - P1 = 1x (x = <B1_P1>) - P2 = <STORE_DATA_BLOCK_NUM> - LC = <L> - <DATA_SUB_PART> - LE = 00 –- present only if paramCase4Command = TRUE If <DATA_SUB_PART> is the last part, generate the following STORE DATA: - CLA = 8x or Cx (x = <CHANNEL_NUMBER>) - INS = E2 - P1 = 9x (x = <B1_P1>) - P2 = <STORE_DATA_BLOCK_NUM> - LC = <L> - <DATA_SUB_PART> - LE = 00 –- present only if paramCase4Command = TRUE Increase the <STORE_DATA_BLOCK_NUM> by 1 End End |

| Method | MTD_TEST_ES8+_GET_BPP_PPK |
| --- | --- |
| Description | Tests the received boundProfilePackage element according to #R_GET_BPP_RESP_OP1_PPK |
| Parameter(s) | -  paramResponse the response to GetBoundProfilePackage -  paramS_MAC the 128 bit SCP03t MACing Session key -  paramS_ENC the 128 bit SCP03t Encryption Session key -  paramPPK_MAC the 128 bit Profile Protection MACing Key -  paramPPK_ENC the 128 bit Profile Protection Encryption Key - paramMetaData the ASN.1 StoreMetadataRequest element associated to a RSP profile |
| Details | Parse paramResponse into #R_GET_BPP_RESP_OP1_PPK and perform the following tests: -  Verify that each element in firstSequenceOf87, sequenceOf88, secondSequenceOf87 and sequenceOf86 has a total length (including tag and length fields) of 1020 or less -  Verify the integrity of each element in firstSequenceOf87, sequenceOf88 and secondSequenceOf87 using paramS_MAC -  Verify that <TRANSACTION_ID_ISC> in #INIT_SC_PROF1 matches <S_TRANSACTION_ID> -  Verify the validity of smdpSign <SM_DP+_SIGN> in #INIT_SC_PROF1 using #PK_SM_DPpb_SIG -  Retrieve #CONF_ISDP_PROF1_SMDP from <CONF_ISDP_PROF1_ENC> using paramS_ENC and validate the content of #CONF_ISDP_PROF1_SMDP -  Construct the complete metadata element from the <SMDP_METADATA_SEG_MAC> segment(s) and verify that it matches paramMetaData -  Retrieve #REPLACE_S_KEYS_REQ from <REPLACE_S_KEYS_REQ_ENC> using paramS_ENC and validate the content of #REPLACE_S_KEYS_REQ -  Verify that the lengths of paramPPK_ENC and paramPPK_MAC in #REPLACE_S_KEYS_REQ are each 16 bytes -  Verify the integrity of each <PPP_OP_PROF1_SEG_PPK> element using paramPPK_MAC -  Retrieve the <UPP_OP_PROF1_SEG> segment(s) from the <PPP_OP_PROF1_SEG_PPK> segment(s) using paramPPK_ENC, construct the complete Profile from the <UPP_OP_PROF1_SEG> segment(s), then verify that the complete Profile matches #UPP_OP_PROF1 |

| Method | MTD_TEST_ES8+_GET_BPP_SK |
| --- | --- |
| Description | Tests the received boundProfilePackage element according to #R_GET_BPP_RESP_OP1_SK |
| Parameter(s) | -  paramResponse the response to GetBoundProfilePackage -  paramS_MAC the 128 bit SCP03t MACing Session key -  paramS_ENC the 128 bit SCP03t Encryption Session key - paramMetaData the ASN.1 StoreMetadataRequest element associated to a RSP profile |
| Details | Parse paramResponse into #R_GET_BPP_RESP_OP1_SK and perform the following tests: -  Verify that each element in firstSequenceOf87, sequenceOf88 and sequenceOf86 has a total length (including tag and length fields) of 1020 or less -  Verify the integrity of each element in firstSequenceOf87, sequenceOf88 and sequenceOf86 using paramSMAC -  Verify that <TRANSACTION_ID_ISC> in #INIT_SC_PROF1 matches <S_TRANSACTION_ID> -  Verify the validity of smdpSign <SM_DP+_SIGN> in #INIT_SC_PROF1 using #PK_SM_DPpb_SIG -  Retrieve #CONF_ISDP_PROF1_SMDP from <CONF_ISDP_PROF1_ENC> using paramS_ENC and validate the content of #CONF_ISDP_PROF1_SMDP -  Construct the complete metadata element from the <SMDP_METADATA_SEG_MAC> segment(s) and verify that it matches paramMetaData -  Retrieve the <UPP_OP_PROF1_SEG> segment(s) from the <PPP_OP_PROF1_SEG_SK> segment(s) using paramS_ENC, then construct the complete Profile from the <UPP_OP_PROF1_SEG> segment(s), then verify that the complete Profile matches #UPP_OP_PROF1 |

| Method | MTD_TLS_CLIENT_KEY_EXCH_ETC |
| --- | --- |
| Description | Finalizes the Transport Layer Security (TLS) handshake in Server authentication mode on ES9+, or ES11 (Client side). |
| Parameter(s) | -  paramClientKeyExchange: ClientKeyExchange message |
| Details | Sends the session key information in TLS ClientKeyExchange message, ChangeCipherSpec and Finished message. |

| Method | MTD_TLS_CLIENT_HELLO |
| --- | --- |
| Description | Sends or checks the Client Hello message used to initiate the Transport Layer Security (TLS) handshake in Server authentication or Mutual authentication mode on ES9+, ES11, ES12 or ES15. |
| Parameter(s) | -  paramTLSversion: TLS protocol version -  paramAlgs: cipher suite types supported -  paramSessionID: Session ID -  paramExts: Extensions data for “supported_signature_algorithms”, “trusted_ca_keys” or other (optional) |
| Details | Sends or receives a TLS ClientHello message according to the parameters defined above. In addition the following parameters will be set: -  The list of compression algorithms supported by the client is not explicitly defined, but by default it will be set to NULL. -  The random of 4 bytes representing time since epoch on client host and 28 random bytes is not explicitly defined but it SHALL be generated by the test tool TLS implementation NOTE: The Supported Elliptic Curves Extension and the Supported Point Formats Extension extensions MAY be sent by the Client. |

| Method | MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH |
| --- | --- |
| Description | Sends or checks the messages to finalize the Transport Layer Security (TLS) handshake in Mutual authentication mode on ES12 or ES15 (Client side). |
| Parameter(s) | -  paramClientCertificate: TLS Client certificate for authentication used in the Client Certificate Message -  paramClientKeyExchange: The Client TLS Ephemeral Key used in the ClientKeyExchange message |
| Details | Sends the TLS Client Certificate, ClientKeyExchange, Certificate Verify, ChangeCipherSpec and Finished message in this order according to the parameters defined above. NOTE 1: The CertificateVerify Message is not explicitly defined in this method but the CLIENT or test tool implementation SHALL be responsible for generating this message. It is the signature of the concatenation of all the data from all messages in this handshake up to, but not including, this message i.e. all handshake messages starting at ClientHello up to, but not including, this message itself using the specified Signature and Hash Algorithm. NOTE 2: ChangeCipherSpec messages, alerts, and any other record type are not handshake messages and are not included in the signature computations. |

| Method | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC |
| --- | --- |
| Description | Sends or checks the replies to the Client Hello in the Transport Layer Security (TLS) handshake in Mutual authentication mode on ES12 or ES15. |
| Parameter(s) | paramTLSVersion: TLS protocol version used in the Server Hello Message paramAlgs: cipher suite selected used in the Server Hello Message paramSessionID: Session ID used in the Server Hello Message paramServerCertificate: TLS Server certificate for authentication used in the Server Certificate Message paramServerTLSEphemeralKey: TLS Server ephemeral key used in the Server Key Exchange Message paramClientCertificateType: type of certificate requested used in the Client Certificate Request Message paramSignatureAndHashAlgorithm: Signature and Hash Algorithm to be verified used in the Client Certificate Request Message paramDistinguishedName: DN of the CI that signed and issued the certificate used in the Client Certificate Request Message |
| Details | Sends or receives a TLS ServerHello, Server Certificate, ServerKeyExchange, Client Certificate Request and ServerHelloDone message in this order according to the parameters defined above. In addition the following parameter will be received: ServerHello - o The random of 4 bytes representing time since epoch on client host and 28 random bytes is not explicitly defined but it SHALL be generated by the Server under test. - ServerKeyExchange - o The ECParameters are not explicitly defined in the ServerKeyExchange message but it SHALL be generated by the Server under test or the test tool implementation. NOTE: The Supported Elliptic Curves Extension and the Supported Point Formats Extension extensions MAY be sent by the CLIENT therefore this method SHALL respond appropriately when used by the SERVER or the S_SERVER. |

| Method | MTD_TLS_SERVER_END |
| --- | --- |
| Description | Send or checks the finalization of the Transport Layer Security (TLS) handshake in Server or Mutual authentication mode on ES9+,ES11, ES12 or ES15 (Server side). |
| Parameter(s) | -  paramChangeCipherSpec: ChangeCipherSpec message -  paramFinish: Finished message |
| Details | Sends a ChangeCipherSpec and Finished message in this order according to the parameters defined above. |

| Method | MTD_TLS_SERVER_HELLO_ETC |
| --- | --- |
| Description | Send or Receives to the Client Hello in the Transport Layer Security (TLS) handshake in Server authentication mode on ES9+, or ES11. |
| Parameter(s) | -  paramTLSversion: TLS protocol version -  paramAlgs: cipher suite selected -  paramSessionID: Session ID -  paramCertificate: TLS server certificate for authentication -  paramServerTLSEphemeralKey: TLS Server ephemeral key. |
| Details | Sends or Receives a TLS ServerHello, Server Certificate, ServerKeyExchange and ServerHelloDone message in this order according to the parameters defined above. NOTE 1: The random of 4 bytes representing time since epoch on client host and 28 random bytes is not explicitly defined in the Server Hello message but it SHALL be generated by the Server under test. NOTE 2: If no parameter mentioned paramServerTLSEphemeralKey, the value SHALL be set as defined in [24] for ServerKeyExchange. No verification required. |

- C.2	Procedures

|  | Procedure | PROC_ES11_VERIFY_EVENT_RETRIEVAL_EVENT_ID |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication on ES11 from S_LPAd to SM-DS under test supplying <EVENT_ID_R> and verifies that the pending Event #EVENT_ENTRY_1 is retrieved. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_LPAd → SM-DS | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |
| 2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_MATCHING_ID_EVENT_ID_R)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_DS_EVENT_ENTRY_1_OK) |

|  | Procedure | PROC_ES11_VERIFY_EVENT_RETRIEVAL_EVENT_ID_ERROR |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication on ES11 from S_LPAd to SM-DS under test supplying <EVENT_ID_R> and verifies that the pending Event #EVENT_ENTRY_1 is not available. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_LPAd → SM-DS | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |
| 2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_MATCHING_ID_EVENT_ID_R)) | MTD_HTTP_RESP( #R_ERROR_8_9_5_3_9) |

|  | Procedure | PROC_ES11_VERIFY_EVENT_RETRIEVAL_NO_EVENT_ID |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication on ES11 from S_LPAd to SM-DS under test supplying no MatchingId and verifies that the pending Event #EVENT_ENTRY_1 is retrieved. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_LPAd → SM-DS | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |
| 2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_MATCHING_ID_OMITTED)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_DS_EVENT_ENTRY_1_OK) |

|  | Procedure | PROC_ES11_VERIFY_EVENT_RETRIEVAL_NO_EVENT_ID_ERROR |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication on ES11 from S_LPAd to SM-DS under test supplying no MatchingId and verifies that no events are available. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_LPAd → SM-DS | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |
| 2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_MATCHING_ID_OMITTED)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_DS_EVENT_ENTRY_EMPTY_OK) |

|  | Procedure | PROC_ES9+_AUTH_CLIENT |  |
| --- | --- | --- | --- |
|  | Description | Authenticate Server procedure without Confirmation Code. #R_AUTH_SERVER_MATCH_ID_DEV_INFO and #AUTH_SERVER_RESP_ACT_CODE_UC_OK are used with the correct MatchingID defined by the Add Profile initiation procedure (Activation Code content or Empty MatchingID). |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_ACT_CODE_UC_OK)) | MTD_HTTP_RESP(#R_AUTH_CLIENT_OK) |

|  | Procedure | PROC_ES9+_AUTH_CLIENT_CC |  |
| --- | --- | --- | --- |
|  | Description | Authenticate Server procedure (via Activation Code) with Confirmation Code. #R_AUTH_SERVER_MATCH_ID_DEV_INFO and #AUTH_SERVER_RESP_ACT_CODE_UC_OK are used with the correct MatchingID defined by the Add Profile initiation procedure (Activation Code content or Empty MatchingID). |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_ACT_CODE_UC_OK)) | MTD_HTTP_RESP(#R_AUTH_CLIENT_OK_CC) |

|  | Procedure | PROC_ES9+_GET_BPP |  |  |
| --- | --- | --- | --- | --- |
|  | Description | Get BPP procedure without Confirmation Code. |  |  |
| Step | Direction | Sequence / Description | Expected result |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) |  | MTD_HTTP_RESP(#R_GET_BPP_RESP_OP1_SK) |

|  | Procedure | PROC_ES9+_GET_BPP_CC |  |  |
| --- | --- | --- | --- | --- |
|  | Description | Get BPP procedure with Confirmation Code. |  |  |
| Step | Direction | Sequence / Description | Expected result |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_CC)) |  | MTD_HTTP_RESP(#R_GET_BPP_RESP_OP1_SK) |

|  | Procedure | PROC_ES9+_AUTH_CLIENT_FAIL_DEF_DP_USE_CASE_INVALID_MATCHING_ID |  |
| --- | --- | --- | --- |
|  | Description | AuthenticateClient fails due to an Invalid Matching ID. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_ACT_CODE_UC_OK)) | MTD_HTTP_RESP( #R_ERROR_8_2_6_3_8) |

|  | Procedure | PROC_ES9+_PROF_DOWNLOAD_DEF_DP_USE_CASE_CANCEL_SESSION_SK |  |
| --- | --- | --- | --- |
|  | Description | End User cancels ongoing Profile Download after the generation of the one-time ECKA key pair, session keys and the generation of the Bound Profile Package. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK) |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_SK) |
| 5 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_POSTPONED)) | MTD_HTTP_RESP( #R_SUCCESS) Cancel Session request accepted by SM-DP+ and ongoing RSP session SHALL enter retry mode. |

|  | Procedure | PROC_ES9+_PROF_DOWNLOAD_DEF_DP_USE_CASE_CANCEL_SESSION_PPK |  |
| --- | --- | --- | --- |
|  | Description | End User cancels ongoing Profile Download after the generation of the one-time ECKA key pair, session keys, profile protection keys and the generation of the Bound Profile Package. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK) |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_PPK) |
| 5 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_POSTPONED)) | MTD_HTTP_RESP( #R_SUCCESS) Cancel Session request accepted by SM-DP+ and ongoing RSP session SHALL enter retry mode. |

|  | Procedure | PROC_ES9+_PROF_DOWNLOAD_DEF_DP_USE_CASE_CC_CANCEL_SESSION_PPK |  |
| --- | --- | --- | --- |
|  | Description | End User cancels ongoing Profile Download after the generation of the one-time ECKA key pair, session keys, profile protection keys and the generation of the Bound Profile Package when a Confirmation Code is required. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK_CC) |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_CC)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_PPK) |
| 5 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_POSTPONED)) | MTD_HTTP_RESP( #R_SUCCESS) Cancel Session request accepted by SM-DP+ and ongoing RSP session SHALL enter retry mode. |

|  | Procedure | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC_EN |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication and then delivers the Bound Profile Package to the LPAd for enable metadata notifications. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK_EN) |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_PPK) |

|  | Procedure | PROC_ES9+_PROF_DOWNLOAD_ACT_CODE_USE_CASE_CANCEL_SESSION |  |
| --- | --- | --- | --- |
|  | Description | End User cancels ongoing Profile Download after the generation of the one-time ECKA key pair, session keys and the generation of the Bound Profile Package. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_ACT_CODE_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK) |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_PPK) |
| 5 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_POSTPONED)) | MTD_HTTP_RESP( #R_SUCCESS) Cancel Session request accepted by SM-DP+ and ongoing RSP session SHALL enter retry mode. |

|  | Procedure | PROC_ES9+_PROF_DOWNLOAD_SM_DS_USE_CASE_CANCEL_SESSION |  |
| --- | --- | --- | --- |
|  | Description | End User cancels ongoing Profile Download after the generation of the one-time ECKA key pair, session keys and the generation of the bound profile package. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_ES9+_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_SMDS_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK) |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_PPK) |
| 5 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_POSTPONED)) | MTD_HTTP_RESP( #R_SUCCESS) Cancel Session request accepted by SM-DP+ and ongoing RSP session shall enter retry mode. |

|  | Procedure | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication for the Profile Download Default SM_DP+ use case without a confirmation code. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT,MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK) |

|  | Procedure | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_CC |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication for the Profile Download Default SM_DP+ use case with a confirmation code. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK_CC) |

|  | Procedure | PROC_ES9+_INIT_AUTH |  |
| --- | --- | --- | --- |
|  | Description | Initiate Authentication procedure. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |

|  | Procedure | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |
| --- | --- | --- | --- |
|  | Description | Verifies that Common Mutual Authentication for the Profile Download Default SM_DP+ use case without a confirmation code fails due to the profile being in the ‘Installed‘ or ‘Error’ state. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP(#R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_ERROR_8_1_1_3_8) |

|  | Procedure | PROC_VERIFY_SESSION_IS_CANCELLED |  |
| --- | --- | --- | --- |
|  | Description | Verify that the RSP session identified by the TransactionID <S_TRANSACTION_ID> has been cancelled by the eUICC (i.e. Common Mutual Authentication and Profile Download procedures SHALL be rejected as long as no GetEUICCChallenge has been requested). |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_LPAd → eUICC | MTD_STORE_DATA_SCRIPT( #PREP_DOWNLOAD_NO_CC) | #R_PREP_DOWN_NO_SESSION SW=0x9000 The transactionId returned in the response SHALL not be checked (any value SHALL be accepted) |
| 2 | S_LPAd → eUICC | MTD_STORE_DATA_SCRIPT( #AUTHENTICATE_SMDP) | #R_AUTH_SERVER_NO_SESSION SW = 0x9000 The transactionId returned in the response SHALL not be checked (any value SHALL be accepted) |

|  | Procedure | PROC_ES9+_PROF_DOWNLOAD_DEF_DP_USE_CASE_CC_CANCEL_SESSION_SK |  |
| --- | --- | --- | --- |
|  | Description | End User cancels ongoing Profile Download after the generation of the one-time ECKA key pair, session keys and the generation of the Bound Profile Package when a Confirmation Code is required. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK_CC) |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_CC)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_SK) |
| 5 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_POSTPONED)) | MTD_HTTP_RESP( #R_SUCCESS) Cancel Session request accepted by SM-DP+ and ongoing RSP session SHALL enter retry mode. |

|  | Procedure | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_CC_RETRY |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication for the Profile Download Default SM_DP+ use case without a confirmation code. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_RETRY_OK) |

|  | Procedure | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_INVALID_CC |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication for the Profile Download Default SM_DP+ use case with an invalid confirmation code provided in the GetBoundProfilePackage. |  |
| Step | Direction | Sequence / Description | Expected result |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_8_2_7_3_8)) | MTD_HTTP_RESP( #R_ERROR_8_2_7_3_8) |

|  | Procedure | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC_RETRY |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication for the Profile Download Default SM_DP+ use case without a confirmation code. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_RETRY_OK) |

|  | Procedure | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |
| --- | --- | --- | --- |
|  | Description | Establishes the Transport Layer Security (TLS) v1.2 connection between the Client (S_)LPAd and (S_)SERVER using Server authentication mode on ES9+ or ES11. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_LPAd → SERVER | MTD_TLS_CLIENT_HELLO( #TLS_VERSION_1_2, #MIN_TLS_CIPHER_SUITES, #S_SESSION_ID_EMPTY, #S_EXT_SHA256_ECDSA) | MTD_TLS_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_SERVER_TLS) |
| 2 | S_LPAd → SERVER | MTD_TLS_CLIENT_KEY_EXCH_ETC( <CLIENT_TLS_EPHEM_KEY>) | MTD_TLS_SERVER_END( #CHANGE_CIPHER_SPEC, <SERVER_FINISHED>) |

|  | Procedure | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication and then delivers the Bound Profile Package to the LPAd. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK) |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_PPK) |

|  | Procedure | PROC_ES9+_VERIFY_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC |  |
| --- | --- | --- | --- |
|  | Description | Verifies that Common Mutual Authentication occurs successfully and that the Bound Profile Package is generated and successfully delivered to the LPAd. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK) OR MTD_HTTP_RESP( #R_AUTH_CLIENT_RETRY_OK) |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_PPK) |

|  | Procedure | PROC_ES9+_VERIFY_PROFILE _NOT_RELEASED_EMPTY_MID |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication on ES9+ from S_LPAd to SM-DP+ under test supplying an empty MatchingId and verifies that there is no pending profile in Released state. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_LPAd → SM-DP+ | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_ERROR_8_2_1_2) |

|  | Procedure | PROC_ES9+_VERIFY_PROFILE_RELEASED_EMPTY_MID_WITH_CC |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication on ES9+ from S_LPAd to SM-DP+ under test supplying an empty MatchingId and verifies that there is at least one pending profile in Released state. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_LPAd → SM-DP+ | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( # R_AUTH_CLIENT_OK_CC) |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_CC)) | MTD_HTTP_RESP(#R_GET_BPP_RESP_OP1_PPK) |

|  | Procedure | PROC_ES9+_VERIFY_PROFILE_RELEASED_WITH_MID_WITH_CC |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication on ES9+ from S_LPAd to SM-DP+ under test supplying a MatchingId set to #MATCHING_ID_1 and verifies that there is a pending profile in Released state. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_LPAd → SM-DP+ | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_ACT_CODE_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK_CC) |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_CC)) | MTD_HTTP_RESP(#R_GET_BPP_RESP_OP1_PPK) |

|  | Procedure | PROC_TLS_INITIALIZATION_MUTUAL_AUTH |  |
| --- | --- | --- | --- |
|  | Description | Establishes the Transport Layer Security (TLS) v1.2 connection between the Client and Server using Mutual authentication mode on ES2+, ES12 or ES15. For Client and Server testing the Server MAY be the SM-DS or the SM-DP+. |  |
| Step | Direction | Sequence / Description | Expected result |
| For Client testing, execute the following steps: |  |  |  |
| 1 | CLIENT → S_SERVER | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_CLIENT_TLS_VER, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) |
| 2 | S_SERVER → CLIENT | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <S_SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_S_SERVER_TLS, <SERVER_TLS_EPHEM_KEY>, #CLIENT_CERT_TYPE, #S_SAH_SHA256_ECDSA, #DIST_NAME_CI) | MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH( #CERT_CLIENT_TLS, <CLIENT_TLS_EPHEM_KEY>) |
| 3 | S_SERVER → CLIENT | MTD_TLS_SERVER_END( #CHANGE_CIPHER_SPEC, <SERVER_FINISHED>) | HTTPS connection established |
| For Server testing, execute the following steps: |  |  |  |
| 1 | S_CLIENT → SERVER | MTD_TLS_CLIENT_HELLO( #TLS_VERSION_1_2, #MIN_TLS_CIPHER_SUITES, #S_SESSION_ID_EMPTY, #S_EXT_SHA256_ECDSA) | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_SERVER_TLS, #CLIENT_CERT_TYPE, <SAH_SHA256_ECDSA>, #DIST_NAME_CI) |
| 2 | S_CLIENT → SERVER | MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH( #CERT_CLIENT_TLS, <CLIENT_TLS_EPHEM_KEY>) | MTD_TLS_SERVER_END( #CHANGE_CIPHER_SPEC, <SERVER_FINISHED>) |

|  | Procedure | PROC_TLS_INITIALIZATION_MUTUAL_AUTH_INV_OID |  |
| --- | --- | --- | --- |
|  | Description | Establishes the Transport Layer Security (TLS) v1.2 connection between the Client and Server using Mutual authentication mode on ES12 or ES15 with a Client Certificate that has an invalid OID. For Client and Server testing the Server MAY be the SM-DS or the SM-DP+. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_CLIENT → SERVER | MTD_TLS_CLIENT_HELLO( #TLS_VERSION_1_2, #MIN_TLS_CIPHER_SUITES, #S_SESSION_ID_EMPTY, #S_EXT_SHA256_ECDSA) | MTD_TLS_MUTUAL_AUTH_SERVER_HELLO_ETC( #TLS_VERSION_1_2, <SEL_TLS_CIPHER_SUITE>, <SESSION_ID_RANDOM>, #CERT_SERVER_TLS, #CLIENT_CERT_TYPE, <SAH_SHA256_ECDSA>, #DIST_NAME_CI) |
| 2 | S_CLIENT → SERVER | MTD_TLS_MUTUAL_AUTH_CLIENT_EXCH( #CERT_S_CLIENT_TLS_INV_OID, <CLIENT_TLS_EPHEM_KEY>) | MTD_TLS_SERVER_END( #CHANGE_CIPHER_SPEC, <SERVER_FINISHED>) |

- Annex D	Commands And Responses

- D.1	ES8+ Requests And Responses

- D.1.1 ES8+ Requests

| Name | Content |
| --- | --- |
| CONF_ISDP_EMPTY | req ConfigureISDPRequest ::={} |
| CONF_ISDP_MAX_LENGTH | req ConfigureISDPRequest ::={ dpProprietaryData { -- size=128 bytes dpOid #S_SM_DP+_OID, additionalSmdpData #ADDITIONAL_SMDP_DATA_MAX_LENGTH } } -- NOTE: Instead of DpProprietaryData ::= SEQUENCE { dpOid OBJECT IDENTIFIER -- additional data objects defined by the -- SM-DP+ MAY follow } -- the following structure is used to test the -- DpProprietaryData size: DpProprietaryData ::= SEQUENCE { dpOid OBJECT IDENTIFIER, additionalSmdpData OCTET STRING OPTIONAL } |
| CONF_ISDP_PROF1 | req ConfigureISDPRequest ::={ dpProprietaryData { dpOid #S_SM_DP+_OID } } |
| CONF_ISDP_PROF1_SMDP | req ConfigureISDPRequest ::={ dpProprietaryData { dpOid #IUT_SM_DP_OID -- additional data objects defined by the SM-DP+ MAY follow }-- optional } |
| CONF_ISDP_SIZE_EXCEEDED | req ConfigureISDPRequest ::={ dpProprietaryData { -- size=129 bytes dpOid #S_SM_DP+_OID, additionalSmdpData #ADDITIONAL_SMDP_DATA_EXCEEDED_MAX } } -- NOTE: Instead of DpProprietaryData ::= SEQUENCE { dpOid OBJECT IDENTIFIER -- additional data objects defined by the -- SM-DP+ MAY follow } -- the following structure is used to test the -- DpProprietaryData size: DpProprietaryData ::= SEQUENCE { dpOid OBJECT IDENTIFIER, additionalSmdpData OCTET STRING OPTIONAL } |
| FULL_METADATA | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS1 } }, profileOwner { mccMnc #MCC_MNC1 }, profilePolicyRules {ppr1} } |
| INIT_SC_INVALID_CRT | req InitialiseSecureChannelRequest ::={ remoteOpId #REMOTE_OP_ID_INSTALL, transactionId <S_TRANSACTION_ID>, controlRefTemplate { keyType #INVALID_KEY_TYPE, keyLen #KEY_LENGTH, hostId #HOST_ID }, smdpOtpk <OTPK_S_SM_DP+_AKA>, smdpSign <S_SM_DP+_SIGN> } |
| INIT_SC_INVALID_OP_ID | req InitialiseSecureChannelRequest ::={ remoteOpId #INVALID_REMOTE_OP_ID, transactionId <S_TRANSACTION_ID>, controlRefTemplate { keyType #KEY_TYPE, keyLen #KEY_LENGTH, hostId #HOST_ID }, smdpOtpk <OTPK_S_SM_DP+_AKA>, smdpSign <S_SM_DP+_SIGN> } |
| INIT_SC_INVALID_SIGN | req InitialiseSecureChannelRequest ::={ remoteOpId #REMOTE_OP_ID_INSTALL, transactionId <S_TRANSACTION_ID>, controlRefTemplate { keyType #KEY_TYPE, keyLen #KEY_LENGTH, hostId #HOST_ID }, smdpOtpk <OTPK_S_SM_DP+_AKA>, smdpSign <S_SM_DP+_SIGN> } The <S_SM_DP+_SIGN> SHALL NOT be computed using the #SK_S_SM_DPpb_SIG but SHALL have the same length as for a valid signature |
| INIT_SC_INVALID_TRANS_ID | req InitialiseSecureChannelRequest ::={ remoteOpId #REMOTE_OP_ID_INSTALL, transactionId <INVALID_TRANSACTION_ID>, controlRefTemplate { keyType #KEY_TYPE, keyLen #KEY_LENGTH, hostId #HOST_ID }, smdpOtpk <OTPK_S_SM_DP+_AKA>, smdpSign <S_SM_DP+_SIGN> } |
| INIT_SC_PROF1 | req InitialiseSecureChannelRequest ::={ remoteOpId #REMOTE_OP_ID_INSTALL, transactionId <TRANSACTION_ID_ISC>, controlRefTemplate { keyType #KEY_TYPE, keyLen #KEY_LENGTH, hostId #IUT_SM_DP_HOST_ID }, smdpOtpk <OTPK_SM_DP+_AKA>, smdpSign <SM_DP+_SIGN> } |
| METADATA_ICCID_MISMATCH | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF2, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1 } |
| METADATA_MCCMNC_MISMATCH | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileOwner { mccMnc #MCC_MNC2 }, profilePolicyRules {ppr2} } |
| METADATA_NO_CLASS | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS1 } } } |
| METADATA_OP_PROF1 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS1 } }, profileOwner { mccMnc #MCC_MNC1 } } |
| METADATA_OP_PROF1_EN | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationEnable }, notificationAddress #TEST_DP_ADDRESS1 } }, profileOwner { mccMnc #MCC_MNC1 } } |
| METADATA_OP_PROF1_INST_DIFF | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS2 } }, profileOwner { mccMnc #MCC_MNC1 } } |
| METADATA_OP_PROF2_MEMRES1 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF2, serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, iconType png, icon #ICON_OP_PROF2, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS2 } }, profileOwner { mccMnc #MCC_MNC2 }, profilePolicyRules { ppr2 } } |
| METADATA_OP_PROF4_MEMRES1 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF4, serviceProviderName #SP_NAME4, profileName #NAME_OP_PROF4, iconType png, icon #ICON_OP_PROF4, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS4 } }, profileOwner { mccMnc #MCC_MNC4 }, profilePolicyRules { ppr1 } } |
| METADATA_OP_PROF2 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF2, serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, iconType png, icon #ICON_OP_PROF2, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS2 } }, profileOwner { mccMnc #MCC_MNC2 } } |
| METADATA_OP_PROF2_NO_INSTALL | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF2, serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, iconType png, icon #ICON_OP_PROF2, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS2 } }, profileOwner { mccMnc #MCC_MNC2 } } |
| METADATA_OP_PROF1_NO_INSTALL | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS1 } }, profileOwner { mccMnc #MCC_MNC1 } } |
| METADATA_OP_PROF2_TEST_DP_ADDRESS1 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF2, serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, iconType png, icon #ICON_OP_PROF2, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS1 } }, profileOwner { mccMnc #MCC_MNC2 } } |
| METADATA_OP_PROF3 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF3, serviceProviderName #SP_NAME3, profileName #NAME_OP_PROF3, iconType png, icon #ICON_OP_PROF3, profileClass operational, profileOwner { mccMnc #MCC_MNC2 }, profilePolicyRules { ppr2 } } |
| METADATA_OP_PROF4 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF4, serviceProviderName #SP_NAME4, profileName #NAME_OP_PROF4, iconType png, icon #ICON_OP_PROF4, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS4 } }, profileOwner { mccMnc #MCC_MNC4 }, profilePolicyRules { ppr1 } } |
| METADATA_OP_PROF5 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF5, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF5, iconType png, icon #ICON_OP_PROF5, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS1 } }, profileOwner { mccMnc #MCC_MNC1 } } |
| METADATA_OP_PROF6 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF6, serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF6, iconType png, icon #ICON_OP_PROF6, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS2 } }, profileOwner { mccMnc #MCC_MNC2 } } |
| METADATA_OP_PROF7 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF7, serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF7, iconType png, icon #ICON_OP_PROF7, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS8 } }, profileOwner { mccMnc #MCC_MNC2 }, profilePolicyRules { ppr2 } } |
| METADATA_OP_PROF8 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF8, serviceProviderName #SP_NAME8, profileName #NAME_OP_PROF8, iconType png, icon #ICON_OP_PROF8, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS8 } }, profileOwner { mccMnc #MCC_MNC2 }, profilePolicyRules { ppr2 } } |
| METADATA_OP_PROF9 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF9, serviceProviderName #SP_NAME9, profileName #NAME_OP_PROF9, profileOwner { mccMnc #MCC_MNC9, gid1 #GID1, gid2 #GID2 }, profilePolicyRules { ppr2 } } |
| METADATA_OP1_GID1GID2_PRESENT | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileOwner { mccMnc #MCC_MNC1, gid1 #GID1, gid2 #GID2 }, profilePolicyRules {ppr2} } |
| METADATA_OP9_GID1GID2_MISSING | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF9, serviceProviderName #SP_NAME9, profileName #NAME_OP_PROF9, profileOwner { mccMnc #MCC_MNC9 } } |
| METADATA_PPR_NO_OWNER | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profilePolicyRules {ppr2} } |
| METADATA_WILDCARD | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileOwner { mccMnc #MCC_MNC_WILDCARD }, profilePolicyRules {ppr2} } |
| METADATA_WITH_JPG | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType jpg, icon #ICON_JPG } |
| METADATA_WITH_NOTIFS | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, notificationConfigurationInfo { { profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS3 }, { profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS2 }, { profileManagementOperation { notificationEnable }, notificationAddress #TEST_DP_ADDRESS2 }, { profileManagementOperation { notificationEnable }, notificationAddress #TEST_DP_ADDRESS3 }, { profileManagementOperation { notificationDisable }, notificationAddress #TEST_DP_ADDRESS3 }, { profileManagementOperation { notificationDisable }, notificationAddress #TEST_DP_ADDRESS4 }, { profileManagementOperation { notificationDelete }, notificationAddress #TEST_DP_ADDRESS1 }, { profileManagementOperation { notificationDelete }, notificationAddress #TEST_DP_ADDRESS3 } } } |
| METADATA_WITH_PPR1_PPR2 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileOwner { mccMnc #MCC_MNC1 }, profilePolicyRules {ppr1,ppr2} } |
| METADATA_WITH_PPR2 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileOwner { mccMnc #MCC_MNC1 }, profilePolicyRules {ppr2} } |
| METADATA_WITH_PPRS_AND_ICON | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileOwner { mccMnc #MCC_MNC1 }, profilePolicyRules {ppr1,ppr2} } |
| METADATA_WITHOUT_ICON | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType jpg } |
| REPLACE_S_KEYS_REQ | req ReplaceSessionKeysRequest ::={ initialMacChainingValue <PPK_INIT_MAC>, ppkEnc <PPK_ENC>, ppkCmac <PPK_MAC> } |
| REPLACE_S_KEYS_REQ_INV_SIZE | req ReplaceSessionKeysRequest ::={ initialMacChainingValue #PPK_INIT_MAC_INV_SIZE, ppkEnc #PPK_ENC_INV_SIZE, ppkCmac #PPK_MAC_INV_SIZE } |
| S_INIT_SC_PROF1 | req InitialiseSecureChannelRequest ::={ remoteOpId #REMOTE_OP_ID_INSTALL, transactionId <S_TRANSACTION_ID>, controlRefTemplate { keyType #KEY_TYPE, keyLen #KEY_LENGTH, hostId #HOST_ID }, smdpOtpk <OTPK_S_SM_DP+_AKA>, smdpSign <S_SM_DP+_SIGN> } |
| SMDP_METADATA_ABS | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1 } |
| SMDP_METADATA_ALL | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #IUT_SM_DP_ADDRESS } }, profileOwner { mccMnc #MCC_MNC1 }, profilePolicyRules { ppr1, ppr2 } } |
| SMDP_METADATA_NON_ASCII | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME_NON_ASCII, profileName #NAME_OP_PROF1_NON_ASCII } |
| SMDP_METADATA_NOTIF_MULTI | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #IUT_SM_DP_ADDRESS }, { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS1 } } } |
| SMDP_METADATA_OP_PROF1_EN | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationEnable }, notificationAddress #IUT_SM_DP_ADDRESS } } } |
| SMDP_METADATA_OP_PROF1_PPR2 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileClass operational, profileOwner { mccMnc #MCC_MNC1 }, profilePolicyRules { ppr2 } } |
| SMDP_METADATA_PN_LONG | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF_LONG } |
| SMDP_METADATA_SPN_LONG | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME_LONG, profileName #NAME_OP_PROF1 } |

- D.2	ES9+ Requests And Responses

- D.2.1	ES9+ Requests

| Name | Content |
| --- | --- |
| AUTH_SERVER_RESP_ACT_CODE_UC_OK | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_ACT_CODE }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_ACT_CODE_UC_OK_EID2 | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_ACT_CODE }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG_EID2, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_ACT_CODE_2_UC_OK | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_ACT_CODE_2 }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_DEF_DP_OK_eUICC_EXT | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2_EXT ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_DEF_DP_OK_UICC_EXT | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2_UICC_EXT ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_DEF_DP_OK_DEVICE_EXT | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2_DEV_EXT ctxParams1 #CTX_PARAMS1_DEVICE_INFO_EXT }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_1_3_8 | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG_EID2, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_4_8 | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2_INSUF_MEM_ERROR, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_2_6_1_EX_BC_cA | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG_INVALID_EX_BC_cA } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_2_6_1_EX_BC_PLC | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG_INVALID_EX_BC_PLC } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_2_6_1_EX_CP | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG_INVALID_EX_CP } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_2_6_1_EX_KU | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG_INVALID_EX_KU } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_2_6_1_SIG | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG_INVALID_SIG } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_2_6_3 | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG_EXPIRED } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_3_6_1_EX_CP | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG_INVALID_EX_CP, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_3_6_1_EX_KU | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG_INVALID_EX_KU, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_3_6_1_SIG | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG_INVALID_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_3_6_1_SUB_ORG | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG_INVALID_SUB_ORG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_3_6_1_SUB_SN | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG_INVALID_SUB_SN, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_3_6_3 | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG_EXPIRED, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_6_1_CHA | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE_INVALID>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_6_1_SIG | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1_INVALID>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_2_5_4_3 | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2_PPR2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_10_1_3_9 | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <INVALID_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_11_1_3_9 | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG_UNKNOWN } |
| AUTH_SERVER_RESP_DEF_DP_UC_OK | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2 ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_SMDP_MATCHING_ID_EMPTY | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_SMDP_MATCHING_ID_OMITTED | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2 ctxParams1 #CTX_PARAMS1_MATCHING_ID_OMITTED }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_SMDS_UC_OK | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_SMDS }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_SMDS_UC_OK_EID2 | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_SMDS }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG_EID2, nextCertInChain #CERT_EUM_SIG } |
| CS_RESP_ERROR_8_1_6_1 | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #IUT_SM_DP_OID, reason postponed }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE_INVALID> } |
| CS_RESP_ERROR_8_8_3_10 | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid <INVALID_SM_DP_OID>, reason postponed }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_RESP_ERROR_8_10_1_3_9 | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <INVALID_TRANSACTION_ID>, smdpOid #IUT_SM_DP_OID, reason postponed }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_RESP_OK_EU_REJ | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #IUT_SM_DP_OID, reason endUserRejection }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_RESP_OK_L_BPP_EXE_ERROR | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #IUT_SM_DP_OID, reason loadBppExecutionError }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_RESP_OK_M_DATA_MISMATCH | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #IUT_SM_DP_OID, reason metadataMismatch }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_RESP_OK_POSTPONED | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #IUT_SM_DP_OID, reason postponed }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_RESP_OK_PPR_NOT_ALLOWED | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #IUT_SM_DP_OID, reason pprNotAllowed }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_RESP_OK_TIMEOUT | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #IUT_SM_DP_OID, reason timeout }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_RESP_OK_UNDEFINED | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #IUT_SM_DP_OID, reason undefinedReason }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CTX_PARAMS1_ACT_CODE | ctx CtxParams1 ::= ctxParamsForCommonAuthentication : { matchingId #MATCHING_ID_1, deviceInfo #S_DEVICE_INFO } |
| CTX_PARAMS1_ACT_CODE_2 | ctx CtxParams1 ::= ctxParamsForCommonAuthentication : { matchingId #MATCHING_ID_2, deviceInfo #S_DEVICE_INFO } |
| CTX_PARAMS1_MATCHING_ID_EMPTY | ctx CtxParams1 ::= ctxParamsForCommonAuthentication : { matchingId #MATCHING_ID_EMPTY, deviceInfo #S_DEVICE_INFO } |
| CTX_PARAMS1_SMDS | ctx CtxParams1 ::= ctxParamsForCommonAuthentication : { matchingId <MATCHING_ID_EVENT>, deviceInfo #S_DEVICE_INFO } |
| EUICC_FIRMWARE_VER | 0x01 00 00 |
| EXT_CARD_RESOURCE_LIMITED_SPACE | The Extended Card Resource Information according to ETSI TS 102 226 and set as: 0x81 <L> #INSTALLED_PROFILES 0x82 <L> #NON_VOLATILE_MEM_LIMITED_SPACE 0x83 <L> #S_VOLATILE_MEM |
| INITIATE_AUTH_DS_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDS_SIGNED1>, "serverSignature1" : <S_SMDS_SIGNATURE1>, "euiccCiPKIdTobeUsed" : <EUICC_CI_PK_ID_TO_BE_USED>, "serverCertificate" : #CERT_S_SM_DSauth_SIG } -- NOTE: select the CI as defined in the note in the chapter 2.1.4 of SGP.23 |
| INITIATE_AUTH_DS_OK_1 | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDS_SIGNED_ADDR1>, "serverSignature1" : <S_SMDS_SIGNATURE1>, "euiccCiPKIdTobeUsed" : <EUICC_CI_PK_ID_TO_BE_USED>, "serverCertificate" : #CERT_S_SM_DSauth_SIG } -- NOTE: select the CI as defined in the note in the chapter 2.1.4 of SGP.23 |
| INITIATE_AUTH_INV_CERT_DS | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDS_SIGNED1>, "serverSignature1" : <S_SMDS_SIGNATURE1>, "euiccCiPKIdTobeUsed" : <EUICC_CI_PK_ID_TO_BE_USED>, -- NOTE: select the CI Key ID in highest priority from the <EUICC_CI_PK_ID_LIST_FOR_SIGNING> "serverCertificate" : #CERT_S_SM_DSauth_INV_SIGN } |
| INITIATE_AUTH_INV_CI_DS | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDS_SIGNED1>, "serverSignature1" : <S_SMDS_SIGNATURE1>, "euiccCiPKIdTobeUsed" : #CI_PK_ID_INV, "serverCertificate" : #CERT_S_SM_DSauth_SIG -- NOTE: select and choose the #CERT_S_SM_DSauth_SIG leading to the CI Key ID in highest priority from the <EUICC_CI_PK_ID_LIST_FOR_SIGNING> } |
| INITIATE_AUTH_INV_SIGN_DS | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDS_SIGNED1>, "serverSignature1" : <S_SMDS_SIGNATURE_INV>, "euiccCiPKIdTobeUsed" : <EUICC_CI_PK_ID_TO_BE_USED>, "serverCertificate" : #CERT_S_SM_DSauth_SIG } -- NOTE: select the CI Key ID in highest priority from the <EUICC_CI_PK_ID_LIST_FOR_SIGNING> and choose the #CERT_S_SM_DSauth_SIG leading to the same Root CI certificate |
| INITIATE_AUTH_INV_SMDS_ADDRESS | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDS_SIGNED_INV_ADDR>, "serverSignature1" : <S_SMDS_SIGNATURE1>, "euiccCiPKIdTobeUsed" : <EUICC_CI_PK_ID_TO_BE_USED>, "serverCertificate" : #CERT_S_SM_DSauth_SIG } -- NOTE: select the CI Key ID in highest priority from the <EUICC_CI_PK_ID_LIST_FOR_SIGNING> and choose the #CERT_S_SM_DSauth_SIG leading to the same Root CI certificate |
| INITIATE_AUTH_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDP_SIGNED1>, "serverSignature1" : <S_SMDP_SIGNATURE1>, "euiccCiPKIdTobeUsed" : <EUICC_CI_PK_ID_TO_BE_USED>, "serverCertificate" : #CERT_S_SM_DPauth_SIG } -- NOTE: select the CI as defined in the note in the chapter 2.1.4 of SGP.23 |
| INITIATE_AUTH_INV_CERT | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDP_SIGNED1>, "serverSignature1" : <S_SMDP_SIGNATURE1>, "euiccCiPKIdTobeUsed" : <EUICC_CI_PK_ID_TO_BE_USED>,-- NOTE: select the CI Key ID in highest priority from the <EUICC_CI_PK_ID_LIST_FOR_SIGNING> "serverCertificate" : #CERT_S_SM_DPauth_INV_SIGN } |
| INITIATE_AUTH_INV_CI | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDP_SIGNED1>, "serverSignature1" : <S_SMDP_SIGNATURE1>, "euiccCiPKIdTobeUsed" : #CI_PKI_ID2, "serverCertificate" : #CERT_S_SM_DPauth_SIG -- NOTE: select and choose the #CERT_S_SM_DPauth_SIG leading to the CI Key ID in highest priority from the <EUICC_CI_PK_ID_LIST_FOR_SIGNING> } |
| INITIATE_AUTH_INV_OID | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDP_SIGNED1>, "serverSignature1" : <S_SMDP_SIGNATURE1>, "euiccCiPKIdTobeUsed" : <EUICC_CI_PK_ID_TO_BE_USED>, "serverCertificate" : #CERT_S_SM_DP2auth_SIG } -- NOTE: select the CI Key ID in highest priority from the <EUICC_CI_PK_ID_LIST_FOR_SIGNING> -- NOTE: serverSignature1 SHALL be calculated correctly, using the secret key related to CERT_S_SM_DP2auth_SIG. |
| INITIATE_AUTH_INV_SIGN | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDP_SIGNED1>, "serverSignature1" : <S_SMDP_SIGNATURE_INV>, "euiccCiPKIdTobeUsed" : <EUICC_CI_PK_ID_TO_BE_USED>, "serverCertificate" : #CERT_S_SM_DPauth_SIG } -- NOTE: select the CI Key ID in highest priority from the <EUICC_CI_PK_ID_LIST_FOR_SIGNING> and choose the #CERT_S_SM_DPauth_SIG leading to the same Root CI certificate |
| INITIATE_AUTH_INV_SMDP+_ADDRESS | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDP_SIGNED_INV_ADDR>, "serverSignature1" : <S_SMDP_SIGNATURE1>, "euiccCiPKIdTobeUsed" : <EUICC_CI_PK_ID_TO_BE_USED>, "serverCertificate" : #CERT_S_SM_DPauth_SIG } -- NOTE: select the CI Key ID in highest priority from the <EUICC_CI_PK_ID_LIST_FOR_SIGNING> and choose the #CERT_S_SM_DPauth_SIG leading to the same Root CI certificate -- NOTE: serverSignature1 SHALL be calculated correctly, using <S_SMDP_SIGNED_INV_ADDR>. |
| LPA_RSP_CAPABILITY | lpaRspCapability LpaRspCapability ::= { crlStaplingV3Support, certChainV3Support } |
| MATCHING_ID_EMPTY |  |
| NON_VOLATILE_MEM_LIMITED_SPACE | '0x00 01' |
| PENDING_NOTIF_DEL1 | response PendingNotification ::= otherSignedNotification :{ tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationDelete }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_DEL2 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationDelete }, notificationAddress #TEST_DP_ADDRESS2, iccid #ICCID_OP_PROF2 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_DEL4 | response PendingNotification ::= otherSignedNotification :{ tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationDelete }, notificationAddress #TEST_DP_ADDRESS4, iccid #ICCID_OP_PROF4 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_DEL5 | response PendingNotification ::= otherSignedNotification :{ tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationDelete }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF5 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_DEL6 | response PendingNotification ::= otherSignedNotification :{ tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationDelete }, notificationAddress #TEST_DP_ADDRESS2, iccid #ICCID_OP_PROF6 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_DIS1 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationDisable }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_DIS5 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationDisable }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF5 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_DIS8 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationDisable }, notificationAddress #TEST_DP_ADDRESS8, iccid #ICCID_OP_PROF8 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_EN1 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationEnable }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_EN2 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationEnable }, notificationAddress #TEST_DP_ADDRESS2, iccid #ICCID_OP_PROF2 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_EN5 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationEnable }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF5 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_EN6 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationEnable }, notificationAddress #TEST_DP_ADDRESS2, iccid #ICCID_OP_PROF6 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PP_VERSION | 0x01 00 00 |
| PREP_DOWNLOAD_RESP_8_1_6_1 | resp PrepareDownloadResponse ::= downloadResponseOk : { euiccSigned2 { transactionId <S_TRANSACTION_ID>, euiccOtpk <BPP_OTPK_EUICC_AKA> }, euiccSignature2 <EUICC_SIGNATURE2_INVALID> } |
| PREP_DOWNLOAD_RESP_8_2_7_3_8 | resp PrepareDownloadResponse ::= downloadResponseOk : { euiccSigned2 { transactionId <S_TRANSACTION_ID>, euiccOtpk <BPP_OTPK_EUICC_AKA>, hashCc <S_HASHED_CC_ERROR> }, euiccSignature2 <EUICC_SIGNATURE2> } |
| PREP_DOWNLOAD_RESP_8_10_1_3_9 | resp PrepareDownloadResponse ::= downloadResponseOk : { euiccSigned2 { transactionId <INVALID_TRANSACTION_ID>, euiccOtpk <BPP_OTPK_EUICC_AKA> }, euiccSignature2 <EUICC_SIGNATURE2> } |
| PREP_DOWNLOAD_RESP | resp PrepareDownloadResponse ::= downloadResponseOk : { euiccSigned2 { transactionId <S_TRANSACTION_ID>, euiccOtpk <BPP_OTPK_EUICC_AKA> }, euiccSignature2 <EUICC_SIGNATURE2> } |
| PREP_DOWNLOAD_RESP_CC | resp PrepareDownloadResponse ::= downloadResponseOk : { euiccSigned2 { transactionId <S_TRANSACTION_ID>, euiccOtpk <BPP_OTPK_EUICC_AKA>, hashCc <S_HASHED_CC> }, euiccSignature2 <EUICC_SIGNATURE2> } |
| PREP_DOWNLOAD_RESP_NEW_OTPK | resp PrepareDownloadResponse ::= downloadResponseOk : { euiccSigned2 { transactionId <S_TRANSACTION_ID>, euiccOtpk <OTPK_EUICC_AKA_NEW> }, euiccSignature2 <EUICC_SIGNATURE2> } |
| PREP_DOWNLOAD_RESP_NEW_OTPK_CC | resp PrepareDownloadResponse ::= downloadResponseOk : { euiccSigned2 { transactionId <S_TRANSACTION_ID>, euiccOtpk <OTPK_EUICC_AKA_NEW>, hashCc <S_HASHED_CC> }, euiccSignature2 <EUICC_SIGNATURE2> } |
| PROFILE_VERSION | 0x02 01 00 |
| RSP_CAPABILITY | rspCapability RspCapability ::= { additionalProfile, rpmSupport, testProfileSupport } |
| RSP_CAPABILITY_EXT | rspCapability RspCapability ::= { additionalProfile, rpmSupport, testProfileSupport, deviceInfoExtensibilitySupport, serviceSpecificDataSupport } |
| S_EUICC_INFO2_INSUF_MEM_ERROR | euiccInfo2 EUICCInfo2 ::= { profileVersion #PROFILE_VERSION, svn #RSP_SVN_H, euiccFirmwareVer #EUICC_FIRMWARE_VER, extCardResource #EXT_CARD_RESOURCE_LIMITED_SPACE, uiccCapability #UICC_CAPABILITY, rspCapability #RSP_CAPABILITY, euiccCiPKIdListForVerification {#EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1}, euiccCiPKIdListForSigning {#EUICC_CI_PK_ID_LIST_FOR_SIGNING_1}, ppVersion #PP_VERSION, sasAcreditationNumber #SAS_ACREDITATION_NUMBER } |
| S_EUICC_INFO2_PPR2 | euiccInfo2 EUICCInfo2 ::= { profileVersion #PROFILE_VERSION, svn #RSP_SVN_H, euiccFirmwareVer #EUICC_FIRMWARE_VER, extCardResource #S_EXT_CARD_RESOURCE, uiccCapability #UICC_CAPABILITY, rspCapability #RSP_CAPABILITY, euiccCiPKIdListForVerification {#EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1}, euiccCiPKIdListForSigning {#EUICC_CI_PK_ID_LIST_FOR_SIGNING_1}, forbiddenProfilePolicyRules { ppr2 }, ppVersion #PP_VERSION, sasAcreditationNumber #SAS_ACREDITATION_NUMBER } |
| S_EXT_CARD_RESOURCE | The Extended Card Resource Information according to ETSI TS 102 226: 0x81 <L> #INSTALLED_PROFILES 0x82 <L> #S_NON_VOLATILE_MEM 0x83 <L> #S_VOLATILE_MEM |
| S_NON_VOLATILE_MEM | 0xA0 00 |
| S_PN_PIR_OK1 | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult successResult : { aid <ISD_P_AID>, simaResponse #SIMA_RESULT_OK } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_INVALID_TRANS_ID | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <INVALID_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult successResult : { aid <ISD_P_AID>, simaResponse #SIMA_RESULT_OK } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_INCORRECT_INPUT_VALUES | response PendingNotification ::= profileInstallationResult : profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId configureISDP, errorReason incorrectInputValues } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_INVALID_SIGN | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId initialiseSecureChannel, errorReason invalidSignature } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_UNSUPPORTED_CRT | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId initialiseSecureChannel, errorReason unsupportedCrtValues } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_UNSUP_REMOTE_OP_TYPE | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId initialiseSecureChannel, errorReason unsupportedRemoteOperationType } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_UNSUP_PROFILE_CLASS | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId storeMetadata, errorReason unsupportedProfileClass } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_SCP03T_STRUCTURE_ERROR | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId storeMetadata, errorReason scp03tStructureError } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_SCP03T_SECURITY_ERROR | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId replaceSessionKeys, errorReason scp03tSecurityError } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_ICCID_ALREADY_EXISTS | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId storeMetadata, errorReason installFailedDueToIccidAlreadyExistsOnEuicc } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_INSUFFICIENT_MEMORY | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId storeMetadata, errorReason installFailedDueToInsufficientMemoryForProfile } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_INSTALL_INTERRUPTION | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId storeMetadata, errorReason installFailedDueToInterruption } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_PE_PROCESSING_ERROR | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId loadProfileElements, errorReason installFailedDueToPEProcessingError } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_DATA_MISMATCH | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId loadProfileElements, errorReason installFailedDueToDataMismatch } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_TEST_PROFILE_INVALID_NAA_KEY | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId loadProfileElements, errorReason testProfileInstallFailedDueToInvalidNaaKey } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_PPR_NOT_ALLOWED | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId storeMetadata, errorReason pprNotAllowed } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_UNKNOWN_ERROR | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId storeMetadata, errorReason installFailedDueToUnknownError } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PENDING_NOTIF_OTHER_INST1 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| S_PENDING_NOTIF_EN1 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationLocalEnable }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| S_PENDING_NOTIF_DIS1 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationLocalDisable }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| S_PENDING_NOTIF_DE1 | response PendingNotification ::= otherSignedNotification :{ tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationLocalDelete }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| S_PENDING_NOTIF_RPM_EN1 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationRpmEnable }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| S_PENDING_NOTIF_RPM_DIS1 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationRpmDisable }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| S_PENDING_NOTIF_RPM_DE1 | response PendingNotification ::= otherSignedNotification :{ tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationRpmDelete }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| S_PENDING_NOTIF_RPM_UM1 | response PendingNotification ::= loadRpmPackageResultSigned : { loadRpmPackageResultDataSigned { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { loadRpmPackageResult }, notificationAddress #IUT_SM_DP_ADDRESS }, smdpOid #IUT_SM_DP_OID, finalResult rpmPackageExecutionResult : { { iccid #ICCID_OP_PROF1, rpmCommandResultData updateMetadataResult : { updateMetadataResult { ok } } } } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_SMDP_SIGNED2 | req SmdpSigned2 ::= { transactionId <S_TRANSACTION_ID>, ccRequiredFlag FALSE } |
| S_SMDP_SIGNED2_CC | req SmdpSigned2 ::= { transactionId <S_TRANSACTION_ID>, ccRequiredFlag TRUE } |
| S_SMDP_SIGNED2_INV_TRANSACTION_ID | req SmdpSigned2 ::= { transactionId <INVALID_TRANSACTION_ID>, ccRequiredFlag FALSE } |
| S_VOLATILE_MEM | '0x01 00' |
| SAS_ACREDITATION_NUMBER | GSMA_SAS_123456789 |
| UICC_CAPABILITY | uiccCapability UICCCapability ::= { contactlessSupport, usimSupport, isimSupport, akaMilenage, akaTuak128, gbaAuthenUsim, eapClient, javacard, multipleUsimSupport } |
| UICC_CAPABILITY_EXT | uiccCapability UICCCapability ::= { contactlessSupport, usimSupport, isimSupport, akaMilenage, akaTuak128, gbaAuthenUsim, eapClient, javacard, multipleUsimSupport, berTlvFileSupport, dfLinkSupport, catTp, getIdentity, profile-a-x25519, profile-b-p256, suciCalculatorApi, unknownServiceSupport } Note: the definition of UICCCapability used above is equivalent to the definition in SGP.22 v2.3 (specific version of [2]) with the additional of a further field called “unknownServiceSupport” after the “suciCalculatorApi” field. |

- D.2.2	ES9+ Responses

| Name | Content |
| --- | --- |
| AUTH_CLIENT_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "profileMetadata" : #METADATA_OP_PROF1, "smdpSigned2" : #S_SMDP_SIGNED2, "smdpSignature2" : <S_SM_DP+_SIGNATURE2>, "smdpCertificate" : #CERT_S_SM_DPpb_SIG } |
| AUTH_CLIENT_OK_CC | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "profileMetadata" : #METADATA_OP_PROF1, "smdpSigned2" : #S_SMDP_SIGNED2_CC, "smdpSignature2" : <S_SM_DP+_SIGNATURE2>, "smdpCertificate" : #CERT_S_SM_DPpb_SIG } |
| AUTH_CLIENT_INV_PB_CERT | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "profileMetadata" : #METADATA_OP_PROF1, "smdpSigned2" : #S_SMDP_SIGNED2, "smdpSignature2" : <S_SM_DP+_SIGNATURE2>, "smdpCertificate" : #CERT_S_SM_DPpb_INV_SIGN } |
| AUTH_CLIENT_INV_CI | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "profileMetadata" : #METADATA_OP_PROF1, "smdpSigned2" : #S_SMDP_SIGNED2, "smdpSignature2" : <S_SM_DP+_SIGNATURE2>, "smdpCertificate" : #CERT_S_SM_DP2pb_SIG } |
| AUTH_CLIENT_INV_SIGN | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "profileMetadata" : #METADATA_OP_PROF1, "smdpSigned2" : #S_SMDP_SIGNED2, "smdpSignature2" : <S_SM_DP+_SIGNATURE2>, "smdpCertificate" : #CERT_S_SM_DPpb_SIG } The <S_SM_DP+_SIGNATURE2> SHALL NOT be computed using the #SK_S_SM_DPpb_SIG but SHALL have the same length as for a valid signature |
| AUTH_CLIENT_INV_TRANSACTION_ID | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "profileMetadata" : #METADATA_OP_PROF1, "smdpSigned2" : #S_SMDP_SIGNED2_INV_TRANSACTION_ID, "smdpSignature2" : <S_SM_DP+_SIGNATURE2>, "smdpCertificate" : #CERT_S_SM_DPpb_SIG } |
| CS_OK_EU_LOAD_BPP_ERROR | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #S_SM_DP+_OID, reason loadBppExecutionError }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_OK_EU_POSTPONED | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #S_SM_DP+_OID, reason postponed }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_OK_EU_REJ | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #S_SM_DP+_OID, reason endUserRejection }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_OK_PPR_NOT_ALLOWED | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #S_SM_DP+_OID, reason pprNotAllowed }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_OK_TIMEOUT | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #S_SM_DP+_OID, reason timeout }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| GET_BPP_LOAD_ERROR | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "boundProfilePackage" : BoundProfilePackage { #S_INIT_SC_PROF1, firstSequenceOf87 { #CONF_ISDP_PROF1 }, sequenceOf88 { <METADATA_OP_PROF1_SEG> … <METADATA_OP_PROF1_SEG> } } } NOTE 1: boundProfilePackage is enconded as base64 therefore the test tool SHALL decode boundProfilePackage to access the ASN.1. NOTE 2: For sequenceOf88 there will be only one or two '88' TLV segments depending on the size of StoreMetadata. |
| GET_BPP_LOAD_ERROR_UNKNOWN_TAG | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "boundProfilePackage" { #S_INIT_SC_PROF1, #UNKNOWN_BPP_SEGMENT, firstSequenceOf87 { #CONF_ISDP_PROF1 }, sequenceOf88 { <METADATA_OP_PROF1_SEG> … <METADATA_OP_PROF1_SEG> }, sequenceOf86 { <PPP_OP_PROF1_SEG_SK> … <PPP_OP_PROF1_SEG_SK> } } } NOTE 1: boundProfilePackage is encoded as base64 therefore the test tool shall decode boundProfilePackage to access the ASN.1. NOTE 2: For sequenceOf88 there will be only one or two '88' TLV segments depending on the size of StoreMetadata. |
| GET_BPP_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "boundProfilePackage" : BoundProfilePackage { #S_INIT_SC_PROF1, firstSequenceOf87 { #CONF_ISDP_PROF1 }, sequenceOf88 { <METADATA_OP_PROF1_SEG> … <METADATA_OP_PROF1_SEG> }, sequenceOf86 { <PPP_OP_PROF1_SEG_SK> … <PPP_OP_PROF1_SEG_SK> } } } NOTE 1: boundProfilePackage is enconded as base64 therefore the test tool SHALL decode boundProfilePackage to access the ASN.1. NOTE 2: For sequenceOf88 there will be only one or two '88' TLV segments depending on the size of StoreMetadata. |
| GET_BPP_OK_PPK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "boundProfilePackage" : BoundProfilePackage { #S_INIT_SC_PROF1, firstSequenceOf87 { 0x87 <L> #CONF_ISDP_PROF1 }, sequenceOf88 { <METADATA_OP_PROF1_SEG> … <METADATA_OP_PROF1_SEG> }, secondSequenceOf87 { 0x87 <L> #REPLACE_S_KEYS_REQ }, sequenceOf86 { <PPP_OP_PROF1_SEG_SK> … <PPP_OP_PROF1_SEG_SK> } } } |
| GET_BPP_INV | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "boundProfilePackage" : BoundProfilePackage { #S_INIT_SC_PROF1, firstSequenceOf87 { 0x87 <L> #CONF_ISDP_PROF1 }, sequenceOf88 { <METADATA_OP_PROF1_SEG> … <METADATA_OP_PROF1_SEG> }, sequenceOf86 { <PPP_OP_PROF1_SEG_SK_INV> … <PPP_OP_PROF1_SEG_SK_INV> } } } |
| PENDING_NOTIF_INST_ADDRESS2 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS2, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_INST1 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| R_AUTH_CLIENT_META_ABS | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <TRANSACTION_ID_AC>, "profileMetadata" : #SMDP_METADATA_ABS, "smdpSigned2" : #SMDP_SIGNED2, "smdpSignature2" : <SMDP_SIGNATURE2>, "smdpCertificate" : #CERT_SM_DPpb_SIG } |
| R_AUTH_CLIENT_META_ALL | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <TRANSACTION_ID_AC>, "profileMetadata" : #SMDP_METADATA_ALL, "smdpSigned2" : #SMDP_SIGNED2, "smdpSignature2" : <SMDP_SIGNATURE2>, "smdpCertificate" : #CERT_SM_DPpb_SIG } |
| R_AUTH_CLIENT_META_LARGE_ICON | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <TRANSACTION_ID_AC>, "profileMetadata" : #SMDP_METADATA_OP_PROF1_2_SEG, "smdpSigned2" : #SMDP_SIGNED2, "smdpSignature2" : <SMDP_SIGNATURE2>, "smdpCertificate" : #CERT_SM_DPpb_SIG } |
| R_AUTH_CLIENT_META_NON_ASCII | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <TRANSACTION_ID_AC>, "profileMetadata" : #SMDP_METADATA_NON_ASCII, "smdpSigned2" : #SMDP_SIGNED2, "smdpSignature2" : <SMDP_SIGNATURE2>, "smdpCertificate" : #CERT_SM_DPpb_SIG } |
| R_AUTH_CLIENT_META_NOTIF_MULTI | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <TRANSACTION_ID_AC>, "profileMetadata" : #SMDP_METADATA_NOTIF_MULTI, "smdpSigned2" : #SMDP_SIGNED2, "smdpSignature2" : <SMDP_SIGNATURE2>, "smdpCertificate" : #CERT_SM_DPpb_SIG } |
| R_AUTH_CLIENT_META_PN_LONG | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <TRANSACTION_ID_AC>, "profileMetadata" : #SMDP_METADATA_PN_LONG, "smdpSigned2" : #SMDP_SIGNED2, "smdpSignature2" : <SMDP_SIGNATURE2>, "smdpCertificate" : #CERT_SM_DPpb_SIG } |
| R_AUTH_CLIENT_META_SPN_LONG | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <TRANSACTION_ID_AC>, "profileMetadata" : #SMDP_METADATA_SPN_LONG, "smdpSigned2" : #SMDP_SIGNED2, "smdpSignature2" : <SMDP_SIGNATURE2>, "smdpCertificate" : #CERT_SM_DPpb_SIG } |
| R_AUTH_CLIENT_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <TRANSACTION_ID_AC>, "profileMetadata" : #SMDP_METADATA_OP_PROF1, "smdpSigned2" : #SMDP_SIGNED2, "smdpSignature2" : <SMDP_SIGNATURE2>, "smdpCertificate" : #CERT_SM_DPpb_SIG } |
| R_AUTH_CLIENT_OK_ALL_NOTIF | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <TRANSACTION_ID_AC>, "profileMetadata" : #SMDP_METADATA_ALL_NOTIF, "smdpSigned2" : #SMDP_SIGNED2, "smdpSignature2" : <SMDP_SIGNATURE2>, "smdpCertificate" : #CERT_SM_DPpb_SIG } |
| R_AUTH_CLIENT_OK_CC | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <TRANSACTION_ID_AC>, "profileMetadata" : #SMDP_METADATA_OP_PROF1, "smdpSigned2" : #SMDP_SIGNED2_CC, "smdpSignature2" : <SMDP_SIGNATURE2>, "smdpCertificate" : #CERT_SM_DPpb_SIG } |
| R_AUTH_CLIENT_OK_EN | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <TRANSACTION_ID_AC>, "profileMetadata" : #SMDP_METADATA_OP_PROF1_EN, "smdpSigned2" : #SMDP_SIGNED2, "smdpSignature2" : <SMDP_SIGNATURE2>, "smdpCertificate" : #CERT_SM_DPpb_SIG } |
| R_AUTH_CLIENT_OK_PPR2 | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <TRANSACTION_ID_AC>, "profileMetadata" : #SMDP_METADATA_OP_PROF1_PPR2, "smdpSigned2" : #SMDP_SIGNED2, "smdpSignature2" : <SMDP_SIGNATURE2>, "smdpCertificate" : #CERT_SM_DPpb_SIG } |
| R_AUTH_CLIENT_RETRY_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <TRANSACTION_ID_AC>, "profileMetadata" : #SMDP_METADATA_OP_PROF1, "smdpSigned2" : #SMDP_SIGNED2_RETRY, "smdpSignature2" : <SMDP_SIGNATURE2>, "smdpCertificate" : #CERT_SM_DPpb_SIG } |
| R_AUTH_CLIENT_RETRY_OK_CC | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <TRANSACTION_ID_AC>, "profileMetadata" : #SMDP_METADATA_OP_PROF1, "smdpSigned2" : #SMDP_SIGNED2_CC_RETRY, "smdpSignature2" : <SMDP_SIGNATURE2>, "smdpCertificate" : #CERT_SM_DPpb_SIG } |
| R_AUTH_SERVER_DS_MATCH_ID_DEV_INFO | resp AuthenticateServerResponse ::authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #TEST_ROOT_DS_ADDRESS, serverChallenge <S_SMDS_CHALLENGE>, euiccInfo2 #R_EUICC_INFO2, -- check only that the field is present but not the values ctxParams1 #CTX_PARAMS1_MATCH_ID_DEV_INFO }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| R_AUTH_SERVER_DS_MATCH_ID_DEV_INFO_1 | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #TEST_DS_ADDRESS1, serverChallenge <S_SMDS_CHALLENGE>, euiccInfo2 #R_EUICC_INFO2, -- check only that the field is present but not the values ctxParams1 #CTX_PARAMS1_MATCH_ID_DEV_INFO }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| R_AUTH_SERVER_MATCH_ID_DEV_INFO | resp AuthenticateServerResponse ::= authenticateResponseOk { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #TEST_DP_ADDRESS1, serverChallenge <S_SMDP_CHALLENGE>, euiccInfo2 #R_EUICC_INFO2, -- check only that the field is present but not the values ctxParams1 #CTX_PARAMS1_MATCH_ID_DEV_INFO }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| R_GET_BPP_RESP_OP1_PPK (Pre-generated PPP for Profiles) | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId": <TRANSACTION_ID_GBPP>, "boundProfilePackage" : BoundProfilePackage { #INIT_SC_PROF1, firstSequenceOf87 { <CONF_ISDP_PROF1_ENC> }, sequenceOf88 { <SMDP_METADATA_SEG_MAC> … <SMDP_METADATA_SEG_MAC> }, secondSequenceOf87 { <REPLACE_S_KEYS_REQ_ENC> }, sequenceOf86 { <PPP_OP_PROF1_SEG_PPK> … <PPP_OP_PROF1_SEG_PPK> } } } NOTE 1: boundProfilePackage is enconded as base64 therefore the test tool SHALL decode boundProfilePackage to access the ASN.1. NOTE 2: For sequenceOf88 there will be only one or two '88' TLV segments depending on the size of StoreMetadata. |
| R_GET_BPP_RESP_OP1_SK (Dynamically-generated PPP for Profiles) | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId": <TRANSACTION_ID_GBPP>, "boundProfilePackage" : BoundProfilePackage { #INIT_SC_PROF1, firstSequenceOf87 { <CONF_ISDP_PROF1_ENC> }, sequenceOf88 { <SMDP_METADATA_SEG_MAC> … <SMDP_METADATA_SEG_MAC> }, sequenceOf86 { <PPP_OP_PROF1_SEG_SK> … <PPP_OP_PROF1_SEG_SK> } } } NOTE 1: boundProfilePackage is enconded as base64 therefore the test tool SHALL decode boundProfilePackage to access the ASN.1. NOTE 2: For sequenceOf88 there will be only one or two '88' TLV segments depending on the size of StoreMetadata. |
| R_HTTP_204_OK | HTTP/1.1 204 No Content X-Admin-Protocol: gsma/rsp/v#RSP_SVN NOTE: If the HTTP response is being received from the server under test, then the "Content-type" header MAY be present. |
| R_INITIATE_AUTH_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <TRANSACTION_ID_IA>, "serverSigned1" : #SERVER_SIGNED1, "serverSignature1" : <SERVER_SIGNATURE1>, "euiccCiPKIdTobeUsed" : #CI_PKI_ID1, "serverCertificate" : #CERT_SM_XXauth_SIG } |
| R_INITIATE_AUTH_OK_2 | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <TRANSACTION_ID_2>, "serverSigned1" : #SERVER_SIGNED1_2, "serverSignature1" : <SERVER_SIGNATURE1_2>, "euiccCiPKIdTobeUsed" : #CI_PKI_ID1, "serverCertificate" : #CERT_SM_XXauth_SIG } |
| SERVER_SIGNED1 | For InitiateAuthentication testing XX = IA, and for AuthenticateClient testing XX = AC: ss1 ServerSigned1 ::= { transactionId <TRANSACTION_ID_SIGNED_IA>, euiccChallenge #S_EUICC_CHALLENGE, serverAddress #SERVER_ADDRESS, serverChallenge <SERVER_CHALLENGE> } |
| SERVER_SIGNED1_2 | ss1_2 ServerSigned1 ::= { transactionId <TRANSACTION_ID_SIGNED_2>, euiccChallenge #S_EUICC_CHALLENGE_2, serverAddress #SERVER_ADDRESS, serverChallenge <SERVER_CHALLENGE_2> } |
| SMDP_METADATA_ALL_NOTIF | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #IUT_SM_DP_ADDRESS } } } |
| SMDP_METADATA_OP_PROF1 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileClass operational } |
| SMDP_METADATA_OP_PROF1_2_SEG | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1_2_SEG, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #IUT_SM_DP_ADDRESS } }, profileOwner { mccMnc #MCC_MNC1 } } |
| SMDP_METADATA_OP_PROF3 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF3, serviceProviderName #SP_NAME3, profileName #NAME_OP_PROF3, profileClass operational, profileOwner { mccMnc #MCC_MNC2 }, profilePolicyRules { ppr2 } } |
| SMDP_SIGNED2 | smdpSigned2 SmdpSigned2 ::= { transactionId <TRANSACTION_ID_SIGNED_AC>, ccRequiredFlag FALSE } |
| SMDP_SIGNED2_CC | smdpSigned2 SmdpSigned2 ::= { transactionId <TRANSACTION_ID_SIGNED_AC>, ccRequiredFlag TRUE } |
| SMDP_SIGNED2_CC_RETRY | smdpSigned2 SmdpSigned2 ::= { transactionId <TRANSACTION_ID_SIGNED_AC>, ccRequiredFlag TRUE, bppEuiccOtpk <BPP_OTPK_EUICC_AKA> } |
| SMDP_SIGNED2_RETRY | smdpSigned2 SmdpSigned2 ::= { transactionId <TRANSACTION_ID_SIGNED_AC>, ccRequiredFlag FALSE, bppEuiccOtpk <BPP_OTPK_EUICC_AKA> } |

- D.3	ES10x Requests And Responses

- D.3.1	ES10x Requests

| Name | Content |
| --- | --- |
| AUTH_SMDP_MATCH_ID | req AuthenticateServerRequest ::= { serverSigned1 { transactionId <S_TRANSACTION_ID>, euiccChallenge <EUICC_CHALLENGE>, serverAddress #TEST_DP_ADDRESS1, serverChallenge <S_SMDP_CHALLENGE> }, serverSignature1 <S_SMDP_SIGNATURE1>, euiccCiPKIdToBeUsed <EUICC_CI_PK_ID_TO_BE_USED>, serverCertificate #CERT_S_SM_DPauth_SIG, ctxParams1 #CTX_PARAMS1_MATCH_ID } |
| AUTH_SMDP_IMEI | req AuthenticateServerRequest ::= { serverSigned1 { transactionId <S_TRANSACTION_ID>, euiccChallenge <EUICC_CHALLENGE>, serverAddress #TEST_DP_ADDRESS1, serverChallenge <S_SMDP_CHALLENGE> }, serverSignature1 <S_SMDP_SIGNATURE1>, euiccCiPKIdToBeUsed <EUICC_CI_PK_ID_TO_BE_USED>, serverCertificate #CERT_S_SM_DPauth_SIG, ctxParams1 #CTX_PARAMS1_IMEI } |
| AUTH_SMDP_INV_CERT | req AuthenticateServerRequest ::= { serverSigned1 { transactionId <S_TRANSACTION_ID>, euiccChallenge <EUICC_CHALLENGE>, serverAddress #TEST_DP_ADDRESS1, serverChallenge <S_SMDP_CHALLENGE> }, serverSignature1 <S_SMDP_SIGNATURE1>, euiccCiPKIdToBeUsed <EUICC_CI_PK_ID_TO_BE_USED>, serverCertificate #CERT_S_SM_DPauth_INV_SIGN, ctxParams1 #CTX_PARAMS1 } |
| AUTH_SMDP_INV_CURV | req AuthenticateServerRequest ::= { serverSigned1 { transactionId <S_TRANSACTION_ID>, euiccChallenge <EUICC_CHALLENGE>, serverAddress #TEST_DP_ADDRESS1, serverChallenge <S_SMDP_CHALLENGE> }, serverSignature1 <RANDOM_SM_DP+_SIGN>, euiccCiPKIdToBeUsed <EUICC_CI_PK_ID_TO_BE_USED>, serverCertificate #CERT_S_SM_DPauth_INV_CURVE, ctxParams1 #CTX_PARAMS1 } |
| AUTH_SMDP_INV_CHALLENGE | req AuthenticateServerRequest ::= { serverSigned1 { transactionId <S_TRANSACTION_ID>, euiccChallenge #S_EUICC_CHALLENGE, serverAddress #TEST_DP_ADDRESS1, serverChallenge <S_SMDP_CHALLENGE> }, serverSignature1 <S_SMDP_SIGNATURE1>, euiccCiPKIdToBeUsed <EUICC_CI_PK_ID_TO_BE_USED>, serverCertificate #CERT_S_SM_DPauth_SIG, ctxParams1 #CTX_PARAMS1 } |
| AUTH_SMDP_INV_OID | req AuthenticateServerRequest ::= { serverSigned1 { transactionId <S_TRANSACTION_ID>, euiccChallenge <EUICC_CHALLENGE>, serverAddress #TEST_DP_ADDRESS1, serverChallenge <S_SMDP_CHALLENGE> }, serverSignature1 <S_SMDP_SIGNATURE1>, euiccCiPKIdToBeUsed <EUICC_CI_PK_ID_TO_BE_USED>, serverCertificate #CERT_S_SM_DPpb_SIG, ctxParams1 #CTX_PARAMS1 } |
| AUTH_SMDS_IMEI | req AuthenticateServerRequest ::= { serverSigned1 { transactionId <S_TRANSACTION_ID>, euiccChallenge <EUICC_CHALLENGE>, serverAddress #TEST_ROOT_DS_ADDRESS, serverChallenge <S_SMDS_CHALLENGE> }, serverSignature1 <S_SMDS_SIGNATURE1>, euiccCiPKIdToBeUsed <EUICC_CI_PK_ID_TO_BE_USED>, serverCertificate #CERT_S_SM_DSauth_SIG, ctxParams1 #CTX_PARAMS1_EVENT_ID_IMEI } |
| AUTH_SMDS_INV_CERT | req AuthenticateServerRequest ::= { serverSigned1 { transactionId <S_TRANSACTION_ID>, euiccChallenge <EUICC_CHALLENGE>, serverAddress #TEST_ROOT_DS_ADDRESS, serverChallenge <S_SMDS_CHALLENGE> }, serverSignature1 <S_SMDS_SIGNATURE1>, euiccCiPKIdToBeUsed <EUICC_CI_PK_ID_TO_BE_USED>, serverCertificate #CERT_S_SM_DSauth_INV_SIGN, ctxParams1 #CTX_PARAMS1_EVENT_ID } |
| AUTH_SMDS_INV_CHALLENGE | req AuthenticateServerRequest ::= { serverSigned1 { transactionId <S_TRANSACTION_ID>, euiccChallenge #S_EUICC_CHALLENGE, serverAddress #TEST_ROOT_DS_ADDRESS, serverChallenge <S_SMDS_CHALLENGE> }, serverSignature1 <S_SMDS_SIGNATURE1>, euiccCiPKIdToBeUsed <EUICC_CI_PK_ID_TO_BE_USED>, serverCertificate #CERT_S_SM_DSauth_SIG, ctxParams1 #CTX_PARAMS1_EVENT_ID } |
| AUTH_SMDS_INV_CURV | req AuthenticateServerRequest ::= { serverSigned1 { transactionId <S_TRANSACTION_ID>, euiccChallenge <EUICC_CHALLENGE>, serverAddress #TEST_ROOT_DS_ADDRESS, serverChallenge <S_SMDS_CHALLENGE> }, serverSignature1 <RANDOM_SM_DS_SIGN>, euiccCiPKIdToBeUsed <EUICC_CI_PK_ID_TO_BE_USED>, serverCertificate #CERT_S_SM_DSauth_INV_CURVE, ctxParams1 #CTX_PARAMS1_EVENT_ID } |
| AUTHENTICATE_SMDP | req AuthenticateServerRequest ::= { serverSigned1 { transactionId <S_TRANSACTION_ID>, euiccChallenge <EUICC_CHALLENGE>, serverAddress #TEST_DP_ADDRESS1, serverChallenge <S_SMDP_CHALLENGE> }, serverSignature1 <S_SMDP_SIGNATURE1>, euiccCiPKIdToBeUsed <EUICC_CI_PK_ID_TO_BE_USED>, serverCertificate #CERT_S_SM_DPauth_SIG, ctxParams1 #CTX_PARAMS1 } |
| AUTHENTICATE_SMDS | req AuthenticateServerRequest ::= { serverSigned1 { transactionId <S_TRANSACTION_ID>, euiccChallenge <EUICC_CHALLENGE>, serverAddress #TEST_ROOT_DS_ADDRESS, serverChallenge <S_SMDS_CHALLENGE> }, serverSignature1 <S_SMDS_SIGNATURE1>, euiccCiPKIdToBeUsed <EUICC_CI_PK_ID_TO_BE_USED>, serverCertificate #CERT_S_SM_DSauth_SIG, ctxParams1 #CTX_PARAMS1_EVENT_ID } |
| CANCEL_SESSION_INV_TRANS_ID | req CancelSessionRequest ::={ transactionId <INVALID_TRANSACTION_ID>, reason endUserRejection } |
| CANCEL_SESSION_REJECT | req CancelSessionRequest ::={ transactionId <S_TRANSACTION_ID>, reason endUserRejection } |
| CANCEL_SESSION_POSTPONED | req CancelSessionRequest ::={ transactionId <S_TRANSACTION_ID>, reason postponed } |
| CANCEL_SESSION_TIMEOUT | req CancelSessionRequest ::={ transactionId <S_TRANSACTION_ID>, reason timeout } |
| CANCEL_SESSION_PPR | req CancelSessionRequest ::={ transactionId <S_TRANSACTION_ID>, reason pprNotAllowed } |
| CANCEL_SESSION_METADATA | req CancelSessionRequest ::={ transactionId <S_TRANSACTION_ID>, reason metadataMismatch } |
| CANCEL_SESSION_LOAD_BPP | req CancelSessionRequest ::={ transactionId <S_TRANSACTION_ID>, reason loadBppExecutionError } |
| CANCEL_SESSION_UNDEF | req CancelSessionRequest ::={ transactionId <S_TRANSACTION_ID>, reason undefinedReason } |
| EUICC_MEMORY_RESET | req EuiccMemoryResetRequest ::= { resetOptions { deleteOperationalProfiles, resetDefaultSmdpAddress } } |
| EUICC_MEMORY_RESET_DEF_SMDPADDRESS | req EuiccMemoryResetRequest ::= { resetOptions { resetDefaultSmdpAddress } } |
| EUICC_MEMORY_RESET_OP_PRO | req EuiccMemoryResetRequest ::= { resetOptions { deleteOperationalProfiles } } |
| GET_CONF_OP_PROF1 | opConfProf1Req ProfileInfoListRequest ::= { searchCriteria iccid: #ICCID_OP_PROF1, tagList '4FB8'H } |
| GET_EID | getEIDReq GetEuiccDataRequest ::= { tagList '5A'H } |
| GET_EID_INVALID | getEIDReq GetEuiccDataRequest ::= { tagList '6B'H } |
| GET_EUICC_CHALLENGE | request GetEuiccChallengeRequest ::= {} |
| GET_EUICC_CONFIGURED_ADDRESSES | request EuiccConfiguredAddressesRequest ::={} |
| GET_EUICC_INFO1 | request GetEuiccInfo1Request::= { } |
| GET_EUICC_INFO2 | request GetEuiccInfo2Request::= { } |
| GET_METADATA_OP_PROF1 | opConfProf1Req ProfileInfoListRequest ::= { searchCriteria iccid: #ICCID_OP_PROF1, tagList '5A9192939495B6B799'H } |
| GET_NEW_METADATA | getupdate1Req ProfileInfoListRequest ::= { searchCriteria iccid: #ICCID_OP_PROF1, tagList '9192939499'H -- names, icon and PPRs } |
| GET_NOTIF_CONF_OP_PROF1 | opConfProf1Req ProfileInfoListRequest ::= { searchCriteria iccid: #ICCID_OP_PROF1, tagList '5AB6'H } |
| GET_PPR_OP_PROF1 | opConfProf1Req ProfileInfoListRequest ::= { searchCriteria iccid: #ICCID_OP_PROF1, tagList '5A99'H } |
| GET_PROFILES_INFO_ALL | request ProfileInfoListRequest::= { } |
| GET_PROFILES_INFO_ICCID_TAGLIST1 | request ProfileInfoListRequest::= { searchCriteria iccid: #ICCID_OP_PROF1, tagList '9F70'H --state } |
| GET_PROFILES_INFO_ICCID_TAGLIST2 | request ProfileInfoListRequest::= { searchCriteria iccid: #ICCID_OP_PROF1, tagList '93'H --icon type } |
| GET_PROFILES_INFO_ICCID_TAGLIST3 | request ProfileInfoListRequest::= { searchCriteria iccid: #ICCID_OP_PROF1, tagList '95'H --Profile Class } |
| GET_PROFILES_INFO_ICCID_TAGLIST4 | request ProfileInfoListRequest::= { searchCriteria iccid: #ICCID_OP_PROF1, tagList 'B6'H --Notification configuration } |
| GET_PROFILES_INFO_ICCID_TAGLIST5 | request ProfileInfoListRequest::= { searchCriteria iccid: #ICCID_OP_PROF3, tagList '99'H --ppr } |
| GET_PROFILES_INFO_OPTAGLIST1 | request ProfileInfoListRequest::= { searchCriteria profileClass: operational, tagList '5A9F70'H -- ICCID and State } |
| GET_PROFILES_INFO_OPTAGLIST2 | request ProfileInfoListRequest::= { searchCriteria profileClass: operational, tagList '909F70'H --Nickname and State } |
| GET_PROFILES_INFO_OPTAGLIST3 | request ProfileInfoListRequest::= { searchCriteria profileClass: operational, tagList '9493'H --Icon, Icon type } |
| GET_PROFILES_INFO_OPTAGLIST4 | request ProfileInfoListRequest::= { searchCriteria profileClass: operational, tagList '949F70'H --Icon, state } |
| GET_PROFILES_INFO_PROFCLASS | request ProfileInfoListRequest::= { searchCriteria profileClass: operational } |
| GET_PROFILES_INFO_TAGLIST_ICCID | request ProfileInfoListRequest::= { tagList '5A'H } |
| GET_PROFILES_INFO_TAGLIST_ICON | request ProfileInfoListRequest::= { tagList '94'H } |
| GET_PROFILES_INFO_TAGLIST_ISDPAID | request ProfileInfoListRequest::= { tagList '4F'H } |
| GET_PROFILES_INFO_TAGLIST_PROFILE_NAME | request ProfileInfoListRequest::= { tagList '92'H } |
| GET_PROFILES_INFO_TAGLIST_PROFILE_NICKNAME | request ProfileInfoListRequest::= { tagList '90'H } |
| GET_PROFILES_INFO_TAGLIST_PROFILE_OWNER | request ProfileInfoListRequest::= { tagList 'B7'H } |
| GET_PROFILES_INFO_TAGLIST_SMDP_PROP_DATA | request ProfileInfoListRequest::= { tagList 'B8'H } |
| GET_PROFILES_INFO_TAGLIST_SP_NAME | request ProfileInfoListRequest::= { tagList '91'H } |
| GET_PROFILES_INFO_TAGLIST1 | request ProfileInfoListRequest::= { tagList '5A9F70'H -- ICCID and State } |
| GET_PROFILES_INFO_TAGLIST2 | request ProfileInfoListRequest::= { tagList '909F70'H --Nickname and State } |
| GET_PROFILES_INFO_TAGLIST3 | request ProfileInfoListRequest::= { tagList '9493'H --Icon, Icon type } |
| GET_PROFILES_INFO_TAGLIST4 | request ProfileInfoListRequest::= { tagList '949F70'H --Icon, state } |
| GET_PROFILES_OWNERS | request ProfileInfoListRequest::= { tagList 'B7'H } |
| GET_RAT | request GetRatRequest ::={} |
| LIST_NOTIF_ALL | request ListNotificationRequest ::= { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete } } |
| LIST_NOTIF_OMITTED | request ListNotificationRequest ::= {} |
| LIST_NOTIF_NONE | request ListNotificationRequest ::= { profileManagementOperation {} } |
| LIST_NOTIF_INSTALL | request ListNotificationRequest ::= { profileManagementOperation { notificationInstall } } |
| LIST_NOTIF_ENABLE | request ListNotificationRequest ::= { profileManagementOperation { notificationEnable } } |
| LIST_NOTIF_DISABLE | request ListNotificationRequest ::= { profileManagementOperation { notificationDisable } } |
| LIST_NOTIF_DELETE | request ListNotificationRequest ::= { profileManagementOperation { notificationDelete } } |
| LIST_NOTIF_INSTALL_ENABLE | request ListNotificationRequest ::= { profileManagementOperation { notificationInstall, notificationEnable } } |
| LIST_NOTIF_DISABLE_DELETE | request ListNotificationRequest ::= { profileManagementOperation { notificationDisable, notificationDelete } } |
| LIST_NOTIF_DISABLE_ENABLE | request ListNotificationRequest ::= { profileManagementOperation { notificationDisable, notificationEnable } } |
| LIST_NOTIF_INSTALL_ENABLE_DISABLE | request ListNotificationRequest ::= { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable } } |
| LIST_NOTIF_ENABLE_DISABLE_DELETE | request ListNotificationRequest ::= { profileManagementOperation { notificationEnable, notificationDisable, notificationDelete } } |
| METADATA_EN_DI_DE_NOTIFS | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, notificationConfigurationInfo { { profileManagementOperation { notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS1 }, { profileManagementOperation { notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS2 } } } |
| PREP_DOWNLOAD_INVALID_CC | req PrepareDownloadRequest ::= { smdpSigned2 { transactionId <S_TRANSACTION_ID>, ccRequiredFlag TRUE }, smdpSignature2 <S_SM_DP+_SIGNATURE2>, smdpCertificate #CERT_S_SM_DPpb_SIG } |
| RETRIEVE_NOTIF_ALL | request RetrieveNotificationsListRequest ::= { searchCriteria profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete } } |
| RETRIEVE_NOTIF_OMITTED | request RetrieveNotificationsListRequest ::= { } |
| RETRIEVE_NOTIF_NONE | request RetrieveNotificationsListRequest ::= { searchCriteria profileManagementOperation {} } |
| RETRIEVE_NOTIF_INSTALL | request RetrieveNotificationsListRequest::= { searchCriteria profileManagementOperation { notificationInstall } } |
| RETRIEVE_NOTIF_ENABLE | request RetrieveNotificationsListRequest::= { searchCriteria profileManagementOperation { notificationEnable } } |
| RETRIEVE_NOTIF_DISABLE | request RetrieveNotificationsListRequest::= { searchCriteria profileManagementOperation { notificationDisable } } |
| RETRIEVE_NOTIF_DELETE | request RetrieveNotificationsListRequest::= { searchCriteria profileManagementOperation { notificationDelete } } |
| RETRIEVE_NOTIF_INSTALL_ENABLE | request RetrieveNotificationsListRequest ::= { searchCriteria profileManagementOperation { notificationInstall, notificationEnable } } |
| RETRIEVE_NOTIF_DISABLE_DELETE | request RetrieveNotificationsListRequest ::= { searchCriteria profileManagementOperation { notificationDisable, notificationDelete } } |
| RETRIEVE_NOTIF_DISABLE_ENABLE | request RetrieveNotificationsListRequest ::= { searchCriteria profileManagementOperation { notificationDisable, notificationEnable } } |
| RETRIEVE_NOTIF_INSTALL_ENABLE_DISABLE | request RetrieveNotificationsListRequest ::= { searchCriteria profileManagementOperation { notificationInstall, notificationEnable, notificationDisable } } |
| PREP_DOWN_INV_CURVE | req PrepareDownloadRequest ::= { smdpSigned2 { transactionId <S_TRANSACTION_ID>, ccRequiredFlag FALSE }, smdpSignature2 <RANDOM_SM_DP+_SIGN>, smdpCertificate #CERT_S_SM_DPpb_INV_CURVE } |
| PREP_DOWNLOAD_CERT_SMDP2 | req PrepareDownloadRequest ::= { smdpSigned2 { transactionId <S_TRANSACTION_ID>, ccRequiredFlag FALSE }, smdpSignature2 <S_SM_DP+_SIGNATURE2>, smdpCertificate #CERT_S_SM_DP2pb_SIG } |
| PREP_DOWNLOAD_INV_CERT | req PrepareDownloadRequest ::= { smdpSigned2 { transactionId <S_TRANSACTION_ID>, ccRequiredFlag FALSE }, smdpSignature2 <S_SM_DP+_SIGNATURE2>, smdpCertificate #CERT_S_SM_DPpb_INV_SIGN } |
| PREP_DOWNLOAD_INV_OID | req PrepareDownloadRequest ::= { smdpSigned2 { transactionId <S_TRANSACTION_ID>, ccRequiredFlag FALSE }, smdpSignature2 <S_SM_DP+_SIGNATURE2>, smdpCertificate #CERT_S_SM_DPauth_SIG } |
| PREP_DOWNLOAD_INV_SIGN | req PrepareDownloadRequest ::= { smdpSigned2 { transactionId <S_TRANSACTION_ID>, ccRequiredFlag FALSE }, smdpSignature2 <S_SM_DP+_SIGNATURE2>, smdpCertificate #CERT_S_SM_DPpb_SIG } NOTE: The <S_SM_DP+_SIGNATURE2> SHALL NOT be computed using the #SK_S_SM_DPpb_SIG but SHALL have the same length as for a valid signature. |
| PREP_DOWNLOAD_INV_TRANS_ID | req PrepareDownloadRequest ::= { smdpSigned2 { transactionId <INVALID_TRANSACTION_ID>, ccRequiredFlag FALSE }, smdpSignature2 <S_SM_DP+_SIGNATURE2>, smdpCertificate #CERT_S_SM_DPpb_SIG } |
| PREP_DOWNLOAD_NO_AUTH | req PrepareDownloadRequest ::= { smdpSigned2 { transactionId <S_TRANSACTION_ID>, ccRequiredFlag FALSE }, smdpSignature2 <RANDOM_SM_DP+_SIGN>, smdpCertificate #CERT_S_SM_DPpb_SIG } |
| PREP_DOWNLOAD_NO_CC | req PrepareDownloadRequest ::= { smdpSigned2 { transactionId <S_TRANSACTION_ID>, ccRequiredFlag FALSE }, smdpSignature2 <S_SM_DP+_SIGNATURE2>, smdpCertificate #CERT_S_SM_DPpb_SIG } |
| PREP_DOWNLOAD_RETRY_CC | req PrepareDownloadRequest ::= { smdpSigned2 { transactionId <S_TRANSACTION_ID>, ccRequiredFlag TRUE, bppEuiccOtpk <OTPK_EUICC_AKA> }, smdpSignature2 <S_SM_DP+_SIGNATURE2>, hashCc <S_HASHED_CC>, smdpCertificate #CERT_S_SM_DPpb_SIG } |
| PREP_DOWNLOAD_WITH_CC | req PrepareDownloadRequest ::= { smdpSigned2 { transactionId <S_TRANSACTION_ID>, ccRequiredFlag TRUE }, smdpSignature2 <S_SM_DP+_SIGNATURE2>, hashCc <S_HASHED_CC>, smdpCertificate #CERT_S_SM_DPpb_SIG } |
| SET_EUICC_CONFIGURED_ADDRESS_1 | request SetDefaultDpAddressRequest::={ defaultDpAddress #TEST_DP_ADDRESS1 } |
| SET_EUICC_CONFIGURED_ADDRESS_2 | request SetDefaultDpAddressRequest::={ defaultDpAddress #TEST_DP_ADDRESS2 } |
| SET_EUICC_CONFIGURED_ADDRESS_EMPTY | request SetDefaultDpAddressRequest::={ defaultDpAddress "" } |
| SET_NICKNAME_EMPTY_OP_PROF1 | setNicknameReq SetNicknameRequest ::= { iccid #ICCID_OP_PROF1, profileNickname "" } |
| SET_NICKNAME_ICCID_UNKNOWN | setNicknameReq SetNicknameRequest ::= { iccid #ICCID_UNKNOWN, profileNickname #NICKNAME2 } |
| SET_NICKNAME_OP_PROF1 | setNicknameReq SetNicknameRequest ::= { iccid #ICCID_OP_PROF1, profileNickname #NICKNAME2 } |

- D.3.2	ES10x Responses

| Name | Content |
| --- | --- |
| NOTIF_METADATA_DELETE1 (NotificationMetadata) | { seqNumber <NOTIF_SEQ_NO_DE1>, profileManagementOperation { notificationDelete }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 } |
| NOTIF_METADATA2_DELETE1 (NotificationMetadata) | { seqNumber <NOTIF_SEQ_NO2_DE1>, profileManagementOperation { notificationDelete }, notificationAddress #TEST_DP_ADDRESS2, iccid #ICCID_OP_PROF1 } |
| NOTIF_METADATA_DISABLE1 (NotificationMetadata) | { seqNumber <NOTIF_SEQ_NO_DI1>, profileManagementOperation { notificationDisable }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 } |
| NOTIF_METADATA2_DISABLE1 (NotificationMetadata) | { seqNumber <NOTIF_SEQ_NO2_DI1>, profileManagementOperation { notificationDisable }, notificationAddress #TEST_DP_ADDRESS2, iccid #ICCID_OP_PROF1 } |
| NOTIF_METADATA_ENABLE1 (NotificationMetadata) | { seqNumber <NOTIF_SEQ_NO_EN1>, profileManagementOperation { notificationEnable }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 } |
| NOTIF_METADATA2_ENABLE1 (NotificationMetadata) | { seqNumber <NOTIF_SEQ_NO2_EN1>, profileManagementOperation { notificationEnable }, notificationAddress #TEST_DP_ADDRESS2, iccid #ICCID_OP_PROF1 } |
| NOTIF_METADATA_ENABLE2 (NotificationMetadata) | { seqNumber <NOTIF_SEQ_NO_EN2>, profileManagementOperation { notificationEnable }, notificationAddress #TEST_DP_ADDRESS2, iccid #ICCID_OP_PROF2 } |
| NOTIF_METADATA_INSTALL1 (NotificationMetadata) | { seqNumber <NOTIF_SEQ_NO_IN1>, profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 } |
| NOTIF_METADATA_INSTALL1_PIR (NotificationMetadata) | { seqNumber <NOTIF_SEQ_NO_IN1_PIR>, profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 } |
| NOTIF_METADATA_INSTALL2 (NotificationMetadata) | { seqNumber <NOTIF_SEQ_NO_IN2>, profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS2, iccid #ICCID_OP_PROF2 } |
| NOTIF_METADATA_INSTALL2_PIR (NotificationMetadata) | { seqNumber <NOTIF_SEQ_NO_IN2_PIR>, profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS2, iccid #ICCID_OP_PROF2 } |
| PPR1_WITH_OWNER_GID (ProfilePolicyAuthorisationRule) | { pprIds { ppr1 }, allowedOperators { { mccMnc #MCC_MNC2, gid1 #GID1, gid2 #GID2 } }, pprFlags {consentRequired} } |
| PPR1_WITHOUT_GID (ProfilePolicyAuthorisationRule) | { pprIds { ppr1 }, allowedOperators { { mccMnc #MCC_MNC4 } }, pprFlags {consentRequired} } |
| PPR2_WITHOUT_CONSENT (ProfilePolicyAuthorisationRule) | { pprIds { ppr2 }, allowedOperators { { mccMnc '92EEEE'H, gid1 ''H, gid2 ''H} }, pprFlags { } } |
| PPRS_ALLOWED (ProfilePolicyAuthorisationRule) | { pprIds { ppr1, ppr2 }, allowedOperators { { mccMnc 'EEEEEE'H, gid1 ''H, gid2 ''H} }, pprFlags {consentRequired} } |
| PROFILE_INFO1 (ProfileInfo) | { iccid #ICCID_OP_PROF1, isdpAid <ISD_P_AID1>, profileState enabled, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational } |
| PROFILE_INFO1_DISABLED (ProfileInfo) | { iccid #ICCID_OP_PROF1, isdpAid <ISD_P_AID1>, profileState disabled, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational } |
| PROFILE_INFO2 (ProfileInfo) | { iccid #ICCID_OP_PROF2, isdpAid <ISD_P_AID2>, profileState disabled, serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, iconType png, icon #ICON_OP_PROF2, profileClass operational } |
| PROFILE_INFO2_ENABLED (ProfileInfo) | { iccid #ICCID_OP_PROF2, isdpAid <ISD_P_AID2>, profileState enabled, serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, iconType png, icon #ICON_OP_PROF2, profileClass operational } |
| PROFILE_INFO3 (ProfileInfo) | { iccid #ICCID_OP_PROF3, isdpAid <ISD_P_AID3>, profileState disabled, profileNickname #NICKNAME3, serviceProviderName #SP_NAME3, profileName #NAME_OP_PROF3, iconType png, icon #ICON_OP_PROF3, profileClass operational } |
| PROFILE_INFO4 (ProfileInfo) | { iccid #ICCID_OP_PROF4, isdpAid <ISD_P_AID4>, profileState disabled, serviceProviderName #SP_NAME4, profileName #NAME_OP_PROF4, iconType png, icon #ICON_OP_PROF4, profileClass operational } |
| PROFILE_INFO4_ENABLED (ProfileInfo) | { iccid #ICCID_OP_PROF4, isdpAid <ISD_P_AID4>, profileState enabled, serviceProviderName #SP_NAME4, profileName #NAME_OP_PROF4, iconType png, icon #ICON_OP_PROF4, profileClass operational } |
| PROFILES_INFO_ICCID_TAGLIST1 (ProfileInfo) | {profileState enabled} |
| PROFILES_INFO_ICCID_TAGLIST2 (ProfileInfo) | {iconType png} |
| PROFILES_INFO_ICCID_TAGLIST3 (ProfileInfo) | {profileClass operational } |
| PROFILES_INFO_ICCID_TAGLIST4 (ProfileInfo) | notificationConfigurationInfo from #METADATA_OP_PROF1 |
| PROFILES_INFO_ICCID_TAGLIST5 (ProfileInfo) | profilePolicyRules from #METADATA_OP_PROF3 |
| PROFILES_INFO_TAGLIST_ICCID (ProfileInfo) | {iccid #ICCID_OP_PROF1}, {iccid #ICCID_OP_PROF2}, {iccid #ICCID_OP_PROF3} |
| PROFILES_INFO_TAGLIST_ICON (ProfileInfo) | {icon #ICON_OP_PROF1}, {icon #ICON_OP_PROF2}, {icon #ICON_OP_PROF3} |
| PROFILES_INFO_TAGLIST_ISDPAID (ProfileInfo) | {isdpAid <ISD_P_AID1>}, {isdpAid <ISD_P_AID2>}, {isdpAid <ISD_P_AID3>} |
| PROFILES_INFO_TAGLIST_PROFILE_NAME (ProfileInfo) | {profileName #NAME_OP_PROF1}, {profileName #NAME_OP_PROF2}, {profileName #NAME_OP_PROF3} |
| PROFILES_INFO_TAGLIST_PROFILE_NICKNAME (ProfileInfo) | {profileNickname #NICKNAME3} |
| PROFILES_INFO_TAGLIST_PROFILE_OWNER (ProfileInfo) | {profileOwner #OWNER_OP_PROF1}, {profileOwner #OWNER_OP_PROF2}, {profileOwner #OWNER_OP_PROF2} |
| PROFILES_INFO_TAGLIST_SMDP_PROP_DATA (ProfileInfo) | {dpProprietaryData #SMDP_PROP_DATA1} |
| PROFILES_INFO_TAGLIST_SP_NAME (ProfileInfo) | {serviceProviderName #SP_NAME1}, {serviceProviderName #SP_NAME2}, {serviceProviderName #SP_NAME3} |
| PROFILES_INFO_TAGLIST1 (ProfileInfo) | { iccid #ICCID_OP_PROF1, profileState enabled }, { iccid #ICCID_OP_PROF2, profileState disabled }, { iccid #ICCID_OP_PROF3, profileState disabled } |
| PROFILES_INFO_TAGLIST2 (ProfileInfo) | { profileState enabled }, { profileState disabled }, { profileState disabled, profileNickname #NICKNAME3 } |
| PROFILES_INFO_TAGLIST3 (ProfileInfo) | { iconType png, icon #ICON_OP_PROF1 }, { iconType png, icon #ICON_OP_PROF2 }, { iconType png, icon #ICON_OP_PROF3 } |
| PROFILES_INFO_TAGLIST4 (ProfileInfo) | { profileState enabled, icon #ICON_OP_PROF1 }, { profileState disabled, icon #ICON_OP_PROF2 }, { profileState disabled, icon #ICON_OP_PROF3 } |
| R_AUTH_SMDP_MATCH_ID | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #TEST_DP_ADDRESS1, serverChallenge <S_SMDP_CHALLENGE>, euiccInfo2 #R_EUICC_INFO2, -- check only that the field is present and has a valid TLV asn.1 structure ctxParams1 #CTX_PARAMS1_MATCH_ID }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| R_AUTH_SMDP_IMEI | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #TEST_DP_ADDRESS1, serverChallenge <S_SMDP_CHALLENGE>, euiccInfo2 #R_EUICC_INFO2, -- check only that the field is present and has a valid TLV asn.1 structure ctxParams1 #CTX_PARAMS1_IMEI }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| R_AUTH_SERVER_INV_CERT | resp AuthenticateServerResponse ::= authenticateResponseError : { transactionId <S_TRANSACTION_ID>, authenticateErrorCode invalidCertificate } |
| R_AUTH_SERVER_INV_SIGN | resp AuthenticateServerResponse ::= authenticateResponseError : { transactionId <S_TRANSACTION_ID>, authenticateErrorCode invalidSignature } |
| R_AUTH_SERVER_INV_CURV | resp AuthenticateServerResponse ::= authenticateResponseError : { transactionId <S_TRANSACTION_ID>, authenticateErrorCode unsupportedCurve } |
| R_AUTH_SERVER_INV_CHALLENGE | resp AuthenticateServerResponse ::= authenticateResponseError : { transactionId <S_TRANSACTION_ID>, authenticateErrorCode euiccChallengeMismatch } |
| R_AUTH_SERVER_INV_CI | resp AuthenticateServerResponse ::= authenticateResponseError : { transactionId <S_TRANSACTION_ID>, authenticateErrorCode ciPKUnknown } |
| R_AUTH_SERVER_INV_OID | resp AuthenticateServerResponse ::= authenticateResponseError : { transactionId <S_TRANSACTION_ID>, authenticateErrorCode invalidOid } |
| R_AUTH_SERVER_NO_SESSION | resp AuthenticateServerResponse ::= authenticateResponseError : { transactionId <S_TRANSACTION_ID>, authenticateErrorCode noSessionContext } |
| R_AUTH_SMDS_IMEI | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #TEST_ROOT_DS_ADDRESS, serverChallenge <S_SMDS_CHALLENGE>, euiccInfo2 #R_EUICC_INFO2, -- check only that the field is present and has a valid TLV asn.1 structure ctxParams1 #CTX_PARAMS1_EVENT_ID_IMEI }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| R_AUTHENTICATE_SMDP | resp AuthenticateServerResponse ::= authenticateResponseOk: { euiccSigned1 #EUICC_SIGNED1, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| R_AUTHENTICATE_SMDS | resp AuthenticateServerResponse ::= authenticateResponseOk: { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #TEST_ROOT_DS_ADDRESS, serverChallenge <S_SMDS_CHALLENGE>, euiccInfo2 #R_EUICC_INFO2, -- check only that the field is present and has a valid TLV asn.1 structure ctxParams1 #CTX_PARAMS1_EVENT_ID }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| R_CANCEL_SESSION_INV_TRANS_ID | resp CancelSessionResponse ::= cancelSessionResponseError : invalidTransactionId |
| R_CANCEL_SESSION_METADATA | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #S_SM_DP+_OID, reason metadataMismatch }, euiccCancelSessionSignature <EUICC_CS_SIGNATURE> } |
| R_CANCEL_SESSION_REJ | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #S_SM_DP+_OID, reason endUserRejection }, euiccCancelSessionSignature <EUICC_CS_SIGNATURE> } |
| R_CANCEL_SESSION_POSTPONED | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #S_SM_DP+_OID, reason postponed }, euiccCancelSessionSignature <EUICC_CS_SIGNATURE> } |
| R_CANCEL_SESSION_TIMEOUT | resp CancelSessionResponse ::= cancelSessionResponseOk { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #S_SM_DP+_OID, reason timeout }, euiccCancelSessionSignature <EUICC_CS_SIGNATURE> } |
| R_CANCEL_SESSION_PPR | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #S_SM_DP+_OID, reason pprNotAllowed }, euiccCancelSessionSignature <EUICC_CS_SIGNATURE> } |
| R_CANCEL_SESSION_LOAD_BPP | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #S_SM_DP+_OID, reason loadBppExecutionError }, euiccCancelSessionSignature <EUICC_CS_SIGNATURE> } |
| R_CANCEL_SESSION_UNDEF | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #S_SM_DP+_OID, reason undefinedReason }, euiccCancelSessionSignature <EUICC_CS_SIGNATURE> } |
| R_CHALLENGE | response GetEuiccChallengeResponse ::= { euiccChallenge <EUICC_CHALLENGE> } |
| R_CONF_OP_PROF1 | resp ProfileInfoListResponse ::= profileInfoListOk :{ { isdpAid <ISD_P_AID>, dpProprietaryData { dpOid #S_SM_DP+_OID, additionalSmdpData #ADDITIONAL_SMDP_DATA_MAX_LENGTH } } } -- NOTE: Instead of DpProprietaryData ::= SEQUENCE { dpOid OBJECT IDENTIFIER -- additional data objects defined by the -- SM-DP+ MAY follow } -- the following structure is used to test the -- DpProprietaryData size: DpProprietaryData ::= SEQUENCE { dpOid OBJECT IDENTIFIER, additionalSmdpData OCTET STRING OPTIONAL } |
| R_DEFAULT_RAT | response GetRatResponse ::= { rat { #PPRS_ALLOWED } } |
| R_DELETE_PROFILE_DISALLOWEDBYPOLICY | respDelProf DeleteProfileResponse ::= { deleteResult disallowedByPolicy } |
| R_DELETE_PROFILE_NOTDISABLESTATE | respDelProf DeleteProfileResponse ::= { deleteResult profileNotInDisabledState } |
| R_DELETE_PROFILE_OK | respDelProf DeleteProfileResponse ::= { deleteResult ok } |
| R_DELETE_PROFILE_ICCID_ISDP_NOTFOUND | resp DeleteProfileResponse ::= { deleteResult iccidOrAidNotFound } |
| R_DISABLE_PROFILE_DISALLOWEDbyPOLICY | resp DisableProfileResponse ::= { disableResult disallowedByPolicy } |
| R_DISABLE_PROFILE_ICCID_ISDP_NOTFOUND | resp DisableProfileResponse ::= { disableResult iccidOrAidNotFound } |
| R_DISABLE_PROFILE_NOT_ENABLE_STATE | resp DisableProfileResponse ::= { disableResult profileNotInEnabledState } |
| R_DISABLE_PROFILE_OK | resp DisableProfileResponse ::= { disableResult ok } |
| R_ENABLE_PROFILE_ICCID_ISDP_NOTFOUND | respEnaPro EnableProfileResponse ::= { enableResult iccidOrAidNotFound } |
| R_ENABLE_PROFILE_NOT_DISABLE_STATE | respEnaPro EnableProfileResponse ::= { enableResult profileNotInDisabledState } |
| R_ENABLE_PROFILE_DISALLOWEDbyPOLICY | respEnaPro EnableProfileResponse ::= { enableResult disallowedByPolicy } |
| R_ENABLE_PROFILE_OK | resp EnableProfileResponse ::= { enableResult ok } |
| R_ES10a_GECA_DS | response EuiccConfiguredAddressesResponse ::= { -- defaultDpAddress SHALL not be present rootDsAddress #TEST_ROOT_DS_ADDRESS } |
| R_ES10a_GECA_DS_DP_1 | response EuiccConfiguredAddressesResponse ::= { defaultDpAddress #TEST_DP_ADDRESS1, rootDsAddress #TEST_ROOT_DS_ADDRESS } |
| R_ES10a_GECA_DS_DP_2 | response EuiccConfiguredAddressesResponse ::= { defaultDpAddress #TEST_DP_ADDRESS2, rootDsAddress #TEST_ROOT_DS_ADDRESS } |
| R_ES10a_SD_DP_A_OK | response SetDefaultDpAddressResponse::= { setDefaultDpAddressResult ok } |
| R_EUICC_INFO1 | response EUICCInfo1::= { svn #RSP_SVN_H, -- for device testing, check only that the field is present and has a valid TLV asn.1 structure euiccCiPKIdListForVerification <EUICC_CI_PK_ID_LIST_FOR_VERIFICATION>, euiccCiPKIdListForSigning <EUICC_CI_PK_ID_LIST_FOR_SIGNING> } |
| R_EUICC_INFO2 | response EUICCInfo2::= { profileVersion #IUT_SIMA_VERSION, svn #RSP_SVN_H, euiccFirmwareVer #IUT_EUICC_FIRMWARE_VER, extCardResource <EXT_CARD_RESOURCE>, uiccCapability #IUT_UICC_CAPABILITY, ts102241Version #IUT_TS102241_VERSION, globalplatformVersion #IUT_GLOBALPLATFORM_VERSION, rspCapability <EUICC_RSP_CAPABILITY>, euiccCiPKIdListForVerification <EUICC_CI_PK_ID_LIST_FOR_VERIFICATION>, euiccCiPKIdListForSigning <EUICC_CI_PK_ID_LIST_FOR_SIGNING>, euiccCategory #IUT_EUICC_CATEGORY, -- OPTIONAL forbiddenProfilePolicyRules <PPR_IDS>, -- OPTIONAL ppVersion #IUT_PP_VERSION, sasAcreditationNumber #IUT_SAS_ACCREDITATION_NUMBER, certificationDataObject { platformLabel #IUT_PLATFORM_LABEL, discoveryBaseURL #IUT_DLOA_URL },-- OPTIONAL treProperties <TRE_PROPERTIES>, -- OPTIONAL treProductReference <TRE_REFERENCE>, -- OPTIONAL additionalEuiccProfilePackageVersions #IUT_EUICC_ADD_PP_VERSIONS -- OPTIONAL /*If no additional eUICC Profile Package version is supported by the eUICC the additionalEuiccProfilePackageVersions field is expected to be absent, or present with an empty content*/ } |
| R_EUICC_MEMORY_RESET_OK | resp EuiccMemoryResetResponse ::= { resetResult ok } |
| R_GET_UPDATE_N1 | resp ProfileInfoListResponse ::= profileInfoListOk :{ { serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profilePolicyRules { ppr2 } } } |
| R_GET_UPDATE_N2 | resp ProfileInfoListResponse ::= profileInfoListOk :{ { serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType jpg, icon #ICON_JPG, profilePolicyRules { ppr1 } } } |
| R_GET_UPDATE_N3 | resp ProfileInfoListResponse ::= profileInfoListOk :{ { serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, iconType png, icon #ICON_OP_PROF1 -- profilePolicyRules SHALL not be present } } |
| R_GET_UPDATE_N4 | resp ProfileInfoListResponse ::= profileInfoListOk :{ { -- serviceProviderName SHALL not be present -- profileName SHALL not be present iconType png, icon #ICON_OP_PROF1 -- profilePolicyRules SHALL not be present } } |
| R_GET_UPDATE_N6 | resp ProfileInfoListResponse ::= profileInfoListOk :{ { serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, iconType png, icon #ICON_OP_PROF1 -- profilePolicyRules SHALL not be present } } |
| R_LIST_NOTIF_DI1_EN2 | response ListNotificationResponse ::= notificationMetadataList : { #NOTIF_METADATA_DISABLE1, #NOTIF_METADATA_ENABLE2 } |
| R_METADATA_UNCHANGED | resp ProfileInfoListResponse ::= profileInfoListOk :{ { serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profilePolicyRules {ppr1,ppr2} } } |
| R_PIR_DATA_MISMATCH | resp ProfileInstallationResult ::= { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS1, ... }, smdpOid #S_SM_DP+_OID, finalResult errorResult : { bppCommandId loadProfileElements, errorReason installFailedDueToDataMismatch, ... } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| R_PIR_OK_PROF9 | response ProfileInstallationResult ::= { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF9 }, smdpOid #S_SM_DP+_OID, finalResult successResult : { aid <ISD_P_AID>, simaResponse #SIMA_RESULT_OK } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| R_PIR_PPR_NOT_ALLOWED | resp ProfileInstallationResult ::= { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS1, ... }, smdpOid #S_SM_DP+_OID, finalResult errorResult : { bppCommandId storeMetadata, errorReason pprNotAllowed } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| R_GET_METADATA_OP_PROF1 | resp ProfileInfoListResponse ::= profileInfoListOk :{ { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS1 } }, profileOwner { mccMnc #MCC_MNC1 }, profilePolicyRules {ppr1} } } |
| R_GET_PROF_NOTIF_CONF | resp ProfileInfoListResponse ::= profileInfoListOk :{ { iccid #ICCID_OP_PROF1, notificationConfigurationInfo { { profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS3 }, { profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS2 }, { profileManagementOperation { notificationEnable }, notificationAddress #TEST_DP_ADDRESS2 }, { profileManagementOperation { notificationEnable }, notificationAddress #TEST_DP_ADDRESS3 }, { profileManagementOperation { notificationDisable }, notificationAddress #TEST_DP_ADDRESS3 }, { profileManagementOperation { notificationDisable }, notificationAddress #TEST_DP_ADDRESS4 }, { profileManagementOperation { notificationDelete }, notificationAddress #TEST_DP_ADDRESS1 }, { profileManagementOperation { notificationDelete }, notificationAddress #TEST_DP_ADDRESS3 } } } } |
| R_ISDR_SELECTION | resp ISDRProprietaryApplicationTemplate::= { svn #RSP_SVN_H } |
| R_LIST_NOTIF_DE1 | response ListNotificationResponse ::= notificationMetadataList : { #NOTIF_METADATA_DELETE1 } |
| R_LIST_NOTIF_DE1_DE1 | response ListNotificationResponse ::= notificationMetadataList : { #NOTIF_METADATA_DELETE1, #NOTIF_METADATA2_DELETE1 } |
| R_LIST_NOTIF_DI1 | response ListNotificationResponse ::= notificationMetadataList : { #NOTIF_METADATA_DISABLE1 } |
| R_LIST_NOTIF_DI1_DE1 | response ListNotificationResponse ::= notificationMetadataList : { #NOTIF_METADATA_DISABLE1, #NOTIF_METADATA_DELETE1 } |
| R_LIST_NOTIF_DI1_DI1 | response ListNotificationResponse ::= notificationMetadataList : { #NOTIF_METADATA_DISABLE1, #NOTIF_METADATA2_DISABLE1 } |
| R_LIST_NOTIF_EN1 | response ListNotificationResponse ::= notificationMetadataList: { #NOTIF_METADATA_ENABLE1 } |
| R_LIST_NOTIF_EN1_EN1 | response ListNotificationResponse ::= notificationMetadataList : { #NOTIF_METADATA_ENABLE1, #NOTIF_METADATA2_ENABLE1 } |
| R_LIST_NOTIF_IN1 | response ListNotificationResponse ::= notificationMetadataList: { #NOTIF_METADATA_INSTALL1 } |
| R_LIST_NOTIF_IN1_IN1_PIR | response ListNotificationResponse ::= notificationMetadataList: { #NOTIF_METADATA_INSTALL1, #NOTIF_METADATA_INSTALL1_PIR } |
| R_LIST_NOTIF_IN1_PIR | response ListNotificationResponse ::= notificationMetadataList: { #NOTIF_METADATA_INSTALL1_PIR } |
| R_LIST_NOTIF_IN1_EN1 | response ListNotificationResponse ::= notificationMetadataList: { #NOTIF_METADATA_INSTALL1, #NOTIF_METADATA_ENABLE1 } |
| R_LIST_NOTIF_IN1_PIR_EN1 | response ListNotificationResponse ::= notificationMetadataList: { #NOTIF_METADATA_INSTALL1_PIR, #NOTIF_METADATA_ENABLE1 } |
| R_LIST_NOTIF_IN2_PIR | response ListNotificationResponse ::= notificationMetadataList: { #NOTIF_METADATA_INSTALL2_PIR } |
| R_LIST_NOTIF_IN2_PIR_IN2 | response ListNotificationResponse ::= notificationMetadataList: { #NOTIF_METADATA_INSTALL2_PIR, #NOTIF_METADATA_INSTALL2 } |
| R_LIST_NOTIF_IN1_PIR_IN2_PIR | response ListNotificationResponse ::= notificationMetadataList: { #NOTIF_METADATA_INSTALL1_PIR, #NOTIF_METADATA_INSTALL2_PIR } |
| R_LIST_NOTIF_NONE | response ListNotificationResponse ::= notificationMetadataList: {} |
| R_LIST_NOTIF_UNDEFINED_ERROR | response ListNotificationResponse ::= listNotificationsResultError : undefinedError |
| R_LIST_NOTIF_EN1_IN2_PIR | response ListNotificationResponse ::= notificationMetadataList: { #NOTIF_METADATA_ENABLE1, #NOTIF_METADATA_INSTALL2_PIR } |
| R_PIR_ICCID_ALREADY_EXIST | resp ProfileInstallationResult ::= { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 }, smdpOid #S_SM_DP+_OID, finalResult errorResult : { bppCommandId storeMetadata, errorReason installFailedDueToIccidAlreadyExistsOnEuicc } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| R_PIR_INVALID_CRT | resp ProfileInstallationResult ::= { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS1 }, smdpOid #S_SM_DP+_OID, finalResult errorResult : { bppCommandId initialiseSecureChannel, errorReason unsupportedCrtValues } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| R_PIR_INVALID_DATA | resp ProfileInstallationResult ::= { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS1 }, smdpOid #S_SM_DP+_OID, finalResult errorResult : { bppCommandId configureISDP, errorReason incorrectInputValues } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| R_PIR_INVALID_OP_ID | resp ProfileInstallationResult ::= { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS1 }, smdpOid #S_SM_DP+_OID, finalResult errorResult : { bppCommandId initialiseSecureChannel, errorReason unsupportedRemoteOperationType } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| R_PIR_INVALID_SIGN | resp ProfileInstallationResult ::= { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS1 }, smdpOid #S_SM_DP+_OID, finalResult errorResult : { bppCommandId initialiseSecureChannel, errorReason invalidSignature } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| R_PIR_INVALID_TRANS_ID | resp ProfileInstallationResult ::= { profileInstallationResultData { transactionId <INVALID_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS1 }, smdpOid #S_SM_DP+_OID, finalResult errorResult : { bppCommandId initialiseSecureChannel, errorReason invalidTransactionId } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| R_PIR_METADATA_INVALID | resp ProfileInstallationResult ::= { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS1, … }, smdpOid #S_SM_DP+_OID, finalResult errorResult : { bppCommandId storeMetadata, errorReason scp03tStructureError OR incorrectInputValues } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| R_PIR_OK | response ProfileInstallationResult ::= { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 }, smdpOid #S_SM_DP+_OID, finalResult successResult : { aid <ISD_P_AID>, simaResponse #SIMA_RESULT_OK } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| R_PIR_PPK_INV | resp ProfileInstallationResult ::= { profileInstallationResultData { ... finalResult errorResult : { bppCommandId replaceSessionKeys, errorReason incorrectInputValues OR scp03tStructureError OR scp03tSecurityError } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| R_PIR_SECU_INVALID | resp ProfileInstallationResult ::= { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, … smdpOid #S_SM_DP+_OID, finalResult errorResult : { bppCommandId loadProfileElements, errorReason incorrectInputValues OR scp03tStructureError OR scp03tSecurityError … } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| R_PREP_DOWN_INV_CURVE | resp PrepareDownloadResponse ::= downloadResponseError : { transactionId <S_TRANSACTION_ID>, downloadErrorCode unsupportedCurve } |
| R_PREP_DOWN_INV_TRANS_ID | resp PrepareDownloadResponse ::= downloadResponseError : { transactionId <INVALID_TRANSACTION_ID>, downloadErrorCode invalidTransactionId } |
| R_PREP_DOWN_NO_SESSION | resp PrepareDownloadResponse ::= downloadResponseError : { transactionId <S_TRANSACTION_ID>, downloadErrorCode noSessionContext } |
| R_PREP_DOWNLOAD_INV_CERT | resp PrepareDownloadResponse ::= downloadResponseError : { transactionId <S_TRANSACTION_ID>, downloadErrorCode invalidCertificate } |
| R_PREP_DOWNLOAD_INV_SIGN | resp PrepareDownloadResponse ::= downloadResponseError : { transactionId <S_TRANSACTION_ID>, downloadErrorCode invalidSignature } |
| R_PREP_DOWNLOAD_NO_CC | resp PrepareDownloadResponse ::= downloadResponseOk : { euiccSigned2 { transactionId <S_TRANSACTION_ID>, euiccOtpk <OTPK_EUICC_AKA> }, euiccSignature2 <EUICC_SIGNATURE2> } |
| R_PREP_DOWNLOAD_WITH_CC | resp PrepareDownloadResponse ::= downloadResponseOk : { euiccSigned2 { transactionId <S_TRANSACTION_ID>, euiccOtpk <OTPK_EUICC_AKA>, hashCc <S_HASHED_CC> }, euiccSignature2 <EUICC_SIGNATURE2> } |
| R_RAT_WITH_OTHER_RULES | response GetRatResponse ::= { rat { #PPR1_WITH_OWNER_GID, #PPR1_WITHOUT_GID, #PPR2_WITHOUT_CONSENT, #PPRS_ALLOWED } } |
| R_REMOVE_NOTIF_NOTHING_TO_DELETE | response NotificationSentResponse ::= { deleteNotificationStatus nothingToDelete } |
| R_REMOVE_NOTIF_OK | response NotificationSentResponse ::= { deleteNotificationStatus ok } |
| R_RETRIEVE_NOTIF_IN1_IN1_PIR | resp RetrieveNotificationsListResponse ::= notificationList : { profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata #NOTIF_METADATA_INSTALL1_PIR, smdpOid #S_SM_DP+_OID, finalResult successResult : { aid <ISD_P_AID>, simaResponse #SIMA_RESULT_OK } }, euiccSignPIR <EUICC_SIGN_PIR> }, otherSignedNotification : { tbsOtherNotification #NOTIF_METADATA_INSTALL1, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } } |
| R_RETRIEVE_NOTIF_IN1_PIR | resp RetrieveNotificationsListResponse ::= notificationList : { profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata #NOTIF_METADATA_INSTALL1_PIR, smdpOid #S_SM_DP+_OID, finalResult successResult : { aid <ISD_P_AID>, simaResponse #SIMA_RESULT_OK } }, euiccSignPIR <EUICC_SIGN_PIR> } } |
| R_RETRIEVE_NOTIF_IN1 | resp RetrieveNotificationsListResponse ::= notificationList : { otherSignedNotification : { tbsOtherNotification #NOTIF_METADATA_INSTALL1, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } } |
| R_RETRIEVE_NOTIF_EN1 | resp RetrieveNotificationsListResponse ::= notificationList : { otherSignedNotification : { tbsOtherNotification #NOTIF_METADATA_ENABLE1, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } } |
| R_RETRIEVE_NOTIF_IN2_PIR | resp RetrieveNotificationsListResponse ::= notificationList : { profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata #NOTIF_METADATA_INSTALL2_PIR, smdpOid #S_SM_DP+_OID2, finalResult successResult : { aid <ISD_P_AID>, simaResponse #SIMA_RESULT_OK } }, euiccSignPIR <EUICC_SIGN_PIR> } } |
| R_RETRIEVE_NOTIF_DI1 | resp RetrieveNotificationsListResponse ::= notificationList : { otherSignedNotification : { tbsOtherNotification #NOTIF_METADATA_DISABLE1, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } } |
| R_RETRIEVE_NOTIF_DE1 | resp RetrieveNotificationsListResponse ::= notificationList : { otherSignedNotification : { tbsOtherNotification #NOTIF_METADATA_DELETE1, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } } |
| R_RETRIEVE_NOTIF_NONE | resp RetrieveNotificationsListResponse ::= notificationList : {} |
| R_RETRIEVE_NOTIF_IN1_PIR_EN1 | resp RetrieveNotificationsListResponse ::= notificationList : { profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata #NOTIF_METADATA_INSTALL1_PIR, smdpOid #S_SM_DP+_OID, finalResult successResult : { aid <ISD_P_AID>, simaResponse #SIMA_RESULT_OK } }, euiccSignPIR <EUICC_SIGN_PIR> }, otherSignedNotification : { tbsOtherNotification #NOTIF_METADATA_ENABLE1, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } } |
| R_RETRIEVE_NOTIF_IN1_PIR_IN2_PIR | resp RetrieveNotificationsListResponse ::= notificationList : { profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata #NOTIF_METADATA_INSTALL1_PIR, smdpOid #S_SM_DP+_OID, finalResult successResult : { aid <ISD_P_AID>, simaResponse #SIMA_RESULT_OK } }, euiccSignPIR <EUICC_SIGN_PIR> }, profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata #NOTIF_METADATA_INSTALL2_PIR, smdpOid #S_SM_DP+_OID2, finalResult successResult : { aid <ISD_P_AID>, simaResponse #SIMA_RESULT_OK } }, euiccSignPIR <EUICC_SIGN_PIR> } } |
| R_RETRIEVE_NOTIF_DI1_DE1 | resp RetrieveNotificationsListResponse ::= notificationList : { otherSignedNotification : { tbsOtherNotification #NOTIF_METADATA_DISABLE1, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG }, otherSignedNotification : { tbsOtherNotification #NOTIF_METADATA_DELETE1, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } } |
| R_RETRIEVE_NOTIF_IN1_EN1 | resp RetrieveNotificationsListResponse ::= notificationList : { otherSignedNotification : { tbsOtherNotification #NOTIF_METADATA_INSTALL1, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG }, otherSignedNotification : { tbsOtherNotification #NOTIF_METADATA_ENABLE1, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } } |
| R_RETRIEVE_NOTIF_EN1_IN2_PIR | resp RetrieveNotificationsListResponse ::= notificationList : { profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata #NOTIF_METADATA_INSTALL2_PIR, smdpOid #S_SM_DP+_OID2, finalResult successResult : { aid <ISD_P_AID>, simaResponse #SIMA_RESULT_OK } }, euiccSignPIR <EUICC_SIGN_PIR> }, otherSignedNotification : { tbsOtherNotification#NOTIF_METADATA_ENABLE1, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } } |
| SMDP_PROP_DATA1 (DpProprietaryData) | { dpOid #S_SM_DP+_OID } |

- D.4	APDU

- D.4.1	APDU Commands

| Name | Content |
| --- | --- |
| DELETE_SSD | - CLA = 80, INS = E4, P1 = 00, P2 = 80, LC = <L> - Data = 4F <L> #SSD_AID - LE = 00 |
| GET_RESPONSE | - CLA = 0x (x = <CHANNEL_NUMBER>), INS = C0, P1 = 00, P2 = 00, LE = <L> |
| GET_MNO_SD | - CLA = 80, INS = F2, P1 = 80, P2 = 02, LC = <L> - Data = 4F 00 - LE = 00 |
| INSTALL_PERSO_RES_ISDP | - CLA = 80, INS = E6, P1 = 20, P2 = 00, LC = 16 - Data = 00 00 10 A0 00 00 05 59 10 10 FF FF FF FF 89 00 00 0F 00 00 00 00 - LE = 00 |
| MANAGE_CHANNEL_OPEN | - CLA = 00, INS = 70, P1 = 00, P2 = 00, LE = 01 |
| READ_BINARY | - CLA = 00, INS = B0, P1 = 00, P2 = 00, LE = <L> |
| SELECT_MF | - CLA = 00, INS = A4, P1 = 00, P2 = 04, LC = <L> - Data = 3F 00 - LE = 00 |
| SELECT_ICCID | - CLA = 00, INS = A4, P1 = 00, P2 = 0C, LC = 02 - Data = 2F E2 |
| SELECT_USIM | - CLA = 00, INS = A4, P1 = 04, P2 = 04, LC = <L> - Data = #USIM_AID - LE = 00 |
| TERMINAL_CAPABILITY_LPAd | - CLA = 80, INS = AA, P1 = 00, P2 = 00, LC = <L> - Data = A9 05 81 00 83 01 07 |
| TERMINAL_PROFILE | - CLA = 80, INS = 10, P1 = 00, P2 = 00, LC = <L> - Data = FF FF FF FF 7F 9D 00 DF BF 00 00 1F E2 00 00 00 C7 EB 00 00 00 01 68 00 50 00 00 00 00 00 02 00 |
| TERMINAL_PROFILE_eUICCProfileStateChanged | - CLA = 80, INS = 10, P1 = 00, P2 = 00, LC = <L> - Data = FF FF FF FF FF FF 1F FF FF 03 02 FF FF 9F FF EF DF FF 0F FF 0F FF FF 0F FF 03 00 3F 7F FF 03 FF FF 20 |

- D.4.2	R-APDU Chaining

During the execution of all sequences related to the eUICC testing (i.e. section 4.2), for commands where the response exceeds 256 bytes, the chaining mechanism defined in ISO/IEC 7816-4 [7], using the 61XX status word and multiple GET RESPONSE commands, SHALL be used.

As an example, the following generic sequence, which describes this mechanism, SHALL apply.

| Step | Direction | Sequence / Description | Result |
| --- | --- | --- | --- |
| 1 | OCE eUICC | Send APDU command on logical channel x | <R_APDU_PART1> SW=0x61XX |
| 2 | OCE eUICC | Send [GET_RESPONSE] on logical channel x with LE='XX' | <R_APDU_PART2> SW=0x61XX |
| 3 | OCE eUICC | Send [GET_RESPONSE] on logical channel x with LE='XX' | <R_APDU_PART3> SW=0x61XX |
| 4 | OCE eUICC | Send [GET_RESPONSE] on logical channel x with LE='XX' | <R_APDU_PART4> SW=0x9000 The complete response is the result of the concatenation of all R-APDUs from <R_APDU_PART1> to <R_APDU_PART4> |

- D.5	ES6 Requests And Responses

- D.5.1	ES6 Requests

| Name | Content |
| --- | --- |
| REMOVE_PPR1 | metadataReq UpdateMetadataRequest ::= { profilePolicyRules {ppr2} } |
| UPD_ICON_REM_PPR2 | metadataReq UpdateMetadataRequest ::= { iconType jpg, icon #ICON_JPG, profilePolicyRules {ppr1} } |
| UPD_NAMES_REM_PPRS | metadataReq UpdateMetadataRequest ::= { serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, profilePolicyRules {} } |
| REMOVE_NAMES_PPRS | metadataReq UpdateMetadataRequest ::= { serviceProviderName "", profileName "", profilePolicyRules {} } |
| UPD_PPR_CONTROL | metadataReq UpdateMetadataRequest ::= { serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, iconType jpg, icon #ICON_JPG, profilePolicyRules {pprUpdateControl, ppr1} } |
| UPD_NO_METADATA | metadataReq UpdateMetadataRequest ::= { } |
| UPD_ICON_NO_TYPE | metadataReq UpdateMetadataRequest ::= { serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, icon #ICON_JPG, profilePolicyRules {} } |
| UPD_ICON_TYPE_ONLY | metadataReq UpdateMetadataRequest ::= { serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, iconType jpg, profilePolicyRules {} } |

- D.6	ES11 Requests And Responses

- D.6.1	ES11 Requests

| Name | Content |
| --- | --- |
| AUTH_SERVER_RESP_MATCHING_ID_EMPTY | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_MATCHING_ID_EVENT_ID | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EVENT_ID }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_MATCHING_ID_EVENT_ID_R | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EVENT_ID_R }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_MATCHING_ID_OMITTED | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_OMITTED }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_SMDS_8_1_2_6_1_EX_BC_cA | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG_INVALID_EX_BC_cA } |
| AUTH_SERVER_RESP_SMDS_8_1_2_6_1_EX_BC_PLC | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG_INVALID_EX_BC_PLC } |
| AUTH_SERVER_RESP_SMDS_8_1_2_6_1_EX_CP | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG_INVALID_EX_CP } |
| AUTH_SERVER_RESP_SMDS_8_1_2_6_1_EX_KU | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG_INVALID_EX_KU } |
| AUTH_SERVER_RESP_SMDS_8_1_2_6_1_SIG | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG_INVALID_SIG } |
| AUTH_SERVER_RESP_SMDS_8_1_2_6_3 | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG_EXPIRED } |
| AUTH_SERVER_RESP_SMDS_8_1_3_6_1_EX_CP | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG_INVALID_EX_CP, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_SMDS_8_1_3_6_1_EX_KU | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG_INVALID_EX_KU, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_SMDS_8_1_3_6_1_SIG | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG_INVALID_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_SMDS_8_1_3_6_1_SUB_ORG | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG_INVALID_SUB_ORG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_SMDS_8_1_3_6_1_SUB_SN | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG_INVALID_SUB_SN, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_SMDS_8_1_3_6_3 | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG_EXPIRED, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_SMDS_8_1_6_1_CHA | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE_INVALID>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_SMDS_8_1_6_1_SIG | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1_INVALID>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| AUTH_SERVER_RESP_SMDS_8_10_1_3_9 | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <INVALID_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| CTX_PARAMS1_MATCHING_ID_EVENT_ID (CtxParams1) | ctxParamsForCommonAuthentication : { matchingId #EVENT_ID_1, deviceInfo #S_DEVICE_INFO } |
| CTX_PARAMS1_MATCHING_ID_EVENT_ID_R (CtxParams1) | ctxParamsForCommonAuthentication : { matchingId <EVENT_ID_R>, deviceInfo #S_DEVICE_INFO } |
| CTX_PARAMS1_MATCHING_ID_OMITTED (CtxParams1) | ctxParamsForCommonAuthentication : { deviceInfo #S_DEVICE_INFO } |

- D.6.2	ES11 Responses

| Name | Content |
| --- | --- |
| AUTH_CLIENT_DS_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "smdsSigned2" : <SMDS_SIGNED2>, "smdsSignature2" : <SMDS_SIGNATURE2> } |
| AUTH_CLIENT_DS_OK1 | { "header" :{ "functionExecutionStatus":{ "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "smdsSigned2" : #SMDS_SIGNED2_1, "smdsSignature2" : <SMDS_SIGNATURE2> } |
| AUTH_CLIENT_DS_OK2 | { "header" :{ "functionExecutionStatus":{ "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "smdsSigned2" : #SMDS_SIGNED2_2, "smdsSignature2" : <SMDS_SIGNATURE2> } |
| AUTH_CLIENT_DS_OK_DSADDR1 | { "header" :{ "functionExecutionStatus":{ "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "smdsSigned2" : #SMDS_SIGNED2_DSADDR1, "smdsSignature2" : <SMDS_SIGNATURE2> } |
| EVENT_ENTRY | { eventId <EVENT_ID>, rspServerAddress <RSP_SERVER_ADDRESS>, eventType <EVENT_TYPE> } |
| EVENT_ENTRY_1 | { eventId #EVENT_ID_1, rspServerAddress #TEST_DP_ADDRESS1, eventType #EVENT_TYPE_DOWNLOAD } |
| EVENT_ENTRY_1_ALT_DS | { eventId #EVENT_ID_1, rspServerAddress #TEST_ALT_DS_ADDRESS1, eventType #EVENT_TYPE_DOWNLOAD } |
| EVENT_ENTRY_1_HASHED_ICCID | { eventId #EVENT_ID_1, rspServerAddress #TEST_DP_ADDRESS1, eventType #EVENT_TYPE_DOWNLOAD, hashedIccids #HASHED_ICCID_OP_PROF1 } |
| EVENT_ENTRY_1_HASHED_SALTED_ICCID | { eventId #EVENT_ID_1, rspServerAddress #TEST_DP_ADDRESS1, eventType #EVENT_TYPE_DOWNLOAD, hashedIccids #HASHED_SALTED_ICCID_OP_PROF1 salt #SALT } |
| EVENT_ENTRY_2 | { eventId #EVENT_ID_2, rspServerAddress #TEST_DP_ADDRESS2, eventType #EVENT_TYPE_DOWNLOAD } |
| EVENT_ENTRY_DSADDR1 | { eventId #EVENT_ID_1, rspServerAddress #TEST_DS_ADDRESS1, eventType #EVENT_TYPE_DOWNLOAD } |
| EVENT_ENTRY_MULTI | { { eventId #EVENT_ID_1, rspServerAddress #TEST_DP_ADDRESS1, eventType #EVENT_TYPE_DOWNLOAD },{ eventId #EVENT_ID_2, rspServerAddress #TEST_DP_ADDRESS2, eventType #EVENT_TYPE_DOWNLOAD } } |
| R_AUTH_CLIENT_DS_EVENT_ENTRY_1_ALT_DS_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "smdsSigned2" : #SMDS_SIGNED2_1_ALT_DS, "smdsSignature2" : <SMDS_SIGNATURE2> } |
| R_AUTH_CLIENT_DS_EVENT_ENTRY_1_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "smdsSigned2" : #SMDS_SIGNED2_1, "smdsSignature2" : <SMDS_SIGNATURE2> } |
| R_AUTH_CLIENT_DS_EVENT_ENTRY_EMPTY_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "smdsSigned2" : #SMDS_SIGNED2_EMPTY, "smdsSignature2" : <SMDS_SIGNATURE2> } |
| R_AUTH_CLIENT_DS_EVENT_ENTRY_MULTI_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "smdsSigned2" : #SMDS_SIGNED2_MULTI, "smdsSignature2" : <SMDS_SIGNATURE2> } |
| SMDS_SIGNED2 | { transactionId <S_TRANSACTIONID>, requestSpecificData : { eventEntries <EVENT_ENTRY> } } |
| SMDS_SIGNED2_1 | { transactionId <S_TRANSACTIONID>, requestSpecificData : { eventEntries #EVENT_ENTRY_1 } } |
| SMDS_SIGNED2_1_ALT_DS | { transactionId <S_TRANSACTIONID>, requestSpecificData : { eventEntries #EVENT_ENTRY_1_ALT_DS } } |
| SMDS_SIGNED2_1_HASHED_ICCID | { transactionId <S_TRANSACTIONID>, requestSpecificData : { eventEntries #EVENT_ENTRY_1_HASHED_ICCID } } |
| SMDS_SIGNED2_1_HASHED_SALTED_ICCID | { transactionId <S_TRANSACTIONID>, requestSpecificData : { eventEntries #EVENT_ENTRY_1_HASHED_SALTED_ICCID } } |
| SMDS_SIGNED2_2 | { transactionId <S_TRANSACTIONID>, requestSpecificData : { eventEntries #EVENT_ENTRY_2 } } |
| SMDS_SIGNED2_DS_ADDR1 | { transactionId <S_TRANSACTIONID>, requestSpecificData : { eventEntries #EVENT_ENTRY_DSADDR1 } } |
| SMDS_SIGNED2_DS_EMPTY | { transactionId <S_TRANSACTIONID>, requestSpecificData : { } } |
| SMDS_SIGNED2_MULTI | { transactionId <S_TRANSACTIONID>, requestSpecificData : { eventEntries #EVENT_ENTRY_MULTI } } |

- D.7	ES12 Requests And Responses

There are no specific ES12 requests or responses defined in the present document.

- D.8	ES15 Requests And Responses

There are no specific ES15 requests or responses defined in the present document.

- D.9	Common Server Responses

For all responses with a JSON component the “subjectIdentifier” and “message” are optional and may or may not be present in the response received from the RSP server.

| Name | Content |
| --- | --- |
| R_ERROR_1_2_4_2 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "1.2", "reasonCode" : "4.2" } } } } |
| R_ERROR_8_1_1_2_2 | - { - "header" : { - "functionExecutionStatus" : { - "status" : "Failed", - "statusCodeData” : { - "subjectCode" : "8.1.1", - "reasonCode" : "2.2" - } - } - } } |
| R_ERROR_8_1_1_3_8 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.1.1", "reasonCode" : "3.8" } } } } |
| R_ERROR_8_1_1_3_10 | - { - "header" : { - "functionExecutionStatus" : { - "status" : "Failed", - "statusCodeData” : { - "subjectCode" : "8.1.1", - "reasonCode" : "3.10" - } - } - } } |
| R_ERROR_8_1_2_6_1 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.1.2", "reasonCode" : "6.1" } } } } |
| R_ERROR_8_1_2_6_3 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.1.2", "reasonCode" : "6.3" } } } } |
| R_ERROR_8_1_3_6_1 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.1.3", "reasonCode" : "6.1" } } } } |
| R_ERROR_8_1_3_6_3 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.1.3", "reasonCode" : "6.3" } } } } |
| R_ERROR_8_1_4_8 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.1", "reasonCode" : "4.8" } } } } |
| R_ERROR_8_1_6_1 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.1", "reasonCode" : "6.1" } } } } |
| R_ERROR_8_2_1_2 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.2", "reasonCode" : "1.2" } } } } |
| R_ERROR_8_2_1_3_3 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData" : { "subjectCode" : "8.2.1", "reasonCode" : "3.3" } } } } |
| R_ERROR_8_2_1_3_9 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData" : { "subjectCode" : "8.2.1", "reasonCode" : "3.9" } } } } |
| R_ERROR_8_2_1_3_10 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.2.1", "reasonCode" : "3.10" } } } } |
| R_ERROR_8_2_3_7 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData" : { "subjectCode" : "8.2", "reasonCode" : "3.7" } } } } |
| R_ERROR_8_2_5_4_3 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.2.5", "reasonCode" : "4.3" } } } } |
| R_ERROR_8_2_6_2_1 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.2.6", "reasonCode" : "2.1" } } } } |
| R_ERROR_8_2_6_3_3 | - { - "header" : { - "functionExecutionStatus" : { - "status" : "Failed", - "statusCodeData” : { - "subjectCode" : "8.2.6", - "reasonCode" : "3.3" - } - } - } } |
| R_ERROR_8_2_6_3_8 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.2.6", "reasonCode" : "3.8" } } } } |
| R_ERROR_8_2_6_3_10 | - { - "header" : { - "functionExecutionStatus" : { - "status" : "Failed", - "statusCodeData” : { - "subjectCode" : "8.2.6", - "reasonCode" : "3.10" - } - } - } } |
| R_ERROR_8_2_7_2_2 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData" : { "subjectCode" : "8.2.7", "reasonCode" : "2.2" } } } } |
| R_ERROR_8_2_7_3_8 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData" : { "subjectCode" : "8.2.7", "reasonCode" : "3.8" } } } } |
| R_ERROR_8_2_7_6_4 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.2.7", "reasonCode" : "6.4" } } } } |
| R_ERROR_8_8_1_3_8 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.8.1", "reasonCode" : "3.8" } } } } |
| R_ERROR_8_8_2_3_1 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.8.2", "reasonCode" : "3.1" } } } } |
| R_ERROR_8_8_3_3_1 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.8.3", "reasonCode" : "3.1" } } } } |
| R_ERROR_8_8_3_10 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.8", "reasonCode" : "3.10" } } } } |
| R_ERROR_8_8_4_3_7 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.8.4", "reasonCode" : "3.7" } } } } |
| R_ERROR_8_8_5_4_10 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.8.5", "reasonCode" : "4.10" } } } } |
| R_ERROR_8_8_5_6_4 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.8.5", "reasonCode" : "6.4" } } } } |
| R_ERROR_8_9_1_3_8 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.9.1", "reasonCode" : "3.8" } } } } |
| R_ERROR_8_9_2_3_1 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.9.2", "reasonCode" : "3.1" } } } } |
| R_ERROR_8_9_3_3_1 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.9.3", "reasonCode" : "3.1" } } } } |
| R_ERROR_8_9_4_2 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.9", "reasonCode" : "4.2" } } } } |
| R_ERROR_8_9_4_3_7 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.9.4", "reasonCode" : "3.7" } } } } |
| R_ERROR_8_9_5_1 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.9", "reasonCode" : "5.1" } } } } |
| R_ERROR_8_9_5_3_3 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.9.5", "reasonCode" : "3.3" } } } } |
| R_ERROR_8_9_5_3_9 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.9.5", "reasonCode" : "3.9" } } } } |
| R_ERROR_8_10_1_3_9 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.10.1", "reasonCode" : "3.9" } } } } |
| R_ERROR_8_11_1_3_9 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.11.1", "reasonCode" : "3.9" } } } } |
| R_ERROR_ANY | - { - "header" : { - "functionExecutionStatus" : { - "status" : "Failed", - "statusCodeData” : { - "subjectCode" : <SUBJECT_CODE_ANY>, - "reasonCode" : <REASON_CODE_ANY> - } - } - } } |
| R_ERROR_SMXX_1_3_8 | The error response will be as follows dependent on the entity under test: -  for SM-DP+ testing on ES9+ SHALL be #R_ERROR_8_8_1_3_8 -  for SM-DS testing on ES11 SHALL be #R_ERROR_8_9_1_3_8 |
| R_ERROR_SMXX_2_3_1 | The error response will be as follows dependent on the entity under test: -  for SM-DP+ testing on ES9+ SHALL be #R_ERROR_8_8_2_3_1 -  for SM-DS testing on ES11 SHALL be #R_ERROR_8_9_2_3_1 |
| R_ERROR_SMXX_3_3_1 | The error response will be as follows dependent on the entity under test: -  for SM-DP+ testing on ES9+ SHALL be #R_ERROR_8_8_3_3_1 -  for SM-DS testing on ES11 SHALL be #R_ERROR_8_9_3_3_1 |
| R_ERROR_SMXX_4_3_7 | The error response will be as follows dependent on the entity under test: -  for SM-DP+ testing on ES9+ SHALL be #R_ERROR_8_8_4_3_7 -  for SM-DS testing on ES11 SHALL be #R_ERROR_8_9_4_3_7 |
| R_SUCCESS | - { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } } } |

- D.10	ES2+ Requests And Responses

D.10.1 ES2+ Requests

D.10.2 ES2+ Responses

| Name | Content |
| --- | --- |
| R_SUCCESS_ICCID1 | - { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } } - "iccid" : "#ICCID_OP_PROF1_NON_SWAP" } |
| R_SUCCESS_MATCHING_ID | - { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } } "matchingId" : <MATCHING_ID> - } |
| R_SUCCESS_MATCHING_ID_EID | - { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } } "matchingId" : <MATCHING_ID> - "eid" : "#EID1" - } |

- Annex E	Profiles

| Profile | GENERIC_PROFILE_STRUCTURE |
| --- | --- |
| Description | Generic Operational Profile ASN.1 structure to be used as a basis for all Profiles used in this specification. |
| Details | headerValue ProfileElement ::= header : { major-version 2, minor-version 3, profileType "GSMA Profile Package", iccid '89019990001234567893'H, eUICC-Mandatory-services { usim NULL, milenage NULL }, eUICC-Mandatory-GFSTEList { -- see Note 1 id-MF, id-USIM } } mfValue ProfileElement ::= mf : { mf-header { mandated NULL, identification 1 }, templateID id-MF, mf { fileDescriptor : { pinStatusTemplateDO '01020A'H } }, ef-pl { fileDescriptor : { -- EF PL modified to use Access Rule 15 within EF ARR securityAttributesReferenced '0F'H } }, ef-iccid { -- swapped ICCID: 98109909002143658739 fillFileContent '98109909002143658739'H }, ef-dir { fileDescriptor { -- Shareable Linear Fixed File -- 4 records, record length: 38 bytes fileDescriptor '42210026'H, efFileSize '98'H }, -- USIM AID: A0000000871002FF33FF018900000100 fillFileContent '61184F10A0000000871002FF33FF01890000010050045553494D'H }, ef-arr { fileDescriptor : { fileDescriptor '42210025'H, lcsi '05'H, efFileSize '022B'H }, fillFileContent : '8001019000800102A406830101950108800158A40683010A950108'H, fillFileOffset : 10, fillFileContent : '800101A40683010195010880015AA40683010A950108'H, fillFileOffset : 15, fillFileContent : '80015BA40683010A950108'H, fillFileOffset : 26, fillFileContent : '800101900080015A9700'H, fillFileOffset : 27, fillFileContent : '800103A406830101950108800158A40683010A950108'H, fillFileOffset : 15, fillFileContent : '800111A40683010195010880014AA40683010A950108'H, fillFileOffset : 15, fillFileContent : '800103A406830101950108800158A40683010A950108840132A406830101950108'H, fillFileOffset : 4, fillFileContent : '800101A406830101950108800102A406830181950108800158A40683010A950108'H, fillFileOffset : 4, fillFileContent : '800101900080011AA406830101950108800140A40683010A950108'H, fillFileOffset : 10, fillFileContent : '800101900080015AA40683010A950108'H, fillFileOffset : 21, fillFileContent : '8001019000800118A40683010A9501088001429700'H, fillFileOffset : 16, fillFileContent : '800101A40683010195010880015A9700'H, fillFileOffset : 21, fillFileContent : '800113A406830101950108800148A40683010A950108'H, fillFileOffset : 15, fillFileContent : '80015EA40683010A950108'H, fillFileOffset : 26, fillFileContent '8001019000800102A010A40683010195 0108A406830102950108800158A40683 010A950108'H } } pukVal ProfileElement ::= pukCodes : { puk-Header { mandated NULL, identification 2 }, pukCodes { { keyReference pukAppl1, pukValue '3030303030303030'H, -- maxNumOfAttemps:9, retryNumLeft:9 maxNumOfAttemps-retryNumLeft 153 }, { keyReference pukAppl2, pukValue '3132333435363738'H }, { keyReference secondPUKAppl1, pukValue '3932393435363738'H, -- maxNumOfAttemps:8, retryNumLeft:8 maxNumOfAttemps-retryNumLeft 136 } } } pinVal ProfileElement ::= pinCodes : { pin-Header { mandated NULL, identification 3 }, pinCodes pinconfig : { { keyReference pinAppl1, pinValue '31323334FFFFFFFF'H, unblockingPINReference pukAppl1 }, { keyReference pinAppl2, pinValue '30303030FFFFFFFF'H, unblockingPINReference pukAppl2 }, { keyReference adm1, pinValue '35363738FFFFFFFF'H, pinAttributes 1 } } } usimValue ProfileElement ::= usim : { usim-header { mandated NULL, identification 4 }, templateID id-USIM, adf-usim { fileDescriptor : { fileID '7FF1'H, dfName 'A0000000871002FF33FF018900000100'H, pinStatusTemplateDO '01810A'H } }, ef-imsi { -- numerical format: 234101943787656 fillFileContent '082943019134876765'H }, ef-arr { fileDescriptor { linkPath '2F06'H } }, ef-ust { -- Service Dialling Numbers, Short Message Storage… fillFileContent '0A2E178CE73204000000000000'H }, ef-spn { -- ASCII format: "GSMA eUICC" fillFileContent '0247534D41206555494343FFFFFFFFFFFF'H }, ef-est { -- Services deactivated fillFileContent '00'H }, ef-acc { -- Access class 4 fillFileContent '0040'H }, ef-ecc { -- Emergency Call Code 911 fillFileContent '19F1FF01'H } } usimPin ProfileElement ::= pinCodes : { pin-Header { mandated NULL, identification 5 }, pinCodes pinconfig : { { keyReference secondPINAppl1, pinValue '39323338FFFFFFFF'H unblockingPINReference secondPUKAppl1, -- PIN is Enabled pinAttributes 1, -- maxNumOfAttemps:2, retryNumLeft:2 maxNumOfAttemps-retryNumLeft 34 } } } akaParamValue ProfileElement ::= akaParameter : { aka-header { mandated NULL, identification 6 }, algoConfiguration algoParameter : { algorithmID milenage, -- RES and MAC 64 bits, CK and IK 128 bits algorithmOptions '01'H, key '000102030405060708090A0B0C0D0E0F'H, opc '0102030405060708090A0B0C0D0E0F00'H, -- rotationConstants uses default: '4000204060'H -- xoringConstants uses default value authCounterMax '010203'H } -- sqnOptions uses default: '02'H -- sqnDelta uses default: '000010000000'H -- sqnAgeLimit uses default: '000010000000'H -- sqnInit uses default: all bytes zero } mnoSdValue ProfileElement ::= securityDomain : { sd-Header { mandated NULL, identification 7 }, instance { applicationLoadPackageAID 'A0000001515350'H, classAID 'A000000151535041'H, instanceAID 'A000000151000000'H, applicationPrivileges '82FC80'H, -- Secured lifeCycleState '0F'H, -- SCP80 supported applicationSpecificParametersC9 '810280008201F08701F0'H, -- other parameters MAY be necessary applicationParameters { -- TAR: B20100, MSL: 12 uiccToolkitApplicationSpecificParametersField '0100000100000002011203B2010000'H } }, keyList { { -- C-ENC + R-ENC keyUsageQualifier '38'H, -- ENC key keyIdentifier '01'H, keyVersionNumber '01'H, keyCompontents { { -- DES mode implicitly known (as an example) keyType '80'H, -- This value MAY be freely changed keyData '112233445566778899AABBCCDDEEFF10'H } } }, { -- C-MAC + R-MAC keyUsageQualifier '34'H, -- MAC key keyIdentifier '02'H, keyVersionNumber '01'H, keyCompontents { { -- DES mode implicitly known (as an example) keyType '80'H, -- This value MAY be freely changed keyData '112233445566778899AABBCCDDEEFF10'H } } }, { -- C-DEK + R-DEK keyUsageQualifier 'C8'H, -- data ENC key keyIdentifier '03'H, keyVersionNumber '01'H, keyCompontents { { -- DES mode implicitly known (as an example) keyType '80'H, -- This value MAY be freely changed keyData '112233445566778899AABBCCDDEEFF10'H } } }, -- AES Token Key (as an example) -- This value MAY be freely changed keyUsageQualifier '81'H, -- MAY be used by SD keyAccess '01'H, -- Key Id 01 keyIdentifier '01'H, keyVersionNumber '70'H, keyCompontents { { -- AES (16 bytes key length) -- This value MAY be freely changed keyType '88'H, -- This value MAY be freely changed keyData 'CDFE56B7B72FAE6A047341F003D7A48D'H } } }, { -- Receipt (the AES scheme SHALL be supported) keyUsageQualifier '44'H, -- MAY be used by SD keyAccess '01'H, -- Key Id 01 keyIdentifier '01'H, keyVersionNumber '71'H, keyCompontents { { -- AES (16 bytes key length) keyType '88'H, -- This value MAY be freely changed keyData '11121314212223243132333441424344'H } } } } } ssdValue ProfileElement ::= securityDomain : { sd-Header { mandated NULL, identification 8 }, instance { applicationLoadPackageAID 'A0000001515350'H, classAID 'A000000151535041'H, instanceAID 'A00000055910100102736456616C7565'H, -- by default extradited under MNO-SD -- Privileges: Security Domain + Trusted Path applicationPrivileges '808000'H, -- Personalized lifeCycleState '0F'H, -- SCP80 supported, extradiction supported applicationSpecificParametersC9 '810280008201F0'H, applicationParameters { -- TAR: 6C7565, MSL: 12 uiccToolkitApplicationSpecificParametersField '01000001000000020112036C756500'H } }, keyList { { -- C-ENC + R-ENC keyUsageQualifier '38'H, keyIdentifier '01'H, keyVersionNumber '01'H, keyCompontents { { -- DES mode implicitly known (as an example) keyType '80'H, -- This value MAY be freely changed keyData '11223344556677881122334455667788'H } } }, { -- C-MAC + R-MAC keyUsageQualifier '34'H, -- MAC key keyIdentifier '02'H, keyVersionNumber '01'H, keyCompontents { { -- DES mode implicitly known (as an example) keyType '80'H, -- This value MAY be freely changed keyData '11223344556677881122334455667788'H } } }, { -- C-DEK + R-DEK keyUsageQualifier 'C8'H, -- data ENC key keyIdentifier '03'H, keyVersionNumber '01'H, keyCompontents { { -- DES mode implicitly known (as an example) keyType '80'H, -- This value MAY be freely changed keyData '11223344556677881122334455667788'H } } } } } rfmUicc ProfileElement ::= rfm : { rfm-header { identification 11 }, -- Instance AID instanceAID ' A00000055910100001'H, tarList { 'B00000'H }, -- cryptographic checksum + counter higher minimumSecurityLevel '12'H, -- full access uiccAccessDomain '00'H, -- full access uiccAdminAccessDomain '00'H } rfmUsim ProfileElement ::= rfm : { rfm-header { identification 12 }, -- Instance AID instanceAID 'A00000055910100002'H, tarList { 'B00020'H }, -- cryptographic checksum + counter higher minimumSecurityLevel '12'H, -- full access uiccAccessDomain '00'H, -- full access uiccAdminAccessDomain '00'H, adfRFMAccess { adfAID 'A0000000871002FF33FF018900000100'H, -- UICC access condition: ADM1 adfAccessDomain '02000100'H, -- UICC access condition: ADM1 adfAdminAccessDomain '02000100'H } } endValue ProfileElement ::= end : { end-header { mandated NULL, identification 99 } } |
| Note 1: The following OIDs are used: id-MF OBJECT IDENTIFIER ::= {joint-iso-itu-t(2) international-organizations(23) simalliance(143) euicc-profile(1) template(2) mf(1)} id-USIM OBJECT IDENTIFIER ::= {joint-iso-itu-t(2) international-organizations(23) simalliance(143) euicc-profile(1) template(2) usim(4)} |  |

| Profile | PROFILE_OPERATIONAL1 |
| --- | --- |
| Description | Operational Profile This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF1, except if defined differently in the test sequence. The Unprotected Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE except that: -  the iccid field SHALL be set to #ICCID_OP_PROF1 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF1 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF1 -  the pinAttributes of pinAppl1 present in the PE_PIN SHALL be set to 6 -  the SCP80 encryption key configured in the PE-SecurityDomain that corresponds to the MNO-SD SHALL be set to #MNO_SCP80_ENC_KEY -  the SCP80 message authentication key configured in the PE-SecurityDomain that corresponds to the MNO-SD SHALL be set to #MNO_SCP80_AUTH_KEY -  the SCP80 data encryption key configured in the PE-SecurityDomain that corresponds to the MNO-SD SHALL be set to #MNO_SCP80_DATA_ENC_KEY -  the instance AID configured in the PE-SecurityDomain that corresponds to the Supplementary Security Domain PE_SSD SHALL be set to #SSD_AID -  the ef-dir present in the PE-MF SHALL be configured with the AID #USIM_AID -  the ef-ust SHALL be set in accordance to #EF_UST1 (service 17 and 18 are not available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H The PROFILE_OPERATIONAL1 UPP is named #UPP_OP_PROF1 in the scope of this document. |

| Profile | PROFILE_OPERATIONAL2 |
| --- | --- |
| Description | Operational Profile This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF2, except if defined differently in the test sequence. The Unprotected Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE except that: -  the iccid field SHALL be set to #ICCID_OP_PROF2 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF2 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF2 -  The pinAttributes of pinAppl1 present in the PE_PIN SHALL be set to 6 -  the ef-ust SHALL be set in accordance to #EF_UST1 (service 17 and 18 are not available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H The PROFILE_OPERATIONAL2 UPP is named #UPP_OP_PROF2 in the scope of this document. |

| Profile | PROFILE_OPERATIONAL3 |
| --- | --- |
| Description | Operational Profile with PPR2 but without notification This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF3, except if defined differently in the test sequence. The Unprotected Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE except that: -  the iccid field SHALL be set to #ICCID_OP_PROF3 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF3 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF3 -  the pinAttributes of pinAppl1 present in the PE_PIN SHALL be set to 6 -  the ef-ust SHALL be set in accordance to #EF_UST1 (service 17 and 18 are not available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H The PROFILE_OPERATIONAL3 UPP is named #UPP_OP_PROF3 in the scope of this document. |

| Profile | PROFILE_OPERATIONAL4 |
| --- | --- |
| Description | Operational Profile with PPR1 and notification This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF4, except if defined differently in the test sequence. The Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE] except that: -  the iccid field SHALL be set to #ICCID_OP_PROF4 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF4 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF4 -  the pinAttributes of pinAppl1 present in the PE_PIN SHALL be set to 6 -  the ef-ust SHALL be set in accordance to #EF_UST1 (service 17 and 18 are not available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H The PROFILE_OPERATIONAL4 UPP is named #UPP_OP_PROF4 in the scope of this document. |

| Profile | PROFILE_OPERATIONAL5 |
| --- | --- |
| Description | Operational Profile with pinAppl1 enabled. This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF5, except if defined differently in the test sequence. The Unprotected Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE except that: -  the iccid field SHALL be set to #ICCID_OP_PROF5 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF5 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF5 -  the pinAppl1 present in the PE_PIN SHALL be enabled and has the value #PO1_PIN1 -  the ef-ust SHALL be set in accordance to #EF_UST1 (service 17 and 18 are not available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H |

| Profile | PROFILE_OPERATIONAL6 |
| --- | --- |
| Description | Operational Profile with pinAppl1 enabled. This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF6, except if defined differently in the test sequence. The Unprotected Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE except that: -  the iccid field SHALL be set to #ICCID_OP_PROF6 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF6 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF6 -  The pinAppl1 present in the PE_PIN SHALL be enabled and has the value #PO2_PIN1 -  the ef-ust SHALL be set in accordance to #EF_UST1 (service 17 and 18 are not available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H |

| Profile | PROFILE_OPERATIONAL7 |
| --- | --- |
| Description | Operational Profile with PPR2 and notification This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF7, except if defined differently in the test sequence. The Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE except that: -  the iccid field SHALL be set to #ICCID_OP_PROF7 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF7 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF7 -  the pinAttributes of pinAppl1 present in the PE_PIN SHALL be set to 6 -  the ef-ust SHALL be set in accordance to #EF_UST1 (service 17 and 18 are not available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H |

| Profile | PROFILE_OPERATIONAL8 |
| --- | --- |
| Description | Operational Profile with PPR2, pinAppl1 enabled and notification This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF8, except if defined differently in the test sequence. The Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE except that: -  the iccid field SHALL be set to #ICCID_OP_PROF8 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF8 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF8 -  The pinAppl1 present in the PE_PIN SHALL be enabled and has the value #PO2_PIN1 -  the ef-ust SHALL be set in accordance to #EF_UST1 (service 17 and 18 are not available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H |

| Profile | PROFILE_OPERATIONAL9 |
| --- | --- |
| Description | Operational Profile with GID1 and GID2 set This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF9, except if defined differently in the test sequence. The Unprotected Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE except that: -  the iccid field SHALL be set to #ICCID_OP_PROF9 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF9 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF9 -  the pinAppl1 present in the PE_PIN SHALL be enabled and has the value #PO1_PIN1 -  the ef-ust SHALL be set to #EF_UST2 (service 17 and 18 are available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H -  the following new Profile Element PE_OPT_USIM SHALL be inserted right after PE_USIM: PE_OPT_USIM. optusimValue ProfileElement ::= opt-usim : { optusim-header { mandated NULL, identification 15 }, templateID id-OPT-USIM, ef-gid1 { fileDescriptor { efFileSize '04'H }, fillFileContent #GID1 }, ef-gid2 { fileDescriptor { efFileSize '04'H }, fillFileContent #GID2 } }. NOTE : The following OIDs are used: id-OPT-USIM OBJECT IDENTIFIER ::= {joint-iso-itu-t(2) international-organizations(23) simalliance(143) euicc-profile(1) template(2) opt-usim(5)}. The PROFILE_OPERATIONAL9 UPP is named #UPP_OP_PROF9 in the scope of this document. |

- Annex F	IUT Settings

- F.1	VOID

- F.2	Platforms Settings

In order to execute the test cases defined in this document, the Platform (i.e. SM-DP+ or SM-DS) provider SHALL deliver following settings:

| SM-DP+ Setting name | Description |
| --- | --- |
| IUT_SM_DP_ADDRESS | FQDN of the SM-DP+ Under Test. |
| IUT_SM_DP_HOST_ID | SM-DP+ Host ID of the SM-DP+ Under Test coded as an ASN.1 octet string. |
| IUT_SM_DP_OID | SM-DP+ OID (as defined in section 1.3) of the SM-DP+ Under Test. |
| IUT_SM-DP+_MAX_NUMBER_DOWNLOAD_ATTEMPTS | Maximum number of download attempts allowed by the SM-DP+. After this number, no further download is allowed. |
| IUT_SM_DP_ADDRESS_ES2_PLUS | FQDN , or FQDN:<port> of the SM-DP+ Under Test ES2+ interface. This Value SHALL be different from the IUT_SM_DP_ADDRESS. |
| SM-DS Setting name | Description |
| IUT_SM_DS_ADDRESS_ES11 | FQDN of the SM-DS Under Test for access on ES11. |
| IUT_SM_DS_ADDRESS_ES12 | FQDN of the SM-DS Under Test for access on ES12. |
| IUT_SM_DS_ADDRESS_ES15 | FQDN of the SM-DS Under Test for access on ES15. |
| Shared Setting name | Description |
| IUT_CLIENT_TLS_VER | Applicability: this IUT setting is applicable for SM-DP+ and Alternative SM-DS. It is not applicable for Root SM-DS. Highest TLS protocol version supported by the Client (SM-DP+ on ES12 or Alternative SM-DS on ES15) under test, which SHALL be at least v1.2. For versions higher than TLS v1.2 backwards compatibility is assumed. |

- F.3	VOID

- F.4	Common Settings

In order to execute the test cases defined in this document, the IUT provider SHALL deliver following settings:

| IUT Setting name | Description |
| --- | --- |
| IUT_RSP_VERSION | Version of SGP.22 supported by the IUT encoded as a string of three integers separated with dots (for example: 2.1.0). In the scope of this specification, this value SHALL indicate one of the versions of SGP.22 for which this specification contains test cases, as specified in section 1.2. |

- Annex G	Initial States

Unless it is defined differently in a particular test case, the IUTs SHALL be set in the following initial state before the test case execution.

- G.1	VOID

- G.2	VOID

- G.3	SM-DP+ and SM-DS

The SM-DP+ SHALL be configured with #CERT_SM_DPauth_SIG, #CERT_SM_DPpb_SIG and #CERT_SM_DP_TLS for both NIST and BRP unless it is specified differently to verify specific configuration (e.g. test cases dedicated for NIST or BRP only).

The SM-DP+ provider SHALL provide the capability to provision the SM-DP+ with Profiles as required by the specific test cases, with the following associated data where required:

- 	Profile Metadata

- 	MatchingID

- 	EID

- 	Confirmation Code

- 	Protected with random keys in advance, or with session keys during an RSP session, as required

- 	Number of retries for receipt of a valid Confirmation Code.

The SM-DP+ provider SHALL provide the capability to expire a download order.

NOTE:	as ES2+ is out of scope in the current version of the present document, proprietary means MAY be used to provide these capabilities.

The SM-DS SHALL be configured with #CERT_SM_DSauth_SIG and #CERT_SM_DS_TLS for both NIST and BRP, unless it is specified differently to verify specific configuration (e.g. test cases dedicated for NIST or BRP only).

For TLS level and for the SM-XX testing, NIST shall be used unless it is specified otherwise.

The SM-DS provider SHALL provide the capability to register an event.

The SM-DS provider SHALL provide the capability to remove the record of a particular EventID having been used from the SM-DS.

- Annex H	Icons and QR Codes

The files for the eUICC Consumer Devices Icons and QR Codes are provided within in SGP.23_AnnexH_Icons.zip and SGP.23_AnnexH_QRCodes.zip packages, which accompany the present document.

- Annex I	Requirements

The requirements used in the specified test cases are provided within SGP_23_AnnexI_Requirements_v1_3.zip package, which accompanies the present document.

- Annex J	VOID

- Annex K	Document Management

- K.1	Document History

| Version |  | Date | CR Number |  | Brief Description of Change |  |  | Approval Authority |  | Editor / Company |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| v1.0 |  | 9th June 2017 |  |  | Initial version of SGP.23 v1.0 Test Specification |  |  | PSMC |  | Yolanda Sanz, GSMA |
| v1.1 |  | 28th Sept 2017 |  |  | Minor version of SGP.23 Test specifications |  |  | RSPLEN |  | Yolanda Sanz, GSMA |
| v1.2 |  | 3rd Jan 2018 |  |  | Minor version of SGP.23 Test specifications |  |  | RSPLEN |  | Yolanda Sanz, GSMA |
| V1.3 |  | 01th August |  |  | Minor version of SGP.23 Test specification |  |  | RSPLEN |  | Yolanda Sanz, GSMA |
| V1.4 |  | 18th Dec |  |  | Minor version of SGP.23 Test specification |  |  | RSPLEN |  | Yolanda Sanz, GSMA |
| V1.5 |  | 30 April 2019 |  |  | Minor version of SGP.23 Test specification |  |  | RSPLEN |  | Marcin Kulczycki, GSMA |
| V1.6 |  | 15 July 2019 |  |  | Minor version of SGP.23 Test specification |  |  | eSIM Group |  | Marcin Kulczycki, GSMA |
| V1.7 |  | 07 July 2020 |  |  | Minor version of SGP.23 Test specification |  |  | ISAG |  | Yolanda Sanz, GSMA |
| V1.8 |  | 22 October 2020 |  |  | Minor version of SGP.23 Test specification |  |  | ISAG |  | Yolanda Sanz, GSMA |
| V1.9 |  | 11 February 2021 | CR1900R01 |  | LPA: To remove non validated TLS test |  |  | ISAG |  | Yolanda Sanz, GSMA |
|  |  |  | CR1901R01 |  | eUICC: Aligment of Applicability table |  |  |  |  |  |
|  |  |  | CR1902R03 |  | LPA: remove TLS critical extension presence test sequence |  |  |  |  |  |
| V1.10 |  | 30 June 2021 | CR11006R00 |  | SM-DP+_ES2+ TestEnviorment |  |  | ISAG |  | Yolanda Sanz, GSMA |
|  |  |  | CR11003R04 |  | SM-DP+_ES2+_DownloadOrder-1. |  |  |  |  |  |
|  |  |  | CR11004R02 |  | SM-DP+ ES2+_DownloadOrder-2,3 |  |  |  |  |  |
|  |  |  | CR11007R02 |  | LPA Simplified End User Confirmation |  |  |  |  |  |
|  |  |  | CR11010R01 |  | LPA Revising User Confirmations in Add Profile with Confirmation Code input |  |  |  |  |  |
|  |  |  | CR11012R01 |  | LPA Set_EditDefaultSM-DP+Address_User_Intent_Verification_Removal |  |  |  |  |  |
|  |  |  | CR11015R01 |  | eUICC Test Enviorment for the eUICC |  |  |  |  |  |
|  |  |  | CR11017R01 |  | eUICC Mirror on integrated eUICCInfo2 |  |  |  |  |  |
|  |  |  | CR11018R01 |  | eUICC Introduction of v2.3 |  |  |  |  |  |
|  |  |  | NA |  | Fix ES2+ test enviorment |  |  |  |  |  |
|  |  |  | CR11019R02 |  | LPA_Update_DeleteProfile_Strong_Confirmation |  |  |  |  |  |
|  |  |  | CR11011R02 |  | Update_eUICCMemoryReset_Strong_Confirmation |  |  |  |  |  |
|  |  |  | CR11016R01 |  | Optional support of Set/Edit Default SM-DP+ Address |  |  |  |  |  |
|  |  |  | CR11020R04 |  | Update_for_LPA45 |  |  |  |  |  |
|  |  |  | CR11005R03 |  | SM-DP+_ES2+_DownloadOrderWithRetry |  |  |  |  |  |
|  |  |  | CR11021R06 |  | Update_eUICCMemoryReset |  |  |  |  |  |
|  |  |  | CR11027R00 |  | Editorial_Definitions_Abbreviations_Reference_SGP.22 |  |  |  |  |  |
|  |  |  | CR11024R04 |  | SM-DP+ ES2+_ConfirmOrder 1-8 |  |  |  |  |  |
|  |  |  | CR11026R00 |  | SM-DP+ ES2+ Test Environment with SM-DS |  |  |  |  |  |
|  |  |  | CR11030R03 |  | SM-DP+ ES2+ ConfirmOrder Errors 1-6 |  |  |  |  |  |
|  |  |  | CR11031R02 |  | SM-DP+ ES2+ ConfirmOrderRetry1-4 |  |  |  |  |  |
|  |  |  | CR11032R00 |  | SM-DP+ Entity SM-DP+ |  |  |  |  |  |
|  |  |  | CR11028R04 |  | SM-DP+_ES2+_CancelOrder-1-3 |  |  |  |  |  |
|  |  |  | CR11029R03 |  | SM-DP+_ES2+_CancelOrderErrors-1-5 |  |  |  |  |  |
|  |  |  | CR11033R01 |  | Update reference to SGP.26 |  |  |  |  |  |
|  |  |  | NA |  | To include the missing part of CR11033R01 |  |  |  |  |  |
|  |  |  | CR11023R06 |  | Update of applicability of PPR sequences |  |  |  |  |  |
|  |  |  | CR11022R06 |  | CancelSession: remove unnecessary usage of profile already installed with PPR1 |  |  |  |  |  |
|  |  |  | CR11034R02 |  | eUICC_Clarify_EuiccInfo2_TRE |  |  |  |  |  |
|  |  |  | CR11035R01 |  | Operator TLS Certificate |  |  |  |  |  |
|  |  |  | CR11036R00 |  | Updates and corrections related to SGP.22 versions |  |  |  |  |  |
|  |  |  | CR11037R01 |  | SIMAlliance references |  |  |  |  |  |
|  |  |  | CR11038R01 |  | LPA fixes |  |  |  |  |  |
|  |  |  | CR11039R01 |  | Optional_support_of_Brainpool_and_FRP_for_TLS |  |  |  |  |  |
|  |  |  | CR11040R01 |  | DpProprietaryData_Additional_Data_Must_be_allowed |  |  |  |  |  |
|  |  |  | CR11041R00 |  | Further SIMAlliance clean up |  |  |  |  |  |
|  |  |  | CR11042R02 |  | eUICCMemoryReset_fixes |  |  |  |  |  |
|  |  |  | CR11043R04 |  | eUICC Profile Package versions |  |  |  |  |  |
| SGP.23-3 v3.1 | 1 December 2023 |  |  | CR11101R02 |  | Fix_ASN1_field_name_and_remove_restriction | ISAG |  | Guido Abate - STMicroelectronics |  |
|  |  |  |  | CR11103R01 |  | Update_AddProfile_with_an_enabled_PPR1_Profile |  |  |  |  |
|  |  |  |  | CR11104R03 |  | Reference_TCA_Test_Spec_v3.1. |  |  |  |  |
|  |  |  |  | CR2211R04 |  | ConditionNbs_Update |  |  |  |  |
|  |  |  |  | CR11201r01 (Ph2) |  | SMDP_FQDN |  |  |  |  |
|  |  |  |  | Editor’s review |  | Removal of tests out of scope: eUICC ad Device tests Removal of the "Requirements” column in test sequences |  |  |  |  |
|  |  |  |  | Editor’s review |  | Fixing typo in 4.2.16.2.1, test sequence 1, IC1 |  |  |  |  |
|  |  |  |  | CR11410R01 |  | ICCID |  |  |  |  |
|  |  |  |  | CR11108R03 |  | RSPCapability bit for Profile Metadata Extensibilty |  |  |  |  |
|  |  |  |  | CR11109R01 |  | ContentType_Case_Insensitivity |  |  |  |  |
|  |  |  |  | CR11111R01 |  | Extended euiccInfo2 - Alt |  |  |  |  |
|  |  |  |  | CR11112R00 |  | Typo in extended deviceInfo |  |  |  |  |
|  |  |  |  | CR11113R01 |  | ES2+_Fixes |  |  |  |  |
|  |  |  |  | CR11115R01 |  | ES2+_Fixes-2 |  |  |  |  |
|  |  |  |  | Editor’s review |  | Yellowed all the test cases not yet reviewed |  |  |  |  |
|  |  |  |  | CR2392R00 |  | Definition of S_DEVICE_INFO_EXT |  |  |  |  |
|  |  |  |  | CR2347R00 |  | Removing_references_SGP.22_v2.x |  |  |  |  |
|  |  |  |  | CR2356R01 |  | TC_ES2+ |  |  |  |  |
|  |  |  |  | CR2358R00 |  | TC_ES2+_Corrections |  |  |  |  |
|  |  |  |  | Editor’s review |  | Certificates names aligned to SGP.23-1’s and SGP.23-2’s |  |  |  |  |
|  |  |  |  | Editor’s review |  | Fixed implementation of CR2356R01 and CR2358R00 and removed duplication of sections 4.3.2.2 and 4.3.3.2 |  |  |  |  |
|  |  |  |  | CR2368R00 |  | ES9+.GetBoundProfilePackage |  |  |  |  |
|  |  |  |  | CR2373R00 |  | SM-DP+ InitiateAuthentication |  |  |  |  |
|  |  |  |  | CR2374R00 |  | SM-DS InitiateAuthentication |  |  |  |  |
|  |  |  |  | CR2379R00 |  | SM-DS_AuthenticateClient_Review |  |  |  |  |
|  |  |  |  | CR2382R00 |  | ES9+.HandleNotification |  |  |  |  |
|  |  |  |  | CR2383R01 |  | ES12 and ES15 RegisterEvent |  |  |  |  |
|  |  |  |  | CR2384R00 |  | ES12 and ES15 DeleteEvent |  |  |  |  |
|  |  |  |  | CR2409R00 |  | SM_DP+_ProfileMetadata |  |  |  |  |
|  |  |  |  | CR2410R00 |  | ES9+.HandleNotification_RPM_UpdateMetadata |  |  |  |  |
|  |  |  |  | CR2411R02 |  | ES9+.AuthClient |  |  |  |  |
|  |  |  |  | CR2413R00 |  | TLS_Client_Mutual_Authentication_for_HTTPS_EstablishmentBRP |  |  |  |  |
|  |  |  |  | CR2414R00 |  | TLS_Server_Mutual_Authentication_for_HTTPS_EstablishmentNIST |  |  |  |  |
|  |  |  |  | CR2415R00 |  | TLS_Server_Mutual_Authentication_for_HTTPS_EstablishmentBRP |  |  |  |  |
|  |  |  |  | CR2416R02 |  | TLS_Server_Authentication_for_HTTPS_EstablishmentNIST |  |  |  |  |
|  |  |  |  | CR2417R00 |  | TLS_Server_Authentication_for_HTTPS_EstablishmentBRP |  |  |  |  |
|  |  |  |  | Editor’s review |  | Implementation of action eSIMWG3.95_AP15: “eumCertificate based in variant O needs to change by nextcertinchain, to apply this change in SGP.23-1 and -3.” |  |  |  |  |

- K.2	Other Information

| Type | Description |
| --- | --- |
| Document Owner | Yolanda Sanz, GSMA |
| Editor / Company | Guido Abate, STMicroelectronics |

It is our intention to provide a quality product for your use. If you find any errors or omissions, please contact us with your comments. You may notify us at prd@gsma.com.

Your comments or suggestions & questions are always welcome.