# Requirements for Multi SIM Devices

# Version 12.0

# 28 April 2026

Security Classification: Non-confidential

Access to and distribution of this document is restricted to the persons permitted by the security classification. This document is subject to copyright protection. This document is to be used only for the purposes for which it has been supplied and information contained in it must not be disclosed or in any other way made available, in whole or in part, to persons other than those permitted under the security classification without the prior written approval of the Association.

Copyright Notice

Copyright © 2026 GSM Association

Disclaimer

The GSM Association (“Association”) makes no representation, warranty or undertaking (express or implied) with respect to and does not accept any responsibility for, and hereby disclaims liability for the accuracy or completeness or timeliness of the information contained in this document. The information contained in this document may be subject to change without prior notice.

Compliance Notice

The information contain herein is in full compliance with the GSM Association’s antitrust compliance policy.

This Permanent Reference Document is classified by GSMA as an Industry Specification, as such it has been developed and is maintained by GSMA in accordance with the provisions set out in GSMA AA.35 - Procedures for Industry Specifications.

Table of Contents

1	Introduction	4

1.1	Overview	4

1.2	In Scope	4

1.3	Out of Scope	4

1.4	References	5

1.5	Definitions	6

1.6	Abbreviations	7

2	Requirements	7

2.1	Number of IMEIs	7

2.2	Use of IMEIs	8

2.2.1	Unblocking / retry	9

2.3	Limitations of specific SIM ports	9

2.4	Operational Mode	11

2.4.1	USAT	12

2.4.2	Single SIM operation	13

2.5	User interface	13

2.5.1	SIM Selection	13

2.5.2	Idle Mode	14

2.5.3	Calls, Data, SMS and MMS	14

2.5.4	Supplementary services	16

2.5.5	SIM PIN	17

2.5.6	Network & Service Provider locks.	17

2.5.7	Contact lists.	18

2.5.8	Network Selection	18

2.5.9	IMS Voice Services	19

2.5.10	User interface on accessories	19

2.6	Automatic optimisation	20

2.7	Application imposed limitations.	21

2.8	User imposed limitations.	21

2.9	Interaction with automatic device configuration	21

2.10	eUICC	22

2.11	NFC	23

2.12	EAP SIM	23

2.13	Performance	23

2.13.1	LTE Performance – non Carrier Aggregation	23

2.13.2	LTE Performance – with Carrier Aggregation	23

2.13.3	WCDMA Performance	23

2.13.4	GSM Performance	24

2.13.5	CDMA/CDMA2000 Performance	24

2.13.6	5G Performance	24

2.14	Automatic call forwarding between SIMs.	24

Annex A	Document Management	27

A.1	Document History	27

A.2	Other Information	27

# Introduction

## Overview

Historically devices with multiple SIM capability have been a major product category only in specific regional markets. As markets have matured, tariffs have emerged targeting particular use cases and as a consequence multi-SIM devices are now more widespread.

Unless well designed, these devices have the capability to break or bypass existing network services. 3GPP specifications define individual network connectivity but do not cover the interactions inherent in multiple simultaneous connections.

## In Scope

This document lays out a minimum set of requirements intended to ensure multi-SIM devices show consistent behaviour.  The requirements relate only to device platform elements such as hardware, protocol stack and operating systems.

In the context of this document, a multi-SIM device is any device that natively accommodates multiple SIMs. This includes:

- The device has a single 3GPP/3GPP2 network connection and a single IMEI (International Mobile Equipment Identifier) with which a single SIM selected from several within the device can be used.

- The device has multiple simultaneous 3GPP/3GPP2 network connections and multiple IMEIs each of which is associated with a particular SIM.

Note: With the advent of IMS, it is possible to have connection to a 3GPP/3GPP2 core network without using a 3GPP/3GPP2 RAN layer. This scenario is in scope.

All combinations of physical SIM and eUICC (removable and embedded) are in scope:

- Physical SIM + Physical SIM

- Physical SIM + eUICC with one or more enabled Profiles

- eUICC with multiple enabled Profiles.

- eUICC + eUICC, each with one or more enabled Profiles

eUICCs with no enabled Profile are in scope but are treated the in the same way as an empty physical SIM slot. SGP.21 and SGP.22 v3.0 onwards define Multiple Enabled Profiles (MEP) to enable multiple Profiles within a single eUICC. Earlier versions of SGP.21 and SGP.22 are limited to enabling only one Profile at a given point in time.

Operations already covered by 3GPP are out of scope. 3GPP added some explicit requirements for Multi SIM devices in Release 16, but these are focussed on network efficiency and do not overlap with the device requirements in this document. However many of the requirements of this document build on 3GPP operations defined for single SIM cases; see below for the relevant 3GPP specifications.

## Out of Scope

Application design (including, but not limited to, RCS) is out of scope.

- NOTE: see TS.61 section 9 for RCS testing of Dual SIM devices

After-market multi-SIM accessories are out of scope.

Installing, enabling, disabling, deleting, etc. of enabled Profiles across multiple eUICCs, or of multiple enabled Profiles within a single eUICC, is out of scope.

Devices that contain more than one SIM/eUICC, but which can only use one SIM/eUICC at any given time. These are sometimes known as passive dual SIM devices.

A subscription that requires neither a SIM nor an eUICC is out of scope.

## References

| Ref | Document Number | Title |
| --- | --- | --- |
| GSMA | TS.06 | IMEI Allocation and Approval Process |
| GSMA | TS.26 | NFC Handset Requirements |
| GSMA | TS.32 | Technical Adaptation of Devices through Late Customisation |
| GSMA | TS.36 | Device Settings Database |
| 3GPP | TS 24.008 | Mobile Radio Interface Layer 3 Specification |
| 3GPP | TS 24.301 | Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS) |
| 3GPP | TS 23.122 | Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode |
| 3GPP | TS 31.102 | Characteristics of the Universal Subscriber Identity Module (USIM) application |
| 3GPP | TS 31.111 | Universal Subscriber Identity Module (USIM) Application Toolkit (USAT) |
| 3GPP | TS 25.331 | Radio Resource Control (RRC); Protocol specification |
| 3GPP | TS 36.331 | E-UTRA Radio Resource Control (RRC); Protocol specification |
| 3GPP2 | C.S0005-F | Upper Layer (Layer 3) Signalling Standard for cdma2000 Spread Spectrum Systems. |
| GSMA | SGP.21 | Remote SIM Provisioning Architecture |
| GSMA | SGP.22 | Remote SIM Provisioning Technical Specification |
| MIIT (PRC) | YDT 3040-2016 | Technical Requirements for LTE/CDMA/TD-SCDMA/WCDMA/GSM (GPRS) Multi-Mode Dual-SIM Multi-Standby User Equipment An English version of this document is available from MIIT. |
| GSMA | TS.09 | Battery Life Measurement and Current Consumption Technique |
| 3GPP | TS 37.901 | User Equipment (UE) application layer data throughput performance. |
| 3GPP | TS 24.080 | Mobile radio interface layer 3 supplementary services specification; Formats and coding |
| 3GPP | TS 24.082 | Call Forwarding (CF) supplementary services; Stage 3 |
| 3GPP | TS 24.173 | IMS Multimedia telephony communication service and supplementary services; Stage 3 |
| 3GPP | TS 24.501 | Non-Access-Stratum (NAS) protocol for 5G System (5GS) |
| 3GPP | TS 38.331 | NR; Radio Resource Control (RRC); Protocol specification |
| GSMA | TS.61 (v3.0 or later) | RCS Test Case Guidelines |

## Definitions

| Term | Definition |
| --- | --- |
| SIM | Subscriber Identity Module; a physical entity that contains keys and ID required to authenticate a user on a mobile network. “SIM” is commonly used to refer to the physical entity that is technically called the UICC (see below). This document uses “SIM” to refer to the physical entity or an equivalent implementation such as an eUICC with an enabled Profile (see below) |
| UICC | Universal Integrated Circuit Card; the physical entity that contains as a minimum the SIM/USIM application |
| USIM | An application that runs on the UICC and provides authentication functions similar to those provided by the SIM in pre-3G systems |
| eUICC | A removable or non-removable UICC which enables the remote and/or local management of Profiles in a secure way |
| Profile | A specific SIM/USIM application contained within an eUICC; as defined in SGP.21 and SGP.22. |
| MEP | An option introduced in SGP.21 & SGP.22 v3.0 that allows a single eUICC to have Multiple Enabled Profiles at the same point in time. |

## Abbreviations

| Abbreviation | Definition |
| --- | --- |
| APN | Access Point Name |
| CS | Circuit Switched |
| DSDA / MSMA | Dual SIM Dual Active / Multi SIM Multi Active |
| DSDS / MSMS | Dual SIM Dual Standby / Multi SIM Multi Standby |
| EAP | Extensible Authentication Protocol |
| IMEI | International Mobile Equipment Identifier |
| IMS | IP Multimedia Subsystem |
| ME | Mobile Equipment |
| MEID | Mobile Equipment Identifier |
| MMS | Multimedia Message Service |
| NFC | Near Field Communications |
| OS | Operating System |
| OTA | Over The Air |
| PDN | Public Data Network |
| RCS | Rich Communication Services |
| SMS | Short Message Service |
| USAT | UMTS SIM Application Toolkit |
| UE | User Equipment |
| UI | User Interface |

# Requirements

Note: As detailed in section 2.10, physical SIMs and eUICCs containing enabled Profiles are equivalent for all requirements except where differences are specifically noted.

## Number of IMEIs

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.1_REQ_1 | In accordance with GSMA TS.06, each simultaneously active SIM in a device SHALL have a unique associated IMEI. |
| TS37_2.1_REQ_2 | The SV value in all IMEISVs SHALL be the same |

Note: 	An active SIM is a SIM for which there is an active logical network connection to a 3GPP/3GPP2 network.

A MEID is specified in 3GPP2; this is identical to the IMEI except that it allows hexadecimal digits where the IMEI only allows decimals. Hence a MEID cannot be used as an IMEI, but an IMEI will function as an MEID. A multi-SIM device must use an ID suitable to all technologies supported. The GSMA TSG (Terminal Steering Group) are not aware of any multi-SIM devices that have a SIM Port only capable of 3GPP2 operations. Accordingly, this document assumes the use of IMEI for all connections.

Over-the-top services that rely on neither 3GPP/3GPP2 radio network nor 3GPP/3GPP2 core are out of scope of TS.06 and are not mandated to have an associated IMEI.

## Use of IMEIs

To ensure the correct operation of regulator-mandated (or voluntary) procedures to block the use of stolen devices on mobile networks, the following requirements must be met:

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.2_REQ_1 | Blocking of all service access from one of the device’s IMEIs SHALL result in the entire device being blocked. Specifically, if a device receives reject #6 “Illegal ME” over one 3GPP/3GPP2 connection, it SHALL block operation on all 3GPP/3GPP2 connections. Similarly, if a Lock until Power-Cycled Order is received over one 3GPP2 connection, the device SHALL block operation on all 3GPP/3GPP2 connections |
| TS37_2.2_REQ_2 | When blocking operation on 3GPP/3GPP2 connections other than the one that triggered the blocking, the device SHALL follow standard 3GPP/3GPP2 protocols. Specifically, any active traffic SHALL be immediately terminated using normal signalling and then a network detach performed |
| TS37_2.2_REQ_3 | When operation is blocked, an appropriate message SHALL be displayed on the user interface. |
| TS37_2.2_REQ_4 | To avoid the need for the user to record all device IMEIs, one IMEI SHALL be designated as primary. |
| TS37_2.2_REQ_5 | The device SHOULD use the “primary IMEI” whenever there is one active SIM / enabled eUICC profile in the device. To eliminate the user impact of modem resets required when changing SIM association, devices that support hot swap of SIMs and/or SIM / enabled eUICC profile selection through software MAY assign primary IMEI at power-on and leave assignment unchanged through subsequent hot swaps / reselections. |
| TS37_2.2_REQ_6 | When more than one active SIM / enabled eUICC profile is present, the device SHOULD use the primary IMEI plus as many other IMEIs as needed to meet the one-IMEI-per SIM / enabled eUICC profile requirement of TS.06 As per TS37_2.2_REQ_5 to eliminate the user impact of modem resets required when changing SIM association, devices that support hot swap of SIMs and/or SIM / enabled eUICC profile selection through software MAY assign primary IMEI at power-on and leave assignment unchanged through subsequent hot swaps / reselections. |
| TS37_2.2_REQ_7 | All device IMEIs SHALL be clearly presented to the user both via box labelling and the 3GPP *#06# command from the user interface |
| TS37_2.2_REQ_8 | The Primary IMEI SHALL be easily identifiable on the box and following the 3GPP *#06# command from the user interface |
| TS37_2.2_REQ_9 | A single IMEI barcode corresponding to the primary IMEI SHALL be printed on the box. |
| TS37_2.2_REQ_10 | The box SHALL list all IMEIs in human readable form |
| TS37_2.2_REQ_11 | To simplify logistics management, IMEIs allocated to a device SHOULD be shown in ascending order. The primary IMEI SHOULD be listed first and have the lowest value. |

### Unblocking / retry

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.2_REQ_12 | After receipt of a blocking reject over a 3GPP connection, retry mechanisms as specified in 3GPP TS24.008 and TS24.301 SHALL be followed. The following scenarios are envisaged by 3GPP: Retry based on T3245 timer. Retry based on UE counter mechanism. Retry following UE power cycle. Retry following SIM removal. |
| TS37_2.2_REQ_13 | After receipt of a blocking reject over a 3GPP2 connection, retry mechanisms as specified in 3GPP2 SHALL be followed |
| TS37_2.2_REQ_14 | Change of SIM associations within a multi-SIM device SHALL trigger retry as this is functionally equivalent to SIM removal. |
| TS37_2.2_REQ_15 | VOID |
| TS37_2.2_REQ_16 | If available, the SIM associated with the connection over which the blocking reject was received SHALL be retried first; if this attach is successful other connections SHALL then be restored. This only applies to timer and counter based retries – retry following power cycle will not have knowledge of an earlier reject. |

## Limitations of specific SIM ports

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.3_REQ_1 | If any of the SIM ports are restricted in the cellular technologies, bearers or bands supported, this SHALL be clearly marked on the device. Preferably this SHOULD be a permanent marking. If permanent marking is incompatible with the device design, then user-removable stickers MAY be used. |
| TS37_2.3_REQ_2 | Device documentation SHALL record the technology bearers and bands supported by each SIM port |

Note: 	A SIM port is the physical and electronic housing provided on a device to accommodate a physical SIM card. See a later section for SIM profiles held in an eUICC.

If all SIM ports support all technologies, then physical marking is not required.

For limitations imposed by software, see the user interface section below.

Ideally documentation SHALL record capability in tabular form, for example:

|  | GSM | WCDMA | LTE | 5G | TD-SCDMA | CDMA2000 |
| --- | --- | --- | --- | --- | --- | --- |
| SIM Port 1 | Data Voice None | Data Voice None | Data IMS Voice Fallback Voice Dual Radio Voice None | Data IMS Voice Fallback Voice None | Data Voice None | Data Voice Dual Radio Voice None |
| … |  |  |  |  |  |  |
| SIM Port n | Data Voice None | Data Voice None | Data IMS Voice Fallback Voice Dual Radio Voice None | Data IMS Voice Fallback Voice None | Data Voice None | Data Voice Dual Radio Voice None |

All supported bearers for each technology on each SIM port shall be ticked. If none are supported then “None” shall be ticked.

Note: 	“Dual Radio Voice” refers to the use of CS voice in CDMA2000 with simultaneous LTE PS data traffic. As such, if the box is checked for LTE it must also be checked for CDMA 2000 (and vice versa)

Additional columns for other technologies – for example narrow band cellular - are permitted.

Additional entries for bearers are also permitted. For example, IMS voice in 2G and 3G is theoretically possible, but at present is not deployed.

Examples of technology limitations include the following, but others are possible:

SIM Port 1 supports 4G/3G/2G while SIM 2 is 2G / 3G.

SIM Port 1 supports 3G/2G while SIM Port 2 is 2G / 3G.

Examples of bearer limitations include the following, but again others are possible:

SIM Port 1 supports voice and data while SIM Port 2 is voice-only.

SIM Port 1 supports IMS and CS voice while SIM Port 2 is CS voice only.

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.3_REQ_3 | “All Mode ”Devices to be sold in the Peoples Republic of China SHALL support both of the cellular technology combinations specified by the requirements in YDT 3040-2016 (see references).These are summarised below Note: Other models of devices which support a subset of the network options below are acceptable in China BUT these are not classified as “All Mode” devices. |

Note: The existing All Mode specification YDT 3040-2016 does not cover 5G devices. Accordingly, the 5G columns below are marked “N/A”. A new specification is in development and its contents will be reflected here when available.

Combination 1:

|  | GSM | WCDMA | LTE | 5G | TD-SCDMA | CDMA2000 |
| --- | --- | --- | --- | --- | --- | --- |
| SIM Port 1 | Data Voice | Data Voice | Data IMS Voice Fallback Voice Dual Radio Voice | N/A | Data Voice | Optional |
| SIM Port 2 | Data Voice | Optional | Optional | N/A | Optional | Data Voice Dual Radio Voice |

Combination 2:

|  | GSM | WCDMA | LTE | 5G | TD-SCDMA | CDMA2000 |
| --- | --- | --- | --- | --- | --- | --- |
| SIM Port 1 | Data Voice | Data Voice | Data IMS Voice Fallback Voice Dual Radio Voice | N/A | Data Voice | Data Voice Dual Radio Voice |
| SIM Port 2 | Data Voice | Optional | Optional | N/A | Optional | Optional |

## Operational Mode

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.4_REQ_1 | Device documentation SHALL record the mode(s) of multi-SIM operation available |

Known operational modes at the time of writing are as follows, but others are possible:

Passive: the device contains two SIMs, but only one can be selected for use at any given time. Passive Dual SIM devices are effectively a single SIM device; the SIMs share a single cellular transceiver and only have logical connection to a single network at any given time. Such devices are outside the scope of this document as beyond a simple UI switch they are single SIM.

Dual SIM Dual Standby (DSDS): both SIMs can be used for idle-mode cellular network connection, but when a cellular radio connection is active the second connection is limited:

As in the passive case, the SIMs in a DSDS device share a single cellular transceiver. Through time multiplexing two radio connections are maintained in idle mode. When in-call on network for one SIM it is no longer possible to read paging on the network of the second SIM, hence that connection is unavailable for the duration of the call. Registration to the second network is maintained.

During a data session, data connection on the primary is on a best effort basis to accommodate reading of the secondary connection paging. Note that DSDS devices that support Voice over Wi-Fi generally allow voice connections to be maintained over the Wi-Fi bearer regardless of the status of the cellular bearers.

Dual SIM Dual Active (DSDA): both SIMs can be used in both idle and connected modes. Each SIM has a dedicated transceiver, meaning that there are no interdependencies on idle or connected mode operation at the modem level. Note that in some DSDA devices the second transceiver may be 2G-only.

Dual SIM Dual IMS, Dual SIM dual VoLTE (DSDI or DSDV): both SIMs can be used for idle-mode cellular network connection, but when a cellular radio connection is active the second connection is limited. Unlike DSDS, calls and texts can still be made & received on the SIM associated with the second connection - these are sent as IMS data over the active cellular radio connection and routed to an appropriate ePDG (the network gateway used for IMS over Wi-Fi).  As for DSDS, DSDI/DSDV devices that support Voice over Wi-Fi generally allow voice connections to be maintained over the Wi-Fi bearer regardless of the status of the cellular bearers. This means that routing of voice over cellular to an ePDG will normally only happen when Wi-Fi is not available.

By extension, Multi SIM Multi Standby (MSMS) and Multi SIM Multi Active (MSMA) are likely in the future. However, if the number of supported SIMs is greater than two, then hybrid modes are also possible.

### USAT

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.4_REQ_2 | When a device is DSDA (or MSMA) USAT commands SHALL be supported on all SIM ports. |
| TS37_2.4_REQ_3 | When a device is DSDS (or MSMS) USAT commands requiring network access SHALL be immediately actioned on the in-call SIM port; If the ME is not able to process USAT commands requiring network access on the other SIM port(s) the ME SHALL inform the SIM that it is unable to process the command ("ME currently unable to process command" or "Network currently unable to process command") as specified in the USAT specification. USAT commands not requiring network access SHALL be supported on all SIM ports. |
| TS37_2.4_REQ_4 | When a device is Passive multi SIM, USAT Commands SHALL be supported on the SIM port selected for use. USAT Commands not requiring network access MAY be supported on the other SIM ports |

### Single SIM operation

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.4_REQ_5 | If there is only one active SIM in a multi-SIM device, the device SHALL behave as Single SIM device. This means that cellular services, SIM toolkit, SIM phonebook etc. are enabled only on the active SIM. This applies to the following scenarios: - One physical SIM is the only SIM present. - One eUICC with an active profile is the only SIM present. - More than one SIM/eUICC is present, but only one is activated in the user interface or through profile management. |

## User interface

### SIM Selection

Selection between SIMs through software is not mandatory.

If software selection of SIMs is implemented, the following requirements apply:

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.5_REQ_1 | SIM selection SHALL be implemented through operating system menus for devices with a display. SIM selection using an application or Web UI MAY be used for devices without a display. |
| TS37_2.5_REQ_2 | For OS, application and Web UI implementations, any restrictions in cellular technologies, bearers or bands accessible under particular configurations SHALL be clearly indicated |
| TS37_2.5_REQ_3 | The device SHALL allow the user to select a preferred SIM for data. |
| TS37_2.5_REQ_4 | If the user does not select a preferred SIM, this setting SHALL default to the SIM with the highest technology generation available. |
| TS37_2.5_REQ_5 | If the device implementation allows the user to configure other limitations (e.g. Preferred SIM for Voice, preferred SIM for SMS, preferred SIM for MMS) the selected options SHALL be clearly indicated. |
| TS37_2.5_REQ_6 | If a multi-SIM device contains a single SIM, that SIM SHALL automatically be selected as the preferred SIM for all services. In this case the user SHALL not be allowed to change the preference |
| TS37_2.5_REQ_7 | If the SIM association with IMEI is dynamically changed, the device SHALL fully detach from the affected 3GPP/3GPP2 network(s) using the original IMEI(s), before beginning new attach procedure(s) with the new IMEI(s) |
| TS37_2.5_REQ_8 | Alteration of SIM association with SIM port SHALL be treated as new SIM insertion – specifically a modem and SIM reset SHALL be performed to ensure that all required parameters are synchronised between SIM and modem |

Note: 	TS37_2.5_REQ_5 applies to device limitations only; limitations arising from subscriber profile SHALL be handled according to 3GPP specifications.

Note: 	TS37_2.5_REQ_7 applies mainly to the case where user action has changed the SIM association. It MAY also apply automatically in certain cases (for example where a SIM has been rendered inactive via OTA programming)

### Idle Mode

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.5_REQ_9 | In idle mode, network identifier, roaming status, technology, and signal strength SHALL be individually displayed for each active SIM. This requirement applies to OS, application and Web UI |
| TS37_2.5_REQ_10 | Operator information for each active SIM SHALL be displayed on the lock-screen if the device has a lock screen |

### Calls, Data, SMS and MMS

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.5_REQ_11 | For mobile terminated calls, SMS and MMS, the user interface SHALL indicate the connection on which the call/SMS/MMS is received |
| TS37_2.5_REQ_12 | For mobile originated calls, SMS and MMS, the user interface SHALL allow the user to select the connection used to make the call. The following selection routes are suggested: There are two voice dial keys on the interface of the device to differentiate two SIMs. There is one voice dial key on the interface of the device. After the user clicks the key, a dialog box is displayed for the user to select the originating SIM. A universal default setting as per requirement 2.5_REQ_5. |
| TS37_2.5_REQ_13 | If the device implements the dialog box option listed in TS37_2.5_REQ_12, this SHALL NOT be shown in the case of an emergency call. Emergency call SHALL be initiated immediately on any available connection. “Emergency camped-on” state MAY be used if the home network is not available. Emergency calls SHALL be handled in accordance with 3GPP specifications. In the case of a device with multiple SIMs present the procedure SHOULD be tried on each SIM until a call is successfully connected. The order in which SIMs are used is for device manufacturers to decide. Following termination of an IMS emergency call on one SIM, the device MAY remain in IMS REGISTERED state on that SIM until the SESSION EXPIRATION timer triggers or there is user action on the other SIM. This is to support emergency service callback. Note that in DSDS devices this has potential to cause missed calls on the second SIM The device SHOULD consider all information it has available to identify applicable emergency numbers (See Note 2). |
| TS37_2.5_REQ_14 | Call logs SHALL indicate the connection on which the call was made/received/missed/rejected |
| TS37_2.5_REQ_15 | SMS logs SHALL indicate the connection on which the SMS was sent/received. |
| TS37_2.5_REQ_16 | If the Device has a data use display, data use SHALL be shown for each connection. Total data use MAY also be shown |
| TS37_2.5_REQ_17 | Cell broadcast configuration SHALL be controlled independently for each SIM |
| TS37_2.5_REQ_18 | The user interface SHALL indicate which connection cell broadcast messages were received over. |
| TS37_2.5_REQ_19 | The device MAY display cell broadcast messages in idle and/or lock screens. If they are shown, then the connection over which they were received SHALL be indicated |
| TS37_2.5_REQ_20 | Calls, SMS and MMS on one SIM SHALL interrupt data traffic on another SIM if the device does not allow both services simultaneously (see Note 1). |
| TS37_2.5_REQ_21 | In a Dual SIM Dual Standby (DSDS) environment, if the data download is suspended / interrupted because of a higher priority event (e.g. incoming call), then it SHALL attempt to resume when applicable (e.g. after hang up of incoming call) and on the relevant SIM (e.g. if started on SIM 1, then resume on SIM 1). |
| TS37_2.5_REQ_22 | In a Dual SIM Dual Standby (DSDS) environment, if the data download is suspended / interrupted because of a higher priority event (e.g. incoming call), and resumption as per TS37_2.5_REQ_21 is not possible (for example due to FTP timeout) then it SHALL restart when applicable (e.g. after hang up of incoming call) and on the relevant SIM (e.g. if started on SIM 1, then restart on SIM 1). |

Note 1: 	TS37_2.5_REQ_20 is relevant to DSDS devices, for example: SIM #1 is chosen as the default data SIM and packet data service is active.

Calls/SMS/MMS of SIM #1 can be used together with the packet data service of SIM #1.

Calls/SMS/MMS of SIM #2 cannot be used together with the packet data service of SIM #1.

Calls/SMS/MMS priority is higher than data service. Thus, when using SIM#2 making phone calls the data service of SIM #1 is shut down and when the SIM#2 finishes the phone call service the data service of SIM#1 can begin again.

There are two acceptable options for interrupting data traffic:

  - Stop data operation without any signalling to the network. Resume through the retry mechanisms normally used when a device loses and then regains coverage.

  - Stop data operation by signalling the network but leave the network registration in place. Resume by way of explicit signalling

Note that if option 1) is implemented then explicit signalling would still be required if the interruption exceeds the data link timeout.

This limitation does not apply to DSDA devices.

Section 9 of GSMA TS.61 “RCS Test Case Guidelines” (v3.0 or later) for guidance on expected behaviour of an RCS client on a Dual SIM device.

Note 2: 	There are many elements that can be considered when identifying applicable emergency numbers. Examples include: EF_ECC files in each SIM / enabled eUICC Profile; ECC list in NAS signalling; device and/or OS customisation; device location combined with a lookup table in the device; SIP 380 signalling redirect. This is not intended as an exhaustive list and it is expected that other methods will arise in future.

### Supplementary services

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.5_REQ_21 | Call forwarding SHALL be controlled independently for each SIM. This applies whether the device is Passive, DSDS or DSDA. |
| TS37_2.5_REQ_22 | Call waiting SHALL be controlled independently for each SIM. This applies whether the device is Passive, DSDS or DSDA. |
| TS37_2.5_REQ_23 | A DSDA device SHALL allow an ongoing call to be placed on hold while a call on the other connection is answered or initiated. |

### SIM PIN

SIM PIN within a single SIM device shall be implemented in accordance with 3GPP standards. Requirements specific to a multiple SIM device are as follows:

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.5_REQ_24 | When asking the user to enter a PIN code, the interface SHALL state which SIM is being accessed. |
| TS37_2.5_REQ_25 | The SIM PIN for each SIM present in the device SHALL operate independently. Specifically, one SIM being blocked SHALL NOT prevent the device from using another (unblocked) SIM |
| TS37_2.5_REQ_26 | When asking the user to enter a PUK code, the interface SHALL state which SIM is being accessed. |

### Network & Service Provider locks.

It is expected that multi-SIM devices will normally be sold through third parties and consequently network / service provider locks will not be activated. However the underlying hardware and software will support the operation, so the following requirements are included for completeness.

It is also possible that multiple locks are implemented in the same device. This may lock all ports to the same network – for example where a network operator sells a multi-SIM device – or lock ports to different networks – for example to support certain roaming propositions.

Network / Service Provider lock on a single connection shall be implemented in accordance with 3GPP standards. Requirements specific to a multiple SIM device are as follows:

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.5_REQ_27 | When asking the user to enter an unlock code, the interface SHALL state which SIM port is being accessed. |
| TS37_2.5_REQ_28 | Network / Service Provider locks SHOULD operate independently. Specifically: One SIM port being locked SHOULD NOT prevent the device from using another (unlocked) SIM port. All SIM ports MAY be locked to a single Network / Service Provider If all SIM ports are locked to a single Network / Service provider, it SHALL be possible to unlock them independently. SIM Ports MAY be locked to different Network / Service Providers One SIM port MAY implement a service provider lock while another SIM port implements a network lock |
| TS37_2.5_REQ_29 | A device MAY implement a network or service provider lock on a SIM port that prevents all device operation unless an appropriate SIM is present in that SIM port. |

### Contact lists.

Read and write of contact details to and from each SIM shall be in accordance with 3GPP. Requirements specific to a multiple SIM device are as follows:

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.5_REQ_30 | The user SHALL be able to access contacts stored in any SIM present in the device |
| TS37_2.5_REQ_31 | Contacts from cloud services integrated with the device operating system and/or stored directly in the device itself SHALL be presented through the same contact manager as those from SIMs |
| TS37_2.5_REQ_32 | Contacts MAY be presented as a single consolidated list. This list SHALL indicate the source (Cloud, Device, SIMx, SIMy etc.) of each contact in the list. Duplicated contacts from different sources MAY be displayed as duplicates or MAY be consolidated to a single entry. If consolidated, all sources of the contact SHALL be indicated. |
| TS37_2.5_REQ_33 | Contacts MAY be presented as a list for each SIM / cloud service. The menu structure and screen headings SHALL indicate which list is being selected / viewed. (Cloud, Device, SIMx, SIMy etc.) |
| TS37_2.5_REQ_34 | When entering a new contact, the user SHALL be asked to select a storage location (SIMx / SIMy / device / cloud service) to which the contact is to be stored. |
| TS37_2.5_REQ_35 | The device MAY offer the option to store contacts to multiple storage locations in one operation |
| TS37_2.5_REQ_36 | When deleting a contact, the user SHALL be asked to select a storage location from which the contact is to be deleted. |
| TS37_2.5_REQ_37 | The device MAY offer the option to delete contacts from multiple storage locations in one operation. |
| TS37_2.5_REQ_38 | The device MAY offer options to copy contacts between any of the storage locations it has available |

### Network Selection

#### Automatic network selection

There are no automatic network selection requirements specific to multi-SIM devices. For each SIM normal 3GPP selection procedures apply. User interface requirements for indication of the network are covered in previous sections of this document.

#### Manual network selection

There are specific requirements relating to manual network selection in a multi-SIM device. These relate entirely to user interface – all protocol level operations follow 3GPP standards.

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.5_REQ_39 | The device SHALL allow manual network selection independently on each SIM. At each stage of selection, the device SHALL indicate the SIM to which the selection relates. Available network technologies SHALL be indicated. These MAY differ between SIMs due to hardware limitations as described in section 2.3 |
| TS37_2.5_REQ_40 | The Device MAY allow simultaneous manual network selection across multiple SIMs. When a network is selected the device SHALL indicate which SIM it is associated with. If a network may be accessed via more than one SIM, the device SHALL allow the desired SIM(s) to be selected. If forbidden PLMNs are included in the list, the SIM(s) for which they are forbidden SHALL be indicated. Available network technologies SHALL be indicated. These MAY differ between SIMs due to hardware limitations as described in section 2.3 |

### IMS Voice Services

Handsets that implement VoLTE and/or VoWiFi services can offer the user options to enable or disable these functions. If such options are presented, there are Multi SIM requirements. There are also additional requirements on status display for devices supporting IMS voice.

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.5_REQ_41 | If a device offers UI options to enable/disable VoLTE, individual options SHALL be provided for each connection that supports VoLTE. |
| TS37_2.5_REQ_42 | An option to enable / disable all VoLTE operation MAY be provided in addition to individual VoLTE enable / disable options as per TS37_2.5_REQ_41. |
| TS37_2.5_REQ_43 | If a device offers UI options to enable/disable VoWiFi, individual options SHALL be provided for each connection that supports VoWiFi. |
| TS37_2.5_REQ_44 | An option to enable / disable all VoWiFi operation MAY be provided in addition to individual VoWiFi enable / disable options as per TS37_2.5_REQ_43. |
| TS37_2.5_REQ_45 | VoLTE registration status SHALL be indicated for each connection. |
| TS37_2.5_REQ_46 | VoWiFi registration status SHALL be indicated for each connection. |

### User interface on accessories

Handsets may connect to accessories such as in-car systems, smart watches etc. If the accessory supports a multi-SIM UI, then the requirements of sections 2.5.1-2.5.9 apply.

When the accessory has a UI designed for a single SIM device, the following device requirements apply:

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.5_REQ_47 | A single combined call history list SHALL be provided to the accessory without indication of the connection associated with each call. |
| TS37_2.5_REQ_48 | An accessory instigated call to a number in the call history SHALL be made using the connection most recently associated with that number |
| TS37_2.5_REQ_49 | An accessory instigated call to a number not in the call history SHALL be made using: - The preferred connection for voice calls (if one is specified) or; - The connection most recently used for a voice call (if no preference is specified) |
| TS37_2.5_REQ_50 | A single combined SMS/MMS history list SHALL be provided to the accessory. |
| TS37_2.5_REQ_51 | An accessory instigated SMS/MMS to a number in the SMS/MMS history SHALL be made using the connection most recently associated with that number |
| TS37_2.5_REQ_52 | An accessory instigated SMS/MMS to a number not in the SMS/MMS history SHALL be made using: - The preferred connection for SMS/MMS (if one is specified) or; - The connection most recently used for SMS/MMS (if no preference is specified) |
| TS37_2.5_REQ_53 | A single combined contact list SHALL be provided to the accessory without indication of the source of each contact. This single list SHALL include contacts from the DUT and any configured cloud service. Inclusion of contacts from SIM phonebooks is optional. |
| TS37_2.5_REQ_54 | Accessory instigated data SHALL use: - The preferred connection for data (if one is specified as per TS37_2.5_REQ_3) or; - The default connection for data (as per TS37_2.5_REQ_4) if no preferred connection is specified |

## Automatic optimisation

Automatic optimisation may be applied in devices which have limitations in the technologies that can be simultaneously supported. This is advantageous in certain region-specific deployments. As it only helps in certain situations, automatic optimisation is not mandatory.

The technique can be problematic if devices are taken outside the regions it is designed for; if automatic optimisation is implemented then the following requirements apply.

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.6_REQ_1 | If an inserted SIM is identified as 2G-only (i.e. not USIM) the device MAY automatically allocate a 2G-only connection to this SIM. |
| TS37_2.6_REQ_2 | A device MAY run signalling discovery protocols to establish subscription status of inserted SIMs. Based on results of the protocol, the device MAY automatically allocate an appropriate connection to each SIM. |
| TS37_2.6_REQ_3 | If automatic optimisation according to TS37_2.6_REQ_1 or TS37_2.6_REQ_2 is active, this SHALL be clearly indicated in the user interface |
| TS37_2.6_REQ_4 | The user SHALL be able to manually override settings allocated under TS37_2.6_REQ_1 and TS37_2.6_REQ_2 |

## Application imposed limitations.

Some applications (for example networks’ customer service apps) require use of the connection associated with a specific SIM.

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.7_REQ_1 | The device SHALL provide appropriate communication to the application if the connection requested by that application is not available. |

It is the responsibility of the application to present appropriate messaging to the user.

## User imposed limitations.

Optionally the device may allow the user to associate a specific application to a specific SIM.

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.8_REQ_1 | The device SHALL provide appropriate communication to the application if the connection associated with that application is not available. |

Again, it is the responsibility of the application to present appropriate messaging to the user.

## Interaction with automatic device configuration

Support of auto configuration is optional but is strongly recommended for connectivity and service configurations.

Where implemented, automatic configuration for each SIM SHALL follow the GSMA Technical Adaptation of Devices Requirements TS.32 (see references). Multi SIM specific requirements are as follows:

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.9_REQ_1 | If the device supports auto-configuration based on the SIM inserted: Voice, Messaging and Data connectivity settings (e.g. PDN / APN) SHALL be configured according to the SIM associated with that connection. If application layer configuration is applied, it SHALL be that applicable to the SIM selected as primary at first power on or following USAT REFRESH command. Radio capability SHALL be auto-configured according to the SIM associated with that connection. Service configurations (e.g. IMS) SHALL be auto configured according to the SIM associated with that connection |
| TS37_2.9_REQ_2 | If only one radio / service configuration can be used, the configuration applied to items indicated in TS37_2.8_REQ_1 SHALL be that applicable to the SIM using the primary IMEI at first power on or following USAT REFRESH command. Note that in the case of service configuration, such a limitation will require “marking” as described earlier in this document. |
| TS37_2.9_REQ_3 | In accordance with TS.32, reconfiguration of the items indicated in TS37_2.9_REQ_1 in case of selecting a new SIM using the primary IMEI is optional, but SHALL be documented if implemented. |

## eUICC

Operation of an eUICC is specified through the GSMA eSIM working group documents SGP.21 and SGP.22. Requirements applicable to Multi SIM devices are as follows:

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.10_REQ_1 | An eUICC with an enabled Profile SHALL be treated as a normal SIM for the purposes of all previous sections of this document. Physical marking requirements are optional for eUICCs. Documentation of technology, band and bearer limitations is mandatory. |
| TS37_2.10_REQ_2 | Mechanisms for eUICC and Profile management (e.g., installation, enabling, disabling & deletion of Profiles) on eUICCs SHALL meet the requirements specified in SGP.21 & SGP.22. |
| TS37_2.10_REQ_3 | User interface operations that indicate associated SIM (contact management, network selection, etc.) MAY indicate whether each SIM is eUICC or non-eUICC. |
| TS37_2.10_REQ_4 | An eUICC with no enabled Profile SHALL be treated as an empty SIM slot for the purposes of all previous sections of this document. |
| TS37_2.10_REQ_5 | An eUICC MAY implement Multiple Enabled Profiles (MEP) as defined in SGP.21 & SGP.22 v3.0 onwards. In this case, each enabled Profile SHALL be treated as a normal SIM in accordance with TS37_2.10_REQ_1 above. |

Management of multiple eUICCs in the same device or of multiple Profiles enabled within the same eUICC is the responsibility of the GSMA eSIM working group. Requirements can be found in SGP.21 and SGP.22.

## NFC

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.11_REQ_1 | NFC operation in a Multi SIM device SHALL be as defined in TS.26 v10 or later |

## EAP SIM

EAP-SIM allows Wireless LAN users to authenticate to a Wireless LAN network using credentials from a SIM card. Clearly this has implications for a Multi SIM device.

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.12_REQ_1 | If a device supports EAP SIM it SHALL be supported on all SIM ports |
| TS37_2.12_REQ_2 | User interface options SHALL allow enable / disable of EAP for each SIM port |
| TS37_2.12_REQ_3 | User interface MAY allow specific Wi-Fi networks to be associated with specific SIM ports |

## Performance

It is expected that a device operating in multi-SIM configuration may show lower data throughput than when operated in single SIM configuration.

Currently only dual SIM dual standby devices are addressed; dual SIM dual active devices are not common enough to establish a baseline.

### LTE Performance – non Carrier Aggregation

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.13_REQ_1 | Applies to Dual SIM Dual Standby (DSDS) devices supporting LTE on at least one connection. When a device is performing LTE data transfer on one SIM and monitoring for LTE, WDCMA or GSM paging on the second SIM: LTE data throughput in both downlink and uplink SHALL be at least 90% of throughput measured on the same device operating with a single SIM. This applies for connected mode configuration as defined in 3GPP TS37.901 and idle mode configuration as defined in GSMA TS.09. |

### LTE Performance – with Carrier Aggregation

Cases where one or both subscriptions use LTE carrier aggregation are not addressed as this technology is not covered by 3GPP TS37.901.

For future study, once devices with this technology become widespread.

### WCDMA Performance

Data transfer over a WCDMA connection, with a second connection in idle mode.

This case is not addressed due to lack of suitable industry agreed configurations. It is noted for future study.

### GSM Performance

Data transfer over a GSM connection, with a second connection in idle mode.

This case is not addressed due to lack of suitable industry agreed configurations. It is noted for future study

### CDMA/CDMA2000 Performance

Cases where one or both subscriptions use CDMA/CDMA2000 are not addressed as there are no equivalents to GSMA PRD TS.09 and 3GPP TS37.901 for this technology.

It is not anticipated that such specifications will become available.

### 5G Performance

#### Stand Alone (SA) configuration.

Data transfer over a 5G standalone connection, with a second connection in idle mode.

This case is not addressed in this version of the specification due to lack of suitable industry agreed configurations. It is noted for future study.

#### Non Stand Alone (NSA) configuration.

Data transfer over a 5G non standalone connection (i.e. using a 4G anchor layer), with a second connection in idle mode.

This case is not addressed in this version of the specification due to lack of suitable industry agreed configurations. It is noted for future study.

## Automatic call forwarding between SIMs.

Some DSDS devices offer automatic call forwarding between connections. When the device is in-call on one connection, incoming calls to the inactive connection are automatically forwarded to the in-call connection.

Incoming calls on the inactive connection will be forwarded to the in-call connection and follow the settings for the in-call connection:

- If call waiting is enabled, they will appear as a call waiting indication in the active call or

- If call waiting is disabled, they will be forwarded to voice mail associated with SIM of the in-call connection rather than going to voicemail associated with the inactive connection.

For case (A), if the user rejects or does not answer the waiting call then call forwarding settings for the SIM associated with the in-call connection will be followed.  This will normally result in the call being sent to voice mail associated with the SIM for the in-call connection.

This service makes use of conventional network 3GPP supplementary services to forward calls when the user is unreachable, but these are configured automatically by the device.

The relevant 3GPP signalling is defined in 3GPP TS 24.080, 24.082 and 24.173.

Clearly such a service can only operate if the relevant network(s) support both call waiting and call forwarding.

If a device supports this service, the following requirements apply:

| Requirement ID | Requirement |
| --- | --- |
| TS37_2.14_REQ_1 | Automatic call forwarding between SIMs SHALL be enabled/disabled by simple controls in the User Interface |
| TS37_2.14_REQ_2 | Automatic call forwarding between SIMs SHALL be individually controlled for each SIM combination. |
| TS37_2.14_REQ_3 | Automatic call forwarding between SIMs SHALL be individually controlled for each direction. i.e. enabling forwarding from SIM1 to SIM2 SHALL NOT automatically enable forwarding from SIM2 to SIM1 |
| TS37_2.14_REQ_4 | When a user accesses the UI for automatic call forwarding between SIMs, the device SHALL use 3GPP supplementary service commands to acquire the current call forwarding settings of each SIM |
| TS37_2.14_REQ_5 | Appropriate automatic call forwarding between SIMs settings SHALL be shown as “ON” if any of the SIMs are found to already be configured to forward to each other when unreachable |
| TS37_2.14_REQ_6 | Devices SHALL NOT change call forwarding settings without a specific user request via the UI |
| TS37_2.14_REQ_7 | Changing the automatic call forwarding between SIMs setting SHALL only be possible when all the associated networks are available. |
| TS37_2.14_REQ_8 | If the user attempts to activate automatic call forwarding between SIMs the device SHALL check all SIMs for currently active call forwarding settings:If any active call forwarding when unreachable settings are found, the user SHALL be informed that activating automatic call forwarding will overwrite the currently active call forwarding settings and given the option to continue or cancel their request for automatic call forwarding between SIMs |
| TS37_2.14_REQ_9 | The user interface for call forwarding between SIMs SHALL show the numbers that will be used for forwarding. These numbers MAY be automatically populated by the device, but if automatically populated the user SHALL be able to edit them. |
| TS37_2.14_REQ_10 | Disabling of automatic call forwarding between SIMs SHALL result in each SIM “forward when unreachable” setting reverting to the setting that applied before automatic forwarding was enabled. |
| TS37_2.14_REQ_11 | Enabling / disabling of automatic call forwarding between SIMs SHALL be controlled by the device sending 3GPP supplementary service “call forwarding when unreachable” configuration messages to the network |
| TS37_2.14_REQ_12 | If enabling or disabling of automatic call forwarding fails the user interface SHALL inform the user. |
| TS37_2.14_REQ_13 | If enabling or disabling of automatic call forwarding fails only in part, the successful configuration MAY be retained. In this case the user interface SHALL inform the user of which element(s) of call forwarding are active. For example, it is possible that forwarding of SIM1 to SIM2 sets up successfully, but SIM2 to SIM1 setup fails. |

Note: Multi SIM devices can be enrolled in the “one number” service. This allows more than one device to be associated with a single MSISDN.

Under one number operation, if call forwarding is set, it applies to all devices using that MSISDN.

Hence turning on automatic call forwarding on a Multi SIM device will affect the operation of devices that are paired with it under one number.

This may have implications for testing of the one number service.

Document Management

Document History

| Version | Date | Brief Description of Change | Approval Authority | Editor / Company |
| --- | --- | --- | --- | --- |
| v1.0 | 14th December 2016 | 1st Version | PSMC#150 TSG#26 | Richard Ormson / Hutchison |
| V2.0 | 12th June 2017 | Updated with changes approved in CR1002 | TSG#28 | Richard Ormson / Hutchison |
| V3.0 | 21st September 2017 | Updated with changes approved in CR1003 | TSG#29 | Richard Ormson / Hutchison |
| V3.1 | 7th November 2017 | Updated with changes approved in CR1004 | TSG | Richard Ormson / Hutchison |
| V4.0 | 14th June 2018 | Updated with changes approved in CR1005 | TSG#32 | Richard Ormson / Hutchison |
| V5.0 | 4th Dec 2018 | Updated with changes approved in CR1006 | TSG#34 | Richard Ormson / Hutchison |
| V6.0 | 4th Dec 2019 | Updated with changes approved in CR1007 | TSG#38 | Richard Ormson / Hutchison |
| V7.0 | April 2020 | Updated with changes approved in CR1008 | TSG#39h | Richard Ormson / Hutchison |
| V8.0 | July 2021 | Updated with changes approved in CR1009 | TSG#44 ISAG#10 | Richard Ormson / Hutchison |
| V9.0 | February 2022 | Updated with changes approved in CR1010 | TSG RMS | Richard Ormson / Hutchison |
| V10.0 | February 2023 | Updated with changes approved in CR1011 | TSG#50 ISAG#27 | Richard Ormson / Hutchison |
| V11.0 | March 2024 | Updated with changes approved in CR1012 | TSG#55 ISAG#40 | Richard Ormson / Hutchison |
| V12.0 | April 2025 | Updated with changes approved in CR1013 | TSG#63 ISAG#61 | Richard Ormson / Hutchison |

Other Information

| Type | Description |
| --- | --- |
| Document Owner | Terminal Steering Group (TSG) |
| Editor / Company | Richard Ormson / Hutchison 3G UK Limited |

It is our intention to provide a quality product for your use. If you find any errors or omissions, please contact us with your comments. You may notify us at prd@gsma.com

Your comments or suggestions & questions are always welcome.