# Technical Adaptation of Devices through Late Customisation

# Version 22.0

# 27 May 2026

Security Classification: Non-confidential

Access to and distribution of this document is restricted to the persons permitted by the security classification. This document is subject to copyright protection. This document is to be used only for the purposes for which it has been supplied and information contained in it must not be disclosed or in any other way made available, in whole or in part, to persons other than those permitted under the security classification without the prior written approval of the Association.

Copyright Notice

Copyright © 2026 GSM Association

Disclaimer

The GSMA makes no representation, warranty or undertaking (express or implied) with respect to and does not accept any responsibility for, and hereby disclaims liability for the accuracy or completeness or timeliness of the information contained in this document. The information contained in this document may be subject to change without prior notice.

Compliance Notice

The information contain herein is in full compliance with the GSMA Antitrust Compliance Policy.

This Permanent Reference Document is classified by GSMA as an Industry Specification, as such it has been developed and is maintained by GSMA in accordance with the provisions set out GSMA AA.35 - Procedures for Industry Specifications.

Table of Contents

1	Introduction	5

1.1	Overview	5

1.2	Scope	5

1.3	Definitions	5

1.4	Abbreviations	6

1.5	References	7

1.6	Conventions	7

2	Technical Adaptation of Devices	8

2.1	Introduction	8

2.2	Late Customisation	8

2.3	Difference between TAD late customisation and MNO Provisioning.	8

2.4	User experience	8

2.5	SIM lock	8

2.6	Software and OS updates	9

2.7	First boot with no SIM card	9

2.8	First SIM configuration	9

2.9	Customisation Package identification (SIM card and customer profile identification)	9

2.10	If no customisation package exists for the Identified MNO/MVNO	10

2.11	If OEM is unable to identify the MNO/MVNO	10

2.12	SIM Swap or factory reset.	10

2.12.1	Factory reset.	10

2.12.2	SIM Swap	11

2.12.3	Use cases.	11

3	Settings	12

3.1	IMS Settings	12

3.2	Access Point Name (APN)’s	12

3.2.1	AOSP APN File	13

3.3	MNO Specific supplementary Settings	13

3.4	Table 1 GSM Settings – See TS.32 Excel Sheet	13

3.5	Table 2 UTRA Settings – See TS.32 Excel Sheet	13

3.6	Table 3 HSPA settings – See TS.32 Excel Sheet	14

3.7	Table 4 LTE Settings – See TS.32 Excel Sheet	14

3.8	Table 5 LTE FGI Bits – See TS.32 Excel Sheet	14

3.9	Table 6 IMS Settings – See TS.32 Excel Sheet	14

3.9.1	Table 6a Test Profiles – See TS.32 Excel Sheet	14

3.10	Table 7 Other Settings – See TS.32 Excel Sheet	14

3.11	Table 8 APN Settings – See TS.32 Excel Sheet	14

3.12	Table 9 5G Settings – See TS.32 Excel Sheet	14

3.13	Table 10 IoT NTN Settings – See TS.32 Excel Sheet	14

3.14	Table 11 5G NTN Settings – See TS.32 Excel Sheet	14

4	Accessing preferred MNO values for Settings and parameters	14

Annex A	Settings List template	15

Annex B	Document Management	16

B.1	Document History	16

B.2	Other Information	18

# Introduction

## Overview

This document provides recommendations on a framework for manufacturers and MNO’s (mobile network operators) so they can technically configure Open Market mobile devices, to ensure the device can support the customer with the services being promoted and offered by the MNO.

The configurations would typically take place upon 1st insertion of a SIM, by a process that is known as Late Customisation, and is referred to in this document as the Technical Adaptation of a Device (TAD)

Without harmonisation of this process across the industry and a clear understanding of which settings must be configured, each manufacturer and MNO face the challenge of having to document, in individual bilateral agreements, the technical settings they need implementing.  Without a clear framework a device is being configured for a 2nd time could inherit some legacy settings from its 1st configuration, unless it is clearly understood which settings should always be configured. It is desirable to avoid such unintentional inheritance as it could compromise the performance of the MNO services and the device itself.

## Scope

This document aims to describe the settings that SHOULD be configured whenever a Technical ‘Late Customisation’ package is deployed to a device. The assumption is that the customisation packages are deployed using a mechanism under the control of the device manufacturer. The deployment mechanism is not in scope of this document.

The document also describes at high level the scenarios when a customisation package would be triggered for deployment.

This document does not specifically cover the Late Customisation of applications layer customisations including deployment of MNO specific apps, UI (User Interface) customisation and branding assets. However, it is recognised that many MNO’s offer services to customers which can use customisation of Application layer features and parameters e.g. start page, bookmarks, Google client ID, speed dial, such settings can be added in table “MNO Supplementary” although it should be noted that implementation whilst strongly encouraged is at the discretion of the OEM (Original Equipment Manufacturer).

Marketing assets such as graphical branding, OS (Operating System) and UI (User Interface) branding and installation of specific apps are not in scope of TS.32.

## Definitions

| Term | Description |
| --- | --- |
| Factory Reset | User-initiated process by which the device is returned to the state it is normally delivered in from the factory. This implies erasing all user-installed applications and data, and restoration of configuration values to those that were applied when the device left the factory. SIM Lock settings and status are NOT changed as part of this process. |
| General Settings | If an MNO is host of MVNOs, they may have General Settings for all their MVNOs that have not provisioned their own Settings. General Settings use the MNOs MCC/MNC values, but do not define the GID/SPN values. The General Settings ensure the functionality of the MNO’s services for their MVNOs. |
| In Life Service | A service provided by the MNO for use on the customer device, when the customer is under contract with that MNO |
| Late Customisation | A term used to describe the process for configuring a device upon 1st SIM insertion by a customer, after the point of sale. Usually managed and deployed by the Manufacturer. |
| Manufacturer | The commercial entity that designs and either directly fabricate or uses a 3rd party fabrication of a Mobile Device, and manages the TAD deployment service. |
| MNO Provisioning | Configuration and activation of an ‘in life’ service by an MNO. Usually managed and deployed by the MNO. |
| Open Market mobile devices | An Open Market mobile device is a device which is sold through a retail channel not tied or aligned with an MNO or MVNO. A device that is intended for potential distribution to more than one MNO. As such, the device should be able to take on the customization values of all MNOs whose SIM cards may end up being inserted into the device. |
| SIM Lock | A SIM lock, network lock or subsidy lock is a technical restriction built into devices by the devices manufacturers for use by services providers to restrict the use of these phones to specific countries and/or networks. Personalisation of Mobile Equipment, as defined in 3GPP TS22.022 (ETSI TS 122 022) allows a device to be restricted to only accepting SIM cards that meet certain criteria. For the purposes of this document, it is focused on network personalisation and/or SP personalisation. |
| TAD | Technical Adaptation of Devices – a terms to describe the technical configuration of a device using Late Customisation |

## Abbreviations

| Term | Description |
| --- | --- |
| 5G-NR | 5G New Radio |
| 5GS | 5G System |
| APN | Access Point Name |
| GID_1 | Elementary File - Group Identifier level 1` |
| GID_2 | Elementary File - Group Identifier level 2` |
| ICCID | Integrated Circuit Card Identifier This is a unique number to identify a Profile in an UICC as defined by ITU-T E.118 [4] |
| IMEI | International Mobile Equipment Identity |
| IMSI | International Mobile Subscriber Identity |
| MCC | Mobile Country Code |
| MNC | Mobile Network Code |
| MNO | Mobile Network Operator |
| MVNO | Mobile Virtual Network Operator |
| NR | New Radio |
| NTN | Non-Terrestrial Network |
| RILTE | Roaming in LTE |
| SIM | Subscriber Identity Module |
| SPN | Service Provider Name |
| OEM | Original Equipment Manufacturer |
| OS | Operating System |
| TAD | Technical Adaptation of Devices – a term to describe the technical configuration of a device using Late Customisation |
| UI | User Interface |
| UICC | Universal Integrated Circuit Card |
| UNI | User Network Interface |
| UPG | UNI Profile Group |
| VoLTE | Voice over LTE |

## References

| Ref | Doc Number | Title |
| --- | --- | --- |
| [1] | GSMA PRD TS.36 | Devices Settings Database specification |
| [2] | GSMA PRD IR.92 | IMS Profile for Voice and SMS |
| [3] | RFC2119 | RFC 2119 - Key words for use in RFCs to Indicate Requirement Levels |
| [4] | ITU-T E.118 | The International Telecommunication Union charge card |
| [5] | GSMA PRD IR.94 | IMS Profile for Conversational Video Service |
| [6] | GSMA PRD IR.51 | IMS Profile for Voice, Video and SMS over untrusted Wi-Fi access |
| [7] | GSMA PRD NG.106 | IMS Profile for Voice, Video and SMS over trusted Wi-Fi access |
| [8] | GSMA PRD NG.102 | IMS Profile for Converged IP Communications |
| [9] | GSMA PRD NG.114 | IMS Profile for Voice, Video and Messaging over 5GS |
| [10] | GSMA PRD IR.25 | VoLTE Roaming Testing |
| [11] | GSMA PRD TS.06 | IMEI Allocation and Approval Process |

## Conventions

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT", "SHOULD", "SHOULD NOT", "RECOMMENDED", "NOT RECOMMENDED", "MAY", and "OPTIONAL" in this document are to be interpreted as described in BCP 14 (RFC2119) [3] (RFC8174) [5] when, and only when, they appear in all capitals, as shown here.

# Technical Adaptation of Devices

## Introduction

This section describes what is and what is not relevant for a TAD late customisation, the high-level process to be used to identify the correct package to be deployed, and when it should be deployed.

## Late Customisation

Late Customisation is the use of techniques to configure devices at the point of SIM detection, rather than pre-configuring the device in the manufacturer’s production line.

A TAD late customisation focuses on the configuration of technical settings for core features supported by the network and device, typically related to Voice calls, messaging and data services.

## Difference between TAD late customisation and MNO Provisioning.

It is important to understand the difference between a TAD late customisation, and MNO provisioning.

TAD through late customisation is implemented on a device by the deployment of a customisation package which is typically managed and deployed by the manufacturer; this process will be triggered at the first use, on insertion of a SIM, or after a Factory Reset of the device. It will customise technical settings that are not normally configurable by the MNO or the end user.

MNO provisioning is carried out by an MNO when enabling an ‘in life’ service for a customer. Settings which can be customised via MNO provisioning would not normally be part of a TAD late customisation package.

However, in some circumstances it may be applicable for a TAD package to set a default value of such settings in a device, in order to provide a common baseline, so that MNO’s can be confident of the devices default configuration, even if a service is not enabled by them.

## User experience

The customer’s experience of a TAD implementation must positive and equivalent to a device with preconfigured software. In particular, the boot time (from power on to the user’s interaction with the manufacturer’s start-up wizard) SHALL be similar.

It is not necessary for the device to notify the user of the customisation being applied, but if the manufacturer chooses to provide such a notification, the wording must be clear and unambiguous. If considered appropriate the information relevant to the TAD customisation can form part of the interaction within the start-up wizard.

## SIM lock

A TAD customisation of an Open Market device SHALL not apply a SIM lock.

A Device that is SIM locked SHALL only be customisable to that SIM’s customisation profile.

## Software and OS updates

If a manufacturer deploys a Software or OS (Operating System) update to a device, which changes any of the parameters and values for settings (as defined in TS.32) of the already configured services and features on the device, the device shall initiate a TAD reconfiguration based the appropriate TAD settings for the MNO profile to restore the device back to the settings configuration prior to the update.

## First boot with no SIM card

If the device is first booted without a SIM, then the ‘out of box’ software configuration SHALL be used until the SIM is inserted for the first time.

Should the TAD configuration mechanisms deployed by the manufacturer require a Factory Reset to deploy the TAD customisation, once insertion of the SIM is carried out, the user SHALL be informed during the initial boot up that any personalisation they make prior to insertion of a SIM, including accounts setup, or user data created on the device (e.g. photos) may be deleted as part of the configuration process once a SIM is inserted.

## First SIM configuration

SIM detection and subsequent customisation SHALL occur with the first SIM inserted into the device, typically as part of the boot up and initialisation of the device.

## Customisation Package identification (SIM card and customer profile identification)

A device implementing TAD customisations SHALL be able to distinguish between the different MNO SIM cards and customers profiles.

It is recommended that manufacturer’s use the following combination of identifiers to ensure they implement the appropriate customisation package. The precise method and algorithm used by the manufacturer to perform any lookup and identification is at their discretion, providing they achieve the required level of granularity without needing user interaction.

SIM card and customer profile identifiers

Mobile Country Code (MCC)

Mobile Network Code (MNC)

IMSI prefix/range (if applicable)

ICCID prefix/range (if applicable)

Service Provider Name (SPN) (if applicable)

GID_1 (if applicable)

GID_2 (if applicable)

- : SIM card and customer profile identifiers

Most countries permit the support of one or more MVNOs (Mobile Virtual Network Operator) on MNO networks, sharing Mobile Country Code (MCC) and Mobile Network Code (MNC).

In the case of an MVNO using a Roaming Brokering service provided by their host MNO, which will involve a device potentially having multiple IMSI (International Mobile Subscriber Identity) and MNC, the inclusion of the ICCID (Integrated Circuit Card Identifier) is strongly recommended, and that OEM’s include this as part of their algorithm for identifying when and what customisation package to deploy.

However, since the ICCID support is optional, the MVNO using a Roaming Brokering service provided by their host MNO shall always provide the needed setting templates for each of the different MCC/MNC/SPN (Service Provider Name) or GID (Group Identifier level) values used.

Therefore, when identifying MVNOs using a Roaming Broker solution, OEMs need to consider that their customization package can be spread across several MCC/MNC values.

It is also possible that different TAD customisations will need to be implemented by the same MNO based on customer profile (Business, Consumer, Post Pay, Pre-Pay etc.).

If the SIM contains a combination of MCC, MNC, IMSI, ICCID, SPN, GID1 and/or GID2 for which there is no exact match in the settings database, the OEM shall progressively ignore elements from the bottom of the list until a match is found. In most cases this will result in use of General Settings of the parent MNO. If a match is still not found, refer to the following section of this document.

For the Identifier fields, more than one complete set of identifiers listed in Table 1 can be provided.

## If no customisation package exists for the Identified MNO/MVNO

If no customisation package exists for the identified MNO/MVNO then the GSMA recommends that manufacturers SHOULD deploy a generic package based on default values identified in section 3, if specified; where default values are not specified the manufacturers preferred value SHOULD be used.

## If OEM is unable to identify the MNO/MVNO

If the MNO/MVNO cannot be identified, the GSMA recommends that manufacturers SHOULD deploy a generic package based on default values identified in section 3, where default values are not specified the manufacturers preferred value SHOULD be used.

## SIM Swap or factory reset.

### Factory reset.

A factory reset SHALL take the device back to ‘out-of-box settings. If the device is restored to out-of-box settings (factory reset) then the TAD customisation should be invoked when a SIM is inserted.

This allows a User to change MNO or to pass their device to a different User who will have an Open market-like device.

The factory reset SHALL delete all user content. It would be appropriate to ensure users are fully aware of this and encouraged to take suitable steps to protect and backup critical data.

### SIM Swap

If a device has been configured with the first SIM card, and later a different SIM card is inserted, the TAD mechanism MAY be triggered.

Any deployment of a TAD customisation package in such a scenario SHALL require interaction and permissions from the customer.

Should such a change be implemented then the manufacturer SHALL be able to configure the device back to the original MNO settings, should the 1st SIM card be re-inserted. Unless the user confirms that they do not require roll back to the original configuration.

When such a use case is encountered the following SHALL always apply.

Reconfigure Specific Technical Settings as defined in this document, the manufacturer must ensure continuity of key services including voice, messaging and data connection.

Minimise impact on the user ensuring a smooth user experience during this configuration.

Protect user content and service configurations not specific to the new MNO, i.e. any account, personal data and files must be backed up for automatic restoration or remain untouched unless the user gives permission to remove them.

Users should be notified that some services such as sending email may not work until they have configured the application based on the requirements of the new MNO, they should be referred to their MNO for these settings.

### Use cases.

Scenario No 1

A device is powered on for the first time with an MNO-1 SIM. The device is configured according to MNO-1’s requirements.

The device is subsequently turned on with an MNO-2 SIM card: The Device will identify if the Manufactures Late Customisation package for the MNO/Device is available.

If a Late customisation package is available, the User is asked if they wish to re-configure the device for the MNO-2 SIM. If yes, then the process’s described in 2.12.2 of this document SHALL be implemented.

Scenario No 2

The User plans to pass the device on someone else, and so performs a factory reset, returning the device to out-of-box settings.

The new User inserts MNO-2 SIM. The TAD customisation SHALL be invoked according to MNO-2’s requirements.

# Settings

The following settings shown in Tables 1 to 7 SHALL always be part of a TAD customisation package.

The parameter/value type that is associated with that setting has been listed.

Default values for the setting have been recommended. These shall be used when deploying a generic TAD customisation, or when an MNO has not provided a specific value. Where no default values exist, the OEM shall use their preferred value for this setting.

Therefore, when implementing a customisation, the following priorities should be applied when selecting which setting is to be used, this is as defined in GSMA PRD TS.32 Annex A or the GSMA TS.36 [1] Network Settings Exchange.

Priority 1: MNO Recommended Value

Priority 2: Where MNO recommended value is blank use TS.32 default Value.

Priority 3: Where both MNO recommended, and Default values are blank the OEM may use their preferred value.

Settings which are defined in this document should not cause conflict with other regulation or mandatory specifications. If this is found to be the case, then the specific settings impacted from this document may be disregarded.

The setting lists below are broken down by high level technology.

Each table contains the:

Setting name

Setting value option -

Default value (unless otherwise specified by an MNO)

Comment – additional notes for clarity

## IMS Settings

IMS settings in Table 6 have been defined by GSMA UPG group. The IMS settings are related to the provision of Operator based communication services as defined in UPG PRDs IR.92 [2], IR.94 [5], IR.51 [6], NG.106 [7], NG.102 [8] & NG.114 [9]. The list of settings in this document represents UPG’s advice on which parameters within its PRDs have optionality and therefore need to be defined by the MNO. If a setting in any UPG PRD is not in this list, then the value for that setting is clearly defined within the appropriate UPG PRD.

## Access Point Name (APN)’s

TS.32 allows the capture of an MNOs APN details, these should be listed in Table 8, when listing an APN, the MNO SHALL provide the information detailed in Table 8 for each of the relevant APN type, these details are the minimum that should be supplied.

It is permissible to list multiple APN’s, e.g. Default APN, MMS APN, Tethering APN, Event APN etc. This can be done by listing multiple supplementary APNs in Table 8.

An MNO SHALL indicate the type of APN in the ‘APN Type’ field e.g. whether it is a Default, MMS or Supplementary APN.

### AOSP APN File

The GSMA notes that APN details for MNOs are also captured in the Android Open-Source Project, which maintains a file listing APN’s, it can be accessed from the link below;

https://android.googlesource.com/device/sample/+/master/etc/apns-full-conf.xml

The file can be updated by any AOSP contributor using normal processes for submission to the AOSP

The APN Settings Table below allows a MNO to indicate if this file also contains their APN information.

## MNO Specific supplementary Settings

The TAD settings list has been defined by GSMA to provide a common group of settings which are always defined in a Late Customisation, however it is recognized that in some circumstances MNO’s may have a requirement to define settings outside of this list.

If such settings are not configurable by the MNO using MNO Provisioning, then it is acceptable for the MNO to define a list of supplementary settings for inclusion in a Late Customisation. However, the manufacturer will need to confirm to the MNO that they can configure these settings as they are outside the scope of the agreed TAD settings list

Such supplementary settings can be added using the Excel Template associated to this document, or using the process defined in GSMA TS.36 [1].

Supplementary settings would use the same format as defined in this document, plus indicate which of the Technology Groupings it belongs to e.g. GSM, UTRA, HSPA, LTE, IMS, Other. The table header below shows the fields to be defined for these supplementary settings:

| Index | Technology Group (e.g. GSM,HSPA,LTE, IMS ) | Setting Name | Permissible Setting Value | Default Value | Additional Comment |
| --- | --- | --- | --- | --- | --- |

It is recommended that supplementary settings should be kept to a minimum by any MNO. The MNO should take care not to include settings where the value is well defined in relevant specifications. MNOs should take care not to define settings which encourage deviation from relevant specifications.

Note: Empty cells in 'Default Values' column indicates that the field as per GSMA default configuration does not have a value.

## Table 1 GSM Settings – See TS.32 Excel Sheet

## Table 2 UTRA Settings – See TS.32 Excel Sheet

## Table 3 HSPA settings – See TS.32 Excel Sheet

## Table 4 LTE Settings – See TS.32 Excel Sheet

## Table 5 LTE FGI Bits – See TS.32 Excel Sheet

## Table 6 IMS Settings – See TS.32 Excel Sheet

Table 6 lists the IMS settings for the control of Operator based communication services.

NOTE:	The index naming scheme “VxLTE” was originally chosen due to the parameters being defined in GSMA PRD IR.92 [2] for the VoLTE service. However, these parameters are generic IMS parameters are also equally applicable to the provision of Operator based communication services over other access types such as Wi-Fi and 5G-NR.

### Table 6a Test Profiles – See TS.32 Excel Sheet

In order to streamline device testing a number of recommended settings (device service profiles) were defined in GSMA PRD IR.25 [10] and reflected in the associated UPG PRDs such as IR.92 [2] etc. Six such service-oriented profiles were defined. The Excel Sheet (Tab Test Profiles) contains the suggested settings of each of the configuration parameters across the six profiles.

## Table 7 Other Settings – See TS.32 Excel Sheet

## Table 8 APN Settings – See TS.32 Excel Sheet

## Table 9 5G Settings – See TS.32 Excel Sheet

## Table 10 IoT NTN Settings – See TS.32 Excel Sheet

## Table 11 5G NTN Settings – See TS.32 Excel Sheet

# Accessing preferred MNO values for Settings and parameters

The TAD customisation shall be implemented by OEM’s for their Late Customisation deployments.

MNO’s and MVNO’s are requested to record their preferred values in the GSMA Network Settings Exchange, which is defined in GSMA TS.36 [1].

If an MNO or MVNO has not entered their data into the Network Settings Exchange then the OEM shall establish a bilateral agreement for the MNO or MVNO to send them their settings, for this purpose it is recommend that the template provided in Annex A of this document is used. This includes a column for MNO recommended Value.

If an MNO/MVNO settings are not available within the GSMA Network Settings Exchange or via bilateral agreement, then the manufacturer SHOULD use the default values to deploy a generic TAD customisation package, where no default values exist the OEM SHOULD use their preferred value for this setting.

For each set of preferred values, the MNO’s and MVNO’s are requested to select all the equipment types (see GSMA TS.06 [11]) these preferred values apply to. More than one set of values can be provided but an equipment type can only be indicated at most on one set of preferred values.

- Settings List template

Use the following Excel template when and OEM requests and MNO or MVNO to bilaterally provide the values for TAD setting.

- Document Management

  - Document History

| Version | Date | Brief Description of Change | Approval Authority | Editor / Company |
| --- | --- | --- | --- | --- |
| 1.0 | 13th July 2016 | New PRD TS.32 | PSMC #146 | Doug Roberts / Orange |
| 2.0 | 24 March 2017 | Changes implemented as per CR1002 | TSG at TSG#27 | Doug Roberts / Orange |
| 3.0 | Sept 2017 | Changes implemented as per CR1003 | TSG at TSG#29 | Doug Roberts / Orange |
| 4.0 | Sept 2018 | Changes implemented as per CR1004 | TSG | Doug Roberts / Orange |
| 5.0 | Oct 2019 | Changes implemented as per CR1005 | TSG | Paul Gosden / GSMA |
| 6.0 | Dec 2019 | Changes implemented as per CR1006 | TSG#38 | Paul Gosden / GSMA |
| 7.0 | April 2020 | Changes implemented as per CR1007 | TSG - email | Tyler Smith/ GSMA |
| 8.0 | July 2020 | Changes implemented as per CR1008 | TSG - email | Paul Gosden / GSMA |
| 9.0 | Jan 2021 | Changes implemented as per CR1010 | TSG - email | Paul Gosden / GSMA |
| 9.1 | Jan 2021 | Changes implemented as per CR1011 | TSG - email | Paul Gosden / GSMA |
| 10.0 | Sept 2021 | Changes implemented as per CR1012 | TSG#45 ISAG#12 | Paul Gosden / GSMA |
| 11.0 | May 2022 | Changes implemented as per CR1013 | TSG via email ISAG#20 | Richard Ormson / 3UK |
| 12.0 | Feb 2023 | Changes implemented as per CR1014 | TSG#50 ISAG#27 | Wayne Cutler / GSMA |
| 13.0 | July 2023 | Changes implemented as per CR1015 | TSG#52 ISAG#32 | Laura White Telstra |
| 14.0 | Oct 2023 | Changes implemented as per CR1016 | TSG#52 ISAG#34 | Mungal Dhanda / Qualcomm |
| 15.0 | Jan 2024 | Changes implemented as per CR1017 | TSG#54 ISAG#37 | Hyewon Lee Apple |
| 16.0 | March 2024 | Changes implemented as per CR1018 to the Excel sheet aligning it with the MS Word Document and NSX and the Alignment of the MS Word document with changes made in the Excel sheet which were not approved APN 1.28 to APN 1.92 | TSG#55 ISAG#40 | Paul Gosden GSMA |
|  |  | Changes implemented as per CR1019 AT&T changes to APN Settings |  | Greg Wong AT&T |
|  |  | CR1020 Changes request by Paradise Mobile |  | Zlatko Zahirovic Paradise Mobile |
|  |  | CR1021 Changes requested by Deutsche Telekom |  | Florian Schmitt Deutsche Telekom |
| 17.0 | July 2024 | CR1022 – Removing the Settings tables so that the information is only in the Excel Sheet Adding new Settings to prevent man in the middle attacks. | TSG#56 ISAG#43 | Paul Gosden /GSMA Richard Ormson / 3UK |
| 17.1 | Nov 2024 | CR1023 v02 Minor CR clarifying that multiple network identifiers are allowed. (This was a non-substantive change requiring TSG approval only) | TSG#57 | Florian-Leon Schmitt Telekom De |
| 18.0 | Jan 2025 | CR1027 v01 (CR1025v03+CR1026v4) CR1026v04 Changes to the Excel Sheet adding OS specific Entitlement Configuration Server Setting CR1025v03 Clarifying the MNO identifier in the MS Word Document | TSG#58 ISAG#48 | Paul Gosden /GSMA |
| 18.1 | Feb 2025 | CR1029 v01 corrects an error in the Excel sheet - 5G Tab, Setting 1.04, the Permissible Setting Value was SA/NSA/A+NSA/Deactivate, the “S” had been removed in error, it should be SA/NSA/SA+NSA/Deactivate | TSG via email 24th Jan 25 This was a Non-Substantive Change requiring only TSG approval. | Paul Gosden /GSMA |
| 19.0 | May 2025 | CR1031 v01 is CR1028 v02 + CR1030 v01 CR1028v02 – In the “Other” tab in the Excel Sheet – “Other 1.30 & 1.31” added. CR1030 v01 – In the “APN” tab of the Excel Sheet, the default value was changed from IPv4 to IPV4v6 | TSG#59 ISAG#52 | Paul Gosden /GSMA |
| 20.0 | July 2025 | Excel Sheet only changes CR1034 v4 Adding 4 new fields: • MMSC URL • MMS Proxy • MMSC Port number • MMS Proxy Port number | TSG#60 ISAG#54 | Paul Gosden /GSMA |
| 21.0 | September 2025 | Addition of NB-IoT NTN and NR NTN parameters | TSG#61 ISAG#56 | Sreenivasa Rao Perisetla / Sateliot |
| 21.1 | Jan 2026 | CR1037 v02 Excel Sheet changes: APN IPv6v4 correction MS Word Doc changes: Adding contact details to Annex B2 | TSG#62 ISAG# | Esther Arellano, Telefónica |
| 22.0 | May 2026 | CR1038 v02 adding LTS NTN Support (Changes to Excel Sheet) | TSG#63 ISAG May 2026 | Sreenivasa Rao Perisetla / Sateliot |

  - Other Information

| Type | Description |
| --- | --- |
| Document Owner | Terminal Steering Group |
| Editor / Company | Paul Gosden / GSMA |

Network Settings Exchange (NSX) details:

Network Settings Exchange - Device Services URL - www.gsma.com/solutions-and-impact/industry-services/device-services/gsma-network-settings-exchange

Contact: nsx@gsma.com

See the NSX home page via the link above for confirmation of which version of TS.32 has been implemented in the NSX Database

It is our intention to provide a quality product for your use. If you find any errors or omissions, please contact us with your comments. You may notify us at prd@gsma.com with your comments or suggestions & questions.