# TS.64 FWA Devices Architecture and Requirements

# Version 5.0

# 22 October 2026

Security Classification: Non-confidential

Access to and distribution of this document is restricted to the persons permitted by the security classification. This document is subject to copyright protection. This document is to be used only for the purposes for which it has been supplied and information contained in it must not be disclosed or in any other way made available, in whole or in part, to persons other than those permitted under the security classification without the prior written approval of the Association.

Copyright Notice

Copyright © 2026 GSM Association

Disclaimer

The GSMA makes no representation, warranty or undertaking (express or implied) with respect to and does not accept any responsibility for, and hereby disclaims liability for the accuracy or completeness or timeliness of the information contained in this document. The information contained in this document may be subject to change without prior notice.

Compliance Notice

The information contain herein is in full compliance with the GSMA Antitrust Compliance Policy.

This Permanent Reference Document is classified by GSMA as an Industry Specification, as such it has been developed and is maintained by GSMA in accordance with the provisions set out GSMA AA.35 - Procedures for Industry Specifications.

Table of Contents

1	Introduction	5

1.1	Overview	5

1.2	Scope	6

1.3	Definitions	6

1.4	Abbreviations	7

1.5	References	11

1.6	Conventions	18

2	FWA Devices Architectures	18

2.1	Indoor FWA Solution	19

2.2	Outdoor FWA Solution (single tenant)	20

2.3	FWA and Network Slicing integration – architectural overview	22

3	FWA Devices Requirements (Common section)	23

3.1	Radio/RRC/NAS common requirements	23

3.2	Quality of Service	24

3.3	Voice Service	24

3.3.1	Voice General Requirements	24

3.3.2	VoIP service: registration and basic call for single-line profile	25

3.3.3	VoIP service: supplementary services for single-line profile	28

3.3.4	FXS interface requirements	32

3.3.5	FAX and POS (Point of Sale) Requirements	32

3.3.6	Voice Quality	33

3.4	Networking Features	34

3.4.1	Interfaces	34

3.4.2	Performance	34

3.4.3	Protocols	36

3.4.4	DHCP	37

3.4.5	NAT & Bridge operation	37

3.4.6	MTU	38

3.4.7	DNS	38

3.4.8	Security	39

3.4.9	Customisation	40

3.4.10	USB Port	40

3.5	Wi-Fi	41

3.5.1	Standards	41

3.5.2	MIMO capabilities, Bandwidth, Modulation and Coding schemes	42

3.5.3	Performance	42

3.5.4	Service Set Identifier (SSID)	43

3.5.5	Channel and Bandwidth Selection	43

3.5.6	Clients	44

3.5.7	Security	44

3.5.8	Wi-Fi protected Setup (WPS)	45

3.5.9	Band Steering	45

3.5.10	Mesh Networks	45

3.5.11	Wi-Fi Diagnostics	46

3.5.12	Wireless Multimedia Extension	46

3.5.13	Customisation	46

3.6	IDU/ODU Interworking and Resilience	47

3.6.1	Common requirements to bridged and routed modes of operation	47

3.6.2	ODU Bridged mode operation.	48

3.6.3	Reliability of IDU-ODU operation	49

3.6.4	ODU Routed mode operation.	49

3.6.5	Tunnels/VPNs	50

3.7	Device Management	50

3.7.1	Common Requirements for IDU and ODU	50

3.7.2	Technical Adaptation of FWA Device	55

3.8	Security	55

3.8.1	Passwords	56

3.8.2	Web UI security requirements	57

3.9	User Data Protection and Privacy	57

3.10	Hardware, Safety, EMC requirements and environment operating conditions	59

3.10.1	General	59

3.11	Stability	60

3.12	User Interface	61

4	Specific Requirements for 4G FWA Devices	61

4.1	Radio/RRC/NAS specific requirements for 4G FWA devices	61

4.2	Antenna Performance Acceptance Values for 4G FWA devices	62

5	Specific Requirements for 5G NSA FWA Devices	63

5.1	Radio/RRC/NAS specific requirements for 5G-FR1 NSA FWA devices.	63

5.2	Radio/RRC/NAS specific requirements for 5G-FR2 NSA FWA devices	65

5.3	Antenna Performance Acceptance Values for 5G NSA FWA devices	65

6	Specific Requirements for 5G SA FWA Devices	66

6.1	Radio/RRC/NAS specific requirements for 5G-FR1 SA FWA devices	67

6.2	Radio/RRC/NAS specific requirements for 5G-FR2 SA FWA devices	68

6.3	Antenna Performance Acceptance Values for 5G SA FWA devices	68

7	5G FWA Device Requirement on URSP Network Slicing Integration	69

7.1	5G FWA URSP Network Slicing Core Device Architectural Support	69

7.2	5G FWA URSP Network Slicing Device Support	70

Annex A	Hardware, Safety and EMC normative references for European Market	71

A.1	Safety	71

A.2	EMC	71

A.3	Radio Spectrum	71

A.4	Environment Protection and Energy Efficiency	72

A.5	Environment Operating Conditions	73

Annex B	Example of Wi-Fi metrics for Wi-Fi diagnostic	73

Annex C	Document Management	73

C.1	Document History	73

C.2	Other Information	74

# Introduction

## Overview

The use of Fixed Wireless Access (FWA) solutions has become quite popular in the Ultrabroadband Internet Access market as a cost-effective solution to provide a good connectivity service to customers, especially in areas in which wireline (FTTx) solutions have not been yet deployed.

A typical FWA Device consists of either an indoor (1-box) or outdoor (2-box) solution that connects via Radio Interface to the network of the Service Provider, and offers to the end-user Wi-Fi connectivity, Fast- or Gigabit-Ethernet ports, and one or two FXS (Foreign eXchange Station) ports for connecting analogue telephones to provide voice service to the end-user.

Several options are available as regards the radio technology for the communication between the FWA Device and the Network, including WiMax. However, in this context we will focus on Devices connecting to the mobile network of service operators via standard 4G/4G+/5G interfaces, as this technology is the natural choice for MNOs and is quickly becoming very popular in the market.

Provided that the 4G/4G+/5G mobile interface through which the FWA Devices connect to the network is well defined and standardized, many other functionalities of FWA Devices have not as yet been standardized; therefore both Operators and Manufacturers need to define and implement many features of the FWA Device and the way in which the services are delivered to the end-user.

So, for example, Remote Management of FWA Devices is an area not clearly defined in standards, although some operators manage such devices via BBF TR-069 ACS (Auto-Configuration Server).

Voice services are another feature where there isn’t a clearly defined standard for FWA Devices. Many Devices offer voice service through the adoption of a GSMA IR.92 VoLTE stack, however, this may not fit the needs of a typical land-line service with PSTN Emulation requirements; therefore, Operators may prefer to implement a VoIP-based stack, however, this requires customized development which requires significant effort in specification definition, implementation and testing.

Another area still not covered by standards so far is the definition of an open architecture for outdoor (2-box) FWA Devices. An outdoor solution is typically consists of an external Antenna, also called OutDoor Unit (ODU), providing connection to the mobile network, and an InDoor Unit (IDU), providing Wi-Fi, networking, Ethernet and Voice Services. The connection between the two Units is typically via a Gigabit-Ethernet connection, with Power-over-Ethernet to provide power supply to the ODU.

In some cases, the 2 boxes (IDU and ODU) are provided by the same manufacturer as a complete solution and the protocol used between the two boxes may be proprietary. Operators may prefer that a standardised interface is implemented between the IDU and ODU, as different market segments (consumer, small business, and enterprise) may require IDU’s with very different features and services.

## Scope

This document specifies a minimum set of requirements for FWA Devices. The proposed approach is to define common requirements for the various mobile technologies (4G, 5G NSA, 5G SA), and delta requirements for the requirements specific to each technology.

The requirements are grouped in functional areas (e.g. Radio, Device Management, Voice); for each area and indicate where the requirements apply to Outdoor or Indoor FWA solutions (or both).

An initial section of the document is dedicated to an overview of FWA Device architectures. This section introduces the Indoor and Outdoor architectures and identifies the areas of requirements to be defined in detail in the subsequent sections. Future enhancements to the FWA architecture are possible, such as the evolution from single-tenant solution to multi-tenant.

The main areas of requirements covered in the document are:

Radio/RRC/NAS

Use of multiple APNs for differentiating the various services

Quality of Service

Voice Service

Networking Features

Wi-Fi

IDU/ODU Interworking and Resilience

Device Management

Security

Please note that 3G FWA Devices and multi-SIM FWA Devices are outside the scope of this document.

## Definitions

| Term | Description |
| --- | --- |
| Bridged mode | In an Outdoor FWA Solution, in Bridged mode operation the OutDoor Unit (ODU) transfers frames (i.e. IP Packets) between each VLAN on the link with the Indoor Unit to the correspondent PDN/PDU Connection on the mobile network, and vice versa. Therefore, in this operation mode, the behaviour of the ODU corresponds to a Layer 2 bridge between networks with different encapsulations. |
| Routed mode | In an Outdoor FWA Solution, in Routed mode operation the OutDoor Unit (ODU) routes frames (i.e., IP Packets) between each VLAN on the link with the Indoor Unit to the PDN/PDU Connections on the mobile network, and vice versa, by means of IP routing and IP routing table. Therefore, in this operation mode, the behaviour of the ODU corresponds to an IP (Layer 3) router. This operation mode normally involves the use of Network Address Translation (NAT) to route IP packets coming from the private IP addressing space with the IDU, to the mobile network, and vice versa. |
| NR FR1 low-bands | Operating bands below 1GHz for NR. |
| NR FR1 mid-bands | Operating bands between 1GHz and 2GHz for NR. |
| NR FR1 high-bands | Operating bands above 2 GHz but less than 6GHz for NR. |
| Up speed | A procedure for changing the voice codec during the active phase of a call, done usually to change to a codec that requires more bandwidth than the one needed for the original codec. |

## Abbreviations

| Term | Description |
| --- | --- |
| 3GPP | 3rd Generation Partnership Project |
| 4G | Fourth-generation technology standard for broadband cellular networks |
| 5G | Fifth-generation technology standard for broadband cellular networks |
| 5QI | 5G QoS Identifier |
| ACS | Auto Configuration Server |
| AES | Advanced Encryption Standard |
| AKA | Authentication and Key Agreement |
| ALG | Application Layer Gateway |
| ANR | Automatic Neighbour Relation |
| AP | Access Point |
| APN | Access Point Name |
| ARP | Address Resolution Protocol |
| BBF | Broadband Forum |
| BRI | Basic Rate Interface |
| CA | Carrier Aggregation |
| CB | Communication Barring |
| CCBS | Completion of Communications to Busy Subscriber |
| CDIV | Communication Diversion |
| CGI | Cell Global Identity |
| CPE | Customer Premise Equipment |
| CSCF | Call Session Control Function |
| CSRF | Cross-Site Request Forgery |
| CW | Communication Waiting |
| CWMP | CPE WAN Management Protocol |
| DC | Direct Current |
| DEI | Drop Eligible Indicator |
| DLNA | Digital Living Network Alliance |
| DNS | Domain Name System |
| DSCP | Differentiated Service Code Point |
| DSS | Dynamic Spectrum Sharing |
| DTMF | Dual-Tone Multi-Frequency signalling |
| ECT | Explicit Communication Transfer |
| EEA | EPS Encryption Algorithm |
| EIA | EPS Integrity Algorithm |
| EIRP | Equivalent Isotropic Radiated Power |
| EMC | Electromagnetic Compatibility |
| EN-DC | E-UTRAN/New Radio Dual Connectivity |
| EPS | Evolved Packet System |
| ETSI | European Telecommunications Standards Institute |
| E-UTRAN | Evolved Universal Mobile Telecommunications System (UMTS) Terrestrial Radio Access |
| FDD | Frequency Division Duplex |
| FQDN | Fully Qualified Domain Name |
| FR | Frequency Range |
| FS | Free Space |
| FTP | File Transfer Protocol |
| FWA | Fixed Wireless Access |
| FXS | Foreign eXchange Station |
| GSM / E-GSM | Global System for Mobile Communication/ Extended-GSM |
| GSMA | GSM Association |
| HLOS | High Level Operating System |
| HTTP | Hypertext Transfer Protocol |
| ICMP | Internet Control Message Protocol |
| IDU | InDoor Unit |
| IEEE | Institute of Electrical and Electronics Engineers |
| IETF | Internet Engineering Task Force |
| IMS | IP Multimedia Subsystem |
| IP | Internet Protocol |
| ISDN | Integrated Services Digital Network |
| ITU-T | International Telecommunication Union – Telecommunication Standardization Bureau |
| LAN | Local Area Network |
| LTE | Long Term Evolution |
| M2M | Machine-to-Machine |
| MAC | Media Access Control |
| MCID | Malicious Communication Identification |
| MIMO | Multiple Input Multiple Output |
| MSB | Message Server Block |
| MSS | Maximum Segment Size |
| MTBF | Mean Time Between Failure |
| MTU | Maximum Transmission Unit |
| MWI | Message Waiting Indication |
| NAS | Non-Access Stratum |
| NAT | Network Address Translation |
| NEA | New radio Encryption Algorithm |
| NIA | New radio Integrity Algorithm |
| NR | New Radio |
| NSA | Non Stand-Alone |
| NSSAI | Network Slice Selection Assistance Information |
| NTP | Network Time Protocol |
| ODU | OutDoor Unit |
| OIP | Originating Identification Presentation |
| OIR | Originating Identification Restriction |
| OS | Operating System |
| OTA | Over The Air (without cable) |
| OTT | Over The Top |
| PC | Power Class |
| PCI | Physical layer Cell Identifier |
| P-CSCF | Proxy-CSCF |
| PCP | Priority Code Point |
| PDN | Packet Data Network |
| PDU | Protocol Data Unit |
| PDSCH | Physical Downlink Shared Channel |
| POE | Power Over Ethernet |
| PSK | Pre-Shared Key |
| PSU | Power Supply Unit |
| QCI | QoS Class Identifier |
| QoE | Quality of Experience |
| QoS | Quality of Service |
| RFC | Request For Comments |
| RPC | Remote Procedure Call |
| RRC | Radio Resource Control |
| RSD | Route Selection Descriptor |
| RSRP | Reference Signal Received Power |
| RSRQ | Reference Signal Received Quality |
| RSSI | Received Signal Strength Indicator |
| RTCP | RTP Control Protocol |
| RTP | Real-time Transport Protocol |
| S-NSSAI | Single Network Slice Selection Assistance Information |
| SA | Stand-Alone |
| SDP | Session Description Protocol |
| SIM/USIM | Subscriber Identity Module / Universal Subscriber Identity Module (in this document, includes embedded SIM) |
| SINR | Signal to Interference plus Noise Ratio |
| SIP | Session Initiation Protocol |
| SUCI | Subscription Concealed Identifier |
| SUPI | Subscription Permanent Identifier |
| SSID | Service Set Identifier |
| TCP | Transmission Control Protocol |
| TD | Traffic Descriptor |
| TDD | Time Division Duplex |
| TIP | Terminating Identification Presentation |
| TIR | Terminating Identification Restriction |
| TLS | Transport Layer Security |
| TRP | Total Radiated Power |
| TRS | Total Radiated Sensitivity |
| TSG | Terminal Steering Group |
| TWAMP | Two-Way Active Measurement Protocol |
| UDP | User Datagram Protocol |
| UE | User Equipment |
| UI | User Interface |
| URI | Uniform Resource Identifier |
| URL | Uniform Resource Locator |
| URSP | UE Route Selection Policy |
| VID | VLAN Identifier |
| VLAN | Virtual LAN |
| VOD | Video On Demand |
| VoLTE | Voice over LTE |
| VoIP | Voice over Internet Protocol |
| VoNR | Voice over NR |
| WAN | Wide Area Network |
| WFA | Wi-Fi Alliance |
| Wi-Fi | A wireless local area networking technology that uses radio waves to provide wireless high-speed Internet access, defined by IEEE 802.11 standards. |
| WMM | Wi-Fi Multimedia |
| WPA | Wi-Fi Protected Access |
| WPS | Wi-Fi Protected Setup |
| WWAN | Wireless Wide Area Network |

## References

| Ref | Doc Number | Title |
| --- | --- | --- |
|  | PRD AA.35 | Policy and Procedures for Official Documents |
|  | RFC 2119 | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997. Available at http://www.ietf.org/rfc/rfc2119.txt |
|  | RFC 8174 | Ambiguity of Uppercase vs Lowercase in RFC 2119 Key Words https://www.rfc-editor.org/info/rfc8174 |
|  | 3GPP TS 23.207 V16.0.0 or later | 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; End-to-end Quality of Service (QoS) concept and architecture (Release 16) |
|  | 3GPP TS 23.203 V16.0.0 or later | 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Policy and charging control architecture (Release 16) |
|  | GSMA IR.92 Version 15.0 or later | IMS Profile for Voice and SMS - Version 15.0 |
|  | 3GPP TS 24.229 V15.2.0 or later | 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3 (Release 15) |
|  | 3GPP TS 23.003 V16.3.0 or later | Digital cellular telecommunications system (Phase 2+) (GSM); Universal Mobile Telecommunications System (UMTS); LTE; 5G; Numbering, addressing and identification. (3GPP TS 23.003 version 16.3.0 Release 16) |
|  | 3GPP TS 23.228 V16.5.0 or later | Digital cellular telecommunications system (Phase 2+) (GSM); Universal Mobile Telecommunications System (UMTS); LTE; IP Multimedia Subsystem (IMS); Stage 2 (3GPP TS 23.228 version 16.5.0 Release 16) |
|  | RFC 3263 | Session Initiation Protocol (SIP): Locating SIP Servers |
|  | RFC 5626 | Managing Client-Initiated Connections in the Session Initiation Protocol (SIP) |
|  | RFC 3261 | SIP: Session Initiation Protocol |
|  | RFC 5341 | The Internet Assigned Number Authority (IANA) tel Uniform Resource Identifier (URI) Parameter Registry |
|  | RFC 3262 | Reliability of Provisional Responses in the Session Initiation Protocol (SIP) |
|  | RFC 3264 | An Offer/Answer Model with the Session Description Protocol (SDP) |
|  | RFC 3311 | The Session Initiation Protocol (SIP) UPDATE Method |
|  | RFC 4733 | RTP Payload for DTMF Digits, Telephony Tones, and Telephony Signals |
|  | ITU-T G.711 (11/88) | Pulse code modulation (PCM) of voice frequencies |
|  | ITU-T G.729 (06/12) or later | Coding of speech at 8 kbit/s using conjugate-structure algebraic-code-excited linear prediction (CS-ACELP) |
|  | ITU-T G.729 Annex A (11/96) or later | Reduced complexity 8 kbit/s CS-ACELP speech codec |
|  | ITU-T G.722 (09/12) or later | 7 kHz audio-coding within 64 kbit/s |
|  | ITU-T G.131 (11/03) or later | Talker echo and its control |
|  | RFC 3550 | RTP: A Transport Protocol for Real-Time Applications |
|  | RFC 3551 | RTP Profile for Audio and Video Conferences with Minimal Control |
|  | RFC 3605 | Real Time Control Protocol (RTCP) attribute in Session Description Protocol (SDP) |
|  | RFC 2198 | RTP Payload for Redundant Audio Data. |
|  | RFC 4961 | Symmetric RTP / RTP Control Protocol (RTCP) |
|  | RFC 5009 | Private Header (P-Header) Extension to the Session Initiation Protocol (SIP) for Authorization of Early Media |
|  | RFC 3960 | Early Media and Ringing Tone Generation in the Session Initiation Protocol (SIP) |
|  | RFC 3325 | Private Extensions to the Session Initiation Protocol (SIP) for Asserted Identity within Trusted Networks |
|  | 3GPP TS 24.607 version 15.0.0 or later | Digital cellular telecommunications system (Phase 2+) (GSM); Universal Mobile Telecommunications System (UMTS); LTE; Originating Identification Presentation (OIP) and Originating Identification Restriction (OIR) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification (3GPP TS 24.607 version 15.0.0 Release 15) |
|  | ETSI EN 300 659-3 V1.3.1 | Access and Terminals (AT); Analogue access to the Public Switched Telephone Network (PSTN); Subscriber line protocol over the local loop for display (and related) services; Part 3: Data link message and parameter codings |
|  | 3GPP TS 24.604 version 15.1.0 or later | Digital cellular telecommunications system (Phase 2+) (GSM); Universal Mobile Telecommunications System (UMTS); LTE; Communication Diversion (CDIV) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification (3GPP TS 24.604 version 15.1.0 Release 15) |
|  | RFC 3842 | A Message Summary and Message Waiting Indication Event Package for the Session Initiation Protocol (SIP) |
|  | 3GPP TS 24.611 version 11.3.0 or later | Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; Anonymous Communication Rejection (ACR) and Communication Barring (CB) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification (3GPP TS 24.611 version 11.3.0 Release 11) |
|  | 3GPP TS 24.610 version 15.1.0 or later | Digital cellular telecommunications system (Phase 2+) (GSM); Universal Mobile Telecommunications System (UMTS); LTE; Communication HOLD (HOLD) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification (3GPP TS 24.610 version 15.1.0 Release 15) |
|  | 3GPP TS 24.615 version 15.0.0 or later | Digital cellular telecommunications system (Phase 2+) (GSM); Universal Mobile Telecommunications System (UMTS); LTE; Communication Waiting (CW) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol Specification (3GPP TS 24.615 version 15.0.0 Release 15) |
|  | 3GPP TS 24.147 version 10.1.0 or later | Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS);LTE; Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3 (3GPP TS 24.147 version 10.1.0 Release 10) |
|  | 3GPP TS 24.642 version 10.2.0 or later | Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS);LTE; Completion of Communications to Busy Subscriber (CCBS) and Completion of Communications by No Reply (CCNR) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol Specification (3GPP TS 24.642 version 10.2.0 Release 10) |
|  | 3GPP TS 24.629 version 15.0.0 or later | Digital cellular telecommunications system (Phase 2+) (GSM); Universal Mobile Telecommunications System (UMTS); LTE; Explicit Communication Transfer (ECT) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification (3GPP TS 24.629 version 15.0.0 Release 15) |
|  | 3GPP TS 24.608 version 10.0.0 or later | Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS);LTE; Terminating Identification Presentation (TIP) and Terminating Identification Restriction (TIR) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification (3GPP TS 24.608 version 10.0.0 Release 10) |
|  | 3GPP TS 24.616 version 10.0.0 or later | Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; Malicious Communication Identification (MCID) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification (3GPP TS 24.616 version 10.0.0 Release 10) |
|  | ETSI ES 201 970 V1.1.1 | Access and Terminals (AT); Public Switched Telephone Network (PSTN); Harmonized specification of physical and electrical characteristics at a 2-wire analogue presented Network Termination Point (NTP) |
|  | ITU-T Q.522 (11/88) | Digital exchange connections, signalling and ancillary functions |
|  | ITU-T T.38 (09/10) | Procedures for real-time Group 3 facsimile communication over IP networks |
|  | ITU-T T.38 (11/15) | Procedures for real-time Group 3 facsimile communication over IP networks |
|  | BBF TR-069 | CPE WAN Management Protocol Issue: 1 Amendment 6 Approval Date: March 2018 CWMP Version: 1.4 |
|  | BBF TR-135 | Data Model for a TR-069 Enabled STB Issue: 1 Amendment 3 Issue Date: November 2012 |
|  | BBF TR-104 | TR-069 Voice Service:2.0 Service Object definition |
|  | BBF TR-140 | TR-069 Data Model for Storage Service Enabled Devices Issue: 1 Amendment 3 Issue Date: May 2017 |
|  | BBF TR-196 | FAP Service:2.0 Femto Access Point Service Data Model |
|  | BBF TR-098 | TR-069 InternetGatewayDevice:1.14 Root Object definition |
|  | BBF TR-181 | TR-069 Device:2.11 Root Object definition |
|  | RFC 5246 | The Transport Layer Security (TLS) Protocol Version 1.2 |
|  | BBF TR-143 | Enabling Network Throughput Performance Tests and Statistical Monitoring Issue: 1 Amendment 1 Corrigendum 2 Issue Date: February 2023 |
|  | BBF TR-390 | Performance Measurement from IP Edge to Customer Equipment using TWAMP Light Issue: 1 Issue Date: May 2017 |
|  | GSMA TS.24 Version 6.0 or later | Operator Acceptance Values for Device Antenna Performance |
|  | 3GPP TS 38.101-1 Version 17.5.0 or later | 5G;NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (3GPP TS 38.101-1 version 17.5.0 Release 17) |
|  | IEEE 802.3ab-1999 or later | IEEE Standard for Information Technology - Telecommunications and information exchange between systems - Local and Metropolitan Area Networks - Part 3: Carrier Sense Multiple Access with Collision Detection (CSMA/CD) Access Method and Physical Layer Specifications - Physical Layer Parameters and Specifications for 1000 Mb/s Operation over 4 pair of Category 5 Balanced Copper Cabling, Type 1000BASE-T |
|  | RFC 791 | INTERNET PROTOCOL DARPA INTERNET PROGRAM PROTOCOL SPECIFICATION |
|  | RFC 8200 | Internet Protocol, Version 6 (IPv6) Specification |
|  | RFC 826 | An Ethernet Address Resolution Protocol -- or -- Converting Network Protocol Addresses to 48.bit Ethernet Address for Transmission on Ethernet Hardware |
|  | RFC 5227 | IPv4 Address Conflict Detection |
|  | RFC 5494 | IANA Allocation Guidelines for the Address Resolution Protocol (ARP) |
|  | RFC 4861 | Neighbor Discovery for IP version 6 (IPv6) |
|  | RFC 792 | INTERNET CONTROL MESSAGE PROTOCOL DARPA INTERNET PROGRAM PROTOCOL SPECIFICATION |
|  | RFC 950 | Internet Standard Subnetting Procedure |
|  | RFC 4884 | Extended ICMP to Support Multi-Part Messages |
|  | RFC 6633 | Deprecation of ICMP Source Quench Messages |
|  | RFC 6918 | Formally Deprecating Some ICMPv4 Message Types |
|  | RFC 4443 | Internet Control Message Protocol (ICMPv6) for the Internet Protocol Version 6 (IPv6) Specification |
|  | RFC 5905 | Network Time Protocol Version 4: Protocol and Algorithms Specification |
|  | RFC 3376 | Internet Group Management Protocol, Version 3 |
|  | RFC 4605 | Internet Group Management Protocol (IGMP) / Multicast Listener Discovery (MLD)-Based Multicast Forwarding ("IGMP/MLD Proxying") |
|  | IEEE 802.1Q-2018 and later | IEEE Standard for Local and Metropolitan Area Networks–Bridges and Bridged Networks |
|  | RFC 2474 | Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers |
|  | RFC 2475 | An Architecture for Differentiated Services |
|  | RFC 2597 | Assured Forwarding PHB Group |
|  | RFC 3260 | New Terminology and Clarifications for Diffserv |
|  | RFC 2663 | IP Network Address Translator (NAT) Terminology and Considerations |
|  | 3GPP TS 23.501 Version 16.5.0 or later | System architecture for the 5G System (5GS) (3GPP TS 23.501 version 16.5.0 Release 16) |
|  | 3GPP TS 24.501 Version 16.5.1 or later | Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3 3GPP TS 24.501 V16.5.1 |
|  | IEEE 802.11n-2009 or later | IEEE Standard for Information technology-- Local and metropolitan area networks-- Specific requirements-- Part 11: Wireless LAN Medium Access Control (MAC)and Physical Layer (PHY) Specifications Amendment 5: Enhancements for Higher Throughput |
|  | IEEE 802.11ac-2013 or later | IEEE Standard for Information technology--Telecommunications and information exchange between systems–Local and metropolitan area networks--Specific requirements--Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications--Amendment 4: Enhancements for Very High Throughput for Operation in Bands below 6 GHz. |
|  | IEEE 802.11ax-2021 or later | IEEE Standard for Information Technology--Telecommunications and Information Exchange between Systems Local and Metropolitan Area Networks--Specific Requirements Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications Amendment 1: Enhancements for High-Efficiency WLAN |
|  | IEEE 802.11k-2008 or later | IEEE Standard for Information technology-- Local and metropolitan area networks-- Specific requirements-- Part 11: Wireless LAN Medium Access Control (MAC)and Physical Layer (PHY) Specifications Amendment 1: Radio Resource Measurement of Wireless LANs |
|  | IEEE 802.11v-2011 or later | IEEE Standard for Information technology-- Local and metropolitan area networks-- Specific requirements-- Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications Amendment 8: IEEE 802.11 Wireless Network Management |
|  | IEEE 802.11r-2008 or later | IEEE Standard for Information technology-- Local and metropolitan area networks-- Specific requirements-- Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications Amendment 2: Fast Basic Service Set (BSS) Transition |
|  | IEC 60529:1989/AMD2:2013/COR1:2019 or later | Degrees of protection provided by enclosures (IP Code) |
|  | ITU-T K.21 (08/22) | Resistibility of telecommunication equipment installed in customer premises to overvoltages and overcurrents |
|  | ITU-T K.44 (10/19) – Cor.1 (12/20) | Resistibility tests for telecommunication equipment exposed to overvoltages and overcurrents – Basic Recommendation |
|  | ITU-T K.45 (11/22) | Resistibility of telecommunication equipment installed in the access and trunk networks to overvoltages and overcurrents |
|  | 3GPP TS 36.101 Version 17.10.0 or later | Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception |
|  | 3GPP TS 38.101-2 Version 17.5.0 or later | NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone |
|  | 3GPP TS 38.214 Version 17.6.0. or later | NR; Physical layer procedures for data |
|  | GSMA TS.32 Version 13.0 or later | Technical Adaptation of Devices through Late Customisation |
|  | GSMA TS.49 Version 1.1 or later | WLAN Antenna Performance Testing |
|  | RFC 2131 | Dynamic Host Configuration Protocol |
|  | RFC 2132 | DHCP Options and BOOTP Vendor Extensions |
|  | RFC 8415 | Dynamic Host Configuration Protocol for IPv6 (DHCPv6) |
|  | RFC 3022 | Traditional IP Network Address Translator (Traditional NAT) |
|  | RFC 4787 | Network Address Translation (NAT) Behavioural Requirements for Unicast UDP |
|  | RFC 6145 | IP/ICMP Translation Algorithm |
|  | 3GPP TS 24.008 Version 18.3.0 or later | Mobile radio interface Layer 3 specification; Core network protocols; Stage 3 |
|  | RFC 1034 | DOMAIN NAMES - CONCEPTS AND FACILITIES |
|  | RFC 1035 | DOMAIN NAMES - IMPLEMENTATION AND SPECIFICATION |
|  | RFC 8484 | DNS Queries over HTTPS (DoH) |
|  | USB 3.1 | USB Implementers Forum, Universal Serial Bus 3.1 Specification |
|  | IEEE 802.11i-2004 | IEEE Standard for information technology-Telecommunications and information exchange between systems-Local and metropolitan area networks-Specific requirements-Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: Amendment 6: Medium Access Control (MAC) Security Enhancements |
|  | WPA3™ - 2022 | Wi-Fi Alliance WPA3™ Specification Version 3.1 |
|  | IEEE 802.1X-2020 | IEEE Standard for Local and Metropolitan Area Networks--Port-Based Network Access Control |
|  | IEEE 802.11be-2024 or later | IEEE Standard for Information technology--Telecommunications and information exchange between systems Local and metropolitan area networks--Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications Amendment 2: Enhancements for Extremely High Throughput (EHT) |

## Conventions

The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” in this document are to be interpreted as described in RFC 2119 [2] and clarified by RFC 8174 [3], when, and only when, they appear in all capitals, as shown here.

# FWA Devices Architectures

A FWA Device offers the typical features of a Home Router (also known as Residential Gateway) and connects to a 3GPP-based network via a Radio Interface.

The two architectural models considered in this document are: indoor FWA solution and outdoor FWA solution.

In the indoor FWA solution, a single box provides all the functions and interfaces needed to deliver the Ultrabroadband Internet services to the end user.

In the outdoor FWA solution, the functions are split between an Outdoor Unit (ODU), which connects to the mobile network via the radio interface, and an Indoor Unit (IDU), which provides all the functions and interfaces for the LAN network: Wi-Fi access point, Voice interface, networking functions (e.g. port mapping, Firewall), etc.

Whilst the indoor solution is clearly a single-tenant solution, different architectural alternatives are possible for outdoor FWA solutions.

Outdoor solutions can be single-tenant or multi-tenant: in a single-tenant solution, an Outdoor Unit is dedicated to a single customer and is connected via a point-to-point link with an Indoor Unit. In a multi-tenant solution, an Outdoor Unit serves multiple customers, and several Indoor Units are connected to it.

This document will be restricted to single-tenant Outdoor solutions.

Another architectural option for outdoor solutions is the specification of the interface between ODUs and IDUs. In some cases, the ODU can only be connected to an IDU from the same manufacturer, and the interface between IDU and ODU may be proprietary. This case is not covered in this document.

Instead, this document defines a standard open interface between ODU and IDU; therefore, ODUs and IDUs from different manufacturers can be matched and combined.

## Indoor FWA Solution

The Indoor FWA solution reference architecture is depicted in Figure 1.

: Indoor FWA Device Reference Architecture

The indoor FWA Device provides the following functionality:

- Internet Service (mandatory): ultra-broadband connectivity to the Internet. Ancillary functions to this connectivity is the option to configure VPN, Port Mapping, Firewall rules, NAT helpers (ALG, Application Layer Gateway), and to customize DNS servers.

- Voice Service (mandatory): the service is provided by the operator by means of VoIP, VoLTE or VoNR technologies. The Indoor FWA Device offers one or more Voice Interfaces to the end-user (typically, an FXS port) and interacts with the IMS Core of the Operator. These technologies are addressed in this document as they represent valid industry standards for Voice services. The choice between the standards will depend on Operator network support, specific voice features required by the market and/or or regulatory obligations.

- Managed Video services (optional): Video on Demand (VOD) or Video Streaming service, managed by the Operator (potentially in partnership with one or more OTT Service Providers), which controls aspects of the service, in order to maximize the Quality of Experience (QoE) for the end user and the efficiency in network resources utilization.

An Indoor FWA Device may provide functionality, e.g. Smart Home control, however, this is outside the scope of this document, which focuses on the above functionality.

The indoor FWA Device normally offers the following interfaces:

- LAN:

  - Ethernet: an FWA Device offers Ethernet LAN interfaces, at least one of which should be Gigabit Ethernet

  - Wi-Fi: an FWA Device offers a Wi-Fi access point. Minimum performance requirements for the Wi-Fi access point are detailed in the specific section 3.5.

- Voice Interfaces: an FWA Device must offer at least one analogue FXS (Foreign eXchange Station) port, to be used in association with a single-line (that is, single-number, single-channel) profile. The availability of two or more FXS interfaces, or other interfaces such as ISDN BRI (Basic Rate Interface) are normally associated with the use of more complex multi-line (multiple-number, multiple-channel) profiles.

- WAN: an FWA Device connects to the network via a radio/mobile interface (4G, 5G NSA, 5G SA). Different PDN connections are used to differentiate quality of service. The requirements are detailed in the specific section of this document.

An Indoor FWA Device is managed through a centralized Device Management platform. An example is a TR-069 AutoConfiguration Server (ACS), operated by the Operator. The remote management provides functionality such as:

- Provisioning: used to configure VoIP account and other VoIP-related parameters (not required for VoLTE-based voice service), APN configurations, Wi-Fi customization and other provisioning activities.

- Assurance: used to perform assurance activities such as re-provisioning, reboot, factory reset, firmware upgrade.

- Monitoring: used to monitor Device operation and performance, for example Device status, VoIP registration status, Wi-Fi statistics and performance, Internet access performance measurements, radio parameters.

The Indoor FWA Device hosts one SIM which allows line identification and authorization to access the network.

## Outdoor FWA Solution (single tenant)

The Outdoor FWA solution reference architecture is depicted in Figure 2.

: Outdoor FWA Device Reference Architecture

As mentioned at the beginning of the section, this document only addresses on an open, standard architecture between the OutDoor Unit (ODU) and InDoor Unit (IDU), so that ODU and IDU from different manufacturers can be used together to deliver the Outdoor FWA Solution.

In this document, the single-tenant solution is considered: therefore, each ODU is coupled via a 1:1 relationship with an IDU and the resultant FWA Solution is exclusively dedicated to a single user.

The OutDoor Unit:

- Hosts either a physical SIM or eSIM which allows line identification and authorization for access to the network;

- Provides connectivity to the network, via a radio interface (4G, 5G NSA, 5G SA). different PDN connections are used to differentiate quality of service. The requirements are detailed in the specific section XXX of this document;

- Connects to the InDoor Unit, via a Gigabit Ethernet Interface, differentiating services by means of VLANs dedicated to Voice, Video and Data services, where each VLAN via a 1 :1 relationship with a PDN connection;

- Is managed through a centralized Device Management platform. An example is a TR-69 AutoConfiguration Server (ACS), operated by the Operator. The remote management serves different purposes, including:

  - Provisioning: used for APN configurations, VLAN configurations and other provisioning activities.

  - Assurance: used to perform assurance activities such as re-provisioning, reboot, factory reset, firmware upgrade, …

  - Monitoring: used to monitor Device operation and performance, for example Device status, Internet access performance measurements, radio parameters.

- Is normally powered by Power over Ethernet from a POE PSU to be installed indoor, which connects via Ethernet to the IDU;

- Is suitable for outdoor installation. That is, the ODU and its accessories have Hardware, EMC and Security features suitable for outdoor installation and are compliant to the current regulations of the country where they are installed.

The InDoor Unit:

- Connects to the OutDoor Unit, via a Gigabit Ethernet interface (at least Gigabit Ethernet), differentiating services by means of VLANs dedicated to Voice, Video and Data services, where each VLAN is mapped by the ODU via a 1:1 relationship with a PDN connection;

- Offers the services normally offered from a Home Router/ Residential Gateway, that is the same services foreseen for the Indoor FWA Device: Internet access (mandatory), Voice (mandatory), managed Video service (optional);

- Offers the same LAN interfaces delivered by the Indoor FWA Device;

- Is managed through a centralized Device Management platform. An example is a TR-69 AutoConfiguration Server (ACS), operated by the Operator. The remote management serves different purposes, including:

  - Provisioning: used to configure VoIP account and other VoIP-related parameters, VLAN configurations, Wi-Fi customization and other provisioning activities.

  - Assurance: used to perform assurance activities such as re-provisioning, reboot, factory reset, firmware upgrade, …

  - Monitoring: used to monitor Device operation and performance, for example Device status, VoIP registration status, Wi-Fi statistics and performance, Internet access performance measurements.

In summary, the InDoor Unit of the Outdoor FWA Solution can be any Home Router/Residential Gateway compliant to the requirements detailed in the following sections, and in particular to the requirements for IDU/ODU interconnection. It is also very similar to an Indoor FWA Device, with the difference that the IDU of an Outdoor FWA Solution does not need a SIM and does not connect directly to the mobile network.

## FWA and Network Slicing integration – architectural overview

5G slicing over URSP (User Equipment Route Selection Policy) was initially introduced to address the need for more efficient and flexible network management. The primary goal was to enable service providers to offer differentiated services to various customer segments by creating multiple virtual networks on a single physical infrastructure. This approach allows for better resource allocation, improved quality of service, and enhanced user experience.

In the context of 5G FWA, URSP can be used to manage how data traffic is routed through the network. This is particularly important for ensuring that different application services (e.g., emailing, browsing, gaming, streaming, voice/video conferencing) are allocated the appropriate Quality of Service and network resources. By using URSP, service providers can create policies that dictate how traffic from FWA devices is handled, ensuring optimal performance and user experience.

The potential deployment can be via 5G SA enabled Indoor FWA or can be via a 5G SA enabled Outdoor FWA to connect to User Equipment.

- Figure 3: URSP on 5G Outdoor FWA Reference Architecture

To realise Network Slicing using URSP via 5G FWA, it is critical that the 5G SA connectivity, regardless of whether it is via an Indoor FWA or an Outdoor FWA, has the appropriate capability enabled. Further, it is important to recognize that, given 5G FWA devices are not natively application aware, not all URSP implementation options are applicable to the 5G FWA equipment type, particularly the Traffic Descriptor (TD) component in the URSP policy (Table 5.2.1, Section 5, 3GPP TS 24.526).  The TD options that are more closely aligned to 5G FWA Equipment type utilization are captured under section 7, 5G FWA Device Requirement on URSP Network Slicing Integration.

# FWA Devices Requirements (Common section)

## Radio/RRC/NAS common requirements

Typically, FWA devices share the radio access resources with other 4G and 5G device categories (e.g., smartphones, M2M modules) so it is essential for mobile operators to increase the spectral efficiency as much as possible, in order to optimize the usage of the valuable FDD and TDD frequency assets.

All this is possible thanks to some transmission techniques that are able to improve device performance and leading to an overall higher spectral efficiency:

- Carrier Aggregation: the ability of the device to receive and/or transmit on multiple bands at the same time.

- MIMO to use multiple antenna elements at the transmitter and the receiver to improve bit rates and channel quality estimation.

- Higher-Order Modulations provide higher data rates within a given bandwidth; the drawback is the reduced robustness to noise and interference.

- Higher Maximum Output Power to compensate propagation losses.

- Uplink Power Class

| TS.64_3.1_REQ_001 | The FWA device SHALL support one (1) SIM/USIM. FWA Devices with multiple SIMs are outside the scope of this document. |
| --- | --- |
| TS.64_3.1_REQ_002 | The FWA device MAY be equipped with one (1) eSIM, instead of a physical SIM. |
| TS.64_3.1_REQ_003 | The Indoor FWA Device (1-box solution) SHALL support the establishment of at least 3 PDNs/PDUs (e.g. for data/remote management, video, and voice services) |
| TS.64_3.1_REQ_004 | The OutDoor Unit of an Outdoor FWA Solution (2-box) SHALL support the establishment of at least 4 PDNs/PDUs (e.g. for remote management of ODU, data/remote management of IDU, video, and voice services) |
| TS.64_3.1_REQ_005 | The FWA Device SHALL allow configurable associations between PDN/PDU connections and services/applications (e.g. voice, video; VLAN settings via Web UI) |
| TS.64_3.1_REQ_006 | The FWA device SHOULD support the establishment of at least 6 PDNs/PDUs |
| TS.64_3.1_REQ_007 | The FWA Device SHALL support the establishment of PDNs/PDUs with IPv4 single stack. |
| TS.64_3.1_REQ_007a | The FWA Device SHALL support the establishment of PDNs/PDUs with IPv6 single stack. |
| TS.64_3.1_REQ_007b | The FWA Device SHALL support the establishment of PDNs/PDUs with IPv4v6 dual stack. |
| TS.64_3.1_REQ_008 | For each PDNs/PDUs, the FWA Device SHALL allow to configure: - Protocol stack (IPv4 or IPv6 or IPv4v6); - Authentication option (PAP/CHAP); - MTU/MSS. |
| TS.64_3.1_REQ_009 | For each PDNs/PDUs, the FWA Device SHALL allow to configure Authentication Username and Password. NOTE: Data network serving the PDN/PDU determines whether Authentication Username and/or Password is required. Typically, this is required if PAP or CHAP authentication is used. |

Further detailed requirements for FWA Device in this area depend on the specific technology considered, therefore they are defined in the chapters dedicated to the various technologies (§ 4.1 for 4G, § 5.1 for 5G-FR1 NSA, § 5.2 for 5G-FR2 NSA, § 6.1 for 5G-FR1 SA, § 6.2 for 5G-FR2 SA).

## Quality of Service

| TS.64_3.2_REQ_001 | For Quality of Service, the FWA Device SHALL comply with the 3GPP standards 3GPP TS 23.207 [4] and 3GPP TS 23.203 [5] regardless of deployment scenario (e.g., 4G, 5G NSA, 5G SA). |
| --- | --- |
| TS.64_3.2_REQ_002 | A wireless service provider utilizes customized QoS for specific category of subscribers including mission critical organizations, government entities and enterprise customers. For the latter case, the FWA Device SHOULD comply with the wireless service provider’s requirements and mandates. |

## Voice Service

- Voice Service requirements apply to the Indoor FWA Device and to the InDoor Unit of an Outdoor FWA Solution.

### Voice General Requirements

| TS.64_3.3.1_REQ_001 | The Indoor FWA Device (1-box solution) SHALL support voice service either by means of VoLTE technology or VoIP technology. |
| --- | --- |
| TS.64_3.3.1_REQ_002 | The InDoor Unit of an Outdoor FWA Solution (2-box) SHALL support voice service uniquely by means of VoIP technology. |
| TS.64_3.3.1_REQ_003 | In case of VoLTE Technology, the FWA Device SHALL be compliant to GSMA IR.92 [6] profile. |
| TS.64_3.3.1_REQ_004 | In case of VoIP technology, the FWA Device SHALL be compliant to 3GPP specification 24.229 [7], with the profile defined in the following sections 3.3.2, 3.3.3, 3.3.6 and, optionally, 3.3.5. |
| TS.64_3.3.1_REQ_005 | In case of VoIP technology, the FWA Device SHALL request a dedicated PDN Connection. NOTE 1: The PDN Connection for VoIP traffic may be characterized with a dedicated QCI. NOTE 2: Voice Traffic includes SIP, RTP, RTCP and DNS traffic used to resolve the P-CSCF FQDN in order to get the P-CSCF addresses. |
| TS.64_3.3,1_REQ_006 | It SHALL be possible to disable all voice features. |
| TS.64_3.3,1_REQ_007 | The FWA Device SHALL be customizable in order not to have any FXS port or other voice interfaces. |

### VoIP service: registration and basic call for single-line profile

| TS.64_3.3.2_REQ_001 | In case of VoIP technology, the FWA Device SHALL be compliant to 3GPP specification TS 24.229 [7]. |
| --- | --- |
| TS.64_3.3.2_REQ_002 | The FWA Device SHALL use the public user identity defined in 3GPP 23.003 [8]. In particular, the UE SHALL use a SIP URL format, with the username part in E.164 telephone number, in international format: e.g. sip:+390612345678@operatordomain.com. |
| TS.64_3.3.2_REQ_003 | The FWA Device SHALL support the private user identity, as specified in 3GPP TS 23.228 [9] par. 4.3.3.1. This parameter is used as username in SIP Digest authentication procedure; it is different from public user identity. |
| TS.64_3.3.2_REQ_004 | The FWA Device SHALL support RFC3263: Location of SIP Servers [10], for the resolution of FQDN of the outbound proxy. The outcome of the process of resolution of the P-CSCF FQDN is a set of P-CSCF IP addresses, ordered by priority. |
| TS.64_3.3.2_REQ_005 | In order to support the P-CSCF FQDN resolution process, the FWA Device SHALL be configured with DNS Servers addresses. Such addresses can be obtained during the Voice PDN connection establishment process. |
| TS.64_3.3.2_REQ_006 | SIP Digest Authentication SHALL be supported. The FWA Device SHALL use digest authentication for the following methods: REGISTER (first registration and registration refresh) and INVITE. |
| TS.64_3.3.2_REQ_007 | The FWA Device SHALL support a password of at least 64 characters for Digest Authentication. |
| TS.64_3.3.2_REQ_008 | The FWA Device SHALL offer a maximum expiry time for Registration of 600000 seconds. |
| TS.64_3.3.2_REQ_009 | In order to improve user experience, the FWA Device SHOULD be compliant to RFC5626 (Managing Client-Initiated Connections in SIP) [11], in particular, as regards the use of reg-id and +sip.instance tags. The latter tag is important in order to handle properly multiple registrations of a User Agent that may occur in case of IP address change of the Device. |
| TS.64_3.3.2_REQ_010 | The FWA Device SHALL support the Retry-After header as defined in RFC3261 [12]. |
| TS.64_3.3.2_REQ_011 | In case of Failure of Registration, the FWA Device SHALL follow the procedures indicated at par. 5.1.1.2.1 of 3GPP TS 24.229 [7]. |
| TS.64_3.3.2_REQ_012 | The FWA Device SHOULD support Registration Event Package according to 3GPP TS 24.229 [7]. |
| TS.64_3.3.2_REQ_013 | The FWA Device SHOULD support P-Associated-URI as defined in 3GPP TS 24.229 [7]. |
| TS.64_3.3.2_REQ_014 | The FWA Device SHALL support Tel-URI in incoming requests, in compliance with RFC 5341 [13]. |
| TS.64_3.3.2_REQ_015 | The FWA Device MAY use Tel-URI in outgoing requests, instead of SIP-URI. |
| TS.64_3.3.2_REQ_016 | The FWA Device SHALL be compliant with mandatory requirements of RFC3261 SIP protocol [12]. |
| TS.64_3.3.2_REQ_017 | The FWA Device SHALL be compliant with mandatory requirements of RFC3262 Reliability of Provisional Responses [14]. |
| TS.64_3.3.2_REQ_018 | The FWA Device SHALL be compliant mandatory requirements of with RFC3264 The Offer/Answer Model [15]. |
| TS.64_3.3.2_REQ_019 | The FWA Device SHALL be compliant with mandatory requirements of RFC3311 The UPDATE method [16]. |
| TS.64_3.3.2_REQ_020 | The FWA Device SHALL support an open numbering plan, that is the length of user selection is not predetermined. |
| TS.64_3.3.2_REQ_021 | The FWA Device SHALL use en-bloc selection towards the IMS network. Overlap selection is not foreseen. |
| TS.64_3.3.2_REQ_022 | In case of selections from user containing * or # digits, those digits SHALL be coded as defined in the Device configuration. |
| TS.64_3.3.2_REQ_023 | The FWA Device SHALL support DSCP marking for SIP, RTP and RTCP traffic, with configurable values for DS Code Point. |
| TS.64_3.3.2_REQ_024 | The FWA Device MAY insert a P-Preferred-Identity header in any initial request for a dialog or request for a standalone transaction as a hint for creation of an asserted identity, as per 3GPP 24.229 [7]. |
| TS.64_3.3.2_REQ_025 | The FWA Device SHALL accept incoming INVITE without SDP, in this case the UE shall perform the SDP offer in the first reliable 1xx or 200 Response, as specified in RFC3262 [14]. |
| TS.64_3.3.2_REQ_026 | The FWA Device SHALL support at least the ITU-T G.711 codec (A-law and µ-law) [18], as specified in RFC3261 [12]. |
| TS.64_3.3.2_REQ_027 | The FWA Device SHOULD support ITU-T G.729a [19] [20] and G.722 [21] codecs. |
| TS.64_3.3.2_REQ_028 | If FWA Device supports more than one codec, the order of Codecs in SDP Offer SHALL be configurable. |
| TS.64_3.3.2_REQ_029 | The FWA Device SHALL send and receive DTMF tones complying with RFC4733 [17]. |
| TS.64_3.3.2_REQ_030 | The RTP Payload Type for DTMF Events SHALL be configurable. |
| TS.64_3.3.2_REQ_031 | The FWA Device SHALL support an echo canceller compliant to ITU-T G.131 [22], for managing delays less than 150 ms. |
| TS.64_3.3.2_REQ_032 | The FWA Device SHALL be compliant with mandatory requirements of RFC 3550 (RTP) [23]. |
| TS.64_3.3.2_REQ_033 | The FWA Device SHALL be compliant with mandatory requirements of RFC 3551 (RTP Profile for Audio and Video Conferences with Minimal Control) [24]. |
| TS.64_3.3.2_REQ_034 | The FWA Device SHALL be compliant with mandatory requirements of RFC 3605 (RTCP) [25]. |
| TS.64_3.3.2_REQ_035 | The FWA Device SHOULD be compliant with mandatory requirements of RFC 2198 (RTP Payload for Redundant Audio Data) [26]. |
| TS.64_3.3.2_REQ_036 | The FWA Device SHALL use Symmetric RTP and RTCP, i.e. the UE shall transmit RTP/RTCP packets using the same UDP port used for receiving RTP/RTCP packets, as stated in RFC 4961 [27]. |
| TS.64_3.3.2_REQ_037 | The FWA Device, for Early Media and Ringing Tone Generation, SHALL be compliant, either to RFC 5009 [28], about P-Early-Media header, or RFC 3960 [29], in particular with the “Gateway Model”. |
| TS.64_3.3.2_REQ_038 | When a FWA Device has sent the SDP offer, it SHALL be prepared to receive media for any streams described by that offer, as defined in RFC 3264 [15], even before receiving any SIP provisional response. |
| TS.64_3.3.2_REQ_039 | In case the FWA Device is compliant to RFC 3960, the FWA Device, for ringing tone generation, SHOULD implement the following local policy, derived from the example contained in RFC 3960 [29]: 1. Unless a 180 (Ringing) response is received, never generate local ringing. 2. If a 180 (Ringing) has been received but there are no incoming media packets or incoming media packets containing silence, generate local ringing. 3. If a 180 (Ringing) has been received and there are incoming media packets, not containing silence, play them and do not generate local ringing. |
| TS.64_3.3.2_REQ_040 | The FWA Device SHALL support RE-INVITE and UPDATE methods. |
| TS.64_3.3.2_REQ_041 | The use of UPDATE method SHALL be accompanied by 100rel/PRACK. |
| TS.64_3.3.2_REQ_042 | The FWA Device SHALL insert the following header in INVITEs: Supported:100rel |
| TS.64_3.3.2_REQ_043 | The following parameters SHALL be provisioned for VoIP Service from the centralized management platform: - phone number/line identity: user part of the public user identity, to be used in SIP registration process and in all outgoing SIP Requests in the From header. - Authentication username: user part of the private user identity, to be used during SIP Digest Authentication. - Authentication password: to be used for user authentication during SIP Digest Authentication. - Outbound proxy: FQDN or IP Address of the proxy (P-CSCF) to which the FWA Device sends all outgoing SIP requests. |
| TS.64_3.3.2_REQ_044 | The following parameters SHALL be configurable in the FWA Device: - SIP Domain: domain portion of SIP URL. - SIP Server Address: configurable as a FQDN or as an IP Address. - SIP Registrar Address: configurable as a FQDN or as an IP Address. - Expire Time for Registration. - VoIP Codec list and priority. - RTP payload type to be used for RTP-events. - DSCP values to be used for both SIP and RTP/RTCP. - Voice Activity Detection on/off. - Silence Suppression on/off. - Dejitter buffer controls: static/dynamic depth. - Coding to be used for * and # characters in SIP signalling (dialled sequences): normal ASCII or URL-encoded. - meaning of a trailing # in user selection not beginning with * or #: end-of-selection marker (speed dial) vs. part of the user selection. - Emergency numbers list. - Conference Factory URI. |
| TS.64_3.3.2_REQ_045 | The FWA Device SHOULD discard silently any SIP/RTP/RTCP packet coming from IP source addresses different from the IP Address(es) of the P-CSCF discovered by the Registration Procedure. |
| TS.64_3.3.2_REQ_046 | The FWA Device SHALL support multiple SIP dialogs within a single session, as specified in RFC3261 [12]. NOTE: as specified in RFC3261 [12], a dialog is defined by Call-ID, ‘From’ tag, and ‘To’ tag. |
| TS.64_3.3.2_REQ_047 | The FWA Device SHALL support incoming INVITE with the Request-URI different from the ‘To’ header, as specified in RFC3261 [12]. NOTE: as an example, this occurs when the incoming INVITE has been diverted by the network due to some Diversion service. |
| TS.64_3.3.2_REQ_048 | The FWA Device SHALL reply to an incoming INVITE with Request-URI different from the SIP identity(-ies) configured on the FWA Device itself, with a 404 Not Found response, as specified in RFC3261 [12]. |

### VoIP service: supplementary services for single-line profile

| TS.64_3.3.3_REQ_001 | The procedures for activation, deactivation and interrogation of Supplementary Services through the FXS interface, are based on the use of specific Feature Access Codes (FAC), that is, dialled sequences normally containing * and # keys. The FWA Device SHALL support the signalling of such Supplementary Services codes towards the IMS network, by means of the Gm Interface, that is sending an INVITE with the Request URI containing the * and # sequence dialled by the user. |
| --- | --- |
| TS.64_3.3.3_REQ_002 | If the FWA Device supports activation/deactivation/interrogation Supplementary Services via the Ut interface towards the IMS, the FAC for activation/deactivation/interrogation SHALL be configurable on the Device. |

#### OIP/OIR – ORIGINATING IDENTITY PRESENTATION/ RESTRICTION

| TS.64_3.3.3.1_REQ_001 | The FWA Device SHALL support the P-Asserted-Identity header according to RFC 3325 [30]. |
| --- | --- |
| TS.64_3.3.3.1_REQ_002 | The FWA Device SHALL support OIP (Originating Identity Presentation) and OIR (Originating Identity Restriction) according to 3GPP TS 24.607 [31]. In particular: • If both From and P-Asserted-Identity headers are present, the UE has to provide the Originating Identity Presentation giving priority to P-A-I header with respect to From header. • If multiple P-A-I headers are present in the incoming messages, the UE SHALL provide OIP according to the P-A-I in tel-URI format. |
| TS.64_3.3.3.1_REQ_003 | For the purpose of OIP Service, the FWA Device SHALL support ETSI EN 300 659 on FXS port. In particular, the cases of OIP service Unsubscribed and Identity Restricted SHALL be treated with the ad-hoc coding, defined in ETSI EN 300 659_3 par. 5.4.4 [32]. |
| TS.64_3.3.3.1_REQ_004 | For the purpose of OIP service on FXS port, the FWA Device SHOULD allow reformat the Identity to be displayed, so for instance to remove international prefix from domestic numbers. |
| TS.64_3.3.3.1_REQ_005 | The FWA Device SHOULD perform OIP service on FXS port also in combination with Call Waiting service. |

#### CDIV – COMMUNICATION DIVERSION

| TS.64_3.3.3.2_REQ_001 | The FWA Device SHALL support network-based communication diversion (CDIV) services according to 3GPP TS 24.604 (Communication Diversion Unconditional, No Reply, On Busy, to Voice Mail) [33]. |
| --- | --- |
| TS.64_3.3.3.2_REQ_002 | The FWA Device SHALL support RFC 3842 “A message summary and Message Waiting Indication Event Package for the Session Initiation Protocol” [34] in order to provide Centralized Voice Mail Service. |

#### ACB/CB/DND – ANONYMOUS CALL REJECTION/COMMUNICATION BARRING/DO NOT DISTURB

| TS.64_3.3.3.3_REQ_001 | The FWA Device SHALL support Anonymous Call Rejection (ACR), Communication Barring (CB) and Do Not Disturb (DND) services according to 3GPP TS 24.611 [35]. |
| --- | --- |

#### HOLD - COMMUNICATION HOLD

| TS.64_3.3.3.4_REQ_001 | The FWA Device SHALL support Communication on Hold service according to 3GPP TS 24.610 [36]. |
| --- | --- |
| TS.64_3.3.3.4_REQ_002 | The Keypad procedures for holding an active call, switching between an active and a held call, terminating an active call and resuming an held call, SHALL be configurable. |
| TS.64_3.3.3.4_REQ_003 | In order to provide PSTN emulation of the hold service, FWA Device SHOULD hold an active call when the local user presses the Register Recall (“R” key) on the telephone; after holding the call, offer dial tone to the telephone, in order to allow dialling a new call. |
| TS.64_3.3.3.4_REQ_004 | When a FWA Device has a held call and user is dialling a new number, it SHOULD terminate new call attempt and return to the held call, if the user presses the Register Recall (“R”) key on the telephone during: dial tone, dialling, ringing, busy, fast busy. |
| TS.64_3.3.3.4_REQ_005 | In order to provide PSTN emulation of the hold service, when a FWA Device has an active call and a held call, if the user of the FWA Device hangs up the telephone, the FWA Device SHOULD: - terminate the active call. - ring back (recall) the FXS port - when the local user answers, resume the held call. - If the local user doesn’t answer in 30 seconds, release the held call. |
| TS.64_3.3.3.4_REQ_006 | In order to provide PSTN emulation of the hold service, when a FWA Device has an active call and a held call, if the remote user terminates the active call, the FWA Device SHOULD: - Play 5 seconds of fast-busy as a “guard tone”, then - Resume held call. |
| TS.64_3.3.3.4_REQ_007 | With reference to TS.64_3.3.3_REQ_015, if the local user hangs up the telephone during the guard tone, the FWA Device SHOULD: - ring back (recall) the FXS port - when the local user answers, resume the held call. - If the local user doesn’t answer in 30 seconds, release the held call. |

#### CW - COMMUNICATION WAITING

| TS.64_3.3.3.5_REQ_001 | The FWA Device SHALL support Communication Waiting service according to 3GPP TS 24.615 [37]. |
| --- | --- |
| TS.64_3.3.3.5_REQ_002 | The FWA Device SHOULD be able to recognize an “INVITE on waiting” when the incoming INVITE has the specific XML CW indication. NOTE: the ability to recognize the “INVITE on waiting” could be needed in order to avoid ringing all phones connected to UE when the Call Waiting service has to be provided. |
| TS.64_3.3.3.5_REQ_003 | When required, the FWA Device SHALL generate a configurable Call Waiting Tone (Frequency, Cadence, Level). |
| TS.64_3.3.3.5_REQ_004 | The Call Waiting service SHALL NOT be applied to FXS port in other states than active conversation (e.g., idle, dialling, ringing, hold, reorder/congestion.) |
| TS.64_3.3.3.5_REQ_005 | The Keypad procedures for answering a waiting call while holding an active call, answering a waiting call while terminating an active call, rejecting a waiting call, SHALL be configurable. |
| TS.64_3.3.3.5_REQ_006 | If the FWA Device supports FAX or POS/modem, it SHOULD a support configurable feature interaction between FAX/POS and Call Waiting service: if a FAX/POS call is ongoing, a new incoming call has not to be served with call waiting service but rejected with a 4xx answer. |
| TS.64_3.3.3.5_REQ_007 | The FWA Device SHOULD allow enabling/disabling CW service on a per-FXS port basis. |
| TS.64_3.3.3.5_REQ_008 | On a FXS port, at most one active call and one held can be handled simultaneously. Hence, the FWA Device equipped with one FXS port SHOULD reject an incoming call with Busy when there are simultaneously an active call and a held call. |

#### CONF - CONFERENCE

| TS.64_3.3.3.6_REQ_001 | The FWA Device SHALL support N-Way Conference service according to 3GPP TS 24.147 [38]. |
| --- | --- |
| TS.64_3.3.3.6_REQ_002 | The Keypad procedures SHALL be configurable. |

#### CCBS – COMMUNICATION COMPLETION ON BUSY SUBSCRIBER

| TS.64_3.3.3.7_REQ_001 | The FWA Device SHALL support CCBS service according to 3GPP TS 24.642 [39]. |
| --- | --- |
| TS.64_3.3.3.7_REQ_002 | The CCBS ring pattern SHALL be played in case the incoming INVITE contains the specific Alert-Info Header: Alert-Info: <urn:alert:service:auto-callback> |

#### ECT – EXPLICIT COMMUNICATION TRANSFER

| TS.64_3.3.3.8_REQ_001 | The FWA Device SHOULD support Explicit Communication Transfer (ECT) service according to 3GPP TS 24.629 [40]. |
| --- | --- |
| TS.64_3.3.3.8_REQ_002 | The Keypad procedures SHALL be configurable. |

#### TIP/TIR – TERMINATING IDENTIFICATION PRESENTATION/TERMINATING IDENTIFICATION RESTRICTION

| TS.64_3.3.3.9_REQ_001 | The FWA Device SHOULD support TIP/TIR services according to 3GPP TS 24.608 [41]. |
| --- | --- |

#### MCID – MALICIOUS COMMUNICATION IDENTIFICATION

| TS.64_3.3.3.10_REQ_001 | The FWA Device SHOULD support MCID service according to 3GPP TS 24.616 [42]. |
| --- | --- |

### FXS interface requirements

- Requirements in this section apply to FWA Devices supporting VoLTE profile and FWA Devices supporting VoIP Profile.

| TS.64_3.3.4_REQ_001 | The FWA Device SHALL support at least one FXS (Foreign eXchange Station) port, compliant to ETSI ES 201 970 [43]. |
| --- | --- |
| TS.64_3.3.4_REQ_002 | The FXS interface SHALL support DTMF Dialling. |
| TS.64_3.3.4_REQ_003 | The FXS interface SHALL recognize Register Recall signal, as specified in ETSI ES 201 970 [43]. |
| TS.64_3.3.4_REQ_004 | The FXS interface SHALL support multiple terminals arrangements; in particular, as regards ringing, the FXS interface SHALL support a value of REN (Ringing Equivalence Number) >=4. |
| TS.64_3.3.4_REQ_005 | The FXS interface SHALL support narrowband voice (3.1kHz net bandwidth, in the range 300-3400 Hz) in conformance to Rec. ITU-T Q.522 [44]. |
| TS.64_3.3.4_REQ_006 | The FXS interface SHOULD support wideband voice (8 kHz bandwidth). |
| TS.64_3.3.4_REQ_007 | The following parameters related to the FXS interface SHALL be customizable: - Supervision Tones: for instance, Dial, Ringing, Busy, Congestion, Call Waiting. Customization shall be possible as regards cadence, frequency and level. - Timers: initial digit timeout, interdigit timeout (Note: interdigit timeout is used to infer the end of the user selection). - Ringing current: customization SHALL be possible as regards cadence, frequency and level. - Polarity Reversal. - Input impedance - TX-gain - RX-gain - DC feed voltage and current limit. - Howler tone control (on/off). |

### FAX and POS (Point of Sale) Requirements

- The requirements in this section apply to a FWA Device supporting VoIP profile.

| TS.64_3.3.5_REQ_001 | The FWA Device SHOULD support FAX and/or POS service using the same FXS port for voice. |
| --- | --- |
| TS.64_3.3.5_REQ_002 | The FWA Device SHOULD support FAX transmission, at least by means of upspeed to ITU-T G.711 [18]. |
| TS.64_3.3.5_REQ_003 | The FWA Device SHOULD support POS/modem transmission, at least by means of upspeed to ITU-T G.711 [18]. |
| TS.64_3.3.5_REQ_004 | The called FWA Device SHOULD send the ReINVITE (upspeed to G.711), when it detects the CED/ANSam in case of FAX; in case of modem/POS, the called FWA Device SHOULD send the ReINVITE (upspeed to G.711), when it detects the modem carrier signal. |
| TS.64_3.3.5_REQ_005 | When performing upspeed to G.711, either initiated by the FWA Device or by the remote party, the FWA Device SHOULD disable, for the ongoing call, all voice signal processing features, such as echo canceller, voice activity detection. NOTE: this is needed as voice signal processing features introduce non-linearities in the coded signal; instead, for the transport of voice-band data over G.711, it is necessary to keep the original signals unmodified from the source to the destination. |
| TS.64_3.3.5_REQ_006 | When performing upspeed to G.711, either initiated by the FWA Device or by the remote party, the FWA Device SHOULD use a static dejitter buffer. |
| TS.64_3.3.5_REQ_007 | The FWA Device SHOULD support T.38 fax transmission, as specified in ITU-T T.38 (09/2010) [45]. In particular: a. The calling FWA Device SHOULD include, in the SDP Offer of the INVITE, the line a=cdsc:1 image udptl t38 b. The called FWA Device SHOULD perform an upspeed to T.38, when it detects the V.21 preamble, by sending a ReINVITE, if in the received INVITE is present the line a=cdsc:1 image udptl t38. |
| TS.64_3.3.5_REQ_008 | The FWA Device MAY support the capabilities for T.38 defined by Annex D of ITU-T T.38 (11/2015) [46]. |
| TS.64_3.3.5_REQ_009 | In case a ReINVITE fails, for example because the remote party is unable to perform upspeed (i.e. a 488 Not Acceptable Here Response is received), the FWA Device SHALL continue the session using the previously negotiated codec(s). In this case, the FWA Device MAY send a ReINVITE to confirm again to the remote party the previously negotiated codecs. |
| TS.64_3.3.5_REQ_010 | The FWA Device SHOULD allow, on a per-FXS port basis, the possibility to choose whether to use only the G.711 codec. |
| TS.64_3.3.5_REQ_011 | When the configuration for use of the G.711 codec only is enabled, the FWA Device SHOULD disable all voice signal processing features, such as echo canceller, voice activity detection. |
| TS.64_3.3.5_REQ_012 | When the configuration for use of the G.711 codec only is enabled, the FWA Device SHOULD use a static dejitter buffer. |
| TS.64_3.3.5_REQ_013 | The FWA Device SHOULD allow the possibility to enable or disable T.38, on a device basis. |

### Voice Quality

- The requirements in this section apply to a FWA Device supporting VoIP profile over a dedicated PDN/PDU connection.

- Requirements for VoLTE/VoNR quality are FFS.

| TS.64_3.3.6_REQ_001 | If the FWA Device supports VoIP, the FWA Device SHALL offer a MOS of at least 3,2 in both directions of voice communication, when the G.729a [19] codec is used in the communication. |
| --- | --- |
| TS.64_3.3.6_REQ_002 | If the FWA Device supports VoIP, the FWA Device SHOULD offer a MOS of at least 3,5 in both directions of voice communication, when the G.729a [19] codec is used in the communication. |
| TS.64_3.3.6_REQ_003 | If the FWA Device supports VoIP, the FWA Device SHALL offer a MOS of at least 3,7 in both directions of voice communication, when the G.711 [18] codec is used in the communication. |
| TS.64_3.3.6_REQ_004 | If the FWA Device supports VoIP, the FWA Device SHOULD offer a MOS of at least 4 in both directions of voice communication, when the G.711 [18] codec is used in the communication. |

## Networking Features

Except where explicitly indicated, the Requirements of this section apply to the Indoor FWA Device, the InDoor Unit and the OutDoor Unit of an Outdoor FWA Solution.

### Interfaces

| TS.64_3.4.1_REQ_001 | The indoor FWA Device and the InDoor Unit of an Outdoor FWA Solution SHALL support at least two Gigabit Ethernet LAN ports, compliant to IEEE 802.3ab standard. |
| --- | --- |
| TS.64_3.4.1_REQ_002 | For the physical interfaces for LAN Ethernet, the 1000BASE-T electrical interface SHOULD be used. |
| TS.64_3.4.1_REQ_003 | Different physical interfaces for LAN Ethernet, compliant to the standards, e.g. 1000BASE-TX, MAY be used in alternative to 1000BASE-T, depending on market and MNOs needs. |
| TS.64_3.4.1_REQ_004 | The InDoor Unit and the OutDoor Unit of an Outdoor FWA Solution SHALL have a connection coherent with the LAN-WAN capability of the Device as defined in the performance requirements below. |
| TS.64_3.4.1_REQ_005 | For the physical interfaces for the IDU-ODU, if Ethernet is used, the BASE-T electrical interface SHOULD be used. |
| TS.64_3.4.1_REQ_006 | If Ethernet is used for the IDU-ODU connection, different physical interfaces, compliant to the standards, e.g. 1000BASE-TX, MAY be used in place of BASE-T, depending on market and MNOs needs. |

### Performance

| TS.64_3.4.2_REQ_001 | The 4G FWA Device SHALL offer an aggregate throughput of at least 1 Gb/s bidirectional between LAN interfaces, either Ethernet or Wi-Fi, irrespective of IPv4 or IPv6 protocol, irrespective of Packet Length. |
| --- | --- |
| TS.64_3.4.2_REQ_002 | The 4G FWA Device SHOULD offer an aggregate throughput of at least 2,5 Gb/s bidirectional between LAN interfaces, either Ethernet or Wi-Fi, irrespective of IPv4 or IPv6 protocol, irrespective of Packet Length. |
| TS.64_3.4.2_REQ_003 | The 5G FWA Device SHALL offer an aggregate throughput of at least 2,5 Gb/s bidirectional between LAN interfaces, either Ethernet or Wi-Fi, irrespective of IPv4 or IPv6 protocol, irrespective of Packet Length. |
| TS.64_3.4.2_REQ_004 | The 5G FWA Device SHOULD offer an aggregate throughput of at least 5 Gb/s bidirectional between LAN interfaces, either Ethernet or Wi-Fi, irrespective of IPv4 or IPv6 protocol, irrespective of Packet Length. |
| TS.64_3.4.2_REQ_005 | The 4G FWA Device SHALL offer a throughput LAN-WAN coherent with the LTE UE Category of the Device, irrespective of IPv4 or IPv6 protocol, irrespective of Packet Length and not affected by the local LAN-LAN throughput. |
| TS.64_3.4.2_REQ_006 | The 5G FWA Device SHALL offer a throughput LAN-WAN coherent with 5G cellular bandwidth of the FWA Device, irrespective of IPv4 or IPv6 protocol, irrespective of Packet Length and not affected by the local LAN-LAN throughput. |
| TS.64_3.4.2_REQ_007 | The use of minimum 4 concurrent PDU/PDN connections to LAN traffic SHALL conform to section 3.1 and not introduce adverse impact on performance. (to be reviewed with 008) |
| TS.64_3.4.2_REQ_008 | The 5G FWA SHOULD support 8 concurrent WWAN-LAN mappings without limiting performance on any mapping |

NOTE: in the Outdoor FWA Solution, requirements TS.64_3.4.2_REQ_001, TS.64_3.4.2_REQ_002, TS.64_3.4.2_REQ_003 and TS.64_3.4.2_REQ_004 apply only to the Indoor Unit – LAN-LAN traffic is managed only by the Indoor Unit.

For TS.64_3.4.2_REQ_005 and TS.64_3.4.2_REQ_006, supported throughputs must align with the maximum WAN throughout. Figures for selected 4G and 5G device categories are as follows:

| Device Category | Maximum Downlink (DL) throughput | Maximum Uplink (UL) throughput |
| --- | --- | --- |
| LTE Category DL11UL5 | 600 Mbps | 75 Mbps |
| LTE Category DL12UL13 | 600 Mbps | 150 Mbps |
| LTE Category DL16UL13 | 975 Mbps | 150 Mbps |
| LTE Category DL18UL18 | 1200 Mbps | 210 Mbps |
| NSA DL12UL13 LTE + 100MHz NR | 2100 Mbps | 200 Mbps |
| NSA DL12UL13 LTE + 140MHz NR | 2700 Mbps | 200 Mbps |
| NSA DL12UL13 LTE + 200MHz NR | 3600 Mbps | 200 Mbps |
| SA 100MHz NR | 1500 Mbps | 100 Mbps |
| SA 140MHz NR | 2100 Mbps | 140 Mbps |
| SA 200MHz NR | 3000 Mbps | 200 Mbps |

NOTE: The table assumes NR channels are TDD configured as 70:30 DL:UL

### Protocols

| TS.64_3.4.3_REQ_001 | The FWA Device SHALL support Internet Protocol version 4 (IPv4), defined in IETF RFC 791 [60]. |
| --- | --- |
| TS.64_3.4.3_REQ_002 | The FWA Device SHALL support Internet Protocol version 6 (IPv6), defined in IETF RFC 8200 [61] and further amendments defined by IETF. |
| TS.64_3.4.3_REQ_003 | The FWA Device SHALL support Address Resolution Protocol (ARP), defined in IETF RFC 826 [62] and further amendments (IETF RFC 5227 [63], IETF RFC 5494 [64]). |
| TS.64_3.4.3_REQ_004 | The FWA Device SHALL support Neighbor Discovery Protocol for IPv6 (NDP) defined in IETF RFC 4861 [65] and further amendments defined by IETF. |
| TS.64_3.4.3_REQ_005 | The FWA Device SHALL support Internet Control Message Protocol (ICMP) defined in IETF RFC 792 [66] and further amendments defined by IETF (RFC 950 [67], RFC 4884 [68], RFC 6633 [69], RFC 6918 [70]). |
| TS.64_3.4.3_REQ_006 | The FWA Device SHALL support Internet Control Message Protocol version 6 for IPv6 (ICMPv6) defined in IETF RFC 4443 [71]. |
| TS.64_3.4.3_REQ_007 | The FWA Device SHALL implement a Network Time Protocol (NTP) client as defined in IETF RFC 5905 [72] and further amendments. |
| TS.64_3.4.3_REQ_008 | The FWA Device SHALL support Internet Group Management Protocol, version 3 (IGMPv3), defined in IETF RFC 3376 [73]. |
| TS.64_3.4.3_REQ_009 | The FWA Device SHALL support IGMP Proxy as defined in IETF RFC 4605 [74]. |
| TS.64_3.4.3_REQ_010 | The FWA Device SHALL support QoS Treatment both at level 2 (p-bits of 802.1q VLAN Tag [75]) and at level 3 (Differentiated Services Code Point of the IP header). |
| TS.64_3.4.3_REQ_011 | The FWA Device SHALL support the Differentiated Services (DiffServ) architecture and behaviours defined in IETF RFC 2474 [76], RFC 2475 [77], RFC 2597 [78], RFC 3260 [79]). |
| TS.64_3.4.3_REQ_012 | The behaviours of traffic classification, marking, remarking, queueing, scheduling, policing, shaping SHALL be applicable both to internally generated traffic and to traffic coming from LAN and destined to the WAN. |
| TS.64_3.4.3_REQ_013 | At least four queues SHALL be supported on the WAN interface, of which one Strict Priority scheduling, and the others with configurable scheduling mechanism (e.g. Weighted Fair Queueing, Weighted Round Robin, ..). |
| TS.64_3.4.3_REQ_014 | The FWA Device SHOULD support a secondary IPv4 addressing on LAN, in order to enable the assignment of public IP addresses to clients in LAN. |
| TS.64_3.4.3_REQ_015 | The FWA Device SHALL support VLAN Tagging, compliant to IEEE 802.1q standard [75]. |

### DHCP

| TS.64_3.4.4_REQ_001 | The FWA Device SHALL support Dynamic Host Configuration Protocol (DHCP) defined in IETF RFC 2131 [98]. |
| --- | --- |
| TS.64_3.4.4_REQ_002 | The FWA Device SHALL support DHCP Options defined in IETF RFC 2132 [99]. |
| TS.64_3.4.4_REQ_003 | The FWA device MAY implement DHCP options 60 and 43 for automatic provision of ACS parameters. |
| TS.64_3.4.4_REQ_004 | The DHCP Server implemented by the FWA Device SHALL manage at least 254 addresses. |
| TS.64_3.4.4_REQ_005 | It SHALL be possible to define any IPv4 Unicast subnet for the private LAN and DHCP pool. |
| TS.64_3.4.4_REQ_006 | The DHCP Server implemented by the FWA Device SHALL support Duplicate Address Detection (DAD) functionality. |
| TS.64_3.4.4_REQ_007 | The DHCP Server implemented by the FWA Device SHALL provide a mechanism for IP reservation on MAC Address basis, assigning the same IP address (if available) at the same MAC Address. |
| TS.64_3.4.4_REQ_008 | The FWA Device SHALL support hostnames presented by the hosts (DHCP clients) with DHCP Option 12. |
| TS.64_3.4.4_REQ_009 | The FWA Device SHALL properly manage the cases of overlapping hostnames and hostnames non presented by clients, by assigning to client’s unambiguous hostnames by means of Option 12. |
| TS.64_3.4.4_REQ_010 | The FWA Device SHALL support Dynamic Host Configuration Protocol for IPv6 (DHCPv6) defined in IETF RFC 8415 [100]. |
| TS.64_3.4.4_REQ_011 | The FWA Device SHALL support Prefix Delegation for IPv6 (DHCPv6) defined in IETF RFC 8415 [100]. |
| TS.64_3.4.4_REQ_012 | The FWA Device SHALL support Prefix Exclude for IPv6 (DHCPv6) defined in IETF RFC 8415 [100]. |

### NAT & Bridge operation

| TS.64_3.4.5_REQ_001 | The FWA Device SHALL support IP Network Address Translator (NAT) as defined in IETF RFC 3022 [101]. |
| --- | --- |
| TS.64_3.4.5_REQ_002 | The Network Address Translator functionality implemented by the FWA Device SHALL be compliant to the behaviours defined in IETF RFC 4787 [102]. |
| TS.64_3.4.5_REQ_003 | The FWA Device SHALL implement a configurable Port Mapping/Virtual Server functionality, allowing the creation of entries for mapping protocols/ports on the WAN side of the FWA Device to an IP address and protocols/ports on the private LAN. |
| TS.64_3.4.5_REQ_004 | It SHALL be possible to configure at least 32 Port Mapping entries. |
| TS.64_3.4.5_REQ_005 | The FWA Device SHALL support CLAT functionality according to IETF RFC 6145 [103]. |
| TS.64_3.4.5_REQ_006 | The FWA Device SHALL support operation in Bridge Mode. In this configuration both DHCP and NAT operations are provided by the network being bridged to. |
| TS.64_3.4.5_REQ_007 | The FWA Device SHALL implement a configurable Application Layer Gateway functionality (ALG), as defined in IETF RFC 2663 [80], at least for the following protocols: SIP, IPSec, PPTP, L2TP. |

### MTU

| TS.64_3.4.6_REQ_001 | The FWA Device SHALL support a default MTU size of 1380 bytes. |
| --- | --- |
| TS.64_3.4.6_REQ_002 | The default MTU size SHALL be customisable according to network requirements. |
| TS.64_3.4.6_REQ_003 | The FWA Device SHALL support network override of the default MTU size in IPv4 operation via Protocol Configuration Options (3GPP TS24.008 [104]). |
| TS.64_3.4.6_REQ_004 | The FWA Device SHALL support network override of the default MTU size in IPv6 operation via Router Advertisement (IETF RFC 4861 [65]). |

### DNS

| TS.64_3.4.7_REQ_001 | The FWA Device SHALL support Domain Name System (DNS) compliant to IETF RFC 1034 [105], RFC 1035 [106] and further amendments defined by IETF. |
| --- | --- |
| TS.64_3.4.7_REQ_002 | The FWA Device SHALL be able, on a configuration basis, to act as DNS Server for the Clients in LAN. |
| TS.64_3.4.7_REQ_003 | The FWA Device SHALL be able to advertise the DNS server(s) to the Clients in LAN via DHCP protocol. On a configuration basis, the advertised DNS server(s) can be: - The FWA Device itself, if it’s configured to act as a DNS Server; - The DNS server addresses received from the network, if the FWA Device is not configured to act as a DNS Server; - Optionally, other DNS Server addresses configured on the FWA Device. |
| TS.64_3.4.7_REQ_004 | The FWA Device SHALL support a configurable Dynamic DNS (DDNS) Service, allowing the FWA Device to be addressable from the Internet with an FQDN. NOTE: some MNOs may wish to remove this service via customization. |
| TS.64_3.4.7_REQ_005 | For the Dynamic DNS service, the FWA Device SHALL send updates to the DDNS server not periodically, but only whenever an IP address change is detected on the Data WAN Interface. |
| TS.64_3.4.7_REQ_006 | For Static DNS operation the FWA Device SHALL support Recursive DNS. |
| TS.64_3.4.7_REQ_007 | For Static DNS operation the FWA Device SHALL NOT support Iterative DNS. |
| TS.64_3.4.7_REQ_008 | The FWA Device SHALL support unencrypted DNS access. |
| TS.64_3.4.7_REQ_009 | The FWA Device SHALL support DNS access via HTTPS (IETF RFC 8484 [107]). |
| TS.64_3.4.7_REQ_010 | The FWA Device SHALL support DNS access via TLS. |
| TS.64_3.4.7_REQ_011 | The FWA device SHALL be protected against DNS Rebind Vulnerability. |
| TS.64_3.4.7_REQ_012 | To prevent DNS spoofing, source ports and Transaction-IDs SHALL be selected randomly by the FWA. |

### Security

| TS.64_3.4.8_REQ_001 | The FWA Device SHALL implement a configurable DeMilitarized Zone (DMZ) functionality, allowing an internal host in LAN to be fully exposed on WAN. |
| --- | --- |
| TS.64_3.4.8_REQ_002 | The FWA Device SHALL implement a configurable Port Binding functionality, allowing binding of the WAN connections to none, one or more LAN interfaces (including Wi-Fi SSIDs). |
| TS.64_3.4.8_REQ_003 | The FWA Device SHALL implement a configurable Filtering functionality, allowing the creation of entries for blocking/allowing the communication of MAC Addresses on LAN towards specific IP address/range, on specific protocols/ports/port range. |
| TS.64_3.4.8_REQ_004 | It SHALL be possible to configure at least 32 Filtering entries. |
| TS.64_3.4.8_REQ_005 | The FWA Device SHOULD implement a configurable (on/off) UPnP Discovery functionality, compliant to UPnP Forum Standard: Device Control Protocols. |
| TS.64_3.4.8_REQ_006 | UPnP functionality SHALL be blocked on the WAN side. |
| TS.64_3.4.8_REQ_007 | If the FWA Device supports UPnP, it SHOULD be disabled in Factory default configuration. |
| TS.64_3.4.8_REQ_008 | If the FWA Device supports UPnP, rules created for one client device SHALL apply only to that device and not to other LAN clients (also for the FWA Device itself). |
| TS.64_3.4.8_REQ_009 | The FWA Device SHOULD implement a configurable VPN functionality, both as a VPN-client and a VPN-server, with L2TP/IPSec PSK or PPTP. |
| TS.64_3.4.8_REQ_010 | The FWA Device SHALL implement a Parental Control functionality, letting the user to configure a list of URLs which access must be denied to all (or a configurable subset of) LAN clients. |
| TS.64_3.4.8_REQ_011 | The FWA Device SHALL implement a per-user device configurable Internet access control functionality, letting the user to configure, for a selected user device, which days of the week/which hours of the day or how many hours per day the Internet access must be allowed/denied. |
| TS.64_3.4.8_REQ_012 | The FWA Device SHALL implement a configurable (at least with on/off behaviours) stateful IPv4 Firewall. |
| TS.64_3.4.8_REQ_013 | The FWA Device SHALL implement a Denial of Service (DoS) protection functionality. |
| TS.64_3.4.8_REQ_014 | The DoS functionality SHALL remain enabled even when the Firewall has been disabled by user configuration |
| TS.64_3.4.8_REQ_015 | The behaviour of the FWA Device to ICMP messages coming from WAN interface SHALL be configurable. |
| TS.64_3.4.8_REQ_016 | The FWA Device SHALL implement a configurable (at least with on/off behaviours) stateful IPv6 Firewall. |
| TS.64_3.4.8_REQ_017 | The IPv6 and IPv4 firewall SHALL be independently configurable |
| TS.64_3.4.8_REQ_018 | The IPv6 and IPv4 firewall status SHALL be presented independently. |
| TS.64_3.4.8_REQ_019 | The FWA Device SHALL NOT allow outgoing traffic originated from a LAN IP address outside the range defined by the FWA Device itself. |
| TS.64_3.4.8_REQ_020 | In Factory Reset condition, the status of the firewall SHALL be enabled. |

### Customisation

| TS.64_3.4.9_REQ_001 | It SHALL be possible for a MNO to customize a FWA Device in order to hide to the end-user any of the functionality detailed in requirements TS.64_3.4.3_REQ_014, TS.64_3.4.5_REQ_003, TS.64_3.4.5_REQ_004, TS.64_3.4.5_REQ_007, TS.64_3.4.7_REQ_004, TS.64_3.4.7_REQ_005, TS.64_3.4.8_REQ_001 to TS.64_3.4.8_REQ_011. |
| --- | --- |

### USB Port

| TS.64_3.4.10_REQ_001 | The FWA Device MAY support a Universal Serial Bus (USB) interface. |
| --- | --- |

The following requirements apply ONLY IF the FWA Device supports a USB Interface for functions other than powering the FWA Device.

| TS.64_3.4.10_REQ_002 | The USB interface SHALL be compliant to the Universal Serial Bus Specification version 3.1 [108] or higher. |
| --- | --- |
| TS.64_3.4.10_REQ_003 | The USB Interface receptacle SHALL be any of Type-A, Type-Micro B or Type-C. |
| TS.64_3.4.10_REQ_004 | The USB Interface SHOULD supply a current of at least 1.5A |
| TS.64_3.4.10_REQ_005 | The FWA Device SHALL integrate the USB Mass Storage Device Class driver, to provide the automatic detection of an USB Mass Storage (such as Hard Disk, CD/DVD ROM, USB memory stick, etc.) connected to the USB Host Port. |
| TS.64_3.4.10_REQ_006 | The FWA Device SHALL integrate the USB Printer Device Class driver, to provide the automatic detection of an USB Printer connected to the USB Host Port. |
| TS.64_3.4.10_REQ_007 | The FWA Device SHOULD use SMBv2 (or higher) protocol to enable the sharing of an USB Mass Storage Hard Disk Devices between LAN hosts. |
| TS.64_3.4.10_REQ_008 | The FWA Device SHOULD use SMBv2 (or higher) protocol to enable the print sharing between the LAN hosts, supporting the standard error messages via SMB protocol. |
| TS.64_3.4.10_REQ_009 | The FWA Device SHALL NOT support SMBv1 protocol. |
| TS.64_3.4.10_REQ_010 | The USB Interface SHALL block firmware upgrade, logging, tracing and similar local management and troubleshooting activities on the FWA Device. |

## Wi-Fi

Requirements in this section apply to the Indoor FWA Solution and to the InDoor Unit (IDU) of the Outdoor FWA Solution. There may be MNO or market specific Wi-Fi requirements for the outdoor unit (ODU).

### Standards

| TS.64_3.5.1_REQ_001 | The FWA Device SHALL integrate a Wi-Fi 4 (IEEE 802.11n [83]) Access Point (AP). |
| --- | --- |
| TS.64_3.5.1_REQ_002 | The FWA Device SHALL integrate a Wi-Fi 5 (IEEE 802.11ac [84]) Access Point (AP). |
| TS.64_3.5.1_REQ_003 | The FWA Device SHOULD integrate a Wi-Fi 6 (IEEE 802.11ax [85]) Access Point (AP). |
| TS.64_3.5.1_REQ_004 | The FWA Device MAY integrate a Wi-Fi 6E (IEEE 802.11ax [85]) Access Point (AP). |
| TS.64_3.5.1_REQ_005 | The FWA Device SHALL be certified (WFA Certification Program): Wi-Fi CERTIFIED 5. |
| TS.64_3.5.1_REQ_006 | If Wi-Fi 6 is supported, the FWA Device SHALL be certified (WFA Certification Program): Wi-Fi CERTIFIED 6. |
| TS.64_3.5.1_REQ_007 | The FWA Device SHALL be certified (WFA Certification Program): WPA3. |
| TS.64_3.5.1_REQ_008 | The FWA Device SHALL be certified (WFA Certification Program): Wi-Fi Protected Setup (PBC). |
| TS.64_3.5.1_REQ_009 | The FWA Device SHOULD integrate a Wi-Fi 7 (IEEE 802.11be [112]) Access Point (AP). |

### MIMO capabilities, Bandwidth, Modulation and Coding schemes

| TS.64_3.5.2_REQ_001 | The Wi-Fi AP of a FWA Device SHALL support at least MIMO 2x2 on all supported frequency bands. |
| --- | --- |
| TS.64_3.5.2_REQ_002 | The Wi-Fi AP of a FWA Device SHOULD support MIMO 4x4 on all supported frequency bands. |
| TS.64_3.5.2_REQ_003 | The Wi-Fi AP of a FWA Device MAY support MIMO higher than 4x4 on some or all supported frequency bands. |
| TS.64_3.5.2_REQ_004 | An 802.11n [83] AP of a FWA Device SHALL support a bandwidth of 40 MHz. |
| TS.64_3.5.2_REQ_005 | An 802.11ac [84] AP of a FWA Device SHALL support a bandwidth of 80 MHz in the 5 GHz band. |
| TS.64_3.5.2_REQ_006 | An 802.11ax [85] AP of a FWA Device SHALL support a bandwidth of 40 MHz in the 2.4 GHz band. |
| TS.64_3.5.2_REQ_007 | An 802.11ax AP of a FWA Device SHALL support a bandwidth of 80 MHz in the 5 GHz band. |
| TS.64_3.5.2_REQ_008 | An 802.11ax AP of a FWA Device SHOULD support a bandwidth of 160 MHz in the 5 GHz band. |
| TS.64_3.5.2_REQ_009 | An 802.11ax (Wi-Fi 6E) AP of a FWA Device SHALL support a bandwidth of 160 MHz in the 6 GHz band, if supported. |
| TS.64_3.5.2_REQ_010 | An 802.11n AP of a FWA Device SHALL support all the Modulation and coding schemes foreseen by the standard, up to 64-QAM with coding 5/6. |
| TS.64_3.5.2_REQ_011 | An 802.11ac AP of a FWA Device SHALL support all the Modulation and coding schemes foreseen by the standard, up to 256-QAM with coding 5/6. |
| TS.64_3.5.2_REQ_012 | An 802.11ax AP of a FWA Device SHALL support all the Modulation and coding schemes foreseen by the standard, up to 1024-QAM with coding 5/6. |

### Performance

| TS.64_3.5.3_REQ_001 | The AP of a FWA Device SHALL offer a throughput coherent with the theoretical maximum physical bit rate attainable by the AP characteristics, at least 70% of Maximum Physical Speed with TCP and UDP traffic in a “clean” environment |
| --- | --- |
| TS.64_3.5.3_REQ_002 | All the Wi-Fi interfaces SHALL NOT exceed the regulatory limits as regards output power level (EIRP). |
| TS.64_3.5.3_REQ_003 | For Wi-Fi antenna performance, the FWA Device SHALL comply to the requirements specified in TS.49 [97]. |

Figures for selected Wi-Fi AP types are as follows:

| Wi-Fi AP Type | Theoretical Maximum Physical bit rate |
| --- | --- |
| 802.11n 2.4 GHz, MIMO 2x2, 40 MHz, 64-QAM | 300 Mbps |
| 802.11n 2.4 GHz, MIMO 4x4, 40 MHz, 64-QAM | 600 Mbps |
| 802.11ac 5 GHz, MIMO 2x2, 80 MHz, 256-QAM | 866 Mbps |
| 802.11ac 5 GHz, MIMO 4x4, 80 MHz, 256-QAM | 1733 Mbps |
| 802.11ax 2.4+5 GHz, MIMO 2x2 + 2x2, 80 MHz, 1024-QAM | 600 + 1200 Mbps |
| 802.11ax 2.4+5 GHz, MIMO 2x2 + 2x2, 160 MHz, 1024-QAM | 600 + 2400 Mbps |
| 802.11ax 2.4+5 GHz, MIMO 4x4 + 4x4, 160 MHz, 1024-QAM | 1200 + 4800 Mbps |

### Service Set Identifier (SSID)

| TS.64_3.5.4_REQ_001 | The AP of a FWA Device SHALL permit the configuration of one main SSID for each supported band. |
| --- | --- |
| TS.64_3.5.4_REQ_002 | The AP of a FWA Device SHALL permit the configuration of at least one guest SSID. |
| TS.64_3.5.4_REQ_003 | The guest SSID(s) SHALL NOT permit the access to the configuration of the FWA Device. |
| TS.64_3.5.4_REQ_004 | The guest SSID(s) SHALL NOT permit traffic between clients in LAN. |
| TS.64_3.5.4_REQ_005 | Each SSID SHALL be configurable to operate on one or more frequency bands. |
| TS.64_3.5.4_REQ_006 | Each SSID SHALL be configurable as regards the Authentication and Security mechanisms adopted. |
| TS.64_3.5.4_REQ_007 | Each SSID SHALL be configurable as regards the SSID broadcasting. |
| TS.64_3.5.4_REQ_008 | The default configuration of the FWA Device SHALL be with the same SSID for all supported bands. |
| TS.64_3.5.4_REQ_009 | Based on MNO requirements, in the default configuration, the SSIDs MAY have an unambiguous, not-repeating value for each deployed FWA Device and not contain any information that consist of or are derived from data or parts of data that depend on the FWA device model itself. |

### Channel and Bandwidth Selection

| TS.64_3.5.5_REQ_001 | The AP of a FWA Device SHALL permit the manual channel selection on all supported bands. |
| --- | --- |
| TS.64_3.5.5_REQ_002 | The AP of a FWA Device SHALL support Automatic Channel Selection on all supported bands, in order to select the less interfered channels. |
| TS.64_3.5.5_REQ_003 | If enabled, the Automatic Channel Selection SHALL be performed every time the AP is turned on. |
| TS.64_3.5.5_REQ_004 | The AP of a FWA Device SHALL support Periodic Automatic Channel Selection. |
| TS.64_3.5.5_REQ_005 | The default value for Periodic Automatic Channel Selection SHOULD be 24 hours. |
| TS.64_3.5.5_REQ_006 | The Periodic Automatic Channel Selection SHALL be configurable by the MNO through customization. |
| TS.64_3.5.5_REQ_007 | The AP of a FWA Device SHALL permit the manual Bandwidth selection on all supported bands. |
| TS.64_3.5.5_REQ_008 | The AP of a FWA Device SHALL support Automatic Bandwidth Selection on all supported bands. |

### Clients

| TS.64_3.5.6_REQ_001 | The AP of a FWA Device SHALL support at least 64 clients. |
| --- | --- |

### Security

| TS.64_3.5.7_REQ_001 | The AP of a FWA Device SHALL support all encryption algorithms foreseen by the supported standards. |
| --- | --- |
| TS.64_3.5.7_REQ_002 | The AP of a FWA Device SHALL support all security and authentication features foreseen by the supported standards. |
| TS.64_3.5.7_REQ_003 | The AP of a FWA Device SHALL support Open Mode security. |
| TS.64_3.5.7_REQ_004 | The AP of a FWA Device SHOULD support Wi-Fi Alliance Wi-Fi Protected Access 2 (WPA2) defined in IEEE 802.11i [109]. NOTE: the support of WPA2 is required for compatibility reasons, although it is recognized the security of WPA2 is already compromised. |
| TS.64_3.5.7_REQ_005 | The AP of a FWA Device SHALL support Wi-Fi Alliance Wi-Fi Protected Access 3 (WPA3) [110]. |
| TS.64_3.5.7_REQ_006 | VOID |
| TS.64_3.5.7_REQ_007 | The AP of a FWA Device SHALL support CTR mode with CBC-MAC Protocol (CCMP) security protocol, also known as Advanced Encryption Standard (AES). |
| TS.64_3.5.7_REQ_008 | The AP of a FWA Device SHALL support WPA2/WPA3 personal mixed mode of operation. |
| TS.64_3.5.7_REQ_009 | The AP of a FWA Device SHOULD support WPA2/WPA3 Enterprise mode of operation. |
| TS.64_3.5.7_REQ_010 | The FWA Device default configuration SHALL be personal mixed mode WPA2/WPA3 with AES. |
| TS.64_3.5.7_REQ_011 | The passphrase (pre-shared key, PSK) configured in factory setting SHALL have a length of at least 20 characters. |
| TS.64_3.5.7_REQ_012 | The passphrase (pre-shared key, PSK) configured in factory setting SHALL NOT contain information that consist of or are derived from data or parts of data that depend on the Device itself. |
| TS.64_3.5.7_REQ_013 | The procedure to set the Wi-Fi passphrase to a different value SHALL be supported by a mechanism showing the strength of the new desired passphrase based on the number of characters and classes of characters (e.g. numbers, letters). |
| TS.64_3.5.7_REQ_014 | The FWA Device SHALL implement a configurable overall Wi-Fi control functionality, letting the user to configure which days of the week/which hours of the day the wireless service is turned ON/OFF. |
| TS.64_3.5.7_REQ_015 | The Wi-Fi control functionality MAY be implemented with finer granularity, e.g. on a per-radio interface basis or on a per-SSID basis. |

### Wi-Fi protected Setup (WPS)

| TS.64_3.5.8_REQ_001 | The AP of a FWA Device SHALL support WPS in order to facilitate the association between clients and the AP of the FWA Device. |
| --- | --- |
| TS.64_3.5.8_REQ_002 | The AP of a FWA Device SHALL support WPS with Push Button mode. |
| TS.64_3.5.8_REQ_003 | The WPS Push Button MAY be either physical or logical (i.e., in the Web UI of the FWA Device) or both. |
| TS.64_3.5.8_REQ_004 | The AP of a FWA Device MAY support WPS PIN mode, with PIN generated by client. |
| TS.64_3.5.8_REQ_005 | The WPS PIN mode with PIN generated by the AP of the FWA Device SHALL be forbidden. |
| TS.64_3.5.8_REQ_006 | WPS Push Button authentication SHALL only allow a limited number of attempts per period of time, to be protected against brute force attacks. |
| TS.64_3.5.8_REQ_007 | The WPS nonce generated by the device SHALL be random. |

### Band Steering

| TS.64_3.5.9_REQ_001 | The AP of a FWA Device SHALL support IEEE 802.11k [86] industry standard for radio resource measurement. |
| --- | --- |
| TS.64_3.5.9_REQ_002 | The AP of a FWA Device SHALL support IEEE 802.11v [87] industry standard to allow configuration of client devices while connected to wireless networks, |
| TS.64_3.5.9_REQ_003 | The AP of a FWA Device SHALL support Band Steering to steer clients from the more congested 2.4 GHz band to the less congested bands (5 GHz, and 6GHz if supported). |
| TS.64_3.5.9_REQ_004 | The Band Steering feature SHALL be manually configurable (ON/OFF selection). |
| TS.64_3.5.9_REQ_005 | The FWA Device SHALL provide a mechanism to manually or automatically configure SSIDs on all supported bands, accordingly with Band Steering settings (i.e. same SSIDs when Band Steering is active). |

### Mesh Networks

Requirements in this section apply to a FWA Device supporting Mesh capability.

| TS.64_3.5.10_REQ_001 | The AP of a FWA Device SHALL comply to the Wi-Fi Alliance Easy Mesh™ R2 (or later) standard. |
| --- | --- |
| TS.64_3.5.10_REQ_002 | The FWA Device SHALL manage the connectivity of Wi-Fi clients and implement AP steering to guarantee that each client is always connected to the best AP and frequency band. |
| TS.64_3.5.10_REQ_003 | The AP of a FWA Device SHALL support IEEE 802.11r [88] industry standard to allow fast transition of clients between BSS (Basic Service Sets), |
| TS.64_3.5.10_REQ_004 | If supported, the 802.11r feature SHALL be configurable (ON/OFF) through customization. |

### Wi-Fi Diagnostics

| TS.64_3.5.11_REQ_001 | The FWA Device SHALL collect Wi-Fi metrics related to its AP and the connected clients as shown in Annex B. |
| --- | --- |
| TS.64_3.5.11_REQ_002 | Wi-Fi Diagnostics data SHALL be available to be collected from the FWA Device by means of TR-069. |
| TS.64_3.5.11_REQ_003 | Additional protocols for allowing the collection of Wi-Fi Diagnostics data from the network MAY be used. |
| TS.64_3.5.11_REQ_004 | The Wi-Fi Diagnostic solution SHALL collect data also from the other APs connected in mesh, as well as from the clients connected to those APs. |

### Wireless Multimedia Extension

| TS.64_3.5.12_REQ_001 | The AP of a FWA Device SHALL support Wi-Fi Alliance Wireless Multimedia Extensions™ (WME™), also known as Wi-Fi Multimedia™ (WMM™), in order to prioritize traffic in the Wireless Network according to Access Categories. |
| --- | --- |
| TS.64_3.5.12_REQ_002 | If WMM™ is supported, the FWA Device SHALL provide the mechanism to enable/disable the feature and to configure the mappings (Access Categories vs DSCP). |

### Customisation

| TS.64_3.5.13_REQ_001 | It SHALL be possible for a MNO to customize the settings of a FWA Device as regards Wi-Fi region/country of operation, enabled bands and channels, power transmission limits, SSIDs, passphrases. |
| --- | --- |
| TS.64_3.5.13_REQ_002 | It SHALL be possible for a MNO to customize a FWA Device in order to hide to the end-user any of the functionality detailed in section 3.5.5, 3.5.7 and 3.5.8. |
| TS.64_3.5.13_REQ_003 | It SHALL be possible for a MNO to customize a FWA Device in order to disable any of the functionality detailed in section 3.5.8, 3.5.9, 3.5.10, 3.5.11. |

## IDU/ODU Interworking and Resilience

### Common requirements to bridged and routed modes of operation

#### APN/VLAN/Service mapping

| TS.64_3.6.1.1_REQ_001 | The OutDoor Unit (ODU) in an Outdoor FWA Solution, SHALL be able to map the traffic received on each PDN Connection / PDU Session, on different VLANs over the interface with the InDoor Unit (IDU), and vice versa. |
| --- | --- |
| TS.64_3.6.1.1_REQ_002 | The IDU SHALL be able to map the different traffic generated by the IDU clients in LAN, and destined to the WAN, on different VLANs over the interface with the ODU, based on Service/VLAN mapping rules defined on the IDU. |
| TS.64_3.6.1.1_REQ_003 | IDU and ODU SHALL be able to manage at least 4 VLANs over the IDU-ODU interface. An example of service/VLAN mapping is: - IDU remote management – VLAN ‘a’ - Data/Internet traffic – VLAN ‘b’ - Voice (VoIP) – VLAN ‘c’ - Video – VLAN ‘d’ Other services may be defined based on MNO and market needs. |
| TS.64_3.6.1.1_REQ_004 | The IDU SHALL be able to manage Differentiated Services QoS Treatment, over the interface with the ODU, based on the Requirements TS.64_3.4.3_REQ_010 to TS.64_3.4.3_REQ_013. |
| TS.64_3.6.1.1_REQ_005 | The ODU SHALL be able to manage Differentiated Services QoS Treatment, over both the interface with the IDU and the interface with the mobile network, based on the Requirements TS.64_3.4.3_REQ_010 to TS.64_3.4.3_REQ_013. |

#### IDU-ODU Networking

| TS.64_3.6.1.2_REQ_001 | The ODU SHALL be configurable in order to operate on each VLAN, either in bridged mode or in routed mode. |
| --- | --- |
| TS.64_3.6.1.2_REQ_002 | The ODU SHALL allow to configure one PDN connection / PDU session to be locally terminated in the ODU itself, that is to operate in routed mode without being mapped on a VLAN with the IDU. NOTE: for example, this connection may be dedicated to ODU Remote Management. |
| TS.64_3.6.1.2_REQ_003 | When using DHCP to assign IP Address to IDU, the ODU SHALL define an IP Address Lease Time and configure it to the IDU by means of DHCP Option 51. |
| TS.64_3.6.1.2_REQ_004 | The ODU SHOULD define an IP Address Lease Time in the interval 1 hour (3600 seconds) - 12 hours (43200 seconds). |
| TS.64_3.6.1.2_REQ_005 | When using DHCP with the IDU, the ODU SHALL be able to provide the IDU with the Interface MTU parameter, by means of DHCP Option 26, over each VLAN, using the IPv4 Link MTU received from the Mobile Network on the corresponding PDN/PDU. |
| TS.64_3.6.1.2_REQ_006 | The ODU and the IDU SHALL use, on each VLAN, Address Resolution Protocol (ARP) or Neighbor Discovery Protocol for IPv6 (NDP) in order to properly maintain the association between IP Addresses and MAC Addresses, on each VLAN. |
| TS.64_3.6.1.2_REQ_007 | The ODU SHALL use a unique MAC Address per each VLAN; the same MAC Address is meant to be used, on a VLAN, both for data traffic exchange with the IDU and for ARP/NDP communications with the IDU. |
| TS.64_3.6.1.2_REQ_008 | The IDU SHALL use a unique MAC Address per each VLAN; the same MAC Address is meant to be used, on a VLAN, both for data traffic exchange with the ODU and for ARP/NDP communications with the ODU. |
| TS.64_3.6.1.2_REQ_009 | The ODU SHALL reply, on each VLAN, to ARP Requests generated by IDU, relating to the IP Address of the Default Gateway, with its own MAC Address dedicated to that VLAN. |
| TS.64_3.6.1.2_REQ_010 | The ODU and the IDU SHOULD support 802.1X authentication [111], where the IDU has the role of the Supplicant, and the ODU has the Role of the Authenticator. |
| TS.64_3.6.1.2_REQ_011 | For the purpose of 802.1X, the ODU MAY have also the role of local Authentication Server. |

### ODU Bridged mode operation.

| TS.64_3.6.2_REQ_001 | The OutDoor Unit (ODU) SHALL be able to operate in bridged mode, over one or more PDNs-PDUs/VLANs. |
| --- | --- |
| TS.64_3.6.2_REQ_002 | In bridged mode operation, the ODU SHALL use DHCP/DHCPv6 to assign to the IDU, on each VLAN, the network parameters received from the mobile network over a PDN/PDU: - IP Address - DNS Servers IP Addresses (DHCP Option 6) Therefore, in bridge mode operation, the IP Address received on each PDN/PDU from the network, is not retained on the ODU itself, but is assigned to the IDU on the VLAN corresponding to that PDN/PDU. |
| TS.64_3.6.2_REQ_003 | The ODU SHALL define, for each VLAN, the following parameters: - Subnet Mask - Default Gateway and assign them to the IDU by means of DHCP/DHCPv6. NOTE 1: this is needed because the mobile network does not provide a UE (specifically, the ODU) with such parameters over a PDN/PDU, while they are needed to properly configure the IDU with DHCP/DHCPv6. NOTE 2: Subnet Mask is DHCP Option 1, Default Gateway is DHCP Option 3 (Router). |
| TS.64_3.6.2_REQ_004 | The ODU MAY define: - For the Subnet Mask, a /30 (255.255.255.252) - For the Default Gateway, the IP Address immediately after or before the one assigned to the IDU on each VLAN, following the rules of the Classless Inter-Domain Routing (CIDR). |

### Reliability of IDU-ODU operation

| TS.64_3.6.3_REQ_001 | If an APN is configured in bridged mode, the ODU SHALL guarantee that the IDU IP configuration will always be the same of the WAN (mobile) IP configuration. |
| --- | --- |
| TS.64_3.6.3_REQ_002 | As soon as the WAN (mobile) IP connection state changes, the ODU SHALL trigger the IDU IP Address renewal by means of a reset of the physical interface with the IDU. |
| TS.64_3.6.3_REQ_003 | In addition to the reset of the physical interface with the IDU, the ODU MAY trigger the IDU IP Address renewal by using the DHCP “ForceRenew” message as specified in RFC 3203. |
| TS.64_3.6.3_REQ_004 | In case of Out-Of-Coverage condition of LTE/5G signal, when the ODU is back to coverage, it SHALL perform Tracking Area Update (TAU) until the relevant timer expires in order to recover its previous IP addresses. |

### ODU Routed mode operation.

| TS.64_3.6.4_REQ_001 | The OutDoor Unit (ODU) SHALL be able to operate in routed mode, over one or more PDNs-PDUs/VLANs. |
| --- | --- |
| TS.64_3.6.4_REQ_002 | In routed mode operation, the ODU SHALL retain for itself the IP Address received from the mobile network over a APN/DNN. |
| TS.64_3.6.4_REQ_003 | In routed mode operation, the ODU SHALL be able to configure the IP Address of the IDU, on each VLAN configured in routed mode, by means of DHCP, using a private IP address pool. |
| TS.64_3.6.4_REQ_004 | In routed mode operation, if DHCP is used, then ODU SHALL provide via DHCP also: - The DNS Server IP Address(es), which can be either the ODU itself or the Servers received from network; - The Default Gateway (Router), which is the IP Address of the ODU over the IDU-ODU connection. |
| TS.64_3.6.4_REQ_005 | In routed mode operation, the ODU SHALL be able to manage statically configured addresses for: - The IP Address of the IDU over the IDU-ODU connection: this is a directly connected interface - The LAN of the IDU: this will be a subnet routed through the IP Address of the IDU. NOTE: static IP Addressing, for the IDU-ODU connection, can be used as an alternative to DHCP. |
| TS.64_3.6.4_REQ_006 | In routed mode operation, the ODU SHALL perform NAT of traffic coming from the IDU and destined to the Network. |

### Tunnels/VPNs

| TS.64_3.6.5_REQ_001 | The IDU SHALL be able to establish, through the ODU, one or more Tunnels or VPN connections, based on IPSec or PPTP or GRE, over one or more VLANs, towards Tunnel/VPN Terminators in the network. |
| --- | --- |

## Device Management

For the proper maintenance of the FWA Devices it is crucial for operators to be able to manage them remotely through an application layer protocol.

For this purpose, TR-069 protocol provides the chance to execute Device management and monitoring operations such as read and write parameters, perform a firmware upgrade etc.

| TS.64_3.7_REQ_001 | The FWA Device SHALL support broadband forum (BBF) TR-069 Device WAN Management Protocol (Issue:6 corrigendum 1 CWMP Version 1.4 or later) [47]. |
| --- | --- |

All objects and parameters describing the many different functions and capabilities of the FWA Devices are hierarchically organised in a XML scheme called “data model”.

The BBF defines two types of CWMP data models:

- Root: used to describe the main functions necessary to CWMP (e.g. interfaces, SW/FW Diagnostics, basic Device information)

- Service: used to provide specific services (e.g. Voice, Set-Top-Box).

For each of service please refer to the following BBF technical reports:

- TR-135: Data Model for a TR-069 Enabled STB (Set-Top-Boxes), Issue 1, Amendment 3 [48]

- TR-104: Provisioning Parameters for VoIP FWA Device, Issue 2 [49]

- TR-140: TR-069 Data Model for Storage Service Enabled Devices, Issue 1, Amendment 3 [50]

- TR-196: Femto Access Point Service Data Model, Issue 2 [51]

### Common Requirements for IDU and ODU

In this chapter a set of common requirements for IDU and ODU FWA Devices has been identified and organised in five main sections:

- RPC methods

- Data model structure

- Security

- Performance monitoring

- Data model parameters

This is a GSMA minimum set of requirements and then MNOs can add extra metrics according to their needs.

#### RPC methods

The technical report TR-069 FWA Device WAN Management Protocol (Issue:6 corrigendum 1 CWMP Version 1.4) provides a summary of all required RPC methods.

| TS.64_3.7.1.1_REQ_001 | The FWA Device SHALL support the following FWA Device RPC Methods, as reported in TR-069 (Issue:6 corrigendum 1 CWMP Version 1.4) technical report, section 3.6 [47]: - GetRPCMethods - SetParameterValues - GetParameterValues - GetParameterNames - SetParameterAttributes - GetParameterAttributes - AddObject - DeleteObject - Reboot - Download |
| --- | --- |
| TS.64_3.7.1.1_REQ_002 | The FWA Device SHALL support the following ACS RPC Methods, as reported in TR-069 (Issue:6 corrigendum 1 CWMP Version 1.4) technical report, section 3.6 [47]: - Inform - TransferComplete |
| TS.64_3.7.1.1_REQ_003 | The FWA Device SHOULD support the following FWA Device RPC Methods: - Upload - FactoryReset |

#### Data model structure

Regarding the CWMP data model structure, two types of root data models have been defined:

- TR-098: Internet Gateway Device Data Model for TR-069 [52]. The data model defined in this specification is DEPRECATED since proved to be inflexible and caused problems in representing complex Device configurations; accordingly, it should be used only by legacy Devices.

- TR-181: Device Data Model for TR-069 [53]. This technical report covers the same functionality of TR-098 plus several extensions as well as IPv6 support and interface stacking mechanism.

| TS.64_3.7.1.2_REQ_001 | For all FWA Devices and upgrades of existing Devices the “Device:2” data model defined in TR-181 Issue 2 [53] SHALL be used. |
| --- | --- |

: Device:2 Data Model Structure – Overview

#### Security

| TS.64_3.7.1.3_REQ_001 | The FWA Device SHALL support TLS 1.2 [54] and any earlier versions that are still valid in standards. |
| --- | --- |
| TS.64_3.7.1.3_REQ_002 | TLS versions later than v1.2 MAY be supported. |
| TS.64_3.7.1.3_REQ_004 | The FWA Device SHALL support the TLS_RSA_WITH_AES_128_CBC_SHA cipher suite. |
| TS.64_3.7.1.3_REQ_005 | Signalling request from the device to the ACS SHALL indicate the latest version of TLS that the device supports. |
| TS.64_3.7.1.3_REQ_006 | The FWA Device SHALL fall back to an earlier version of TLS if that is requested by the ACS. |
| TS.64_3.7.1.3_REQ_007 | The FWA Device SHALL support the mandatory cipher suites for all supported TLS versions. |

#### Performance monitoring

| TS.64_3.7.1.4_REQ_001 | In order to measure the Device performance in terms of downlink and uplink throughput and latency, the FWA Device SHALL be compliant to BBF TR-143 [55] technical report. |
| --- | --- |
| TS.64_3.7.1.4_REQ_002 | The FWA Device SHALL support both FWA Device initiated and Network Initiated diagnostics TR-143 approaches. |
| TS.64_3.7.1.4_REQ_003 | The FWA Device SHALL support both size-based and time-based TR-143 throughput testing approaches. |
| TS.64_3.7.1.4_REQ_004 | In order to increase the rate that a specific link can be tested, the FWA Device SHOULD support TCP multi-thread for TR-143 downlink and uplink throughput test. |
| TS.64_3.7.1.4_REQ_005 | The FWA Device SHALL support both FTP and HTTP transport for Download and Upload diagnostics. |
| TS.64_3.7.1.4_REQ_006 | The FWA Device SHALL support UDP Echo Plus methodology for latency and packet loss evaluation (One Way Packet Loss, Round Trip Delay, One Way IP Packet Delay Variation). |
| TS.64_3.7.1.4_REQ_007 | The FWA Device SHOULD support TR-390 for Performance Measurement from IP Edge to Customer Equipment using TWAMP Light [56]. |

#### Data model parameters

This section presents a baseline data model. Note that parameters will only be used if they are relevant to the operator's service offering.

| TS.64_3.7.1.5_REQ_001 | The Data model of the FWA Device SHALL contain a set of data object concerning the ACS parameter: - ACS Url - ACS Username - ACS Password - Connection Request URL - Connection Request Username - Connection Request Password - Periodic Inform Interval |
| --- | --- |
| TS.64_3.7.1.5_REQ_002 | The Data model of the FWA Device SHALL contain a set of data object concerning APNs info, making a clear distinction between the various APN types (Management, Data, Voice): - APN Name - IP Address and Subnet Mask - IP stack (IPv4, IPv6, IPv4v6) - APN Authentication Type - APN Username - APN Password |
| TS.64_3.7.1.5_REQ_003 | The Data model of the FWA Device SHALL contain a set of data object concerning FWA Device info: - Serial Number - IMEI - Software/Firmware Version |
| TS.64_3.7.1.5_REQ_004 | The Data model of the FWA Device SHALL contain a set of data object concerning SIM info: - MSISDN - IMSI - ICCID |
| TS.64_3.7.1.5_REQ_005 | The Data model of the FWA Device SHALL contain a set of data object concerning network info: - PLMN ID - Global Cell ID - TAC - PCI |
| TS.64_3.7.1.5_REQ_006 | The Data model of the FWA Device SHALL contain a set of data object concerning radio condition, making a clear distinction between 4G and 5G NSA/SA parameter: - Connected bands - RSRP - RSRQ - SINR |
| TS.64_3.7.1.5_REQ_007 | If the FWA Device supports VoIP, the Data model SHALL contain a set of data object concerning voice service, as specified in TS.64_3.3.2_REQ_043. |
| TS.64_3.7.1.5_REQ_008 | If the FWA Device supports Voice service (VoIP or VoLTE), the Data model SHALL contain the status of SIP registration. |
| TS.64_3.7.1.5_REQ_009 | If the FWA Device supports Wi-Fi access point, the Data model SHALL contain a set of data object concerning Wi-Fi service: - SSID - Channel - Password |
| TS.64_3.7.1.5_REQ_010 | The Data model of the FWA Device SHALL contain a set of data object concerning networking (eg. VLAN ID). |
| TS.64_3.7.1.5_REQ_011 | The Data model of the FWA Device SHALL contain a set of data object concerning Devices connected to the FWA Device, both via Wi-Fi or Ethernet: - Host name - Connected IP Address - MAC Address |
| TS.64_3.7.1.5_REQ_012 | The Data model of the FWA Device SHALL contain a set of data object concerning TR-143 Diagnostic Parameter: - Download URL - Interface - Ethernet Priority - DSCP - BOM Time - EOM Time - Test Bytes Received - Diagnostic Max Connection for multi-thread connections |
| TS.64_3.7.1.5_REQ_013 | The Data model of the FWA Device SHALL contain a set of data object concerning NTP service. |

### Technical Adaptation of FWA Device

| TS.64_3.7.2_REQ_001 | If the FWA Device implements autoconfiguration based on SIM insertion, this SHOULD be able to use the available settings as defined in GSMA TS.32 [96]. |
| --- | --- |

## Security

| TS.64_3.8_REQ_001 | To prevent attacks on secured connections and on the FWA itself, the FWA Device in factory condition setting SHALL have cryptographic keys and password that are unique, random, uncorrelated between passwords themselves and between passwords and serial numbers. |
| --- | --- |
| TS.64_3.8_REQ_002 | To be able to react to newly appearing exploits of soft- or hardware vulnerabilities of the FWA Device or any of its components, the FWA Device SHALL have a functionality to update the firmware (operating system and applications) using a firmware package. |
| TS.64_3.8_REQ_003 | The FWA Device SHALL support remote firmware upgrade via the WAN connection. |
| TS.64_3.8_REQ_004 | The FWA Device SHALL be able to verify the authenticity of the firmware package before it is installed on the FWA Device |
| TS.64_3.8_REQ_005 | The FWA Device SHALL NOT install an unsigned firmware package |
| TS.64_3.8_REQ_006 | The FWA Device SHALL support a Factory Reset feature, deleting all personal data and end-user setting from the FWA Device. |
| TS.64_3.8_REQ_007 | The FWA Device SHALL provide at least two different methods to the end-user to request the factory reset, e.g. Web UI and pinhole/reset button on the case of the FWA Device. |
| TS.64_3.8_REQ_008 | The FWA Device SHALL expose, on any interface, a defined and declared set of services (see section 2.1), identified by specific TCP and/or UDP ports. |
| TS.64_3.8_REQ_009 | The FWA Device SHALL allow the customization of the active services and corresponding open ports, based on MNOs requirements. |
| TS.64_3.8_REQ_010 | Any other port on any interface of the FWA Device, out of those defined by MNO custom requirements, SHALL be closed at all times. |
| TS.64_3.8_REQ_011 | If the FWA Device has a serial interface to access the Device, it SHALL be closed by default, unless otherwise required by the MNO. |

### Passwords

| TS.64_3.8.1_REQ_001 | Access to the FWA Device configuration SHALL be secured by a password, initially when new, after user customization and after factory reset. |
| --- | --- |
| TS.64_3.8.1_REQ_002 | The factory preset password SHALL NOT contain information derived from data or parts of data related to the FWA Device itself (e.g. manufacturer, model name, MAC address, IMEI, …) |
| TS.64_3.8.1_REQ_003 | If the FWA Device does not implement a forced password customization at the first access of the user to the Web UI, the preset password SHALL be unique for each device of the same manufacturer. |
| TS.64_3.8.1_REQ_004 | Preset passwords and customized passwords SHALL fulfil the following requirements: - contain at least 8 characters (12+ recommended) - contain at least: - one uppercase letter [A-Z] - one lowercase letter [a-z] - one special character [e.g. ?, !, $, etc.] - • one numeric character [0-9] |
| TS.64_3.8.1_REQ_005 | Different security requirements for passwords MAY be defined, based on MNO requirements. |
| TS.64_3.8.1_REQ_006 | The FWA Device SHALL allow an authenticated end-user to change the password after entering the previous password. |
| TS.64_3.8.1_REQ_007 | The FWA Device SHALL prevent the user from entering a weak password. NOTE: A password such as Abcd1234 whilst compliant to TS.64_3.8.1_REQ_004 can still be weak. |
| TS.64_3.8.1_REQ_008 | Password based authentication SHALL be protected against brute force attacks. NOTE: A suitable solution is reducing the amount of login attempts in a certain time span (e.g. tarpit) or equivalent techniques. |
| TS.64_3.8.1_REQ_009 | After authentication, the session of the authenticated end-user SHALL be protected against session hijacking attacks. NOTE: Example protection mechanisms are a session time out and the use of a CSRF token. |
| TS.64_3.8.1_REQ_009.1 | If a session timeout is supported, the session timeout SHALL be customizable based on MNO requirement. |
| TS.64_3.8.1_REQ_009.2 | If a session timeout is supported, the session timeout SHALL have a default value of 30 minutes, |
| TS.64_3.8.1_REQ_010 | If the FWA device supports SAMBA or DLNA, the end-user SHALL be capable of setting a password to protect access to media devices connected to the FWA device. |

### Web UI security requirements

| TS.64_3.8.2_REQ_001 | The FWA Device SHALL NOT have, in factory reset condition, accounts undocumented to the end-user. |
| --- | --- |
| TS.64_3.8.2_REQ_001.1 | The manufacturer SHALL provide a formal declaration about absence of hidden accounts in the FWA Device. |
| TS.64_3.8.2_REQ_002 | In factory setting condition, the FWA Device SHALL allow end-user access to the configuration only using the LAN or Wi-Fi interface. |
| TS.64_3.8.2_REQ_003 | To maintain a secure operation, the FWA Device SHALL provide the necessary security relevant information to the authenticated end-user. |
| TS.64_3.8.2_REQ_004 | The FWA Device MAY offer to the end-user remote access to the configuration via WAN interface, e.g. through a Web server. |
| TS.64_3.8.2_REQ_005 | IF the FWA Device offers remote configuration, this communication SHALL be encrypted using strong protocols and cipher suites. |
| TS.64_3.8.2_REQ_006 | IF the FWA Device offers remote configuration, the status of this functionality (active/ inactive) SHALL be made available to the end-user. |
| TS.64_3.8.2_REQ_007 | IF the FWA Device offers remote configuration, the status of this functionality SHALL be disabled in factory setting condition. |
| TS.64_3.8.2_REQ_008 | If the FWA Device provides an option to save and/or restore the configuration to a file, this function SHALL be available only to successfully authenticated users. |
| TS.64_3.8.2_REQ_009 | Credentials and security-sensitive data SHALL be stored securely in the FWA Device. |
| TS.64_3.8.2_REQ_010 | Hard-coded credentials in device software SHALL NOT be used. |
| TS.64_3.8.2_REQ_011 | If the FWA Device provides an option to save and/or restore the configuration to a file, its content SHALL be encrypted. |
| TS.64_3.8.2_REQ_012 | The FWA Device SHALL allow the end-user to display the version number of the firmware currently installed on the Device. |

## User Data Protection and Privacy

In the provision of the FWA service, the FWA Device processes user personal data, for which privacy requirements apply.

Such data fall into some categories:

- Data essential for the basic services of the FWA Device: for example, the MAC Addresses of the hosts connected in LAN to the FWA Device are needed in order to properly route data traffic to/from these hosts.

- Data useful to provide additional services of the FWA Device: for example, the call log, that is the list of received, placed and missed voice calls, may be a useful service provided by the FWA Device, although it is based on sensitive data.

- Data useful to provide additional services offered by the MNO using a combination of features local to the FWA Device and remotely located in the network: for example, MAC Addresses and hostnames may be used by Wi-Fi diagnostics applications, to provide the end-user hints and advice to optimize the Wi-Fi performance in the home network.

| TS.64_3.9_REQ_001 | The nature and amount of user personal data being stored, processed and transmitted by the FWA Device SHALL be clearly identified and declared. |
| --- | --- |
| TS.64_3.9_REQ_002 | Personal data SHALL be processed by the FWA Device in compliance with the Data Protection and Privacy regulations in effect in the country where the FWA Device is deployed. |
| TS.64_3.9_REQ_003 | Personal data SHALL be stored in the FWA Device ONLY IF permitted by the Data Protection and Privacy regulations in effect in the country where the FWA Device is deployed and following the mandatory prescriptions. NOTE: for example, a regulator might require anonymised data to be presented, e.g. anonymizing the last digits of a phone number or a significant portion of a hostname/MAC Address. |
| TS.64_3.9_REQ_004 | If permitted, the storage of personal data onboard the FWA Device SHALL be done in an encrypted manner. |
| TS.64_3.9_REQ_005 | Personal data in the FWA Device SHALL be presented to the end user only upon authenticated access and by means of secure protocols. |
| TS.64_3.9_REQ_006 | VOID. |
| TS.64_3.9_REQ_007 | Personal data in the FWA Device SHALL NOT be shared with other parties other than the MNO providing the service. |
| TS.64_3.9_REQ_008 | If permitted, the transmission of personal data SHALL only occur via authenticated sessions and encrypted protocols between the FWA Device and the MNO network. NOTE: Permitted data as defined by the Regulator where the FWA Device is sold. |

## Hardware, Safety, EMC requirements and environment operating conditions

### General

| TS.64_3.10.1_REQ_001 | The FWA Device SHALL comply to the safety, health and environmental protection regulations of the market where it is meant to be used. |
| --- | --- |
| TS.64_3.10.1_REQ_002 | The FWA Device SHALL comply to the ElectroMagnetic Compatibility (EMC) regulations of the market where it is meant to be used. |
| TS.64_3.10.1_REQ_003 | The FWA Device SHALL comply to the eco-design and energy efficiency regulations of the market where it is meant to be used. |
| TS.64_3.10.1_REQ_004 | The FWA Device SHALL comply to the restrictions of use of hazardous materials and waste management regulations of the market where it is meant to be used. |
| TS.64_3.10.1_REQ_005 | For the Indoor FWA Device and the InDoor Unit of the Outdoor FWA Solution, the maximum allowed temperature on the different parts of the enclosure, in any environmental condition during stationary use, shall be not higher than 55°C. |
| TS.64_3.10.1_REQ_006 | The FWA Device SHALL comply to the noise emissions regulations of the market where it is meant to be used. |
| TS.64_3.10.1_REQ_007 | Regarding the insulation against water and dust, the OutDoor Unit of the Outdoor FWA Solution SHALL be compliant at least to the class IP65 of the international standard IEC 60529 [89]. |
| TS.64_3.10.1_REQ_008 | Regarding the electrical resistibility, the Indoor FWA Device and the InDoor Unit of the Outdoor FWA Solution SHALL comply with basic requirements defined by Rec. ITU-T K.21 [90] (Resistibility of telecommunication equipment installed in customer premises to overvoltages and overcurrents) and K.44 [91] (Resistibility tests for telecommunication equipment exposed to overvoltages and overcurrents – Basic Recommendation). |
| TS.64_3.10.1_REQ_009 | Regarding the electrical resistibility, the OutDoor Unit of the Outdoor FWA Solution, including the external power supply, SHALL comply with basic requirements defined by Rec. ITU-T K.21 [90] (Resistibility of telecommunication equipment installed in customer premises to overvoltages and overcurrents), K.44 [91] (Resistibility tests for telecommunication equipment exposed to overvoltages and overcurrents – Basic Recommendation) and K.45 [92] (Resistibility of telecommunication equipment installed in the access and trunk networks to overvoltages and overcurrents). |
| TS.64_3.10.1_REQ_010 | The FWA Device SHALL have a MTBF (Mean Time Between Failure) not shorter than 7 years at 30 °C. |
| TS.64_3.10.1_REQ_011 | In countries where there’re no safety requirements, the FWA Device SHOULD comply with the requirements mandatory for the CE marking. |

NOTE 1: examples or mandatory regulatory requirements for some markets:

- CE Marking for the operation in the Economic European Area (EEA).

- FCC regulatory requirements for the operation in the US market.

NOTE 2: some operators may require additional certifications.

NOTE 3: See Annex A for detailed list of European norms to be fulfilled in EU for EMC, Safety & Radio aspects.

## Stability

| TS.64_3.11_REQ_001 | In case of loss of power, when the power is restored the FWA Device SHALL return automatically to the operational state, with all services (e.g., data, voice) restored according to the configuration of the device prior the power interruption. |
| --- | --- |
| TS.64_3.11_REQ_002 | In case of loss of radio signal(s), when the radio signal is restored the FWA Device SHALL return automatically to the operational state, with all services (e.g., data, voice) restored according to the configuration of the device prior the radio signal interruption. |
| TS.64_3.11_REQ_003 | For data service, in normal operating conditions, the FWA Device SHALL offer a service availability equal or greater than 99.95%. NOTE: this objective considers only the availability of the Device itself, not the availability of the network. |
| TS.64_3.11_REQ_004 | In normal operating conditions, the FWA Device SHALL offer a service availability for voice service of at least 99.5%, maintaining the mandatory voice quality targets defined in sect. 3.3.6. NOTE: this objective considers only the availability of the Device itself, not the availability of the network. |
| TS.64_3.11_REQ_005 | The FWA Device SHALL maintain uninterrupted voice (SIP protocol) registration for at least 72 consecutive hours, during which the Device is idle for Voice. |
| TS.64_3.11_REQ_006 | At the end of the idle period of req. TS.64_3.11.0_REQ_005, the FWA Device SHALL be able to receive and make phone calls reliably, as well as to transmit and receive IP data user packets reliably. |
| TS.64_3.11_REQ_007 | If voice service is supported, the FWA Device SHALL support long-lasting voice calls (1.5 hours at least), maintaining the mandatory voice quality targets defined in sect. 3.3.6. |
| TS.64_3.11_REQ_008 | As regards the performance targets detailed in sect. 3.4.2 and 3.5.3, with reference to 3GPP TS 36.101 [93] (for 4G), TS 38.101-1 [94] (5G FR1) and TS 38.101-2 [95] (5G FR2), the Indoor FWA Device and the InDoor Unit of the Outdoor FWA Solution SHALL offer: - 90% of each of the targets, in the “normal conditions” temperature range. |
| TS.64_3.11_REQ_009 | As regards the performance targets detailed in sect. 3.4.2 and 3.5.3, with reference to 3GPP TS 36.101 [93] (for 4G), TS 38.101-1 [94] (5G FR1) and TS 38.101-2 [95] (5G FR2), the Indoor FWA Device and the InDoor Unit of the Outdoor FWA Solution SHALL offer: - At least 75% of each of the targets, in “extreme conditions” temperature range (outside the “normal condition” range). |
| TS.64_3.11_REQ_010 | As regards the performance targets detailed in sect. 3.4.2, with reference 3GPP TS 36.101 [93] (for 4G), TS 38.101-1 [94] (5G FR1) and TS 38.101-2 [95] (5G FR2), the OutDoor Unit of the Outdoor FWA Solution SHALL offer: - 75% of each of the targets, in the “extreme conditions” temperature range. |

## User Interface

| TS.64_3.12_REQ_001 | The FWA Device SHALL offer a Web UI to the end user for customizing the configuration of the FWA Device. |
| --- | --- |
| TS.64_3.12_REQ_002 | The FWA Device SHALL comply to the requirements defined in sect. 3.8.2 as regards Web UI Security. |
| TS.64_3.12_REQ_003 | The Web UI SHOULD permit the configuration of all the service features relevant for the end user. |
| TS.64_3.12_REQ_004 | The Web UI SHALL be customizable based on MNO requirements. |

# Specific Requirements for 4G FWA Devices

## Radio/RRC/NAS specific requirements for 4G FWA devices

| TS.64_4.1_REQ_001 | The FWA device SHALL be compliant with 3GPP E-UTRAN Access Stratum Release 12 baseline or later. |
| --- | --- |
| TS.64_4.1_REQ_002 | The FWA device SHALL support EEA0 Null Encryption Algorithm. |
| TS.64_4.1_REQ_003 | The FWA device SHALL support 128-EEA1 EPS Encryption Algorithm (based on SNOW 3G algorithm) for both RRC signalling ciphering and User Plane ciphering. |
| TS.64_4.1_REQ_004 | The FWA device SHALL support 128-EEA2 EPS Encryption Algorithm (based on AES algorithm) for both RRC signalling ciphering and User Plane ciphering. |
| TS.64_4.1_REQ_005 | The FWA device SHOULD support 128-EEA3 EPS Encryption Algorithm (based on ZUC algorithm) for both RRC signalling ciphering and User Plane ciphering. |
| TS.64_4.1_REQ_006 | The FWA device SHALL support EIA0 Null Integrity Algorithm. |
| TS.64_4.1_REQ_007 | The FWA device SHALL support 128-EIA1 EPS Integrity Algorithm (based on SNOW 3G algorithm) for both RRC and NAS signalling integrity protection. |
| TS.64_4.1_REQ_008 | The FWA device SHALL support 128-EIA2 EPS Integrity Algorithm (based on AES algorithm) for both RRC and NAS signalling integrity protection. |
| TS.64_4.1_REQ_009 | The FWA device SHOULD support 128-EIA3 EPS Integrity Algorithm (based on ZUC algorithm) for both RRC and NAS signalling integrity protection. |
| TS.64_4.1_REQ_010 | The FWA device SHALL support EPS-AKA Authentication Protocol. |
| TS.64_4.1_REQ_011 | In order to support the transmission techniques reported above, the FWA device SHALL support ue-CategoryDL 11 and ue-CategoryUL 5 or higher and all fallback configurations foreseen by the standard. |
| TS.64_4.1_REQ_012 | The FWA device SHOULD support ue-CategoryDL 12 and ue-CategoryUL 13 (Uplink CA support) or higher and all fallback configurations foreseen by the standard. NOTE: if the FWA device supports this requirement, then TS.64_4.1_REQ_001 requirement does not apply. |
| TS.64_4.1_REQ_013 | The FWA device SHALL support at least 3DL LTE Carrier Aggregation capability. |
| TS.64_4.1_REQ_014 | The FWA device SHOULD support 2UL LTE Carrier Aggregation capability. |
| TS.64_4.1_REQ_015 | The FWA device SHALL support MIMO 4x4 capability at least on one LTE mid-band (e.g., LTE B3 for Europe/Asia or B2 for US). |
| TS.64_4.1_REQ_016 | The FWA device SHALL support 256QAM modulation for downlink. |
| TS.64_4.1_REQ_017 | The FWA device SHALL support 64QAM modulation for uplink. |
| TS.64_4.1_REQ_018 | The FWA device SHOULD support 256QAM modulation for uplink. |
| TS.64_4.1_REQ_019 | The FWA device SHALL support standardized QCIs as specified in 3GPP TS 23.203 [5]. |
| TS.64_4.1_REQ_020 | The FWA device SHOULD support operator specific QCIs as specified in 3GPP TS 23.203 [5]. |
| TS.64_4.1_REQ_021 | The FWA device SHALL support periodical intra-frequency ANR measurements for reporting to the network the Strongest Cells and related CGI (Cell Global Identity). |
| TS.64_4.1_REQ_022 | The FWA device SHALL support periodical inter-frequency ANR measurements for reporting to the network the Strongest Cells and related CGI (Cell Global Identity). |

## Antenna Performance Acceptance Values for 4G FWA devices

As described in GSMA TS.24 [57], the GSMA Terminal Steering Group have reviewed results of antenna performance tests, aligning test methods and performance values to be used as guidelines for acceptable performance of antennas in Mobile devices to facilitate alignment and agreement among the various operators representing America, Europe, and Asia.

| TS.64_4.2_REQ_001 | The FWA device SHALL be compliant to TS.24 for Antenna Performance acceptance values [57]. |
| --- | --- |

GSMA TS.24 (Operator Acceptance Values for Device Antenna Performance) version 6 onwards has specific focus on FWA Devices, as they have different form factor which may improve Antenna performance.

# Specific Requirements for 5G NSA FWA Devices

| TS.64_5_REQ_001 | The FWA device SHALL support standardized QCIs as specified in 3GPP TS 23.203 [5]. |
| --- | --- |
| TS.64_5_REQ_002 | The FWA device SHOULD support operator-specific QCIs as specified in 3GPP TS 23.203 [5]. |
| TS.64_5_REQ_003 | The FWA device SHALL be compliant with 3GPP E-UTRAN and NR Access Stratum Release 16 baseline or later. |
| TS.64_5_REQ_004 | The FWA device SHALL support periodical ANR measurements for reporting via the 4G network the Strongest NR Cells and related CGI (Cell Global Identity) when in ENDC operation. (reportCGI-NR-EN-DC-r15) (nr-CGI-Reporting-ENDC) |
| TS.64_5_REQ_005 | The FWA device SHOULD support periodical inter-RAT ANR measurements for reporting via the 4G network the Strongest NR Cells and related CGI (Cell Global Identity) when not in ENDC operation. (reportCGI-NR-NoEN-DC-r15) |
| TS.64_5_REQ_006 | The FWA device SHALL support periodical inter-RAT ANR measurements for reporting via the NR network the Strongest 4G Cells and related CGI (Cell Global Identity). (eutra-CGI-Reporting) |
| TS.64_5_REQ_007 | The FWA device SHALL support periodical intra-RAT ANR measurements for reporting via the NR network the Strongest NR Cells and related CGI (Cell Global Identity). (nr-CGI-Reporting) |

## Radio/RRC/NAS specific requirements for 5G-FR1 NSA FWA devices.

| TS.64_5.1_REQ_001 | The FWA device SHALL support EN-DC (Option 3x). |
| --- | --- |
| TS.64_5.1_REQ_002 | The FWA device SHALL support DSS technology. |
| TS.64_5.1_REQ_003 | The FWA device SHALL support rateMatchingResrcSetSemi-Static Information element in the Capability Information message. NOTE: It indicates the UE supports receiving PDSCH with resource mapping that excludes the REs corresponding to resource sets configured with RB-symbol level granularity following the semi-static configuration as specified in TS 38.214. |
| TS.64_5.1_REQ_004 | The FWA device SHALL support rateMatchingResrcSetDynamic Information element in the Capability Information message. NOTE: It indicates the UE supports receiving PDSCH with resource mapping that excludes the REs corresponding to resource sets configured with RB-symbol level granularity based on dynamic indication in the scheduling DCI as specified in TS 38.214. |
| TS.64_5.1_REQ_005 | The FWA device SHALL support rateMatchingLTE-CRS Information element at least for one FDD mid-band (e.g. n1,n3) in the Capability Information message. NOTE: It indicates the UE supports receiving PDSCH with resource mapping that excludes the REs determined by the higher layer configuration LTE-carrier configuring common RS, as specified in TS 38.214. |
| TS.64_5.1_REQ_006 | The FWA device SHOULD support AdditionalDMRS-DL-Alt Information element in the Capability Information message. NOTE: It indicates the support of the alternative additional DMRS position for coexistence with LTE CRS. |
| TS.64_5.1_REQ_007 | The FWA device SHALL support NR SRS antenna switching 1T4R in 5G NR TDD high-bands (e.g. n77/n78). |
| TS.64_5.1_REQ_008 | The FWA device SHALL support NR SRS antenna switching 1T2R in 5G NR TDD mid- and low-bands. |
| TS.64_5.1_REQ_009 | The FWA device SHOULD support 2DL NR Inter-Band Carrier Aggregation. |
| TS.64_5.1_REQ_010 | The FWA device SHALL support UL split bearer to transmit concurrently on LTE and NR. |
| TS.64_5.1_REQ_011 | The FWA device SHALL support MIMO 4x4 DL capability on NR mid-bands (e.g. NR bands n77/n78). |
| TS.64_5.1_REQ_012 | The FWA device SHALL support 4Rx diversity on NR bands. |
| TS.64_5.1_REQ_013 | The FWA device SHOULD support 8Rx diversity on NR bands. |
| TS.64_5.1_REQ_014 | The FWA device MAY support more than 8Rx diversity on NR bands. |
| TS.64_5.1_REQ_015 | The FWA device SHALL support 256QAM modulation for downlink. |
| TS.64_5.1_REQ_016 | The FWA device SHALL support 64QAM modulation for uplink. |
| TS.64_5.1_REQ_017 | The FWA device SHOULD support 256QAM modulation for uplink. |
| TS.64_5.1_REQ_018 | The FWA device SHALL support power class 3 (23 dBm). |
| TS.64_5.1_REQ_019 | The FWA device SHOULD support power class 2 (26 dBm) in compliance with 3GPP TS 38.101-1 [58]. |
| TS.64_5.1_REQ_020 | The FWA device SHOULD support power class 1.5 (29 dBm) in compliance with 3GPP TS 38.101-1 [58]. |
| TS.64_5.1_REQ_021 | The FWA device SHALL support 15 kHz Sub-Carrier Spacing in FR1 NR bands. |
| TS.64_5.1_REQ_022 | The FWA device SHALL support 30 kHz Sub-Carrier Spacing in FR1 NR bands. |

## Radio/RRC/NAS specific requirements for 5G-FR2 NSA FWA devices

If the Device supports 5G FR2, the following requirements apply.

| TS.64_5.2_REQ_001 | The FWA device SHALL support EN-DC (Option 3x). |
| --- | --- |
| TS.64_5.2_REQ_002 | The FWA device SHALL support 2DL contiguous NR Carrier Aggregation. |
| TS.64_5.2_REQ_003 | The FWA device SHOULD support 2UL contiguous NR Carrier Aggregation. |
| TS.64_5.2_REQ_004 | The FWA device SHALL support UL split bearer to transmit concurrently on LTE and NR. |
| TS.64_5.2_REQ_005 | The FWA device SHALL support MIMO 2x2 DL capability on NR FR2 bands (e.g., NR bands n257/n258). |
| TS.64_5.2_REQ_006 | The FWA device SHOULD support MIMO 4x4 DL capability on NR FR2 bands (e.g., NR bands n257/n258). |
| TS.64_5.2_REQ_007 | The FWA device SHALL support 64QAM modulation for downlink. |
| TS.64_5.2_REQ_008 | The FWA device SHOULD support 256QAM modulation for downlink. |
| TS.64_5.2_REQ_009 | The FWA device SHALL support 64QAM modulation for uplink. |
| TS.64_5.2_REQ_010 | The FWA device SHOULD support 256QAM modulation for uplink. |
| TS.64_5.2_REQ_011 | The FWA device SHALL support power class 3 (23 dBm). |
| TS.64_5.2_REQ_012 | The FWA device SHOULD support power class 2 (26 dBm). |
| TS.64_5.2_REQ_013 | The FWA device SHOULD support power class 1 (31 dBm). |
| TS.64_5.2_REQ_014 | The FWA device SHALL support 100 MHz channel bandwidth in FR2 NR TDD bands (e.g., n257/n258). |
| TS.64_5.2_REQ_015 | The FWA device SHOULD support 200 MHz channel bandwidth in FR2 NR TDD bands (e.g., n257/n258). |
| TS.64_5.2_REQ_016 | The FWA device SHOULD support 400 MHz channel bandwidth in FR2 NR TDD bands (e.g., n257/n258). |
| TS.64_5.2_REQ_017 | The FWA device SHALL support 60 KHz Sub-Carrier Spacing in FR2 NR TDD bands (e.g., n257/n258). |
| TS.64_5.2_REQ_018 | The FWA device SHALL support 120 KHz Sub-Carrier Spacing in FR2 NR TDD bands (e.g., n257/n258). |

## Antenna Performance Acceptance Values for 5G NSA FWA devices

| TS.64_5.3_REQ_001 | The FWA device SHALL be compliant to TS.24 [57] for Antenna Performance acceptance values. |
| --- | --- |

# Specific Requirements for 5G SA FWA Devices

| TS.64_6_REQ_001 | The FWA device SHALL support Option 2 SA deployment option. |
| --- | --- |
| TS.64_6_REQ_002 | The FWA device SHOULD support Option 4 NSA deployment option. |
| TS.64_6_REQ_003 | The FWA device SHALL be compliant with 3GPP NR Access Stratum Release 16 baseline or later. |
| TS.64_6_REQ_004 | The FWA device SHALL support standardized 5QIs as specified in 3GPP TS 23.501[81]. |
| TS.64_6_REQ_005 | The FWA device SHALL support NEA0 Null Encryption Algorithm. |
| TS.64_6_REQ_006 | The FWA device SHALL support 128-NEA1 New radio Encryption Algorithm (based on SNOW 3G algorithm). |
| TS.64_6_REQ_007 | The FWA device SHALL support 128-NEA2 New radio Encryption Algorithm (based on AES algorithm). |
| TS.64_6_REQ_008 | The FWA device SHOULD support 128-NEA3 New radio Encryption Algorithm (based on ZUC algorithm). |
| TS.64_6_REQ_009 | The FWA device SHALL support 128-NIA1 New radio Integrity Algorithm (based on SNOW 3G algorithm). |
| TS.64_6_REQ_010 | The FWA device SHALL support 128-NIA2 New radio Integrity Algorithm (based on AES algorithm). |
| TS.64_6_REQ_011 | The FWA device SHOULD support 128-NIA3 New radio Integrity Algorithm (based on ZUC algorithm). |
| TS.64_6_REQ_012 | The FWA device SHALL support 5G-AKA Authentication Protocol Algorithm. |
| TS.64_6_REQ_013 | The FWA device SHALL support EAP-AKA Authentication Protocol Algorithm. |
| TS.64_6_REQ_014 | The FWA device SHALL support Initial 5GC Registration with SUCI, as per 3GPP TS 24.501 [82]. |
| TS.64_6_REQ_015 | The FWA device SHALL support 5G Slicing User Equipment Route Selection Policy (URSP) parameters. |
| TS.64_6_REQ_016 | The FWA device SHALL support 5G Slicing Network Slice Selection Assistance Information (NSSAI) parameters, as per 3GPP TS 24.501 [82]. |
| TS.64_6_REQ_017 | The FWA device SHALL support SST (Slice/Service Type) and SD (Slice Differentiator) parameters. |
| TS.64_6_REQ_018 | The FWA device SHALL support standardised SST values, as specified in 3GPP TS 23.501 [81]. |

## Radio/RRC/NAS specific requirements for 5G-FR1 SA FWA devices

| TS.64_6.1_REQ_001 | The FWA device SHALL support 2DL NR Carrier Aggregation. |
| --- | --- |
| TS.64_6.1_REQ_002 | The FWA Device SHALL support all combinations of FDD and TDD duplexing (i.e., 2F, 2T, F+T and T+F) in 2DL NR Carrier Aggregation. |
| TS.64_6.1_REQ_003 | The FWA device SHOULD support 3DL NR Carrier Aggregation. |
| TS.64_6.1_REQ_004 | The FWA device MAY support 4DL NR Carrier Aggregation or higher order. |
| TS.64_6.1_REQ_005 | The FWA device SHOULD support 2UL NR Carrier Aggregation. |
| TS.64_6.1_REQ_006 | The FWA device SHALL support 15 kHz Sub-Carrier Spacing in FR1 NR bands. |
| TS.64_6.1_REQ_007 | The FWA device SHALL support 30 kHz Sub-Carrier Spacing in FR1 NR bands. |
| TS.64_6.1_REQ_008 | The FWA device SHALL support MIMO 4x4 DL capability on NR high-bands (e.g. NR bands n77/n78). |
| TS.64_6.1_REQ_009 | The FWA device SHALL support MIMO 2x2 UL capability on NR high-bands (e.g. NR bands n77/n78). |
| TS.64_6.1_REQ_010 | The FWA device SHALL support 4Rx diversity on NR bands. |
| TS.64_6.1_REQ_011 | The FWA device SHOULD support 8Rx diversity on NR bands. |
| TS.64_6.1_REQ_012 | The FWA device MAY support more than 8Rx diversity on NR bands. |
| TS.64_6.1_REQ_013 | The FWA device SHALL support 256QAM modulation for downlink. |
| TS.64_6.1_REQ_014 | The FWA device MAY support 1024QAM modulation for downlink on NR TDD FR1 high-bands (e.g. n77/n78). |
| TS.64_6.1_REQ_015 | The FWA device SHALL support 64QAM modulation for uplink. |
| TS.64_6.1_REQ_016 | The FWA device SHOULD support 256QAM modulation for uplink. |
| TS.64_6.1_REQ_017 | The FWA device SHALL support power class 3 (23 dBm). |
| TS.64_6.1_REQ_018 | The FWA device SHOULD support power class 2 (26 dBm) in compliance with 3GPP TS 38.101-1 [58]. |
| TS.64_6.1_REQ_019 | The FWA device SHOULD support power class 1.5 (29 dBm) in compliance with 3GPP TS 38.101-1 [58]. |

## Radio/RRC/NAS specific requirements for 5G-FR2 SA FWA devices

| TS.64_6.2_REQ_001 | The FWA device SHALL support 2DL intra-band contiguous NR Carrier Aggregation. |
| --- | --- |
| TS.64_6.2_REQ_002 | The FWA device SHOULD support 4DL intra-band contiguous NR Carrier Aggregation. |
| TS.64_6.2_REQ_003 | The FWA device SHOULD support 8DL intra-band contiguous NR Carrier Aggregation. |
| TS.64_6.2_REQ_004 | The FWA device SHOULD support 2UL intra-band contiguous NR Carrier Aggregation. |
| TS.64_6.2_REQ_005 | The FWA device MAY support 4UL intra-band contiguous NR Carrier Aggregation. |
| TS.64_6.2_REQ_006 | The FWA device SHALL support 100 MHz channel bandwidth in FR2 NR TDD bands (e.g., n257/n258). |
| TS.64_6.2_REQ_007 | The FWA device SHOULD support 200 MHz channel bandwidth in FR2 NR TDD bands (e.g., n257/n258). |
| TS.64_6.2_REQ_008 | The FWA device SHOULD support 400 MHz channel bandwidth in FR2 NR TDD bands (e.g., n257/n258). |
| TS.64_6.2_REQ_009 | The FWA device SHALL support 60 kHz Sub-Carrier Spacing in FR2 NR TDD bands (e.g., n257/n258). |
| TS.64_6.2_REQ_010 | The FWA device SHALL support 120 kHz Sub-Carrier Spacing in FR2 NR TDD bands (e.g., n257/n258). |
| TS.64_6.2_REQ_011 | The FWA device SHALL support MIMO 2x2 DL capability on NR FR2 bands (e.g., NR bands n257/n258). |
| TS.64_6.2_REQ_012 | The FWA device SHOULD support MIMO 4x4 DL capability on NR FR2 bands (e.g., NR bands n257/n258). |
| TS.64_6.2_REQ_013 | The FWA device SHALL support 64QAM modulation for downlink. |
| TS.64_6.2_REQ_014 | The FWA device SHOULD support 256QAM modulation for downlink. |
| TS.64_6.2_REQ_015 | The FWA device SHALL support 64QAM modulation for uplink. |
| TS.64_6.2_REQ_016 | The FWA device SHOULD support 256QAM modulation for uplink. |
| TS.64_6.2_REQ_017 | The FWA device SHALL support power class 3 (23 dBm). |
| TS.64_6.2_REQ_018 | The FWA device SHOULD support power class 2 (26 dBm) in compliance with 3GPP TS 38.101-1 [58]. |
| TS.64_6.2_REQ_019 | The FWA device SHOULD support power class 1 (31 dBm) in compliance with 3GPP TS 38.101-1 [58]. |

## Antenna Performance Acceptance Values for 5G SA FWA devices

| TS.64_6.3_REQ_001 | The FWA device SHALL be compliant to TS.24 [57] for Antenna Performance acceptance values. |
| --- | --- |

# 5G FWA Device Requirement on URSP Network Slicing Integration

The implementation of URSP requires consideration of various UE types and their ability to manage application traffic identification and routing. While smartphones utilise existing operating system and app development frameworks to support URSP, equipment such as FWA must have the required URSP implementation explicitly defined, including the details that are specified in any TD regarding the traffic types required to be identified by the FWA, and the required implementation for establishing slices as detailed in RSD. The requirements outlined in sections 7.1 and 7.2 are provided to explicitly highlight the key considerations.

## 5G FWA URSP Network Slicing Core Device Architectural Support

| TS.64_7.1_REQ_001 | The 5G modem integrated in the 5G Indoor or Outdoor Device SHALL expose S-NSSAI subscription information available for 5G Indoor or Outdoor device and its HLOS to consume. |
| --- | --- |
| TS.64_7.1_REQ_002 | The 5G Indoor or Outdoor Device SHALL ensure its HLOS is capable of supporting URSP and can consume 5G modem exposed S-NSSAI subscription information (i.e., Traffic Descriptor, Route Selection Descriptor). |
| TS.64_7.1_REQ_003 | The 5G Indoor or Outdoor Device SHALL support a minimum four concurrent slices. |
| TS.64_7.1_REQ_004 | The 5G Indoor or Outdoor Device SHOULD support a minimum eight concurrent slices. |

## 5G FWA URSP Network Slicing Device Support

| TS.64_7.2_REQ_001 | The 5G Indoor and Outdoor FWA Device SHALL support Match-All TD Type. |
| --- | --- |
| TS.64_7.2_REQ_002 | The 5G Indoor and Outdoor FWA Device SHALL support IP 3-tuple TD Type. |
| TS.64_7.2_REQ_003 | The 5G Indoor and Outdoor FWA Device SHALL support IPv4 Remote Address in IP 3-tuple TD Type. |
| TS.64_7.2_REQ_004 | The 5G Indoor and Outdoor FWA Device SHALL support IPv6 Remote Address/Prefix Length in IP 3-tuple TD Type. |
| TS.64_7.2_REQ_005 | The 5G Indoor and Outdoor FWA Device SHALL support Single Remote Port in IP 3-tuple TD Type. |
| TS.64_7.2_REQ_005 | The 5G Indoor and Outdoor FWA Device SHALL support Remote Port Range in IP 3-tuple TD Type. |
| TS.64_7.2_REQ_007 | The 5G Indoor and Outdoor FWA Device SHALL support Protocol ID / Next Header Field in IP 3-tuple TD Type. |
| TS.64_7.2_REQ_008 | The 5G Indoor and Outdoor FWA Device SHALL support Type of Service TD Type. |
| TS.64_7.2_REQ_009 | The 5G Indoor and Outdoor FWA Device SHALL support S-NSSAI Type RSD. |
| TS.64_7.2_REQ_010 | The 5G Indoor and Outdoor FWA Device SHALL support DNN Type RSD. |
| TS.64_7.2_REQ_011 | The 5G Indoor and Outdoor FWA Device SHALL support PDU Session Type RSD. |
| TS.64_7.2_REQ_012 | If no PDU Session Type is specified in the RSD as IPv4 only, the 5G FWA Device SHALL set the PDU Session Type as IPv4 in the PDU Establishment Request. |
| TS.64_7.2_REQ_013 | If no PDU Session Type is specified in the RSD as IPv6 only, the 5G FWA Device SHALL set the PDU Session Type as IPv6 in the PDU Establishment Request. |
| TS.64_7.2_REQ_014 | If no PDU Session Type is specified in the RSD, the 5G FWA Device SHOULD set the PDU Session Type as IPv4v6 in the PDU Establishment Request. |
| TS.64_7.2_REQ_015 | The 5G Indoor and Outdoor FWA Device SHALL configure local traffic steering implementation (e.g., routing table) following the received URSP TD and RSD definition. |
| TS.64_7.2_REQ_016 | The 5G Indoor and Outdoor FWA Device SHALL continue to evaluate network assigned URSP rules stored locally when in LTE fallback and follow the DNN definition defined in RSD to a defined APN. |
| TS.64_7.2_REQ_017 | The 5G Indoor and Outdoor FWA Device SHALL steer traffic to the next priority matching slice with matching TD and RSD when target slice fails to be configured. |
| TS.64_7.2_REQ_018 | 5G Indoor and Outdoor FWA Device SHALL steer traffic to the Match-All slice as the final traffic steering fall back option. |

- Hardware, Safety and EMC normative references for European Market

Safety

- EN 50385:2017 (Product standard to demonstrate the compliance of base station equipment with electromagnetic field exposure limits (110 MHz - 100 GHz), when placed on the market).

- EN 62311:2020. Assessment of electronic and electrical equipment related to human exposure restrictions for electromagnetic fields (0 Hz - 300 GHz).

- IEC EN 62368-1:2023 (Audio/video, information and communication technology equipment - Part 1: Safety requirements).

EMC

- IEC EN 55032:2015 + A1:2020 - class B limits (Electromagnetic compatibility of multimedia equipment - Emission requirements).

- IEC EN 55035:2017 + A11: 2020 (Electromagnetic compatibility of multimedia equipment. Immunity requirements).

- ETSI EN 301 489-1 V2.2.3 (EMNC Standard for Radio Equipment and services – Part1: Common Requirements).

- ETSI EN 301 489-17 V3.2.4 (Part 17: Specific conditions for Broadband Data Transmission Systems).

- ETSI EN 301 489-19 V2.2.1 Specific conditions for Receive Only Mobile Earth Stations (ROMES) operating in the 1,5 GHz band providing data communications and GNSS receivers operating in the RNSS band (ROGNSS) providing positioning, navigation, and timing data.

- ETSI EN 301 489-52 V1.2.2 (Part 52: Specific conditions for Cellular Communications User Equipment’s).

- IEC EN 61000-3-2:2014 (limitation of harmonic currents injected into the public supply system).

Radio Spectrum

- ETSI EN 300 328 V2.2.2 (Data transmission equipment operating in the 2,4 GHz band; Harmonised Standard for access to radio spectrum).

- ETSI EN 301 893 V2.1.1 (5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU).

- ETSI EN 301 908-1 V15.1.1 (IMT cellular networks; Harmonised Standard covering the essential requirements of article 3.2 of the Directive 2014/53/EU; Part 1: Introduction and common requirements).

- ETSI EN 301 908-2 V13.1.1 MT cellular networks; Harmonised Standard for access to radio spectrum; Part 2: CDMA Direct Spread (UTRA FDD) User Equipment (UE).

- ETSI EN 301 908-13 V13.2.1 (IMT cellular networks; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU; Part 13: Evolved Universal Terrestrial Radio Access (E-UTRA) User Equipment (UE)).

- Draft ETSI 301 908-25 V15.1.1 IMT cellular networks - Harmonised Standard for access to radio spectrum - Part 1: Introduction and common requirements Release 15.

- EN 303 413 V1.2.1 Satellite Earth Stations and Systems (SES); Global Navigation Satellite System (GNSS) receivers; Radio equipment operating in the 1164 MHz to 1300 MHz and 1559 MHz to 1610 MHz frequency bands; Harmonised Standard for access to radio spectrum.

- ETSI TS 138 521-1 V17.5.0 5G; NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 standalone (3GPP TS 38.521-1 version 17.5.0 Release 17).

- ETSI TS 138 521-3 V17.5.0 5G; NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios (3GPP TS 38.521-3 version 17.5.0 Release 17).

- ETSI TS 38 521-3 V16.4.0 5G New Radio User Equipment Conformance Specification – Radio Transmission and Reception Part 3: Range 1 and Range 2 Interworking Operation with other Radios.

Environment Protection and Energy Efficiency

- EN 63000:2018 Technical documentation for the assessment of electrical and electronic products with respect to the restriction of hazardous substances

- EN 50564:2011: Electrical and electronic household and office equipment - Measurement of low power consumption

- Directive 2014/53/EU Radio equipment, and the related conformity assessment procedure.

- Energy related Products (ErP) Directive 2009/125/EC and related Commission Regulation (EC) no. 2023/826.

- Directives RoHS 2011/65/UE and WEEE 2012/19/UE, and the related conformity assessment procedures.

- For the operation in the European Union (EU), considering the obligations of the applicable Directives, the manufacturer of the FWA Device shall provide the full text of the Declaration of Conformity (DoC) and the Technical File, which shall include the Technical Documentation (TD) compliant with the conformity assessment procedure used under the applicable product Directives.

Environment Operating Conditions

- ETSI ETS 300 019-1-1 [] as regards storage for equipment in class 1.1 (Weather protected, partly temperature-controlled storage locations)

- ETSI ETS 300 019-1-2 [] as regards transportation for equipment in class 2.3 (Public transportation).

- For the Indoor FWA Device and the InDoor Unit of the Outdoor FWA Solution, ETSI EN 300 019-1-3 [] as regards stationary use in Environmental Class T 3.1 “Temperature-controlled locations”.

- For the OutDoor Unit of the Outdoor FWA Solution, ETSI EN 300 019-1-4 [] as regards stationary use in Environmental Class T 4.1 “Non-weather protected locations”.

- For the OutDoor Unit of the Outdoor FWA Solution, IEC EN 60068-2-52 as regards the corrosion produced by salt-laden atmosphere - severity (1).

- ETS 300 019-2-3 as regards the mechanical stress, (severity Class T3.2).

a.	IEC EN 60068-2-27 Test Ea – Shock: according to, with test severity defined for the Class T3.2 by the standard ETS 300 019-2-3

b.	IEC EN 60068-2-64 Test Fh – Random: according to, with test severity defined for the Class T3.2 by the standard

c.	IEC EN 60068-2-6 Test Fc – Sinusoidal Vibration: according to, with test severity defined for the Class T3.2 by the standard ETS 300 019-2-3

d.	IEC EN 60068-2-32 Test Ed – Free fall from a height of 1m.

Example of Wi-Fi metrics for Wi-Fi diagnostic

Wi-Fi metrics related to the AP of the FWA Device, and the connected clients are defined in Wi-Fi Data Elements™ specified by Wi-Fi Alliance and the corresponding TR-181 data model specified by BroadBand Forum.

Document Management

Document History

| Version | Date | Brief Description of Change | Approval Authority | Editor / Company |
| --- | --- | --- | --- | --- |
| 1.0 | Oct 2023 | New PRD | TSG#53 ISAG#34 | Paolo Ferrabone Telecom Italia |
| 2.0 | May 2025 | Implementing changes in CR1041 v02 | TSG#59 ISAG#52 | Paolo Ferrabone Telecom Italia |
| 2.1 | Sept 2025 | Implementing changes in CR1042 v01 | TSG#61 Only | Paolo Ferrabone Telecom Italia |
| 3.0 | Jan 2026 | Implementing changes in CR1040 v07 | TSG#62 ISAG#58 | Paolo Ferrabone Telecom Italia |
| 4.0 | March 2026 | Implementing changes in CR1054 v02 | TSG#63 ISAG#60 | Paolo Ferrabone Telecom Italia |
| 5.0 | Oct 2026 | Implementing changes in CR1056 v04 | TSG#65 ISAG Oct 2026 | Paolo Ferrabone Telecom Italia |

Other Information

| Type | Description |
| --- | --- |
| Document Owner | GSMA Terminal Steering Group (TSG) |
| Editor / Company | Paolo Ferrabone / Telecom Italia |

It is our intention to provide a quality product for your use. If you find any errors or omissions, please contact us with your comments. You may notify us at prd@gsma.com

Your comments or suggestions & questions are always welcome.

Maximum Transmission Unit (MTU) is the largest Layer 3 (IP) packet a network interface can transmit, while Maximum Segment Size (MSS) is the largest chunk of data (payload) TCP will send in a single segment (Layer 4). Their relationship is:

MSS = MTU - IP header - TCP header

MTU and MSS are normally measured in bytes.

For example, standard Ethernet supports an MTU of 1500 bytes. With IPv4, this leads to:

MSS = 1500 - 20 - 20 = 1460 bytes