# RSP Test Specification for the Device

# SGP.23-2 Version 3.1

# 01 December 2023

Security Classification: Non-confidential

Access to and distribution of this document is restricted to the persons permitted by the security classification. This document is confidential to the Association and is subject to copyright protection. This document is to be used only for the purposes for which it has been supplied and information contained in it must not be disclosed or in any other way made available, in whole or in part, to persons other than those permitted under the security classification without the prior written approval of the Association.

Copyright Notice

Copyright © 2023 GSM Association

Disclaimer

The GSM Association (“Association”) makes no representation, warranty or undertaking (express or implied) with respect to and does not accept any responsibility for, and hereby disclaims liability for the accuracy or completeness or timeliness of the information contained in this document. The information contained in this document may be subject to change without prior notice.

Antitrust Notice

The information contain herein is in full compliance with the GSM Association’s antitrust compliance policy.

Table of Contents

1	Introduction	6

1.1	Overview	6

1.2	Scope	6

1.3	Definition of Terms	6

1.4	Abbreviations	7

1.5	Document Cross-references	7

1.6	Conventions	9

2	Testing Rules	9

2.1	Applicability	9

2.1.1	Format of the Optional Features Table	9

2.1.2	Format of the Applicability Table	9

2.1.3	Applicability and Notations	9

2.1.4	Optional Features Table	10

2.1.5	Applicability Table	12

2.2	General Consideration	18

2.2.1	Test Case Definition	18

2.2.2	Test Cases Format	18

2.2.3	VOID	23

2.2.4	General Rules for Device Testing	23

2.2.5	Pass Criteria	26

2.2.6	Future Study	26

2.2.7	VOID	26

3	Testing Architecture	26

3.1	Testing Scope	26

3.2	Testing Execution	28

3.2.1	VOID	28

3.2.2	VOID	28

3.2.3	Device/LPAd - Test Environment	28

3.2.4	VOID	30

3.2.5	VOID	30

4	Interface Compliance Testing	30

4.1	General Overview	30

4.2	VOID	30

4.3	VOID	30

4.4	LPAd Interfaces	31

4.4.1	ES10a (LPA -- eUICC): GetEuiccConfiguredAddresses	31

4.4.2	ES10a (LPA -- eUICC): SetDefaultDpAddress	31

4.4.3	ES10b (LPA -- eUICC): PrepareDownload	31

4.4.4	ES10b (LPA -- eUICC): LoadBoundProfilePackage	31

4.4.5	ES10b (LPA -- eUICC): GetEUICCChallenge	31

4.4.6	ES10b (LPA -- eUICC): GetEUICCInfo	31

4.4.7	ES10b (LPA -- eUICC): ListNotification	31

4.4.8	ES10b (LPA -- eUICC): RetrieveNotificationsList	31

4.4.9	ES10b (LPA -- eUICC): RemoveNotificationFromList	31

4.4.10	ES10b (LPA -- eUICC): LoadCRL	31

4.4.11	ES10b (LPA -- eUICC): AuthenticateServer	31

4.4.12	ES10b (LPA -- eUICC): CancelSession	31

4.4.13	ES10c (LPA -- eUICC): GetProfilesInfo	31

4.4.14	ES10c (LPA -- eUICC): EnableProfile	31

4.4.15	ES10c (LPA -- eUICC): DisableProfile	31

4.4.16	ES10c (LPA -- eUICC): DeleteProfile	32

4.4.17	ES10c (LPA -- eUICC): eUICCMemoryReset	32

4.4.18	ES10c (LPA -- eUICC): GetEID	32

4.4.19	ES10c (LPA -- eUICC): SetNickname	32

4.4.20	ES10b (LPA -- eUICC): GetRAT	32

4.4.21	ES9+ (LPA -- SM-DP+): InitiateAuthentication	32

4.4.22	ES9+ (LPA -- SM-DP+): GetBoundProfilePackage	47

4.4.23	ES9+ (LPA -- SM-DP+): AuthenticateClient	56

4.4.24	ES9+ (LPA – SM-DP+): HandleNotification	74

4.4.25	ES9+ (LPA – SM-DP+): CancelSession	81

4.4.26	ES9+ (LPA – SM-DP+): HTTPS	97

4.4.27	ES11 (LPA – SM-DS): InitiateAuthentication	102

4.4.28	ES11 (LPA – SM-DS): AuthenticateClient	107

4.4.29	ES11 (LPA -- SM-DS): HTTPS	114

4.4.30	ES9+ (LPA -- SM-DP+): AuthenticateClient – RPM Package Download	118

4.4.32	ES9+ (LPA – SM-DP+): HandleNotification for RPM Package Download	137

4.5	VOID	142

4.6	VOID	142

4.7	VOID	142

5	Procedure - Behaviour Testing	142

5.1	General Overview	142

5.2	VOID	142

5.3	VOID	142

5.4	Device Procedures	142

5.4.1	Local Profile Management - Add Profile	142

5.4.2	Local Profile Management – ListProfiles	168

5.4.3	Local Profile Management - SetNickname	169

5.4.4	Local Profile Management - Delete Profile	172

5.4.5	Local Profile Management - Enable Profile	177

5.4.6	Local Profile Management- Disable Profile	182

5.4.7	Local eUICC Management - Retrieve EID Process	185

5.4.8	Local eUICC Management - eUICC Memory Reset Process	186

5.4.9	Local eUICC Management–- eUICC Test Memory Reset Process	193

5.4.10	Local eUICC Management – Set/Edit Default SM-DP+ Address Process	193

5.4.11	Device Power On – Profile Discovery	196

5.4.12	RPM Command Execution - Enable Profile	199

5.4.13	RPM Command Execution - Disable Profile	203

5.4.14	RPM Command Execution - Delete Profile	208

5.4.15	RPM Command Execution – List Profile Info	213

5.4.16	RPM Command Execution – Update Metadata	221

6	VOID	225

7	VOID	225

Annex A	Constants	226

A.1	Generic Constants	226

A.2	Test Certificates and Test Keys	238

Annex B	Dynamic Content	242

Annex C	Methods And Procedures	255

C.1	Methods	255

C.2	Procedures	274

Annex D	Commands And Responses	294

D.1	ES8+ Requests And Responses	294

D.1.1 ES8+ Requests	294

D.2	ES9+ Requests And Responses	324

D.2.1	ES9+ Requests	324

D.2.2	ES9+ Responses	360

D.3	VOID	373

D.4	VOID	373

D.5	VOID	373

D.6	ES11 Requests And Responses	373

D.6.1	ES11 Requests	373

D.6.2	ES11 Responses	380

D.7	VOID	382

D.8	VOID	382

D.9	Common Server Responses	382

D.10	ES2+ Requests And Responses	390

Annex E	Profiles	392

Annex F	IUT Settings	407

F.1	VOID	407

F.2	VOID	407

F.3	Device Settings	407

F.4	VOID	408

Annex G	Initial States	408

G.1	Device	408

G.1.1 Device (default)	408

G.1.2 Companion Device connected to a Primary Device	408

G.1.3 Test eUICC Settings	409

G.2	VOID	410

G.3	VOID	410

Annex H	Icons and QR Codes	411

Annex I	Requirements	412

Annex J	VOID	413

Annex K	Document Management	414

K.1	Document History	414

K.2	Other Information	419

# Introduction

## 1.1	Overview

The main aim of the GSMA Remote SIM Provisioning specifications [2] & [3] is to provide solution for the Remote SIM Provisioning of Consumer Devices. The adoption of this technical solution will provide the basis for global interoperability between different Operator deployment scenarios, for example network equipment (e.g. Subscription Manager Data Preparation (SM-DP+)) and various eUICC platforms.

This Test Plan provides a set of test cases to be used for testing the implementations of the provisioning system specifications documents [2] & [3]. This document offers to the involved entities an unified test strategy and ensures interoperability between different implementations.

## 1.2	Scope

This document is intended for:

Parties which develop test tools and platforms

Vendors (Device and eUICC Manufacturers, SM-DP+ and SM-DS Providers)

Operators

The Test Plan consists of a set of relevant test cases for the Device/LPA testing The only Implementations Under Test (IUT) within this document is the LPA. Test cases for the eUICC are defined in [30], Test cases for the Servers (SM-DP+, SM-DS) are defined in [31].

The testing scopes developed in this document are:

Interface compliance testing: Test cases to verify the compliance of the interfaces within the system.

System behaviour testing: Test cases to verify the functional behaviour of the system.

Each test case specified within this Test Plan refers to one or more requirements.

The Test Plan contains test cases for the following versions of SGP.22:

- GSMA RSP Technical Specification V3.1 [2]

This document includes an applicability table providing an indication whether test cases are relevant for a specific Device/LPA.

## 1.3	Definition of Terms

In addition to the terms which are defined below, the terms defined in SGP.22 [2] also apply

| Term | Description |
| --- | --- |
| End User | The person using the Device. |
| Integrated eUICC Test Interface | An external interface provided by its manufacturer for the purpose of testing eUICC functionality. |
| Standalone Device | A Device which provides all the capabilities to be able to be used in an RSP environment and needs no other Device for the purpose of Remote SIM Provisioning. |
| Test Plan | Current document describing the test cases that allow the RSP ecosystem to be tested. |

## 1.4	Abbreviations

In addition to the abbreviations which are defined below, the abbreviations defined in SGP.22 [2] also apply.

| Abbreviation | Description |
| --- | --- |
| APDU | Application Protocol Data Unit |
| ATR | Answer To Reset |
| C-APDU | Command APDU |
| CCID | (USB) Chip Card Interface Device |
| DER TLV | Distinguished Encoding Rules - Tag Length Value |
| FCP | File Control Parameters |
| HW | Hardware |
| IUT | Implementation Under Test |
| KVN | Key Version Number |
| OCE | Off-Card Entity |
| OS | Operating System |
| PIR | Profile Installation Result |
| POR | Proof Of Receipt |
| R-APDU | Response APDU |
| SoC | System on a Chip |
| SP | Service Provider |
| SSD | Supplemental Security Domain |
| USB | Universal Serial Bus |

## 1.5	Document Cross-references

| Ref | Document Number | Title |
| --- | --- | --- |
| [1] | SGP.02 | GSMA "Remote Provisioning of Embedded UICC Technical specification" V3.1 |
| [2] | SGP.22 | RSP Technical Specification V3.1 |
| [3] | SGP.21 | RSP Architecture V3.1 |
| [4] | eUICC Profile Package | Trusted Connectivity Alliance (formerly SIMalliance) eUICC Profile Package: Interoperable Format Technical Specification V2.0 or later |
| [5] | ETSI TS 102 221 | Smart Cards; UICC-Terminal interface |
| [6] | GPC_SPE_034 | GlobalPlatform Card Specification v.2.3 |
| [7] | ISO/IEC 7816-4:2013 | Identification cards – Integrated circuit cards - Part 4: Organization, security and commands for interchange |
| [8] | RFC 5639 | Elliptic Curve Cryptography (ECC) Brainpool Standard Curves and Curve Generation |
| [9] | ANSSI ECC FRP256V1 | Avis relatif aux paramètres de courbes elliptiques définis par l'Etat français. JORF n°0241 du 16 octobre 2011 page 17533. texte n° 30. 2011 |
| [10] | ITU E.118 | The international telecommunication charge card |
| [11] | NIST SP 800-56A | NIST Special Publication SP 800-56A: Recommendation for Pair-Wise Key Establishment Schemes Using Discrete Logarithm Cryptography (Revision 2), May 2013 |
| [12] | 3GPP TS 23.003 | Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); Numbering, addressing and identification |
| [13] | ETSI TS 102 225 | Secured packet structure for UICC based applications; Release 12 |
| [14] | ETSI TS 102 226 | Remote APDU structure for UICC based applications; Release 9 |
| [15] | TS.26 | GSMA NFC Handset Requirements V9.0 |
| [16] | ITU-T X.690 (11/2008) | ASN.1 Encoding Rules: Specification of Basic Encoding Rules (BER), Canonical Encoding Rules (CER) and Distinguished Encoding Rules (DER) including Corrigendum 1 and 2 |
| [17] | ETSI TS 102 241 | Smart cards; UICC Application Programming Interface (UICC API) for Java Card™ |
| [18] | 3GPP TS 31.102 | Characteristics of the Universal Subscriber Identity Module (USIM) application |
| [19] | GPC_SPE_095 | GlobalPlatform Card - Digital Letter of Approval - Version 1.0 |
| [20] | RFC 2119 | Key words for use in RFCs to Indicate Requirement Levels, S. Bradner http://www.ietf.org/rfc/rfc2119.txt |
| [21] | Void |  |
| [22] | 3GPP TS 23.040 | Technical realization of the Short Message Service (SMS) |
| [23] | TCA Test | Trusted Connectivity Alliance (TCA) eUICC Profile Package: Interoperable Format Test Specification Version 2.3.1 |
| [24] | RFC 4492 | Elliptic Curve Cryptography (ECC) Cipher Suites for Transport Layer Security (TLS) |
| [25] | SGP.26 | RSP Test Certificates Definition v1.5 |
| [26] | 3GPP TS 29.002 | Mobile Application Part (MAP) specification |
| [27] | RFC 5246 | The Transport Layer Security (TLS) Protocol Version 1.2 |
| [28] | GSMA PRD AA.35 | Procedures for Industry Specifications Product |
| [29] | CCID Rev 1.1 | CCID Specification for Integrated Circuit(s) Cards Interface Devices |
| [30] | SGP.23-1 v3.1 | RSP Test Specification for the eUICC |
| [31] | SGP.23-3 v3.1 | RSP Test Specification for Servers |

## 1.6	Conventions

The key words "SHALL", "SHALL NOT", "SHOULD", "SHOULD NOT", and "MAY" in this document SHALL be interpreted as described in RFC 2119 [20].

# Testing Rules

## Applicability

### Format of the Optional Features Table

The columns in Table 4 have the following meaning:

| Column | Meaning |
| --- | --- |
| Option | The optional feature supported or not by the implementation. |
| Mnemonic | The mnemonic column contains mnemonic identifiers for each item. |

- Table 1: Format of the Optional Features Table

### Format of the Applicability Table

The applicability of every test in Table 5 is formally expressed by the use of a Boolean expression defined in the following clause.

The columns in Table 5 have the following meaning:

| Column | Meaning |
| --- | --- |
| Test case | The "Test case" column gives a reference to the test case number detailed in the present document and is required to validate the implementation of the corresponding item in the "Name" column. |
| Name | In the "Name" column, a short non-exhaustive description of the test is found. |
| Version | This column indicates which test cases are applicable for the given SGP.22 version. See clause 2.1.3 'Applicability and Notations'. |
| Test Env. | Test environment used for executing the test case. |

- Table 2: Format of the Applicability Table

### Applicability and Notations

The following notations are used for the Applicability column:

| Applicability code | Meaning |
| --- | --- |
| M | mandatory - the capability is required to be supported. |
| N/A | not applicable - in the given context, it is impossible to use the capability. |
| Ci | conditional - the requirement on the capability depends on the support of other items. "i" is an integer identifying an unique conditional status expression which is defined immediately following the table. For nested conditional expressions, the syntax "IF ... THEN (IF ... THEN ... ELSE...) ELSE ..." is to be used to avoid ambiguities. |

- Table 3: Applicability and Notations

### Optional Features Table

The supplier of the implementation SHALL state the support of possible options in Table 5.

| Device Options | Mnemonic |
| --- | --- |
| The Device supports LPAd | O_D_LPAD |
| The Device supports GSM/GERAN | O_D_GSM_GERAN |
| The Device supports UMTS/UTRAN | O_D_UMTS_UTRAN |
| The Device supports cdma2000 1X | O_D_CDMA2000_1X |
| The Device supports cdma2000 HRPD | O_D_CDMA2000_HRPD |
| The Device supports cdma2000 eHRPD | O_D_CDMA2000_EHRPD |
| The Device supports LTE/E-UTRAN | O_D_LTE |
| The Device supports NFC as defined in TS26 | O_D_NFC_TS26 |
| Initiation of the Enable Profile procedure is allowed on a Profile that is enabled already | O_D_ENPROF |
| Initiation of the Enable Profile procedure is allowed even if the currently enabled Profile has PPR1 | O_D_ENPREVPPR1 |
| Device supports only cellular connectivity (see NOTE 1) | O_D_ONLY_CELLULAR_CONNECTIVITY |
| Device offers a user interface to enter a PIN for user authentication | O_D_PIN |
| Device allows the End User to initiate the disabling or deletion of an enabled Profile with ppr1 | O_D_DISDELPPR1 |
| Device allows the End User to initiate the deletion of a Profile with ppr2 | O_D_DELPPR2 |
| Initiation of the Disable Profile procedure is allowed on a Profile that is disabled already | O_D_DISPROF |
| Initiation of Disable Profile procedure is allowed even if the currently enabled Profile has PPR1 | O_D_DISPPR1 |
| Device retries after unsuccessful CC entry attempt | O_D_CC_RETRY |
| The Device provides the LUI functionality to postpone Profile Download | O_D_EU_POSTPONED |
| The Device provides the LUI functionality to postpone Profile Download after GetBPP | O_D_EU_POSTPONED_AFTER_GETBPP |
| Device supports Power-on Profile discovery | O_D_POW_ON_PROF_DISCOVERY |
| The Device provides the LUI functionality to reject Profile Download | O_D_EU_REJECT |
| The Device supports Set/Edit Nickname procedure as defined in SGP.22 [2] section 3.2.6 and displaying the profile's Nickname | O_D_NICKNAME |
| The Device supports additional verification of TLS certificate content (i.e. key usage, extended key usage and certificate policy) | O_D_TLS_FULL_VERIFICATION |
| The Device supports Add Profile and Enable Profile in one combined operation (See NOTE 2) | O_D_ADD_ENABLE_COMBINED |
| The Device supports Add Profile and Enable Profile as separated operations (See NOTE 2) | O_D_ADD_ENABLE_SEPARATED |
| Initiation of Add Profile procedure is allowed even if the currently enabled Profile has PPR1 | O_D_ADDPREPPR1 |
| The Device supports Set/Edit Default SM-DP+ Address procedure as defined in SGP.22 [2] section 3.3.4 | O_D_DEFAULT_DP_ADDRESS |
| The Device supports a removable eUICC and downloading a profile containing PPRs to the removable eUICC. | O_D_REMOVABLE_DOWNLOAD_PPR |
| The Device supports a non-removable eUICC and eUICC RAT configurations in which PPR1 is allowed and End User Consent is required. | O_D_EMB_ALLOWS_PPR1_EUC_REQ |
| The Device supports a non-removable eUICC and eUICC RAT configurations in which PPR1 is allowed and End User Consent is NOT required. | O_D_EMB_ALLOWS_PPR1_EUC_NOT_REQ |
| The Device supports a non-removable eUICC and eUICC RAT configurations in which PPR2 is allowed and End User Consent is required. | O_D_EMB_ALLOWS_PPR2_EUC_REQ |
| The Device supports a non-removable eUICC and eUICC RAT configurations in which PPR2 is allowed and End User Consent is NOT required. | O_D_EMB_ALLOWS_PPR2_EUC_NOT_REQ |
| The Device supports NR EPC | O_D_NR_EPC |
| The Device supports NR 5GC | O_D_NR_5GC |
| The Device supports E-UTRAN 5GC | O_D_EUTRAN_5GC |
| The Device supports RPM | O_D_RPM |
| The Device is Enterprise capable | O_D_ENTERPRISE |
| The Device supports indication of supported Card Application Toolkit letter classes | O_D_CAT_CLASSES |
| The Device supports Multiple Enabled Profiles | O_D_MEP |
| The Device supports Activation QR code scanning. | O_D_QR_SCANNING |
| NOTE 1: Devices which supports O_D_ONLY_CELLULAR_CONNECTIVITY are out of scope of the current version of this document. - NOTE 2: The Device SHALL support at least one of O_D_ADD_ENABLE_COMBINED or O_D_ADD_ENABLE_SEPARATED. It is valid to support both options. |  |

- Table 4: Options

### Applicability Table

Table 5 specifies the applicability of each test case. See clause 2.1.2 for the format of this table.

| Test case | Name | V3.1 | Test Env. |
| --- | --- | --- | --- |
| LPAd Interfaces Compliance Testing |  |  |  |
| 4.4.21.2.1 | TC_LPAd_InitiateAuthentication_Nominal | C007 |  |
| 4.4.21.2.2 | TC_LPAd_InitiateAuthentication_ErrorCases | C007 |  |
| 4.4.21.2.3 | TC_LPAd_InitiateAuthentication_Nominal_V3 | C007 |  |
| 4.4.21.2.4 | TBD | C007 |  |
| 4.4.21.2.5 | TC_LPAd_InitiateAuthentication_Norminal_RPM | C303 |  |
| 4.4.21.2.6 | TC_LPAd_InitiateAuthentication_ErrorCases_RPM | C303 |  |
| 4.4.22.2.1 | TC_LPAd_ES9+_GetBoundProfilePackage_Nominal | C007 |  |
| 4.4.22.2.2 | TC_LPAd_ES9+_GetBoundProfilePackage_Retry | C005 |  |
| 4.4.22.2.3 | TC_LPAd_ES9+_GetBoundProfilePackage_Error | C007 |  |
| 4.4.22.2.4 | TC_LPAd_ES9+_GetBoundProfilePackage_Retry_Reuse_oldOTPK | C302 |  |
| 4.4.23.2.1 | TC_LPAd_AuthenticatClient_Nominal | C007 |  |
| 4.4.23.2.2 | TC_LPAd_AuthenticateClient_ErrorCases | C007 |  |
| 4.4.23.2.3 | TC_LPAd_Authenticate_Client_Nominal_V3 Only the test sequences #01 | C007 |  |
| 4.4.23.2.3 | TC_LPAd_Authenticate_Client_Nominal_V3 Only the test sequences #02 | C303 |  |
| 4.4.24.2.1 | TC_LPAd_ES9+_HandleNotification_Nominal | C007 |  |
| 4.4.25.2.1 | TC_LPAd_ES9+_CancelSession_Nominal All test sequences except the sequence #02 | C007 |  |
| 4.4.25.2.1 | TC_LPAd_ES9+_CancelSession_Nominal Only the test sequences #02 | C023 |  |
| 4.4.25.2.2 | TC_LPAd_ES9+_CancelSession_EndUserPostponed_Nominal | C008 |  |
| 4.4.25.2.3 | TC_LPAd_ES9+_CancelSession_Error | C026 |  |
| 4.4.25.2.4 | TC_LPAd_ES9+_CancelSession_PPRs Only test sequence #1 | C045 |  |
| 4.4.25.2.4 | TC_LPAd_ES9+_CancelSession_PPRs Only test sequence #2 | C046 |  |
| 4.4.25.2.5 | TC_LPAd_ ES9+_CancelSession_AuthenticateClient_RPM | C303 |  |
| 4.4.26.2.1 | TC_LPAd_HTTPS_Nominal | C007 |  |
| 4.4.26.2.2 | TC_LPAd_HTTPS_ErrorCases | C007 |  |
| 4.4.26.2.3 | TC_LPAd_HTTPS_Nominal_Variants_V3 | C007 |  |
| 4.4.27.2.1 | TC_LPAd_ES11_InitiateAuthentication_Nominal | C007 |  |
| 4.4.27.2.2 | TC_LPAd_ES11_InitiateAuthentication_ErrorCases | C007 |  |
| 4.4.28.2.1 | TC_LPAd_ES11_AuthenticateClient_Nominal | C007 |  |
| 4.4.28.2.2 | TC_LPAd_ES11_AuthenticateClient_ErrorCases | C007 |  |
| 4.4.29.2.1 | TC_LPAd_HTTPS_Nominal | C007 |  |
| 4.4.29.2.2 | TC_LPAd_HTTPS_Error | C007 |  |
| 4.4.30.2.1 | TC_LPAd_Authenticate_Client_Nominal_RPM | C303 |  |
| 4.4.30.2.2 | TC_LPAd_Authenticate_Client_Nominal_RPM_Enterprise_Device | C304 |  |
| 4.4.32.2.1 | TC_LPAd_RPM_ HandleNotification for RPM Package Download | C303 |  |
| Procedure - Behaviour Testing |  |  |  |
| 5.4.1.2.1 | TC_LPAd_AddProfile_Manual_Entry | C035 |  |
| 5.4.1.2.2 | TC_LPAd_AddProfile_QRCode_scanning | C307 |  |
| 5.4.1.2.3 | TC_LPAd_AddProfile_ActivationCode_InvalidFormat_QRCode | C308 |  |
| 5.4.1.2.4 | TC_LPAd_AddProfile_ActivationCode_InvalidFormat_ManualEntry | C309 |  |
| 5.4.1.2.5 | TC_LPAd_AddProfile_ConfirmationCode_smdpSigned2_QR | C307 |  |
| 5.4.1.2.6 | TC_LPAd_AddProfile_ConfirmationCode_smdpSigned2_Manual_Entry | C035 |  |
| 5.4.1.2.7 | TC_LPAd_AddProfile_default_SM-DP+_address | C035 |  |
| 5.4.1.2.8 | TC_LPAd_AddProfile _with_ConfirmationCode Only test sequence #01 | C307 |  |
| 5.4.1.2.8 | TC_LPAd_AddProfile _with_ConfirmationCode Only test sequence #02 | C035 |  |
| 5.4.1.2.9 | TC_LPAd_AddProfile_PPRs Only test sequence #1 | C047 |  |
| 5.4.1.2.9 | TC_LPAd_AddProfile_PPRs Only test sequence #2 | C048 |  |
| 5.4.1.2.9 | TC_LPAd_AddProfile_PPRs Only test sequence #3 | C051 |  |
| 5.4.1.2.11 | TC_LPAd_AddProfile_Security_Errors | C007 |  |
| 5.4.1.2.12 | TC_LPAd_AddProfile_Empty_MatchingID Only test sequence #01 | C307 |  |
| 5.4.1.2.12 | TC_LPAd_AddProfile_Empty_MatchingID Only test sequence #02 | C035 |  |
| 5.4.1.2.13 | TC_LPAd_AddEnabledProfile_Manual_Entry | C034 |  |
| 5.4.1.2.14 | TC_LPAd_AddEnabledProfile Only test sequence #01 | C306 |  |
| 5.4.1.2.14 | TC_LPAd_AddEnabledProfile Only test sequence #02 | C034 |  |
| 5.4.1.2.15 | TC_LPAd_AddEnabledProfile_ConfirmationCode_smdpSigned2_QR | C306 |  |
| 5.4.1.2.16 | TC_LPAd_AddEnableProfile_ConfirmationCode_smdpSigned2_Manual_Entry | C034 |  |
| 5.4.1.2.17 | TC_LPAd_AddEnabledProfile_default_SM-DP+_address | C034 |  |
| 5.4.1.2.18 | TC_LPAd_AddEnableProfile_ with_ConfirmationCode Only test sequence #01 | C306 |  |
| 5.4.1.2.18 | TC_LPAd_AddEnableProfile_with_ConfirmationCode Only test sequence #02 | C034 |  |
| 5.4.1.2.19 | TC_LPAd_AddEnabledProfile_PPRs Only test sequence #1 | C049 |  |
| 5.4.1.2.19 | TC_LPAd_AddEnabledProfile_PPRs Only test sequence #2 | C050 |  |
| 5.4.1.2.19 | TC_LPAd_AddEnabledProfile_PPRs Only test sequence #3 | C052 |  |
| 5.4.1.2.20 | TC_LPAd_AddEnableProfile_Empty_MatchingID Only test sequence #01 | C306 |  |
| 5.4.1.2.20 | TC_LPAd_AddEnableProfile_Empty_MatchingID Only test sequence #02 | C034 |  |
| 5.4.2.2.1 | TC_LPAd_ListProfiles | C007 |  |
| 5.4.3.2.1 | TC_LPAd_SetNickname | C027 |  |
| 5.4.3.2.2 | TC_LPAd_EditNickname | C027 |  |
| 5.4.4.2.1 | TC_LPAd_DeleteProfile_Disabled_without_PPR | C007 |  |
| 5.4.4.2.2 | TC_LPAd_DeleteProfile_Enabled_without_PPR | C009 |  |
| 5.4.4.2.3 | TC_LPAd_DeleteProfile_Error_with_PPR1 | C012 |  |
| 5.4.4.2.4 | TC_LPAd_DeleteProfile_Error_Disabled_with_PPR2 | C013 |  |
| 5.4.4.2.5 | TC_LPAd_DeleteProfile_Error_Enabled_with_PPR2 | C014 |  |
| 5.4.4.2.6 | TC_LPAd_DeleteProfile_Security_Errors | C007 |  |
| 5.4.4.2.7 | TC_LPAd_DeleteProfiles_MEP_Enabled_without_PPR | C305 |  |
| 5.4.5.2.1 | TC_LPAd_EnableProfile | C009 |  |
| 5.4.5.2.2 | TC_LPAd_EnableProfile_ImplicitDisable | C009 |  |
| 5.4.5.2.3 | TC_LPAd_EnableProfile_Error_ProfileAlreadyEnabled | C010 |  |
| 5.4.5.2.4 | TC_LPAd_EnableProfile_Error_PPR1Set | C011 |  |
| 5.4.5.2.6 | TC_LPAd_EnableProfile_MEP | C305 |  |
| 5.4.6.2.1 | TC_LPAd_DisableProfile | C009 |  |
| 5.4.6.2.2 | TC_LPAd_DisableProfile_Error_ProfileAlreadyDisabled | C017 |  |
| 5.4.6.2.3 | TC_LPAd_DisableProfile_Error_PPR1Set | C018 |  |
| 5.4.6.2.5 | TC_LPAd_DisableProfile_MEP | C305 |  |
| 5.4.7.2.1 | TC_LPAd_RetrieveEID | C004 |  |
| 5.4.8.2.1 | TC_LPAd_eUICCMemoryReset Only test sequence #1 and test sequence #5 | C007 |  |
| 5.4.8.2.1 | TC_LPAd_eUICCMemoryReset Only test sequence #2 and test sequence #3 | C044 |  |
| 5.4.8.2.1 | TC_LPAd_eUICCMemoryReset Only test sequence #4 | C043 |  |
| 5.4.8.2.2 | TC_LPAd_eUICCMemoryResetWithPINVerification | C009 |  |
| 5.4.8.2.3 | TC_LPAd_eUICCMemoryReset_MEP | C305 |  |
| 5.4.10.2.1 | TC_LPAd_Set/Edit Default SM-DP+ Address | C041 |  |
| 5.4.11.2.1 | TC_LPAd_DevicePowerOnProfileDiscovery_SM-DP+_address | C022 |  |
| 5.4.11.2.2 | TC_LPAd_DevicePowerOnProfileDiscovery_SM-DS | C022 |  |
| 5.4.12.2.1 | TC_LPAd_RPM_Command_Execution_EnableProfile | C303 |  |
| 5.4.13.2.1 | TC_LPAd_RPM_Command_Execution_DisableProfile | C303 |  |
| 5.4.14.2.1 | TC_LPAd_RPM_Command_Execution_DeleteProfile | C303 |  |
| 5.4.15.2.1 | TC_LPAd_RPM_Command_Execution_ListProfileInfo | C303 |  |
| 5.4.16.2.1 | TC_LPAd_RPM_Command_Execution_UpdateMetadata | C303 |  |

- Table 5: Applicability of Tests

| Conditional item | Condition |
| --- | --- |
| C004 | IF (O_D_LPAD) THEN M ELSE N/A |
| C005 | IF (O_D_CC_RETRY AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY) THEN M ELSE N/A |
| C007 | IF (O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY) THEN M ELSE N/A |
| C008 | IF (O_D_LPAD AND O_D_EU_POSTPONED AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY) THEN M ELSE N/A |
| C009 | IF (O_D_LPAD AND O_D_PIN AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY) THEN M ELSE N/A |
| C010 | IF (O_D_LPAD AND O_D_ENPROF) THEN M ELSE N/A |
| C011 | IF (O_D_LPAD AND (O_D_REMOVABLE_DOWNLOAD_PPR OR O_D_ALLOWS_PPR1_EUC_REQ OR O_D_ALLOWS_PPR1_EUC_NOT_REQ) AND O_D_ENPREVPPR1 AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY) THEM M ELSE N/A |
| C012 | IF (O_D_LPAD AND (O_D_REMOVABLE_DOWNLOAD_PPR OR O_D_ALLOWS_PPR1_EUC_REQ OR O_D_ALLOWS_PPR1_EUC_NOT_REQ) AND O_D_DISDELPPR1 AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY) THEN M ELSE N/A |
| C013 | IF (O_D_LPAD AND (O_D_REMOVABLE_DOWNLOAD_PPR OR O_D_ALLOWS_PPR2_EUC_REQ OR O_D_ALLOWS_PPR2_EUC_NOT_REQ) AND O_D_DELPPR2 AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY) THEN M ELSE N/A |
| C014 | IF (O_D_LPAD AND O_D_PIN AND (O_D_REMOVABLE_DOWNLOAD_PPR OR O_D_ALLOWS_PPR2_EUC_REQ OR O_D_ALLOWS_PPR2_EUC_NOT_REQ) AND O_D_DELPPR2 AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY) THEN M ELSE N/A |
| C017 | IF (O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY AND O_D_DISPROF) THEN M ELSE N/A |
| C018 | IF (O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY AND (O_D_REMOVABLE_DOWNLOAD_PPR OR O_D_ALLOWS_PPR1_EUC_REQ OR O_D_ALLOWS_PPR1_EUC_NOT_REQ) AND O_D_DISPPR1) THEN M ELSE N/A |
| C022 | IF (O_D_LPAD AND O_D_POW_ON_PROF_DISCOVERY AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY) THEN M ELSE N/A |
| C023 | IF (O_D_LPAD AND O_D_EU_REJECT AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY) THEN M ELSE N/A |
| C026 | IF ((O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY) AND (O_D_EU_POSTPONED OR O_D_EU_REJECT)) THEN M ELSE N/A |
| C027 | IF (O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY AND O_D_NICKNAME) THEN M ELSE N/A |
| C034 | IF (O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY AND O_D_ADD_ENABLE_COMBINED AND NOT O_D_QR_SCANNING) THEN M ELSE NA |
| C035 | IF (O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY AND O_D_ADD_ENABLE_SEPARATED AND NOT O_D_QR_SCANNING) THEN M ELSE NA |
| C041 | IF (O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY AND O_D_DEFAULT_DP_ADDRESS) THEN M ELSE N/A |
| C043 | IF (O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY AND (O_D_REMOVABLE_DOWNLOAD_PPR OR O_D_EMB_ALLOWS_PPR1_EUC_REQ OR O_D_EMB_ALLOWS_PPR1_EUC_NOT_REQ)) THEN M ELSE N/A |
| C044 | IF (O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY AND (O_D_REMOVABLE_DOWNLOAD_PPR OR O_D_EMB_ALLOWS_PPR2_EUC_REQ OR O_D_EMB_ALLOWS_PPR2_EUC_NOT_REQ)) THEN M ELSE N/A |
| C045 | IF ((O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY) AND (O_D_EU_POSTPONED OR O_D_EU_REJECT) AND (O_D_REMOVABLE_DOWNLOAD_PPR OR O_D_EMB_ALLOWS_PPR1_EUC_REQ)) THEN M ELSE N/A |
| C046 | IF ((O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY) AND (O_D_EU_POSTPONED OR O_D_EU_REJECT) AND (O_D_REMOVABLE_DOWNLOAD_PPR OR O_D_EMB_ALLOWS_PPR2_EUC_REQ)) THEN M ELSE N/A |
| C047 | IF (O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY AND O_D_ADD_ENABLE_SEPARATED AND (O_D_REMOVABLE_DOWNLOAD_PPR OR O_D_EMB_ALLOWS_PPR1_EUC_REQ)) THEN M ELSE NA |
| C048 | IF (O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY AND O_D_ADD_ENABLE_SEPARATED AND (O_D_REMOVABLE_DOWNLOAD_PPR OR O_D_EMB_ALLOWS_PPR2_EUC_REQ)) THEN M ELSE NA |
| C049 | IF (O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY AND O_D_ADD_ENABLE_COMBINED AND (O_D_REMOVABLE_DOWNLOAD_PPR OR O_D_EMB_ALLOWS_PPR1_EUC_REQ)) THEN M ELSE NA |
| C050 | IF (O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY AND O_D_ADD_ENABLE_COMBINED AND (O_D_REMOVABLE_DOWNLOAD_PPR OR O_D_EMB_ALLOWS_PPR2_EUC_REQ)) THEN M ELSE NA |
| C051 | IF (O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY AND O_D_ADD_ENABLE_SEPARATED AND O_D_ADDPREPPR1 AND (O_D_REMOVABLE_DOWNLOAD_PPR OR O_D_EMB_ALLOWS_PPR1_EUC_REQ OR O_D_EMB_ALLOWS_PPR1_EUC_NOT_REQ) THEN M ELSE N/A |
| C052 | IF (O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY AND O_D_ADD_ENABLE_COMBINED AND O_D_ADDPREPPR1 AND (O_D_REMOVABLE_DOWNLOAD_PPR OR O_D_EMB_ALLOWS_PPR1_EUC_REQ OR O_D_EMB_ALLOWS_PPR1_EUC_NOT_REQ) THEN M ELSE N/A |
| Conditions applicable to SGP.23 v3.1 only |  |
| C302 | IF (O_D_EU_POSTPONED_AFTER_GETBPP AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY) THEN M ELSE N/A |
| C303 | IF O_D_RPM THEN M ELSE N/A |
| C304 | IF O_D_RPM AND O_D_ENTERPRISE THEN M ELSE N/A |
| C305 | IF O_D_MEP THEN M ELSE N/A |
| C306 | IF (O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY AND O_D_ADD_ENABLE_COMBINED AND O_D_QR_SCANNING) THEN M ELSE N/A |
| C307 | IF (O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY AND O_D_ADD_ENABLE_SEPARATED AND O_D_QR_SCANNING) THEN M ELSE N/A |
| C308 | IF (O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY AND O_D_QR_SCANNING) THEN M ELSE N/A |
| C309 | IF (O_D_LPAD AND NOT O_D_ONLY_CELLULAR_CONNECTIVITY AND NOT O_D_QR_SCANNING) THEN M ELSE N/A |

- Table 6: Conditional Items Referenced by Table 6

Note: Conditions C0XX which are missing in Table 6 are present in an earlier version of SGP.23 but are not used in the current version.

## General Consideration

This section contains some general considerations about the test cases defined in this document. Note that some external test specifications are referred to in chapter 7. Consequently, the following sub sections SHALL only apply for test cases defined in sections 4 and 5 and 6.

### Test Case Definition

Test descriptions are independent.

For each test described in this document, a chapter provides a general description of the initial conditions applicable for the whole test. This description is completed by specific configurations to each individual sub-case.

It is implicitly assumed that all entities under test SHALL be compliant with the initial states described in Annex G. An initial state SHALL be considered as a pre-requisite to execute all the test cases described in this Test Plan.

After completing the test, the configuration is reset before the execution of the following test.

### Test Cases Format

Here is an explanation of the way to define the test cases in chapters 4, 5 and 6.

4.X.Y.Z Test Cases 4.X.Y.Z.1 TC_IUT_TestName1 General Initial Conditions. Entity; Description of the general initial condition. Entity1; Test case - general condition 1. Entity2; Test case - general condition 2. Test Sequence #01: Short Description Description of the aim of the test sequence N°1 Initial Conditions. Entity; Description of the initial condition. Entity1; Test sequence N°1 - initial condition 1. Entity2; Test sequence N°1 - initial condition 2. Step; Direction; Sequence / Description; Expected result. IC1; Entity1 → Entity2; Command or Message to send from Entity1 to Entity2; Expected result N°1.1. 1; Entity1 → Entity2; Command or Message to send from Entity1 to Entity2; 1- expected result N°1.2 2- expected result N°1.3. 2; Entity2 → Entity3; Command or Message to send from Entity2 to Entity3. Test Sequence #02 Description of the aim of the test sequence N°2 Step; Direction; Sequence / Description; Expected result. 1; Entity1 → Entity2; Command or Message to send from Entity1 to Entity2. 2; Entity2 → Entity3; Command or Message to send from Entity2 to Entity3; 1- expected result N°2.1 2- expected result N°2.2. 4.X.Y.Z.2 TC_IUT_TestName2 …

The test cases TC_IUT_TestName1 and TC_IUT_TestName2 are referenced in Table 5 that allows indicating the applicability of the tests.

In the test case TC_IUT_TestName1, the requirements REQ1 and REQ2 are respectively covered by the test sequences #01 and #02.

Note: For some test cases, requirements to be covered are not listed in the test sequences. In that case, references to sections in GSMA RSP Technical Specification [2] covered by the test sequences are indicated in the Conformance Requirements References section of the test case.

The test sequence #01 SHALL be executed if and only if these conditions are met:

Test case - general condition 1

Test case - general condition 2

Test sequence N°1 - initial condition 1

Test sequence N°1 - initial condition 2

The test sequence #02 SHALL be executed if and only if these conditions are met:

Test case - general condition 1

Test case - general condition 2

The tables defining the different initial conditions are optional.

Initial Conditions are intended to be reached dynamically using the Test Tool when possible.

No additional operation SHALL be done prior to the test sequence besides those indicated in the Initial Conditions (e.g. no other Profiles SHALL be present on the eUICC besides those defined in the Initial Conditions).

In the test sequence #01:

the step IC1 corresponds to an additional Initial Condition

in the step N°1, if the expected results N°1 and N°2 are validated, the requirement REQ1 (or a part of the REQ1) SHALL be considered as implemented

Note that all initial states (described in Annex G) SHALL be implemented by the entity under test whatever the test cases to execute.

In addition, following 2.2.1 sub sections present all information (e.g. Methods, Constants…) that MAY be referenced in test sequences.

After execution of each test sequence a clean-up procedure (CU) SHALL be executed to restore the IUT to the Common Initial State as defined in Annex G.

#### Methods and Procedures

A method is referenced as follow:

- 	MTD_NAME_OF_THE_METHOD(PARAM1, PARAM2…)

The key word “NO_PARAM” SHALL be set in method call if the related optional parameter is not used.

All methods and their related parameters are described in Annex C.1.

A procedure is a generic sub-sequence and is referenced as follow:

PROC_NAME_OF_THE_PROCEDURE

All procedures are described in Annex C.2.

The implementation of these methods and procedures is under the responsibility of the test tool providers.

#### Constants and Dynamic Content

A constant (e.g. text, ASN.1 structure, hexadecimal string, icon, URI, integer, EID, AID…) is referenced as follow:

- 	#NAME_OF_THE_CONSTANT

All constants are defined in Annex A.

When provided as an ASN.1 value notation, a constant SHALL be encoded in DER TLV (as specified in ITU-T X.690 [16]) by the test tool.

A dynamic content (e.g. TLV, ASN.1 structure, signature, integer, AID, one-time key pair…) is referenced as follow:

- 	<NAME_OF_THE_VARIABLE>

All dynamic contents are defined in Annex B.

A dynamic content is either generated by an IUT or by a test tool provider.

#### Requests and Responses

An ASN.1 or a JSON request is referenced as follow:

- 	#NAME_OF_THE_REQUEST

An ASN.1 or a JSON response is referenced as follows:

- 	#R_NAME_OF_THE_RESPONSE

Each ASN.1 or JSON request and response MAY refer to a constant or a dynamic content. All these structures are defined in Annex D.

When provided as an ASN.1 value notation, a request or a response SHALL be encoded in DER TLV (as specified in ITU-T X.690 [16]) by the test tool.

When an ASN.1 element definition contains three points (i.e. “…”), it means that fields MAY be present but SHALL not be checked by the test tool.

In the following example, several fields MAY be part of the ProfileInfoListResponse but only the profileNickname SHALL be verified.

resp ProfileInfoListResponse ::=

profileInfoListOk :{

{ 
       ...
       profileNickname #NICKNAME
       ...

}

}

This rule applies also for Constants definition.

Some ASN.1 SEQUENCE components have a DEFAULT value (for example, profileClass in StoreMetadataRequest). In this specification, when values are specified in ASN.1 syntax and the DEFAULT value is intended, two different formulations (both of which are valid) may be used:

the relevant component is specified with the DEFAULT value;

the relevant component is missing entirely.

These are logically equivalent and lead to the same DER encoding. In both cases, the following rules apply:

When the test tool is sending the DER value, it SHALL NOT include the component (as per DER rules).

When the test tool is checking a received DER value from the entity under test, it SHALL check that the component is NOT present.

Test tools SHALL consider two BIT STRINGs to be equivalent if the BIT STRINGs have the same DER encoding. For example, '0101'B shall be considered to be equivalent to '010100'B.

NOTE:	this is equivalent to removing any trailing zero bits from the BIT STRINGs in "bstring" notation (e.g. '010100'B  '0101'B) and then comparing the strings textually.

NOTE:	according to the DER format, the encoding of transmitted values will remove the trailing zeroes. The definition above allows for values which are specified using ASN.1 value notation and are not transmitted, such as values specified in the Annexes of the current document, including IUT settings which might be specified by a user of the current document and may contain trailing zeroes in the ASN.1 value notation.

#### APDUs

A C-APDU is referenced as follow:

- 	[NAME_OF_THE_CAPDU]

All C-APDUs are defined in Annex D.4.

An R-APDU is referenced as follow:

- 	[R_NAME_OF_THE_RAPDU]

All R-APDUs are defined in Annex D.4.

Each APDU MAY refer to a constant or a dynamic content.

The APDU TERMINAL RESPONSE SHALL be dynamically generated by the test tool according to the related proactive command. Therefore, this particular command is not referenced with brackets in this specification. If not explicitly defined in the step, the general result SHALL be set by default to “Command performed successfully” (i.e. 0x83 01 00).

#### Profiles

In order to execute the test cases described in this document, Operational, Test and Provisioning Profiles are necessary. All these Profiles are defined in Annex E with the Profile Metadata content and the corresponding Profile Package as defined in the eUICC Profile Package Specification [4].

A Profile is referenced as follow:

- 	PROFILE_OPERATIONALx with x the identifier of the Operational Profile

or

- 	PROFILE_TESTx with x the identifier of the Test Profile

or

- 	PROFILE_PROVISIONINGx with x the identifier of the Provisioning Profile

NOTE:	Test Profiles and Provisioning Profiles are out of the scope of this version of test specification.

#### IUT Settings

For the purpose of some test cases, Device and eUICC manufacturers and Platforms (i.e. SM-DP+, SM-DS) providers need to give some information related to their products to the test tools providers (e.g. supported Java Card version).

An IUT setting is referenced as follow:

- 	#IUT_NAME_OF_SETTING

All these settings are defined in Annex F.

#### Referenced Requirements

All requirements referenced in this document by their identifiers are present and described in Annex I. These requirements have been extracted from the specifications:

- 	GSMA RSP Technical Specification [2]

- 	GSMA RSP Architecture [3]

### VOID

### General Rules for Device Testing

#### Default Profile Download, install and enable Process on the Device Under Test

By default, when an Operational Profile needs to be downloaded, installed (and if necessary enabled) on the (Test) eUICC resided in the Device Under Test (e.g. as mentioned in an initial condition), the following rules apply except if it is defined differently in the Test Case.

The default way to execute the Profile download SHALL be the Add Profile procedure with Activation Code #ACTIVATION_CODE_1. The way to apply the Activation Code (manual typing or QR code scanning) depends on the Device/LPAd implementation. In order to execute the Common Mutual Authentication procedure and the Sub-procedure Profile Download and Installation (End User Confirmation), the following responses SHALL be sent by the S_SM-DP+:

- 	#INITIATE_AUTH_OK

with the <EUICC_CI_PK_ID_TO_BE_USED> set to the CI for signing indicated as highest priority in euiccCiPKIdListForSigning in the <R_EUICC_INFO1>

with the #CERT_S_SM_DPauth_SIG leading to the same CI as the one chosen for signing

with the SM-DP+ address #TEST_DP_ADDRESS1

- 	#AUTH_CLIENT_OK

with the #CERT_S_SM_DPpb_SIG leading to the same CI as the one chosen for signing

Metadata of the downloaded Profile instead of #METADATA_OP_PROF1

- 	#GET_BPP_OK with the content of the installed Profile (no session keys used)

Before running a test sequence, and after establishing the Initial conditions, all pending Notifications (sent on the best-effort basis as soon as connectivity is available as defined in section 3.5 of SGP.22 [2]) SHALL have been acknowledged by the simulated SM-DP+(s). S_SM-DP+(s) SHALL be run with suitable addresses in order to receive and acknowledge all pending Notifications (including install, enable, disable and delete). The addresses which are required depend on the server address used for recent profile downloads (typically #TEST_DP_ADDRESS1 to receive and acknowledge PIR), and the notificationAddress values in the Metadata of recently downloaded Profiles (for otherSignedNotification). Each S_SM_DP+ SHALL use the TLS certificate corresponding to its address (CERT_S_SM_DP_TLS, CERT_S_SM_DP2_TLS, etc).

If only O_D_ADD_ENABLE_COMBINED is supported by the DUT, the user might have to perform actions in a particular manner in order to achieve the initial conditions related to enabled/disabled state of profiles (for example: disable a profile after installing, install profiles in a particular order, re-enable an initial profile after installing a subsequent profile).

If the test case requires a Profile Download to be initiated via SM-DS:

The mechanism used to initiate this is device-specific.

If the device is using Power-on Profile Discovery the following applies:

when it is supported, the value of the configuration parameter for Device Power-on Profile discovery is 'Enabled'.

the Device has to be powered-off and then powered-on before each test sequence.

#### LUI Settings and Result Verification Criteria

Some Initial Conditions require the “The protection of access to the LUI is disabled” setting. It means that no protection mechanism is enforced upon entry to the LUI as defined in SGP.22 [2].

The way to perform Strong Confirmation SHALL be executed by the S_EndUser according to the description provided by the Device Vendor in #IUT_LPAd_Confirmation.

For operations for which SGP.21 [3] and SGP.22 [2] do not require Confirmation – i.e. only User Intent is required (for example, Enable Profile, Disable Profile, Set/Edit Nickname): if the Device requests Confirmation from the User, the Test Tool SHALL NOT treat this as a failure.

For operations for which SGP.21 [3] and SGP.22 [2] require Simple Confirmation: if the Device requests Strong Confirmation from the User, the Test Tool SHALL NOT treat this as a failure.

Some of the Expected Results on the IUT side expect “No Error”. In this case the Test Tool SHALL verify that there is no error message appears on the UI of the DUT.

The End User SHALL follow the LUI requests to successfully complete the Profile Download process. Any combined confirmation for consecutive Local Profile Management Operations SHALL be avoided by the End User unless it is explicitly required by the test procedure. E.g.: upon installation of a new Profile, the LPA could propose ‘add Profile’ and ‘enable’ into one single step with a single confirmation only (e.g. “Do you want to install Profile ‘ProfileName’ on your Device and enable it? Yes / No / Install only”) In this case the End User will select the confirmation only for the single actual operation (i.e. select “Install only”).

NOTE:	When combined Add and Enable Profile operations are to be initiated, various device implementations are possible. Examples (non-exhaustive):

- The user initiates the Add Profile operation first, with the Enable operation being incorporated later in the process, for example, at the confirmation stage.

- The user initiates a composite "Add and Enable Profile" operation at the start of the process.

If a test sequence requires Add Profile initiation and only O_D_ADD_ENABLE_COMBINED is supported by the DUT, then Add Profile initiation SHALL be interpreted to mean that the combined Add and Enable Profile operations are to be initiated, taking into account the note above regarding various device implementations.

#### TLS Testing Rules and Recommendations

The TLS connection may be rejected either:

- by sending a TLS alert, or

- by closing of the TCP connection, though TLS handshake completed, or

- TLS handshake not completed without sending a TLS alert, or

- No further RSP communication has been initiated by LPAd on ES9+/ES11 within the #IUT_LPAd_SESSION_CLOSE_TIMEOUT

Please note that this is not an exhaustive list, and acting as guidelines for the test tools.

Unless it is defined differently in test case, the S_SM-DP or S_SM_DS is configured to respond to the “ClientHello” in the following way:

- Certificates that chain to eSIM CA SHALL be used, even if the received ClientHello message contains the "server_name" extension with the v3-specific FQDN

### Pass Criteria

A test execution is considered as successful only if the test procedure was fully carried out successfully.

A test execution is considered as failed if the tested feature provides an unexpected behaviour.

A test execution is considered as inconclusive when the pass criteria cannot be evaluated due to issues during the setup of the initial conditions (including the ICx steps) or during the execution of steps in which no requirement is referenced.

### Future Study

Some of the test cases or test sequences described in this Test Plan are FFS (For Future Study). This MAY mean that some clarifications are expected at the requirement level to conclude on a test method. As consequence, the corresponding test SHALL not be executed.

### VOID

# 3	Testing Architecture

## 3.1	Testing Scope

All the interfaces, intended to be tested in the scope of this document, are presented hereafter:

| Interface | Between |  | Description |
| --- | --- | --- | --- |
| ES2+ | Operator | SM-DP+ | Used by the Operator to order Profiles for specific eUICCs as well as other administrative functions. NOTE: this interface is out of scope of this specification. |
| ES8+ | SM-DP+ | eUICC | Provides a secure end-to-end channel between the SM-DP+ and the eUICC for the administration of the ISD-P and the associated Profile during download and installation. It provides Perfect Forward Secrecy. |
| ES9+ | SM-DP+ | LPD | Used to provide a secure transport between the SM-DP+ and the LPA (LPD) for the delivery of the Bound Profile Package and the delivery of Remote Profile Management Commands. |
| ES11 | LDS | SM-DS | Used by the LDS to retrieve Event Records for the respective eUICC. |

- Table 7: Interfaces Descriptions

## 3.2	Testing Execution

This chapter aims to describe the different testing environments and equipments to allow the test cases to be executed.

To permit the execution of the different test cases described in this Test Plan, specific simulators SHALL be used. The simulators that have been defined are listed hereafter:

- 	S_SM-DP+: the SM-DP+ Simulator

- 	S_SM-DS: the SM-DS Simulator

- 	S_EndUser: the End User Simulator that acts as an End User. This simulator MAY be either a person (i.e. a Tester) or a software that simulates the End User interactions.

- 	S_SERVER: the HTTPs server Simulator for the purpose of TLS testing. The S_SERVER MAY be S_SM-DP+ or S_SM-DS depending on the component under test.

- 	Implementation of these simulators remains under the responsibility of the test tool providers.

- 	The aim of all the test cases is to verify the compliance of an Actor/Component (i.e. LPAd, Device).

Following notations are used:

- 	S_ComponentName for a simulated component

- 	ComponentName for the Implementation Under Test (IUT)

- 	Where ComponentName is indicated by CLIENT, SERVER

- 	Depending on the component under test, the CLIENT MAY be the SM-DP+ or the SM-DS. The Operator component is currently out of scope.

- Depending on the component under test, the SERVER MAY be the SM-DP+ or the SM-DS. The Operator component is currently out of scope.

- The use of "-- optional" in any ASN.1 elements defined within this document indicate that the test tool SHALL allow for the value either being present with that value, or being absent.

### 3.2.1	VOID

### 3.2.2	VOID

### 3.2.3	Device/LPAd - Test Environment

The following test environment is used for all LPAd Interfaces related test cases as defined in chapter 4.4 and 5.4 (unless it is specified differently in the specific test case). Following conditions apply:

- 	The Device contains an eUICC configured with Test Certificates and Test Keys

- 	The Test eUICC is either soldered or removable. In case the eUICC is removable, it SHALL NOT be removed during testing

- 	The Test eUICC is only used for LPAd testing and SHALL not be considered as an IUT

- 	The Test eUICC SHALL not support LPAe

- 	The Test eUICC SHALL be compliant with the GSMA RSP Technical Specification [2]

- 	SM-DP+ Simulator(s) SHALL be implemented by the test tools

- 	SM-DS Simulator(s) SHALL be implemented by the test tools

- 	End User Simulator SHALL be used (S_EndUser)

- 	No modification of the Device HW is required

- 	If the IUT is a Companion Device it has to be connected to a Primary Device as defined by the Device Vendor. The Device Vendor SHALL provide detailed information about which RSP functionality requires a Primary Device.

- 	No modification of the Device OS is required for the usage of S_EndUser

- 	Test Root Certificate SHALL be configured in the Device

#### 3.2.3.1	General (Device/LPAd) Test Environment

The Test Environment consists of:

- 	IUT: Device, or Companion Device supporting the LPAd with a Test eUICC connected to a Primary Device

- 	S_SM-DP+: a simulated SM-DP+ supporting a connection used by the Device to establish ES9+, (ES8+)

- 	S_SM-DS: a simulated SM-DS supporting a connection used by the Device to establish ES11

- 	S_EndUser

In case the Device supports a connection method different from Cellular Network it is expected that this connection method is used.

NOTE: 	Device that supports only Cellular Networks is out of scope for this specification.

#### 3.2.3.2	Device – Test Environment

If the IUT is a Device as defined in SGP21/SGP.22 [2] it SHALL provide at least one method to establish the IP connection to the S_SM-DP+, or S_SM-DS.

When executing a test case with an IUT matching this definition, default Initial States as defined in G.1.1 apply unless it is specified differently in the specific test case.

#### 3.2.3.3	Companion Device connected to a Primary Device – Test Environment

The Companion Device is connected to a Primary Device.

Device Vendors SHALL provide the mechanism to connect the Primary Device to the Companion Device.

User interaction and the verification of User Intents can be performed on the User Interface of the Primary Device or the companion Device.

The Companion Device MAY connect to the S_SM-DP+, or S_SM-DS directly, or MAY use a connection offered by the Primary Device.

To connect to the SM-DP+ or the SM-DS the Companion Device uses a connection offered by the Primary Device.

Initial State as defined in G.1.2 applies unless otherwise stated in the test case.

### 3.2.4	VOID

### 3.2.5	VOID

# 4	Interface Compliance Testing

## 4.1	General Overview

This section focuses on the implementation of the different interfaces according to the GSMA RSP Technical Specification [2]. The aim is to verify the compliance of all interfaces within the system.

## 4.2	VOID

## 4.3	VOID

## 4.4	LPAd Interfaces

### 4.4.1	ES10a (LPA -- eUICC): GetEuiccConfiguredAddresses

This test case is defined as FFS and not applicable for this version of test specification.

### 4.4.2	ES10a (LPA -- eUICC): SetDefaultDpAddress

This test case is defined as FFS and not applicable for this version of test specification.

### 4.4.3	ES10b (LPA -- eUICC): PrepareDownload

This test case is defined as FFS and not applicable for this version of test specification.

### 4.4.4	ES10b (LPA -- eUICC): LoadBoundProfilePackage

This test case is defined as FFS and not applicable for this version of test specification.

### 4.4.5	ES10b (LPA -- eUICC): GetEUICCChallenge

This test case is defined as FFS and not applicable for this version of test specification.

### 4.4.6	ES10b (LPA -- eUICC): GetEUICCInfo

This test case is defined as FFS and not applicable for this version of test specification.

### 4.4.7	ES10b (LPA -- eUICC): ListNotification

This test case is defined as FFS and not applicable for this version of test specification.

### 4.4.8	ES10b (LPA -- eUICC): RetrieveNotificationsList

This test case is defined as FFS and not applicable for this version of test specification.

### 4.4.9	ES10b (LPA -- eUICC): RemoveNotificationFromList

This test case is defined as FFS and not applicable for this version of test specification.

### 4.4.10	ES10b (LPA -- eUICC): LoadCRL

This test case is defined as FFS and not applicable for this version of test specification.

### 4.4.11	ES10b (LPA -- eUICC): AuthenticateServer

This test case is defined as FFS and not applicable for this version of test specification.

### 4.4.12	ES10b (LPA -- eUICC): CancelSession

This test case is defined as FFS and not applicable for this version of test specification.

### 4.4.13	ES10c (LPA -- eUICC): GetProfilesInfo

This test case is defined as FFS and not applicable for this version of test specification.

### 4.4.14	ES10c (LPA -- eUICC): EnableProfile

This test case is defined as FFS and not applicable for this version of test specification.

### 4.4.15	ES10c (LPA -- eUICC): DisableProfile

This test case is defined as FFS and not applicable for this version of test specification.

### 4.4.16	ES10c (LPA -- eUICC): DeleteProfile

This test case is defined as FFS and not applicable for this version of test specification.

### 4.4.17	ES10c (LPA -- eUICC): eUICCMemoryReset

This test case is defined as FFS and not applicable for this version of test specification.

### 4.4.18	ES10c (LPA -- eUICC): GetEID

This test case is defined as FFS and not applicable for this version of test specification.

### 4.4.19	ES10c (LPA -- eUICC): SetNickname

This test case is defined as FFS and not applicable for this version of test specification.

### 4.4.20	ES10b (LPA -- eUICC): GetRAT

This test case is defined as FFS and not applicable for this version of test specification.

### 4.4.21	ES9+ (LPA -- SM-DP+): InitiateAuthentication

#### 4.4.21.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

Section 2.1, 2.4a.1.3

Section 2.6.6.2, 2.10.1

Section 3.0.1, 3.1.3, 3.7.2, 3.7.3

Section 5.6.1, 5.6.3, 5.7.14a

Section 6.2

Section 6.3

Section 6.5.1, 6.5.1.1, 6.5.1.2, 6.5.1.3, 6.5.1.4, 6.5.2, 6.5.2.6

Section 6.6.2.2

#### 4.4.21.2	Test Cases

##### 4.4.21.2.1	TC_LPAd_InitiateAuthentication_Nominal

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1). |
| eUICC | There is no default SM-DP+ address configured. |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |

Test Sequence #01 Nominal: Initiate Authentication

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, <LPA_RSP_CAPABILITY>)) • Extract <EUICC_CHALLENGE> Verify if: - • <LPA_RSP_CAPABILITY> contains - crlStaplingV3Support set to '1' - certChainV3Support set to '1' - signedSmdsResponseV3Support set to '1' |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_OK) | Next step of common mutual authentication procedure is performed (i.e. AuthenticateClient request is sent). |

##### 4.4.21.2.2	TC_LPAd_InitiateAuthentication_ErrorCases

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1). |
| Device | The protection of access to the LUI is disabled. |
| eUICC | There is no default SM-DP+ address configured. |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |

Test Sequence #01 Error: Invalid SM-DP+ Address

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, <LPA_RSP_CAPABILITY>)) |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_8_1_3_8) | LPAd aborts AddProfile procedure |
| 2 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.InitiateAuthentication requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #02 Error: Unsupported Security Configuration

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, <LPA_RSP_CAPABILITY>)) |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_8_2_3_1) | LPAd aborts AddProfile procedure |
| 2 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.InitiateAuthentication requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #03 Error: Unsupported SVN

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, <LPA_RSP_CAPABILITY>)) |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_8_3_3_1) | LPAd aborts AddProfile procedure RQ56_008 RQ56_011 |
| 2 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.InitiateAuthentication requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #04 Error: Unavailable SM-DP+ Certificate

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, <LPA_RSP_CAPABILITY>)) |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_8_4_3_7) | LPAd aborts AddProfile procedure |
| 2 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.InitiateAuthentication requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #05 Error:  Invalid SM-DP+ Certificate

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, <LPA_RSP_CAPABILITY>)) |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_INV_CERT) | LPAd aborts AddProfile procedure |
| 2 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.InitiateAuthentication or ES9+.AuthenticateClient requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #06 Error:  Invalid SM-DP+ Signature

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, <LPA_RSP_CAPABILITY>)) |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_INV_SIGN) | LPAd aborts AddProfile procedure |
| 2 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.InitiateAuthentication or ES9+.AuthenticateClient requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #07 Error: Invalid SM-DP+ Address sent by the SM-DP+

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, <LPA_RSP_CAPABILITY>)) |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_INV_SMDP+_ADDRESS) | LPAd informs the S_EndUser and aborts the AddProfile procedure |
| 2 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.InitiateAuthentication or ES9+.AuthenticateClient requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #08 Error: Unsupported CI Key ID

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, <LPA_RSP_CAPABILITY>)) |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_INV_CI) | LPAd aborts AddProfile procedure |
| 2 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.InitiateAuthentication or ES9+.AuthenticateClient requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #09 Error: Invalid SM-DP+ OID

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated, #ACTIVATION_CODE_2 is provided to the LPAd on request from the S_EndUser. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_2 (PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, <LPA_RSP_CAPABILITY>)) |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_INV_OID) | LPAd informs the S_EndUser and aborts the AddProfile procedure |
| 2 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.InitiateAuthentication or ES9+.AuthenticateClient requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

##### 4.4.21.2.3	TC_LPAd_InitiateAuthentication_Nominal_V3

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) |
| eUICC | There is no default SM-DP+ address configured |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |
| S_SM-DP+ | Variant A certificates are included in certificates chain for TLS procedures. |

Test Sequence #01 Nominal: Initiate Authentication – V3 Variant

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, #IUT_LPA_RSP_CAPABILITY) ) • Extract <EUICC_CHALLENGE> |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_OK_VARIANT_FLEX) | Next step of common mutual authentication procedure is performed (i.e. AuthenticateClient request is sent). |

##### 4.4.21.2.4	    TC_LPAd_InitiateAuthentication_ErrorCases_V3

TBD

##### 4.4.21.2.5     TC_LPAd_ InitiateAuthentication_Norminal_RPM

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled |
| eUICC | There is no default SM-DP+ address configured The eUICC supports RPM |
| LPAd | RPM operation is enabled in the LPA by the End User |
| S_SM-DP+ | Variant A certificates are included in certificates chain for TLS procedures. |

Test Sequence #01 Nominal: Initiate Authentication for RPM – Variant A, with Polling Address and allowed GSMA CI RootCA public key identifier

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_CONF_EN_CI_PKI_RPM is loaded and disabled. eUICC returns CTX_PARAMS1_RPM_ICCID1 (sends by the LPAd) in euiccSigned1 in the response to ES10.AuthenticateServer. |
| LPAd | Update Profile operation is initiated and PROFILE_OPERATIONAL1 is selected. Polling Address and the Allowed GSMA CI RootCA public key identifier (#EUICC_CI_PKI_RPM) are retrieved from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, #IUT_LPA_RSP_CAPABILITY) ) • Extract <EUICC_CHALLENGE> |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_OK_VARIANT_A) | Next step of common mutual authentication procedure is performed (i.e. AuthenticateClient request is sent). |

Test Sequence #02 Nominal: Initiate Authentication for RPM – Default SM-DP+ Address and allowed GSMA CI RootCA public key identifier

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with # METADATA_OP_PROF1_RPM_CONF_EN_NO_POLLING_ADDRESS is loaded and disabled. eUICC returns CTX_PARAMS1_RPM_ICCID1 (sends by the LPAd) in euiccSigned1 in the response to ES10.AuthenticateServer. |
| LPAd | Update Profile operation is initiated and PROFILE_OPERATIONAL1 is selected. Default SM-DP+ Address (#TEST_DP_ADDRESS1) and the Allowed GSMA CI RootCA public key identifier (#EUICC_CI_PKI_RPM) are set using ES10a.SetDefaultDpAddress and retrieved using the ES10a.GetEuiccConfiguredData. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, #IUT_LPA_RSP_CAPABILITY) ) • Extract <EUICC_CHALLENGE> |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_OK_VARIANT_A) | Next step of common mutual authentication procedure is performed (i.e. AuthenticateClient request is sent). |

Test Sequence #03 Nominal: Initiate Authentication for RPM – Root SM-DS Address as Polling Address and allowed GSMA CI RootCA public key identifier

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_CONF_EN_POL_SMDS_CI_PKI_RPM is loaded and disabled. eUICC returns CTX_PARAMS1_RPM_ICCID1 (sent by the LPAd) in euiccSigned1 in the response to ES10.AuthenticateServer. |
| LPAd | Update Profile operation is initiated and PROFILE_OPERATIONAL1 is selected. Polling Address and the Allowed GSMA CI RootCA public key identifier (#PK_CI_ECDSA_RPM) are retrieved from the ProfileInfo, and the Polling Address indicates the S_SM-DS address #TEST_ROOT_DS_ADDRESS. |
| S_SM-DS | S_SM-DP+ (#TEST_DP_ADDRESS1) performed RPM Download Event Registration to the S_SM-DS (#TEST_ROOT_DS_ADDRESS) with #EVENT_ID_1 |
| S_SM-DP+ | There is a pending RPM package order for #EVENT_ID_1 (PROFILE_OPERATIONAL1) |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES11 |  |  |
| 1 | LPAd → S_SM-DS | Send ES11.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_ROOT_DS_ADDRESS, #IUT_LPA_RSP_CAPABILITY) ) • Extract <EUICC_CHALLENGE> |
| 2 | S_SM-DS → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_DS_OK_VARIANT_A) | Next step of common mutual authentication procedure is performed (i.e. AuthenticateClient request is sent). |

Test Sequence #04 Nominal: Initiate Authentication for RPM –  Root SM-DS Address and no default SM-DP+ address configured

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_CONF_EN_NO_POLLING_ADDRESS is loaded and disabled. eUICC returns CTX_PARAMS1_RPM_ICCID1 (sent by the LPAd) in euiccSigned1 in the response to ES10.AuthenticateServer. |
| eUICC | S_SM_DS address #TEST_ROOT_DS_ADDRESS is configured in eUICC and no Default S_SM-DP+ is set in eUICC. |
| LPAd | Update Profile operation is initiated and PROFILE_OPERATIONAL1 is selected. The Root SM-DS Address (#TEST_ROOT_DS_ADDRESS) is retrieved using the ES10a.GetEuiccConfiguredData. |
| S_SM-DS | S_SM-DP+ (#TEST_DP_ADDRESS1) performed RPM Download Event Registration to the S_SM-DS (#TEST_ROOT_DS_ADDRESS) with #EVENT_ID_1 |
| S_SM-DP+ | There is a pending RPM package order for #EVENT_ID_1 (PROFILE_OPERATIONAL1) |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES11 |  |  |
| 1 | LPAd → S_SM-DS | Send ES11.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_ROOT_DS_ADDRESS, #IUT_LPA_RSP_CAPABILITY) ) • Extract <EUICC_CHALLENGE> |
| 2 | S_SM-DS → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_DS_OK_VARIANT_A) | Next step of common mutual authentication procedure is performed (i.e. AuthenticateClient request is sent). |

Test Sequence #05 Nominal: Initiate Authentication for RPM –  previous Profile Download session with pending RPM

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | Installation of PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_CONF_EN_POL_DP8 is triggered using the Activation Code (#ACTIVATION_CODE_1) with S_SM-DP+ address (#TEST_DP_ADDRESS1). |
| S_SM-DP+ | There is a pending RPM package order for (PROFILE_OPERATIONAL1). During Mutual Authentication for the Profile Download S_SM-DP+ sends the AuthenticateClientOk with #S_SMDP_SIGNED2_RPM_PENDING. |
| eUICC | PROFILE_OPERATIONAL1 with is loaded and is disabled. |
| LPAd | No Update Profile operation is initiated for RPM download and it is triggered by the rpmPending received from the Profile Download session. |
| eUICC | eUICC returns CTX_PARAMS1_RPM_ICCID1 (sent by the LPAd) in euiccSigned1 in the response to ES10.AuthenticateServer for RPM download. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9 |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, #IUT_LPA_RSP_CAPABILITY) ) • Extract <EUICC_CHALLENGE> |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_OK_VARIANT_A) | Next step of common mutual authentication procedure is performed (i.e. AuthenticateClient request is sent). |

Test Sequence #06 Nominal: Initiate Authentication for RPM –  previous RPM Download session with pending RPM

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_CONF_EN_POL_DP8 is loaded and is disabled |
| LPAd | Update Profile operation for RPM download to enable PROFILE_OPERATIONAL1 is initiated. |
| S_SM-DP+ | There is a pending RPM package, Enable profile order for (PROFILE_OPERATIONAL1). During Mutual Authentication for the RPM Download S_SM-DP+ sends the #AUTH_CLIENT_RPM_OK_AND_RPM_PENDING. S_SM-DP+ has another pending RPM package for (PROFILE_OPERATIONAL1), eg: Disable profile order. |
| eUICC | PROFILE_OPERATIONAL1 is enabled using the RPM download. |
| LPAd | No Update Profile operation is initiated again for the pending RPM download (it is triggered by the rpmPending received from the previous RPM Download session). |
| eUICC | eUICC returns CTX_PARAMS1_RPM_ICCID1 (sent by the LPAd) in euiccSigned1 in the response to ES10.AuthenticateServer for RPM download. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9 |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, #IUT_LPA_RSP_CAPABILITY) ) • Extract <EUICC_CHALLENGE> |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_OK_VARIANT_A) | Next step of common mutual authentication procedure is performed (i.e. AuthenticateClient request is sent). |

##### 4.4.21.2.6     TC_LPAd_ InitiateAuthentication_ErrorCases_RPM

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled |
| eUICC | There is no default SM-DP+ address configured The eUICC supports RPM |
| LPAd | RPM operation is enabled in the LPA by the End User |
| S_SM-DP+ | Variant A certificates are included in certificates chain for TLS procedures. |

Test Sequence #01 Error: InitiateAuthentication for RPM, Polling Address and not matching allowed GSMA CI Root CA public key identifier

The purpose of this test sequence is to ensure that LPAd stops the RPM procedure if there is a restriction to a single allowed GSMA CI RootCA public key identifier and it does not match the Subject Key Identifier of the GSMA Root CI corresponding to CERT.XXauth.SIG.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_CONF_EN_CI_PKI_RAND is loaded and disabled. eUICC returns CTX_PARAMS1_RPM_ICCID1 (sends by the LPAd) in euiccSigned1 in the response to ES10.AuthenticateServer. |
| LPAd | Update Profile operation is initiated and PROFILE_OPERATIONAL1 is selected. Polling Address and the Allowed GSMA CI RootCA public key identifier (#EUICC_CI_PKI_RANDOM) are retrieved from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, #IUT_LPA_RSP_CAPABILITY) ) • Extract <EUICC_CHALLENGE> |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_OK_VARIANT_A) | LPAd aborts Update Profile procedure. |
| 3 | LPAd → S_SM-DP+ | No retry of Update Profile operation | No ES9+.InitiateAuthentication requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. |

Test Sequence #02 Error: InitiateAuthentication for RPM, Default SM-DP+ Address and not matching allowed GSMA CI Root CA public key identifier

The purpose of this test sequence is to ensure that LPAd stops the RPM procedure if there is a restriction to a single allowed GSMA CI RootCA public key identifier and it does not match the Subject Key Identifier of the GSMA Root CI corresponding to CERT.XXauth.SIG.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_CONF_EN_NO_POLLING_ADDRESS is loaded and disabled. eUICC returns CTX_PARAMS1_RPM_ICCID1 (sends by the LPAd) in euiccSigned1 in the response to ES10.AuthenticateServer. |
| LPAd | Update Profile operation is initiated and PROFILE_OPERATIONAL1 is selected. Default SM-DP+ Address (#TEST_DP_ADDRESS1) and the Allowed GSMA CI RootCA public key identifier (#EUICC_CI_PKI_RANDOM) are set using ES10a.SetDefaultDpAddress and retrieved using the ES10a.GetEuiccConfiguredData. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, #IUT_LPA_RSP_CAPABILITY) ) • Extract <EUICC_CHALLENGE> |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_OK_VARIANT_A) | LPAd aborts Update Profile procedure. |
| 3 | LPAd → S_SM-DP+ | No retry of Update Profile operation | No ES9+.InitiateAuthentication requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. |

Test Sequence #03 Error: InitiateAuthentication for RPM, Root SM-DS Address as Polling Address and not matching allowed GSMA CI Root CA public key identifier

The purpose of this test sequence is to ensure that LPAd stops the RPM procedure if there is a restriction to a single allowed GSMA CI RootCA public key identifier and it does not match the Subject Key Identifier of the GSMA Root CI corresponding to CERT.XXauth.SIG.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_CONF_EN_POL_SMDS_CI_PKI_RAND is loaded and disabled. eUICC returns CTX_PARAMS1_RPM_ICCID1 (sent by the LPAd) in euiccSigned1 in the response to ES10.AuthenticateServer. |
| LPAd | Update Profile operation is initiated and PROFILE_OPERATIONAL1 is selected. Polling Address and the Allowed GSMA CI RootCA public key identifier (#PK_CI_ECDSA_RANDOM) are retrieved from the ProfileInfo, and the Polling Address indicates the S_SM-DS address #TEST_ROOT_DS_ADDRESS. |
| S_SM-DS | S_SM-DP+ (#TEST_DP_ADDRESS1) performed RPM Download Event Registration to the S_SM-DS (#TEST_ROOT_DS_ADDRESS) with #EVENT_ID_1 |
| S_SM-DP+ | There is a pending RPM package order for #EVENT_ID_1 (PROFILE_OPERATIONAL1) |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES11 |  |  |
| 1 | LPAd → S_SM-DS | Send ES11.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_ROOT_DS_ADDRESS, #IUT_LPA_RSP_CAPABILITY) ) • Extract <EUICC_CHALLENGE> |
| 2 | S_SM-DS → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_DS_OK_VARIANT_A) | LPAd aborts Update Profile procedure. |
| 3 | LPAd → S_SM-DS | No retry of Update Profile operation | No ES9+.InitiateAuthentication requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. |

Test Sequence #04 Error: InitiateAuthentication for RPM, Pending RPM in previous RSP Session and not matching GSMA CI Root CA public key indicator in AC

The purpose of this test sequence is to ensure that LPAd stops the RPM procedure if there is a restriction to a single allowed GSMA CI RootCA public key identifier and it does not match the Subject Key Identifier of the GSMA Root CI corresponding to CERT.XXauth.SIG while using the SM-DP+ address from previous RSP Session.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | Installation of PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_CONF_EN_POL_DP8 is triggered using the Activation Code (#ACTIVATION_CODE_1) with S_SM-DP+ address (#TEST_DP_ADDRESS1). |
| S_SM-DP+ | There is a pending RPM package order for (PROFILE_OPERATIONAL1). During Mutual Authentication for the Profile Download S_SM-DP+ sends, - #CERT_S_SM_DPauth_ECDSA in Initiate Authentication response. the AuthenticateClientOk with #S_SMDP_SIGNED2_RPM_PENDING. |
| eUICC | PROFILE_OPERATIONAL1 with is loaded and is disabled. |
| LPAd | No Update Profile operation is initiated for RPM download and it is triggered by the rpmPending received from the Profile Download session. |
| S_SM-DP+ | During Mutual Authentication for the RPM Download S_SM-DP+ sends #CERT_S_SM_DPauth_PK_CI2_ECDSA in Initiate Authentication response. |
| eUICC | eUICC returns CTX_PARAMS1_RPM_ICCID1 (sent by the LPAd) in euiccSigned1 in the response to ES10.AuthenticateServer for RPM download. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, #IUT_LPA_RSP_CAPABILITY) ) • Extract <EUICC_CHALLENGE> |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_OK_DIFF_CERT_VARIANT_A) | LPAd aborts Update Profile procedure. |
| 3 | LPAd → S_SM-DP+ | No retry of Update Profile operation | No ES9+.InitiateAuthentication requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. |

### 4.4.22	ES9+ (LPA -- SM-DP+): GetBoundProfilePackage

#### 4.4.22.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Requirements

- 

- 	Section 3.0.1, 3.1.3,

- 	Section 5.6.2,

- 	Section 6.5, 6.5.2.7

#### 4.4.22.2	Test Cases

##### 4.4.22.2.1	TC_LPAd_ES9+_GetBoundProfilePackage_Nominal

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| eUICC | There is no default SM-DP+ address configured. |

Test Sequence #01 Nominal: Get BPP using S-ENC and S-MAC without Confirmation Code

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.GetBoundProfilePackage method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_GET_BPP, MTD_GET_BPP(<S_TRANSACTION_ID>, #R_PREP_DOWNLOAD_NO_CC)) Verify: • If <S_TRANSACTION_ID> is the same as in #R_PREP_DOWNLOAD_NO_CC • <EUICC_SIGNATURE2> using the #PK_EUICC_SIG |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#GET_BPP_OK) | No error, see NOTE. |
| NOTE: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL confirm End User Intent if requested. |  |  |  |

Test Sequence #02 Nominal: Get BPP using S-ENC and S-MAC with Confirmation Code

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_3. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_3 (associated with PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT_CC |  |  |
| IC4 | LPAd → S_EndUser | LPAd requests the Confirmation Code from the S_EndUser. | #CONFIRMATION_CODE1 is provided by manual entry. |
| 1 | LPAd → S_SM-DP+ | Send ES9+.GetBoundProfilePackage method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_GET_BPP, MTD_GET_BPP(<S_TRANSACTION_ID>, #R_PREP_DOWNLOAD_WITH_CC)) Verify if: • <S_TRANSACTION_ID> is the same as in #R_PREP_DOWNLOAD_WITH_CC • <EUICC_SIGNATURE2> using the #PK_EUICC_SIG • <S_HASHED_CC> = MTD_GENERATE_HASHED_CC(#CONFIRMATION_CODE1, <S_TRANSACTION_ID>) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#GET_BPP_OK) | No error, see NOTE. |
| NOTE: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL confirm End User Intent if requested. |  |  |  |

Test Sequence #03 Nominal: Get BPP using PPK-ENC and PPK-MAC without Confirmation Code

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.GetBoundProfilePackage method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_GET_BPP, MTD_GET_BPP(<S_TRANSACTION_ID>, #R_PREP_DOWNLOAD_NO_CC)) Verify: • If <S_TRANSACTION_ID> is the same as in #R_PREP_DOWNLOAD_NO_CC • <EUICC_SIGNATURE2> using the #PK_EUICC_SIG |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#GET_BPP_OK_PPK) | No error, see NOTE. |
| NOTE: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL confirm End User Intent if requested. |  |  |  |

Test Sequence #04 Nominal: Get BPP using PPK-ENC and PPK-MAC with Confirmation Code

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_3. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_3 (associated with PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT_CC |  |  |
| IC4 | LPAd → S_EndUser | LPAd requests the Confirmation Code from the S_EndUser. |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.GetBoundProfilePackage method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_GET_BPP, MTD_GET_BPP(<S_TRANSACTION_ID>, #R_PREP_DOWNLOAD_WITH_CC)) Verify if: • <S_TRANSACTION_ID> is the same as in #R_PREP_DOWNLOAD_WITH_CC • <EUICC_SIGNATURE2> using the #PK_EUICC_SIG • <S_HASHED_CC> = MTD_GENERATE_HASHED_CC(#CONFIRMATION_CODE1, <S_TRANSACTION_ID>) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#GET_BPP_OK_PPK) | No error, see NOTE. |
| NOTE: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL confirm End User Intent if requested. |  |  |  |

##### 4.4.22.2.2	TC_LPAd_ES9+_GetBoundProfilePackage_Retry

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| eUICC | There is no default SM-DP+ address configured. |

Test Sequence #01 Nominal: Get BPP Retry after incorrect Confirmation Code

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_3. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_3 (associated with PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT_CC |  |  |
| IC4 | LPAd → S_EndUser | LPAd requests the Confirmation Code from the S_EndUser. | #CONFIRMATION_CODE2 is provided by manual entry. |
| IC5 | LPAd → S_SM-DP+ | Send ES9+.GetBoundProfilePackage method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_GET_BPP, MTD_GET_BPP(<S_TRANSACTION_ID>, #R_PREP_DOWNLOAD_WITH_CC)) Verify if: <S_HASHED_CC> = MTD_GENERATE_HASHED_CC(#CONFIRMATION_CODE2, <S_TRANSACTION_ID>) |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#R_ERROR_8_2_7_3_8) | Continue to step 2 |
| 2 | S_SM-DP+ closes TLS session (unless ,LPAd has already closed TLS session) |  |  |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 4 | PROC_ES9+_INIT_AUTH |  |  |
| 5 | PROC_ES9+_AUTH_CLIENT_CC |  |  |
| 6 | LPAd → S_EndUser | LPAd requests the Confirmation Code from the S_EndUser. | #CONFIRMATION_CODE1 is provided by manual entry. |
| 7 | LPAd → S_SM-DP+ | Send ES9+.GetBoundProfilePackage method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_GET_BPP, MTD_GET_BPP(<S_TRANSACTION_ID>, #R_PREP_DOWNLOAD_WITH_CC)) Verify if: • If <S_TRANSACTION_ID> is the same as in #R_PREP_DOWNLOAD_WITH_CC • <EUICC_SIGNATURE2> using the #PK_EUICC_SIG • <S_HASHED_CC> = MTD_GENERATE_HASHED_CC(#CONFIRMATION_CODE1, <S_TRANSACTION_ID>) |
| 8 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#GET_BPP_OK) | No error, see NOTE 1. |
| NOTE: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL confirm End User Intent if requested. |  |  |  |

##### 4.4.22.2.3	TC_LPAd_ES9+_GetBoundProfilePackage_Error

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| eUICC | There is no default SM-DP+ address configured. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (associated with PROFILE_OPERATIONAL1). |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |

Test Sequence #01 Error: Wrong eUICC Signature

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT |  |  |
| IC4 | LPAd → S_SM-DP+ | Send ES9+.GetBoundProfilePackage method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_GET_BPP, MTD_GET_BPP(<S_TRANSACTION_ID>, #R_PREP_DOWNLOAD_NO_CC)) |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_1_6_1) | LPAd aborts AddProfile procedure NOTE: The LPAd MAY retry by restarting the Profile download and installation procedure. |

Test Sequence #02 Error: BPP Not Available

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT |  |  |
| IC4 | LPAd → S_SM-DP+ | Send ES9+.GetBoundProfilePackage method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_GET_BPP, MTD_GET_BPP(<S_TRANSACTION_ID>, #R_PREP_DOWNLOAD_NO_CC)) |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_2_3_7) | LPAd aborts AddProfile procedure NOTE: the LPAd MAY retry by restarting the Profile download and installation procedure. |

Test Sequence #03 Error: Unknown TransactionID received by SM-DP+

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT |  |  |
| IC4 | LPAd → S_SM-DP+ | Send ES9+.GetBundProfilePackage method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_GET_BPP, MTD_GET_BPP(<S_TRANSACTION_ID>, #R_PREP_DOWNLOAD_NO_CC)) |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_10_1_3_9) | LPAd aborts AddProfile procedure NOTE: the LPAd MAY retry by restarting the Profile download and installation procedure. |

Test Sequence #04 Error: Missing Confirmation Code

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT |  |  |
| IC4 | LPAd → S_SM-DP+ | Send ES9+.GetBoundProfilePackage method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #R_PREP_DOWNLOAD_NO_CC)) |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_2_7_2_2) | LPAd aborts AddProfile procedure NOTE: the LPAd MAY retry by restarting the Profile download and installation procedure. |

Test Sequence #05 Error: VOID

Test Sequence #06 Error: Wrong Confirmation Code

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_3. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_3 (associated with PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT_CC |  |  |
| IC4 | LPAd → S_EndUser | LPAd requests the Confirmation Code from the S_EndUser. | #CONFIRMATION_CODE2 is provided by manual entry. |
| IC5 | LPAd → S_SM-DP+ | Send ES9+.GetBoundProfilePackage method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_GET_BPP, MTD_GET_BPP(<S_TRANSACTION_ID>, #R_PREP_DOWNLOAD_WITH_CC)) |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_2_7_3_8) | LPAd aborts AddProfile procedure NOTE: The LPAd MAY retry by restarting the Profile download and installation procedure |

Test Sequence #07 Error: Maximum number of Confirmation Code retries exceeded

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_3. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_3 (associated with PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT_CC |  |  |
| IC4 | LPAd → S_EndUser | LPAd requests the Confirmation Code from the S_EndUser. | #CONFIRMATION_CODE2 is provided by manual entry. |
| IC5 | LPAd → S_SM-DP+ | Send ES9+.GetBoundProfilePackage method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_GET_BPP, MTD_GET_BPP(<S_TRANSACTION_ID>, #R_PREP_DOWNLOAD_WITH_CC)) |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_2_7_6_4) | LPAd aborts AddProfile procedure The LPAd SHALL NOT retry by restarting the Profile download and installation procedure. |

##### 4.4.22.2.4	TC_LPAd_ES9+_GetBoundProfilePackage_Retry_Reuse_oldOTPK

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| eUICC | There is no default SM-DP+ address configured. |

Test Sequence #01 Nominal: Get BPP Retry with Old Keys

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (associated with PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT |  |  |
| IC4 | LPAd → S_SM-DP+ | Send ES9+.GetBoundProfilePackage method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_GET_BPP, MTD_GET_BPP(<S_TRANSACTION_ID>, #R_PREP_DOWNLOAD_NO_CC)) Extract the <OTPK_EUICC_AKA> and reuse the same value in step 10. |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#GET_BPP_OK) | No error. The LPAd MAY display any relevant part of the Profile Metadata and offers the S_EndUser to postpone the Profile installation. Request for Strong Confirmation, if not requested before and not aborted. |
| 2 | S_EndUser → LPAd | End User Postpone is performed within the period as defined in #IUT_EU_CONFIRMATION_TIMEOUT |  |
| 3 | LPAd → S_SM-DP+ | Send ES9+.CancelSession method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_OK_EU_POSTPONED)) |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_SUCCESS) | No Error |
| 5 | S_SM-DP+ closes TLS session (unless LPAd has already closed TLS session) |  |  |
| 6 | Restart “Add Profile” Procedure as defined in the initial conditions. |  |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 8 | PROC_ES9+_INIT_AUTH |  |  |
| 9 | LPAd → S_SM-DP+ | Send ES9+.AuthenticateClient method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |
| 10 | S_SM-DP+ → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_OK_OLD_KEYS) | No Error |
| 11 | LPAd → S_SM-DP+ | Send ES9+.GetBoundProfilePackage method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_GET_BPP, MTD_GET_BPP(<S_TRANSACTION_ID>, #R_PREP_DOWNLOAD_NO_CC)) The <OTPK_EUICC_AKA> present in euiccSigned2 may be the same or different as in #AUTH_CLIENT_OK_OLD_KEYS. |
| 12 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#GET_BPP_OK) | No error, see NOTE 1. |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. Request for Strong Confirmation, if not requested before and not aborted. |  |  |  |

### 4.4.23	ES9+ (LPA -- SM-DP+): AuthenticateClient

#### 4.4.23.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

Section 2.1, 2.6.6.2, 2.10.1

Section 3.0.1, 3.1.3, 3.1.3.2, 3.2.7, 3.7.2

Section 4.2, 4.3

Section 5.6, 5.6.1, 5.6.3

Section 5.7.5

Section 6.2

Section 6.3

#### 4.4.23.2	Test Cases

##### 4.4.23.2.1	TC_LPAd_AuthenticateClient_Nominal

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| eUICC | There is no default SM-DP+ address configured. |

Test Sequence #01 Nominal: Authenticate Client without Confirmation Code

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, <LPA_RSP_CAPABILITY>)) • Extract <EUICC_CHALLENGE>, <LPA_RSP_CAPABILITY> |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#INITIATE_AUTH_OK) | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) Verify: • if #R_AUTH_SERVER_MATCH_ID_DEV_INFO used with the #MATCHING_ID_1 • If <S_TRANSACTION_ID> is the same as in #INITIATE_AUTH_OK • <EUICC_SIGNATURE1> using the #PK_EUICC_SIG • if <S_SMDP_CHALLENGE> present in the #R_AUTH_SERVER_MATCH_ID_DEV_INFO is the same as in <S_SMDP_SIGNED1> present in #INITIATE_AUTH_OK • for #DEVICE_INFO: - The value of the TAC corresponds to the first 8 digits of #IUT_IMEI and is represented as a string of 4 octets that is coded as a Telephony Binary Coded Decimal String as defined in 3GPP TS 29.002 [26] and 3GPP TS 23.003 [12] - if IMEI is present then its value corresponds to #IUT_IMEI and is represented as a string of 8 octets that is coded as a Telephony Binary Coded Decimal String as defined in 3GPP TS 29.002 [26] and 3GPP TS 23.003 [12] except that the last octet contains the check digit (in high nibble) and an'F' filler (in low nibble) - if O_D_GSM_GERAN then gsmSupportedRelease is set to the highest release as defined in #IUT_GSM_GERAN_REL. – if O_D_UMTS_UTRAN then utranSupportedRelease is set to the highest release as defined in #IUT_UMTS_UTRAN_REL. – if O_D_CDMA2000_1X then cdma2000onexSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_1X_REL. – if O_D_CDMA2000_HRPD then cdma2000hrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_HRPD_REL. The value R is either 1, 2 or 3 for Rev 0, A or B respectively. – if O_D_CDMA2000_EHRPD then cdma2000ehrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_EHRPD_REL. – if O_D_LTE then eutranSupportedRelease (or eutranEpcSupportedRelease if #IUT_RSP_VERSION is v2.2.2 or higher) is set to the highest release as defined in #IUT_LTE_EUTRAN_REL. – if O_D_NFC_TS26 then contactlessSupportedRelease is set to the highest release as defined in #IUT_NFC_REL. – that rspCrlSupportedVersion is not present. – if O_D_NR_EPC then nrEpcSupportedRelease is set to the highest release as defined in #IUT_NR_EPC_REL, – if O_D_NR_5GC then nr5gcSupportedRelease is set to the highest release as defined in #IUT_NR_5GC_REL, – if O_D_EUTRAN_5GC then eutran5gcSupportedRelease is set to the highest release as defined in #IUT_EUTRAN_5GC_REL, - if O_D_CAT_CLASSES then catSupportedClasses is set to the set of supported Card Application Toolkit letter classes as defined in #IUT_CAT_CLASSES. - lpaSvn is present and set to #IUT_RSP_VERSION_HIGHEST - euiccFormFactorType is present and set to the eUICC form factor as defined in #IUT_EUICC_FORM_FACTOR. - lpaRspCapability is present and equal to <LPA_RSP_CAPABILITY> in IC2 For each of the options O_D_GSM_GERAN, O_D_UMTS_UTRAN, O_D_CDMA2000_1X, O_D_CDMA2000_HRPD, O_D_CDMA2000_EHRPD, O_D_LTE, O_D_NFC_TS26, O_D_CAT_CLASSES, O_D_NR_EPC, O_D_NR_5GC or O_D_EUTRAN_5GC, if the option is not set, verify that the corresponding field in DeviceCapabilities is not present. |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_OK) | Next step of profile download procedure is performed (i.e. GetBoundProfilePackage request is sent). |

Test Sequence #02 Nominal: Authenticate Client with Confirmation Code

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_3. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_3 (associated with PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, <LPA_RSP_CAPABILITY>)) • Extract <EUICC_CHALLENGE> |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#INITIATE_AUTH_OK) | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) Verify: • if #R_AUTH_SERVER_MATCH_ID_DEV_INFO used with the #MATCHING_ID_3 • If <S_TRANSACTION_ID> is the same as in #INITIATE_AUTH_OK • <EUICC_SIGNATURE1> using the #PK_EUICC_SIG • if <S_SMDP_CHALLENGE> present in the #R_AUTH_SERVER_MATCH_ID_DEV_INFO is the same as in <S_SMDP_SIGNED1> present in #INITIATE_AUTH_OK • for #DEVICE_INFO: - The value of the TAC corresponds to the first 8 digits of #IUT_IMEI and is represented as a string of 4 octets that is coded as a Telephony Binary Coded Decimal String as defined in 3GPP TS 29.002 [26] and 3GPP TS 23.003 [12] - if IMEI is present then its value corresponds to #IUT_IMEI and is represented as a string of 8 octets that is coded as a Telephony Binary Coded Decimal String as defined in 3GPP TS 29.002 [26] and 3GPP TS 23.003 [12] except that the last octet contains the check digit (in high nibble) and an 'F' filler (in low nibble) - if O_D_GSM_GERAN then gsmSupportedRelease is set to the highest release as defined in #IUT_GSM_GERAN_REL. – if O_D_UMTS_UTRAN then utranSupportedRelease is set to the highest release as defined in #IUT_UMTS_UTRAN_REL. – if O_D_CDMA2000_1X then cdma2000onexSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_1X_REL. – if O_D_CDMA2000_HRPD then cdma2000hrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_HRPD_REL. The value R is either 1, 2 or 3 for Rev 0, A or B respectively. – if O_D_CDMA2000_EHRPD then cdma2000ehrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_EHRPD_REL – if O_D_LTE then eutranSupportedRelease (or eutranEpcSupportedRelease if #IUT_RSP_VERSION is v2.2.2 or higher) is set to the highest release as defined in #IUT_LTE_EUTRAN_REL. – if O_D_NFC_TS26 then contactlessSupportedRelease is set to the highest release as defined in #IUT_NFC_REL. – that rspCrlSupportedVersion is not present. – if O_D_NR_EPC then nrEpcSupportedRelease is set to the highest release as defined in #IUT_NR_EPC_REL, – if O_D_NR_5GC then nr5gcSupportedRelease is set to the highest release as defined in #IUT_NR_5GC_REL, – if O_D_EUTRAN_5GC then eutran5gcSupportedRelease is set to the highest release as defined in #IUT_EUTRAN_5GC_REL, - if O_D_CAT_CLASSES then catSupportedClasses is set to the set of supported Card Application Toolkit letter classes as defined in #IUT_CAT_CLASSES. - lpaSvn is present and set to #IUT_RSP_VERSION_HIGHEST - euiccFormFactorType is present and set to the eUICC form factor as defined in #IUT_EUICC_FORM_FACTOR. - lpaRspCapability is present and equal to <LPA_RSP_CAPABILITY> in IC2 For each of the options O_D_GSM_GERAN, O_D_UMTS_UTRAN, O_D_CDMA2000_1X, O_D_CDMA2000_HRPD, O_D_CDMA2000_EHRPD, O_D_LTE, O_D_NFC_TS26, O_D_CAT_CLASSES, O_D_NR_EPC, O_D_NR_5GC or O_D_EUTRAN_5GC, if the option is not set, verify that the corresponding field in DeviceCapabilities is not present. |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_OK_CC) | Next step of profile download procedure is performed (i.e. GetBoundProfilePackage request is sent). |

Test Sequence #03 Nominal: Authenticate Client with Confirmation Code Retry

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_3. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_3 (associated with PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description |  | Expected result |
| --- | --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT_CC |  |  |  |
| IC4 | LPAd → S_EndUser | LPAd requests the Confirmation Code from the S_EndUser. | #CONFIRMATION_CODE2 is provided by manual entry. |  |
| IC5 | LPAd → S_SM-DP+ | Send ES9+.GetBoundProfilePackage method | Verify if: <S_HASHED_CC> = MTD_GENERATE_HASHED_CC(#CONFIRMATION_CODE2, <S_TRANSACTION_ID>) |  |
| IC6 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#R_ERROR_8_2_7_3_8) |  |  |
| IC7 | Restart Add Profile procedure if O_D_CC_RETRY not supported |  |  |  |
| IC8 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |  |
| IC9 | PROC_ES9+_INIT_AUTH |  |  |  |
| IC10 | S_SM-DP+ → LPAd | Send ES9+.AuthenticateClient method |  | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_OK_CC) |  | Next step of profile download procedure is performed (i.e. GetBoundProfilePackage request is sent). |

##### 4.4.23.2.2	TC_LPAd_AuthenticateClient_ErrorCases

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| S_SM-DP+ | There is a pending Profile download order for MATCHING_ID_1 (PROFILE_OPERATIONAL1). |
| eUICC | There is no default SM-DP+ address configured. |
| Device | The protection of access to the LUI is disabled. |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |

Test Sequence #01 Error: VOID

Test Sequence #02 Error: VOID

Test Sequence #03 Error: Invalid eUICC or EUM Certificate

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.AuthenticateClient method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_1_3_6_1) | LPAd aborts AddProfile procedure |
| 3 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #04 Error: Expired eUICC Certificate

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.AuthenticateClient method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_1_3_6_3) | LPAd aborts AddProfile procedure |
| 3 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #05 Error: Invalid eUICC Signature or serverChallenge

| Step | Direction | Sequence / Description | Expected result | REQ |
| --- | --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.AuthenticateClient method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |  |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_1_6_1) | LPAd aborts AddProfile procedure |  |
| 3 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |  |

Test Sequence #06 Error: Insufficient Memory

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.AuthenticateClient Method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_1_4_8) | LPAd aborts AddProfile procedure |
| 3 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #07 Error: Unknown CI Root Key

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.AuthenticateClient Method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_11_1_3_9) | LPAd aborts AddProfile procedure |
| 3 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #08 Error: Profile not Allowed (Not in 'released' State)

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.AuthenticateClient Method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_2_1_2) | LPAd aborts AddProfile procedure |
| 3 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #09 Error: Unknown TransactionID

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.AuthenticateClient Method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_10_1_3_9) | LPAd aborts AddProfile procedure |
| 3 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #10 Error: Refused MatchingID

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.AuthenticateClient Method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_2_6_3_8) | LPAd aborts AddProfile procedure |
| 3 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #11 Error: Refused EID

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.AuthenticateClient Method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_1_1_3_8) | LPAd aborts AddProfile procedure |
| 3 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #12 Error: No Eligible Profile for this eUICC/Device

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.AuthenticateClient Method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_2_5_4_3) | LPAd aborts AddProfile procedure |
| 3 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #13 Error: Expired Download Order

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.AuthenticateClient Method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_8_5_4_10) | LPAd aborts AddProfile procedure |
| 3 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #14 Error: Maximum Number of Retries Exceeded

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.AuthenticateClient Method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_8_5_6_4) | LPAd aborts AddProfile procedure |
| 3 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #15 Error: Invalid SM-DP+(pb) certificate

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.AuthenticateClient method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #AUTH_CLIENT_INV_PB_CERT) | LPAd aborts AddProfile procedure (See NOTE) |
| 3 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |
| NOTE: Before the AddProfile procedure is aborted, the LPAd may request for Confirmation from the S_EndUser. In this case the S_EndUser SHALL give the Confirmation. |  |  |  |

Test Sequence #16 Error: Different OID for SM-DP+ Certificates (CERT.DPpb.SIG and CERT.DPauth.SIG not belonging to the same entity)

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| 1 | LPAd → S_SM-DP+ | AuthenticateClient | MTD_HTTP_REQ(#TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#AUTH_CLIENT_INV_CI) | LPAd aborts AddProfile procedure (See NOTE) |
| 3 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |
| NOTE: Before the AddProfile procedure is aborted, the LPAd may request for Confirmation from the S_EndUser. In this case the S_EndUser SHALL give the Confirmation. |  |  |  |

Test Sequence #17 Error: Invalid SM-DP+ signature (smdpSignature2)

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.AuthenticateClient Method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #AUTH_CLIENT_INV_SIGN) | LPAd aborts AddProfile procedure (See NOTE) |
| 3 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |
| NOTE: Before the AddProfile procedure is aborted, the LPAd may request for Confirmation from the S_EndUser. In this case the S_EndUser SHALL give the Confirmation. |  |  |  |

Test Sequence #18 Error: Invalid TransactionID sent by SM-DP+

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.AuthenticateClient Method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #AUTH_CLIENT_INV_TRANSACTION_ID) | LPAd aborts AddProfile procedure (See NOTE) |
| 3 | LPAd → S_SM-DP+ | No Profile download action | No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |
| NOTE: Before the AddProfile procedure is aborted, the LPAd may request for Confirmation from the S_EndUser. In this case the S_EndUser SHALL give the Confirmation. |  |  |  |

##### 4.4.23.2.3	TC_LPAd_AuthenticateClient_Nominal_V3

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled |
| eUICC | There is no default SM-DP+ address configured |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |
| S_SM-DP+ | Variant A certificates are included in certificate chain for TLS procedures |

Test Sequence #01 Nominal: Authenticate Client V3 Variant without Confirmation Code

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, <LPA_RSP_CAPABILITY>) ) • Extract <EUICC_CHALLENGE> |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#INITIATE_AUTH_OK_VARIANT_FLEX) | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3) ) Verify that : • #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3 used with the #MATCHING_ID_1 • <S_TRANSACTION_ID> is the same as in #INITIATE_AUTH_OK_VARIANT_A • <EUICC_SIGNATURE1> using the #PK_EUICC_SIG • <S_SMDP_CHALLENGE> present in the #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3 is the same as in <S_SMDP_SIGNED1_V3> present in #INITIATE_AUTH_OK_VARIANT_FLEX • for #DEVICE_INFO (deviceCapabilities) verify that: - The value of the TAC corresponds to the first 8 digits of #IUT_IMEI and is represented as a string of 4 octets that is coded as a Telephony Binary Coded Decimal String as defined in 3GPP TS 29.002 [26] and 3GPP TS 23.003 [12] - if IMEI is present then its value corresponds to #IUT_IMEI and is represented as a string of 8 octets that is coded as a Telephony Binary Coded Decimal String as defined in 3GPP TS 29.002 [26] and 3GPP TS 23.003 [12] except that the last octet contains the check digit (in high nibble) and an'F' filler (in low nibble) - if O_D_GSM_GERAN then gsmSupportedRelease is set to the highest release as defined in #IUT_GSM_GERAN_REL. – if O_D_UMTS_UTRAN then utranSupportedRelease is set to the highest release as defined in #IUT_UMTS_UTRAN_REL. – if O_D_CDMA2000_1X then cdma2000onexSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_1X_REL. – if O_D_CDMA2000_HRPD then cdma2000hrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_HRPD_REL. The value R is either 1, 2 or 3 for Rev 0, A or B respectively. – if O_D_CDMA2000_EHRPD then cdma2000ehrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_EHRPD_REL. – if O_D_LTE then eutranSupportedRelease is set to the highest release as defined in #IUT_LTE_EUTRAN_REL. – if O_D_NFC_TS26 then contactlessSupportedRelease is set to the highest release as defined in #IUT_NFC_REL. – that rspCrlSupportedVersion is not present. – if O_D_NR_EPC then nrEpcSupportedRelease is set to the highest release as defined in #IUT_NR_EPC_REL, – if O_D_NR_5GC then nr5gcSupportedRelease is set to the highest release as defined in #IUT_NR_5GC_REL, – if O_D_EUTRAN_5GC then eutran5gcSupportedRelease is set to the highest release as defined in #IUT_EUTRAN_5GC_REL, - if O_D_CAT_CLASSES then catSupportedClasses is set to the set of supported Card Application Toolkit letter classes as defined in #IUT_CAT_CLASSES. - lpaSvn is present and set to #IUT_RSP_VERSION_HIGHEST - euiccFormFactorType , is present and set to the eUICC form factor as defined in #IUT_EUICC_FORM_FACTOR. - lpaRspCapability is present and equal to <LPA_RSP_CAPABILITY> in IC2 For each of the options O_D_GSM_GERAN, O_D_UMTS_UTRAN, O_D_CDMA2000_1X, O_D_CDMA2000_HRPD, O_D_CDMA2000_EHRPD, O_D_LTE, O_D_NFC_TS26, O_D_CAT_CLASSES, O_D_NR_EPC, O_D_NR_5GC or O_D_EUTRAN_5GC if the option is not set, verify that the corresponding field in DeviceCapabilities is not present. |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_OK) | Next step of profile download procedure is performed (i.e. GetBoundProfilePackage request is sent). |

Test Sequence #02 Nominal: Authenticate Client - response with RPM pending

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_RPM_CONF_EN is loaded and disabled. eUICC returns CTX_PARAMS1_RPM_ICCID1 (sent by the LPAd) in euiccSigned1 in the response to ES10.AuthenticateServer. |
| LPAd | Update Profile operation is initiated and PROFILE_OPERATIONAL1 is selected. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) There is another pending RPM package order and S_SM-DP+ sends AuthenticationClient Response with rpmPending. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, #IUT_LPA_RSP_CAPABILITY) ) • Extract <EUICC_CHALLENGE> |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#INITIATE_AUTH_OK_VARIANT_A) | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3) ) Verify that: • #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3 used with the #MATCHING_ID_1 • <S_TRANSACTION_ID> is the same as in #INITIATE_AUTH_OK • <EUICC_SIGNATURE1> using the #PK_EUICC_ECDSA • <S_SMDP_CHALLENGE> present in the #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3 is the same as in <S_SMDP_SIGNED1> present in #INITIATE_AUTH_OK_VARIANT_A • operationType in #CTX_PARAMS_RPM_ICCID1 is set to #OP_TYPE_RPM • matchingIdSource in #CTX_PARAMS1_RPM_ICCID1 is set to NULL • for #DEVICE_INFO (deviceCapabilities) verify that: - TAC is BCD coded as 4 octets acc. to 3GPP TS 23.003 - if IMEI is present then it is BCD coded as 8 octets acc. to 3GPP TS 23.003 - if O_D_GSM_GERAN then gsmSupportedRelease is set to the highest release as defined in #IUT_GSM_GERAN_REL. – if O_D_UMTS_UTRAN then utranSupportedRelease is set to the highest release as defined in #IUT_UMTS_UTRAN_REL. – if O_D_CDMA2000_1X then cdma2000onexSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_1X_REL. – if O_D_CDMA2000_HRPD then cdma2000hrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_HRPD_REL. The value R is either 1, 2 or 3 for Rev 0, A or B respectively. – if O_D_CDMA2000_EHRPD then cdma2000ehrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_EHRPD_REL. – if O_D_LTE then eutranSupportedRelease is set to the highest release as defined in #IUT_LTE_EUTRAN_REL. – if O_D_NFC_TS26 then contactlessSupportedRelease is set to the highest release as defined in #IUT_NFC_REL. – if O_D_CRL then rspCrlSupportedVersion is set to the highest release as defined in #IUT_RSP_VERSION . – if O_D_NR_EPC then nrEpcSupportedRelease is set to the highest release as defined in #IUT_NR_EPC_REL, – if O_D_NR_5GC then nr5gcSupportedRelease is set to the highest release as defined in #IUT_NR_5GC_REL, – if O_D_EUTRAN_5GC then eutran5gcSupportedRelease is set to the highest release as defined in #IUT_EUTRAN_5GC_REL, - lpaSvn, the highest SGP.22 version of the LPA as defined in #IUT_RSP_VERSION_HIGHEST. - catSupportedClasses, the set of supported Card Application Toolkit letter classes as defined in #IUT_CAT_CLASSES. - euiccFormFactorType , the eUICC form factor as defined in #IUT_EUICC_FORM_FACTOR. For each of the options O_D_GSM_GERAN, O_D_UMTS_UTRAN, O_D_CDMA2000_1X, O_D_CDMA2000_HRPD, O_D_CDMA2000_EHRPD, O_D_LTE, O_D_NFC_TS26, O_D_CRL, O_D_NR_EPC, O_D_NR_5GC or O_D_EUTRAN_5GC if the option is not set, verify that the corresponding field in DeviceCapabilities is not present. • for #DEVICE_INFO (lpaRspCapabilities) verify that: - lpaRspCapabilities is set to #IUT_LPA_RSP_CAPABILITY |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_RPM_OK_AND_RPM_PENDING ) | Next step of RPM Package handling procedure is performed (i.e. ES9+.HandleNotification is sent). LPA shall initiate another Mutual Authentication with same S_SM-DP+ after completing to first RPM download is completed. |

##### 4.4.23.2.4	    TC_LPAd_AuthenticateClient_ErrorCases_V3

TBD

### 4.4.24	ES9+ (LPA – SM-DP+): HandleNotification

#### 4.4.24.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Requirements

- 	Section 3.0.1, 3.1.1

- 	Section 5.6.4

- 	Section 6.5.1, 6.5.2

#### 4.4.24.2	Test Cases

##### 4.4.24.2.1	TC_LPAd_ES9+_HandleNotification_Nominal

Throughout all the test cases the maximum number of Notifications simultaneously tested has been set as to two as there is not minimum defined in SGP.21 [3] or SGP.22 [2] for the number of Notifications that can be stored by the eUICC.

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (associated with PROFILE_OPERATIONAL1). |
| S_SM-DP+ | S_SM-DP+(1) is configured with #TEST_DP_ADDRESS1 and #CERT_S_SM_DP_TLS. S_SM-DP+(2) is configured with #TEST_DP_ADDRESS2 and #CERT_S_SM_DP2_TLS. |
| Device | The protection of access to the LUI is disabled. |
| eUICC | There is no default SM-DP+ address configured. |

Test Sequence #01 Nominal: Successful PIR and Install Notifications to the Same SM-DP+ Address

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1 for PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT |  |  |
| IC4 | PROC_ES9+_GET_BPP (s NOTE 1) |  |  |
| 1 | LPAd → S_SM-DP+(1) | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF(#R_PIR_OK)) • Verify the euiccSign <EUICC_SIGN_PIR> using the #PK_EUICC_SIG |
| 2 | S_SM-DP+(1) → LPAd | #R_HTTP_204_OK | No error exhibited by the LPAd. The LPAd MAY inform the End User of the success status indicated by the Profile Installation Result. |
| 3 | LPAd → S_SM-DP+(1) | Establish an HTTPs connection if previously closed |  |
| 4 | LPAd → S_SM-DP+(1) | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #PENDING_NOTIF_INST1)) sent within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG |
| 5 | S_SM-DP+(1) → LPAd | #R_HTTP_204_OK | No error exhibited by the LPAd |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL confirm End User Intent if requested and SHALL not abort the session. NOTE 2: The timeout SHALL start after the PIR is received. NOTE 3: In case the AddProfile initiation was combined with “Enable” (i.e. O_D_ADD_ENABLE_SEPARATED is not supported), any subsequent Enable Notification is not part of the test sequence. |  |  |  |

Test Sequence #02 Nominal: Successful PIR and Enable Notifications to the Same SM-DP+ Address

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1 for PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_EN instead of #METADATA_OP_PROF1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT |  |  |
| IC4 | PROC_ES9+_GET_BPP (s. NOTE 1) |  |  |
| 1 | LPAd → S_SM-DP+(1) | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF(#R_PIR_OK)) • Verify the euiccSign <EUICC_SIGN_PIR> using the #PK_EUICC_SIG |
| 2 | S_SM-DP+(1) → LPAd | #R_HTTP_204_OK | No error exhibited by the LPAd. The LPAd MAY inform the End User of the success status indicated by the Profile Installation Result. |
| 3 | S_EndUser → LPAd | If PROFILE_OPERATIONAL1 is not already enabled (see NOTE 3), initiate the Enable Profile operation for PROFILE_OPERATIONAL1. | PROFILE_OPERATIONAL1 is enabled |
| 4 | LPAd → S_SM-DP+(1) | Establish an HTTPs connection if previously closed |  |
| 5 | LPAd → S_SM-DP+(1) | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #PENDING_NOTIF_EN1)) sent within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG |
| 6 | S_SM-DP+(1) → LPAd | #R_HTTP_204_OK | No error exhibited by the LPAd |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL confirm End User Intent if requested and SHALL not abort the session. NOTE 2: The timeout SHALL start after the initiation of the Enable Profile operation. NOTE 3: PROFILE_OPERATIONAL1 is expected to be already enabled only in the case that the device supports only O_D_ADD_ENABLE_COMBINED. |  |  |  |

Test Sequence #03 Nominal: Disable and Delete Notifications to the Same SM-DP+ Address

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | PROFILE_OPERATIONAL1 is in the Enabled state. |

| Step | Direction | Sequence / Description | Expected result |  |
| --- | --- | --- | --- | --- |
| 1 | S_EndUser → LPAd | Initiate the Disable Profile operation for PROFILE_OPERATIONAL1 | PROFILE_OPERATIONAL1 is disabled |  |
| 2 | LPAd → S_SM-DP+(1) | Establish an HTTPs connection if previously closed |  |  |
| 3 | LPAd → S_SM-DP+(1) | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF(#PENDING_NOTIF_DIS1)) sent within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT (see NOTE 1) Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG |  |
| 4 | S_SM-DP+(1) → LPAd | #R_HTTP_204_OK | No error exhibited by the LPAd |  |
| 5 | LPAd → S_SM-DP+(1) | Establish an HTTPs connection if previously closed |  |  |
| 6 | S_EndUser → LPAd | Initiate the Delete Profile operation for PROFILE_OPERATIONAL1 | Successful End User Intent verified PROFILE_OPERATIONAL1 is deleted |  |
| 7 | LPAd → S_SM-DP+(1) | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF(#PENDING_NOTIF_DEL1)) sent within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT (see NOTE 2) Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG |  |
| 8 | S_SM-DP+(1) → LPAd | #R_HTTP_204_OK | No error exhibited by the LPAd |  |
| NOTE 1: The timeout SHALL start after the initiation of the Disable Profile operation. NOTE 2: The timeout SHALL start after the End User Intent verification. |  |  |  |  |

Test Sequence #04 Nominal: Enable and Disable Notifications with Different SM-DP+ Addresses

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | PROFILE_OPERATIONAL2 is installed on the eUICC. |
| eUICC | PROFILE_OPERATIONAL1 is in the Enabled state. |
| eUICC | PROFILE_OPERATIONAL2 is in the Disabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser → LPAd | Initiate the Enable Profile operation for PROFILE_OPERATIONAL2 | PROFILE_OPERATIONAL2 is enabled |
| 2 | LPAd → S_SM-DP+(1) | Establish an HTTPs connection if previously closed |  |
| 3 | LPAd → S_SM-DP+(1) | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF(#PENDING_NOTIF_DIS1)) sent within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG |
| 4 | S_SM-DP+(1) → LPAd | #R_HTTP_204_OK | No error exhibited by the LPAd |
| 5 | LPAd → S_SM-DP+(2) | Establish an HTTPs connection |  |
| 6 | LPAd → S_SM-DP+(2) | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS2, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF(#PENDING_NOTIF_EN2)) sent within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG |
| 7 | S_SM-DP+(2) → LPAd | #R_HTTP_204_OK | No error exhibited by the LPAd |
| NOTE 1: Steps 2,3 and 4 can be executed in parallel to the steps 5, 6 and 7. NOTE 2: The timeout SHALL start after the initiation of the Enable Profile operation. |  |  |  |

Test Sequence #05 Nominal: Different SM-DP+ Addresses in PIR and Install Notifications

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1 for PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_INST_DIFF instead of #METADATA_OP_PROF1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT |  |  |
| IC4 | PROC_ES9+_GET_BPP(s. NOTE 1) |  |  |
| 1 | LPAd → S_SM-DP+(1) | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF(#R_PIR_OK)) |
| 2 | S_SM-DP+(1) → LPAd | #R_HTTP_204_OK | No error exhibited by the LPAd. The LPAd MAY inform the End User of the success status indicated by the Profile Installation Result. |
| 3 | LPAd → S_SM-DP+(2) | Establish an HTTPs connection |  |
| 4 | LPAd → S_SM-DP+(2) | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS2, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF(#PENDING_NOTIF_INST_ADDRESS2)) sent within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG |
| 5 | S_SM-DP+(2) → LPAd | #R_HTTP_204_OK | No error exhibited by the LPAd |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL confirm End User Intent if requested and SHALL not abort the session. NOTE 2: Steps 1 and 2 can be executed in parallel to the steps 3,4 and 5. NOTE 3: The timeout SHALL start after the End User Intent verification. |  |  |  |

Test Sequence #06 Nominal: Profile Download with PIR Failed

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1 for PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT |  |  |
| IC4 | LPAd → S_SM-DP+(1) | Send ES9+.GetBoundProfilePackage method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_GET_BPP, MTD_GET_BPP(<S_TRANSACTION_ID>, #R_PREP_DOWNLOAD_NO_CC)) |
| IC5 | S_SM-DP+(1) → LPAd | MTD_HTTP_RESP(#GET_BPP_INV) | No error exhibited by the LPAd, s. note 1. |
| 1 | LPAd → S_SM-DP+(1) | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF(#R_PIR_SECU_INVALID)) • Verify the euiccSign <EUICC_SIGN_PIR> using the #PK_EUICC_SIG |
| 2 | S_SM-DP+(1) → LPAd | #R_HTTP_204_OK | No error exhibited by the LPAd. The LPAd MAY inform the End User of the error status indicated by the Profile Installation Result. |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL confirm End User Intent if requested and SHALL not abort the session. |  |  |  |

Test Sequence #07 Nominal: Successful PIR and Install Notifications after Connectivity Interruption

This Test Sequence is FFS.

Test Sequence #08 Nominal: No Acknowledge for Successful PIR results in No Further Notifications

The purpose of this test case is to verify that the next Notification of a group is not sent until LPA receives a successful response from the SM-DP+ for the previous Notification.

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1 for PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_1 as <MATCHING_ID> |  |  |
| IC4 | PROC_ES9+_GET_BPP (s. NOTE 1) |  |  |
| 1 | LPAd → S_SM-DP+(1) | Send ES9+.HandleNotification method initiated | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF(#R_PIR_OK)) |
| 2 | LPAd → S_SM-DP+(1) | No ES9+.HandleNotification method sent | No ES9+.HandleNotification requests are sent within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT OR TLS Session closed independent of timeout. |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL confirm End User Intent if requested and SHALL not abort the session. NOTE 2: The timeout in Step 3 SHALL start after the End User Intent verification. |  |  |  |

### 4.4.25	ES9+ (LPA – SM-DP+): CancelSession

#### 4.4.25.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

Section 2.9.2.1, 2.9.2.4

Section 2.10.1

Section 3.0.1, 3.0.2, 3.1.3, 3.1.3.2, 3.1.5, 3.2.7, 3.7.2

Section 5.6.5

Section 6.5.2.10

#### 4.4.25.2	Test Cases

##### 4.4.25.2.1	TC_LPAd_ES9+_CancelSession_Nominal

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| eUICC | There is no default SM-DP+ address configured. |

Test Sequence #01 Nominal: Profile Download with PPR1 not allowed due to Operational Profile already present

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed and disabled on the eUICC. |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_4. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_4 (associated with PROFILE_OPERATIONAL4). |
| S_SM-DP+ | The S_SM-DP+ is configured to ignore the forbidden PPR during the eligibility check. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT Extract <S_TRANSACTION_ID> |  |  |
| IC4 | PROC_ES9+_GET_BPP with #METADATA_OP_PROF4 used in #GET_BPP_OK This step is conditional – occurs only if ES9+.CancelSession method was not sent before (e.g. request for Confirmation was required after ES9+.AuthenticateClient method) |  |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.CancelSession method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_OK_PPR_NOT_ALLOWED)) Verify: •<EUICC_CANCEL_SESSION_SIGNATURE> with the #PK_EUICC_SIG •<S_TRANSACTION_ID> is the same as in IC3 |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#R_SUCCESS) | If Step 1 was performed directly after IC3: No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. OR If Step 1 was performed after IC4: No ES9+.HandleNotification requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. |

Test Sequence #02 Nominal: End User rejection

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (associated with PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT Extract <S_TRANSACTION_ID> |  |  |
| IC4 | PROC_ES9+_GET_BPP This step is conditional – occurs only if ES9+.CancelSession method was not sent before (e.g. request for Confirmation was required after ES9+.AuthenticateClient method) |  |  |
| 1 | LPAd → S_EndUser | Request for Confirmation if not requested before. | The LPA provides means for the End User Confirmation/Rejection of the Profile Download. |
| 2 | S_EndUser → LPAd | End User Rejection (or failed confirmation) is performed within the period as defined in #IUT_EU_CONFIRMATION_TIMEOUT |  |
| 3 | LPAd → S_SM-DP+ | Send ES9+.CancelSession method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_OK_EU_REJ)) Verify: •<EUICC_CANCEL_SESSION_SIGNATURE> with the #PK_EUICC_SIG •<S_TRANSACTION_ID> is the same as in IC3 |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_SUCCESS) | If Step 1 was performed directly after IC3: No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. OR If Step 1 was performed after IC4: No ES9+.HandleNotification requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. |

Test Sequence #03 Nominal: Load BPP Error

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description | Expected result |  |
| --- | --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT Extract <S_TRANSACTION_ID> |  |  |  |
| IC4 | LPAd → S_SM-DP+ | Send ES9+.GetBoundProfilePackage method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_GET_BPP, MTD_GET_BPP(<S_TRANSACTION_ID>, #R_PREP_DOWNLOAD_NO_CC)) |  |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #GET_BPP_LOAD_ERROR) | Continue to step 2 (End User Confirmation) if requested, otherwise continue with Step 3 |  |
| 2 | LPAd → S_EndUser | Request for Confirmation if not requested before. | End User Intent successfully verified. |  |
| 3 | LPAd → S_SM-DP+ | Send ES9+.CancelSession method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_OK_EU_LOAD_BPP_ERROR)) Verify: •<EUICC_CANCEL_SESSION_SIGNATURE> with the #PK_EUICC_SIG •<S_TRANSACTION_ID> is the same as in IC3 |  |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_SUCCESS) | No ES9+.HandleNotification requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. |  |

Test Sequence #04 Nominal: End User Timeout

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (associated with PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT Extract <S_TRANSACTION_ID> |  |  |
| IC4 | PROC_ES9+_GET_BPP This step is conditional – occurs only if ES9+.CancelSession method was not sent before (e.g. request for Confirmation was required after ES9+.AuthenticateClient method) |  |  |
| 1 | LPAd → S_EndUser | Request for Confirmation if not requested before. | The LPA provides means for the End User Confirmation of the Profile Download. |
| 2 | S_EndUser → LPAd | No End User Rejection or Confirmation is performed within the period as defined in #IUT_EU_CONFIRMATION_TIMEOUT |  |
| 3 | LPAd → S_SM-DP+ | Send ES9+.CancelSession method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_OK_TIMEOUT)) Verify: •<EUICC_CANCEL_SESSION_SIGNATURE> with the #PK_EUICC_SIG •<S_TRANSACTION_ID> is the same as in IC3 |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_SUCCESS) | If Step 1 was performed directly after IC3: No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. OR If Step 1 was performed after IC4: No ES9+.HandleNotification requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. |

Test Sequence #05 Nominal: Load BPP Error due to unknown TAG

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT Extract <S_TRANSACTION_ID> |  |  |
| IC4 | LPAd → S_SM-DP+ | Send ES9+.GetBoundProfilePackage method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_GET_BPP, MTD_GET_BPP(<S_TRANSACTION_ID>, #R_PREP_DOWNLOAD_NO_CC)) |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#GET_BPP_LOAD_ERROR_UNKNOWN_TAG) | Continue to step 2 (End User Confirmation) if requested, otherwise continue with Step 3 |
| 2 | LPAd → S_EndUser | Request for Confirmation if not requested before. | End User Intent successfully verified. |
| 3 | LPAd → S_SM-DP+ | Send ES9+.CancelSession method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_OK_EU_LOAD_BPP_ERROR)) Verify: •<EUICC_CANCEL_SESSION_SIGNATURE> with the #PK_EUICC_SIG •<S_TRANSACTION_ID> is the same as in IC3 |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#R_SUCCESS) | No ES9+.HandleNotification requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. |

##### 4.4.25.2.2	TC_LPAd_ES9+_CancelSession_EndUserPostponed_Nominal

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| eUICC | There is no default SM-DP+ address configured. |

Test Sequence #01 Nominal: End User Postponed

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (associated with PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT Extract <S_TRANSACTION_ID> |  |  |
| IC4 | PROC_ES9+_GET_BPP This step is conditional – occurs only if ES9+.CancelSession method was not sent before (e.g. request for Confirmation was required after ES9+.AuthenticateClient method) |  |  |
| 1 | LPAd → S_EndUser | Request for Confirmation if not requested before. | The LPA provides means for the End User Confirmation/Rejection of the Profile Download. |
| 2 | S_EndUser → LPAd | End User Postpone is performed within the period as defined in #IUT_EU_CONFIRMATION_TIMEOUT |  |
| 3 | LPAd → S_SM-DP+ | Send ES9+.CancelSession method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_OK_EU_POSTPONED)) Verify: •<EUICC_CANCEL_SESSION_SIGNATURE> with the #PK_EUICC_SIG •<S_TRANSACTION_ID> is the same as in IC3 |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_SUCCESS) | If Step 1 was performed directly after IC3: No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. OR If Step 1 was performed after IC4: No ES9+.HandleNotification requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. |

##### 4.4.25.2.3	TC_LPAd_ES9+_CancelSession_Error

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| eUICC | There is no default SM-DP+ address configured. |

Test Sequence #01 Error: Unknown TransactionID after End User Rejection/Postpone

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (associated with PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_1 as <MATCHING_ID> Extract <S_TRANSACTION_ID> |  |  |
| IC4 | PROC_ES9+_GET_BPP This step is conditional – occurs only if ES9+.CancelSession method was not sent before (e.g. request for Confirmation was required after ES9+.AuthenticateClient method) |  |  |
| IC5 | LPAd → S_EndUser | Request for Confirmation if not requested before. | The LPA provides means for the End User Confirmation/Rejection of the Profile Download. |
| IC6 | S_EndUser → LPAd | End User Postpone/Rejection (or failed confirmation) is performed within the period as defined in #IUT_EU_CONFIRMATION_TIMEOUT |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.CancelSession method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_OK_EU_REJ)) OR MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_OK_EU_POSTPONED)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #R_ERROR_8_10_1_3_9) | No error after receiving the HTTPs response. (See NOTE) |
| NOTE: The LPA MAY either stop or retry sending ES9+.CancelSession method. |  |  |  |

Test Sequence #02 Error: Invalid eUICC Signature after End User Rejection/Postpone

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (associated with PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_1 as <MATCHING_ID> Extract <S_TRANSACTION_ID> |  |  |
| IC4 | PROC_ES9+_GET_BPP This step is conditional – occurs only if ES9+.CancelSession method was not sent before (e.g. request for Confirmation was required after ES9+.AuthenticateClient method) |  |  |
| IC5 | LPAd → S_EndUser | Request for Confirmation if not requested before. | The LPA provides means for the End User Confirmation/Rejection of the Profile Download. |
| IC6 | S_EndUser → LPAd | End User Postpone/Rejection (or failed confirmation) is performed within the period as defined in #IUT_EU_CONFIRMATION_TIMEOUT |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.CancelSession method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_OK_EU_REJ)) OR MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_OK_EU_POSTPONED)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#R_ERROR_8_1_6_1) | No error after receiving the HTTPs response. The LPA SHALL stop the procedure: no ES9+.CancelSession requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. |

Test Sequence #03 Error: Invalid SM-DP+ OID after End User Rejection/Postpone

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (associated with PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_1 as <MATCHING_ID> Extract <S_TRANSACTION_ID> |  |  |
| IC4 | PROC_ES9+_GET_BPP This step is conditional – occurs only if ES9+.CancelSession method was not sent before (e.g. request for Confirmation was required after ES9+.AuthenticateClient method) |  |  |
| IC5 | LPAd → S_EndUser | Request for Confirmation if not requested before. | The LPA provides means for the End User Confirmation/Rejection of the Profile Download. |
| IC6 | S_EndUser → LPAd | End User Postpone/Rejection (or failed confirmation) is performed within the period as defined in #IUT_EU_CONFIRMATION_TIMEOUT |  |
| 1 | LPAd → S_SM-DP+ | Send ES9+.CancelSession method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_OK_EU_REJ)) OR MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_OK_EU_POSTPONED)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#R_ERROR_8_8_3_10) | No error after receiving the HTTPs response. The LPA SHALL stop the procedure: no ES9+.CancelSession requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT.. |

##### 4.4.25.2.4	TC_LPAd_ES9+_CancelSession_PPRs

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| eUICC | There is no default SM-DP+ address configured. |

Test Sequence #01 Nominal: End User rejection/postpone after PPR1 consent requested

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| eUICC | - The Test eUICC’s RAT is configured as follows: PPR1 is allowed and End User Consent is required for #MCC_MNC4 with gid1 and gid2 absent. |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_4. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_4 (associated with PROFILE_OPERATIONAL4). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT Extract <S_TRANSACTION_ID> |  |  |
| IC4 | PROC_ES9+_GET_BPP with #METADATA_OP_PROF4 used in #GET_BPP_OK This step is conditional – occurs only if ES9+.CancelSession method was not sent before (e.g. request for Confirmation was required after ES9+.AuthenticateClient method) |  |  |
| 1 | LPAd → S_EndUser | Request for Confirmation if not requested before. | The LPA provides means for the End User Confirmation/Rejection of the Profile Download. Either Strong Confirmation is asked by showing relevant information concerning the PPR(s); or Simple Confirmation is asked on the Profile download. |
| 2 | S_EndUser → LPAd | End User Postpone/Rejection (or failed confirmation) is performed within the period as defined in #IUT_EU_CONFIRMATION_TIMEOUT |  |
| 3 | LPAd → S_SM-DP+ | Send ES9+.CancelSession method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_OK_EU_REJ)) OR MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_OK_EU_POSTPONED)) Verify: •<EUICC_CANCEL_SESSION_SIGNATURE> with the #PK_EUICC_SIG •<S_TRANSACTION_ID> is the same as in IC3 |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#R_SUCCESS) | If Step 1 was performed directly after IC3: No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. OR If Step 1 was performed after IC4: No ES9+.HandleNotification requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. |

Test Sequence #02 Nominal: End User rejection/posptone after PPR2 consent requested

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| eUICC | - The Test eUICC’s RAT is configured as follows: PPR2 is allowed and End User Consent is required for #MCC_MNC2 with gid1 and gid2 absent. |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_3_NO_CC. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_3 (associated with PROFILE_OPERATIONAL3). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT Extract <S_TRANSACTION_ID> |  |  |
| IC4 | PROC_ES9+_GET_BPP with #METADATA_OP_PROF3 used in #GET_BPP_OK This step is conditional – occurs only if ES9+.CancelSession method was not sent before (e.g. request for Confirmation was required after ES9+.AuthenticateClient method) |  |  |
| 1 | LPAd → S_EndUser | Request for Confirmation if not requested before. | The LPA provides means for the End User Confirmation/Rejection of the Profile Download. Either Strong Confirmation is asked by showing relevant information concerning the PPR(s); or Simple Confirmation is asked on the Profile download. |
| 2 | S_EndUser → LPAd | End User Postpone/Rejection (or failed confirmation) is performed within the period as defined in #IUT_EU_CONFIRMATION_TIMEOUT |  |
| 3 | LPAd → S_SM-DP+ | Send ES9+.CancelSession method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_OK_EU_REJ)) OR MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_OK_EU_POSTPONED)) Verify: •<EUICC_CANCEL_SESSION_SIGNATURE> with the #PK_EUICC_SIG •<S_TRANSACTION_ID> is the same as in IC3 |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#R_SUCCESS) | If Step 1 was performed directly after IC3: No ES9+.GetBoundProfilePackage requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. OR If Step 1 was performed after IC4: No ES9+.HandleNotification requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. |

##### 4.4.25.2.5	TC_LPAd_ES9+_CancelSession_AuthenticateClient_RPM

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled |
| eUICC | There is no default SM-DP+ address configured The eUICC supports RPM |
| LPAd | RPM operation is enabled in the LPA by the End User |
| S_SM-DP+ | Variant A certificates are included in certificates chain for TLS procedures. |

Test Sequence #01 Nominal:  RPM Disabled by User

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_RPM_CONF_EN is loaded and disabled. eUICC returns CTX_PARAMS1_RPM_ICCID1 (sends by the LPAd) in euiccSigned1 in the response to ES10.AuthenticateServer. |
| LPAd | Update Profile operation is initiated and PROFILE_OPERATIONAL1 is selected. Polling Address is retrieved from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. RPM operation is disabled in the LPA by the End User. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, #IUT_LPA_RSP_CAPABILITY) ) • Extract <EUICC_CHALLENGE> |
| IC3 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#INITIATE_AUTH_OK_VARIANT_A) | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3) ) |
| IC4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_RPM_OK) | LPAd initiates Cancel Session |
| 1 | LPAd → S_SM-DP+ | Send ES9+.CancelSession method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_OK_RPM_DISABLED) ) Verify: •<EUICC_CANCEL_SESSION_SIGNATURE> with the #PK_EUICC_ECDSA •<S_TRANSACTION_ID> is the same as in #INITIATE_AUTH_OK_VARIANT_A |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#R_SUCCESS) | No ES9+ Handle Notification with Load RPM Package Result is sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. |

Test Sequence #02 Nominal:  RPM Package download – Reject on Strong Confirmation

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_DELETE is loaded and disabled. eUICC returns CTX_PARAMS1_RPM_ICCID1 (sent by the LPAd) in euiccSigned1 in the response to ES10.AuthenticateServer. |
| LPAd | Update Profile operation is initiated and PROFILE_OPERATIONAL1 is selected. Polling Address is retrieved from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) with RPM Command for Delete profile. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, #IUT_LPA_RSP_CAPABILITY) ) • Extract <EUICC_CHALLENGE> |
| IC3 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#INITIATE_AUTH_OK_VARIANT_A) | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3) ) |
| IC4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_RPM_DELETE_OK ) |  |
| 1 | LPAd → S_EndUser | Request for User Confirmation. | LPAd initiates Confirmation Request for Strong Confirmation on the execution of RPM Command for Delete Profile, |
| 2 | S_EndUser → LPAd | End User Rejects the RPM Command within the period as defined in #IUT_EU_CONFIRMATION_TIMEOUT |  |
| 3 | LPAd → S_SM-DP+ | Send ES9+.CancelSession method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_OK_RPM_EU_REJECT) ) Verify: •<EUICC_CANCEL_SESSION_SIGNATURE> with the #PK_EUICC_ECDSA •<S_TRANSACTION_ID> is the same as in #INITIATE_AUTH_OK_VARIANT_A |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#R_SUCCESS) | No ES9+ Handle Notification with Load RPM Package Result is sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. |

Test Sequence #03 Nominal: RPM Package download – Reject on Confirmation – Disable Profile

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_DISABLE is loaded and enabled. eUICC returns CTX_PARAMS1_RPM_ICCID1 (sent by the LPAd) in euiccSigned1 in the response to ES10.AuthenticateServer. |
| LPAd | Update Profile operation is initiated and PROFILE_OPERATIONAL1 is selected. Polling Address is retrieved from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) with RPM Command for Disable profile. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, #IUT_LPA_RSP_CAPABILITY) ) • Extract <EUICC_CHALLENGE> |
| IC3 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#INITIATE_AUTH_OK_VARIANT_A) | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3) ) |
| IC4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_RPM_DISABLE_OK ) |  |
| 1 | LPAd → S_EndUser | Request for User Confirmation. | LPAd initiates Confirmation Request for Simple Confirmation on the execution of RPM Command for Disable Profile, |
| 2 | S_EndUser → LPAd | End User Rejects the RPM Command within the period as defined in #IUT_EU_CONFIRMATION_TIMEOUT |  |
| 3 | LPAd → S_SM-DP+ | Send ES9+.CancelSession method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_OK_RPM_EU_REJECT) ) Verify: •<EUICC_CANCEL_SESSION_SIGNATURE> with the #PK_EUICC_ECDSA •<S_TRANSACTION_ID> is the same as in #INITIATE_AUTH_OK_VARIANT_A |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#R_SUCCESS) | No ES9+ Handle Notification with Load RPM Package Result is sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT. |

### 4.4.26	ES9+ (LPA – SM-DP+): HTTPS

#### 4.4.26.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

Section 2.1

Section 2.6.6, 2.6.7.1

Section 3.0.1

Section 5.6

Section 6

Section 6.1

#### 4.4.26.2	Test Cases

##### 4.4.26.2.1	TC_LPAd_HTTPS_Nominal_Variant_O

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| eUICC | There is no default SM-DP+ address configured. |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |

Test Sequence #01 Nominal: HTTPS Session Establishment

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | LPAd → S_SM-DP+ | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_TLS_VERSION, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) Verify the following: • #IUT_TLS_VERSION SHALL be 1.2 or higher • <TLS_CIPHER_SUITES> SHALL contain at least TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 and NOT contain TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 • <EXT_SHA256_ECDSA> SHALL have at least the 'supported_signature_algorithms' extension set with HashAlgorithm sha256 (04) and SignatureAlgorithm ecdsa (03). |
| 2 | S_SM-DP+ → LPAd | MTD_TLS_SERVER_HELLO_ETC(#TLS_VERSION_1_2, #S_TLS_CIPHER_SUITE, <S_SESSION_ID_SERVER>, #CERT_S_SM_DP_TLS, NO_PARAM) | MTD_TLS_CLIENT_KEY_EXCH_ETC(<CLIENT_TLS_EPHEM_KEY>) |
| 3 | S_SM-DP+ → LPAd | Finalize TLS Handshake (send Server ChangeCipherSpec and Finished messages) | HTTPS connection established |

Test Sequence #02 Nominal: non-reuse of session keys

The purpose of this test sequence is to verify that the LPAd is not reusing ephemeral keys from the previous session.

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ Extract <CLIENT_TLS_EPHEM_KEY> Extract <SESSION_ID_CLIENT> and <S_SESSION_ID_SERVER> |  |  |
| IC2 | Terminate TLS session and restart “Add Profile” Procedure as define in the initial conditions. |  |  |
| 1 | LPAd → S_SM-DP+ | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_TLS_VERSION, <TLS_CIPHER_SUITES>, <SESSION_ID_CLEINT>, <EXT_SHA256_ECDSA>) Verify the following: • #IUT_TLS_VERSION SHALL be 1.2 or higher • <TLS_CIPHER_SUITES> SHALL be at least TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 and NOT contain TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 • if <SESSION_ID_CLIENT> is non-empty then it SHALL be different from <SESSION_ID_CLIENT> and <S_SESSION_ID_SERVER> extracted in IC1. • <EXT_SHA256_ECDSA> SHALL have at least the ‘supported_signature_algorithms’ extension set with HashAlgorithm sha256 (04) and SignatureAlgorithm ecdsa (03). |
| 2 | S_SM-DP+ → LPAd | MTD_TLS_SERVER_HELLO_ETC(#TLS_VERSION_1_2, #S_TLS_CIPHER_SUITE, <S_SESSION_ID_SERVER>, #CERT_S_SM_DP_TLS, NO_PARAM) | MTD_TLS_CLIENT_KEY_EXCH_ETC(<CLIENT_TLS_EPHEM_KEY>) Verify if • <CLIENT_TLS_EPHEM_KEY> is different from the one used by LPAd in IC1 |
| 3 | S_SM-DP+ → LPAd | Finalize TLS Handshake (send Server ChangeCipherSpec and Finished messages) | HTTPS connection established |

##### 4.4.26.2.2	TC_LPAd_HTTPS_ErrorCases

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |

Test Sequence #01 Error: Invalid (SM-DP+) TLS Certificate signature

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | LPAd → S_SM-DP+ | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_TLS_VERSION, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) |
| 2 | S_SM-DP+ → LPAd | MTD_TLS_SERVER_HELLO_ETC(#TLS_VERSION_1_2, #S_TLS_CIPHER_SUITE, <S_SESSION_ID_SERVER>, #CERT_S_SM_DP_TLS_INV_SIG, NO_PARAM) Note: if the LPAd sends an Alert during or after any of the messages sent by the S_SM-DP+ in MTD_TLS_SERVER_HELLO_ETC, then the S_SM-DP+ might not send the messages specified in MTD_TLS_SERVER_HELLO_ETC which occur after the Alert. | LPAd MAY send a TLS Alert. LPAd aborts AddProfile procedure. |
| 3 | LPDd → S_SM-DP+ | TLS 1.2 close | The TLS connection is rejected. |

Test Sequence #02 Error: Expired TLS Certificate

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | LPAd → S_SM-DP+ | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_TLS_VERSION, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) |
| 2 | S_SM-DP+ → LPAd | MTD_TLS_SERVER_HELLO_ETC(#TLS_VERSION_1_2, #S_TLS_CIPHER_SUITE, <S_SESSION_ID_SERVER>, #CERT_S_SM_DP_TLS_EXPIRED, NO_PARAM) Note: if the LPAd sends an Alert during or after any of the messages sent by the S_SM-DP+ in MTD_TLS_SERVER_HELLO_ETC, then the S_SM-DP+ might not send the messages specified in MTD_TLS_SERVER_HELLO_ETC which occur after the Alert. | LPAd MAY send a TLS Alert. LPAd aborts AddProfile procedure |
| 3 | LPDd → S_SM-DP+ | TLS 1.2 close | The TLS connection is rejected. |

Test Sequence #03 Error: VOID

Test Sequence #04 Error: VOID

Test Sequence #05 Error: VOID

Test Sequence #06 Error: VOID

Test Sequence #07 Error: Invalid TLS Certificate based on Invalid CI (Invalid Curve)

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | Power-on the Device |  |  |
| 1 | LPAd → S_SM-DP+ | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_TLS_VERSION, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) |
| 2 | S_SM-DP+ → LPAd | MTD_TLS_SERVER_HELLO_ETC(#TLS_VERSION_1_2, #S_TLS_CIPHER_SUITE, <S_SESSION_ID_SERVER>, #CERT_S_SM_DP_TLS_INV_CURVE, NO_PARAM) Note: if the LPAd sends an Alert during or after any of the messages sent by the S_SM-DP+ in MTD_TLS_SERVER_HELLO_ETC, then the S_SM-DP+ might not send the messages specified in MTD_TLS_SERVER_HELLO_ETC which occur after the Alert. | LPAd MAY send a TLS Alert. LPAd aborts AddProfile procedure |
| 3 | LPDd → S_SM-DP+ | TLS 1.2 close | The TLS connection is rejected. |

##### 4.4.26.2.3	TC_LPAd_HTTPS_Nominal_Variants_V3

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| eUICC | There is no default SM-DP+ address configured. |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |

Test Sequence #01 Nominal: HTTPS Session Establishment Variant A

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | LPAd → S_SM-DP+ | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_TLS_VERSION, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) Verify the following: • #IUT_TLS_VERSION SHALL be 1.2 or higher • <TLS_CIPHER_SUITES> SHALL contain at least TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 and NOT contain TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 • <EXT_SHA256_ECDSA> SHALL have at least the 'supported_signature_algorithms' extension set with HashAlgorithm sha256 (04) and SignatureAlgorithm ecdsa (03). |
| 2 | S_SM-DP+ → LPAd | MTD_TLS_SERVER_HELLO_ETC(#TLS_VERSION_1_2, #S_TLS_CIPHER_SUITE, <S_SESSION_ID_SERVER>, #CERT_S_SM_DP_TLS, #CERT_S_SM_DP_SubCA_ECDSA) | MTD_TLS_CLIENT_KEY_EXCH_ETC(<CLIENT_TLS_EPHEM_KEY>) |
| 3 | S_SM-DP+ → LPAd | Finalize TLS Handshake (send Server ChangeCipherSpec and Finished messages) | HTTPS connection established |

Test Sequence #02 Nominal: HTTPS Session Establishment Variant B

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | LPAd → S_SM-DP+ | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_TLS_VERSION, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) Verify the following: • #IUT_TLS_VERSION SHALL be 1.2 or higher • <TLS_CIPHER_SUITES> SHALL contain at least TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 and NOT contain TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 • <EXT_SHA256_ECDSA> SHALL have at least the 'supported_signature_algorithms' extension set with HashAlgorithm sha256 (04) and SignatureAlgorithm ecdsa (03). |
| 2 | S_SM-DP+ → LPAd | MTD_TLS_SERVER_HELLO_ETC(#TLS_VERSION_1_2, #S_TLS_CIPHER_SUITE, <S_SESSION_ID_SERVER>, #CERT_S_SM_DP_TLS, #CERT_S_CI_SubCA_SIG) | MTD_TLS_CLIENT_KEY_EXCH_ETC(<CLIENT_TLS_EPHEM_KEY>) |
| 3 | S_SM-DP+ → LPAd | Finalize TLS Handshake (send Server ChangeCipherSpec and Finished messages) | HTTPS connection established |

Test Sequence #03 Nominal: HTTPS Session Establishment Variant C

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | LPAd → S_SM-DP+ | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_TLS_VERSION, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) Verify the following: • #IUT_TLS_VERSION SHALL be 1.2 or higher • <TLS_CIPHER_SUITES> SHALL contain at least TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 and NOT contain TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 • <EXT_SHA256_ECDSA> SHALL have at least the 'supported_signature_algorithms' extension set with HashAlgorithm sha256 (04) and SignatureAlgorithm ecdsa (03). |
| 2 | S_SM-DP+ → LPAd | MTD_TLS_SERVER_HELLO_ETC(#TLS_VERSION_1_2, #S_TLS_CIPHER_SUITE, <S_SESSION_ID_SERVER>, #CERT_S_SM_DP_TLS, #CERT_S_SM_DP_SubCAList_SIG) | MTD_TLS_CLIENT_KEY_EXCH_ETC(<CLIENT_TLS_EPHEM_KEY>) |
| 3 | S_SM-DP+ → LPAd | Finalize TLS Handshake (send Server ChangeCipherSpec and Finished messages) | HTTPS connection established |

### 4.4.27	ES11 (LPA – SM-DS): InitiateAuthentication

#### 4.4.27.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

Section 3.0.1, 3.1.3

Section 5.8.1

Section 6.5.2.11

Section 6.6.2.6

#### 4.4.27.2	Test Cases

##### 4.4.27.2.1	TC_LPAd_ES11_InitiateAuthentication_Nominal

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| Device | The Profile Download is initiated using SM-DS (see section 2.2.4.1). |
| S_SM-DS | S_SM-DP+ (#TEST_DP_ADDRESS1) performed Profile download Event Registration to the S_SM-DS (#TEST_ROOT_DS_ADDRESS) with #EVENT_ID_1. |
| eUICC | There is no default SM-DP+ address configured. |

Test Sequence #01 Nominal: Initiate Authentication

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES11 |  |  |
| 1 | LPAd → S_SM-DS | Send ES11.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_ROOT_DS_ADDRESS) , <LPA_RSP_CAPABILITY>) • Extract <EUICC_CHALLENGE> Verify if: - • <LPA_RSP_CAPABILITY> contains - crlStaplingV3Support set to '1' - certChainV3Support set to '1' - signedSmdsResponseV3Support set to '1' |
| 2 | S_SM-DS → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_DS_OK) | Next step of common mutual authentication procedure is performed (i.e. AuthenticateClient request is sent). |

##### 4.4.27.2.2	TC_LPAd_ES11_InitiateAuthentication_ErrorCases

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| Device | The Profile Download is initiated using SM-DS (see section 2.2.4.1). |
| S_SM-DS | S_SM-DP+ (#TEST_DP_ADDRESS1) performed Profile download Event Registration to the S_SM-DS (#TEST_ROOT_DS_ADDRESS) with #EVENT_ID_1 (see NOTE). |
| eUICC | There is no default SM-DP+ address configured. |
| NOTE: The S_SM_DP+ does not need to be available to the LPAd for profile download during test sequence execution, as the LPAd is not expected to receive the smdpAddress. |  |

Test Sequence #01 Error: Invalid SM-DS Address

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES11 |  |  |
| IC2 | LPAd → S_SM-DS | Send ES11.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_ROOT_DS_ADDRESS, <LPA_RSP_CAPABILITY>)) |
| 1 | S_SM-DS → LPAd | MTD_HTTP_RESP(#R_ERROR_8_9_1_3_8) | LPAd aborts AddProfile procedure |
| 2 | LPAd → S_SM-DS | No Profile download action | No ES11.InitiateAuthentication requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #02 Error: Unsupported Security Configuration

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES11 |  |  |
| IC2 | LPAd → S_SM-DS | Send ES11.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_ROOT_DS_ADDRESS, <LPA_RSP_CAPABILITY>)) |
| 1 | S_SM-DS → LPAd | MTD_HTTP_RESP(#R_ERROR_8_9_2_3_1) | LPAd aborts AddProfile procedure |
| 2 | LPAd → S_SM-DS | No Profile download action | No ES11.InitiateAuthentication requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #03 Error: Unsupported SVN

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES11 |  |  |
| IC2 | LPAd → S_SM-DS | Send ES11.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_ROOT_DS_ADDRESS, <LPA_RSP_CAPABILITY>)) |
| 1 | S_SM-DS → LPAd | MTD_HTTP_RESP(#R_ERROR_8_9_3_3_1) | LPAd aborts AddProfile procedure |
| 2 | LPAd → S_SM-DS | No Profile download action | No ES11.InitiateAuthentication requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #04 Error: Unavailable SM-DS Certificate

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES11 |  |  |
| IC2 | LPAd → S_SM-DS | Send ES11.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_ROOT_DS_ADDRESS, <LPA_RSP_CAPABILITY>)) |
| 1 | S_SM-DS → LPAd | MTD_HTTP_RESP(#R_ERROR_8_9_4_3_7) | LPAd aborts AddProfile procedure |
| 2 | LPAd → S_SM-DS | No Profile download action | No ES11.InitiateAuthentication requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #05 Error:  Invalid SM-DS Certificate

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES11 |  |  |
| IC2 | LPAd → S_SM-DS | Send ES11.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_ROOT_DS_ADDRESS, <LPA_RSP_CAPABILITY>)) |
| 1 | S_SM-DS → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_INV_CERT_DS) | LPAd aborts AddProfile procedure |
| 2 | LPAd → S_SM-DS | No Profile download action | No ES11.InitiateAuthentication or ES11.AuthenticateClient requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #06 Error:  Invalid SM-DS Signature

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES11 |  |  |
| IC2 | LPAd → S_SM-DS | Send ES11.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_ROOT_DS_ADDRESS, <LPA_RSP_CAPABILITY>)) |
| 1 | S_SM-DS → LPAd | MTD_HTTP_RESP(#INITIATE_AUTH_INV_SIGN_DS) | LPAd aborts AddProfile procedure |
| 2 | LPAd → S_SM-DS | No Profile download action | No ES11.InitiateAuthentication or ES11.AuthenticateClient requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #07 Error: Invalid SM-DS Address sent by the SM-DS

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES11 |  |  |
| IC2 | LPAd → S_SM-DS | Send ES11.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_ROOT_DS_ADDRESS, <LPA_RSP_CAPABILITY>)) |
| 1 | S_SM-DS → LPAd | MTD_HTTP_RESP(#INITIATE_AUTH_INV_SMDS_ADDRESS) | LPAd informs the S_EndUser and aborts the AddProfile procedure |
| 2 | LPAd → S_SM-DS | No Profile download action | No ES11.InitiateAuthentication or ES11.AuthenticateClient requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #08 Error: Unsupported CI Key ID

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES11 |  |  |
| IC2 | LPAd → S_SM-DS | Send ES11.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_ROOT_DS_ADDRESS, <LPA_RSP_CAPABILITY>)) |
| 1 | S_SM-DS → LPAd | MTD_HTTP_RESP(#INITIATE_AUTH_INV_CI_DS) | LPAd aborts AddProfile procedure |
| 2 | LPAd → S_SM-DS | No Profile download action | No ES11.InitiateAuthentication or ES11.AuthenticateClient requests are sent within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

### 4.4.28	ES11 (LPA – SM-DS): AuthenticateClient

#### 4.4.28.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

Section 3.0.1, 3.1.3, 3.1.3.2

Section 4.2

Section 5.8.2

Section 6.2

Section 6.3

Section 6.5.1, 6.5.1.1, 6.5.1.2, 6.5.1.3, 6.5.1.4, 6.5.2, 6.5.2.12

Section 6.6.2.7

#### 4.4.28.2	Test Cases

##### 4.4.28.2.1	TC_LPAd_ES11_AuthenticateClient_Nominal

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| Device | The Profile Download is initiated using SM-DS (see section 2.2.4.1). |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| eUICC | There is no default SM-DP+ address configured. |

Test Sequence #01 Nominal: Authenticate Client with empty MatchingID

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| S_SM-DS | S_SM-DP+ (#TEST_DP_ADDRESS1) performed Profile download Event Registration to the root S_SM-DS (#TEST_ROOT_DS_ADDRESS) with #EVENT_ID_1 for #EID1. |
| S_SM-DP+ | There is a pending Profile download order for #EVENT_ID_1 (PROFILE_OPERATIONAL1) (see NOTE). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES11 |  |  |
| IC2 | LPAd → S_SM-DS | Send ES11.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_ROOT_DS_ADDRESS, <LPA_RSP_CAPABILITY>)) • Extract <EUICC_CHALLENGE> |
| 1 | S_SM-DS → LPAd | MTD_HTTP_RESP(#INITIATE_AUTH_DS_OK) | MTD_HTTP_REQ(#TEST_ROOT_DS_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_DS_MATCH_ID_DEV_INFO)) Verify: • If <S_TRANSACTION_ID> is the same as in #INITIATE_AUTH_DS_OK • <EUICC_SIGNATURE1> using the #PK_EUICC_SIG • if matchingId field in #R_AUTH_SERVER_DS_MATCH_ID_DEV_INFO is missing OR matchingId field is present and <MATCHING_ID> is empty • if <S_SMDS_CHALLENGE> present in the #R_AUTH_SERVER_MATCH_ID_DEV_INFO is the same as in <S_SMDS_SIGNED1> present in #INITIATE_AUTH_DS_OK • for #DEVICE_INFO: - The value of the TAC corresponds to the first 8 digits of #IUT_IMEI and is represented as a string of 4 octets that is coded as a Telephony Binary Coded Decimal String as defined in 3GPP TS 29.002 [26] and 3GPP TS 23.003 [12] - if IMEI is present then its value corresponds to #IUT_IMEI and is represented as a string of 8 octets that is coded as a Telephony Binary Coded Decimal String as defined in 3GPP TS 29.002 [26] and 3GPP TS 23.003 [12] except that the last octet contains the check digit (in high nibble) and an 'F' filler (in low nibble) - if O_D_GSM_GERAN then gsmSupportedRelease is set to the highest release as defined in #IUT_GSM_GERAN_REL. – if O_D_UMTS_UTRAN then utranSupportedRelease is set to the highest release as defined in #IUT_UMTS_UTRAN_REL. – if O_D_CDMA2000_1X then cdma2000onexSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_1X_REL. – if O_D_CDMA2000_HRPD then cdma2000hrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_HRPD_REL. The value R is either 1, 2 or 3 for Rev 0, A or B respectively. – if O_D_CDMA2000_EHRPD then cdma2000ehrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_EHRPD_REL. – if O_D_LTE then eutranSupportedRelease (or eutranEpcSupportedRelease if #IUT_RSP_VERSION is v2.2.2 or higher) is set to the highest release as defined in #IUT_LTE_EUTRAN_REL. – if O_D_NFC_TS26 then contactlessSupportedRelease is set to the highest release as defined in #IUT_NFC_REL. – that rspCrlSupportedVersion is not present , – if O_D_NR_EPC then nrEpcSupportedRelease is set to the highest release as defined in #IUT_NR_EPC_REL, – if O_D_NR_5GC then nr5gcSupportedRelease is set to the highest release as defined in #IUT_NR_5GC_REL, – if O_D_EUTRAN_5GC then eutran5gcSupportedRelease is set to the highest release as defined in #IUT_EUTRAN_5GC_REL, - if O_D_CAT_CLASSES then catSupportedClasses is set to the set of supported Card Application Toolkit letter classes as defined in #IUT_CAT_CLASSES. - lpaSvn is present and set to #IUT_RSP_VERSION_HIGHEST - euiccFormFactorType is present and set to the eUICC form factor as defined in #IUT_EUICC_FORM_FACTOR. - lpaRspCapability is present and equal to <LPA_RSP_CAPABILITY> in IC2 For each of the options O_D_GSM_GERAN, O_D_UMTS_UTRAN, O_D_CDMA2000_1X, O_D_CDMA2000_HRPD, O_D_CDMA2000_EHRPD, O_D_LTE, O_D_NFC_TS26, O_D_CAT_CLASSES, O_D_NR_EPC, O_D_NR_5GC or O_D_EUTRAN_5GC, if the option is not set, verify that the corresponding field in DeviceCapabilities is not present. |
| 2 | S_SM-DS → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_DS_OK1) | Next step of profile discovery procedure is performed (ie. ES9+.InitiateAuthentication request is sent) |

Test Sequence #02 Nominal: Authenticate Client with MatchingID set to EventID

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| S_SM-DS | The Alternative S_SM-DS(2) (#TEST_DS_ADDRESS1) performed Profile download Event Registration to the root S_SM-DS(1) (#TEST_ROOT_DS_ADDRESS) with #EVENT_ID_1 for #EID1. |
| S_SM-DS | S_SM-DP+ (#TEST_DP_ADDRESS1) performed Profile download Event Registration to the Alternative S_SM-DS(2) (#TEST_DS_ADDRESS1) with #EVENT_ID_2 for #EID1. |
| S_SM-DP+ | There is a pending Profile download order for #EVENT_ID_2 (PROFILE_OPERATIONAL1) (see NOTE). |

| Step | Direction | Sequence/ Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES11 |  |  |
| IC2 | LPAd → S_SM-DS(1) | Send ES11.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION(<EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_ROOT_DS_ADDRESS, <LPA_RSP_CAPABILITY>)) • Extract <EUICC_CHALLENGE> |
| 1 | S_SM-DS(1) → LPAd | MTD_HTTP_RESP(#INITIATE_AUTH_DS_OK) | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #R_AUTH_SERVER_DS_MATCH_ID_DEV_INFO)) Verify: • if matchingId field in #R_AUTH_SERVER_DS_MATCH_ID_DEV_INFO is missing OR matchingId field is present and <MATCHING_ID> is empty |
| 2 | S_SM-DS(1) → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_DS_OK_DSADDR1) |  |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES11 with #TEST_DS_ADDRESS1 and #CERT_S_SM_DS2_TLS |  |  |
| 4 | LPAd → S_SM-DS(2) | Send ES11.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DS_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION(<EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DS_ADDRESS1, <LPA_RSP_CAPABILITY>)) • Extract <EUICC_CHALLENGE> |
| 5 | S_SM-DS(2) → LPAd | MTD_HTTP_RESP(#INITIATE_AUTH_DS_OK_1) | MTD_HTTP_REQ(#TEST_DS_ADDRESS1 , #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_DS_MATCH_ID_DEV_INFO_1)) Verify: • if <MATCHING_ID> is set to #EVENT_ID_1 |
| 6 | S_SM-DS(2) → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_DS_OK2) | Next step of profile discovery procedure is performed (ie. ES9+.InitiateAuthentication request is sent) |

##### 4.4.28.2.2	TC_LPAd_ES11_AuthenticateClient_ErrorCases

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| Device | The Profile Download is initiated using SM-DS (see section 2.2.4.1). |
| S_SM-DP+ | There is a pending Profile download order for #EVENT_ID_1 (PROFILE_OPERATIONAL1) (see NOTE). |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| eUICC | There is no default SM-DP+ address configured. |
| NOTE: The S_SM_DP+ does not need to be available to the LPAd for profile download during test sequence execution, as the LPAd is not expected to receive the smdpAddress. |  |

Test Sequence #01 Error: VOID

Test Sequence #02 Error: VOID

Test Sequence #03 Error: Invalid eUICC or EUM Certificate

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES11 |  |  |
| IC2 | PROC_ES11_INIT_AUTH |  |  |
| 1 | LPAd → S_SM-DS | Send ES11.AuthenticateClient method | MTD_HTTP_REQ(#TEST_ROOT_DS_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_DS_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DS → LPAd | MTD_HTTP_RESP(#R_ERROR_8_1_3_6_1) | LPAd aborts AddProfile procedure |
| 3 | LPAd → S_SM-DS | No Profile download action | No requests are sent on ES11 within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #04 Error: Expired eUICC or EUM Certificate

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES11 |  |  |
| IC2 | PROC_ES11_INIT_AUTH |  |  |
| 1 | LPAd → S_SM-DS | Send ES11.AuthenticateClient method | MTD_HTTP_REQ(#TEST_ROOT_DS_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_DS_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DS → LPAd | MTD_HTTP_RESP(#R_ERROR_8_1_3_6_3) | LPAd aborts AddProfile procedure |
| 3 | LPAd → S_SM-DS | No Profile download action | No requests are sent on ES11 within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #05 Error: Invalid eUICC signature or serverChallenge

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES11 |  |  |
| IC2 | PROC_ES11_INIT_AUTH |  |  |
| 1 | LPAd → S_SM-DS | Send ES11.AuthenticateClient method | MTD_HTTP_REQ(#TEST_ROOT_DS_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_DS_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DS → LPAd | MTD_HTTP_RESP(#R_ERROR_8_1_6_1) | LPAd aborts AddProfile procedure |
| 3 | LPAd → S_SM-DS | No Profile download action | No requests are sent on ES11 within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #06 Error: Unknown TransactionID

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES11 |  |  |
| IC2 | PROC_ES11_INIT_AUTH |  |  |
| 1 | LPAd → S_SM-DS | Send ES11.AuthenticateClient method | MTD_HTTP_REQ(#TEST_ROOT_DS_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_DS_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DS → LPAd | MTD_HTTP_RESP(#R_ERROR_8_10_1_3_9) | LPAd aborts AddProfile procedure |
| 3 | LPAd → S_SM-DS | No Profile download action | No requests are sent on ES11 within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

Test Sequence #07 Error: Unknown Event Record

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| S_SM-DS | The Alternative S_SM-DS (#TEST_DS_ADDRESS1) performed Profile download Event Registration to the root S_SM-DS (#TEST_ROOT_DS_ADDRESS) with #EVENT_ID_1 for #EID1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES11 |  |  |
| IC2 | LPAd → S_SM-DS | Send ES11.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_ROOT_DS_ADDRESS, <LPA_RSP_CAPABILITY>)) • Extract <EUICC_CHALLENGE> |
| IC3 | S_SM-DS → LPAd | MTD_HTTP_RESP(#INITIATE_AUTH_DS_OK) | MTD_HTTP_REQ(#TEST_ROOT_DS_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_DS_MATCH_ID_DEV_INFO)) |
| IC4 | S_SM-DS → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_DS_OK_DSADDR1) | No Error |
| IC5 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES11 with #TEST_DS_ADDRESS1 and #CERT_S_SM_DS2_TLS |  |  |
| IC6 | LPAd → S_SM-DS | Send ES11.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DS_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION(<EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DS_ADDRESS1, <LPA_RSP_CAPABILITY>)) • Extract <EUICC_CHALLENGE> |
| IC7 | S_SM-DS → LPAd | MTD_HTTP_RESP(#INITIATE_AUTH_DS_OK_1) | MTD_HTTP_REQ(#TEST_DS_ADDRESS1 , #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_DS_MATCH_ID_DEV_INFO_1)) |
| 1 | S_SM-DS → LPAd | MTD_HTTP_RESP(#R_ERROR_8_9_5_3_9) | LPAd aborts AddProfile procedure |
| 2 | LPAd → S_SM-DS | No Profile download action | No requests are sent on ES11 within the timeout #IUT_LPAd_SESSION_CLOSE_TIMEOUT in Annex F. |

### 4.4.29	ES11 (LPA -- SM-DS): HTTPS

#### 4.4.29.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

Section 2.6.6, 2.6.7.1

Section 4.5.2.2

Section 5.8

Section 6

Section 6.1

#### 4.4.29.2	Test Cases

##### 4.4.29.2.1	TC_LPAd_ES11_HTTPS_Nominal_Variant_O

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| Device | The Profile Download is initiated using SM-DS (see section 2.2.4.1). |
| S_SM-DS | S_SM-DP+ (#TEST_DP_ADDRESS1) performed Profile download Event Registration to the S_SM-DS (#TEST_ROOT_DS_ADDRESS) with #EVENT_ID_1. |
| eUICC | There is no default SM-DP+ address configured. |

Test Sequence #01 Nominal: HTTPS Session Establishment

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | LPAd → S_SM-DS | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO(#IUT_TLS_VERSION, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) Verify the following: • #IUT_TLS_VERSION SHALL be 1.2 or higher • <TLS_CIPHER_SUITES> SHALL contain at least TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 and NOT contain TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 • <EXT_SHA256_ECDSA> SHALL have at least the'supported_signature_algorithms' extension set with HashAlgorithm sha256 (04) and SignatureAlgorithm ecdsa (03). |
| 2 | S_SM-DS → LPAd | MTD_TLS_SERVER_HELLO_ETC(#TLS_VERSION_1_2, #S_TLS_CIPHER_SUITE, <S_SESSION_ID_SERVER>, #CERT_S_SM_DS_TLS, NO_PARAM) | MTD_TLS_CLIENT_KEY_EXCH_ETC(<CLIENT_TLS_EPHEM_KEY>) |
| 3 | S_SM-DS → LPAd | Finalize TLS Handshake (send Server ChangeCipherSpec and Finished messages) | HTTPS connection established |

Test Sequence #02 Nominal: non-reuse of session keys

The purpose of this test sequence is to verify that the LPAd is not reusing ephemeral keys from the previous session.

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES11 Extract <CLIENT_TLS_EPHEM_KEY> Extract <SESSION_ID_CLIENT> and <S_SESSION_ID_SERVER> |  |  |
| IC2 | Terminate TLS session and Initiate Profile Download using SM-DS (see section 2.2.4.1). |  |  |
| 1 | LPAd → S_SM-DS | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_TLS_VERSION, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) Verify the following: • #IUT_TLS_VERSION SHALL be 1.2 or higher • <TLS_CIPHER_SUITES> SHALL be at least TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 and NOT contain TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 • if <SESSION_ID_CLIENT> is non-empty then it SHALL be different from <SESSION_ID_CLIENT> and <S_SESSION_ID_SERVER> extracted in IC1. • <EXT_SHA256_ECDSA> SHALL have at least the'supported_signature_algorithms' extension set with HashAlgorithm sha256 (04) and SignatureAlgorithm ecdsa (03). |
| 2 | S_SM-DS → LPAd | MTD_TLS_SERVER_HELLO_ETC(#TLS_VERSION_1_2, #S_TLS_CIPHER_SUITE, <S_SESSION_ID_SERVER>, #CERT_S_SM_DS_TLS, NO_PARAM) | MTD_TLS_CLIENT_KEY_EXCH_ETC(<CLIENT_TLS_EPHEM_KEY>) Verify if • <CLIENT_TLS_EPHEM_KEY> is different from the one used by LPAd in IC1 |
| 3 | S_SM-DS → LPAd | Finalize TLS Handshake (send Server ChangeCipherSpec and Finished messages) | HTTPS connection established |

##### 4.4.29.2.2	TC_LPAd_ES11_HTTPS_Error

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| Device | The Profile Download is initiated using SM-DS (see section 2.2.4.1). |
| S_SM-DS | S_SM-DP+ (#TEST_DP_ADDRESS1) performed Profile download Event Registration to the S_SM-DS (#TEST_ROOT_DS_ADDRESS) with #EVENT_ID_1. |
| eUICC | There is no default SM-DP+ address configured. |

Test Sequence #01 Error: Invalid (SM-DS) TLS Certificate signature

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | LPAd → S_SM-DS | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_TLS_VERSION, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) |
| 2 | S_SM-DS → LPAd | MTD_TLS_SERVER_HELLO_ETC(#TLS_VERSION_1_2, #S_TLS_CIPHER_SUITE, <S_SESSION_ID_SERVER>, #CERT_S_SM_DS_TLS_INV_SIG, NO_PARAM) Note: if the LPAd sends an Alert during or after any of the messages sent by the S_SM-DP+ in MTD_TLS_SERVER_HELLO_ETC, then the S_SM-DP+ might not send the messages specified in MTD_TLS_SERVER_HELLO_ETC which occur after the Alert. | LPAd MAY send a TLS Alert. LPAd aborts AddProfile procedure |
| 3 | LPDd → S_SM-DS | TLS 1.2 close | The TLS connection is rejected. |

Test Sequence #02 Error: Expired TLS Certificate

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | LPAd → S_SM-DS | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_TLS_VERSION, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) |
| 2 | S_SM-DS → LPAd | MTD_TLS_SERVER_HELLO_ETC(#TLS_VERSION_1_2, #S_TLS_CIPHER_SUITE, <S_SESSION_ID_SERVER>, #CERT_S_SM_DS_TLS_EXPIRED, NO_PARAM) Note: if the LPAd sends an Alert during or after any of the messages sent by the S_SM-DP+ in MTD_TLS_SERVER_HELLO_ETC, then the S_SM-DP+ might not send the messages specified in MTD_TLS_SERVER_HELLO_ETC which occur after the Alert. | LpaD may send a TLS Alert. LPAd aborts AddProfile procedure |
| 3 | LPDd → S_SM-DS | TLS 1.2 close | The TLS connection is rejected. |

Test Sequence #03 Error: VOID

Test Sequence #04 Error: VOID

Test Sequence #05 Error: VOID

Test Sequence #06 Error: VOID

Test Sequence #07 Error: Invalid TLS Certificate based on Invalid CI (Invalid Curve)

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | LPAd → S_SM-DS | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_TLS_VERSION, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) |
| 2 | S_SM-DS → LPAd | MTD_TLS_SERVER_HELLO_ETC(#TLS_VERSION_1_2, #S_TLS_CIPHER_SUITE, <S_SESSION_ID_SERVER>, #CERT_S_SM_DS_TLS_INV_CURVE, NO_PARAM) Note: if the LPAd sends an Alert during or after any of the messages sent by the S_SM-DP+ in MTD_TLS_SERVER_HELLO_ETC, then the S_SM-DP+ might not send the messages specified in MTD_TLS_SERVER_HELLO_ETC which occur after the Alert. | LPAd MAY send a TLS Alert. LPAd aborts AddProfile procedure |
| 3 | LPDd → S_SM-DS | TLS 1.2 close | The TLS connection is rejected. |

### 4.4.30	ES9+ (LPA -- SM-DP+): AuthenticateClient – RPM Package Download

#### 4.4.30.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]

Requirements

- 2.6.6.2, 2.10.1

- 3.0.1

- 3.2.7, 3.7.2

- 3.7.2

- 5.6.3

#### 4.4.30.2	Test Cases

##### 4.4.30.2.1     TC_LPAd_AuthenticateClient_Nominal_RPM

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled |
| eUICC | There is no default SM-DP+ address configured The eUICC supports RPM |
| LPAd | RPM operation is enabled in the LPA by the End User |
| S_SM-DP+ | Variant A certificates are included in certificates chain for TLS procedures. |

Test Sequence #01 Nominal: Authenticate Client Variant A for RPM, Polling Address: SM-DP+

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_RPM_CONF_EN is loaded and disabled. eUICC returns CTX_PARAMS1_RPM_ICCID1 (sends by the LPAd) in euiccSigned1 in the response to ES10.AuthenticateServer. |
| LPAd | Update Profile operation is initiated and PROFILE_OPERATIONAL1 is selected. Polling Address is retrieved from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 |  | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, #IUT_LPA_RSP_CAPABILITY) ) • Extract <EUICC_CHALLENGE> |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#INITIATE_AUTH_OK_VARIANT_A) | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3) ) Verify that: • #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3 used with the #MATCHING_ID_1 • <S_TRANSACTION_ID> is the same as in #INITIATE_AUTH_OK • <EUICC_SIGNATURE1> using the #PK_EUICC_ECDSA • <S_SMDP_CHALLENGE> present in the #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3 is the same as in <S_SMDP_SIGNED1> present in #INITIATE_AUTH_OK_VARIANT_A • operationType in #CTX_PARAMS_RPM_ICCID1 is set to #OP_TYPE_RPM • matchingIdSource in #CTX_PARAMS1_RPM_ICCID1 is set to NULL • for #DEVICE_INFO (deviceCapabilities) verify that: - TAC is BCD coded as 4 octets acc. to 3GPP TS 23.003 - if IMEI is present then it is BCD coded as 8 octets acc. to 3GPP TS 23.003 - if O_D_GSM_GERAN then gsmSupportedRelease is set to the highest release as defined in #IUT_GSM_GERAN_REL. – if O_D_UMTS_UTRAN then utranSupportedRelease is set to the highest release as defined in #IUT_UMTS_UTRAN_REL. – if O_D_CDMA2000_1X then cdma2000onexSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_1X_REL. – if O_D_CDMA2000_HRPD then cdma2000hrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_HRPD_REL. The value R is either 1, 2 or 3 for Rev 0, A or B respectively. – if O_D_CDMA2000_EHRPD then cdma2000ehrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_EHRPD_REL. – if O_D_LTE then eutranSupportedRelease is set to the highest release as defined in #IUT_LTE_EUTRAN_REL. – if O_D_NFC_TS26 then contactlessSupportedRelease is set to the highest release as defined in #IUT_NFC_REL. – if O_D_CRL then rspCrlSupportedVersion is set to the highest release as defined in #IUT_RSP_VERSION . – if O_D_NR_EPC then nrEpcSupportedRelease is set to the highest release as defined in #IUT_NR_EPC_REL, – if O_D_NR_5GC then nr5gcSupportedRelease is set to the highest release as defined in #IUT_NR_5GC_REL, – if O_D_EUTRAN_5GC then eutran5gcSupportedRelease is set to the highest release as defined in #IUT_EUTRAN_5GC_REL, - lpaSvn is present. - catSupportedClasses, the set of supported Card Application Toolkit letter classes as defined in #IUT_CAT_CLASSES. - euiccFormFactorType , the eUICC form factor as defined in #IUT_EUICC_FORM_FACTOR. For each of the options O_D_GSM_GERAN, O_D_UMTS_UTRAN, O_D_CDMA2000_1X, O_D_CDMA2000_HRPD, O_D_CDMA2000_EHRPD, O_D_LTE, O_D_NFC_TS26, O_D_CRL, O_D_NR_EPC, O_D_NR_5GC or O_D_EUTRAN_5GC if the option is not set, verify that the corresponding field in DeviceCapabilities is not present. • for #DEVICE_INFO (lpaRspCapabilities) verify that: - lpaRspCapabilities is set to #IUT_LPA_RSP_CAPABILITY |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_RPM_OK) | No Error LPAd initiates Simple Confirmation. S_EndUser accepts RPM Command. |

Test Sequence #02 Nominal: Authenticate Client Variant B for RPM

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_RPM_CONF_EN is loaded and disabled. eUICC returns CTX_PARAMS1_RPM_ICCID1 (sends by the LPAd) in euiccSigned1 in the response to ES10.AuthenticateServer. |
| LPAd | Update Profile operation is initiated and PROFILE_OPERATIONAL1 is selected. Polling Address is retrieved from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 |  | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, #IUT_LPA_RSP_CAPABILITY) ) • Extract <EUICC_CHALLENGE> |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#INITIATE_AUTH_OK_VARIANT_B) | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3) ) Verify: • if #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3 used with the #MATCHING_ID_1 • If <S_TRANSACTION_ID> is the same as in #INITIATE_AUTH_OK • <EUICC_SIGNATURE1> using the #PK_EUICC_SIG • if <S_SMDP_CHALLENGE> present in the #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3 is the same as in <S_SMDP_SIGNED1> present in #INITIATE_AUTH_OK_VARIANT_B • if operationType in #CTX_PARAMS_RPM_ICCID1 is set to #OP_TYPE_RPM • if matchingIdSource in #CTX_PARAMS1_RPM_ICCID1 is set to NULL • for #DEVICE_INFO (deviceCapabilities) verify: - TAC is BCD coded as 4 octets acc. to 3GPP TS 23.003 - if IMEI is present then it is BCD coded as 8 octets acc. to 3GPP TS 23.003 - if O_D_GSM_GERAN then gsmSupportedRelease is set to the highest release as defined in #IUT_GSM_GERAN_REL. – if O_D_UMTS_UTRAN then utranSupportedRelease is set to the highest release as defined in #IUT_UMTS_UTRAN_REL. – if O_D_CDMA2000_1X then cdma2000onexSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_1X_REL. – if O_D_CDMA2000_HRPD then cdma2000hrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_HRPD_REL. The value R is either 1, 2 or 3 for Rev 0, A or B respectively. – if O_D_CDMA2000_EHRPD then cdma2000ehrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_EHRPD_REL. – if O_D_LTE then eutranSupportedRelease is set to the highest release as defined in #IUT_LTE_EUTRAN_REL. – if O_D_NFC_TS26 then contactlessSupportedRelease is set to the highest release as defined in #IUT_NFC_REL. – if O_D_CRL then rspCrlSupportedVersion is set to the highest release as defined in #IUT_RSP_VERSION . – if O_D_NR_EPC then nrEpcSupportedRelease is set to the highest release as defined in #IUT_NR_EPC_REL, – if O_D_NR_5GC then nr5gcSupportedRelease is set to the highest release as defined in #IUT_NR_5GC_REL, – if O_D_EUTRAN_5GC then eutran5gcSupportedRelease is set to the highest release as defined in #IUT_EUTRAN_5GC_REL, - lpaSvn is present. - catSupportedClasses, the set of supported Card Application Toolkit letter classes as defined in #IUT_CAT_CLASSES. - euiccFormFactorType , the eUICC form factor as defined in #IUT_EUICC_FORM_FACTOR. For each of the options O_D_GSM_GERAN, O_D_UMTS_UTRAN, O_D_CDMA2000_1X, O_D_CDMA2000_HRPD, O_D_CDMA2000_EHRPD, O_D_LTE, O_D_NFC_TS26, O_D_CRL, O_D_NR_EPC, O_D_NR_5GC or O_D_EUTRAN_5GC if the option is not set, verify that the corresponding field in DeviceCapabilities is not present. • for #DEVICE_INFO (lpaRspCapabilities) verify: - lpaRspCapabilities is set to #IUT_LPA_RSP_CAPABILITY |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_RPM_OK) | No Error LPAd initiates Simple Confirmation. S_EndUser accepts RPM Command. |

Test Sequence #03 Nominal: Authenticate Client Variant C for RPM

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_RPM_CONF_EN is loaded and disabled. eUICC returns CTX_PARAMS1_RPM_ICCID1 (sends by the LPAd) in euiccSigned1 in the response to ES10.AuthenticateServer. |
| LPAd | Update Profile operation is initiated and PROFILE_OPERATIONAL1 is selected. Polling Address is retrieved from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 |  | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, #IUT_LPA_RSP_CAPABILITY) ) • Extract <EUICC_CHALLENGE> |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#INITIATE_AUTH_OK_VARIANT_C) | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3) ) Verify: • if #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3 used with the #MATCHING_ID_1 • If <S_TRANSACTION_ID> is the same as in #INITIATE_AUTH_OK • <EUICC_SIGNATURE1> using the #PK_EUICC_SIG • if <S_SMDP_CHALLENGE> present in the #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3 is the same as in <S_SMDP_SIGNED1> present in #INITIATE_AUTH_OK_VARIANT_C • if operationType in #CTX_PARAMS_RPM_ICCID1 is set to #OP_TYPE_RPM • if matchingIdSource in #CTX_PARAMS1_RPM_ICCID1 is set to NULL • for #DEVICE_INFO (deviceCapabilities) verify: - TAC is BCD coded as 4 octets acc. to 3GPP TS 23.003 - if IMEI is present then it is BCD coded as 8 octets acc. to 3GPP TS 23.003 - if O_D_GSM_GERAN then gsmSupportedRelease is set to the highest release as defined in #IUT_GSM_GERAN_REL. – if O_D_UMTS_UTRAN then utranSupportedRelease is set to the highest release as defined in #IUT_UMTS_UTRAN_REL. – if O_D_CDMA2000_1X then cdma2000onexSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_1X_REL. – if O_D_CDMA2000_HRPD then cdma2000hrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_HRPD_REL. The value R is either 1, 2 or 3 for Rev 0, A or B respectively. – if O_D_CDMA2000_EHRPD then cdma2000ehrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_EHRPD_REL. – if O_D_LTE then eutranSupportedRelease is set to the highest release as defined in #IUT_LTE_EUTRAN_REL. – if O_D_NFC_TS26 then contactlessSupportedRelease is set to the highest release as defined in #IUT_NFC_REL. – if O_D_CRL then rspCrlSupportedVersion is set to the highest release as defined in #IUT_RSP_VERSION . – if O_D_NR_EPC then nrEpcSupportedRelease is set to the highest release as defined in #IUT_NR_EPC_REL, – if O_D_NR_5GC then nr5gcSupportedRelease is set to the highest release as defined in #IUT_NR_5GC_REL, – if O_D_EUTRAN_5GC then eutran5gcSupportedRelease is set to the highest release as defined in #IUT_EUTRAN_5GC_REL, - lpaSvn is present. - catSupportedClasses, the set of supported Card Application Toolkit letter classes as defined in #IUT_CAT_CLASSES. - euiccFormFactorType , the eUICC form factor as defined in #IUT_EUICC_FORM_FACTOR. For each of the options O_D_GSM_GERAN, O_D_UMTS_UTRAN, O_D_CDMA2000_1X, O_D_CDMA2000_HRPD, O_D_CDMA2000_EHRPD, O_D_LTE, O_D_NFC_TS26, O_D_CRL, O_D_NR_EPC, O_D_NR_5GC or O_D_EUTRAN_5GC if the option is not set, verify that the corresponding field in DeviceCapabilities is not present. • for #DEVICE_INFO (lpaRspCapabilities) verify: - lpaRspCapabilities is set to #IUT_LPA_RSP_CAPABILITY |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_RPM_OK) | No Error LPAd initiates Simple Confirmation. S_EndUser accepts RPM Command. |

Test Sequence #04 Nominal: Authenticate Client Variant A for RPM, with PPR

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_PPR_RPM_CONF_EN is loaded and disabled. eUICC returns CTX_PARAMS1_RPM_ICCID1 (sends by the LPAd) in euiccSigned1 in the response to ES10.AuthenticateServer. |
| LPAd | Update Profile operation is initiated and PROFILE_OPERATIONAL1 is selected. Polling Address is retrieved from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 |  | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, #IUT_LPA_RSP_CAPABILITY) ) • Extract <EUICC_CHALLENGE> |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#INITIATE_AUTH_OK_VARIANT_A) | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3) ) Verify: • if #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3 used with the #MATCHING_ID_1 • If <S_TRANSACTION_ID> is the same as in #INITIATE_AUTH_OK • <EUICC_SIGNATURE1> using the #PK_EUICC_SIG • if <S_SMDP_CHALLENGE> present in the #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3 is the same as in <S_SMDP_SIGNED1> present in #INITIATE_AUTH_OK_VARIANT_A • if operationType in #CTX_PARAMS_RPM_ICCID1 is set to #OP_TYPE_RPM • if matchingIdSource in #CTX_PARAMS1_RPM_ICCID1 is set to NULL • for #DEVICE_INFO (deviceCapabilities) verify: - TAC is BCD coded as 4 octets acc. to 3GPP TS 23.003 - if IMEI is present then it is BCD coded as 8 octets acc. to 3GPP TS 23.003 - if O_D_GSM_GERAN then gsmSupportedRelease is set to the highest release as defined in #IUT_GSM_GERAN_REL. – if O_D_UMTS_UTRAN then utranSupportedRelease is set to the highest release as defined in #IUT_UMTS_UTRAN_REL. – if O_D_CDMA2000_1X then cdma2000onexSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_1X_REL. – if O_D_CDMA2000_HRPD then cdma2000hrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_HRPD_REL. The value R is either 1, 2 or 3 for Rev 0, A or B respectively. – if O_D_CDMA2000_EHRPD then cdma2000ehrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_EHRPD_REL. – if O_D_LTE then eutranSupportedRelease is set to the highest release as defined in #IUT_LTE_EUTRAN_REL. – if O_D_NFC_TS26 then contactlessSupportedRelease is set to the highest release as defined in #IUT_NFC_REL. – if O_D_CRL then rspCrlSupportedVersion is set to the highest release as defined in #IUT_RSP_VERSION . – if O_D_NR_EPC then nrEpcSupportedRelease is set to the highest release as defined in #IUT_NR_EPC_REL, – if O_D_NR_5GC then nr5gcSupportedRelease is set to the highest release as defined in #IUT_NR_5GC_REL, – if O_D_EUTRAN_5GC then eutran5gcSupportedRelease is set to the highest release as defined in #IUT_EUTRAN_5GC_REL, - lpaSvn, the highest SGP.22 version of the LPA as defined in #LPA_RSP_VERSION_HIGHEST. - catSupportedClasses, the set of supported Card Application Toolkit letter classes as defined in #IUT_CAT_CLASSES. - euiccFormFactorType , the eUICC form factor as defined in #IUT_EUICC_FORM_FACTOR. For each of the options O_D_GSM_GERAN, O_D_UMTS_UTRAN, O_D_CDMA2000_1X, O_D_CDMA2000_HRPD, O_D_CDMA2000_EHRPD, O_D_LTE, O_D_NFC_TS26, O_D_CRL, O_D_NR_EPC, O_D_NR_5GC or O_D_EUTRAN_5GC if the option is not set, verify that the corresponding field in DeviceCapabilities is not present. • for #DEVICE_INFO (lpaRspCapabilities) verify: - lpaRspCapabilities is set to #IUT_LPA_RSP_CAPABILITY |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_RPM_OK) | No Error - LPAd initiates Simple Confirmation - LPAd may display relevant information concerning the Profile and PPR(s) to the S_EndUser and requests the User Consent. It may be combined with Simple Confirmation. - S_EndUser approves RPM Command. |

Test Sequence #05 Nominal: Authenticate Client Variant A for RPM – PPR Update

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_CONF_UPDATE_MD_PPR is loaded and disabled. eUICC returns CTX_PARAMS1_RPM_ICCID1 (sent by the LPAd) in euiccSigned1 in the response to ES10.AuthenticateServer. |
| LPAd | Update Profile operation is initiated and PROFILE_OPERATIONAL1 is selected. Polling Address is retrieved from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) with RPM Command Update Metadata for removing PPRs. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 |  | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, #IUT_LPA_RSP_CAPABILITY) ) • Extract <EUICC_CHALLENGE> |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#INITIATE_AUTH_OK_VARIANT_A) | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3) ) Verify that: • #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3 used with the #MATCHING_ID_1 • <S_TRANSACTION_ID> is the same as in #INITIATE_AUTH_OK • <EUICC_SIGNATURE1> using the #PK_EUICC_ECDSA • <S_SMDP_CHALLENGE> present in the #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3 is the same as in <S_SMDP_SIGNED1> present in #INITIATE_AUTH_OK_VARIANT_A • operationType in #CTX_PARAMS_RPM_ICCID1 is set to #OP_TYPE_RPM • matchingIdSource in #CTX_PARAMS1_RPM_ICCID1 is set to NULL • for #DEVICE_INFO (deviceCapabilities) verify that: - TAC is BCD coded as 4 octets acc. to 3GPP TS 23.003 - if IMEI is present then it is BCD coded as 8 octets acc. to 3GPP TS 23.003 - if O_D_GSM_GERAN then gsmSupportedRelease is set to the highest release as defined in #IUT_GSM_GERAN_REL. – if O_D_UMTS_UTRAN then utranSupportedRelease is set to the highest release as defined in #IUT_UMTS_UTRAN_REL. – if O_D_CDMA2000_1X then cdma2000onexSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_1X_REL. – if O_D_CDMA2000_HRPD then cdma2000hrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_HRPD_REL. The value R is either 1, 2 or 3 for Rev 0, A or B respectively. – if O_D_CDMA2000_EHRPD then cdma2000ehrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_EHRPD_REL. – if O_D_LTE then eutranSupportedRelease is set to the highest release as defined in #IUT_LTE_EUTRAN_REL. – if O_D_NFC_TS26 then contactlessSupportedRelease is set to the highest release as defined in #IUT_NFC_REL. – if O_D_CRL then rspCrlSupportedVersion is set to the highest release as defined in #IUT_RSP_VERSION . – if O_D_NR_EPC then nrEpcSupportedRelease is set to the highest release as defined in #IUT_NR_EPC_REL, – if O_D_NR_5GC then nr5gcSupportedRelease is set to the highest release as defined in #IUT_NR_5GC_REL, – if O_D_EUTRAN_5GC then eutran5gcSupportedRelease is set to the highest release as defined in #IUT_EUTRAN_5GC_REL, - lpaSvn is present - catSupportedClasses, the set of supported Card Application Toolkit letter classes as defined in #IUT_CAT_CLASSES. - euiccFormFactorType , the eUICC form factor as defined in #IUT_EUICC_FORM_FACTOR. For each of the options O_D_GSM_GERAN, O_D_UMTS_UTRAN, O_D_CDMA2000_1X, O_D_CDMA2000_HRPD, O_D_CDMA2000_EHRPD, O_D_LTE, O_D_NFC_TS26, O_D_CRL, O_D_NR_EPC, O_D_NR_5GC or O_D_EUTRAN_5GC if the option is not set, verify that the corresponding field in DeviceCapabilities is not present. • for #DEVICE_INFO (lpaRspCapabilities) verify that: - lpaRspCapabilities is set to #IUT_LPA_RSP_CAPABILITY |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_RPM_UM_PPR_OK ) | LPA may optionally initiate User Consent. and S_EndUser approves RPM Command. Next step of RPM Package handling procedure is performed (i.e. ES9+.HandleNotification is sent). |

##### 4.4.30.2.2     TC_LPAd_AuthenticateClient_Nominal_RPM_Enterprise Device

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled |
| eUICC | There is no default SM-DP+ address configured The eUICC supports RPM and Enterprise profiles |
| LPAd | Variant A certificates are included in certificates chain for TLS procedures. RPM operation is enabled in the LPA by the End User |

Test Sequence #01 Nominal: Authenticate Client Variant A for RPM, Update Reference Enterprise Rule

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Entity |  |  | Description of the initial condition |  |
| eUICC |  |  | PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_UM_ENT_RULES is loaded and disabled. eUICC returns CTX_PARAMS1_RPM_ICCID1 (sends by the LPAd) in euiccSigned1 in the response to ES10.AuthenticateServer. |  |
| LPAd |  |  | Update Profile operation is initiated and PROFILE_OPERATIONAL1 is selected. Polling Address is retrieved from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |  |
| S_SM-DP+ |  |  | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) for Update Reference Enterprise Rules |  |
| Step | Direction | Sequence / Description |  | Expected result |
| IC1 |  | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| IC2 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method |  | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1, #IUT_LPA_RSP_CAPABILITY) ) • Extract <EUICC_CHALLENGE> |
| 1 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#INITIATE_AUTH_OK_VARIANT_A) |  | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3) ) Verify: • if #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3 used with the #MATCHING_ID_1 • If <S_TRANSACTION_ID> is the same as in #INITIATE_AUTH_OK • <EUICC_SIGNATURE1> using the #PK_EUICC_SIG • if <S_SMDP_CHALLENGE> present in the #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3 is the same as in <S_SMDP_SIGNED1> present in #INITIATE_AUTH_OK_VARIANT_A • if operationType in #CTX_PARAMS_RPM_ICCID1 is set to #OP_TYPE_RPM • if matchingIdSource in #CTX_PARAMS1_RPM_ICCID1 is set to NULL • for #DEVICE_INFO (deviceCapabilities) verify: - TAC is BCD coded as 4 octets acc. to 3GPP TS 23.003 - if IMEI is present then it is BCD coded as 8 octets acc. to 3GPP TS 23.003 - if O_D_GSM_GERAN then gsmSupportedRelease is set to the highest release as defined in #IUT_GSM_GERAN_REL. – if O_D_UMTS_UTRAN then utranSupportedRelease is set to the highest release as defined in #IUT_UMTS_UTRAN_REL. – if O_D_CDMA2000_1X then cdma2000onexSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_1X_REL. – if O_D_CDMA2000_HRPD then cdma2000hrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_HRPD_REL. The value R is either 1, 2 or 3 for Rev 0, A or B respectively. – if O_D_CDMA2000_EHRPD then cdma2000ehrpdSupportedRelease is set to the highest release as defined in #IUT_CDMA2000_EHRPD_REL. – if O_D_LTE then eutranSupportedRelease is set to the highest release as defined in #IUT_LTE_EUTRAN_REL. – if O_D_NFC_TS26 then contactlessSupportedRelease is set to the highest release as defined in #IUT_NFC_REL. – if O_D_CRL then rspCrlSupportedVersion is set to the highest release as defined in #IUT_RSP_VERSION . – if O_D_NR_EPC then nrEpcSupportedRelease is set to the highest release as defined in #IUT_NR_EPC_REL, – if O_D_NR_5GC then nr5gcSupportedRelease is set to the highest release as defined in #IUT_NR_5GC_REL, – if O_D_EUTRAN_5GC then eutran5gcSupportedRelease is set to the highest release as defined in #IUT_EUTRAN_5GC_REL, - lpaSvn, the highest SGP.22 version of the LPA as defined in #LPA_RSP_VERSION_HIGHEST. - catSupportedClasses, the set of supported Card Application Toolkit letter classes as defined in #IUT_CAT_CLASSES. - euiccFormFactorType , the eUICC form factor as defined in #IUT_EUICC_FORM_FACTOR. For each of the options O_D_GSM_GERAN, O_D_UMTS_UTRAN, O_D_CDMA2000_1X, O_D_CDMA2000_HRPD, O_D_CDMA2000_EHRPD, O_D_LTE, O_D_NFC_TS26, O_D_CRL, O_D_NR_EPC, O_D_NR_5GC or O_D_EUTRAN_5GC if the option is not set, verify that the corresponding field in DeviceCapabilities is not present. • for #DEVICE_INFO (lpaRspCapabilities) verify: - lpaRspCapabilities is set to #IUT_LPA_RSP_CAPABILITY |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_RPM_OK) |  | No Error - LPAd initiates Simple Confirmation - LPAd may display relevant information concerning the Enterprise Rules to the S_EndUser and requests the User Consent. It may be combined with Simple Confirmation. - S_EndUser approves RPM Command. |

### 4.4.32	ES9+ (LPA – SM-DP+): HandleNotification for RPM Package Download

#### 4.4.32.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

2.6.6.2, 2.10.1

3.0.1

3.2.7, 3.7.2

3.7.2

5.5.3, 5.6.3, 5.7.14a

6.6.2.4

#### 4.4.32.2	Test Cases

##### 4.4.32.2.1     TC_LPAd_RPM_ HandleNotification for RPM Package Download

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled |
| Device | RPM operation is enabled in the LPA by the End User |
| eUICC | There is no default SM-DP+ address configured The eUICC supports RPM |
| S_SM-DP+ | Variant A certificates are included in certificates chain for TLS procedures. |

Test Sequence #01 Nominal: Successful RPR and Other Notification for RPM Enable operation to the Same SM-DP+ Address

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_CONF_EN_NOTIF_CONF is installed and disabled. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Update Profile operation | LPAd retrieves Polling Address from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH_AND_AUTH_CLIENT_REQ_V3 |  |  |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( MTD_AUTH_CLIENT_RPM_PKG_REQ_FOR_SINGLE_CMND( enable, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, <S_SM_DP+_SIGNATURE3>, NO_PARAM, NO_PARAM, NO_PARAM ) ) | No error |
| 5 | LPAd → S_EndUser | Request for User Confirmation. | LPAd initiates Confirmation Request for Simple Confirmation |
| 6 | S_EndUser | Accepts the RPM command | PROFILE_OPERATIONAL1 is enabled |
| 7 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( MTD_RESP_RPR_FOR_SINGLE_CMND( enableResult, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, 0, -- OK response #NOTIF_METADATA_PROF1_DP1_RPR, #S_SM_DP+_OID, NO_PARAM, NO_PARAM, NO_PARAM) ) ) |
| 8 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |
| 9 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF(#PENDING_NOTIF_RPM_EN1) ) |
| 10 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |

Test Sequence #02 Nominal: Successful RPR and Other Notifications to different SM-DP+ Address (implicit disabling of the formerly enabled Profile)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_CONF_EN_NOTIF_CONF2 is installed and disabled. |
| eUICC | The PROFILE_OPERATIONAL2 with #METADATA_OP_PROF2_RPM_CONF_EN_OWNER_OID1_AND_NOTIF_CONF (without PPR1 present) is loaded and Enabled on the eUICC. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Update Profile operation | LPAd retrieves Polling Address from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH_AND_AUTH_CLIENT_REQ_V3 |  |  |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( MTD_AUTH_CLIENT_RPM_PKG_REQ_FOR_SINGLE_CMND( enable, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, <S_SM_DP+_SIGNATURE3>, NO_PARAM, NO_PARAM, NO_PARAM ) ) | No error |
| 5 | LPAd → S_EndUser | Request for User Confirmation. | LPAd initiates Confirmation Request for Simple Confirmation |
| 6 | S_EndUser | Accepts the RPM command | PROFILE_OPERATIONAL1 is enabled |
| 7 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS2, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #PENDING_NOTIF_IMPLICIT_DIS2) ) |
| 8 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |
| 9 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( MTD_RESP_RPR_FOR_SINGLE_CMND( enableResult, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, 0, -- OK response #NOTIF_METADATA_PROF1_DP1_RPR, #S_SM_DP+_OID, NO_PARAM, NO_PARAM, NO_PARAM) ) ) |
| 10 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |
| 11 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS2, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( #PENDING_NOTIF_RPM_EN2) ) |
| 12 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |

## 4.5	VOID

## 4.6	VOID

## 4.7	VOID

# 5	Procedure - Behaviour Testing

## 5.1	General Overview

## 5.2	VOID

## 5.3	VOID

## 5.4	Device Procedures

### 5.4.1	Local Profile Management - Add Profile

#### 5.4.1.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

Section 2.9.2.1

Section 3.0.1, 3.1.3, 3.1.3.2

Section 3.2, 3.2.5

Section 4.1, 4.4

Annex C.1, C.3

#### 5.4.1.2	Test Cases

##### 5.4.1.2.1	TC_LPAd_AddProfile_Manual_Entry

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Add a new Operational Profile by using Activation Code (manual entry)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1). |
| eUICC | There is no default SM-DP+ address configured. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Add Profile operation | LPAd requests the Activation Code from the End User |
| 2 | S_EndUser→ LPAd | Provide #ACTIVATION_CODE_1 by manual entry | No error |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 4 | PROC_ES9+_INIT_AUTH |  |  |
| 5 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_1 as <MATCHING_ID> |  |  |
| 6 | PROC_ES9+_GET_BPP (see NOTE 1) |  |  |
| 7 | LPAd → S_EndUser | Request for Confirmation, if not requested before. | End User Intent successfully verified, if not verified before. |
| 8 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 9 | S_EndUser → LPAd | List Profile operation is initiated | PROFILE_OPERATIONAL1 is displayed in Disabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. |  |  |  |

Test Sequence #02 Nominal: Add a new Operational Profile by using Activation Code (manual entry) with Confirmation Code

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_3 (PROFILE_OPERATIONAL1) associated with #CONFIRMATION_CODE1. |
| eUICC | There is no default SM-DP+ address configured. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Add Profile operation | LPAd requests the Activation Code from the S_End User |
| 2 | S_EndUser→ LPAd | Provide #ACTIVATION_CODE_3 by manual entry | No error |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 4 | PROC_ES9+_INIT_AUTH |  |  |
| 5 | PROC_ES9+_AUTH_CLIENT_CC with #MATCHING_ID_3 as <MATCHING_ID> |  |  |
| 6 | LPAd → S_EndUser | LPAd requests the Confirmation Code from the S_End User. | CONFIRMATION_CODE1 is provided by manual entry. |
| 7 | PROC_ES9+_GET_BPP_CC (see NOTE 1) |  |  |
| 8 | LPAd → S_EndUser | Request for Confirmation, if not requested before. (see NOTE 2) | End User Intent successfully verified, if not verified before. (see NOTE 2) |
| 9 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 10 | S_EndUser → LPAd | Initiate List Profile operation | PROFILE_OPERATIONAL1 is displayed in Disabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. NOTE 2: The LPAd MAY skip this request for Confirmation. If so, it SHALL NOT be regarded as a failure. |  |  |  |

##### 5.4.1.2.2	TC_LPAd_AddProfile_QRcode_scanning

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Add a new Operational Profile by using Activation Code (QR code scanning)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1). |
| eUICC | There is no default SM-DP+ address configured. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Add Profile operation | LPAd requests the Activation Code from the End User |
| 2 | S_EndUser→ LPAd | Provide #ACTIVATION_CODE_1 by scanning the QR code | No error |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 4 | PROC_ES9+_INIT_AUTH |  |  |
| 5 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_1 as <MATCHING_ID> |  |  |
| 6 | PROC_ES9+_GET_BPP (see NOTE 1) |  |  |
| 7 | LPAd → S_EndUser | Request for Confirmation, if not requested before. | End User Intent successfully verified, if not verified before. |
| 8 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 9 | S_EndUser→ LPAd | Initiate List Profile operation | PROFILE_OPERATIONAL1 is displayed in Disabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. |  |  |  |

##### 5.4.1.2.3	TC_LPAd_AddProfile_ActivationCode_InvalidFormat_QRcode

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| eUICC | The PROFILE_OPERATIONAL1 is not installed on the eUICC. |

Test Sequence #01 Error: Add a new Operational Profile by using wrongly formatted Activation Code (QR code scanning)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | There is no default SM-DP+ address configured. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Add Profile operation | Activation Code is requested from the End User by LPAd |
| 2 | S_EndUser→ LPAd | Provide #ACTIVATION_CODE_INVALID_FORMAT by scanning the QR code | LPAd provides an error message to the EndUser |
| 3 | S_EndUser→ LPAd | Initiate List Profile operation | PROFILE_OPERATIONAL1 is not displayed |

##### 5.4.1.2.4	TC_LPAd_AddProfile_ActivationCode_InvalidFormat_ManualEntry

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Error: Add a new Operational Profile by using wrongly formatted Activation Code (Manual entry)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | There is no default SM-DP+ address configured. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Add Profile operation | Activation Code is requested from the End User by LPAd |
| 2 | S_EndUser→ LPAd | Provide #ACTIVATION_CODE_INVALID_FORMAT by manual entry | LPAd provides an error message to the EndUser |
| 3 | S_EndUser→ LPAd | Initiate List Profile operation | PROFILE_OPERATIONAL1 is not displayed |

##### 5.4.1.2.5	TC_LPAd_AddProfile_ConfirmationCode_smdpSigned2_QR

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Add a new Operational Profile by using Activation Code (QR code scanning) with confirmation code indicated only in smdpSigned2

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) which requires confirmation code. |
| eUICC | There is no default SM-DP+ address configured. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Add Profile operation | Activation Code is requested from the End User by LPAd |
| 2 | S_EndUser→ LPAd | Provide #ACTIVATION_CODE_1 by scanning the QR code | No error |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 4 | PROC_ES9+_INIT_AUTH |  |  |
| 5 | PROC_ES9+_AUTH_CLIENT_CC with #MATCHING_ID_1 as <MATCHING_ID> |  |  |
| 6 | LPAd → S_EndUser | Request the Confirmation Code from the S_End User. | #CONFIRMATION_CODE1 is provided by manual entry. |
| 7 | PROC_ES9+_GET_BPP_CC (see NOTE 1) |  |  |
| 8 | LPAd → S_EndUser | Request for Confirmation, if not requested before. (see NOTE 2) | End User Intent successfully verified, if not verified before. (see NOTE 2) |
| 9 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 10 | S_EndUser→ LPAd | Initiate List Profile operation | PROFILE_OPERATIONAL1 is displayed in Disabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. NOTE 2: The LPAd MAY skip this request for Confirmation. If so, it SHALL NOT be regarded as a failure. |  |  |  |

##### 5.4.1.2.6	TC_LPAd_AddProfile_ConfirmationCode_smdpSigned2_Manual_Entry

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Add a new Operational Profile by using Activation Code (manual entry) with confirmation code indicated only in smdpSigned2

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) which requires confirmation code. |
| eUICC | There is no default SM-DP+ address configured. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Add Profile operation | Activation Code is requested from the End User by LPAd |
| 2 | S_EndUser→ LPAd | Provide #ACTIVATION_CODE_1 by manual entry | No error |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9 |  |  |
| 4 | PROC_ES9+_INIT_AUTH |  |  |
| 5 | PROC_ES9+_AUTH_CLIENT_CC with #MATCHING_ID_1 as <MATCHING_ID> |  |  |
| 6 | LPAd → S_EndUser | Request the Confirmation Code from the S_End User. | #CONFIRMATION_CODE1 is provided by manual entry. |
| 7 | PROC_ES9+_GET_BPP_CC (see NOTE 1) |  |  |
| 8 | LPAd → S_EndUser | Request for Confirmation, if not requested before. (see NOTE 2) | End User Intent successfully verified, if not verified before. (see NOTE 2) |
| 9 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 10 | S_EndUser → LPAd | Initiate List Profile operation | PROFILE_OPERATIONAL1 is displayed in Disabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. NOTE 2: The LPAd MAY skip this request for Confirmation. If so, it SHALL NOT be regarded as a failure. |  |  |  |

##### 5.4.1.2.7	TC_LPAd_AddProfile_default_SM-DP+_address

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Add a new Operational Profile by using the default SM-DP+ Address

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for PROFILE_OPERATIONAL1 linked to the EID of the eUICC. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Add Profile operation See NOTE1 | No error |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH |  |  |
| 4 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_EMPTY as <MATCHING_ID> or missing MatchingID data object |  |  |
| 5 | PROC_ES9+_GET_BPP (see NOTE 2) |  |  |
| 6 | LPAd → S_EndUser | Request for Confirmation, if not requested before. | End User Intent successfully verified, if not verified before. |
| 7 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 8 | S_EndUser → LPAd | Initiate List Profile operation | PROFILE_OPERATIONAL1 is displayed in Disabled state |
| NOTE 1: The Profile download by default SM-DP+ address MAY be implemented in different ways (e.g. some Device MAY implement a separate LUI menu for this function, some Device MAY request first the activation code, etc.). In order to enforce that the default SM-DP+ address is used the user SHALL not enter the Activation Code in case it is requested. NOTE 2: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. |  |  |  |

##### 5.4.1.2.8	TC_LPAd_AddProfile _with_ConfirmationCode

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Add a new Operational Profile by using Activation Code (QR code scanning) with confirmation code

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_3 (PROFILE_OPERATIONAL1). |
| eUICC | There is no default SM-DP+ address configured. |

| Step | Direction | Sequence / Description | Expected result. |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Add Profile operation | Activation Code is requested from the End User by LPAd |
| 2 | S_EndUser→ LPAd | Provide#ACTIVATION_CODE_3 by scanning the QR code |  |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 4 | PROC_ES9+_INIT_AUTH |  |  |
| 5 | PROC_ES9+_AUTH_CLIENT_CC with #MATCHING_ID_3 as <MATCHING_ID> |  |  |
| 6 | LPAd → S_EndUser | Request the Confirmation Code from the S_End User. | #CONFIRMATION_CODE1 is provided by manual entry. |
| 7 | PROC_ES9+_GET_BPP_CC (see NOTE 1) |  |  |
| 8 | LPAd → S_EndUser | Request for Confirmation, if not requested before. (see NOTE 2) | End User Intent successfully verified, if not verified before. (see NOTE 2) |
| 9 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 10 | S_EndUser→ LPAd | Initiate List Profile operation | PROFILE_OPERATIONAL1 is displayed in Disabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. NOTE 2: The LPAd MAY skip this request for Confirmation. If so, it SHALL NOT be regarded as a failure. |  |  |  |

Test Sequence #02 Nominal: Add a new Operational Profile by using Activation Code (manual) with confirmation code

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_3 (PROFILE_OPERATIONAL1). |
| eUICC | There is no default SM-DP+ address configured. |

| Step | Direction | Sequence / Description | Expected result. |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Add Profile operation | Activation Code is requested from the End User by LPAd |
| 2 | S_EndUser→ LPAd | Provide#ACTIVATION_CODE_3 by manual entry |  |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 4 | PROC_ES9+_INIT_AUTH |  |  |
| 5 | PROC_ES9+_AUTH_CLIENT_CC with #MATCHING_ID_3 as <MATCHING_ID> |  |  |
| 6 | LPAd → S_EndUser | Request the Confirmation Code from the S_End User. | #CONFIRMATION_CODE1 is provided by manual entry. |
| 7 | PROC_ES9+_GET_BPP_CC (see NOTE 1) |  |  |
| 8 | LPAd → S_EndUser | For LPAd supporting SGP.22 v2.2.2 or earlier: Request for Confirmation, if not requested before. (see NOTE 2) | For LPAd supporting SGP.22 v2.2.2 or earlier: End User Intent successfully verified, if not verified before. (see NOTE 2) |
| 9 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 10 | S_EndUser→ LPAd | Initiate List Profile operation | PROFILE_OPERATIONAL1 is displayed in Disabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. NOTE 2: The LPAd MAY skip this request for Confirmation. If so, it SHALL NOT be regarded as a failure. |  |  |  |

##### 5.4.1.2.9	TC_LPAd_AddProfile_PPRs

Test Sequence #01 Nominal: End User Confirmation after PPR1 consent requested

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The Test eUICC’s RAT is configured as follows: PPR1 is allowed and End User Consent is required for #MCC_MNC4 with gid1 and gid2 absent. |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_4. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_4 (associated with PROFILE_OPERATIONAL4). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT Extract <S_TRANSACTION_ID> |  |  |
| IC4 | PROC_ES9+_GET_BPP with #METADATA_OP_PROF4 used in #GET_BPP_OK |  |  |
| 1 | LPAd → S_EndUser | Request for Confirmation if not requested before. | The LPA provides means for the End User Confirmation/Rejection of the Profile Download either at this point or at a previous point of the procedure. Either Strong Confirmation is asked by showing relevant information concerning the PPR(s); or Simple Confirmation is asked on the Profile download. (See NOTE) |
| 2 | S_EndUser → LPAd | End User Confirmation is performed within the period as defined in #IUT_EU_CONFIRMATION_TIMEOUT |  |
| 3 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 4 | S_EndUser → LPAd | List Profile operation is initiated | PROFILE_OPERATIONAL4 is displayed in Disabled state |
| NOTE: The request for this End User consent/Confirmation for the installation of Profile Policy Rules and Profile download MAY be combined into a single prompt. |  |  |  |

Test Sequence #02 Nominal: End User Confirmation after PPR2 consent requested

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The Test eUICC’s RAT is configured as follows: PPR2 is allowed and End User Consent is required for #MCC_MNC2 with gid1 and gid2 absent. |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_3_NO_CC. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_3 (associated with PROFILE_OPERATIONAL3). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT Extract <S_TRANSACTION_ID> |  |  |
| IC4 | PROC_ES9+_GET_BPP with #METADATA_OP_PROF3 used in #GET_BPP_OK |  |  |
| 1 | LPAd → S_EndUser | Request for Confirmation if not requested before. | The LPA provides means for the End User Confirmation/Rejection of the Profile Download either at this point or at a previous point of the procedure Either Strong Confirmation is asked by showing relevant information concerning the PPR(s); or Simple Confirmation is asked on the Profile download. (See NOTE) |
| 2 | S_EndUser → LPAd | End User Confirmation is performed within the period as defined in #IUT_EU_CONFIRMATION_TIMEOUT |  |
| 3 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 4 | S_EndUser → LPAd | List Profile operation is initiated | PROFILE_OPERATIONAL3 is displayed in Disabled state |
| NOTE: The request for this End User consent/Confirmation for the installation of Profile Policy Rules and Profile download MAY be combined into a single prompt. |  |  |  |

Test Sequence #03 Nominal: Profile with PPR1 already present

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| eUICC | The Test eUICC’s RAT is configured as follows: PPR1 is allowed for #MCC_MNC4 with gid1 and gid2 absent (with End User Consent either required or not required). |
| eUICC | The PROFILE_OPERATIONAL4 with PPR1 is installed and enabled on the eUICC. |
| LPAd | Add Profile operation is initiated by using #ACTIVATION_CODE_1. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (associated with PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT Extract <S_TRANSACTION_ID> |  |  |
| IC4 | PROC_ES9+_GET_BPP |  |  |
| 1 | LPAd → S_EndUser | Request for Confirmation if not requested before. | The LPA provides means for the End User Confirmation/Rejection of the Profile Download. End User advised about a Profile with PPR1 already present and the End User consent is requested if not requested before. |
| 2 | S_EndUser → LPAd | End User Confirmation is performed within the period as defined in #IUT_EU_CONFIRMATION_TIMEOUT |  |
| 3 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 4 | S_EndUser → LPAd | List Profile operation is initiated | PROFILE_OPERATIONAL1 is displayed in Disabled state |

##### 5.4.1.2.10	VOID

##### 5.4.1.2.11	TC_LPAd_AddProfile_Security_Errors

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| eUICC | The PROFILE_OPERATIONAL1 is not installed on the eUICC. |

Test Sequence #01 Error: Stop Add Profile Operation if No Confirmation Provided

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1). |
| eUICC | There is no default SM-DP+ address configured. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | S_EndUser→ LPAd | Initiate Add Profile operation | LPAd requests the Activation Code from the End User |
| IC2 | S_EndUser→ LPAd | Provide #ACTIVATION_CODE_1 | No error |
| IC3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ (See NOTE 2) |  |  |
| IC4 | PROC_ES9+_INIT_AUTH |  |  |
| IC5 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_1 as <MATCHING_ID> |  |  |
| IC6 | PROC_ES9+_GET_BPP (see NOTE 1) |  |  |
| 1 | LPAd → S_EndUser | Request for Confirmation, if not requested before. The End User SHALL not provide Confirmation. | The LPAd stops the Add Profile procedure automatically or provide means to stop the procedure by the End User. |
| 2 | S_EndUser → LPAd | List Profile operation is initiated | PROFILE_OPERATIONAL1 is not displayed |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction if there is a way to abort it in step 1. NOTE 2: Step IC6 is conditional – occurs only if Step 1 (Request for Confirmation) was not executed before. |  |  |  |

##### 5.4.1.2.12	 TC_LPAd_AddProfile_Empty_MatchingID

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Add a new Operational Profile by using empty MatchingID (QR code entry)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for PROFILE_OPERATIONAL1 linked to the EID of the eUICC. |

| Step | Direction | Sequence / Description | Expected result |  |
| --- | --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Add Profile operation | LPAd requests the Activation Code from the End User |  |
| 2 | S_EndUser→ LPAd | Provide #ACTIVATION_CODE_5 by scanning the QR code | No error |  |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |  |
| 4 | PROC_ES9+_INIT_AUTH |  |  |  |
| 5 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_EMPTY as <MATCHING_ID> |  |  |  |
| 6 | PROC_ES9+_GET_BPP (see NOTE 1) |  |  |  |
| 7 | LPAd → S_EndUser | Request for Confirmation, if not requested before. | End User Intent successfully verified, if not verified before. |  |
| 8 | PROC_ES9+_HANDLE_NOTIF |  |  |  |
| 9 | S_EndUser → LPAd | Initiate List Profile operation | PROFILE_OPERATIONAL1 is displayed in Disabled state |  |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. |  |  |  |  |

Test Sequence #02 Nominal: Add a new Operational Profile by using empty MatchingID (manual entry)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for PROFILE_OPERATIONAL1 linked to the EID of the eUICC. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Add Profile operation | LPAd requests the Activation Code from the End User |
| 2 | S_EndUser→ LPAd | Provide #ACTIVATION_CODE_5 by manual entry | No error |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 4 | PROC_ES9+_INIT_AUTH |  |  |
| 5 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_EMPTY as <MATCHING_ID> |  |  |
| 6 | PROC_ES9+_GET_BPP (see NOTE 1) |  |  |
| 7 | LPAd → S_EndUser | Request for Confirmation, if not requested before. | End User Intent successfully verified, if not verified before. |
| 8 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 9 | S_EndUser → LPAd | Initiate List Profile operation | PROFILE_OPERATIONAL1 is displayed in Disabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. |  |  |  |

##### 5.4.1.2.13	TC_LPAd_AddEnableProfile_Manual_Entry

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Add and Enable a new Operational Profile by using Activation Code (manual entry)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1). |
| eUICC | There is no default SM-DP+ address configured. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate combined Add and Enable Profile operations | LPAd requests the Activation Code from the End User |
| 2 | S_EndUser→ LPAd | Provide #ACTIVATION_CODE_1 by manual entry | No error |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 4 | PROC_ES9+_INIT_AUTH |  |  |
| 5 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_1 as <MATCHING_ID> |  |  |
| 6 | PROC_ES9+_GET_BPP (see NOTE 1) |  |  |
| 7 | LPAd → S_EndUser | Request for Confirmation, if not requested before. | End User Intent successfully verified,if not verified before. |
| 8 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 9 | S_EndUser → LPAd | List Profile operation is initiated | PROFILE_OPERATIONAL1 is displayed in Enabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. |  |  |  |

Test Sequence #02 Nominal: Add and Enable a new Operational Profile by using Activation Code (manual entry) with Confirmation Code

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_3 (PROFILE_OPERATIONAL1) associated with #CONFIRMATION_CODE1. |
| eUICC | There is no default SM-DP+ address configured. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate combined Add and Enable Profile operations | LPAd requests the Activation Code from the S_End User |
| 2 | S_EndUser→ LPAd | Provide #ACTIVATION_CODE_3 by manual entry | No error |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 4 | PROC_ES9+_INIT_AUTH |  |  |
| 5 | PROC_ES9+_AUTH_CLIENT_CC with #MATCHING_ID_3 as <MATCHING_ID> |  |  |
| 6 | LPAd → S_EndUser | LPAd requests the Confirmation Code from the S_End User. | CONFIRMATION_CODE1 is provided by manual entry. |
| 7 | PROC_ES9+_GET_BPP_CC (see NOTE 1) |  |  |
| 8 | LPAd → S_EndUser | Request for Confirmation, if not requested before. (see NOTE 2) | End User Intent successfully verified if not verified before. (see NOTE 2) |
| 9 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 10 | S_EndUser → LPAd | Initiate List Profile operation | PROFILE_OPERATIONAL1 is displayed in Enabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. NOTE 2: The LPAd MAY skip this request for Confirmation. If so, it SHALL NOT be regarded as a failure. |  |  |  |

##### 5.4.1.2.14	TC_LPAd_AddEnableProfile

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Add and Enable a new Operational Profile by using Activation Code (QR code scanning)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1). |
| eUICC | There is no default SM-DP+ address configured. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate combined Add and Enable Profile operations | LPAd requests the Activation Code from the End User |
| 2 | S_EndUser→ LPAd | Provide #ACTIVATION_CODE_1 by scanning the QR code | No error |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 4 | PROC_ES9+_INIT_AUTH |  |  |
| 5 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_1 as <MATCHING_ID> |  |  |
| 6 | PROC_ES9+_GET_BPP (see NOTE 1) |  |  |
| 7 | LPAd → S_EndUser | Request for Confirmation, if not requested before. | End User Intent successfully verified,if not verified before. |
| 8 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 9 | S_EndUser→ LPAd | Initiate List Profile operation | PROFILE_OPERATIONAL1 is displayed in Enabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. |  |  |  |

Test Sequence #02 Nominal: Add and Enable a new Operational Profile by using Activation Code (manual)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1). |
| eUICC | There is no default SM-DP+ address configured. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate combined Add and Enable Profile operations | LPAd requests the Activation Code from the End User |
| 2 | S_EndUser→ LPAd | Provide #ACTIVATION_CODE_1 by manual entry | No error |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 4 | PROC_ES9+_INIT_AUTH |  |  |
| 5 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_1 as <MATCHING_ID> |  |  |
| 6 | PROC_ES9+_GET_BPP (see NOTE 1) |  |  |
| 7 | LPAd → S_EndUser | Request for Confirmation, if not requested before. | End User Intent successfully verified,if not verified before. |
| 8 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 9 | S_EndUser→ LPAd | Initiate List Profile operation | PROFILE_OPERATIONAL1 is displayed in Enabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. |  |  |  |

##### 5.4.1.2.15	TC_LPAd_AddEnableProfile_ConfirmationCode_smdpSigned2_QR

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Add and Enable a new Operational Profile by using Activation Code (QR code scanning) with confirmation code indicated only in smdpSigned2

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) which requires confirmation code. |
| eUICC | There is no default SM-DP+ address configured. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate combined Add and Enable Profile operations | Activation Code is requested from the End User by LPAd |
| 2 | S_EndUser→ LPAd | Provide #ACTIVATION_CODE_1 by scanning the QR code | No error |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 4 | PROC_ES9+_INIT_AUTH |  |  |
| 5 | PROC_ES9+_AUTH_CLIENT_CC with #MATCHING_ID_1 as <MATCHING_ID> |  |  |
| 6 | LPAd → S_EndUser | Request the Confirmation Code from the S_End User. | #CONFIRMATION_CODE1 is provided by manual entry. |
| 7 | PROC_ES9+_GET_BPP_CC (see NOTE 1) |  |  |
| 8 | LPAd → S_EndUser | Request for Confirmation, if not requested before. (see NOTE 2) | End User Intent successfully verified if not verified before. (see NOTE 2) |
| 9 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 10 | S_EndUser→ LPAd | Initiate List Profile operation | PROFILE_OPERATIONAL1 is displayed in Enabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. NOTE 2: The LPAd MAY skip this request for Confirmation. If so, it SHALL NOT be regarded as a failure. |  |  |  |

##### 5.4.1.2.16 TC_LPAd_AddEnableProfile_ConfirmationCode_smdpSigned2_Manual_Entry

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Add and Enable a new Operational Profile by using Activation Code (manual entry) with confirmation code indicated only in smdpSigned2

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) which requires confirmation code. |
| eUICC | There is no default SM-DP+ address configured. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate combined Add and Enable Profile operations | Activation Code is requested from the End User by LPAd |
| 2 | S_EndUser→ LPAd | Provide #ACTIVATION_CODE_1 by manual entry | No error |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9 |  |  |
| 4 | PROC_ES9+_INIT_AUTH |  |  |
| 5 | PROC_ES9+_AUTH_CLIENT_CC with #MATCHING_ID_1 as <MATCHING_ID> |  |  |
| 6 | LPAd → S_EndUser | Request the Confirmation Code from the S_End User. | #CONFIRMATION_CODE1 is provided by manual entry. |
| 7 | PROC_ES9+_GET_BPP_CC (see NOTE 1) |  |  |
| 8 | LPAd → S_EndUser | Request for Confirmation, if not requested before. (see NOTE 2) | End User Intent successfully verified, if not verified before. (see NOTE 2) |
| 9 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 10 | S_EndUser → LPAd | Initiate List Profile operation | PROFILE_OPERATIONAL1 is displayed in Enabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. NOTE 2: The LPAd MAY skip this request for Confirmation. If so, it SHALL NOT be regarded as a failure. |  |  |  |

##### 5.4.1.2.17	TC_LPAd_AddEnableProfile_default_SM-DP+_address

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Add and Enable a new Operational Profile by using the default SM-DP+ Address

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for PROFILE_OPERATIONAL1 linked to the EID of the eUICC. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate combined Add and Enable Profile operations See NOTE1 | No error |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH |  |  |
| 4 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_EMPTY as <MATCHING_ID> or missing MatchingID data object |  |  |
| 5 | PROC_ES9+_GET_BPP (see NOTE 2) |  |  |
| 6 | LPAd → S_EndUser | Request for Confirmation, if not requested before. | End User Intent successfully verified, if not verified before. |
| 7 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 8 | S_EndUser → LPAd | Initiate List Profile operation | PROFILE_OPERATIONAL1 is displayed in Enable state |
| NOTE 1: The Profile download by default SM-DP+ address MAY be implemented in different ways (e.g. some Device MAY implement a separate LUI menu for this function, some Device MAY request first the activation code, etc.). In order to enforce that the default SM-DP+ address is used the user SHALL not enter the Activation Code in case it is requested. NOTE 2: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. |  |  |  |

##### 5.4.1.2.18	TC_LPAd_AddEnableProfile_with_ConfirmationCode

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Add and Enable a new Operational Profile by using Activation Code (QR code scanning) with confirmation code

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_3 (PROFILE_OPERATIONAL1). |
| eUICC | There is no default SM-DP+ address configured. |

| Step | Direction | Sequence / Description | Expected result. |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate combined Add and Enable Profile operations | Activation Code is requested from the End User by LPAd |
| 2 | S_EndUser→ LPAd | Provide#ACTIVATION_CODE_3 by scanning the QR code |  |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 4 | PROC_ES9+_INIT_AUTH |  |  |
| 5 | PROC_ES9+_AUTH_CLIENT_CC with #MATCHING_ID_3 as <MATCHING_ID> |  |  |
| 6 | LPAd → S_EndUser | Request the Confirmation Code from the S_End User. | #CONFIRMATION_CODE1 is provided by manual entry. |
| 7 | PROC_ES9+_GET_BPP_CC (see NOTE 1) |  |  |
| 8 | LPAd → S_EndUser | Request for Confirmation, if not requested before. (see NOTE 2) | End User Intent successfully verified,if not verified before. (see NOTE 2) |
| 9 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 10 | S_EndUser→ LPAd | Initiate List Profile operation | PROFILE_OPERATIONAL1 is displayed in Enabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. NOTE 2: The LPAd MAY skip this request for Confirmation. If so, it SHALL NOT be regarded as a failure. |  |  |  |

Test Sequence #02 Nominal: Add and Enable a new Operational Profile by using Activation Code (manual) with confirmation code

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_3 (PROFILE_OPERATIONAL1). |
| eUICC | There is no default SM-DP+ address configured. |

| Step | Direction | Sequence / Description | Expected result. |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate combined Add and Enable Profile operations | Activation Code is requested from the End User by LPAd |
| 2 | S_EndUser→ LPAd | Provide#ACTIVATION_CODE_3 by manual entry |  |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 4 | PROC_ES9+_INIT_AUTH |  |  |
| 5 | PROC_ES9+_AUTH_CLIENT_CC with #MATCHING_ID_3 as <MATCHING_ID> |  |  |
| 6 | LPAd → S_EndUser | Request the Confirmation Code from the S_End User. | #CONFIRMATION_CODE1 is provided by manual entry. |
| 7 | PROC_ES9+_GET_BPP_CC (see NOTE 1) |  |  |
| 8 | LPAd → S_EndUser | For LPAd supporting SGP.22 v2.2.2 or earlier: Request for Confirmation, if not requested before. (see NOTE 2) | For LPAd supporting SGP.22 v2.2.2 or earlier: End User Intent successfully verified,if not verified before. (see NOTE 2) |
| 9 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 10 | S_EndUser→ LPAd | Initiate List Profile operation | PROFILE_OPERATIONAL1 is displayed in Enabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. NOTE 2: The LPAd MAY skip this request for Confirmation. If so, it SHALL NOT be regarded as a failure. |  |  |  |

##### 5.4.1.2.19	TC_LPAd_AddEnableProfile_PPRs

Test Sequence #01 Nominal: End User Confirmation after PPR1 consent requested

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The Test eUICC’s RAT is configured as follows: PPR1 is allowed and End User Consent is required for #MCC_MNC4 with gid1 and gid2 absent. |
| LPAd | Add and Enable Profile operation is initiated by using #ACTIVATION_CODE_4. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_4 (associated with PROFILE_OPERATIONAL4). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT Extract <S_TRANSACTION_ID> |  |  |
| IC4 | PROC_ES9+_GET_BPP with #METADATA_OP_PROF4 used in #GET_BPP_OK |  |  |
| 1 | LPAd → S_EndUser | Request for Confirmation if not requested before. | The LPA provides means for the End User Confirmation/Rejection of the Profile Download either at this point or at a previous point of the procedure Either Strong Confirmation is asked by showing relevant information concerning the PPR(s); or Simple Confirmation is asked on the Profile download. (See NOTE) |
| 2 | S_EndUser → LPAd | End User Confirmation is performed within the period as defined in #IUT_EU_CONFIRMATION_TIMEOUT |  |
| 3 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 4 | S_EndUser → LPAd | List Profile operation is initiated | PROFILE_OPERATIONAL4 is displayed in Enabled state |
| NOTE: The request for this End User consent/Confirmation for the installation of Profile Policy Rules and Profile download MAY be combined into a single prompt. |  |  |  |

Test Sequence #02 Nominal: End User Confirmation after PPR2 consent requested

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The Test eUICC’s RAT is configured as follows: PPR2 is allowed and End User Consent is required for #MCC_MNC2 with gid1 and gid2 absent. |
| LPAd | Add and Enable Profile operation is initiated by using #ACTIVATION_CODE_3_NO_CC. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_3 (associated with PROFILE_OPERATIONAL3). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT Extract <S_TRANSACTION_ID> |  |  |
| IC4 | PROC_ES9+_GET_BPP with #METADATA_OP_PROF3 used in #GET_BPP_OK |  |  |
| 1 | LPAd → S_EndUser | Request for Confirmation if not requested before. | The LPA provides means for the End User Confirmation/Rejection of the Profile Download either at this point or at a previous point of the procedure Either Strong Confirmation is asked by showing relevant information concerning the PPR(s); or Simple Confirmation is asked on the Profile download. (See NOTE) |
| 2 | S_EndUser → LPAd | End User Confirmation is performed within the period as defined in #IUT_EU_CONFIRMATION_TIMEOUT |  |
| 3 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 4 | S_EndUser → LPAd | List Profile operation is initiated | PROFILE_OPERATIONAL3 is displayed in Enabled state |
| NOTE: The request for this End User consent/Confirmation for the installation of Profile Policy Rules and Profile download MAY be combined into a single prompt. |  |  |  |

Test Sequence #03 Nominal: Profile with PPR1 already present

| Initial Conditions |
| --- |
| Entity | Description of the initial condition |
| eUICC | The Test eUICC’s RAT is configured as follows: PPR1 is allowed for #MCC_MNC4 with gid1 and gid2 absent (with End User Consent either required or not required). |
| eUICC | The PROFILE_OPERATIONAL4 with PPR1 is installed and enabled on the eUICC. |
| LPAd | Add and Enable Profile operation is initiated by using #ACTIVATION_CODE_1. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (associated with PROFILE_OPERATIONAL1). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| IC2 | PROC_ES9+_INIT_AUTH |  |  |
| IC3 | PROC_ES9+_AUTH_CLIENT Extract <S_TRANSACTION_ID> |  |  |
| IC4 | PROC_ES9+_GET_BPP |  |  |
| 1 | LPAd → S_EndUser | Request for Confirmation if not requested before. | The LPA provides means for the End User Confirmation/Rejection of the Profile Download. End User advised about a Profile with PPR1 already present and the End User consent is requested if not requested before. |
| 2 | S_EndUser → LPAd | End User Confirmation is performed within the period as defined in #IUT_EU_CONFIRMATION_TIMEOUT |  |
| 3 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 4 | S_EndUser → LPAd | List Profile operation is initiated | PROFILE_OPERATIONAL1 is displayed in Disabled state |

##### 5.4.1.2.20	TC_LPAd_AddEnableProfile_Empty_MatchingID

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Add and Enable a new Operational Profile by using empty MatchingID (QR code entry)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for PROFILE_OPERATIONAL1 linked to the EID of the eUICC. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate combined Add and Enable Profile operations | LPAd requests the Activation Code from the End User |
| 2 | S_EndUser→ LPAd | Provide #ACTIVATION_CODE_5 by scanning the QR code | No error |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 4 | PROC_ES9+_INIT_AUTH |  |  |
| 5 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_EMPTY as <MATCHING_ID> |  |  |
| 6 | PROC_ES9+_GET_BPP (see NOTE 1) |  |  |
| 7 | LPAd → S_EndUser | Request for Confirmation, if not requested before. | End User Intent successfully verified, if not verified before. |
| 8 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 9 | S_EndUser → LPAd | Initiate List Profile operation | PROFILE_OPERATIONAL1 is displayed in Enabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. |  |  |  |

Test Sequence #02 Nominal: Add and Enable a new Operational Profile by using empty MatchingID (manual entry)

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for PROFILE_OPERATIONAL1 linked to the EID of the eUICC. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate combined Add and Enable Profile operations | LPAd requests the Activation Code from the End User |
| 2 | S_EndUser→ LPAd | Provide #ACTIVATION_CODE_5 by manual entry | No error |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 4 | PROC_ES9+_INIT_AUTH |  |  |
| 5 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_EMPTY as <MATCHING_ID> |  |  |
| 6 | PROC_ES9+_GET_BPP (see NOTE 1) |  |  |
| 7 | LPAd → S_EndUser | Request for Confirmation, if not requested before. | End User Intent successfully verified, if not verified before. |
| 8 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 9 | S_EndUser → LPAd | Initiate List Profile operation | PROFILE_OPERATIONAL1 is displayed in Enabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. |  |  |  |

### 5.4.2	Local Profile Management – ListProfiles

#### 5.4.2.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

Section 3.2, 3.2.4

Section 4.4

#### 5.4.2.2	Test Cases

##### 5.4.2.2.1	TC_LPAd_ListProfiles

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL2 is installed on the eUICC. |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: List the Profiles and their current state

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is Enabled. |
| eUICC | The PROFILE_OPERATIONAL2 is Disabled. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser → LPAd | Request the list of Profiles | Display PROFILE_OPERATIONAL1 with Enabled state and the PROFILE_OPERATIONAL2 with Disabled state in human readable format. |

### 5.4.3	Local Profile Management - SetNickname

#### 5.4.3.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

Section 3.2, 3.2.6

#### 5.4.3.2	Test Cases

##### 5.4.3.2.1	TC_LPAd_SetNickname

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Add a Nickname on a Disabled Operational Profile

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is Disabled. |
| eUICC | The Nickname of the PROFILE_OPERATIONAL1 is not defined . |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Select PROFILE_OPERATIONAL1. Indicates the intention to change the Profile Nickname of PROFILE_OPERATIONAL1. | LPA offers to the End User a way to enter the Nickname. |
| 2 | S_EndUser→ LPAd | Set the Profile Nickname of the PROFILE_OPERATIONAL1 to #NICKNAME2 | LPAd sets the Profile Nickname (No Error) |
| 3 | Exit the UI menu |  |  |
| 4 | S_EndUser→ LPAd | Perform an LUI dependent action to display the NickName of PROFILE_OPERATIONAL1. | Profile Nickname of PROFILE_OPERATIONAL1 equals to #NICKNAME2 |
| 5 | Power off then power on the Device |  |  |
| 6 | S_EndUser→ LPAd | Perform an LUI dependent action to display the NickName of PROFILE_OPERATIONAL1. | Profile Nickname of PROFILE_OPERATIONAL1 equals to #NICKNAME2 |

Test Sequence #02 Nominal: Add a Nickname on an Enabled Operational Profile

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL2 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL2 is Enabled. |
| eUICC | The Nickname of the PROFILE_OPERATIONAL2 is not defined. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Select PROFILE_OPERATIONAL2. Indicates the intention to change the Profile Nickname of PROFILE_OPERATIONAL2 | LPA offers to the End User a way to enter the nickname. |
| 2 | S_EndUser→ LPAd | Set the Profile Nickname of the PROFILE_OPERATIONAL2 to #NICKNAME3 | LPAd sets the Profile Nickname (No Error) |
| 3 | Exit the UI menu |  |  |
| 4 | S_EndUser→ LPAd | Perform an LUI dependent action to display the NickName ofPROFILE_OPERATIONAL2 | Profile Nickname of PROFILE_OPERATIONAL2 equals to #NICKNAME3 |
| 5 | Power off then power on the Device |  |  |
| 6 | S_EndUser→ LPAd | Perform an LUI dependent action to display the NickName of PROFILE_OPERATIONAL2. | Profile Nickname of PROFILE_OPERATIONAL2 equals to #NICKNAME3 |

##### 5.4.3.2.2	TC_LPAd_EditNickname

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Edit the Nickname on a Disabled Operational Profile

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is Disabled. |
| eUICC | The Nickname of the PROFILE_OPERATIONAL1 is equal to #NICKNAME1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Select PROFILE_OPERATIONAL1 Indicates the intention to change the Profile Nickname of PROFILE_OPERATIONAL1 | Profile Nickname equals to #NICKNAME1 LPA offers to the End User a way to enter a new Nickname. |
| 2 | S_EndUser→ LPAd | Set the Profile Nickname of the PROFILE_OPERATIONAL1 to #NICKNAME2 | LPAd sets the Profile Nickname (No Error) |
| 3 | Exit the UI menu |  |  |
| 4 | S_EndUser→ LPAd | Perform an LUI dependent action to display the NickName ofPROFILE_OPERATIONAL1 | Profile Nickname equals to #NICKNAME2 |
| 5 | Power off then power on the Device |  |  |
| 6 | S_EndUser→ LPAd | Perform an LUI dependent action to display the NickName ofPROFILE_OPERATIONAL1 | Profile Nickname equals to #NICKNAME2 |

Test Sequence #02 Nominal: Edit the Nickname on an Enabled Operational Profile

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL2 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL2 is Enabled. |
| eUICC | The Nickname of the PROFILE_OPERATIONAL2 is equal to #NICKNAME3. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Select PROFILE_OPERATIONAL2 Indicates the intention to change the Profile Nickname of PROFILE_OPERATIONAL2 | Profile Nickname equals to #NICKNAME3 LPA offers to the End User a way to enter a new Nickname. |
| 2 | S_EndUser→ LPAd | Set the Profile Nickname of the PROFILE_OPERATIONAL2 to #NICKNAME4 | LPAd sets the Profile Nickname (No Error) |
| 3 | Exit the UI menu |  |  |
| 4 | S_EndUser→ LPAd | Perform an LUI dependent action to display the NickName of PROFILE_OPERATIONAL2 | Profile Nickname equals to #NICKNAME4 |
| 5 | Power off then power on the Device |  |  |
| 6 | S_EndUser→ LPAd | Perform an LUI dependent action to display the NickName ofPROFILE_OPERATIONAL2 | Profile Nickname equals to #NICKNAME4 |

### 5.4.4	Local Profile Management - Delete Profile

#### 5.4.4.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

Section 3.2, 3.2.3

Section 3.5

#### 5.4.4.2	Test Cases

##### 5.4.4.2.1	TC_LPAd_DeleteProfile_Disabled_without_PPR

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Deleting Disabled Profile, No PPRs

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL2 with #METADATA_OP_PROF2_TEST_DP_ADDRESS1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is in Disabled state. |
| eUICC | The PROFILE_OPERATIONAL2 is in Disabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Delete Profile procedure is initiated for PROFILE_OPERATIONAL1 | Strong Confirmation is requested by the LPAd and confirmed by the End User. |
| 2 | LPAd → S_SM-DP+ | Delete Notification containing #ICCID_OP_PROF1 is sent by the LPAd | The delete Notification as defined below is received by the S_SM-DP+ within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT MTD_HANDLE_NOTIF(#PENDING_NOTIF_DEL1) Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG |
| 3 | S_EndUser→ LPAd | Request for List Profiles | Installed Operational Profiles with their current states are displayed in a human readable format. PROFILE_OPERATIONAL1 is not shown. |
| NOTE: The timeout in Step 2 SHALL start after the End User Intent verification. |  |  |  |

##### 5.4.4.2.2	TC_LPAd_DeleteProfile_Enabled_without_PPR

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Deleting Enabled Profile, No PPRs

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL5 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL2 with #METADATA_OP_PROF2_TEST_DP_ADDRESS1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL5 is in Enabled state. |
| eUICC | The PROFILE_OPERATIONAL2 is in Disabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Delete Profile procedure for PROFILE_OPERATIONAL5 | Strong Confirmation is requested by the LPAd and confirmed by the End User. |
| 2 | LPAd → S_SM-DP+ | Send Disable Notification containing #ICCID_OP_PROF5 | The disable Notification as defined below is received by the S_SM-DP+ within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT MTD_HANDLE_NOTIF(#PENDING_NOTIF_DIS5) Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG See NOTE |
| 3 | LPAd → S_SM-DP+ | Send Delete Notification containing #ICCID_OP_PROF5 | The delete Notification as defined below is received by the S_SM-DP+ within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT MTD_HANDLE_NOTIF(#PENDING_NOTIF_DEL5) Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG See NOTE |
| 4 | S_EndUser→ LPAd | Request for List Profiles | Installed Operational Profiles with their current states are displayed in a human readable format. PROFILE_OPERATIONAL5 is not shown. |
| 5 | S_EndUser→ Device | Power off then power on the Device | During Device boot up no PIN entry is requested from the End User. |
| NOTE: The timeout SHALL start after the End User Intent verification. |  |  |  |

##### 5.4.4.2.3	TC_LPAd_DeleteProfile_Error_with_PPR1

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Error: Deleting Enabled Profile, PPR1 set

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The Test eUICC’s RAT is configured as follows: PPR1 is allowed for #MCC_MNC4 with gid1 and gid2 absent (with End User Consent either required or not required). |
| eUICC | The PROFILE_OPERATIONAL4 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL4 is in Enabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Delete Profile procedure is initiated for PROFILE_OPERATIONAL4 | Strong Confirmation is requested by the LPAd and confirmed by the End User. See NOTE 1 and NOTE 2 |
| 2 | S_EndUser→ LPAd | Request for List Profiles | Installed Operational Profiles with their current states are displayed in a human readable format. PROFILE_OPERATIONAL4 is shown in Enabled state. |
| NOTE 1: The LPAd MAY check the policy rules of the Profiles and give a warning to the End User. The procedure can be continued after the warning and the End User shall continue the procedure. NOTE 2: The LPAd MAY display an error indicating that the deletion of the Profile is failed. |  |  |  |

##### 5.4.4.2.4	TC_LPAd_DeleteProfile_Error_Disabled_with_PPR2

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Error: Deleting Disabled Profile, PPR2 set

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The Test eUICC’s RAT is configured as follows: PPR2 is allowed for #MCC_MNC2 with gid1 and gid2 absent (with End User Consent either required or not required). |
| eUICC | The PROFILE_OPERATIONAL7 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL7 is in Disabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Delete Profile procedure is initiated for PROFILE_OPERATIONAL7 | Strong Confirmation is requested by the LPAd and confirmed by the End User. See NOTE 1 and NOTE 2 |
| 2 | S_EndUser→ LPAd | Request for List Profiles | Installed Operational Profiles with their current states are displayed in a human readable format. PROFILE_OPERATIONAL7 is shown in Disabled state. |
| NOTE 1: The LPAd MAY check the policy rules of the Profiles and give a warning to the End User. The procedure can be continued after the warning and the End User shall continue the procedure. NOTE 2: The LPAd MAY display an error indicating that the deletion of the Profile is failed. |  |  |  |

##### 5.4.4.2.5	TC_LPAd_DeleteProfile_Error_Enabled_with_PPR2

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Error: Deleting Enabled Profile, PPR2 set

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The Test eUICC’s RAT is configured as follows: PPR2 is allowed for #MCC_MNC2 with gid1 and gid2 absent (with End User Consent either required or not required). |
| eUICC | The PROFILE_OPERATIONAL8 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL8 is in Enabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Delete Profile procedure for PROFILE_OPERATIONAL8 | Strong Confirmation is requested by the LPAd and confirmed by the End User. See NOTE 2 and NOTE 3 |
| 2 | LPAd → S_SM-DP+ | Send Disable Notification containing #ICCID_OP_PROF8 | The disable Notification as defined below is received by the S_SM-DP+ within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT MTD_HANDLE_NOTIF (#PENDING_NOTIF_DIS8) Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG See NOTE 1 |
| 3 | S_EndUser→ LPAd | Request for List Profiles | Installed Operational Profiles with their current states are displayed in a human readable format. PROFILE_OPERATIONAL8 is shown in Disabled state. |
| 4 | S_EndUser→ Device | Power off then power on the Device | During Device boot up no PIN entry is requested from the End User. |
| NOTE 1: The timeout SHALL start after the End User Intent verification. NOTE 2: The LPAd MAY check the policy rules of the Profiles and give a warning to the End User. The procedure can be continued after the warning and the End User shall continue the procedure. NOTE 3: The LPAd MAY display an error indicating that the deletion of the Profile is failed. |  |  |  |

##### 5.4.4.2.6	TC_LPAd_DeleteProfile_Security_Errors

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Error: Stop Delete Profile Operation if No Confirmation Provided

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is in Disabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Delete Profile procedure is initiated for PROFILE_OPERATIONAL1. The End User SHALL not provide Confirmation. | The LPAd stops the Delete Profile procedure. |
| 2 | S_EndUser→ LPAd | Request for List Profiles | Installed Operational Profiles with their current states are displayed in a human readable format. PROFILE_OPERATIONAL1 is shown in Disabled state. |

##### 5.4.4.2.7	TC_LPAd_DeleteProfiles_MEP_Enabled_without_PPR

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Deleting one of two Enabled Profiles, No PPRs

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL2 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is in Enabled state. |
| eUICC | The PROFILE_OPERATIONAL2 is in Enabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Delete Profile procedure for PROFILE_OPERATIONAL1 | Strong Confirmation is requested by the LPAd and confirmed by the End User. |
| 2 | LPAd → S_SM-DP+ | Send Disable Notification containing #ICCID_OP_PROF1 | The disable Notification as defined below is received by the S_SM-DP+ within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT MTD_HANDLE_NOTIF(#PENDING_NOTIF_DIS1) Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG See NOTE |
| 3 | LPAd → S_SM-DP+ | Send Delete Notification containing #ICCID_OP_PROF1 | The delete Notification as defined below is received by the S_SM-DP+ within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT MTD_HANDLE_NOTIF(#PENDING_NOTIF_DEL1) Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG See NOTE |
| 4 | S_EndUser→ LPAd | Request for List Profiles | Installed Operational Profiles with their current states are displayed in a human readable format. PROFILE_OPERATIONAL1 is not shown and PROFILE_OPERATIONAL2 is shown in Enabled state. |
| NOTE: The timeout (steps 2 and 3) SHALL start after the End User Intent verification. |  |  |  |

### 5.4.5	Local Profile Management - Enable Profile

#### 5.4.5.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

Section 3.2, 3.2.1, 3.2.4

Section 3.5

#### 5.4.5.2	Test Cases

##### 5.4.5.2.1	TC_LPAd_EnableProfile

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| Device | The End User gets presented a list of installed (operational) Profiles with their current state. |

Test Sequence #01 Nominal: Enable a formerly disabled Profile

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL5 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL5 is in Disabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser → LPAd | Initiate the Enable Profile operation for PROFILE_OPERATIONAL5 | PROFILE_OPERATIONAL5 is enabled |
| 2 | LPAd → S_SM-DP+ | Send the Enable Notification containing #ICCID_OP_PROF5 | The Enable Notification MTD_HANDLE_NOTIF(#PENDING_NOTIF_EN5) is received by the S_SM-DP+ within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_ECDSA S_SM-DP+ SHALL return #R_HTTP_204_OK |
| 3 | S_EndUser → Device | Enter #PO1_PIN1 to authenticate the user | Successful End User authentication for the selected application |
| 4 | S_EndUser → LPAd | Request List Profiles | PROFILE_OPERATIONAL5 is shown in Enabled state. |
| NOTE: The timeout SHALL start after the initiation of the Enable Profile operation. |  |  |  |

##### 5.4.5.2.2	TC_LPAd_EnableProfile_ImplicitDisable

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Enable a Profile with implicit disabling of the formerly enabled Profile

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL5 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL6 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL5 is in Enabled state. |
| eUICC | The PROFILE_OPERATIONAL6 is in Disabled state. |

| Step | Direction | Sequence / Description | Expected result |  |
| --- | --- | --- | --- | --- |
| 1 | S_EndUser → LPAd | Initiate the Enable Profile operation for PROFILE_OPERATIONAL6 |  |  |
| 2 | LPAd → S_SM-DP+(1) | Disable Notification containing #ICCID_OP_PROF5 is sent by the LPAd | The Disable Notification MTD_HANDLE_NOTIF(#PENDING_NOTIF_DIS5) is received by the S_SM-DP+ (configured with #TEST_DP_ADDRESS1) within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_ECDSA S_SM-DP+ SHALL return #R_HTTP_204_OK |  |
| 3 | LPAd → S_SM-DP+(2) | Send the Enable Notification containing #ICCID_OP_PROF6 | The Enable Notification MTD_HANDLE_NOTIF(#PENDING_NOTIF_EN6) is received by the S_SM-DP+ (configured with #TEST_DP_ADDRESS2) within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_ECDSA S_SM-DP+ SHALL return #R_HTTP_204_OK |  |
| 4 | S_EndUser → Device | Enter #PO2_PIN1 to authenticate the user | Successful End User authentication for the selected application |  |
| 5 | S_EndUser → LPAd | Request List Profiles | Installed Operational Profiles with their current states are displayed in a human readable format. PROFILE_OPERATIONAL6 is shown in Enabled state. |  |
| NOTE 1: The Notifications (steps 2 and 3) MAY be sent sequentially in either order or in parallel. NOTE 2: The timeout (steps 2 and 3) SHALL start after the initiation of the Enable Profile operation. |  |  |  |  |

##### 5.4.5.2.3	TC_LPAd_EnableProfile_Error_ProfileAlreadyEnabled

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Error: Enable an already enabled Profile

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is in Enabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser → LPAd | Initiate the Enable Profile operation for PROFILE_OPERATIONAL1 |  |
| 2 | LPAd → S_EndUser | Result of the Profile enabling | Enable Profile procedure terminates indicating an error |

##### 5.4.5.2.4	TC_LPAd_EnableProfile_Error_PPR1Set

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled |

Test Sequence #01 Error: Enabled Profile when a formerly enabled Profile has set PPR1

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The Test eUICC’s RAT is configured as follows: PPR1 is allowed for #MCC_MNC4 with gid1 and gid2 absent (with End User Consent either required or not required). |
| eUICC | The PROFILE_OPERATIONAL4 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL4 is in Enabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | S_EndUser → LPAd | Install PROFILE_OPERATIONAL1 and ensure that it is disabled (see NOTE 1) | PROFILE_OPERATIONAL1 is installed and disabled |
| 1 | S_EndUser → LPAd | Initiate the Enable Profile operation for PROFILE_OPERATIONAL1 | See NOTE 2 and NOTE 3 |
| 2 | S_EndUser LPAd | Request List Profiles | Installed Operational Profiles with their current states are displayed in a human readable format. PROFILE_OPERATIONAL4 is shown in Enabled state. |
| NOTE 1: If the device supports only O_D_ADD_ENABLE_COMBINED, any attempt to automatically enable the profile is expected to fail. NOTE 2: The LPAd MAY check the policy rules of the Profiles and give a warning to the End User. The procedure can be continued after the warning and the End User shall continue the procedure. NOTE 3: The LPAd MAY display an error indicating that the enabling of the Profile is failed. |  |  |  |

##### 5.4.5.2.5	Void

##### 5.4.5.2.6	TC_LPAd_EnableProfile_MEP

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| Device | The End User gets presented a list of installed (operational) Profiles with their current state. |

Test Sequence #01 Nominal: Enable two Disabled Profiles

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL2 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is in Disabled state. |
| eUICC | The PROFILE_OPERATIONAL2 is in Disabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser → LPAd | Initiate the Enable Profile operation for PROFILE_OPERATIONAL1 | PROFILE_OPERATIONAL1 is enabled |
| 2 | S_EndUser → LPAd | Initiate the Enable Profile operation for PROFILE_OPERATIONAL2 | PROFILE_OPERATIONAL2 is enabled |
| 3 | LPAd → S_SM-DP+ | Send the Enable Notification containing #ICCID_OP_PROF1 | The Enable Notification MTD_HANDLE_NOTIF(#PENDING_NOTIF_EN1) is received by the S_SM-DP+ within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG S_SM-DP+ SHALL return #R_HTTP_204_OK |
| 4 | LPAd → S_SM-DP+ | Send the Enable Notification containing #ICCID_OP_PROF2 | The Enable Notification MTD_HANDLE_NOTIF(#PENDING_NOTIF_EN2) is received by the S_SM-DP+ within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG S_SM-DP+ SHALL return #R_HTTP_204_OK |
| 5 | S_EndUser → LPAd | Request List Profiles | PROFILE_OPERATIONAL1 and PROFILE_OPERATIONAL2 are shown in Enabled state. |
| NOTE 1: The Notifications (steps 3 and 4) MAY be sent sequentially in either order or in parallel. NOTE 2: The timeout (steps 3 and 4) SHALL start after the initiation of the Enable Profile operation. |  |  |  |  |

### 5.4.6	Local Profile Management- Disable Profile

#### 5.4.6.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

Section 3.2, 3.2.2, 3.2.4

Section 3.5

#### 5.4.6.2	Test Cases

##### 5.4.6.2.1	TC_LPAd_DisableProfile

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Disable an Enabled Profile

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is in Enabled state. |
| eUICC | There is no default SM-DP+ address configured. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser → LPAd | Initiate the Disable Profile operation for PROFILE_OPERATIONAL1 | PROFILE_OPERATIONAL1 is disabled |
| 2 | LPAd → S_SM-DP+ | Send the Disable Notification containing #ICCID_OP_PROF1 | The Disable Notification MTD_HANDLE_NOTIF(#PENDING_NOTIF_DIS1) is received by the S_SM-DP+ within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_ECDSA S_SM-DP+ SHALL return #R_HTTP_204_OK |
| 3 | S_EndUser → LPAd | Request List Profiles | Installed Operational Profile(s) with their current states are displayed in a human readable format. PROFILE_OPERATIONAL1 is shown in Disabled state. |
| NOTE: The timeout SHALL start after the initiation of the Disable Profile operation. |  |  |  |

##### 5.4.6.2.2	TC_LPAd_DisableProfile_Error_ProfileAlreadyDisabled

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Error: Disable an already disabled Profile

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is in Disabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate the Disable Profile operation for PROFILE_OPERATIONAL1 |  |
| 2 | LPAd → S_EndUser | Result of the Profile disabling | The Disable Profile procedure terminates indicating a failure |

##### 5.4.6.2.3	TC_LPAd_DisableProfile_Error_PPR1Set

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Error: Disable an Enabled Profile with PPR1 set

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The Test eUICC’s RAT is configured as follows: PPR1 is allowed for #MCC_MNC4 with gid1 and gid2 absent (with End User Consent either required or not required). |
| eUICC | The PROFILE_OPERATIONAL4 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL4 is in Enabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser → LPAd | Initiate the Disable Profile operation for PROFILE_OPERATIONAL4 | See NOTE 1 and NOTE 2 |
| 2 | S_EndUser → LPAd | Request List Profiles | Installed Operational Profiles with their current states are displayed in a human readable format PROFILE_OPERATIONAL4 is shown in Enabled state |
| NOTE 1: The LPAd MAY check the policy rules of the Profiles and give a warning to the End User. The procedure can be continued after the warning and the End User shall continue the procedure. NOTE 2: The LPAd MAY display an error indicating that the disabling of the Profile is failed. |  |  |  |

##### 5.4.6.2.4	VOID

##### 5.4.6.2.5	TC_LPAd_DisableProfile_MEP

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Disable two Enabled Profiles

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL2 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is in Enabled state. |
| eUICC | The PROFILE_OPERATIONAL2 is in Enabled state. |
| eUICC | There is no default SM-DP+ address configured. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser → LPAd | Initiate the Disable Profile operation for PROFILE_OPERATIONAL1 | PROFILE_OPERATIONAL1 is disabled |
| 2 | LPAd → S_SM-DP+ | Send the Disable Notification containing #ICCID_OP_PROF1 | The Disable Notification MTD_HANDLE_NOTIF(#PENDING_NOTIF_DIS1) is received by the S_SM-DP+ within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG S_SM-DP+ SHALL return #R_HTTP_204_OK |
| 3 | S_EndUser → LPAd | Request List Profiles | Installed Operational Profile(s) with their current states are displayed in a human readable format. PROFILE_OPERATIONAL1 is shown in Disabled state and PROFILE_OPERATIONAL2 is shown in Enabled state. |
| 4 | S_EndUser → LPAd | Initiate the Disable Profile operation for PROFILE_OPERATIONAL2 | PROFILE_OPERATIONAL2 is disabled |
| 5 | LPAd → S_SM-DP+ | Send the Disable Notification containing #ICCID_OP_PROF2 | The Disable Notification MTD_HANDLE_NOTIF(#PENDING_NOTIF_DIS2) is received by the S_SM-DP+ within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG S_SM-DP+ SHALL return #R_HTTP_204_OK |
| 6 | S_EndUser → LPAd | Request List Profiles | Installed Operational Profile(s) with their current states are displayed in a human readable format. PROFILE_OPERATIONAL1 and PROFILE_OPERATIONAL2 are shown in Disabled state. |
| NOTE 1: The Notifications (steps 2 and 5) MAY be sent sequentially in either order or in parallel. NOTE 2: The timeout (steps 2 and 5) SHALL start after the initiation of the Enable Profile operation. |  |  |  |

### 5.4.7	Local eUICC Management - Retrieve EID Process

#### 5.4.7.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

Section 3.3.1

#### 5.4.7.2	Test Cases

##### 5.4.7.2.1	TC_LPAd_RetrieveEID

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Retrieve EID

The purpose of this test is to check if the Device is capable to display the stored EID in as QR code or in text string format.

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser → LPAd | Request to display EID. (See NOTE) | EID is displayed. |
| 2 | LPAd → S_EndUser | Presentation of the EID | The LPA presents the #EID1 to the End User as a text string and/or as a QR code. If the EID is represented as text string, the text SHALL be identical to #EID1 If the #EID1 is shown as a QR code it SHALL be either #EID1_QR_CODE1 or #EID1_QR_CODE2 with or without blank spaces. |
| NOTE: LPAd may display the EID by default. |  |  |  |

### 5.4.8	Local eUICC Management - eUICC Memory Reset Process

#### 5.4.8.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

Section 3.2.4

Section 3.3.2, 3.3.4

Section 3.5

#### 5.4.8.2	Test Cases

##### 5.4.8.2.1	TC_LPAd_eUICCMemoryReset

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | No proactive session is ongoing. NOTE: These test cases MAY fail due to the fact that a proactive is ongoing but it is impossible to determine that this is the case. In this instance it is recommended to repeat the test. |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: eUICC Memory Reset, Operational Profile installed, no Operational Profile enabled

The purpose of this test is to check the basic functions of the eUICC Memory Reset. An installed but not enabled Operational Profile SHALL be deleted.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is in Disabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser → LPAd | Initiate the eUICC Memory Reset for operational profiles | Strong Confirmation is requested by the LPAd and confirmed by the End User. |
| 2 | LPAd → S_SM-DP+ | Delete Notification containing #ICCID_OP_PROF1 is sent by the LPAd | The Delete Notification MTD_HANDLE_NOTIF(#PENDING_NOTIF_DEL1) is received by the S_SM-DP+ within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG The S_SM-DP+ SHALL return #R_HTTP_204_OK |
| 3 | S_EndUser → LPAd | Request List Profiles | Installed Operational Profiles with their current states are displayed in a human readable format No Operational Profile is available |
| NOTE: The timeout (step 2) SHALL start after the End User Intent verification. |  |  |  |

Test Sequence #02 Nominal: eUICC Memory Reset, Operational Profile with PPR2 installed, no Operational Profile enabled

The purpose of this test is to check if an initiated eUICC Memory Reset deletes an installed but not enabled Operational Profile with PPR2 ('Deletion of this Profile is not allowed').

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The Test eUICC’s RAT is configured as follows: PPR2 is allowed for #MCC_MNC2 with gid1 and gid2 absent (with End User Consent either required, or not required). |
| eUICC | The PROFILE_OPERATIONAL2 is installed on the eUICC with #METADATA_OP_PROF2_MEMRES1. |
| eUICC | The PROFILE_OPERATIONAL2 is in Disabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser → LPAd | Initiate the eUICC Memory Reset for operational profiles | Strong Confirmation is requested by the LPAd and confirmed by the End User. |
| 2 | LPAd → S_SM-DP+ | Delete Notification containing #ICCID_OP_PROF2 is sent by the LPAd | The Delete Notification MTD_HANDLE_NOTIF(#PENDING_NOTIF_DEL2) is received by the S_SM-DP+ within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG The S_SM-DP+ SHALL return #R_HTTP_204_OK |
| 3 | S_EndUser → LPAd | Request List Profiles | Installed Operational Profiles with their current states are displayed in a human readable format No Operational Profile is available |
| NOTE: The timeout (step 2) SHALL start after the End User Intent verification. |  |  |  |

Test Sequence #03 Nominal: eUICC Memory Reset, Operational Profile with PPR2 installed and enabled

The purpose of this test is to check if an initiated eUICC Memory Reset deletes an installed and enabled Operational Profile with PPR2 (‘'Deletion of this Profile is not allowed’').

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The Test eUICC’s RAT is configured as follows: PPR2 is allowed for #MCC_MNC2 with gid1 and gid2 absent (with End User Consent either required, or not required). |
| eUICC | The PROFILE_OPERATIONAL2 is installed on the eUICC with #METADATA_OP_PROF2_MEMRES1. |
| eUICC | The PROFILE_OPERATIONAL2 is in Enabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser → LPAd | Initiate the eUICC Memory Reset for operational profiles | Strong Confirmation is requested by the LPAd and confirmed by the End User. |
| 2 | LPAd → S_SM-DP+ | Delete Notification containing #ICCID_OP_PROF2 is sent by the LPAd | The Delete Notification MTD_HANDLE_NOTIF(#PENDING_NOTIF_DEL2) is received by the S_SM-DP+ within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG The S_SM-DP+ SHALL return #R_HTTP_204_OK See NOTE 2 |
| 3 | S_EndUser → LPAd | Request List Profiles | Installed Operational Profiles with their current states are displayed in a human readable format No Operational Profile is available RQ32_053 RQ33_012 |
| NOTE 1: The timeout (step 2) SHALL start after the End User Intent verification. NOTE 2: A Disable Notification for PROFILE_OPERATIONAL2 MAY be sent before the Delete Notification. This notification SHALL NOT be checked. |  |  |  |

Test Sequence #04 Nominal: eUICC Memory Reset, Operational Profile with PPR1 installed and enabled

The purpose of this test is to check if an initiated eUICC Memory Reset deletes an installed and enabled Operational Profile with PPR1 (‘'Disabling of this Profile is not allowed’').

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The Test eUICC’s RAT is configured as follows: PPR1 is allowed for #MCC_MNC4 with gid1 and gid2 absent (with End User Consent either required, or not required). |
| eUICC | The PROFILE_OPERATIONAL4 is installed on the eUICC with #METADATA_OP_PROF4_MEMRES1. |
| eUICC | The PROFILE_OPERATIONAL4 is in Enabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser → LPAd | Initiate the eUICC Memory Reset for operational profiles | Strong Confirmation is requested by the LPAd and confirmed by the End User. |
| 2 | LPAd → S_SM-DP+ | Delete Notification containing #ICCID_OP_PROF4 is sent by the LPAd | The Delete Notification MTD_HANDLE_NOTIF(#PENDING_NOTIF_DEL4) is received by the S_SM-DP+ within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG The S_SM-DP+ SHALL return #R_HTTP_204_OK See NOTE 2 |
| 3 | S_EndUser → LPAd | Request List Profiles | Installed Operational Profiles with their current states are displayed in a human readable format No Operational Profile is available |
| NOTE 1: The timeout (step 2) SHALL start after the End User Intent verification. NOTE 2: A Disable Notification for PROFILE_OPERATIONAL4 MAY be sent before the Delete Notification. This notification SHALL NOT be checked. |  |  |  |

Test Sequence #05 Nominal: eUICC Memory Reset, multiple Operational Profiles are installed, an Operational Profile is enabled

The purpose of this test is to check if an initiated eUICC Memory Reset deletes all Operational Profiles installed and send the required Notifications to the appropriate SM-DP+.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is in Enabled state. |
| eUICC | The PROFILE_OPERATIONAL2 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL2 is in Disabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser → LPAd | Initiate the eUICC Memory Reset for operational profiles | Strong Confirmation is requested by the LPAd and confirmed by the End User. |
| 2 | LPAd → S_SM-DP+(1) | Delete Notifications containing #ICCID_OP_PROF1 is sent by the LPAd | The Delete Notification MTD_HANDLE_NOTIF(#PENDING_NOTIF_DEL1) is received by the S_SM-DP+ (configured with #TEST_DP_ADDRESS1) within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT The S_SM-DP+ SHALL return #R_HTTP_204_OK See NOTE 3 |
| 3 | LPAd → S_SM-DP+(2) | Delete Notification containing #ICCID_OP_PROF2 is sent by the LPAd | The Delete Notification MTD_HANDLE_NOTIF(#PENDING_NOTIF_DEL2) is received by the S_SM-DP+ (configured with #TEST_DP_ADDRESS2) within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG The S_SM-DP+ SHALL return #R_HTTP_204_OK |
| 4 | S_EndUser → LPAd | Request List Profiles | Installed Operational Profiles with their current states are displayed in a human readable format No Operational Profile is available |
| NOTE 1: The Delete Notifications (steps 2 and 3) MAY be sent sequentially in either order or in parallel. NOTE 2: The timeout (steps 2 and 3) SHALL start after the End User Intent verification. NOTE 3: A Disable Notification for PROFILE_OPERATIONAL1 MAY be sent before the Delete Notification. This notification SHALL NOT be checked. |  |  |  |

##### 5.4.8.2.2	TC_LPAd_eUICCMemoryResetWithPINVerification

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | No proactive session is ongoing. NOTE: these test cases may fail due to the fact that a proactive session is ongoing but it is impossible to determine that this is the case. In this instance it is recommended to repeat the test. |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: eUICC Memory Reset, installed and enabled Operational Profile with PIN verification

The purpose of this test is to check if an initiated eUICC Memory Reset deletes an installed and enabled Operational Profile  with PIN verification enabled.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL5 is installed on the eUICC with #METADATA_OP_PROF5. |
| eUICC | The PROFILE_OPERATIONAL5 is in Enabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser → LPAd | Initiate the eUICC Memory Reset for operational profiles | Strong Confirmation is requested by the LPAd and confirmed by the End User. |
| 2 | LPAd → S_SM-DP+ | Delete Notification containing #ICCID_OP_PROF5 is sent by the LPAd | The Delete Notification MTD_HANDLE_NOTIF(#PENDING_NOTIF_DEL5) is received by the S_SM-DP+ within the timeout (#IUT_LPAd_NOTIFICATION_TIMEOUT, + #IUT_LPAd_READY_AFTER_REBOOT_TIMEOUT) Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG The S_SM-DP+ SHALL return #R_HTTP_204_OK See NOTE 3 |
| 3 | Device | Power off then power on the Device If the Device does not automatically power off and power on, the S_EndUser SHALL power off and power on the Device. | During Device boot up no PIN entry is requested from the End User. |
| 4 | S_EndUser → LPAd | Request List Profiles | Installed Operational Profiles with their current states are displayed in a human readable format No Operational Profile is available |
| NOTE 1: The Delete Notification (step 2) can be sent at any step after having successfully initiated the eUICC Memory Reset. NOTE 2: The timeout (step 2) SHALL start after the End User Intent verification. NOTE 3: A Disable Notification for PROFILE_OPERATIONAL5 MAY be sent before the Delete Notification. This notification SHALL NOT be checked. |  |  |  |

Test Sequence #02 Nominal: VOID

##### 5.4.8.2.3	TC_LPAd_eUICCMemoryReset_MEP

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | No proactive session is ongoing. NOTE: These test cases MAY fail due to the fact that a proactive is ongoing but it is impossible to determine that this is the case. In this instance it is recommended to repeat the test. |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: eUICC Memory Reset, multiple Operational Profiles are installed, two Operational Profiles are enabled

The purpose of this test is to check if an initiated eUICC Memory Reset deletes all Operational Profiles installed and send the required Notifications to the appropriate SM-DP+.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is in Enabled state. |
| eUICC | The PROFILE_OPERATIONAL2 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL2 is in Enabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser → LPAd | Initiate the eUICC Memory Reset for operational profiles | Strong Confirmation is requested by the LPAd and confirmed by the End User. |
| 2 | LPAd → S_SM-DP+(1) | Delete Notifications containing #ICCID_OP_PROF1 is sent by the LPAd | The Delete Notification MTD_HANDLE_NOTIF(#PENDING_NOTIF_DEL1) is received by the S_SM-DP+ (configured with #TEST_DP_ADDRESS1) within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT The S_SM-DP+ SHALL return #R_HTTP_204_OK See NOTE 3 |
| 3 | LPAd → S_SM-DP+(2) | Delete Notification containing #ICCID_OP_PROF2 is sent by the LPAd | The Delete Notification MTD_HANDLE_NOTIF(#PENDING_NOTIF_DEL2) is received by the S_SM-DP+ (configured with #TEST_DP_ADDRESS2) within the timeout #IUT_LPAd_NOTIFICATION_TIMEOUT Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_SIG The S_SM-DP+ SHALL return #R_HTTP_204_OK |
| 4 | S_EndUser → LPAd | Request List Profiles | Installed Operational Profiles with their current states are displayed in a human readable format No Operational Profile is available |
| NOTE 1: The Delete Notifications (steps 2 and 3) MAY be sent sequentially in either order or in parallel. NOTE 2: The timeout (steps 2 and 3) SHALL start after the End User Intent verification. NOTE 3: Disable Notifications for PROFILE_OPERATIONAL1 and PROFILE_OPERATIONAL2 MAY be sent before the Delete Notification. These notifications SHALL NOT be checked. |  |  |  |

### 5.4.9	Local eUICC Management–- eUICC Test Memory Reset Process

This section is defined as FFS and not applicable for this version of test specification.

### 5.4.10	Local eUICC Management – Set/Edit Default SM-DP+ Address Process

#### 5.4.10.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

Section 3.3.4

#### 5.4.10.2	Test Cases

##### 5.4.10.2.1	TC_LPAd_Set/Edit Default SM-DP+ Address

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |

Test Sequence #01 Nominal: Set Default SM-DP+ Address where no Default Address has been set before

The purpose of this test is to set a default SM-DP+ address on a eUICC where no SM-DP+ default address is stored.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | No value is assigned to the Default SM-DP+ field. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser → LPAd | Initiate the function to retrieve the configured address | The LPAd retrieves the Default SM-DP+ Address and presents it to the EndUser The current Default SM-DP+ Address is empty respectively no Default SM-DP+ Address is shown |
| 2 | S_EndUser → LPAd | If required, initiate the function to enter #TEST_DP_ADDRESS1 as the new Default SM-DP+ address or enter directly #TEST_DP_ADDRESS1 as the new Default SM-DP+. | Successful End User Intent verified as defined in SGP.21 [3] for Simple Confirmation, if not verified before. (see NOTE 1) |
| 3 | S_EndUser → LPAd | Initiate the function to retrieve the configured address | The LPAd retrieves the Default SM-DP+ Address and presents it to the EndUser The current Default SM-DP+ Address #TEST_DP_ADDRESS1 is shown |
| NOTE 1: The LPAd MAY skip this request for Confirmation. If so, it SHALL NOT be regarded as a failure. |  |  |  |

Test Sequence #02 Nominal: Edit the Default SM-DP+ Address and store it on the eUICC

The purpose of this test is to edit an existing default SM-DP+ address on a eUICC and to ensure that the changes are stored.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The Default SM-DP+ field is set to #TEST_DEFAULT_DP_ADDRESS_1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser → LPAd | Initiate the function to retrieve the configured address | The LPAd retrieves the Default SM-DP+ Address and presents it to the EndUser The current Default SM-DP+ Address is #TEST_DEFAULT_DP_ADDRESS_1 |
| 2 | S_EndUser → LPAd | If required, initiate the function to enter #TEST_DP_ADDRESS1 as the new Default SM-DP+ address or enter directly #TEST_DP_ADDRESS1 as the new Default SM-DP+. | Successful End User Intent verified as defined in SGP.21 [3] for Simple Confirmation, if not verified before. (see NOTE 1) |
| 3 | S_EndUser → LPAd | Initiate the function to retrieve the configured address | The LPAd retrieves the Default SM-DP+ Address and presents it to the EndUser The current Default SM-DP+ Address #TEST_DP_ADDRESS1 is shown |
| NOTE 1: The LPAd MAY skip this request for Confirmation. If so, it SHALL NOT be regarded as a failure. |  |  |  |

Test Sequence #03 Nominal: Edit the Default SM-DP+ Address and store a Default Address with an empty value

The purpose of this test is to edit an existing Default SM-DP+ address on a eUICC and to ensure that the changes are stored even if the new Default Address value is empty.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The Default SM-DP+ field is set to #TEST_DEFAULT_DP_ADDRESS_1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser → LPAd | Initiate the function to retrieve the configured address | The LPAd retrieves the Default SM-DP+ Address and presents it to the EndUser The current Default SM-DP+ Address is #TEST_DEFAULT_DP_ADDRESS_1 |
| 2 | S_EndUser → LPAd | If required, initiate the function to enter “” (empty value) as the new Default SM-DP+ address or enter directly “” as the new Default SM-DP+. | Successful End User Intent verified as defined in SGP.21 [3] for Simple Confirmation, if not verified before. (see NOTE 1) |
| 3 | S_EndUser → LPAd | Initiate the function to retrieve the configured address | The LPAd retrieves the Default SM-DP+ Address and presents it to the EndUser The current Default SM-DP+ Address is empty respectively no Default SM-DP+ Address is shown |
| NOTE 1: The LPAd MAY skip this request for Confirmation. If so, it SHALL NOT be regarded as a failure. |  |  |  |

### 5.4.11	Device Power On – Profile Discovery

#### 5.4.11.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

Section 3.4.4

#### 5.4.11.2	Test Cases

##### 5.4.11.2.1	TC_LPAd_DevicePowerOnProfileDiscovery_SM-DP+_address

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| Device | The setting of the configuration parameter for Device Power-on Profile discovery is 'Enabled'. |
| Device | The Device is powered off. |

Test Sequence #01 Nominal: Power-on Profile discovery by using the default SM-DP+ Address

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for PROFILE_OPERATIONAL1 linked to the EID of the eUICC. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | Power on the Device |  |  |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | PROC_ES9+_INIT_AUTH |  |  |
| 3 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_EMPTY as <MATCHING_ID> or missing MatchingID data object |  |  |
| 4 | PROC_ES9+_GET_BPP (see NOTE 1) |  |  |
| 5 | LPAd → S_EndUser | Request for Confirmation, if not requested before. | End User Intent successfully verified, if not verified before. |
| 6 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 7 | LPAd → S_EndUser | Initiate List Profile operation | PROFILE_OPERATIONAL1 is displayed in Disabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. |  |  |  |

##### 5.4.11.2.2	TC_LPAd_DevicePowerOnProfileDiscovery_SM-DS

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled. |
| Device | The setting of the configuration parameter for Device Power-on Profile discovery is 'Enabled'. |
| Device | The Device is powered off. |

Test Sequence #01 Nominal: Power-on Profile discovery by using the SM-DS

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DS | S_SM-DP+ (#TEST_DP_ADDRESS1) performed Profile download Event Registration to the S_SM-DS (#TEST_ROOT_DS_ADDRESS) with #EVENT_ID_1. |
| S_SM-DP+ | There is a pending Profile download order for #EVENT_ID_1 (PROFILE_OPERATIONAL1). |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| eUICC | There is no default SM-DP+ address configured. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | Power-on the Device |  |  |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES11 |  |  |
| 2 | PROC_ES11_INIT_AUTH |  |  |
| 3 | PROC_ES11_AUTH_CLIENT with #MATCHING_ID_EMPTY as <MATCHING_ID> or missing MatchingID data object |  |  |
| 4 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 5 | PROC_ES9+_INIT_AUTH |  |  |
| 6 | PROC_ES9+_AUTH_CLIENT with #EVENT_ID_1 as <MATCHING_ID> |  |  |
| 7 | PROC_ES9+_GET_BPP (see NOTE 1) |  |  |
| 8 | LPAd → S_EndUser | Request for Confirmation, if not requested before. | End User Intent successfully verified , if not verified before. |
| 9 | PROC_ES9+_HANDLE_NOTIF |  |  |
| 10 | LPAd → S_EndUser | Initiate List Profile operation | PROFILE_OPERATIONAL1 is displayed in Disabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. |  |  |  |

### 5.4.12	RPM Command Execution - Enable Profile

#### 5.4.12.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

2.6.6.2, 2.10.1

3.0.1

3.2.7, 3.7.2

3.7.3

5.5.3, 5.6.3, 5.7.14a

5.7.16

#### 5.4.12.2	Test Cases

##### 5.4.12.2.1	TC_LPAd_RPM_Command_Execution_EnableProfile

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled |
| Device | RPM operation is enabled in the LPA by the End User |
| eUICC | There is no default SM-DP+ address configured The eUICC supports RPM |
| S_SM-DP+ | Variant A certificates are included in certificates chain for TLS procedures. |

Test Sequence #01 Nominal: RPM Enable a formerly disabled Profile

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Entity |  | Description of the initial condition |  |  |
| eUICC |  | PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_RPM_CONF_EN is installed and disabled. |  |  |
| S_SM-DP+ |  | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) |  |  |
| Step | Direction |  | Sequence / Description | Expected result |
| 1 | S_EndUser→ LPAd |  | Initiate Update Profile operation | LPAd retrievesPolling Address from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |  |
| 3 | PROC_ES9+_INIT_AUTH_V3 |  |  |  |
| 4 | PROC_ES9+_AUTH_CLIENT_RPM with #MATCHING_ID_1 as <MATCHING_ID> |  |  |  |
| 5 | LPAd → S_EndUser |  | Request for User Confirmation. | LPAd initiates Confirmation Request for Simple Confirmation |
| 6 | S_EndUser |  | Accepts the RPM command | PROFILE_OPERATIONAL1 is enabled |
| 7 | PROC_ES9+_HANDLE_NOTIF_RPM_OK |  |  |  |
| 8 | S_EndUser → LPAd |  | List Profile operation is initiated | PROFILE_OPERATIONAL1 is displayed in Enabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. |  |  |  |  |

Test Sequence #02 Nominal: RPM Command - RPM Enable a Profile with implicit disabling of the formerly enabled Profile

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_RPM_CONF_EN is installed and disabled. |
| eUICC | The PROFILE_OPERATIONAL2 with #METADATA_OP_PROF2_RPM_CONF_EN_OWNER_OID1 (without PPR1 present) is loaded and Enabled on the eUICC. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Update Profile operation | LPAd retrievesPolling Address from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH_AND_AUTH_CLIENT_REQ_V3 |  |  |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( MTD_AUTH_CLIENT_RPM_PKG_REQ_FOR_SINGLE_CMND( enable, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, <S_SM_DP+_SIGNATURE3>, NO_PARAM, NO_PARAM, NO_PARAM ) ) | No error |
| 5 | LPAd → S_EndUser | Request for User Confirmation. | LPAd initiates Confirmation Request for Simple Confirmation |
| 6 | S_EndUser | Accepts the RPM command | PROFILE_OPERATIONAL1 is enabled |
| 7 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( MTD_RESP_RPR_FOR_SINGLE_CMND( enableResult, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, 0, -- OK response #NOTIF_METADATA_PROF1_DP1_RPR, #S_SM_DP+_OID, NO_PARAM, NO_PARAM, NO_PARAM) ) ) |
| 8 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |
| 9 | S_EndUser → LPAd | List Profile operation is initiated | PROFILE_OPERATIONAL1 is in Enabled state PROFILE_OPERATIONAL2 is in Disabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. |  |  |  |

Test Sequence #03 Error: RPM Command Result Data Error -  Enable Profile, Disallowed Managing SM-DP+

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_CONF_EN_DP_OID2 is installed and disabled. |
| S_SM-DP+ | There is a pending RPM package order with OID 1 for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) The SM-DP+ that sent the RPM Command is not included in the Managing SM-DP+ List in the Profile Metadata (rpmConfiguration) of PROFILE_OPERATIONAL1. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Update Profile operation | LPAd retrieves Polling Address from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH_V3 |  |  |
| 4 | PROC_ES9+_AUTH_CLIENT_RPM with #MATCHING_ID_1 as <MATCHING_ID> and #RPM_PKG_EN is included in <S_SMDP_SIGNED3> |  |  |
| 5 | LPAd → S_EndUser | Request for User Confirmation. | LPAd initiates Confirmation Request for Simple Confirmation |
| 6 | S_EndUser | Accepts the RPM command | The LPAd MAY inform the End User of the status indicating the error disallowedManagingDp. |
| 7 | PROC_ES9+_HANDLE_NOTIF_RPR_ERROR( MTD_RESP_RPR_FOR_SINGLE_CMND( enableResult, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, 1, -- error response #NOTIF_METADATA_PROF1_DP1_RPR, #S_SM_DP+_OID, NO_PARAM, NO_PARAM, disallowedManagingDp ) ) |  |  |
| 8 | S_EndUser → LPAd | List Profile operation is initiated | PROFILE_OPERATIONAL1 is in Disabled state |

Test Sequence #04 Error: RPM Command Result Data Error -  Enable Profile, Disallowed by Policy

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 has been loaded on the eUICC with #METADATA_OP_PROF1_RPM_CONF_EN and is disabled. |
| eUICC | The PROFILE_OPERATIONAL2 with #METADATA_OP_PROF2_RPM_CONF_EN_OWNER_OID1_PPR1 has been loaded before the PROFILE_OPERATIONAL1 and is Enabled. |
| S_SM-DP+ | There is a pending RPM package order with OID 1 for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Update Profile operation | LPAd retrievesPolling Address from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH_V3 |  |  |
| 4 | PROC_ES9+_AUTH_CLIENT_RPM with #MATCHING_ID_1 as <MATCHING_ID> and #RPM_PKG_EN is included in <S_SMDP_SIGNED3> |  |  |
| 5 | LPAd → S_EndUser | Request for User Confirmation. | LPAd initiates Confirmation Request for Simple Confirmation |
| 6 | S_EndUser | Accepts the RPM command | The LPAd MAY inform the End User of the status indicating the error disallowedByPolicy. |
| 7 | PROC_ES9+_HANDLE_NOTIF_RPR_ERROR( MTD_RESP_RPR_FOR_SINGLE_CMND( enableResult, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, 1, -- error response #NOTIF_METADATA_PROF1_DP1_RPR, #S_SM_DP+_OID, NO_PARAM, NO_PARAM, disallowedByPolicy ) ) |  |  |
| 8 | S_EndUser → LPAd | List Profile operation is initiated | PROFILE_OPERATIONAL1 is in Disabled state |

### 5.4.13	RPM Command Execution - Disable Profile

#### 5.4.13.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

2.6.6.2, 2.10.1

3.0.1

3.2.7, 3.7.2

3.7.3

5.5.3, 5.6.3, 5.7.14a

5.7.17

#### 5.4.13.2	Test Cases

##### 5.4.13.2.1	TC_LPAd_RPM_Command_Execution_DisableProfile

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled |
| Device | RPM operation is enabled in the LPA by the End User |
| eUICC | There is no default SM-DP+ address configured The eUICC supports RPM |
| S_SM-DP+ | Variant A certificates are included in certificates chain for TLS procedures. |

Test Sequence #01 Nominal: RPM Command - Disable an Enabled Profile without PPR

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_RPM_CONF_ALL is installed and Enabled. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) for Disabling the Profile |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Update Profile operation | LPAd retrieves Polling Address from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH_AND_AUTH_CLIENT_REQ_V3 |  |  |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( MTD_AUTH_CLIENT_RPM_PKG_REQ_FOR_SINGLE_CMND( disable, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, <S_SM_DP+_SIGNATURE3>, NO_PARAM, NO_PARAM, NO_PARAM ) ) | No error |
| 5 | LPAd → S_EndUser | Request for User Confirmation. | LPAd initiates Confirmation Request for Simple Confirmation |
| 6 | S_EndUser | Accepts the RPM command | PROFILE_OPERATIONAL1 is disabled |
| 7 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( MTD_RESP_RPR_FOR_SINGLE_CMND( disableResult, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, 0, -- OK response #NOTIF_METADATA_PROF1_DP1_RPR, #S_SM_DP+_OID, NO_PARAM, NO_PARAM, NO_PARAM) ) ) |
| 8 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |
| 9 | S_EndUser → LPAd | List Profile operation is initiated | PROFILE_OPERATIONAL1 is in Disabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. |  |  |  |

Test Sequence #02 Error: RPM Command Result Data Error -  Disable Profile, ICCID Not Found

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_CONF_ALL is installed and Enabled. (PPR1 is not set in the Metadata) |
| eUICC | The Operational Profile identified by the ICCID #ICCID_OP_PROFX is not loaded. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) for Disabling the Profile. |
| S_SM-DP+ | ICCID in the RPM package is set to #ICCID_OP_PROFX. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Update Profile operation | LPAd retrieves Polling Address from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH_AND_AUTH_CLIENT_REQ_V3 |  |  |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( MTD_AUTH_CLIENT_RPM_PKG_REQ_FOR_SINGLE_CMND( disable, <S_TRANSACTION_ID>, #ICCID_OP_PROFX, <S_SM_DP+_SIGNATURE3>, NO_PARAM, NO_PARAM, NO_PARAM ) ) | No error |
| 5 | LPAd → S_EndUser | Request for User Confirmation. | LPAd initiates Confirmation Request for Simple Confirmation |
| 6 | S_EndUser | Accepts the RPM command | The LPAd MAY inform the End User of the status indicating the error iccidOrAidNotFound. |
| 7 | PROC_ES9+_HANDLE_NOTIF_RPR_ERROR( MTD_RESP_RPR_FOR_SINGLE_CMND( disableResult, <S_TRANSACTION_ID>, #ICCID_OP_PROFX, 1, -- error response #NOTIF_METADATA_PROF1_DP1_RPR, #S_SM_DP+_OID, NO_PARAM, NO_PARAM, iccidOrAidNotFound ) ) |  |  |
| 8 | S_EndUser → LPAd | List Profile operation is initiated | PROFILE_OPERATIONAL1 is in Enabled state |

Test Sequence #03 Error: RPM Command Result Data Error -  Disable Profile, Profile is not in Enabled state

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 has been loaded on the eUICC with #METADATA_OP_PROF1_RPM_CONF_ALL and it is in Disabled state. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) for Disabling the Profile. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Update Profile operation | LPAd retrieves Polling Address from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH_AND_AUTH_CLIENT_REQ_V3 |  |  |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( MTD_AUTH_CLIENT_RPM_PKG_REQ_FOR_SINGLE_CMND( disable, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, <S_SM_DP+_SIGNATURE3>, NO_PARAM, NO_PARAM, NO_PARAM ) ) | No error |
| 5 | LPAd → S_EndUser | Request for User Confirmation. | LPAd initiates Confirmation Request for Simple Confirmation |
| 6 | S_EndUser | Accepts the RPM command | The LPAd MAY inform the End User of the status indicating the error profileNotInEnabledState. |
| 7 | PROC_ES9+_HANDLE_NOTIF_RPR_ERROR( MTD_RESP_RPR_FOR_SINGLE_CMND( disableResult, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, 1, -- error response #NOTIF_METADATA_PROF1_DP1_RPR, #S_SM_DP+_OID, NO_PARAM, NO_PARAM, profileNotInEnabledState ) ) |  |  |
| 8 | S_EndUser → LPAd | List Profile operation is initiated | PROFILE_OPERATIONAL1 is in Enabled state |

### 5.4.14	RPM Command Execution - Delete Profile

#### 5.4.14.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

2.6.6.2, 2.10.1

3.0.1

3.2.7, 3.7.2

3.7.3

5.5.3, 5.6.3, 5.7.14a

5.7.18

#### 5.4.14.2	Test Cases

##### 5.4.14.2.1	TC_LPAd_RPM_Command_Execution_DeleteProfile

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled |
| Device | RPM operation is enabled in the LPA by the End User |
| eUICC | There is no default SM-DP+ address configured The eUICC supports RPM |
| S_SM-DP+ | Variant A certificates are included in certificates chain for TLS procedures. |

Test Sequence #01 Nominal: RPM Command - Delete a Disabled Profile without PPR

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_RPM_CONF_ALL is installed and Disabled. |
| eUICC | The PROFILE_OPERATIONAL2 with #METADATA_OP_PROF2_RPM_CONF_EN_OWNER_OID1 (without PPR1 present) is loaded and Enabled on the eUICC. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) for Deleting the Profile |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Update Profile operation | LPAd retrieves Polling Address from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH_AND_AUTH_CLIENT_REQ_V3 |  |  |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( MTD_AUTH_CLIENT_RPM_PKG_REQ_FOR_SINGLE_CMND( delete, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, <S_SM_DP+_SIGNATURE3>, NO_PARAM, NO_PARAM, NO_PARAM ) ) | No error |
| 5 | LPAd → S_EndUser | Request for User Confirmation. | LPAd initiates Confirmation Request for Strong Confirmation |
| 6 | S_EndUser | Accepts the RPM command | PROFILE_OPERATIONAL1 is deleted |
| 7 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( MTD_RESP_RPR_FOR_SINGLE_CMND( deleteResult, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, 0, -- OK response #NOTIF_METADATA_PROF1_DP1_RPR, #S_SM_DP+_OID, NO_PARAM, NO_PARAM, NO_PARAM) ) ) |
| 8 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |
| 9 | S_EndUser → LPAd | List Profile operation is initiated | PROFILE_OPERATIONAL2 is in Enabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. |  |  |  |

Test Sequence #02 Nominal: RPM Command - Delete a Disabled Profile without PPR2 and with PPR1

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_CONF_ALL_PPR1 is installed and Disabled. |
| eUICC | The PROFILE_OPERATIONAL2 with #METADATA_OP_PROF2_RPM_CONF_EN_OWNER_OID1 is loaded and Enabled on the eUICC. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) for Deleting the Profile |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Update Profile operation | LPAd retrieves Polling Address from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH_AND_AUTH_CLIENT_REQ_V3 |  |  |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( MTD_AUTH_CLIENT_RPM_PKG_REQ_FOR_SINGLE_CMND( delete, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, <S_SM_DP+_SIGNATURE3>, NO_PARAM, NO_PARAM, NO_PARAM ) ) | No error See NOTE 1 below. |
| 5 | LPAd → S_EndUser | Request for User Confirmation. | LPAd initiates Confirmation Request for Strong Confirmation LPAd may request End User Consent and it may combine with the Confirmation |
| 6 | S_EndUser | Accepts the RPM command | PROFILE_OPERATIONAL1 is deleted |
| 7 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( MTD_RESP_RPR_FOR_SINGLE_CMND( deleteResult, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, 0, -- OK response #NOTIF_METADATA_PROF1_DP1_RPR, #S_SM_DP+_OID, NO_PARAM, NO_PARAM, NO_PARAM) ) ) |
| 8 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |
| 9 | S_EndUser → LPAd | List Profile operation is initiated | PROFILE_OPERATIONAL2 is in Enabled state |
| NOTE 1: The LPAd MAY display any relevant part of the Profile Metadata and MAY offer the S_EndUser to postpone or reject the Profile installation. The S_EndUser SHALL not abort the transaction. |  |  |  |

Test Sequence #03 Error: RPM Command Result Data Error -  Delete Profile, Profile is not in Disabled state

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 has been loaded on the eUICC with #METADATA_OP_PROF1_RPM_CONF_ALL and it is in Enabled state. (PPR2 is not set in the Metadata) |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) for Disabling the Profile. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Update Profile operation | LPAd retrieves Polling Address from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH_AND_AUTH_CLIENT_REQ_V3 |  |  |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( MTD_AUTH_CLIENT_RPM_PKG_REQ_FOR_SINGLE_CMND( delete, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, <S_SM_DP+_SIGNATURE3>, NO_PARAM, NO_PARAM, NO_PARAM ) ) | No error |
| 5 | LPAd → S_EndUser | Request for User Confirmation. | LPAd initiates Confirmation Request for Simple Confirmation |
| 6 | S_EndUser | Accepts the RPM command | The LPAd MAY inform the End User of the status indicating the error profileNotInDisabledState. |
| 7 | PROC_ES9+_HANDLE_NOTIF_RPR_ERROR( MTD_RESP_RPR_FOR_SINGLE_CMND( deleteResult, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, 1, -- error response #NOTIF_METADATA_PROF1_DP1_RPR, #S_SM_DP+_OID, NO_PARAM, NO_PARAM, profileNotInDisabledState ) ) |  |  |
| 8 | S_EndUser → LPAd | List Profile operation is initiated | PROFILE_OPERATIONAL1 is in Enabled state |

Test Sequence #04 Error: RPM Command Result Data Error -  Delete Profile, disallowed by Policy

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 has been loaded with #METADATA_OP_PROF1_RPM_CONF_DE_PPR2 and is Disabled on the eUICC. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) for Disabling the Profile. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Update Profile operation | LPAd retrieves Polling Address from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH_AND_AUTH_CLIENT_REQ_V3 |  |  |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( MTD_AUTH_CLIENT_RPM_PKG_REQ_FOR_SINGLE_CMND( delete, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, <S_SM_DP+_SIGNATURE3>, NO_PARAM, NO_PARAM, NO_PARAM ) ) | No error |
| 5 | LPAd → S_EndUser | Request for User Confirmation. | LPAd initiates Confirmation Request for Simple Confirmation |
| 6 | S_EndUser | Accepts the RPM command | The LPAd MAY inform the End User of the status indicating the error disallowedByPolicy. |
| 7 | PROC_ES9+_HANDLE_NOTIF_RPR_ERROR( MTD_RESP_RPR_FOR_SINGLE_CMND( disableResult, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, 1, -- error response #NOTIF_METADATA_PROF1_DP1_RPR, #S_SM_DP+_OID, NO_PARAM, NO_PARAM, disallowedByPolicy ) ) |  |  |
| 8 | S_EndUser → LPAd | List Profile operation is initiated | PROFILE_OPERATIONAL1 is in Disable state |

### 5.4.15	RPM Command Execution – List Profile Info

#### 5.4.15.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

2.6.6.2, 2.10.1

3.0.1

3.2.7, 3.7.2

3.7.3

5.5.3, 5.6.3, 5.7.14a

5.7.15

#### 5.4.15.2	Test Cases

##### 5.4.15.2.1	TC_LPAd_RPM_Command_Execution_ListProfileInfo

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled |
| Device | RPM operation is enabled in the LPA by the End User |
| eUICC | There is no default SM-DP+ address configured |
| S_SM-DP+ | The eUICC supports RPM |

Test Sequence #01 Nominal: RPM Command - ListProfileInfo_by ICCID

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_CONF_ALL is installed and Enabled. |
| eUICC | The PROFILE_OPERATIONAL2 with #METADATA_OP_PROF2_RPM_CONF_ALL_OWNER2 is loaded and Disabled on the eUICC. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) for ListProfileInfo the Profile |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Update Profile operation | LPAd retrieves Polling Address from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH_AND_AUTH_CLIENT_REQ_V3 |  |  |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( MTD_AUTH_CLIENT_RPM_PKG_REQ_FOR_SINGLE_CMND_LIST_PROFILE_INFO ( <S_TRANSACTION_ID>, #ICCID_OP_PROF1, <S_SM_DP+_SIGNATURE3>, NO_PARAM, NO_PARAM ) ) | No error No user Confirmation |
| 5 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( MTD_RESP_RPR_FOR_SINGLE_CMND( listProfileInfoResult, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, 0, -- OK response #NOTIF_METADATA_PROF1_DP1_RPR, #S_SM_DP+_OID, #PROFILE_INFO1, NO_PARAM, NO_PARAM) ) ) |
| 6 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |

Test Sequence #02 Nominal: RPM Command - ListProfileInfo_with ICCID  and tagList PPR present

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_CONF_ALL_PPR1 is installed and Enabled. |
| eUICC | The PROFILE_OPERATIONAL2 with #METADATA_OP_PROF2_RPM_CONF_ALL_OWNER2 is loaded and Disabled on the eUICC. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) for ListProfileInfo the Profile |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Update Profile operation | LPAd retrieves Polling Address from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH_AND_AUTH_CLIENT_REQ_V3 |  |  |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( MTD_AUTH_CLIENT_RPM_PKG_REQ_FOR_SINGLE_CMND_LIST_PROFILE_INFO ( <S_TRANSACTION_ID>, #ICCID_OP_PROF1, <S_SM_DP+_SIGNATURE3>, NO_PARAM, ‘99’H ) ) | No error No user Confirmation |
| 5 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( MTD_RESP_RPR_FOR_SINGLE_CMND( listProfileInfoResult, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, 0, -- OK response #NOTIF_METADATA_PROF1_DP1_RPR, #S_SM_DP+_OID, #PROFILES_INFO_TAGLIST_PPR1, NO_PARAM, NO_PARAM) ) ) |
| 6 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |

Test Sequence #03 Nominal: RPM Command - ListProfileInfo_with Profile Owner ID  and tagList with multiple Tags present

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_CONF_ALL_PPR1 is installed and Enabled. |
| eUICC | The PROFILE_OPERATIONAL2 with #METADATA_OP_PROF2_RPM_CONF_ALL_OWNER2 is loaded and Disabled on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL3 with #METADATA_OP_PROF3_RPM_CONF_ALL is loaded and Disabled on the eUICC. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) for ListProfileInfo the Profile |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Update Profile operation | LPAd retrieves Polling Address from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH_AND_AUTH_CLIENT_REQ_V3 |  |  |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( MTD_AUTH_CLIENT_RPM_PKG_REQ_FOR_SINGLE_CMND_LIST_PROFILE_INFO ( <S_TRANSACTION_ID>, NO_PARAM, <S_SM_DP+_SIGNATURE3>, #S_PROFILE_OWNER_OID, ‘BB9ABC5A’H ) ) | No error No user Confirmation |
| 5 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( MTD_RESP_RPR_FOR_SINGLE_CMND( listProfileInfoResult, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, 0, -- OK response #NOTIF_METADATA_PROF1_DP1_RPR, #S_SM_DP+_OID, { #PROFILES_INFO_MULT_TAGS_PROFILE1, #PROFILES_INFO_MULT_TAGS_PROFILE3}, NO_PARAM, NO_PARAM) ) ) |
| 6 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |

Test Sequence #04 Error: RPM Command - ListProfileInfo_ICCID specified, SM-DP+ OID not in Managing SM-DP+ list

The purpose of this Test Sequence is to ensure RPM Command ListProfileInfo is not executed if the SM-DP+ that sent the RPM Command is not included in the Managing SM-DP+ List in the Profile Metadata (rpmConfiguration) and LPA sends the Notification with error ‘disallowedManaging’ to the SM-DP+.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with #METADATA_OP_PROF1_RPM_CONF_ALL_DP_OID2 is installed and Enabled. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) for RPM commands ListProfileInfo (TagList with rpmConfiguration). |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Update Profile operation | LPAd retrieves Polling Address from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH_AND_AUTH_CLIENT_REQ_V3 |  |  |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( MTD_AUTH_CLIENT_RPM_PKG_REQ_FOR_SINGLE_CMND_LIST_PROFILE_INFO ( <S_TRANSACTION_ID>, #ICCID_OP_PROF1, <S_SM_DP+_SIGNATURE3>, NO_PARAM, ‘5A’H ) ) | No error No user Confirmation |
| 5 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( MTD_RESP_RPR_FOR_SINGLE_CMND( listProfileInfoResult, <S_TRANSACTION_ID>, NO_PARAM, #ICCID_OP_PROF1, 1, -- error response #NOTIF_METADATA_PROF1_DP1_RPR, #S_SM_DP+_OID, NO_PARAM, NO_PARAM, disallowedManaging) ) ) |
| 6 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |

Test Sequence #05 Error: RPM Command_ListProfileInfo_- Profile Owner ID specified, allowed CI public key identifier does not match

The purpose of this test sequence is to ensure RPM Command ListProfileInfo is not executed if Subject Key Identifier of the CI corresponding to CERT.DPauth.ECDSA attached to the ongoing session does not match with the Allowed CI Public Key Identifier in the Profile Metadata (rpmConfiguration) and LPA sends the Notification with error ‘ciPKMismatch’ to the SM-DP+.

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with # METADATA_OP_PROF1_RPM_CONF_ALL_CI_PKI_RAND is installed and Enabled. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) for ListProfileInfo (with TagList with multiple tags) |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Update Profile operation | LPAd retrieves Polling Address from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH_AND_AUTH_CLIENT_REQ_V3 |  |  |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( MTD_AUTH_CLIENT_RPM_PKG_REQ_FOR_SINGLE_CMND_LIST_PROFILE_INFO ( <S_TRANSACTION_ID>, NO_PARAM, <S_SM_DP+_SIGNATURE3>, #S_PROFILE_OWNER_OID, ‘BB9ABC5A’H ) ) | No error No user Confirmation |
| 5 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( MTD_RESP_RPR_FOR_SINGLE_CMND( listProfileInfoResult, <S_TRANSACTION_ID>, NO_PARAM, #ICCID_OP_PROF1, 1, -- error response #NOTIF_METADATA_PROF1_DP1_RPR, #S_SM_DP+_OID, NO_PARAM, NO_PARAM, ciPKMismatch) ) ) |
| 6 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |

### 5.4.16	RPM Command Execution – Update Metadata

#### 5.4.16.1	Conformance Requirements

References

GSMA RSP Technical Specification [2]:

2.6.6.2, 2.10.1

3.0.1

3.2.7, 3.7.2

3.7.3

5.5.3, 5.6.3, 5.7.14a

#### 5.4.16.2	Test Cases

##### 5.4.16.2.1	TC_LPAd_RPM_Command_Execution_UpdateMetadata

| General Initial Conditions |  |
| --- | --- |
| Entity | Description of the general initial condition |
| Device | The protection of access to the LUI is disabled |
| Device | RPM operation is enabled in the LPA by the End User |
| eUICC | There is no default SM-DP+ address configured The eUICC supports RPM |
| S_SM-DP+ | Variant A certificates are included in certificates chain for TLS procedures. |

Test Sequence #01 Nominal: RPM Command - UpdateMetadata – Remove PPR1

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | PROFILE_OPERATIONAL1 with # METADATA_OP_PROF1_RPM_CONF_UPDATE_MD_PPR is installed and Enabled. |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) for Update Metadata. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Update Profile operation | LPAd retrieves Polling Address from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH_AND_AUTH_CLIENT_REQ_V3 |  |  |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( MTD_AUTH_CLIENT_RPM_PKG_REQ_FOR_SINGLE_CMND( updateMetadata, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, <S_SM_DP+_SIGNATURE3>, profilePolicyRules {ppr2}, NO_PARAM, NO_PARAM ) ) | No error See NOTE 1 below. |
| 5 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( MTD_RESP_RPR_FOR_SINGLE_CMND( updateMetadataResult, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, 0, -- OK response #NOTIF_METADATA_PROF1_DP1_RPR, #S_SM_DP+_OID, NO_PARAM, NO_PARAM, NO_PARAM) ) ) |
| 6 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |
| NOTE 1: The LPAd MAY ask for the S_End_User consent by showing relevant information concerning the Profile and PPR(s). This information MAY include the consequences of the Profile Policy Rule |  |  |  |

Test Sequence #02 Nominal: RPM Command - UpdateMetadata – RPM  Configuration

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 has been installed on the eUICC with #METADATA_OP_PROF1_RPM_CONF_ALL_PPR1 |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) for Update Metadata. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Update Profile operation | LPAd retrieves Polling Address from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH_AND_AUTH_CLIENT_REQ_V3 |  |  |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( MTD_AUTH_CLIENT_RPM_PKG_REQ_FOR_SINGLE_CMND( updateMetadata, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, <S_SM_DP+_SIGNATURE3>, rpmConfiguration #RPM_CONFIG_OP_PROF1, NO_PARAM, NO_PARAM ) ) | No error |
| 5 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( MTD_RESP_RPR_FOR_SINGLE_CMND( updateMetadataResult, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, 0, -- OK response #NOTIF_METADATA_PROF1_DP1_RPR, #S_SM_DP+_OID, NO_PARAM, NO_PARAM, NO_PARAM) ) ) |
| 6 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |

Test Sequence #03 Nominal: RPM Command UpdateMetadata – Multiple Tags

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 has been installed on the eUICC with #METADATA_OP_PROF1_RPM_CONF_ALL_PPR1 |
| S_SM-DP+ | There is a pending RPM package order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) for Update Metadata. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| 1 | S_EndUser→ LPAd | Initiate Update Profile operation | LPAd retrieves Polling Address from the ProfileInfo, and the Polling Address indicates the SM-DP+ address #TEST_DP_ADDRESS1. |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH_AND_AUTH_CLIENT_REQ_V3 |  |  |
| 4 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( MTD_AUTH_CLIENT_RPM_PKG_REQ_FOR_SINGLE_CMND( updateMetadata, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, <S_SM_DP+_SIGNATURE3>, #MULTIPLE_TAGS_OP_PROF1, NO_PARAM, NO_PARAM ) ) | No error |
| 5 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF( MTD_RESP_RPR_FOR_SINGLE_CMND( updateMetadataResult, <S_TRANSACTION_ID>, #ICCID_OP_PROF1, 0, -- OK response #NOTIF_METADATA_PROF1_DP1_RPR, #S_SM_DP+_OID, NO_PARAM, NO_PARAM, NO_PARAM) ) ) |
| 6 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |

# 6	VOID

# 7	VOID

- Annex A	Constants

- A.1	Generic Constants

| Name | Content |
| --- | --- |
| ACTIVATION_CODE_1 | 1$#TEST_DP_ADDRESS1$#MATCHING_ID_1 ACTIVATION_CODE_1.png as defined in Annex H |
| ACTIVATION_CODE_2 | 1$#TEST_DP_ADDRESS1$#MATCHING_ID_2$#S_SM_DP+_OID ACTIVATION_CODE_2.png as defined in Annex H |
| ACTIVATION_CODE_3 | 1$#TEST_DP_ADDRESS1$#MATCHING_ID_3$$1 ACTIVATION_CODE_3.png as defined in Annex H |
| ACTIVATION_CODE_3_NO_CC | 1$#TEST_DP_ADDRESS1$#MATCHING_ID_3 ACTIVATION_CODE_3_NO_CC.png as defined in Annex H |
| ACTIVATION_CODE_4 | 1$#TEST_DP_ADDRESS1$#MATCHING_ID_4 ACTIVATION_CODE_4.png as defined in Annex H |
| ACTIVATION_CODE_5 | 1$#TEST_DP_ADDRESS1$#MATCHING_ID_EMPTY ACTIVATION_CODE_5.png as defined in Annex H |
| ACTIVATION_CODE_INVALID_FORMAT | 1#TEST_DP_ADDRESS1$#MATCHING_ID_1 ACTIVATION_CODE_INVALID_FORMAT.png as defined in Annex H |
| ACTIVATION_CODE_WITH_CI_PK_IND_RAND | 1#TEST_DP_ADDRESS1$#MATCHING_ID_1$#S_SM_DP+_OID$$#PK_CI_ECDSA_RANDOM ACTIVATION_CODE_INVALID_FORMAT.png as defined in Annex H |
| ADDITIONAL_SMDP_DATA_EXCEEDED_MAX | 0x01 02 03…76 77 78 -- additional data objects defined by the S_SM-DP+ depending on the length of the SM-DP+ OID, to ensure that total length of dpProprietaryData is 129 bytes |
| ADDITIONAL_SMDP_DATA_MAX_LENGTH | 0x01 02 03…75 76 77 -- additional data objects defined by the S_SM-DP+ depending on the length of the SM-DP+ OID, to ensure that total length of dpProprietaryData is 128 bytes |
| CHANGE_CIPHER_SPEC | 1 |
| CLIENT_CERT_TYPE | 64. The Certificate Type requested from the client by the server in the Certificate Request message as ecdsa_sign(64). |
| CONFIRMATION_CODE1 | 0102030405 |
| CONFIRMATION_CODE2 | ABCDEFGHIJ |
| CTX_PARAMS1_MATCH_ID_DEV_INFO (CtxParams1) | ctxParamsForCommonAuthentication : { matchingId <MATCHING_ID>, -- OPTIONAL - see NOTE #DEVICE_INFO } NOTE: the matchingId field may be present (with value <MATCHING_ID>) or may be absent. The presence or absence of matchingId may be checked in individual test cases. |
| CTX_PARAMS1_MATCH_ID_DEV_INFO_V3 (CtxParams1) | ctxParamsForCommonAuthentication : { matchingId <MATCHING_ID>, deviceInfo #DEVICE_INFO, operationType { profileDownload }, OPTIONAL matchingIdSource activationCode : NULL, vendorSpecificExtension #VENDOR_SPECIFIC_EXT –- OPTIONAL } |
| CTX_PARAMS1_RPM_ICCID1 | ctxParamsForCommonAuthentication : { #DEVICE_INFO, operationType { rpm }, iccid #ICCID_OP_PROF1, matchingIdSource none NULL, vendorSpecificExtension #VENDOR_SPECIFIC_EXT –- OPTIONAL } |
| DEVICE_INFO | deviceInfo { tac ..., deviceCapabilities { ... }, imei #IUT_IMEI -- Optional preferredLanguages ..., -- OPTIONAL deviceTestMode ..., -- OPTIONAL lpaRspCapability <LPA_RSP_CAPABILITY> }-- Check only that the field is present and has a valid TLV asn.1 structure unless it is defined differently in the individual test cases. |
| EF_UST1 | 0x0A 2E 14 8C E7 32 04 00 00 00 00 00 00 -- NOTE: Service n°17 (GID1) and n°18 (GID2) not available |
| EF_UST2 | 0x0A 2E 17 8C E7 32 04 00 00 00 00 00 00 -- NOTE: Service n°17 (GID1) and n°18 (GID2) available |
| EID1 | 0x89 04 90 32 12 34 51 23 45 12 34 56 78 90 12 35 |
| EID1_QR_CODE1 | QR code which decodes as: EID:89049032123451234512345678901235 |
| EID1_QR_CODE2 | QR code which decodes as: EID:89 04 90 32 12 34 51 23 45 12 34 56 78 90 12 35 |
| EID2 | 0x89 04 90 32 11 23 41 23 40 12 34 56 78 90 13 75 |
| ENT_CONF_REF_ENT_RULE | { enterpriseOid #S_ENTERPRISE_OID, enterpriseName #ENTERPRISE_NAME1, enterpriseRules { referenceEnterpriseRule, onlyThisProfileCanBeEnabled, onlyEnterpriseProfilesCanBeInstalled } } |
| EUICC_CI_PK_ID_LIST_FOR_SIGNING_1 | #CI_PKI_ID1, #CI_PKI_ID2 |
| EUICC_CI_PK_ID_LIST_FOR_SIGNING_2 | #CI_PKI_ID3, #CI_PKI_ID4 |
| EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1 | #CI_PKI_ID1, #CI_PKI_ID2 |
| EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_2 | #CI_PKI_ID3, #CI_PKI_ID4 |
| EUICC_INFO1_8_8_2_3_1 | euiccInfo1_8_8_2_3_1 EUICCInfo1 ::= { svn #RSP_SVN, euiccCiPKIdListForVerification { #EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1 }, euiccCiPKIdListForSigning { #EUICC_CI_PK_ID_LIST_FOR_SIGNING_2 } } |
| EUICC_INFO1_8_8_3_3_1_HIGHER | euiccInfo1_8_8_3_3_1 EUICCInfo1 ::= { svn #RSP_SVN_HIGHER, euiccCiPKIdListForVerification { #EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1 }, euiccCiPKIdListForSigning { #EUICC_CI_PK_ID_LIST_FOR_SIGNING_1 } } |
| EUICC_INFO1_8_8_3_3_1_LOWER | euiccInfo1_8_8_3_3_1 EUICCInfo1 ::= { svn #RSP_SVN_LOWER, euiccCiPKIdListForVerification { #EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1 }, euiccCiPKIdListForSigning { #EUICC_CI_PK_ID_LIST_FOR_SIGNING_1 } } |
| EUICC_INFO1_8_8_4_3_7 | euiccInfo1_8_8_4_3_7 EUICCInfo1 ::= { svn #RSP_SVN, euiccCiPKIdListForVerification { #EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_2 }, euiccCiPKIdListForSigning { #EUICC_CI_PK_ID_LIST_FOR_SIGNING_1 } } |
| EUICC_SIGNED1 | { transactionId <S_TRANSACTION_ID>, serverAddress #TEST_DP_ADDRESS1, serverChallenge <S_SMDP_CHALLENGE>, euiccInfo2 #R_EUICC_INFO2, -- check only that the field is present and has a valid TLV asn.1 structure ctxParams1 #CTX_PARAMS1 } |
| EVENT_ID_1 | 07399-BGH7E-T8779 |
| EVENT_ID_2 | 07399-BGH7E-T8778 |
| EXT_SHA256_RSA | TLS extension data for "supported_signature_algorithms" set as: o HashAlgorithm sha256 (04) and o SignatureAlgorithm rsa (01). |
| FUNCTION_CALL_ID_1 | 0000-0000-0000-0001 |
| FUNCTION_CALL_ID_2 | 0000-0000-0000-0002 |
| GID1 | 0x47 53 4D 41 |
| GID2 | 0x52 53 50 FF |
| HOST_ID | 0x47 53 4D 41 20 53 4D 2D 58 58 -- NOTE: 'GSMA SM-XX' in ASCII |
| ICCID_OP_PROF1 | 0x98 92 09 01 21 43 65 87 09 F5 |
| ICCID_OP_PROF2 | 0x98 92 09 01 32 54 76 98 10 F9 |
| ICCID_OP_PROF3 | 0x98 92 09 01 43 65 87 09 21 F5 |
| ICCID_OP_PROF4 | 0x98 92 09 01 54 76 98 10 32 F9 |
| ICCID_OP_PROF5 | 0x98 92 09 01 65 87 09 21 43 F5 |
| ICCID_OP_PROF6 | 0x98 92 09 01 76 98 10 32 54 F9 |
| ICCID_OP_PROF7 | 0x98 92 09 01 87 09 21 43 65 F5 |
| ICCID_OP_PROF8 | 0x98 92 09 01 98 10 32 54 76 F9 |
| ICCID_OP_PROF9 | 0x98 92 09 01 21 43 65 87 76 F5 |
| ICCID_OP_PROFX | 0x98 92 09 01 43 65 87 09 FF FF |
| ICCID_UNKNOWN | 0x98 92 01 0A 21 43 65 87 09 F8 |
| ICON_JPG | ICON_JPG.jpg as defined in Annex H |
| ICON_OP_PROF1 | profile_O1.png as defined in Annex H |
| ICON_OP_PROF1_2_SEG | profile_O1_2_SEG.png as defined in Annex H |
| ICON_OP_PROF2 | profile_O2.png as defined in Annex H |
| ICON_OP_PROF3 | profile_O3.png as defined in Annex H |
| ICON_OP_PROF4 | profile_O4.png as defined in Annex H |
| ICON_OP_PROF5 | profile_O5.png as defined in Annex H |
| ICON_OP_PROF6 | profile_O6.png as defined in Annex H |
| ICON_OP_PROF7 | profile_O7.png as defined in Annex H |
| ICON_OP_PROF8 | profile_O8.png as defined in Annex H |
| IMSI_OP_PROF1 | 0x08 29 99 18 11 32 54 76 98 |
| IMSI_OP_PROF2 | 0x08 29 99 28 11 32 54 76 97 |
| IMSI_OP_PROF3 | 0x08 29 99 28 11 32 54 76 96 |
| IMSI_OP_PROF4 | 0x08 29 99 48 43 65 87 09 21 |
| IMSI_OP_PROF5 | 0x08 29 99 18 11 32 54 76 98 |
| IMSI_OP_PROF6 | 0x08 29 99 28 11 32 54 76 97 |
| IMSI_OP_PROF7 | 0x08 29 99 28 43 65 87 09 21 |
| IMSI_OP_PROF8 | 0x08 29 99 28 43 65 87 09 21 |
| IMSI_OP_PROF9 | 0x08 29 99 98 43 65 87 09 21 |
| INSTALLED_PROFILES | 0x00 |
| INVALID_KEY_TYPE | 0x80 |
| INVALID_REMOTE_OP_ID | 8 |
| ISD_R_AID | 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 00 01 00 |
| KEY_LENGTH | 0x10 |
| KEY_TYPE | 0x88 |
| MATCHING_ID_1 | 04386-AGYFT-A74Y8-3F815 |
| MATCHING_ID_2 | 04386-AGYFT-A74Y8-3F816 |
| MATCHING_ID_3 | 04386-AGYFT-A74Y8-3F817 |
| MATCHING_ID_4 | 04386-AGYFT-A74Y8-3F818 |
| MCC_MNC_WILDCARD | 0x92 F9 EE |
| MCC_MNC1 | 0x92 F9 18 |
| MCC_MNC2 | 0x92 F9 28 |
| MCC_MNC4 | 0x92 F9 48 |
| MCC_MNC9 | 0x92 F9 98 |
| MIN_TLS_CIPHER_SUITES | The minimum TLS cipher suites proposed by the Client: o TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 o TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 |
| MNO_SCP80_AUTH_KEY | 0x11 22 33 44 55 66 77 88 99 AA BB CC DD EE FF 10 |
| MNO_SCP80_DATA_ENC_KEY | 0x99 AA BB CC DD EE FF 10 11 22 33 44 55 66 77 88 |
| MNO_SCP80_ENC_KEY | 0x66 77 88 99 AA BB CC DD 11 22 33 44 55 EE FF 10 |
| NAME_OP_PROF_LONG | Operational Profile Name with long name of sixty four characters NOTE: the exact text above SHOULD be used, as it is exactly 64 characters long. |
| NAME_OP_PROF1 | Operational Profile Name 1 |
| NAME_OP_PROF1_NON_ASCII | Operational Profile Name UTF-8 encoding: 0x4F 70 65 72 61 74 69 6F 6E 61 6C 20 50 72 6F 66 69 6C 65 20 4E 61 6D 65 20 E4 BD A0 E5 A5 BD |
| NAME_OP_PROF2 | Operational Profile Name 2 |
| NAME_OP_PROF3 | Operational Profile Name 3 |
| NAME_OP_PROF4 | Operational Profile Name 4 |
| NAME_OP_PROF5 | Operational Profile Name 5 |
| NAME_OP_PROF6 | Operational Profile Name 6 |
| NAME_OP_PROF7 | Operational Profile Name 7 |
| NAME_OP_PROF8 | Operational Profile Name 8 |
| NAME_OP_PROF9 | Operational Profile Name 9 |
| NICKNAME1 | Nickname 1 |
| NICKNAME2 | Nickname 2 |
| NICKNAME3 | Nickname 3 |
| NICKNAME4 | Nickname 4 |
| OWNER_OP_PROF1 | { mccMnc #MCC_MNC1 } |
| OWNER_OP_PROF2 | { mccMnc #MCC_MNC2 } |
| PATH_AUTH_CLIENT | /gsma/rsp2/es9plus/authenticateClient |
| PATH_CANCEL_ORDER | /gsma/rsp2/es2plus/cancelOrder |
| PATH_CANCEL_SESSION | /gsma/rsp2/es9plus/cancelSession |
| PATH_CONFIRM_ORDER | /gsma/rsp2/es2plus/confirmOrder |
| PATH_DELETE_EVENT | /gsma/rsp2/es12/deleteEvent |
| PATH_DOWNLOAD_ORDER | /gsma/rsp2/es2plus/downloadOrder |
| PATH_GET_BPP | /gsma/rsp2/es9plus/getBoundProfilePackage |
| PATH_HANDLE_NOTIF | /gsma/rsp2/es9plus/handleNotification |
| PATH_INITIATE_AUTH | /gsma/rsp2/es9plus/initiateAuthentication |
| PATH_REGISTER_EVENT | /gsma/rsp2/es12/registerEvent |
| PO1_PIN1 | 0x32 34 36 38 FF FF FF FF |
| PO2_PIN1 | 0x33 35 37 39 FF FF FF FF |
| PPK_ENC_INV_SIZE | 0x01 02 03 04 05 06 07 08 09 0A 0B 0C 0D 0E 0F 10 0D 0E 0F 10 0D 0E 0F 10 |
| PPK_INIT_MAC_INV_SIZE | 0x05 0A 04 0B 03 0C 02 0D 01 0E 00 0F 09 01 08 02 09 01 08 02 09 01 08 02 |
| PPK_MAC_INV_SIZE | 0x01 0E 00 0F 09 01 08 02 05 0A 04 0B 03 0C 02 0D 03 0C 02 0D 03 0C 02 0D |
| PROFILE_STATUS_AVAILABLE | Available |
| PROP_TLS_CIPHER_SUITES | The TLS cipher suites proposed by the Client: o TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 o TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 o TLS_RSA_WITH_AES_128_CBC_SHA o TLS_RSA_WITH_AES_256_CBC_SHA256 o TLS_DHE_RSA_WITH_AES_256_CBC_SHA256 |
| REMOTE_OP_ID_INSTALL | 1 |
| RPM_PKG_DELETE | rpmPackage { { continueOnFailure NULL, rpmCommandDetails delete : { iccid #ICCID_OP_PROF1 } } |
| RPM_PKG_DISABLE | rpmPackage { { continueOnFailure NULL, rpmCommandDetails disable : { iccid #ICCID_OP_PROF1 } } |
| RPM_PKG_EN | rpmPackage { { continueOnFailure NULL, rpmCommandDetails enable : { iccid #ICCID_OP_PROF1 } } |
| RPM_PKG_UM_PPR | rpmPackage { { continueOnFailure NULL, rpmCommandDetails updateMetadata : { iccid #ICCID_OP_PROF1, updateMetadataRequest { profilePolicyRules {ppr2}} } } |
| RPM_PKG_UM_REF_ENT_RULE | rpmPackage { { continueOnFailure NULL, rpmCommandDetails updateMetadata : { iccid #ICCID_OP_PROF1, updateMetadataRequest { enterpriseConfiguration #ENT_CONF_REF_ENT_RULE } } } |
| RSP_SVN_LOWEST | eUICC lowest svn encoded as the value part of an ASN.1 VersionType (e.g. 0x02 00 00) |
| RSP_SVN_HIGHEST | eUICC highest svn encoded as the value part of an ASN.1 VersionType (e.g. 0x03 01 00) |
| RSP_SVN | This field is set to #IUT_RSP_VERSION (e.g. 2.1.0) |
| RSP_SVN_H | This field is set to #IUT_RSP_VERSION encoded as the value part of an ASN.1 VersionType (e.g. 0x02 01 00) |
| RSP_SVN_HIGHER | 100.0.0 |
| RSP_SVN_LOWER | 0.0.0 |
| S_DEVICE_CAP_EXT | deviceCapExt DeviceCapExt ::= { unknownServiceSupport2 } using the following definition of DeviceCapExt: DeviceCapExt ::= INTEGER { unknownServiceSupport1 (0), unknownServiceSupport2 (1) } |
| S_DEVICE_INFO | deviceInfo { tac #S_TAC, deviceCapabilities { gsmSupportedRelease '050000'H, utranSupportedRelease '080000'H, cdma2000onexSupportedRelease '010000'H, cdma2000hrpdSupportedRelease '010000'H, cdma2000ehrpdSupportedRelease '020000'H, eutranSupportedRelease '020000'H, contactlessSupportedRelease '090000'H, rspCrlSupportedVersion #RSP_SVN_H } } |
| S_DEVICE_INFO_EXT | deviceInfo DeviceInfo { tac #S_TAC, deviceCapabilities { gsmSupportedRelease '050000'H, utranSupportedRelease '080000'H, cdma2000onexSupportedRelease '010000'H, cdma2000hrpdSupportedRelease '010000'H, cdma2000ehrpdSupportedRelease '020000'H, eutranEpcSupportedRelease '020000'H, contactlessSupportedRelease '090000'H, rspCrlSupportedVersion, #RSP_SVN_H nrEpcSupportedRelease '0F0000'H, nr5gcSupportedRelease '0F0000'H, eutran5gcSupportedRelease '0F0000'H, unknownServiceSupport #S_DEVICE_CAP_EXT } } Note: the definition of DeviceInfo used above is equivalent to the definition in SGP.22 v2.3 (specific version of [2]) with the additional of a further field called “unknownServiceSupport” of type DeviceCapExt (see #S_DEVICE_CAP_EXT) after the “eutran5gcSupportedRelease” field. |
| S_DEVICE_INFO_IMEI | deviceInfo { tac #S_TAC, deviceCapabilities { gsmSupportedRelease '050000'H, utranSupportedRelease '080000'H, cdma2000onexSupportedRelease '01000'H, eutranSupportedRelease '020000'H }, imei #S_IMEI } |
| S_EUICC_CHALLENGE | 0x01 02 03 04 05 06 07 08 01 02 03 04 05 06 07 08 |
| S_EUICC_CHALLENGE_2 | 0x21 22 23 24 25 26 27 28 21 22 23 24 25 26 27 28 |
| S_EUICC_INFO1 | euiccInfo1 EUICCInfo1 ::= { svn #RSP_SVN, euiccCiPKIdListForVerification { #EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1 }, euiccCiPKIdListForSigning { #EUICC_CI_PK_ID_LIST_FOR_SIGNING_1 } } |
| S_EUICC_INFO2 | euiccInfo2 EUICCInfo2 ::= { profileVersion #PROFILE_VERSION, svn #RSP_SVN_H, euiccFirmwareVer #EUICC_FIRMWARE_VER, extCardResource #S_EXT_CARD_RESOURCE, uiccCapability #UICC_CAPABILITY, rspCapability #RSP_CAPABILITY, euiccCiPKIdListForVerification {#EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1}, euiccCiPKIdListForSigning {#EUICC_CI_PK_ID_LIST_FOR_SIGNING_1}, ppVersion #PP_VERSION, sasAcreditationNumber #SAS_ACREDITATION_NUMBER } |
| S_EUICC_INFO2_UICC_EXT | euiccInfo2 EUICCInfo2 ::= { profileVersion #PROFILE_VERSION, svn #RSP_SVN_H, euiccFirmwareVer #EUICC_FIRMWARE_VER, extCardResource #S_EXT_CARD_RESOURCE, uiccCapability #UICC_CAPABILITY_EXT, rspCapability #RSP_CAPABILITY, euiccCiPKIdListForVerification {#EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1}, euiccCiPKIdListForSigning {#EUICC_CI_PK_ID_LIST_FOR_SIGNING_1}, ppVersion #PP_VERSION, sasAcreditationNumber #SAS_ACREDITATION_NUMBER } |
| S_EUICC_INFO2_DEV_EXT | euiccInfo2 EUICCInfo2 ::= { profileVersion #PROFILE_VERSION, svn #RSP_SVN_H, euiccFirmwareVer #EUICC_FIRMWARE_VER, extCardResource #S_EXT_CARD_RESOURCE, uiccCapability #UICC_CAPABILITY, rspCapability #RSP_CAPABILITY_EXT, euiccCiPKIdListForVerification {#EUICC_CI_PK_ID_LIST_FOR_VERIFICATION_1}, euiccCiPKIdListForSigning {#EUICC_CI_PK_ID_LIST_FOR_SIGNING_1}, ppVersion #PP_VERSION, sasAcreditationNumber #SAS_ACREDITATION_NUMBER } |
| S_EXT_SHA256_ECDSA | TLS extension data for "supported_signature_algorithms" set as: o HashAlgorithm sha256 (04) and o SignatureAlgorithm ecdsa (03). |
| S_IMEI | 0x00 00 00 00 11 11 11 11 |
| S_MNO_F_REQ_ID | “S_MNO” |
| S_PROFILE_OWNER_OID | 2.888.9 |
| S_PROFILE_OWNER_OID2 | 2.888.99 |
| S_SAH_SHA256_ECDSA | Signature And Hash Algorithm extension sent in the CertificateRequest message set as: o HashAlgorithm sha256 (04) and o SignatureAlgorithm ecsda(3). |
| S_SESSION_ID_EMPTY | Empty TLS session ID to identify a new session, with the Length set as ‘zero’. |
| S_SM_DP_SVN | 0x03 00 00 |
| S_SM_DP+_F_REQ_ID | “S_SM_DP_PLUS” |
| S_SM_DP+_OID | 2.999.10 |
| S_SM_DP+_OID2 | 2.999.12 |
| S_SM_DP+_OID4 | 2.999.14 |
| S_SM_DP+_OID8 | 2.999.18 |
| S_SM_DS_F_REQ_ID | “S_SM_DS” |
| S_SM_DS_OID | 2.999.15 |
| S_TAC | 0x00 00 00 00 |
| S_TLS_CIPHER_SUITE | TLS cipher suite selected as follows: o TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 |
| SERVER_ADDRESS | FQDN of the SERVER Under Test which can be one of the following depending on the entity under test: -  #IUT_SM_DP_ADDRESS -  #IUT_SM_DS_ADDRESS_ES11 |
| SIMA_RESULT_OK | simaresp EUICCResponse ::= { peStatus { {status ok} } } |
| SP_NAME_LONG | SP Name as thirty two characters NOTE: the exact text above SHOULD be used, as it is exactly 32 characters long. |
| SP_NAME_NON_ASCII | SP Name UTF-8 encoding: 0x53 50 20 4E 61 6D 65 20 E3 83 AB |
| SP_NAME1 | SP Name 1 |
| SP_NAME2 | SP Name 2 |
| SP_NAME3 | SP Name 3 |
| SP_NAME4 | SP Name 4 |
| SP_NAME8 | SP Name 8 |
| SP_NAME9 | SP Name 9 |
| SSD_AID | 0xA0 00 00 05 59 10 10 01 02 73 64 56 61 6C 75 65 |
| TEST_ALT_DS_ADDRESS | testaltsmds.example.com |
| TEST_DEFAULT_DP_ADDRESS_1 | testdefaultsmdpplus1.example.com |
| TEST_DP_ADDRESS1 | testsmdpplus1.example.com |
| TEST_DP_ADDRESS2 | testsmdpplus2.example.com |
| TEST_DP_ADDRESS3 | testsmdpplus3.example.com |
| TEST_DP_ADDRESS4 | testsmdpplus4.example.com |
| TEST_DP_ADDRESS8 | testsmdpplus8.example.com |
| TEST_DS_ADDRESS1 | testsmds1.example.com |
| TEST_HRI_ADDRESS3 | testhriserver3.example.com |
| TEST_PCMP_ADDRESS1 | testpcmp1.example.com |
| TEST_ROOT_DS_ADDRESS | testrootsmds.example.com |
| TLS_VERSION_1_1 | 1.1 |
| TLS_VERSION_1_2 | 1.2 The minimum TLS Version supported by the Server. |
| UNKNOWN_BPP_SEGMENT | 0xC9 05 01 02 03 04 05 |
| UNKNOWN_SERVER_ADDRESS | unknownserver.example.com |
| UNSUP_TLS_CIPHER_SUITES | The TLS cipher suites proposed by the Client: o TLS_RSA_WITH_AES_128_CBC_SHA o TLS_RSA_WITH_AES_256_CBC_SHA256 |
| UPP_OP_PROF1 | The Unprotected Profile Package related to the PROFILE_OPERATIONAL1 (see Annex E). |
| UPP_OP_PROF2 | The Unprotected Profile Package related to the PROFILE_OPERATIONAL2 (see Annex E). |
| UPP_OP_PROF3 | The Unprotected Profile Package related to the PROFILE_OPERATIONAL3 (see Annex E). |
| UPP_OP_PROF4 | The Unprotected Profile Package related to the PROFILE_OPERATIONAL4 (see Annex E). |
| UPP_OP_PROF9 | The Unprotected Profile Package related to the PROFILE_OPERATIONAL9 (see Annex E). |
| USIM_AID | 0xA0 00 00 00 87 10 02 FF 33 FF 01 89 00 00 01 00 |
| VENDOR_SPECIFIC_EXT | Additional extensions to include the OID of the vendor and the vendorSpecificData defined by vendor, coded as ASN.1 SEQUENCE_OF_SEQUENCE (VendorSpecificExtension). |

- A.2	Test Certificates and Test Keys

All ECC certificates and keys described below are based on descriptions and security requirements (such as algorithms to be used) from SGP.22[2].

NOTE: 	SGP.26 [25] contains test keys, valid test certificates and instructions for how to generate invalid certificates. All test keys and test certificates used in the present document are bundled with SGP.26 [25].

| Name | Description |
| --- | --- |
| CERT_CI_SIG | Certificate of the CI for its Public SIG Key |
| CERT_CI_SubCA_SIG | The intermediate SubCA Certificate through which #CERT_EUM_SIG is chained to the #CERT_CI_SIG in Variants A and C. This certificate contains the same Extension for subjectAltName value as in #CERT_CI_SIG of that same GSMA CI. |
| CERT_EUICC_SIG | Certificate of the eUICC for its Public key CERT.EUICC.SIG in the X.509 format signed by the EUM with SK.EUM.SIG |
| CERT_EUM_SIG | Certificate of the EUM for its Public ECDSA key CERT.EUM.ECDSA in the X.509 format signed by the requested CI with SK.CI.ECDSA. |
| CERT_S_SERVER_TLS | CERT.SERVER.TLS certificate of the S_SERVER, based on NIST or Brainpool for this version of the specification, where the Certificate MAY be one of the following depending on the role of the simulator: -  #CERT_S_SM_DP_TLS on ES9+ -  #CERT_S_SM_DS_TLS on ES11 or ES12 |
| CERT_S_SM_DPauth_PK_CI2_SIG | Certificate of the S_SM-DP+ for its Public key used for SM-DP+ authentication. This certificate contains the OID #S_SM_DP+_OID. The Subject Key Identifier of the GSMA CI RootCA Certificate corresponding to this SM-DPauth certificate is #PK_CI2_SIG. |
| CERT_S_SM_DS_SubCA_SIG | The intermediate SubCA Certificate through which #CERT_S_SM_DSauth_SIG and the #CERT_S_SM_DSpb_SIG and the #CERT_S_SM_DS_TLS are chained to the #CERT_CI_SIG in Variant A or to the #CERT_CI_SubCA_SIG in Variant C. This certificate contains the Extension for subjectAltName as OID #S_SM_DS_OID. |
| CERT_S_SM_DP_SubCA_SIG | The intermediate SubCA Certificate through which #CERT_S_SM_DPauth_SIG and the #CERT_S_SM_DPpb_SIG and the #CERT_S_SM_DP_TLS are chained to the #CERT_CI_SIG in Variant A or to the #CERT_CI_SubCA_SIG in Variant C. This certificate contains the Extension for subjectAltName as OID #S_SM_DP+_OID. |
| CERT_S_SM_DP_TLS | CERT.DP.TLS certificate of the S_SM-DP+, based on the same CI as defined in #IUT_LPAd_CI. |
| CERT_S_SM_DP2_TLS | CERT.DP.TLS certificate of the S_SM-DP+, based on the same CI as defined in #IUT_LPAd_CI. Contains different SM-DP+ hostname (FQDN) as #CERT_S_SM_DP2_TLS. |
| CERT_S_SM_DP4_TLS | CERT.DP.TLS certificate of the S_SM-DP+, based on the same CI as defined in #IUT_LPAd_CI. Contains the SM-DP+ hostname (FQDN) #TEST_DP_ADDRESS4 and OID value #S_SM_DP+_OID4. |
| CERT_S_SM_DP8_TLS | CERT.DP.TLS certificate of the S_SM-DP+, based on the same CI as defined in #IUT_LPAd_CI. Contains the SM-DP+ hostname (FQDN) #TEST_DP_ADDRESS8 and OID value #S_SM_DP+_OID8. |
| CERT_S_SM_DP_TLS_EXPIRED | Expired CERT.DP.TLS certificate of the S_SM-DP+ with a valid signature, correctly formatted as X.509 certificate. |
| CERT_S_SM_DP_TLS_INV_CURVE | CERT.DP.TLS certificate of the S_SM-DP+, based on the different CI as defined in #IUT_LPAd_CI, not based on the curves defined in SGP.22[2]. |
| CERT_S_SM_DP_TLS_INV_SIG | Invalid CERT.DP.TLS certificate of the S_SM-DP+ with an invalid signature with the same tag and length as a valid signature, correctly formatted as X.509 certificate. |
| CERT_S_SM_DPauth_SIG | Certificate of the S_SM-DP+ for its Public SIG key used for SM-DP+ authentication. This certificate contains the OID #S_SM_DP+_OID. |
| CERT_S_SM_DP2auth_SIG | Certificate of the S_SM-DP+ for its Public key used for SM-DP+ authentication. This certificate contains the OID #S_SM_DP+_OID2. |
| CERT_S_SM_DPauth_INV_SIGN | Invalid certificate of the S_SM-DP+ for its Public key used for authentication. This certificate contains the OID #S_SM_DP+_OID and contains an invalid signature (i.e. not generated with the #SK_CI_SIG but with the same tag and length as a valid signature) |
| CERT_S_SM_DPauth_INV_CURVE | Certificate of the S_SM-DP+ for its Public ECDSA key used for Authentication. This certificate contains the OID #S_SM_DP+_OID and a public key based on a curve different from the curves defined in SGP.22[2]. |
| CERT_S_SM_DSauth_INV_CURVE | Certificate of the S_SM-DS for its Public key used for Authentication. This certificate contains the OID #S_SM_DS_OID and a public key based on a curve different from the curves defined in SGP.22[2]. |
| CERT_S_SM_DPpb_SIG | Certificate of the S_SM-DP+ for its Public key used for Profile Package Binding. This certificate contains the OID #S_SM_DP+_OID. |
| CERT_S_SM_DPpb_INV_SIGN | Invalid certificate of the S_SM-DP+ for its Public key used for Profile Package Binding. This certificate contains the OID #S_SM_DP+_OID and contains an invalid signature (i.e. not generated with the #SK_CI_SIG but with the same tag and length as a valid signature) |
| CERT_S_SM_DPpb_INV_CURVE | Certificate of the S_SM-DP+ for its Public key used for Profile Package Binding. This certificate contains the OID #S_SM_DP+_OID and a public key based on a curve different from the curves defined in SGP.22[2]. |
| CERT_S_SM_DP2pb_SIG | Certificate of the S_SM-DP+ for its Public key used for Profile Package Binding. This certificate contains the OID #S_SM_DP+_OID2. |
| CERT_S_SM_DS_TLS | CERT.DS.TLS certificate of the S_SM-DS based on the same CI as defined in #IUT_LPAd_CI based on NIST or Brainpool for this version of the specification |
| CERT_S_SM_DS2_TLS | CERT.DS.TLS certificate of the S_SM-DS based on the same CI as defined in #IUT_LPAd_CI based on NIST or Brainpool for this version of the specification. Contains different SM-DS hostname (FQDN) as #CERT_S_SM_DS2_TLS. |
| CERT_S_SM_DS_TLS_EXPIRED | Expired CERT.DS.TLS certificate of the S_SM-DS with a valid signature, correctly formatted as X.509 certificate. |
| CERT_S_SM_DS_TLS_INV_CURVE | CERT.DP.TLS certificate of the S_SM-DP+, based on the different CI as defined in #IUT_LPAd_CI, not based on the curves defined in SGP.22[2]. |
| CERT_S_SM_DS_TLS_INV_SIG | Invalid CERT.DS.TLS certificate of the S_SM_DS with an invalid signature with the same tag and length as a valid signature, correctly formatted as X.509 certificate. |
| CERT_S_SM_DSauth_SIG | Certificate of the S_SM-DS for its Public key used for SM-DS authentication. This certificate contains the OID #S_SM_DS_OID. |
| CERT_S_SM_DSauth_INV_SIGN | Invalid certificate of the S_SM-DS for its Public key used for SM-DS authentication. This certificate contains an invalid signature, (i.e. not generated with the #SK_CI_SIG but with the same tag and length as a valid signature) |
| CERT_S_SM_DP_SubCAList_SIG (CertificateChain) | #CERT_S_SM_DP_SubCA_SIG CERT_CI_SubCA_SIG |
| CI_PKI_ID1 | The CI Subject Key Identifier as defined in SGP.26 [25]. |
| CI_PKI_ID2 | 0x21 22 23 24 25 26 27 28 29 2A 2B 2C 2D 2E 2F 30 31 32 33 |
| CRL_LIST | The list of CRLs needed to verify the revocation status of each Certificate that contains a cRLDistributionPoints extension in the returned Certificate chain. |
| PK_S_SM_DP_TLS | Public key of CERT.DP.TLS of the S_SM-DP+. |
| PK_S_SM_DPauth_SIG | Public Key of the S_SM-DP+, contained within #CERT_S_SM_DPauth_SIG |
| PK_S_SM_DPpb_SIG | Public Key of the S_SM-DP+, contained within #CERT_S_SM_DPpb_SIG |
| PK_S_SM_DS_TLS | Public key of CERT_S_DS_TLS of the S_SM-DS. |
| PK_SM_DPauth_SIG | Public Key of the SM-DP+, contained within #CERT_SM_DPauth_SIG |
| PK_SM_DPpb_SIG | Public Key of the SM-DP+, contained within #CERT_SM_DPpb_SIG |
| PK_SM_DSauth_SIG | Public Key of the SM-DS, contained within #CERT_SM_DSauth_SIG |
| SK_CI_SIG | Private Key of the CI |
| SK_EUICC_SIG | Private key of the eUICC for creating signatures |
| SK_S_SM_DPauth_SIG | Private Key of the of S_SM-DP+ for creating signatures for SM-DP+ authentication |
| SK_S_SM_DSauth_SIG | Private Key of the of S_SM-DS for creating signatures for SM-DS authentication |
| SK_S_SM_DPpb_SIG | Private key of the S_SM-DP+ used to provide signatures for Profile binding |

- Annex B	Dynamic Content

| Variable | Description |
| --- | --- |
| ANY_SW_IN_ERROR | Any Status Word in error (different from 0x9000) |
| BPP | Content of a Bound Profile Package to download within the eUICC. |
| BPP_SEG_A0 | Bound Profile Package TLV segment containing the tag and length fields of the firstSequenceOf87 TLV plus the first 0x87 TLV containing the ConfigureISDP command |
| BPP_SEG_A1 | Bound Profile Package following TLV segment array, as defined in SGP.22 [2] – section 2.5.5: - array first element containing the Tag and length fields of the sequenceOf88 TLV - array following elements containing each of the ‘88’ TLVs containing the StoreMetadata command |
| BPP_SEG_A2 | Bound Profile Package TLV segment containing the Tag and length fields of the secondSequenceOf87 TLV plus the first '87' TLV, containing the ReplaceSessionKeys command |
| BPP_SEG_A3 | Bound Profile Package following TLV segment array, as defined in SGP.22 [2] – section 2.5.5: - array first element containing the tag and length fields of the sequenceOf86 TLV - array following elements containing each of the '86' TLVs containing the Protected Profile Package (PPP) |
| BPP_SEG_INIT | Bound Profile Package TLV segment containing the tag and length fields of the BoundProfilePackage TLV plus the initialiseSecureChannelRequest command |
| C_APDUS_SCRIPT | List of Command APDUs formatted as an expanded structure with definite length coding as defined in ETSI TS 102 226 [14]. |
| CC | SCP80 cryptographic checksum as defined in ETSI TS 102 225 [13] (8 bytes long). |
| CHANNEL_NUMBER | The logical channel number newly opened in the eUICC. If no logical channel is opened, the value is set to 0x00 (i.e. Basic Channel). |
| CI_PKI_RANDOM | Random Subject Key Identifier of the PK CI different from all the PK CI Identifiers defined in SGP.26 [25]. This random value has the same length as the ones defined in SGP.26 [25]. |
| CLIENT_TLS_EPHEM_KEY | Client's ephemeral key and associated information. |
| CONF_ISDP_PROF1_ENC | An element of firstSequenceOf87, consisting of #CONF_ISDP_PROF1_SMDP protected with <S_ENC> and <S_MAC> and encapsulated in a TLV with tag 0x87, length <L> to a maximum size of 1020 bytes including the tag and length fields. |
| EUICC_CANCEL_SESSION_SIGNATURE | euiccCancelSessionSignature is created using the SK.EUICC.SIG signed over euiccCancelSessionSigned coded as ASN.1 OCTET STRING. |
| EUICC_CANCEL_SESSION_SIGNATURE_INVALID | eUICC signature randomly generated and coded as an ASN.1 OCTET STRING not equal to <EUICC_CANCEL_SESSION_SIGNATURE> but with the same length as a valid signature |
| EUICC_CHALLENGE | Random eUICC challenge, coded as asn.1 OCTET STRING, 16 bytes. |
| EUICC_CI_PK_ID_LIST_FOR_SIGNING | List of CI Public Key Identifiers supported on the eUICC for signature creation, coded as ASN.1 sequence of SubjectKeyIdentifier. The CI Public Key Identifier is from the list of possible CI Public Key Identifier. This possible CI Public Key Identifiers as supported by the eUICC will be defined later on. |
| EUICC_CI_PK_ID_LIST_FOR_SIGNING_V3 | List of CI Public Key Identifiers supported on the eUICC for signature creation that can be verified by a certificate chain Variant A, B or C, coded as ASN.1 sequence of SubjectKeyIdentifier. The CI Public Key Identifiers are from the list of possible identifiers as defined in SGP.26 [25]. |
| EUICC_CI_PK_ID_LIST_FOR_VERIFICATION | List of CI Public Key Identifiers supported on the eUICC for signature verification, coded as ASN.1 sequence of SubjectKeyIdentifier. The CI Public Key Identifier is from the list of possible CI Public Key Identifier. This possible CI Public Key Identifiers as supported by the eUICC will be defined later on. |
| EUICC_CI_PK_ID_TO_BE_USED | CI Public Key Identifier to be used by the eUICC for signature, coded as ASN.1 sequence of SubjectKeyIdentifier. |
| EUICC_CS_SIGNATURE | The eUICC cancel session signature computed using the #SK_EUICC_ECDSA across the EuiccCancelSessionSigned present in the CancelSessionResponse structure |
| EUICC_NEXT_CERT | Next Certificate in the eUICC Certificates Chain. eg : If Variant A Certificates are used it is the intermediate certificate CERT_EUM_SubCA_ECDSA |
| EUICC_OTHER_CERTS | Other Certificates in the eUICC Certificates Chain. eg : If Variant A Certificates are used it is the CERT_EUM_ECDSA. |
| EUICC_RSP_CAPABILITY | RspCapability of the eUICC, coded as ASN.1 BIT STRING (4 bits) to be used for indication of additionalProfile, loadcrlSupport, rpmSupport , testProfileSupport, deviceInfoExtensibilitySupport, osUpdateSupport, lpaApiSupport, enterpriseProfilesSupport, lpaProxySupport, crlStaplingV3Support, certChainV3VerificationSupport, serviceProviderMessageSupport, signedEventRecordsV3Support |
| EUICC_SIGN_PIR | The eUICC signature of the Profile Installation Result (PIR). The input data used to generate the <EUICC_SIGN_PIR> is the profileInstallationResultData TLV. |
| EUICC_SIGN_RPR | The eUICC signature of the Load RPM Package Result (RPR). The input data used to generate the <EUICC_SIGN_RPR> is the loadRpmPackageResultDataSigned and smdpSignature3. euiccSignRPR shall be created using the SK.EUICC.ECDSA and verified using the PK.EUICC.ECDSA. |
| EUICC_SIGNATURE1 | The eUICC signature 1 (euiccSignature1) computed using #SK_EUICC_ECDSA across the euiccSigned1 present in the AuthenticateServerResponse structure, coded as ASN.1 OCTET STRING. |
| EUICC_SIGNATURE1_INVALID | eUICC signature randomly generated and coded as an ASN.1 OCTET STRING not equal to <EUICC_SIGNATURE1> |
| EUICC_SIGNATURE2 | The eUICC signature 2 (euiccSignature2) computed using the #SK_EUICC_ECDSA across the following data objects: - euiccSigned2 - smdpSignature2 present in the PrepareDownloadRequest structure |
| EUICC_SIGNATURE2_INVALID | eUICC signature randomly generated and coded as an ASN.1 OCTET STRING not equal to <EUICC_SIGNATURE2> |
| EVENT_ID | An EventID value in String format, generated by the SM-DP+ during Event Record registration. |
| EVENT_ID_D | An EventID value in String format, generated by the Alternative SM-DS during cascaded Event Record deletion on ES15 |
| EVENT_ID_R | The EventID value in String format generated by the Alternative SM-DS during cascaded Event Record registration on ES15. |
| EXT_CARD_RESOURCE | Extended Card Resource Information according to ETSI TS 102 226 [14], coded as ASN.1 OCTET STRING. 'Number of installed application' value field is '00'. |
| EXT_SHA256_ECDSA | TLS extension data for "supported_signature_algorithms" set as a minimum of HashAlgorithm sha256 (04) and SignatureAlgorithm ecdsa (03). |
| FORWARDING_INDICATOR_ANY | Any boolean value (TRUE/FALSE) |
| FREE_MEM_OP_PROF_INSTALLED | Non-volatile memory (tag 0x82) available in the eUICC when two or more PROFILE_OPERATIONAL are installed |
| FREE_MEM_OP_PROF1_DELETED | Non-volatile memory (tag 0x82) available in the eUICC after PROFILE_OPERATIONAL1 deletion |
| FREE_MEM_OP_PROF1_INSTALLED | Non-volatile memory (tag 0x82) available in the eUICC when only PROFILE_OPERATIONAL1 is installed |
| FREE_MEMORY_NO_PROFILE | Non-volatile memory (tag 0x82) available in the eUICC when there is no Profile installed |
| FUNCTION_CALL_ID | The function call ID generated by the entity that calls the function |
| FUNCTION_REQ_ID | The function requester ID |
| INVALID_SM_DP_OID | SM-DP+ OID (as defined in section 1.3) not equal to #IUT_SM_DP_OID |
| INVALID_TRANSACTION_ID | A Transaction Identifier generated by the S_SM-DP+ or the S_SM-DS that SHALL be different from <S_TRANSACTION_ID> if exists. Otherwise, a random value is generated. |
| ISD_P_AID | The ISD-P AID newly created in the eUICC. This AID value is in the range from 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 00 10 00 to 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 FF FF 00. Last byte is set to '00' as defined in SGP.02[1]. |
| ISD_P_AID1 | The ISD-P AID created in the eUICC for the PROFILE_OPERATIONAL1. This AID value belongs to the range from 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 00 10 00 to 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 FF FF 00. Last byte is set to '00' as defined in SGP.02[1]. |
| ISD_P_AID2 | The ISD-P AID created in the eUICC for the PROFILE_OPERATIONAL2. This AID value belongs to the range from 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 00 10 00 to 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 FF FF 00. Last byte is set to '00' as defined in SGP.02[1]. |
| ISD_P_AID3 | The ISD-P AID created in the eUICC for the PROFILE_OPERATIONAL3. This AID value belongs to the range from 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 00 10 00 to 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 FF FF 00. Last byte is set to '00' as defined in SGP.02[1]. |
| ISD_P_AID4 | The ISD-P AID created in the eUICC for the PROFILE_OPERATIONAL4. This AID value belongs to the range from 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 00 10 00 to 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 FF FF 00. Last byte is set to '00' as defined in SGP.02[1]. |
| ISD_P_AIDX | An invalid ISD-P AID not present on the eUICC. This AID value is in the range from 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 00 10 00 to 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 FF FF 00. |
| L | Exact length of the corresponding tag or of the remaining data. |
| LPA_RSP_CAPABILITY | LpaRspCapability, coded as ASN.1 BIT STRING to be used for indication of RSP capabilities of the Device. |
| MATCHING_ID | Unique identifier as defined in [2]. The content can be either empty, or the value of the EventID, or the value of the Activation Code token. |
| MATCHING_ID_EVENT | A Unique identifier of an Event for a specific EID generated by the SM-DP+ / SM-DS. |
| METADATA_OP_PROF1_SEG | The #METADATA_OP_PROF1 is mac-ed with <S_MAC> and split as necessary into segments of a maximum size of 1020 bytes (including the tag, length field, and MAC), |
| MNO_SCP80_COUNTER | SCP80 counter of the MNO-SD related to the KVN 0x01 (5 bytes long). Initial value is set to 0x00 00 00 00 01 and is incremented by one each time a secured packet is sent. |
| NB_EXECUTED_C_APDUS | Number of executed Command TLV objects as defined in ETSI TS 102 226 [14]. |
| NOTIF_SEQ_NO_DE1 | The Sequence Number of the Delete Notification related to the PROFILE_OPERATIONAL1. |
| NOTIF_SEQ_NO_DI1 | The Sequence Number of the Disable Notification related to the PROFILE_OPERATIONAL1. |
| NOTIF_SEQ_NO_EN1 | The Sequence Number of the Enable Notification related to the PROFILE_OPERATIONAL1. |
| NOTIF_SEQ_NO_EN2 | The Sequence Number of the Enable Notification related to the PROFILE_OPERATIONAL2. |
| NOTIF_SEQ_NO_IN1 | The Sequence Number of the Install Notification related to the PROFILE_OPERATIONAL1. |
| NOTIF_SEQ_NO_IN1_PIR | The Sequence Number of the Install Notification (PIR) related to the PROFILE_OPERATIONAL1. |
| NOTIF_SEQ_NO_IN2 | The Sequence Number of the Install Notification related to the PROFILE_OPERATIONAL2. |
| NOTIF_SEQ_NO_IN2_PIR | The Sequence Number of the Install Notification (PIR) related to the PROFILE_OPERATIONAL2. |
| NOTIF_SEQ_NO_PROF1_RPR | The Sequence Number of the Notification RPM Package Result related to the PROFILE_OPERATIONAL1. |
| NOTIF_SEQ_NO2_DE1 | The Sequence Number of the second Delete Notification related to the PROFILE_OPERATIONAL1. |
| NOTIF_SEQ_NO2_DI1 | The Sequence Number of the second Disable Notification related to the PROFILE_OPERATIONAL1. |
| NOTIF_SEQ_NO2_EN1 | The Sequence Number of the second Enable Notification related to the PROFILE_OPERATIONAL1. |
| OTPK_EUICC_ECKA | One-time Public Key generated by the eUICC for ECKA. Depending on the eUICC configuration, this key is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| OTPK_EUICC_AKA_NEW | One-time Public Key of the eUICC for ECKA used for the BPP which is a new generated value different from <OTPK_EUICC_ECKA> |
| PPK_ENC | Random PPK-ENC value (16 bytes key length). This value is different from <S_ENC> value. |
| PPK_INIT_MAC | Random initial MAC chaining value (16 bytes). This value is different from the <S_MAC_CHAIN> value. |
| PPK_MAC | Random PPK-MAC value (16 bytes key length). This value is different from <S_MAC> value. |
| PPP_OP_PROF1_SEG_PPK | An element of sequenceOf86, consisting of a <UPP_OP_PROF1_SEG> protected with <PPK_ENC> and <PPK_MAC> and encapsulated in a TLV with tag 0x86 length <L>, up to a maximum size of 1020 bytes including the tag and length field. |
| PPP_OP_PROF1_SEG_SK | An element of sequenceOf86, consisting of a <UPP_OP_PROF1_SEG> segment protected with <S_ENC> and <S_MAC> and encapsulated in a TLV with tag 0x86, length <L>, up to a maximum size of 1020 bytes including the tag and length field. |
| PPP_OP_PROF1_SEG_SK_INV | <PPP_OP_PROF1_SEG_SK> modified (wrong encryption) |
| PPR_IDS | Forbidden Profile Policy Rules. This PPR list MAY be empty or MAY contain either PPR1 or PPR2 or both. |
| PROPRIETARY_DATA | Proprietary Data returned by the eUICC as part of FCI template |
| R_APDU_PARTx | Sub-part of a R-APDU (see Annex D.4.2) |
| R_EUICC_INFO1 | The eUICC information EUICCInfo1 coded as an ASN.1 SEQUENCE, as defined in SGP.22 v3.1[2]. |
| R_EUICC_INFO2 | The eUICC information EUICCInfo2 coded as an ASN.1 SEQUENCE, as defined in SGP.22 v3.1[2]. |
| RANDOM_SM_DP+_SIGN | Random SM-DP+ signature (i.e. content of the tag 0x5F37) with a size corresponding to a valid one. |
| RANDOM_SM_DS_SIGN | Random SM-DS signature (i.e. content of the tag 0x5F37) with a size corresponding to a valid one. |
| REASON_CODE_ANY | Any Reason Code, as defined in SGP.22 [2] – section 5.2.6.2 |
| REPLACE_S_KEYS_REQ_ENC | An element of secondSequenceOf87, consisting of #REPLACE_S_KEYS_REQ protected with <S_ENC> and <S_MAC> and encapsulated in a TLV with tag 0x87, up to a maximum size of 1020 bytes including the tag and length field. |
| RSP_SERVER_ADDRESS | RSP Server address in FQDN format where the operation corresponding to the Event can be processed. |
| S_ENC | SCP03T Encryption Session key (128 bits length) resulting from the key agreement with eUICC. |
| S_HASHED_CC | Hashed Confirmation Code generated by the LPA. When generated by the S_LPAd, the S_LPAd SHALL use #CONFIRMATION_CODE1 in the calculation unless otherwise specified. |
| S_HASHED_CC_ERROR | A random generated hash value of the Confirmation Code not equal to S_HASHED_CC. |
| S_INIT_MAC | SCP03T Initial MAC chaining value (128 bits length) resulting from the key agreement with eUICC. |
| S_MAC | SCP03T MACing Session key (128 bits length) resulting from the key agreement with eUICC. |
| S_MAC_CHAIN | Current MAC chaining value used for SCP03t BPP protection. |
| S_SEL_TLS_CIPHER_SUITE | TLS cipher suite selected by the Server set as follows: o TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 if present in <TLS_CIPHER_SUITES>, otherwise o TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256. |
| S_SERVER_RSP_CAPABILITY_NONE | RspCapability of the S_SERVER is coded as ASN.1 BIT STRING (2 bits) to be used for indication of crlStaplingV3Support, eventListSigningV3Support and set this as no bits are set (NONE). |
| S_SESSION_CONTEXT | { serverSvn #S_SM_DP_SVN, crlStaplingV3Used FALSE } |
| S_SESSION_ID_SERVER | Random value of the TLS session_id in ServerHello which is different from <SESSION_ID_CLIENT>. This value is non-empty. |
| S_SM_DP+_SIGN | The S_SM-DP+ signature (smdpSign), computed using the #SK_S_SM_DPpb_ECDSA across the following data objects: - remoteOpId - transactionId - controlRefTemplate - smdpOtpk - euiccOtpk, as provided earlier in the prepareDownloadResponse data object |
| S_SM_DP+_SIGNATURE2 | The ASN.1 OCTET STRING encoded SM-DP+ signature 2 (field smdpSignature2) computed using the private key related to the server certificate (field smdpCertificate) present in the PrepareDownloadRequest structure. This signature SHALL be generated across the following data objects: - smdpSigned2 - euiccSignature1 present in the AuthenticateServerResponse structure |
| S_SM_DP+_SIGNATURE3 | The ASN.1 OCTET STRING encoded SM-DP+ signature 3 (field smdpSignature3) computed using the private key (Compute the smdpSignature3 over the concatenated data object of smdpSigned3 and euiccSignature1 using the SK.DPauth.SIG related to the server certificate (field smdpCertificate) present in the InitiateAuthenticationOk Response structure. This signature SHALL be generated across the following data objects: - smdpSigned3 euiccSignature1 present in the AuthenticateServerResponse structure |
| S_SMDP_CHALLENGE | The SM-DP+ Challenge (serverChallenge) randomly chosen by the simulated SM-DP+ to be signed later by the eUICC for the eUICC authentication, coded as ASN.1 OCTET STRING of 16 bytes. |
| S_SMDP_SIGNATURE_INV | <S_SMDP_SIGNATURE1> NOT computed with the #SK_S_SM_DPauth_ECDSA but with the same length as a valid signature |
| S_SMDP_SIGNATURE1 | The ASN.1 OCTET STRING encoded SM-DP+ signature (field serverSignature1) computed using the private key related to the server certificate (field serverCertificate) present in the AuthenticateServerRequest structure. |
| S_SMDP_SIGNED_INV_ADDR | <S_SMDP_SIGNED1> with a different SM-DP+ address (#TEST_DP_ADDRESS2 instead of #TEST_DP_ADDRESS1) |
| S_SMDP_SIGNED1 (ServerSigned1) | { transactionId <S_TRANSACTION_ID>, euiccChallenge <EUICC_CHALLENGE>, serverAddress #TEST_DP_ADDRESS1, serverChallenge <S_SMDP_CHALLENGE>, sessionContext { serverSvn #IUT_RSP_VERSION_HIGHEST, crlStaplingV3Used TRUE }, serverRspCapability { crlStaplingV3Support, cancelForEmptySpnPnSupport } } |
| S_SMDP_SIGNED1_V3 (ServerSigned1) | { transactionId <S_TRANSACTION_ID>, euiccChallenge <EUICC_CHALLENGE>, serverAddress #TEST_DP_ADDRESS1, serverChallenge <S_SMDP_CHALLENGE>, sessionContext { serverSvn #IUT_RSP_VERSION_HIGHEST, crlStaplingV3Used TRUE, euiccCiPKIdToBeUsedV3 <EUICC_CI_PK_ID_TO_BE_USED_V3> }, serverRspCapability { crlStaplingV3Support, cancelForEmptySpnPnSupport } } NOTE : select the CI Key ID in highest priority from the <EUICC_CI_PK_ID_LIST_FOR_SIGNING_V3> |
| S_SMDP_SIGNED2_OLD_KEYS | smdpSigned2 SmdpSigned2 ::= { transactionId <S_TRANSACTION_ID>, ccRequiredFlag FALSE, bppEuiccOtpk <OTPK_EUICC_ECKA> } |
| S_SMDS_CHALLENGE | The SM-DS Challenge (serverChallenge) randomly chosen by the simulated SM-DS to be signed later by the eUICC for the eUICC authentication, coded as ASN.1 OCTET STRING of 16 bytes. |
| S_SMDS_SIGNATURE_INV | <S_SMDS_SIGNATURE1> NOT computed with the #SK_S_SM_DSauth_ECDSA but with the same length as a valid signature |
| S_SMDS_SIGNATURE1 | The SM-DS signature 1 (serverSignature1) computed using #SK_S_SM_DSauth_ECDSA across the serverSigned1 present in the AuthenticateServerRequest structure, coded as ASN.1 OCTET STRING |
| S_SMDS_SIGNED_ADDR1 (ServerSigned1) | { transactionId <S_TRANSACTION_ID>, euiccChallenge <EUICC_CHALLENGE>, serverAddress #TEST_DS_ADDRESS1, serverChallenge <S_SMDS_CHALLENGE>, sessionContext { serverSvn #IUT_RSP_VERSION_HIGHEST, crlStaplingV3Used TRUE }, serverRspCapability { crlStaplingV3Support, eventListSigningV3Support } } |
| S_SMDS_SIGNED_INV_ADDR | <S_SMDS_SIGNED1> with a different SM-DS address (#TEST_DP_ADDRESS1 instead of #TEST_ROOT_DS_ADDRESS) |
| S_SMDS_SIGNED1 (ServerSigned1) | { transactionId <S_TRANSACTION_ID>, euiccChallenge <EUICC_CHALLENGE>, serverAddress #TEST_ROOT_DS_ADDRESS, serverChallenge <S_SMDS_CHALLENGE>, sessionContext { serverSvn #IUT_RSP_VERSION_HIGHEST, crlStaplingV3Used TRUE }, serverRspCapability { crlStaplingV3Support, eventListSigningV3Support } } |
| S_SMDS_SIGNED1_V3 (ServerSigned1) | { transactionId <S_TRANSACTION_ID>, euiccChallenge <EUICC_CHALLENGE>, serverAddress #TEST_ROOT_DS_ADDRESS, serverChallenge <S_SMDS_CHALLENGE>, sessionContext { serverSvn #IUT_RSP_VERSION_HIGHEST, crlStaplingV3Used TRUE, euiccCiPKIdToBeUsedV3 <EUICC_CI_PK_ID_TO_BE_USED_V3> }, serverRspCapability { crlStaplingV3Support, eventListSigningV3Support } } NOTE : select the CI Key ID in highest priority from the <EUICC_CI_PK_ID_LIST_FOR_SIGNING_V3> |
| S_TRANSACTION_ID | The TransactionID (Unique Transaction Identifier) generated by the (S_)SM-DP+, or (S_)SM-DS which is used to uniquely identify the RSP session and to correlate the multiple ESXX request messages that belong to the same RSP session. This value (binary value) can start from 0x01 and can be increased by 1 each time a Profile is downloaded in the eUICC. 1-16 bytes (ASN.1 OCTET STRING). |
| SAH_SHA256_ECDSA | Signature And Hash Algorithm extension sent in the CertificateRequest message set as a minimum of: HashAlgorithm sha256 (04) and SignatureAlgorithm ecdsa (03). |
| SEL_TLS_CIPHER_SUITE | TLS cipher suite selected by the Server |
| SEQ_NUMBER | Sequence Number related to a Notification Metadata generated by the eUICC. |
| SERVER_CHALLENGE | Random value generated by the SM-XX server under test coded as ASN.1 OCTET STRING of 16 bytes which can be one of the following depending on the entity under test: - <SMDP_CHALLENGE> - <SMDS_CHALLENGE> |
| SERVER_CHALLENGE_2 | Random value generated by the SM-XX server under test coded as ASN.1 OCTET STRING of 16 bytes which can be one of the following depending on the entity under test: - <SMDP_CHALLENGE_2> - <SMDS_CHALLENGE_2> |
| SERVER_FINISHED | The first protected message with the negotiated algorithms, keys, and secrets. It is the Hash of the concatenation of all the data from all messages in this handshake up to, but not including, this message i.e. all handshake messages starting at ClientHello up to, but not including, this Finished message itself. NOTE: ChangeCipherSpec messages, alerts, and any other record type are not handshake messages and are not included in the hash computations. Also, HelloRequest messages are omitted from handshake hashes. |
| SERVER_NAME_V3 | The v3-specific FQDN SHALL be the concatenation of the string "rsp3-" and the "base" UTF-8 encoded FQDN. v3-specific FQDN is computed from the "base" FQDN known from the RSP Server (e.g. an address retrieved from the eUICC, an address read from an Activation code) |
| SERVER_SIGNATURE1 | Server signature (serverSignature1) which can be one of the following depending on the entity under test: - SM-DP+ signature (serverSignature1) generated over #SERVER_SIGNED1 using SK.DPauth.ECDSA, coded as ASN.1 OCTET STRING - SM-DS signature (serverSignature1) generated over #SERVER_SIGNED1 using SK.DSauth.ECDSA, coded as ASN.1 OCTET STRING |
| SERVER_SIGNATURE1_2 | SERVER signature (serverSignature1) which can be one of the following depending on the entity under test: - SM-DP signature (serverSignature1) generated over #SERVER_SIGNED1_2 using SK.DPauth.ECDSA, coded as ASN.1 OCTET STRING - SM-DS signature (serverSignature1) generated over #SERVER_SIGNED1_2 using SK.DSauth.ECDSA, coded as ASN.1 OCTET STRING |
| SERVER_TLS_EPHEM_KEY | Server's ephemeral key and associated information. |
| SESSION_ID_CLIENT | Random or empty value of the TLS session_id in ClientHello. |
| SESSION_ID_RANDOM | Random value of the TLS session. |
| SHS | Shared Secret resulting from the key agreement with eUICC. |
| SM_DP+_SIGN | The SM-DP+ signature in ES8+/InitialiseSecureChannelRequest/smdpSign. |
| SMDP_CHALLENGE | Random value generated by the SM-DP+ coded as ASN.1 OCTET STRING of 16 bytes. |
| SMDP_CHALLENGE_2 | Random value generated by the SM-DP+ coded as ASN.1 OCTET STRING of 16 bytes. |
| SMDP_CHALLENGE_INVALID | SM-DP+ Challenge randomly generated by the simulated SM-DP+ coded as ASN.1 OCTET STRING of 16 bytes not equal to <SMDP_CHALLENGE>. |
| SMDP_METADATA_SEG_MAC | An element of sequenceOf88, consisting of a segment of maximum size 1008 bytes protected with <S_MAC> and encapsulated in a TLV with tag 0x88, length <L>, up to a maximum size of 1020 bytes including the tag and length field. |
| SMDP_SIGNATURE2 | SM-DP+ signature (smdpSignature2) generated over smdpSigned2 using SK.DPauth.ECDSA, coded as ASN.1 OCTET STRING |
| SMDS_CHALLENGE | Random value generated by the SM-DS coded as ASN.1 OCTET STRING of 16 bytes. |
| SMDS_CHALLENGE_2 | Random value generated by the SM-DS coded as ASN.1 OCTET STRING of 16 bytes. |
| SMDS_CHALLENGE_INVALID | SM-DS Challenge randomly generated by the simulated SM-DS coded as ASN.1 OCTET STRING of 16 bytes not equal to <SMDS_CHALLENGE>. |
| STORE_DATA_BLOCK_NUM | The STORE DATA block number coded sequentially from 0x00 to 0xFF. If the value 0xFF has been reached and more STORE DATA commands are needed to complete the transfer, the numbering restarts and the next STORE DATA block number is set to 0x00. |
| SUBJECT_CODE_ANY | Any Subject Code, as defined in SGP.22 [2] – section 5.2.6.1 |
| TBS_EUICC_NOTIF_SIG | The eUICC signature generated over tbsOtherNotification. NotificationMetadata, coded as ASN.1 OCTET STRING. |
| TLS_CIPHER_SUITES | TLS cipher suite list supported by LPAd or the Client (SM-DP+ or SM-DS) under test. |
| TRANSACTION_ID_2 | A unique Transaction ID generated by an SM-DP+ or an SM-DS within the scope and lifetime of each SM-DP+ or SM-DS to uniquely identify the ongoing RSP session as OCTET STRING of up to 16 bytes. |
| TRANSACTION_ID_AC | A unique Transaction ID generated by an SM-DP+ or an SM-DS within the scope and lifetime of each SM-DP+ or SM-DS to uniquely identify the ongoing RSP session used by the AuthenticateClient function as OCTET STRING of up to 16 bytes. |
| TRANSACTION_ID_GBPP | A unique Transaction ID generated by an SM-DP+ within the scope and lifetime of each SM-DP+ to uniquely identify the ongoing RSP session used by the GetBoundProfilePackage function as OCTET STRING of up to 16 bytes. |
| TRANSACTION_ID_IA | A unique Transaction ID generated by an SM-DP+ or an SM-DS within the scope and lifetime of each SM-DP+ or an SM-DS to uniquely identify the ongoing RSP session used by the InitiateAuthentication function as OCTET STRING of up to 16 bytes. |
| TRANSACTION_ID_ISC | A unique Transaction ID generated by an SM-DP+ within the scope and lifetime of each SM-DP+ to uniquely identify the ongoing RSP session used by the InitialiseSecureChannelRequest function as OCTET STRING of up to 16 bytes. |
| TRANSACTION_ID_SIGNED | A unique Transaction ID generated by an SM-DP+ or an SM-DS within the scope and lifetime of each SM-DP+ or SM-DS to uniquely identify the ongoing RSP session as OCTET STRING of up to 16 bytes signed as part of #SERVER_SIGNED1 |
| TRANSACTION_ID_SIGNED_2 | A unique Transaction ID generated by an SM-DP+ or an SM-DS within the scope and lifetime of each SM-DP+ or SM-DS to uniquely identify the ongoing RSP session as OCTET STRING of up to 16 bytes signed as part of #SERVER_SIGNED1 |
| TRANSACTION_ID_SIGNED_AC | A unique Transaction ID generated by an SM-DP+ or an SM-DS within the scope and lifetime of each SM-DP+ or SM-DS to uniquely identify the ongoing RSP session used by the AuthenticateClient function as OCTET STRING of up to 16 bytes. |
| TRANSACTION_ID_SIGNED_IA | A unique Transaction ID generated by an SM-DP+ or an SM-SD within the scope and lifetime of each SM-DP+ or SM-DS to uniquely identify the ongoing RSP session used by the InitiateAuthentication function as OCTET STRING of up to 16 bytes. |
| TRE_PROPERTIES | The value of the treProperties field in EUICCInfo2. |
| TRE_REFERENCE | The value of the treProductReference field in EUICCInfo2. |
| UPP_OP_PROF1_SEG | A segment of the #UPP_OP_PROF1, with a maximum size of 1007 bytes. |
| UPP_OP_PROF2_SEG | A segment of the #UPP_OP_PROF2, with a maximum size of 1007 bytes. |

- Annex C	Methods And Procedures

This section describes methods and procedures used in the interfaces compliance test cases. They are part of test cases and SHALL not be executed in standalone mode.

- C.1	Methods

If the method is used in the “expected result” column, all parameters SHALL be verified by the simulated entity (test tool). If the method is used in the “Sequence / Description” column, the command SHALL be generated by the simulated entity.

| Method | MTD_AUTH_CLIENT_RPM_PKG_REQ_FOR_SINGLE_CMND |
| --- | --- |
| Description | This Method can be used for creating Authenticate Client response with LoadRpmPackageRequest containing an rpmPackage with a single RPM command. |
| Parameter(s) | -  paramComandDetailsChoice: The rpmCommandDetails choice (Mandatory) -  paramTransactionId: The Transaction Id (Mandatory) -  paramIccidValue: The ICCID within RPM Command (Mandatory) -  paramSmdpSignature3: The SM-DP+ Signature3 (Mandatory) -  paramUpdateMetadataRequest: The UpdateMetadataRequest if required -  paramDpiRpmValue: The dpiRpm within RPM Command if required -  paramRpmPending if required Parameters paramUpdateMetadataRequest and paramDpiRpmValue shall be passed and can be empty depend on the paramComandDetailsChoice. |
| Details | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : paramTransactionId, "smdpSigned3" : { transactionId paramTransactionId, rpmPackage { MTD_RPM_PKG_REQ_FOR_SINGLE_CMND( paramComandDetailsChoice, paramIccidValue, paramUpdateMetadataRequest, paramDpiRpmValue, paramRpmPending ) } - }, - "smdpSignature3" : paramSmdpSignature3 } |

| Method | MTD_AUTH_CLIENT_RPM_PKG_REQ_FOR_SINGLE_CMND_LIST_PROFILE_INFO |
| --- | --- |
| Description | This Method can be used for creating Authenticate Client response with LoadRpmPackageRequest containing an rpmPackage with a single RPM command ListProfileInfo and Search Criteria as ICCID. |
| Parameter(s) | - paramTransactionId: The Transaction Id (Mandatory) -  paramIccidValue: The ICCID within RPM Command (Mandatory) -  paramSmdpSignature3: The SM-DP+ Signature3 (Mandatory) -  paramProfileOwnerOidValue: The ProfileOwnerOid for seachCriteria in ListProfileInfo if required - paramTagListValue: The TagList if required - Parameters paramTagListValue and paramProfileOwnerOidValue shall be passed and can be empty. |
| Details | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : paramTransactionId, "smdpSigned3" : { transactionId paramTransactionId, rpmPackage { MTD_RPM_PKG_REQ_FOR_SINGLE_CMND_LIST_PROFILE_INFO( paramIccidValue, paramTagListValue, paramProfileOwnerOidValue ) } }, - "smdpSignature3" : paramSmdpSignature3 } |

| Method | MTD_AUTHENTICATE_CLIENT |
| --- | --- |
| Description | Generates or verifies the JSON formatted AuthenticateClient request |
| Parameter(s) | - paramTransactionId: random 16 byte identifier encoded as String Hexadecimal. - paramAuthenticateServerResponse: server authentication response structured as ASN.1 encoded as base 64. |
| Details | JSON body { "transactionId" : paramTransactionId, "authenticateServerResponse" : paramAuthenticateServerResponse } |

| Method | MTD_CANCEL_ORDER |
| --- | --- |
| Description | Sends and checks the JSON formatted CancelOrder request |
| Parameter(s) | -  paramFunctionRequesterId -  paramFunctionCallId -  paramIccid: identification of the targeted profile (mandatory) -  paramEID: EID of the targeted eUICC (conditional) -  paramMatchingId: matching ID generated by the Operator (conditional) -  paramProfileStatus: final Profile status indicator (mandatory) |
| Details | JSON requestHeader { "header" : { "functionRequesterIdentifier" : "paramFunctionRequesterId", "functionCallIdentifier" : "paramFunctionCallId" } JSON body { "iccid" : paramIccid "eid" : paramEID, "matchingId" : paramMatchingId "profileStatus" : paramProfileStatus } } Note: if some of the value of the parameters above are not provided, those parameters are not included as part of JSON body |

| Method | MTD_CANCEL_SESSION |
| --- | --- |
| Description | Sends or verifies the JSON formatted CancelSession request |
| Parameter(s) |  paramTransactionId: random 16 byte identifier.  paramCancelSessionResponse: eUICC information structured as ASN.1 encoded as base 64. |
| Details | JSON body { "transactionId" : paramTransactionId, "cancelSessionResponse" : paramCancelSessionResponse } |

| Method | MTD_CHECK_SMS_POR |
| --- | --- |
| Description | Check the content of the SMS POR containing the response of the ES6.UpdateMetadata request |
| Parameter(s) | paramExpectedSW: the expected Status Word of the last STORE DATA command |
| Details | Parse and retrieve the SCP80 response packet from the SMS. SCP80 response status code SHALL be equal to 0x00 – POR OK. The additional data from the response packet SHALL be formatted as an expanded structure with definite length as defined in ETSI TS 102 226 [14] and contains the following TLV: AB <L> 80 <L> <NB_EXECUTED_C_APDUS> -- Number of executed C-APDUs 23 <L> 00 90 00 –- R-APDU of the INSTALL FOR PERSONALIZATION command 23 <L> paramExpectedSW –- SW of the last STORE DATA command executed <NB_EXECUTED_C_APDUS> SHALL be equal to the number of executed C-APDUs (i.e. one INSTALL FOR PERSONALIZATION + n STORE DATA command(s)) |

| Method | MTD_ CONFIRM_ORDER |
| --- | --- |
| Description | Sends and checks the JSON formatted ConfirmOrder request |
| Parameter(s) | -  paramFunctionRequesterId -  paramFunctionCallId -  paramIccid: identification of the targeted profile (mandatory) -  paramEID: EID of the targeted eUICC (conditional) -  paramMatchingId: matching ID generated by the Operator (optional) -  paramConfirmationCode: confirmation code provided by the Operator (optional) -  paramSmdsAddress: SM-DS to be used for event registration (conditional) -  paramReleaseFlag: boolean indicating if the profile shall be released (mandatory) |
| Details | JSON requestHeader { "header" : { "functionRequesterIdentifier" : "paramFunctionRequesterId", "functionCallIdentifier" : "paramFunctionCallId" } JSON body { "iccid" : paramIccid "eid" : paramEID, "matchingId" : paramMatchingId "confirmationCode" : paramConfirmationCode "smdsAddress" : paramSmdsAddress "releaseFlag" : paramReleaseFlag } } Note: if some of the value of the parameters above are not provided, those parameters are not included as part of JSON body |

| Method | MTD_DELETE_EVENT |
| --- | --- |
| Description | Sends and checks the JSON formatted DeleteEvent request |
| Parameter(s) | -  paramFunctionRequesterId: identification of the function requester. -  paramFunctionCallId: identification of the function call. -  paramEID: EID of the targeted eUICC -  paramEventId: unique Identification of the Event to be registered |
| Details | JSON requestHeader { "header" : { "functionRequesterIdentifier" : "paramFunctionRequesterId", "functionCallIdentifier" : "paramFunctionCallId" } JSON body { "eid" : paramEID, "eventId" : paramEventId } } |

| Method | MTD_DISABLE_PROFILE |
| --- | --- |
| Description | Generate the ASN.1 DisableProfileRequest structure according to the input parameters. |
| Parameter(s) | -  paramIccidValue: The ICCID of the Profile to Disable (optional) -  paramIsdpAidValue: The ISD-P AID of the Profile to Disable (optional) -  paramRefreshFlag: Boolean, TRUE if refreshFlag SHALL be set, FALSE otherwise Either paramIccidValue or paramIsdpAidValue is passed as a parameter. |
| Details | IF paramIccidValue is provided Then req DisableProfileRequest::= { profileIdentifier iccid : paramIccidValue, refreshFlag paramRefreshFlag } Else req DisableProfileRequest::= { profileIdentifier isdpAid : paramIsdpAidValue, refreshFlag paramRefreshFlag } End if |

| Method | MTD_ DOWNLOAD_ORDER |
| --- | --- |
| Description | Sends and checks the JSON formatted DownloadOrder request |
| Parameter(s) | -  paramFunctionRequesterId -  paramFunctionCallId -  paramEID: EID of the targeted eUICCoptional -  paramIccid: identification of the targeted profile (conditional) -  paramProfileType: identification of the targeted profile type (conditional) |
| Details | JSON requestHeader { "header" : { "functionRequesterIdentifier" :"paramFunctionRequesterID", "functionCallIdentifier" :"paramFunctionCallID" } JSON body { "eid" : paramEID, "iccid" : paramIccid "profileType" : paramProfileType } } Note: if some of the value of the parameters above are not provided, those parameters are not included as part of JSON body |

| Method | MTD_ENABLE_PROFILE |
| --- | --- |
| Description | Generate the ASN.1 EnableProfileRequest structure according to the input parameters. |
| Parameter(s) | -  paramIccidValue: The ICCID of the Profile to Disable (optional) -  paramIsdpAidValue: The ISD-P AID of the Profile to Disable (optional) -  paramRefreshFlag: Boolean, TRUE if refreshFlag SHALL be set, FALSE otherwise Either paramIccidValue or paramIsdpAidValue is passed as a parameter. |
| Details | IF paramIccidValue is provided Then req EnableProfileRequest ::= { profileIdentifier iccid : paramIccidValue, refreshFlag paramRefreshFlag } Else req EnableProfileRequest ::= { profileIdentifier isdpAid : paramIsdpAidValue, refreshFlag paramRefreshFlag } End if |

| Method | MTD_DELETE_PROFILE |
| --- | --- |
| Description | Generate the ASN.1 DeleteProfileRequest structure according to the input parameters. |
| Parameter(s) | -  paramIccidValue: The ICCID of the Profile to Delete (optional) -  paramIsdpAidValue: The ISD-P AID of the Profile to Delete (optional) Either paramIccidValue or paramIsdpAidValue is passed as a parameter. |
| Details | IF paramIccidValue is provided Then req DeleteProfileRequest ::= iccid : paramIccidValue Else req DeleteProfileRequest ::= isdpAid : paramIsdpAidValue End if |

| Method | MTD_GET_PROFILE_INFO |
| --- | --- |
| Description | Generate the ASN.1 ProfileInfoListRequest according to the input parameters. |
| Parameter(s) |  paramIccidValue: The ICCID of the Profile  paramIsdpAidValue: The ISD-P AID of the Profile Either paramIccidValue or paramIsdpAidValue is passed as a parameter. |
| Details | IF paramIccidValue is provided Then req ProfileInfoListRequest::= { searchCriteria iccid: paramIccidValue } Else req ProfileInfoListRequest::= { searchCriteria isdpAid: paramIsdpAidValue } End If |

| Method | MTD_GENERATE_HASHED_CC |
| --- | --- |
| Description | Generate an Hashed Confirmation Code based on the Confirmation Code and the Transaction ID given in parameter. |
| Parameter(s) |  paramConfirmationCode: The Confirmation Code (plain)  paramTransactionId: The Transaction ID (plain) |
| Details | Generate a SHA-256 of the paramConfirmationCode. Concatenate the obtained hash value with the paramTransactionId. Generate and return a SHA-256 of these two concatenated elements. |

| Method | MTD_GET_BPP |
| --- | --- |
| Description | Generates or verifies the JSON formatted GetBoundProfilePackage request |
| Parameter(s) |  paramTransactionId: random 16 byte identifier.  paramPrepareDownloadResponse structured as ASN.1 encoded as base 64. |
| Details | JSON body { "transactionId" : paramTransactionId, "prepareDownloadResponse" : paramPrepareDownloadResponse } |

| Method | MTD_HANDLE_NOTIF |
| --- | --- |
| Description | Generates or verifies the JSON formatted HandleNotification request |
| Parameter(s) | paramPendingNotification: PendingNotification data object |
| Details | JSON body { "pendingNotification" : paramPendingNotification } |

| Method | MTD_HTTP_REQ |
| --- | --- |
| Description | Sends or verifies a secured HTTP request message delivering a JSON object payload using a network to an off-card entity. |
| Parameter(s) | - paramServerAddress: Target Server address - paramFunctionPath: Function path - paramRequestMessage: JSON Request message |
| Details | HTTP POST paramFunctionPath HTTP/1.1 Host: paramServerAddress User-Agent: See NOTE 1 X-Admin-Protocol:gsma/rsp/v<2.1.0> Content-Type: application/json;charset=UTF-8 Content-Length: <L> paramRequestMessage NOTE 1: The "User-Agent" field may contain additional information after a semicolon. The additional information shall not be checked. The HTTP POST request may contain additional header fields. These shall not be checked. |

| Method | MTD_HTTP_RESP |
| --- | --- |
| Description | Sends or verifies a secured HTTP response message delivering a JSON object payload using a network to an off-card entity. |
| Parameter(s) |  paramResponseMessage: JSON Response message |
| Details | HTTP/1.1 200 (OK) X-Admin-Protocol: gsma/rsp/v<2.1.0> Content-Type: application/json;charset=UTF-8 Content-Length: <L> paramResponseMessage The HTTP response may contain additional header fields. These shall not be checked. |

| Method | MTD_INITIATE_AUTHENTICATION |
| --- | --- |
| Description | Generates or verifies the JSON formatted Initiate Authentication request on ES9+ or ES11 as applicable. |
| Parameter(s) | - paramEUICCChallenge: random 16 byte challenge coded as base 64 - paramEUICCInfo1: eUICC information structured coded as base 64 - paramServerAddress: FQDN of the Server. - paramLpaRspCapability: value of lpaRspCapability, supported by LPA |
| Details | JSON body { "euiccChallenge" : paramEUICCChallenge, "euiccInfo1" : paramEUICCInfo1, "smdpAddress" : paramServerAddress "lpaRspCapability " : paramLpaRspCapability } |

| Method | MTD_REGISTER_EVENT |
| --- | --- |
| Description | Send or checks the JSON formatted RegisterEvent request |
| Parameter(s) | -  paramFunctionRequesterId: identification of the function requester. -  paramFunctionCallId: identification of the function call. -  paramEID: EID of the targeted eUICC -  paramRspServerAddress: Address of the Server sending the RegisterEvent formatted as FQDN -  paramEventId: unique Identification of the Event to be registered -  paramForwardingIndicator: TRUE if registration has to be made to the Root SM-DS; FALSE if this is not to be made to the Root SM-DS |
| Details | JSON requestHeader { "header" : { "functionRequesterIdentifier" : "paramFunctionRequesterId", "functionCallIdentifier" : "paramFunctionCallId" } JSON body { "eid" : paramEID, "rspServerAddress" : paramRspServerAddress, "eventId" : paramEventId, "forwardingIndicator" : paramForwardingIndicator } } |

| Method | MTD_REMOVE_NOTIF |
| --- | --- |
| Description | Constructs the command data for RemoveNotificationFromList |
| Parameter(s) | -  paramSeqNumber: the sequence number to be removed |
| Details | request NotificationSentRequest ::= { seqNumber paramSeqNumber } |

| Method | MTD_RESP_RPR_FOR_SINGLE_CMND |
| --- | --- |
| Description | This Method can be used for verifying the rpmPackageResult resulted from a single RPM command. |
| Parameter(s) | -  paramRpmCommandResultDataChoice: The rpmCommandDetails choice (Mandatory) -  paramTransactionId: The Transaction Id (Mandatory) -  paramIccidValue: The ICCID within the RPM Command received -  paramRprErrorMask: The RPR error mask as defined below. (Mandatory) -  paramNotificationMetadata: The notification Metadata. -  paramSmdpOid: The SM-DP+ OID. -  paramProfileInfoValue: The Profile Info response -  paramPcmpAddressValue: The pcmp Address. - paramRpmCommandResultError: The Rpm Command Result error - paramRprErrorMask is defined as, - 0 – OK result (RPM command successful) - 1 - RpmCommandResultDataError is present - 2 - RpmProcessingTerminated is present - 3 - LoadRpmPackageErrorCode is present - 4 - LoadRpmPackageErrorCodeNotSigned is present - Parameters paramNotificationMetadata and paramSmdpOid are mandatory for all the cases except for paramRprErrorMask is 4 (loadRpmPackageErrorCodeNotSigned). - Parameters paramProfileInfoValue and paramPcmpAddressValue shall be passed and can be empty depend on the paramRpmCommandResultDataChoice for paramRprErrorMask is 0 (the successful responses). - Parameter paramRpmCommandResultError is mandatory if paramRprErrorMask > 0 (all error cases) and can be empty for paramRprErrorMask = 0 |
| Details | If paramRprErrorMask = 0 Then -- OK response If paramRpmCommandResultDataChoice = enableResult or disableResult or deleteResult or updateMetadataResult Then response LoadRpmPackageResult ::= loadRpmPackageResultSigned : { loadRpmPackageResultDataSigned { transactionId paramTransactionId, notificationMetadata paramNotificationMetadata, smdpOid paramSmdpOid, finalResult rpmPackageExecutionResult : { { iccid paramIccidValue, rpmCommandResultData paramRpmCommandResultDataChoice : { paramRpmCommandResultDataChoice ok } } } }, euiccSignRPR <EUICC_SIGN_RPR> } Else if paramRpmCommandResultDataChoice = listProfileInfoResult Then response LoadRpmPackageResult ::= loadRpmPackageResultSigned : { loadRpmPackageResultDataSigned { transactionId paramTransactionId, notificationMetadata paramNotificationMetadata, smdpOid paramSmdpOid, finalResult rpmPackageExecutionResult : { { rpmCommandResultData listProfileInfoResult : { profileInfoListOk : {paramProfileInfoValue } } } } }, euiccSignRPR <EUICC_SIGN_RPR> } Else if paramRpmCommandResultDataChoice = contactPcmpResult Then response LoadRpmPackageResult ::= loadRpmPackageResultSigned : { loadRpmPackageResultDataSigned { transactionId paramTransactionId, notificationMetadata paramNotificationMetadata, smdpOid paramSmdpOid, finalResult rpmPackageExecutionResult : { { iccid paramIccidValue, rpmCommandResultData contactPcmpResult : { contactPcmpResponseOk : { pcmpAddress paramPcmpAddressValue } } } } }, euiccSignRPR <EUICC_SIGN_RPR> } Else If paramRprErrorMask = 1 Then -- rpmCommandResultDataError is present If paramRpmCommandResultDataChoice = listProfileInfoResult Then response LoadRpmPackageResult ::= loadRpmPackageResultSigned : { loadRpmPackageResultDataSigned { transactionId paramTransactionId, notificationMetadata paramNotificationMetadata, smdpOid paramSmdpOid, finalResult rpmPackageExecutionResult : { { rpmCommandResultData listProfileInfoResult : { profileInfoListError : { paramRpmCommandResultError } } } } }, euiccSignRPR <EUICC_SIGN_RPR> } Else if paramRpmCommandResultDataChoice = contactPcmpResult Then response LoadRpmPackageResult ::= loadRpmPackageResultSigned : { loadRpmPackageResultDataSigned { transactionId paramTransactionId, notificationMetadata paramNotificationMetadata, smdpOid paramSmdpOid, finalResult rpmPackageExecutionResult : { { rpmCommandResultData contactPcmpResult: { contactPcmpResponseError: { paramRpmCommandResultError } } } } }, euiccSignRPR <EUICC_SIGN_RPR> } Else response LoadRpmPackageResult ::= loadRpmPackageResultSigned : { loadRpmPackageResultDataSigned { transactionId paramTransactionId, notificationMetadata paramNotificationMetadata, smdpOid paramSmdpOid, finalResult rpmPackageExecutionResult : { { iccid paramIccidValue, rpmCommandResultData paramRpmCommandResultDataChoice : { paramRpmCommandResultDataChoice paramRpmCommandResultError } } } }, euiccSignRPR <EUICC_SIGN_RPR> } Else If paramRprErrorMask = 2 Then -- rpmProcessingTerminated is present response LoadRpmPackageResult ::= loadRpmPackageResultSigned : { loadRpmPackageResultDataSigned { transactionId paramTransactionId, notificationMetadata paramNotificationMetadata, smdpOid paramSmdpOid, finalResult rpmPackageExecutionResult : { { rpmCommandResultData rpmProcessingTerminated : { paramRpmCommandResultError } } } }, euiccSignRPR <EUICC_SIGN_RPR> } Else If paramRprErrorMask = 3 Then -- loadRpmPackageErrorCode is present response LoadRpmPackageResult ::= loadRpmPackageResultSigned : { loadRpmPackageResultDataSigned { transactionId paramTransactionId, notificationMetadata paramNotificationMetadata, smdpOid paramSmdpOid, finalResult loadRpmPackageErrorCode : { paramRpmCommandResultError } }, euiccSignRPR <EUICC_SIGN_RPR> } Else If paramRprErrorMask = 4 Then -- loadRpmPackageErrorCodeNotSigned is present response LoadRpmPackageResult ::= loadRpmPackageResultNotSigned : { transactionId paramTransactionId, loadRpmPackageErrorCodeNotSigned paramRpmCommandResultError } End if |

| Method | MTD_RETRIEVE_NOTIF_SEQ_NUM |
| --- | --- |
| Description | Constructs the command data for RetrieveNotificationsList filtered by sequence number |
| Parameter(s) | -  paramSeqNumber: the sequence number to be retrieved |
| Details | request RetrieveNotificationsListRequest ::= { searchCriteria seqNumber paramSeqNumber } |

| Method | MTD_RPM_PKG_REQ_FOR_SINGLE_CMND |
| --- | --- |
| Description | This Method can be used for creating rpmPackage with single RPM command. |
| Parameter(s) | -  paramComandDetailsChoice: The rpmCommandDetails choice (Mandatory) -  paramIccidValue: The ICCID within RPM Command (Mandatory) -  paramUpdateMetadataRequest: The UpdateMetadataRequest if required -  paramDpiRpmValue: The dpiRpm within RPM Command if required -  paramRpmPending if required - Parameters paramUpdateMetadataRequest and paramDpiRpmValue shall be passed or shall be empty depend on the paramComandDetailsChoice. NULL shall be passed as parameter paramRpmPending if RPM command is triggered due to a rpmPending indication from a previous session. Otherwise it is not required. |
| Details | If paramComandDetailsChoice = enable or disable or delete Then { continueOnFailure NULL, rpmCommandDetails paramComandDetailsChoice : { iccid paramIccidValue } } Else if paramComandDetailsChoice = updateMetadata Then { continueOnFailure NULL, rpmCommandDetails updateMetadata : { iccid paramIccidValue updateMetadataRequest {paramUpdateMetadataRequest} } } Else if paramComandDetailsChoice = contactPcmp Then If paramDpiRpmValue is not present Then { continueOnFailure NULL, rpmCommandDetails contactPcmp : { iccid paramIccidValue } } Else { continueOnFailure NULL, rpmCommandDetails contactPcmp : { iccid paramIccidValue, dpiRpm paramDpiRpmValue } } End if |

| Method | MTD_RPM_PKG_REQ_FOR_SINGLE_CMND_LIST_PROFILE_INFO |
| --- | --- |
| Description | This Method can be used for creating rpmPackage with single RPM command ListProfileInfo and Search Criteria as ICCID. |
| Parameter(s) | - paramIccidValue: The ICCID within RPM Command (Mandatory)  paramTagListValue The TagList if required  paramProfileOwnerOidValue: The ProfileOwnerOid for seachCriteria in ListProfileInfo if required Parameters paramTagListValue and paramProfileOwnerOidValue shall be passed and can be empty. |
| Details | If paramTagListValue is not present Then If paramProfileOwnerOidValue is not present Then { continueOnFailure NULL, rpmCommandDetails listProfileInfo : { searchCriteria iccid : {paramIccidValue} } } Else { continueOnFailure NULL, rpmCommandDetails listProfileInfo : { searchCriteria profileOwnerOid : {paramProfileOwnerOidValue} } } Else If paramProfileOwnerOidValue is not present Then { continueOnFailure NULL, rpmCommandDetails listProfileInfo : { searchCriteria iccid : {paramIccidValue}, tagList { paramTagListValue} } } Else if { continueOnFailure NULL, rpmCommandDetails listProfileInfo : { searchCriteria profileOwnerOid : {paramProfileOwnerOidValue}, tagList { paramTagListValue} } } End if |

| Method | MTD_SELECT |
| --- | --- |
| Description | Generates the SELECT command as defined in GlobalPlatform Card Specification [6]. |
| Parameter(s) | -  paramAID: the AID to select |
| Details | - CLA = 0x or 4x (x = <CHANNEL_NUMBER>) - INS = A4 - P1 = 04 - P2 = 00 - LC = <L> - paramAID - LE = 00 |

| Method | MTD_SEND_SMS_PP |
| --- | --- |
| Description | Generate and send an envelope SMS-PP download to the MNO-SD |
| Parameter(s) |  paramApdusList: the list of APDUs (plain) to send |
| Details | Generate and send the following envelope: 80 C2 00 00 <L> D1 <L> 02 02 83 81 -- Device identity Tag 06 07 91 33 86 09 40 00 F0 -- Address Tag (TON/NPI/..) 0B <L> -- SMS TPDU 44 -- SMS-DELIVER 05 85 02 13 F2 -- TP-Originating-Address 7F -- TP-Protocol-Identifier F6 -- TP-Data-Coding-Scheme 71 30 12 41 55 74 40 -- TP-Service-Centre-Time-Stamp <L> -- TP-User-Data-Length 02 -- User-Data-Header-Length 70 -- IEIa 00 -- IEIDLa <L> -- Command Packet Length (2 bytes) <L> -- Command Header Length (1 byte) 12 21 -- SPI 00 -- KIC 15 –- KID (SCP80 Keyset version 0x01 in Triple DES) B2 01 00 –- MNO-SD TAR <MNO_SCP80_COUNTER> 00 -- Padding Counter <CC> -- Cryptographic checksum <C_APDUS_SCRIPT> -- Command APDUs script <C_APDUS_SCRIPT> SHALL contain the paramApdusList (i.e. each APDU is named <APDU1>; <APDU2>; …; <APDUn> here after) formatted as an expanded structure with definite length as defined in ETSI TS 102 226 [14]: AA <L> 22 <L> <APDU1> 22 <L> <APDU2> … 22 <L> <APDUn> The Cryptographic checksum <CC> SHALL be generated in Triple DES (outer-CBC mode using two different keys) with the #MNO_SCP80_AUTH_KEY as defined in ETSI TS 102 225 [13]. If the command packet length is higher than 140 bytes, it SHALL be sent over several envelopes: SMS concatenation as defined in 3GPP TS 23.040 [22] SHALL be used. |

| Method | MTD_STORE_DATA |
| --- | --- |
| Description | Generates the STORE DATA command (Case 4) as defined in GlobalPlatform Card Specification [6]. |
| Parameter(s) | -  paramCommandData: the command data |
| Details | - CLA = 8x or Cx (x = <CHANNEL_NUMBER>) - INS = E2 - P1 = 91 - P2 = 00 - LC = <L> - paramCommandData - LE = 00 |

| Method | MTD_STORE_DATA_Case3 |
| --- | --- |
| Description | Generates the STORE DATA command (Case3) as defined in GlobalPlatform Card Specification [6]. |
| Parameter(s) | -  paramCommandData: the command data |
| Details | - CLA = 8x or Cx (x = <CHANNEL_NUMBER>) - INS = E2 - P1 = 90 - P2 = 00 - LC = <L> - paramCommandData |

| Method | MTD_STORE_DATA_SCRIPT |
| --- | --- |
| Description | Generate (multiple) STORE DATA command(s) by breaking the data into smaller components (if needed) for transmission. |
| Parameter(s) | -  paramTLVDataToTransmit: TLVs array or single TLV to transfer to the eUICC -  paramCase4Command (optional parameter, default value = TRUE): TRUE if the APDU is a Case 4 command, FALSE if the APDU is a Case 3 command |
| Details | For each element of paramTLVDataToTransmit If the size of the element is greater than 255 bytes, split the element in several blocks of 255 bytes. The last block MAY be shorter. Each block is named <DATA_SUB_PART> here after. If the element is up to 255 bytes, <DATA_SUB_PART> contains the value of the element. The bit b1 of P1 in the STORE DATA commands is named <B1_P1> here after and is defined as below: If paramCase4Command = TRUE Then <B1_P1> = 1 Else <B1_P1> = 0 End If Set <STORE_DATA_BLOCK_NUM> to 0 For each <DATA_SUB_PART> If <DATA_SUB_PART> is an intermediate part, generate the following STORE DATA: - CLA = 8x or Cx (x = <CHANNEL_NUMBER>) - INS = E2 - P1 = 1x (x = <B1_P1>) - P2 = <STORE_DATA_BLOCK_NUM> - LC = <L> - <DATA_SUB_PART> - LE = 00 –- present only if paramCase4Command = TRUE If <DATA_SUB_PART> is the last part, generate the following STORE DATA: - CLA = 8x or Cx (x = <CHANNEL_NUMBER>) - INS = E2 - P1 = 9x (x = <B1_P1>) - P2 = <STORE_DATA_BLOCK_NUM> - LC = <L> - <DATA_SUB_PART> - LE = 00 –- present only if paramCase4Command = TRUE Increase the <STORE_DATA_BLOCK_NUM> by 1 End End |

| Method | MTD_TEST_ES8+_GET_BPP_PPK |
| --- | --- |
| Description | Tests the received boundProfilePackage element according to #R_GET_BPP_RESP_OP1_PPK |
| Parameter(s) | -  paramResponse the response to GetBoundProfilePackage -  paramS_MAC the 128 bit SCP03t MACing Session key -  paramS_ENC the 128 bit SCP03t Encryption Session key -  paramPPK_MAC the 128 bit Profile Protection MACing Key -  paramPPK_ENC the 128 bit Profile Protection Encryption Key - paramMetaData the ASN.1 StoreMetadataRequest element associated to a RSP profile |
| Details | Parse paramResponse into #R_GET_BPP_RESP_OP1_PPK and perform the following tests: -  Verify that each element in firstSequenceOf87, sequenceOf88, secondSequenceOf87 and sequenceOf86 has a total length (including tag and length fields) of 1020 or less -  Verify the integrity of each element in firstSequenceOf87, sequenceOf88 and secondSequenceOf87 using paramS_MAC -  Verify that <TRANSACTION_ID_ISC> in #INIT_SC_PROF1 matches <S_TRANSACTION_ID> -  Verify the validity of smdpSign <SM_DP+_SIGN> in #INIT_SC_PROF1 using #PK_SM_DPpb_ECDSA -  Retrieve #CONF_ISDP_PROF1_SMDP from <CONF_ISDP_PROF1_ENC> using paramS_ENC and validate the content of #CONF_ISDP_PROF1_SMDP -  Construct the complete metadata element from the <SMDP_METADATA_SEG_MAC> segment(s) and verify that it matches paramMetaData -  Retrieve #REPLACE_S_KEYS_REQ from <REPLACE_S_KEYS_REQ_ENC> using paramS_ENC and validate the content of #REPLACE_S_KEYS_REQ -  Verify that the lengths of paramPPK_ENC and paramPPK_MAC in #REPLACE_S_KEYS_REQ are each 16 bytes -  Verify the integrity of each <PPP_OP_PROF1_SEG_PPK> element using paramPPK_MAC -  Retrieve the <UPP_OP_PROF1_SEG> segment(s) from the <PPP_OP_PROF1_SEG_PPK> segment(s) using paramPPK_ENC, construct the complete Profile from the <UPP_OP_PROF1_SEG> segment(s), then verify that the complete Profile matches #UPP_OP_PROF1 |

| Method | MTD_TEST_ES8+_GET_BPP_SK |
| --- | --- |
| Description | Tests the received boundProfilePackage element according to #R_GET_BPP_RESP_OP1_SK |
| Parameter(s) | -  paramResponse the response to GetBoundProfilePackage -  paramS_MAC the 128 bit SCP03t MACing Session key -  paramS_ENC the 128 bit SCP03t Encryption Session key - paramMetaData the ASN.1 StoreMetadataRequest element associated to a RSP profile |
| Details | Parse paramResponse into #R_GET_BPP_RESP_OP1_SK and perform the following tests: -  Verify that each element in firstSequenceOf87, sequenceOf88 and sequenceOf86 has a total length (including tag and length fields) of 1020 or less -  Verify the integrity of each element in firstSequenceOf87, sequenceOf88 and sequenceOf86 using paramSMAC -  Verify that <TRANSACTION_ID_ISC> in #INIT_SC_PROF1 matches <S_TRANSACTION_ID> -  Verify the validity of smdpSign <SM_DP+_SIGN> in #INIT_SC_PROF1 using #PK_SM_DPpb_ECDSA -  Retrieve #CONF_ISDP_PROF1_SMDP from <CONF_ISDP_PROF1_ENC> using paramS_ENC and validate the content of #CONF_ISDP_PROF1_SMDP -  Construct the complete metadata element from the <SMDP_METADATA_SEG_MAC> segment(s) and verify that it matches paramMetaData -  Retrieve the <UPP_OP_PROF1_SEG> segment(s) from the <PPP_OP_PROF1_SEG_SK> segment(s) using paramS_ENC, then construct the complete Profile from the <UPP_OP_PROF1_SEG> segment(s), then verify that the complete Profile matches #UPP_OP_PROF1 |

| Method | MTD_TLS_CLIENT_KEY_EXCH_ETC |
| --- | --- |
| Description | Finalizes the Transport Layer Security (TLS) handshake in Server authentication mode on ES9+, or ES11 (Client side). |
| Parameter(s) | -  paramClientKeyExchange: ClientKeyExchange message |
| Details | Sends the session key information in TLS ClientKeyExchange message, ChangeCipherSpec and Finished message. |

| Method | MTD_TLS_CLIENT_HELLO |
| --- | --- |
| Description | Sends or checks the Client Hello message used to initiate the Transport Layer Security (TLS) handshake in Server authentication or Mutual authentication mode on ES9+, ES11, ES12 or ES15. |
| Parameter(s) | -  paramTLSversion: TLS protocol version -  paramAlgs: cipher suite types supported -  paramSessionID: Session ID -  paramExts: Extensions data for “supported_signature_algorithms”, “trusted_ca_keys”, “server_name” (as defined in RFC 6066) or other |
| Details | Sends or receives a TLS ClientHello message according to the parameters defined above. In addition, the following parameters will be set: -  The list of compression algorithms supported by the client is not explicitly defined, but by default it will be set to NULL. -  The random of 4 bytes representing time since epoch on client host and 28 random bytes is not explicitly defined but it SHALL be generated by the test tool TLS implementation NOTE1: The Supported Elliptic Curves Extension and the Supported Point Formats Extension extensions MAY be sent by the Client. NOTE2: Only the extensions explicitly specified in paramExts while using the method SHALL be verified. Other extensions MAY be present. |

| Method | MTD_TLS_SERVER_HELLO_ETC |
| --- | --- |
| Description | Send or Receives to the Client Hello in the Transport Layer Security (TLS) handshake in Server authentication mode on ES9+, or ES11. |
| Parameter(s) | -  paramTLSversion: TLS protocol version -  paramAlgs: cipher suite selected -  paramSessionID: Session ID -  paramCertificate: TLS server certificate for authentication - paramOtherCertsInChain (optional): TLS other server certificate(s) in the chain for auth. -  |
| Details | Sends or Receives a TLS ServerHello, Server Certificate, ServerKeyExchange and ServerHelloDone message in this order according to the parameters defined above. NOTE 1: The random of 4 bytes representing time since epoch on client host and 28 random bytes is not explicitly defined in the Server Hello message but it SHALL be generated by the Server under test. NOTE 2: Server_TLS_Ephemeral_Key, the value SHALL be set as defined in [24] for ServerKeyExchange. No verification required. |

- C.2	Procedures

|  | Procedure | PROC_ES11_AUTH_CLIENT |  |
| --- | --- | --- | --- |
|  | Description | Authenticate Server procedure and Event Retrieval from SM-SD. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | LPAd → S_SM-DS | Send ES11.AuthenticateClient method | MTD_HTTP_REQ(#TEST_ROOT_DS_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_DS_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DS → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_DS_OK) | No error |

|  | Procedure | PROC_ES11_VERIFY_EVENT_RETRIEVAL_EVENT_ID |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication on ES11 from S_LPAd to SM-DS under test supplying <EVENT_ID_R> and verifies that the pending Event #EVENT_ENTRY_1 is retrieved. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_LPAd → SM-DS | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |
| 2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_MATCHING_ID_EVENT_ID_R)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_DS_EVENT_ENTRY_1_OK) |

|  | Procedure | PROC_ES11_VERIFY_EVENT_RETRIEVAL_EVENT_ID_ERROR |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication on ES11 from S_LPAd to SM-DS under test supplying <EVENT_ID_R> and verifies that the pending Event #EVENT_ENTRY_1 is not available. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_LPAd → SM-DS | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |
| 2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_MATCHING_ID_EVENT_ID_R)) | MTD_HTTP_RESP( #R_ERROR_8_9_5_3_9) |

|  | Procedure | PROC_ES11_VERIFY_EVENT_RETRIEVAL_NO_EVENT_ID |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication on ES11 from S_LPAd to SM-DS under test supplying no MatchingId and verifies that the pending Event #EVENT_ENTRY_1 is retrieved. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_LPAd → SM-DS | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |
| 2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_MATCHING_ID_OMITTED)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_DS_EVENT_ENTRY_1_OK) |

|  | Procedure | PROC_ES11_VERIFY_EVENT_RETRIEVAL_NO_EVENT_ID_ERROR |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication on ES11 from S_LPAd to SM-DS under test supplying no MatchingId and verifies that no events are available. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_LPAd → SM-DS | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |
| 2 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DS_ADDRESS_ES11)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DS | MTD_HTTP_REQ( #IUT_SM_DS_ADDRESS_ES11, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_MATCHING_ID_OMITTED)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_DS_EVENT_ENTRY_EMPTY_OK) |

|  | Procedure | PROC_ES11_INIT_AUTH |  |
| --- | --- | --- | --- |
|  | Description | Initiate Authentication procedure with SM-DS. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | LPAd → S_SM-DS | Send ES11.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_ROOT_DS_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_ROOT_DS_ADDRESS, <LPA_RSP_CAPABILITY>)) |
| 2 | S_SM-DS → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_DS_OK) | No error |

|  | Procedure | PROC_EUICC_INITIALIZATION_SEQUENCE |  |
| --- | --- | --- | --- |
|  | Description | Initialize communication between the S_Device and the eUICC. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_Device → eUICC | RESET | ATR present |
| 2 | S_Device → eUICC | [SELECT_MF] | FCP Template present SW=0x9000 |
| 3 | S_Device → eUICC | [TERMINAL_CAPABILITY_LPAd] | SW=0x9000 |
| 4 | S_Device → eUICC | [TERMINAL_PROFILE] | Toolkit initialization THEN SW=0x9000 |

|  | Procedure | PROC_EUICC_INITIALIZATION_SEQUENCE_eUICCProfileStateChanged |  |
| --- | --- | --- | --- |
|  | Description | Initialize communication between the S_Device and the eUICC. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_Device → eUICC | RESET | ATR returned by eUICC |
| 2 | S_Device → eUICC | [SELECT_MF] | FCP Template present SW=0x9000 |
| 3 | S_Device → eUICC | [TERMINAL_CAPABILITY_LPAd] | SW=0x9000 |
| 4 | S_Device → eUICC | [TERMINAL_PROFILE_eUICCProfileStateChanged] | Toolkit initialization THEN SW=0x9000 |

|  | Procedure | PROC_OPEN_LOGICAL_CHANNEL_AND_SELECT_ISDR |  |
| --- | --- | --- | --- |
|  | Description | The LPAd opens a logical channel and selects the ISD-R. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_LPAd → eUICC | [MANAGE_CHANNEL_OPEN] | Extract the <CHANNEL_NUMBER> from response data SW=0x9000 |
| 2 | S_LPAd → eUICC | MTD_SELECT(#ISD_R_AID) | SW=0x9000 |

|  | Procedure | PROC_ES9+_AUTH_CLIENT |  |
| --- | --- | --- | --- |
|  | Description | Authenticate Server procedure without Confirmation Code. #R_AUTH_SERVER_MATCH_ID_DEV_INFO is used with the correct MatchingID defined by the Add Profile initiation procedure (Activation Code content or Empty MatchingID). |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | LPAd → S_SM-DP+ | Send ES9+.AuthenticateClient method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_OK) | No error |

|  | Procedure | PROC_ES9+_AUTH_CLIENT_CC |  |
| --- | --- | --- | --- |
|  | Description | Authenticate Server procedure (via Activation Code) with Confirmation Code. #R_AUTH_SERVER_MATCH_ID_DEV_INFO and #AUTH_SERVER_RESP_ACT_CODE_UC_OK are used with the correct MatchingID defined by the Add Profile initiation procedure (Activation Code content or Empty MatchingID). |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | LPAd → S_SM-DP+ | Send ES9+.AuthenticateClient method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_OK_CC) | No error |

|  | Procedure | PROC_ES9+_GET_BPP |  |
| --- | --- | --- | --- |
|  | Description | Get BPP procedure without Confirmation Code. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | LPAd → S_SM-DP+ | Send ES9+.GetBoundProfilePackage method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_GET_BPP, MTD_GET_BPP(<S_TRANSACTION_ID>, #R_PREP_DOWNLOAD_NO_CC)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#GET_BPP_OK) | No error |

|  | Procedure | PROC_ES9+_GET_BPP_CC |  |
| --- | --- | --- | --- |
|  | Description | Get BPP procedure with Confirmation Code. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | LPAd → S_SM-DP+ | Send ES9+.GetBoundProfilePackage method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_GET_BPP, MTD_GET_BPP(<S_TRANSACTION_ID>, #R_PREP_DOWNLOAD_WITH_CC)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP(#GET_BPP_OK) | No error |

|  | Procedure | PROC_ES9+_HANDLE_NOTIF |  |
| --- | --- | --- | --- |
|  | Description | Handle Notification procedure. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF(#R_PIR_OK)) See NOTE 2 |
| 2 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |
| NOTE 1: Other Notifications MAY be sent within the same HTTPS session. NOTE 2: The values of notificationAddress, iccid and smdpOid used in #R_PIR_OK MAY vary depending on the context (ICCID of the downloaded profile, used SM-DP+ address and certificate). |  |  |  |

|  | Procedure | PROC_ES9+_HANDLE_NOTIF_RPR_ERROR |  |
| --- | --- | --- | --- |
|  | Description | Handle Notification procedure for LoadRpmPackageResult with Errors. |  |
|  | Parameter(s) | paramRpmResultError : Parameter with rpmPackageResult containing rpmCommandResultDataError or rpmProcessingTerminated or loadRpmPackageErrorCode or loadRpmPackageErrorCodeNotSigned |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF(paramRpmResultError)) |
| 2 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |

|  | Procedure | PROC_ES9+_HANDLE_NOTIF_RPM_OK |  |
| --- | --- | --- | --- |
|  | Description | Handle Notification procedure for LoadRpmPackageResult. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF(#R_RPM_OK)) |
| 2 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |

|  | Procedure | PROC_ES9+_AUTH_CLIENT_FAIL_DEF_DP_USE_CASE_INVALID_MATCHING_ID |  |
| --- | --- | --- | --- |
|  | Description | AuthenticateClient fails due to an Invalid Matching ID. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_ACT_CODE_UC_OK)) | MTD_HTTP_RESP( #R_ERROR_8_2_6_3_8) |

|  | Procedure | PROC_ES9+_AUTH_CLIENT_RPM |  |
| --- | --- | --- | --- |
|  | Description | Authenticate Server procedure without Confirmation Code. #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3 and #AUTH_SERVER_RESP_RPM_OK are used with the correct MatchingID for the pending RPM package order. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | LPAd → S_SM-DP+ | Send ES9+.AuthenticateClient method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP (#AUTH_CLIENT_RPM_OK) | No error |

|  | Procedure | PROC_ES9+_PROF_DOWNLOAD_DEF_DP_USE_CASE_CANCEL_SESSION_SK |  |
| --- | --- | --- | --- |
|  | Description | End User cancels ongoing Profile Download after the generation of the one-time ECKA key pair, session keys and the generation of the Bound Profile Package. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK) |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_SK) |
| 5 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_POSTPONED)) | MTD_HTTP_RESP( #R_SUCCESS) Cancel Session request accepted by SM-DP+ and ongoing RSP session SHALL enter retry mode. |

|  | Procedure | PROC_ES9+_PROF_DOWNLOAD_DEF_DP_USE_CASE_CANCEL_SESSION_PPK |  |
| --- | --- | --- | --- |
|  | Description | End User cancels ongoing Profile Download after the generation of the one-time ECKA key pair, session keys, profile protection keys and the generation of the Bound Profile Package. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK) |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_PPK) |
| 5 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_POSTPONED)) | MTD_HTTP_RESP( #R_SUCCESS) Cancel Session request accepted by SM-DP+ and ongoing RSP session SHALL enter retry mode. |

|  | Procedure | PROC_ES9+_PROF_DOWNLOAD_DEF_DP_USE_CASE_CC_CANCEL_SESSION_PPK |  |
| --- | --- | --- | --- |
|  | Description | End User cancels ongoing Profile Download after the generation of the one-time ECKA key pair, session keys, profile protection keys and the generation of the Bound Profile Package when a Confirmation Code is required. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK_CC) |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_CC)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_PPK) |
| 5 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_POSTPONED)) | MTD_HTTP_RESP( #R_SUCCESS) Cancel Session request accepted by SM-DP+ and ongoing RSP session SHALL enter retry mode. |

|  | Procedure | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC_EN |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication and then delivers the Bound Profile Package to the LPAd for enable metadata notifications. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK_EN) |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_PPK) |

|  | Procedure | PROC_ES9+_PROF_DOWNLOAD_ACT_CODE_USE_CASE_CANCEL_SESSION |  |
| --- | --- | --- | --- |
|  | Description | End User cancels ongoing Profile Download after the generation of the one-time ECKA key pair, session keys and the generation of the Bound Profile Package. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_ACT_CODE_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK) |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_PPK) |
| 5 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_POSTPONED)) | MTD_HTTP_RESP( #R_SUCCESS) Cancel Session request accepted by SM-DP+ and ongoing RSP session SHALL enter retry mode. |

|  | Procedure | PROC_ES9+_PROF_DOWNLOAD_SM_DS_USE_CASE_CANCEL_SESSION |  |
| --- | --- | --- | --- |
|  | Description | End User cancels ongoing Profile Download after the generation of the one-time ECKA key pair, session keys and the generation of the bound profile package. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_ES9+_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_SMDS_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK) |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_PPK) |
| 5 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_POSTPONED)) | MTD_HTTP_RESP( #R_SUCCESS) Cancel Session request accepted by SM-DP+ and ongoing RSP session shall enter retry mode. |

|  | Procedure | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication for the Profile Download Default SM_DP+ use case without a confirmation code. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT,MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK) |

|  | Procedure | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_CC |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication for the Profile Download Default SM_DP+ use case with a confirmation code. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK_CC) |

|  | Procedure | PROC_ES9+_INIT_AUTH |  |
| --- | --- | --- | --- |
|  | Description | Initiate Authentication procedure. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, #TEST_DP_ADDRESS1)) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_OK) | No error |

|  | Procedure | PROC_ES9+_INIT_AUTH_AND_AUTH_CLIENT_REQ_V3 |  |
| --- | --- | --- | --- |
|  | Description | Initiate Authentication procedure. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, <LPA_RSP_CAPABILITY>) ) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_OK_VARIANT_A) | No error |
| 3 | LPAd → S_SM-DP+ | Send ES9+.AuthenticateClient method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3)) Next step of common mutual authentication procedure is performed by the S_SM-DP+. |

|  | Procedure | PROC_ES9+_INIT_AUTH_V3 |  |
| --- | --- | --- | --- |
|  | Description | Initiate Authentication procedure. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | LPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, #R_EUICC_INFO1, <LPA_RSP_CAPABILITY>) ) |
| 2 | S_SM-DP+ → LPAd | MTD_HTTP_RESP( #INITIATE_AUTH_OK_VARIANT_A) | No error, Next step of common mutual authentication procedure is performed. |

|  | Procedure | PROC_ES9+_VERIFY_CMA_PD_DEF_SMDP_ADDRESS_NO_CC_FAIL |  |
| --- | --- | --- | --- |
|  | Description | Verifies that Common Mutual Authentication for the Profile Download Default SM_DP+ use case without a confirmation code fails due to the profile being in the ‘Installed‘ or ‘Error’ state. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP(#R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_ERROR_8_1_1_3_8) |

|  | Procedure | PROC_VERIFY_SESSION_IS_CANCELLED |  |
| --- | --- | --- | --- |
|  | Description | Verify that the RSP session identified by the TransactionID <S_TRANSACTION_ID> has been cancelled by the eUICC (i.e. Common Mutual Authentication and Profile Download procedures SHALL be rejected as long as no GetEUICCChallenge has been requested). |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_LPAd → eUICC | MTD_STORE_DATA_SCRIPT( #PREP_DOWNLOAD_NO_CC) | #R_PREP_DOWN_NO_SESSION SW=0x9000 The transactionId returned in the response SHALL not be checked (any value SHALL be accepted) |
| 2 | S_LPAd → eUICC | MTD_STORE_DATA_SCRIPT( #AUTHENTICATE_SMDP) | #R_AUTH_SERVER_NO_SESSION SW = 0x9000 The transactionId returned in the response SHALL not be checked (any value SHALL be accepted) |

|  | Procedure | PROC_ES9+_PROF_DOWNLOAD_DEF_DP_USE_CASE_CC_CANCEL_SESSION_SK |  |
| --- | --- | --- | --- |
|  | Description | End User cancels ongoing Profile Download after the generation of the one-time ECKA key pair, session keys and the generation of the Bound Profile Package when a Confirmation Code is required. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK_CC) |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_CC)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_SK) |
| 5 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_CANCEL_SESSION, MTD_CANCEL_SESSION( <S_TRANSACTION_ID>, #CS_RESP_OK_POSTPONED)) | MTD_HTTP_RESP( #R_SUCCESS) Cancel Session request accepted by SM-DP+ and ongoing RSP session SHALL enter retry mode. |

|  | Procedure | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_CC_RETRY |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication for the Profile Download Default SM_DP+ use case without a confirmation code. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_RETRY_OK) |

|  | Procedure | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_INVALID_CC |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication for the Profile Download Default SM_DP+ use case with an invalid confirmation code provided in the GetBoundProfilePackage. |  |
| Step | Direction | Sequence / Description | Expected result |
| IC1 | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_CC |  |  |
| 1 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_8_2_7_3_8)) | MTD_HTTP_RESP( #R_ERROR_8_2_7_3_8) |

|  | Procedure | PROC_ES9+_CMA_PD_DEF_SMDP_ADDRESS_UC_NO_CC_RETRY |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication for the Profile Download Default SM_DP+ use case without a confirmation code. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_RETRY_OK) |

|  | Procedure | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |
| --- | --- | --- | --- |
|  | Description | Establishes the Transport Layer Security (TLS) v1.2 connection between the Client (S_)LPAd and (S_)SERVER using Server authentication mode on ES9+ or ES11 with Variant O certificate. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | LPAd → S_SERVER | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_TLS_VERSION, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) |
| 2 | S_SERVER → LPAd | MTD_TLS_SERVER_HELLO_ETC(#TLS_VERSION_1_2, #S_TLS_CIPHER_SUITE, <S_SESSION_ID_SERVER>, #CERT_S_SERVER_TLS, NO_PARAM) | MTD_TLS_CLIENT_KEY_EXCH_ETC(<CLIENT_TLS_EPHEM_KEY>) |
| 3 | S_SERVER → LPAd | Finalize TLS Handshake (send Server ChangeCipherSpec and Finished messages) | HTTPS connection established |

|  | Procedure | PROC_TLS_INITIALIZATION_SERVER_AUTH_VARIANT_A |  |
| --- | --- | --- | --- |
|  | Description | Establishes the Transport Layer Security (TLS) v3 connection between the Client LPAd and S_SERVER using Server authentication mode on ES9+ or ES11 as defined in SGP.22 v3 [2] for Variant A. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | LPAd → S_SERVER | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_TLS_VERSION, <TLS_CIPHER_SUITES>, #SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) |
| 2 | S_SERVER → LPAd | MTD_TLS_SERVER_HELLO_ETC( #TLS_VERSION_1_2, #S_TLS_CIPHER_SUITE, <SESSION_ID_RANDOM>, #CERT_S_SERVER_TLS, #CERT_S_SM_DP_SubCA_SIG) | MTD_TLS_CLIENT_KEY_EXCH_ETC(<CLIENT_TLS_EPHEM_KEY>) |
| 3 | S_SERVER → LPAd | Finalize TLS Handshake (send Server ChangeCipherSpec and Finished messages) | HTTPS connection established |

|  | Procedure | PROC_ES9+_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC_NO_CC |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication and then delivers the Bound Profile Package to the LPAd. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK) |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_PPK) |

|  | Procedure | PROC_ES9+_VERIFY_PROFILE_DOWNLOAD_DEF_SMDP_ADDRESS_UC |  |
| --- | --- | --- | --- |
|  | Description | Verifies that Common Mutual Authentication occurs successfully and that the Bound Profile Package is generated and successfully delivered to the LPAd. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_AUTH_CLIENT_OK) OR MTD_HTTP_RESP( #R_AUTH_CLIENT_RETRY_OK) |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP)) | MTD_HTTP_RESP( #R_GET_BPP_RESP_OP1_PPK) |

|  | Procedure | PROC_ES9+_VERIFY_PROFILE _NOT_RELEASED_EMPTY_MID |  |  |
| --- | --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication on ES9+ from S_LPAd to SM-DP+ under test supplying an empty MatchingId and verifies that there is no pending profile in Released state. |  |  |
| Step | Direction | Sequence / Description | Expected result |  |
| 1 | S_LPAd → SM-DP+ | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |  |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( #R_ERROR_8_2_1_2) |  |

|  | Procedure | PROC_ES9+_VERIFY_PROFILE_RELEASED_EMPTY_MID_WITH_CC |  |
| --- | --- | --- | --- |
|  | Description | Performs Common Mutual Authentication on ES9+ from S_LPAd to SM-DP+ under test supplying an empty MatchingId and verifies that there is at least one pending profile in Released state. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | S_LPAd → SM-DP+ | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |
| 2 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( #S_EUICC_CHALLENGE, #S_EUICC_INFO1, #IUT_SM_DP_ADDRESS)) | MTD_HTTP_RESP( #R_INITIATE_AUTH_OK) |
| 3 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT( <S_TRANSACTION_ID>, #AUTH_SERVER_RESP_DEF_DP_UC_OK)) | MTD_HTTP_RESP( # R_AUTH_CLIENT_OK_CC) |
| 4 | S_LPAd → SM-DP+ | MTD_HTTP_REQ( #IUT_SM_DP_ADDRESS, #PATH_GET_BPP, MTD_GET_BPP( <S_TRANSACTION_ID>, #PREP_DOWNLOAD_RESP_CC)) | MTD_HTTP_RESP(#R_GET_BPP_RESP_OP1_PPK) |

- Annex D	Commands And Responses

- D.1	ES8+ Requests And Responses

- D.1.1 ES8+ Requests

| Name | Content |
| --- | --- |
| CONF_ISDP_EMPTY | req ConfigureISDPRequest ::={} |
| CONF_ISDP_MAX_LENGTH | req ConfigureISDPRequest ::={ dpProprietaryData { -- size=128 bytes dpOid #S_SM_DP+_OID, additionalSmdpData #ADDITIONAL_SMDP_DATA_MAX_LENGTH } } -- NOTE: Instead of DpProprietaryData ::= SEQUENCE { dpOid OBJECT IDENTIFIER -- additional data objects defined by the -- SM-DP+ MAY follow } -- the following structure is used to test the -- DpProprietaryData size: DpProprietaryData ::= SEQUENCE { dpOid OBJECT IDENTIFIER, additionalSmdpData OCTET STRING OPTIONAL } |
| CONF_ISDP_PROF1 | req ConfigureISDPRequest ::={ dpProprietaryData { dpOid #S_SM_DP+_OID } } |
| CONF_ISDP_PROF1_SMDP | req ConfigureISDPRequest ::={ dpProprietaryData { dpOid #IUT_SM_DP_OID }-- optional } |
| CONF_ISDP_SIZE_EXCEEDED | req ConfigureISDPRequest ::={ dpProprietaryData { -- size=129 bytes dpOid #S_SM_DP+_OID, additionalSmdpData #ADDITIONAL_SMDP_DATA_EXCEEDED_MAX } } -- NOTE: Instead of DpProprietaryData ::= SEQUENCE { dpOid OBJECT IDENTIFIER -- additional data objects defined by the -- SM-DP+ MAY follow } -- the following structure is used to test the -- DpProprietaryData size: DpProprietaryData ::= SEQUENCE { dpOid OBJECT IDENTIFIER, additionalSmdpData OCTET STRING OPTIONAL } |
| FULL_METADATA | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationLocalEnable, notificationLocalDisable, notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS1 } }, profileOwner { mccMnc #MCC_MNC1 }, profilePolicyRules {ppr1} } |
| INIT_SC_INVALID_CRT | req InitialiseSecureChannelRequest ::={ remoteOpId #REMOTE_OP_ID_INSTALL, transactionId <S_TRANSACTION_ID>, controlRefTemplate { keyType #INVALID_KEY_TYPE, keyLen #KEY_LENGTH, hostId #HOST_ID }, smdpOtpk <OTPK_S_SM_DP+_ECKA>, smdpSign <S_SM_DP+_SIGN> } |
| INIT_SC_INVALID_OP_ID | req InitialiseSecureChannelRequest ::={ remoteOpId #INVALID_REMOTE_OP_ID, transactionId <S_TRANSACTION_ID>, controlRefTemplate { keyType #KEY_TYPE, keyLen #KEY_LENGTH, hostId #HOST_ID }, smdpOtpk <OTPK_S_SM_DP+_ECKA>, smdpSign <S_SM_DP+_SIGN> } |
| INIT_SC_INVALID_SIGN | req InitialiseSecureChannelRequest ::={ remoteOpId #REMOTE_OP_ID_INSTALL, transactionId <S_TRANSACTION_ID>, controlRefTemplate { keyType #KEY_TYPE, keyLen #KEY_LENGTH, hostId #HOST_ID }, smdpOtpk <OTPK_S_SM_DP+_ECKA>, smdpSign <S_SM_DP+_SIGN> } The <S_SM_DP+_SIGN> SHALL NOT be computed using the #SK_S_SM_DPpb_ECDSA but SHALL have the same length as for a valid signature |
| INIT_SC_INVALID_TRANS_ID | req InitialiseSecureChannelRequest ::={ remoteOpId #REMOTE_OP_ID_INSTALL, transactionId <INVALID_TRANSACTION_ID>, controlRefTemplate { keyType #KEY_TYPE, keyLen #KEY_LENGTH, hostId #HOST_ID }, smdpOtpk <OTPK_S_SM_DP+_ECKA>, smdpSign <S_SM_DP+_SIGN> } |
| INIT_SC_PROF1 | req InitialiseSecureChannelRequest ::={ remoteOpId #REMOTE_OP_ID_INSTALL, transactionId <TRANSACTION_ID_ISC>, controlRefTemplate { keyType #KEY_TYPE, keyLen #KEY_LENGTH, hostId #IUT_SM_DP_HOST_ID }, smdpOtpk <OTPK_SM_DP+_ECKA>, smdpSign <SM_DP+_SIGN> } |
| METADATA_ICCID_MISMATCH | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF2, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1 } |
| METADATA_MCCMNC_MISMATCH | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileOwner { mccMnc #MCC_MNC2 }, profilePolicyRules {ppr2} } |
| METADATA_NO_CLASS | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationLocalEnable, notificationLocalDisable, notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS1 } } } |
| METADATA_OP_PROF1 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationLocalEnable, notificationLocalDisable, notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS1 } }, profileOwner { mccMnc #MCC_MNC1 } } |
| METADATA_OP_PROF1_EN | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationLocalEnable }, notificationAddress #TEST_DP_ADDRESS1 } }, profileOwner { mccMnc #MCC_MNC1 } } |
| METADATA_OP_PROF1_ENT_RULES_RPM_CONF_EN | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, profileOwner { mccMnc #MCC_MNC1 }, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType {enable} } }, pollingAddress #TEST_DP_ADDRESS1, profileOwnerOid #S_PROFILE_OWNER_OID }, enterpriseConfiguration { enterpriseOid #S_ENTERPRISE_OID, enterpriseName #ENTERPRISE_NAME1, enterpriseRules { referenceEnterpriseRule, onlyEnterpriseProfilesCanBeEnabled, onlyEnterpriseProfilesCanBeInstalled } } } |
| METADATA_OP_PROF1_INST_DIFF | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS2 } }, profileOwner { mccMnc #MCC_MNC1 } } |
| METADATA_OP_PROF1_PPR_RPM_CONF_EN | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, profileOwner { mccMnc #MCC_MNC1 }, profilePolicyRules {ppr1, ppr2}, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType {enable} } }, pollingAddress #TEST_DP_ADDRESS1, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF1_RPM_CONF_ALL | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, profileOwner { mccMnc #MCC_MNC1 }, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType {enable, disable, delete, listProfileInfo, contactPcmp}, tagList ‘99’H –- Tag for PPR } }, pollingAddress #TEST_DP_ADDRESS1, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF1_RPM_CONF_ALL_CI_PKI_RAND | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, profileOwner { mccMnc #MCC_MNC1 }, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType { enable, disable, delete, listProfileInfo, contactPcmp} tagList ‘99BB9ABC5A’H –- Tag for PPR, rpmConfiguration, hriServerAddress, lprConfiguration and ICCID } }, pollingAddress #TEST_DP_ADDRESS1, allowedCiPKId <CI_PKI_RANDOM>, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF1_RPM_CONF_ALL_DP_OID2 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, profileOwner { mccMnc #MCC_MNC1 }, profilePolicyRules {ppr1}, hriServerAddress { #TEST_HRI_ADDRESS1 }, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID2, rpmType {enable, disable, delete, listProfileInfo, contactPcmp} tagList ‘99BB9ABC5A’H –- Tag for PPR, rpmConfiguration, hriServerAddress, lprConfiguration and ICCID } }, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF1_RPM_CONF_ALL_PPR1 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, profileOwner { mccMnc #MCC_MNC1 }, profilePolicyRules {ppr1}, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType {enable, disable, delete, listProfileInfo, contactPcmp}, tagList ‘99’H –- Tag for PPR } }, pollingAddress #TEST_DP_ADDRESS1, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF1_RPM_CONF_DE_PPR2 | metadataReq StoreMetadataRequest:: = { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, profileOwner { mccMnc #MCC_MNC1 }, profilePolicyRules {ppr2}, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType {delete} } }, pollingAddress #TEST_DP_ADDRESS1, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF1_RPM_CONF_DELETE | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, profileOwner { mccMnc #MCC_MNC1 }, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType {delete} } }, pollingAddress #TEST_DP_ADDRESS1, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF1_RPM_CONF_DISABLE | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, profileOwner { mccMnc #MCC_MNC1 }, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType {disable} } }, pollingAddress #TEST_DP_ADDRESS1, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF1_RPM_CONF_EN | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, profileOwner { mccMnc #MCC_MNC1 }, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType {enable} } }, pollingAddress #TEST_DP_ADDRESS1, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF1_RPM_CONF_EN_CI_PKI_RPM | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, profileOwner { mccMnc #MCC_MNC1 }, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType {enable}, } }, pollingAddress #TEST_DP_ADDRESS1, allowedCiPKId #PK_CI_ECDSA_RPM, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF1_RPM_CONF_EN_CI_PKI_RAND | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, profileOwner { mccMnc #MCC_MNC1 }, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType {enable}, } }, pollingAddress #TEST_DP_ADDRESS1, allowedCiPKId #PK_CI_ECDSA_RANDOM, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF1_RPM_CONF_EN_DP_OID2 | metadataReq StoreMetadataRequest:: = { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileClass operational, profileOwner { mccMnc #MCC_MNC1 }, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID2, rpmType {enable} } }, pollingAddress #TEST_DP_ADDRESS1, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF1_RPM_CONF_EN_NO_POLLING_ADDRESS | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, profileOwner { mccMnc #MCC_MNC1 }, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType {enable} } }, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF1_RPM_CONF_EN_NOTIF_CONF | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationRpmEnable, notificationRpmDisable, notificationRpmDelete }, notificationAddress #TEST_DP_ADDRESS1 } }, profileOwner { mccMnc #MCC_MNC1 }, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType {enable} } }, pollingAddress #TEST_DP_ADDRESS1, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF1_RPM_CONF_EN_NOTIF_CONF2 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationRpmEnable, notificationRpmDisable, notificationRpmDelete }, notificationAddress #TEST_DP_ADDRESS2 } }, profileOwner { mccMnc #MCC_MNC1 }, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType {enable} } }, pollingAddress #TEST_DP_ADDRESS1, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF1_RPM_CONF_EN_POL_DP8 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, profileOwner { mccMnc #MCC_MNC1 }, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType {enable}, } }, pollingAddress #TEST_DP_ADDRESS8, allowedCiPKId #PK_CI_ECDSA_RANDOM, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF1_RPM_CONF_EN_POL_SMDS_CI_PKI_RPM | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, profileOwner { mccMnc #MCC_MNC1 }, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType {enable}, } }, pollingAddress #TEST_ROOT_DS_ADDRESS, allowedCiPKId #PK_CI_ECDSA_RPM, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF1_RPM_CONF_EN_POL_SMDS_CI_PKI_RAND | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, profileOwner { mccMnc #MCC_MNC1 }, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType {enable}, } }, pollingAddress #TEST_ROOT_DS_ADDRESS, allowedCiPKId #PK_CI_ECDSA_RANDOM, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF1_RPM_CONF_UPDATE_MD_PPR | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, profileOwner { mccMnc #MCC_MNC1 }, profilePolicyRules {ppr1, ppr2}, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, tagList ‘99’H –- Tag for PPR } }, pollingAddress #TEST_DP_ADDRESS1, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF1_RPM_UM_ENT_RULES | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, profileOwner { mccMnc #MCC_MNC1 }, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, tagList ‘BD’H –- Tag for Enterprise Configuration } }, pollingAddress #TEST_DP_ADDRESS1, profileOwnerOid #S_PROFILE_OWNER_OID }, enterpriseConfiguration { enterpriseOid #S_ENTERPRISE_OID, enterpriseName #ENTERPRISE_NAME1, enterpriseRules { onlyEnterpriseProfilesCanBeEnabled, onlyEnterpriseProfilesCanBeInstalled } } } |
| METADATA_OP_PROF2_MEMRES1 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF2, serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, iconType png, icon #ICON_OP_PROF2, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationLocalDisable, notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS2 } }, profileOwner { mccMnc #MCC_MNC2 }, profilePolicyRules { ppr2 } } |
| METADATA_OP_PROF4_MEMRES1 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF4, serviceProviderName #SP_NAME4, profileName #NAME_OP_PROF4, iconType png, icon #ICON_OP_PROF4, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationLocalDisable, notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS4 } }, profileOwner { mccMnc #MCC_MNC4 }, profilePolicyRules { ppr1 } } |
| METADATA_OP_PROF2 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF2, serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, iconType png, icon #ICON_OP_PROF2, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationLocalEnable, notificationLocalDisable, notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS2 } }, profileOwner { mccMnc #MCC_MNC2 } } |
| METADATA_OP_PROF2_NO_INSTALL | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF2, serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, iconType png, icon #ICON_OP_PROF2, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationLocalEnable, notificationLocalDisable, notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS2 } }, profileOwner { mccMnc #MCC_MNC2 } } |
| METADATA_OP_PROF2_RPM_CONF_ALL_OWNER2 | metadataReq StoreMetadataRequest::= { iccid #ICCID_OP_PROF2, serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, iconType png, icon #ICON_OP_PROF2, profileClass operational, profileOwner { mccMnc #MCC_MNC2 }, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType {enable, disable, delete, listProfileInfo, contactPcmp}, tagList ‘99’H –- Tag for PPR } }, pollingAddress #TEST_DP_ADDRESS2, profileOwnerOid #S_PROFILE_OWNER_OID2 } } |
| METADATA_OP_PROF2_RPM_CONF_EN_OWNER_OID1 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF2, serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, iconType png, icon #ICON_OP_PROF2, profileClass operational, profileOwner { mccMnc #MCC_MNC2 }, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType {enable} } }, pollingAddress #TEST_DP_ADDRESS1, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF2_RPM_CONF_EN_OWNER_OID1_AND_NOTIF_CONF | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF2, serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, iconType png, icon #ICON_OP_PROF2, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationLocalDisable, notificationRpmEnable, notificationRpmDisable, notificationRpmDelete }, notificationAddress #TEST_DP_ADDRESS2 } }, profileOwner { mccMnc #MCC_MNC2 }, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType {enable} } }, pollingAddress #TEST_DP_ADDRESS2, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF2_RPM_CONF_EN_OWNER_OID1_PPR1 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF2, serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, iconType png, icon #ICON_OP_PROF2, profileClass operational, profileOwner { mccMnc #MCC_MNC2 }, profilePolicyRules {ppr1}, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType {enable} } }, pollingAddress #TEST_DP_ADDRESS1, profileOwnerOid #S_PROFILE_OWNER_OID } } |
| METADATA_OP_PROF1_NO_INSTALL | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationLocalEnable, notificationLocalDisable, notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS1 } }, profileOwner { mccMnc #MCC_MNC1 } } |
| METADATA_OP_PROF2_TEST_DP_ADDRESS1 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF2, serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, iconType png, icon #ICON_OP_PROF2, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationLocalEnable, notificationLocalDisable, notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS1 } }, profileOwner { mccMnc #MCC_MNC2 } } |
| METADATA_OP_PROF3 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF3, serviceProviderName #SP_NAME3, profileName #NAME_OP_PROF3, iconType png, icon #ICON_OP_PROF3, profileClass operational, profileOwner { mccMnc #MCC_MNC2 }, profilePolicyRules { ppr2 } } |
| METADATA_OP_PROF3_RPM_CONF_ALL | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF3, serviceProviderName #SP_NAME3, profileName #NAME_OP_PROF3, iconType png, icon #ICON_OP_PROF3, profileClass operational, profileOwner { mccMnc #MCC_MNC3 }, hriServerAddress { #TEST_HRI_ADDRESS3 }, rpmConfiguration { managingDpList { { managingDP #S_SM_DP+_OID, rpmType {enable, disable, delete, listProfileInfo, contactPcmp} tagList ‘99BB9ABC5A’H –- Tag for PPR, rpmConfiguration, hriServerAddress, lprConfiguration and ICCID } }, pollingAddress #TEST_DP_ADDRESS3, profileOwnerOid #S_PROFILE_OWNER_OID }, lprConfiguration { pcmpAddress #TEST_PCMP_ADDRESS3 } } |
| METADATA_OP_PROF4 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF4, serviceProviderName #SP_NAME4, profileName #NAME_OP_PROF4, iconType png, icon #ICON_OP_PROF4, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationLocalEnable, notificationLocalDisable, notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS4 } }, profileOwner { mccMnc #MCC_MNC4 }, profilePolicyRules { ppr1 } } |
| METADATA_OP_PROF5 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF5, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF5, iconType png, icon #ICON_OP_PROF5, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationLocalEnable, notificationLocalDisable, notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS1 } }, profileOwner { mccMnc #MCC_MNC1 } } |
| METADATA_OP_PROF6 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF6, serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF6, iconType png, icon #ICON_OP_PROF6, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationLocalEnable, notificationLocalDisable, notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS2 } }, profileOwner { mccMnc #MCC_MNC2 } } |
| METADATA_OP_PROF7 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF7, serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF7, iconType png, icon #ICON_OP_PROF7, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationLocalEnable, notificationLocalDisable, notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS8 } }, profileOwner { mccMnc #MCC_MNC2 }, profilePolicyRules { ppr2 } } |
| METADATA_OP_PROF8 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF8, serviceProviderName #SP_NAME8, profileName #NAME_OP_PROF8, iconType png, icon #ICON_OP_PROF8, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationLocalEnable, notificationLocalDisable, notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS8 } }, profileOwner { mccMnc #MCC_MNC2 }, profilePolicyRules { ppr2 } } |
| METADATA_OP_PROF9 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF9, serviceProviderName #SP_NAME9, profileName #NAME_OP_PROF9, profileOwner { mccMnc #MCC_MNC9, gid1 #GID1, gid2 #GID2 }, profilePolicyRules { ppr2 } } |
| METADATA_OP1_GID1GID2_PRESENT | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileOwner { mccMnc #MCC_MNC1, gid1 #GID1, gid2 #GID2 }, profilePolicyRules {ppr2} } |
| METADATA_OP9_GID1GID2_MISSING | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF9, serviceProviderName #SP_NAME9, profileName #NAME_OP_PROF9, profileOwner { mccMnc #MCC_MNC9 } } |
| METADATA_PPR_NO_OWNER | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profilePolicyRules {ppr2} } |
| METADATA_WILDCARD | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileOwner { mccMnc #MCC_MNC_WILDCARD }, profilePolicyRules {ppr2} } |
| METADATA_WITH_JPG | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType jpg, icon #ICON_JPG } |
| METADATA_WITH_NOTIFS | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, notificationConfigurationInfo { { profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS3 }, { profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS2 }, { profileManagementOperation { notificationLocalEnable }, notificationAddress #TEST_DP_ADDRESS2 }, { profileManagementOperation { notificationLocalEnable }, notificationAddress #TEST_DP_ADDRESS3 }, { profileManagementOperation { notificationLocalDisable }, notificationAddress #TEST_DP_ADDRESS3 }, { profileManagementOperation { notificationLocalDisable }, notificationAddress #TEST_DP_ADDRESS4 }, { profileManagementOperation { notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS1 }, { profileManagementOperation { notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS3 } } } |
| METADATA_WITH_PPR1_PPR2 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileOwner { mccMnc #MCC_MNC1 }, profilePolicyRules {ppr1,ppr2} } |
| METADATA_WITH_PPR2 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileOwner { mccMnc #MCC_MNC1 }, profilePolicyRules {ppr2} } |
| METADATA_WITH_PPRS_AND_ICON | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileOwner { mccMnc #MCC_MNC1 }, profilePolicyRules {ppr1,ppr2} } |
| METADATA_WITHOUT_ICON | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType jpg } |
| REPLACE_S_KEYS_REQ | req ReplaceSessionKeysRequest ::={ initialMacChainingValue <PPK_INIT_MAC>, ppkEnc <PPK_ENC>, ppkCmac <PPK_MAC> } |
| REPLACE_S_KEYS_REQ_INV_SIZE | req ReplaceSessionKeysRequest ::={ initialMacChainingValue #PPK_INIT_MAC_INV_SIZE, ppkEnc #PPK_ENC_INV_SIZE, ppkCmac #PPK_MAC_INV_SIZE } |
| S_INIT_SC_PROF1 | req InitialiseSecureChannelRequest ::={ remoteOpId #REMOTE_OP_ID_INSTALL, transactionId <S_TRANSACTION_ID>, controlRefTemplate { keyType #KEY_TYPE, keyLen #KEY_LENGTH, hostId #HOST_ID }, smdpOtpk <OTPK_S_SM_DP+_ECKA>, smdpSign <S_SM_DP+_SIGN> } |
| SMDP_METADATA_ABS | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1 } |
| SMDP_METADATA_ALL | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationLocalEnable, notificationLocalDisable, notificationLocalDelete }, notificationAddress #IUT_SM_DP_ADDRESS } }, profileOwner { mccMnc #MCC_MNC1 }, profilePolicyRules { ppr1, ppr2 } } |
| SMDP_METADATA_NON_ASCII | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME_NON_ASCII, profileName #NAME_OP_PROF1_NON_ASCII } |
| SMDP_METADATA_NOTIF_MULTI | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationLocalEnable, notificationLocalDisable, notificationLocalDelete }, notificationAddress #IUT_SM_DP_ADDRESS }, { profileManagementOperation { notificationInstall, notificationLocalEnable, notificationLocalDisable, notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS1 } } } |
| SMDP_METADATA_OP_PROF1_EN | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationLocalEnable }, notificationAddress #IUT_SM_DP_ADDRESS } } } |
| SMDP_METADATA_OP_PROF1_PPR2 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileClass operational, profileOwner { mccMnc #MCC_MNC1 }, profilePolicyRules { ppr2 } } |
| SMDP_METADATA_PN_LONG | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF_LONG } |
| SMDP_METADATA_SPN_LONG | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME_LONG, profileName #NAME_OP_PROF1 } |

- D.2	ES9+ Requests And Responses

- D.2.1	ES9+ Requests

| Name | Content |
| --- | --- |
| AUTH_SERVER_RESP_ACT_CODE_UC_OK_EID2 | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_ACT_CODE }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA_EID2, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_ACT_CODE_2_UC_OK | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_ACT_CODE_2 }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_DEF_DP_OK_eUICC_EXT | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2_UICC_EXT ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_DEF_DP_OK DEVICE_EXT | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2_DEV_EXT ctxParams1 #CTX_PARAMS1_DEVICE_INFO_EXT }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_1_3_8 | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA_EID2, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_4_8 | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2_INSUF_MEM_ERROR, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_2_6_1_EX_BC_cA | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA_INVALID_EX_BC_cA } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_2_6_1_EX_BC_PLC | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA_INVALID_EX_BC_PLC } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_2_6_1_EX_CP | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA_INVALID_EX_CP } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_2_6_1_EX_KU | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA_INVALID_EX_KU } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_2_6_1_SIG | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA_INVALID_SIG } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_2_6_3 | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA_EXPIRED } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_3_6_1_EX_CP | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA_INVALID_EX_CP, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_3_6_1_EX_KU | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA_INVALID_EX_KU, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_3_6_1_SIG | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA_INVALID_SIG, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_3_6_1_SUB_ORG | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA_INVALID_SUB_ORG, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_3_6_1_SUB_SN | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA_INVALID_SUB_SN, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_3_6_3 | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA_EXPIRED, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_6_1_CHA | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE_INVALID>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_1_6_1_SIG | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1_INVALID>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_2_5_4_3 | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2_PPR2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_10_1_3_9 | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <INVALID_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_DEF_DP_UC_8_11_1_3_9 | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA_UNKNOWN } |
| AUTH_SERVER_RESP_DEF_DP_UC_OK | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2 ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_SMDP_MATCHING_ID_EMPTY | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_SMDP_MATCHING_ID_OMITTED | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2 ctxParams1 #CTX_PARAMS1_MATCHING_ID_OMITTED }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_SMDS_UC_OK | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_SMDS }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_SMDS_UC_OK_EID2 | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DP_ADDRESS, serverChallenge <SMDP_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_SMDS }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA_EID2, eumCertificate #CERT_EUM_ECDSA } |
| CS_RESP_ERROR_8_1_6_1 | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #IUT_SM_DP_OID, reason postponed }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE_INVALID> } |
| CS_RESP_ERROR_8_8_3_10 | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid <INVALID_SM_DP_OID>, reason postponed }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_RESP_ERROR_8_10_1_3_9 | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <INVALID_TRANSACTION_ID>, smdpOid #IUT_SM_DP_OID, reason postponed }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_RESP_OK_EU_REJ | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #IUT_SM_DP_OID, reason endUserRejection }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_RESP_OK_L_BPP_EXE_ERROR | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #IUT_SM_DP_OID, reason loadBppExecutionError }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_RESP_OK_M_DATA_MISMATCH | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #IUT_SM_DP_OID, reason metadataMismatch }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_RESP_OK_POSTPONED | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #IUT_SM_DP_OID, reason postponed }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_RESP_OK_PPR_NOT_ALLOWED | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #IUT_SM_DP_OID, reason pprNotAllowed }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_RESP_OK_TIMEOUT | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #IUT_SM_DP_OID, reason timeout }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_RESP_OK_UNDEFINED | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #IUT_SM_DP_OID, reason undefinedReason }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CTX_PARAMS1_ACT_CODE | ctx CtxParams1 ::= ctxParamsForCommonAuthentication : { matchingId #MATCHING_ID_1, deviceInfo #S_DEVICE_INFO } |
| CTX_PARAMS1_ACT_CODE_2 | ctx CtxParams1 ::= ctxParamsForCommonAuthentication : { matchingId #MATCHING_ID_2, deviceInfo #S_DEVICE_INFO } |
| CTX_PARAMS1_MATCHING_ID_EMPTY | ctx CtxParams1 ::= ctxParamsForCommonAuthentication : { matchingId #MATCHING_ID_EMPTY, deviceInfo #S_DEVICE_INFO } |
| CTX_PARAMS1_SMDS | ctx CtxParams1 ::= ctxParamsForCommonAuthentication : { matchingId <MATCHING_ID_EVENT>, deviceInfo #S_DEVICE_INFO } |
| EUICC_FIRMWARE_VER | 0x01 00 00 |
| EXT_CARD_RESOURCE_LIMITED_SPACE | The Extended Card Resource Information according to ETSI TS 102 226 and set as: 0x81 <L> #INSTALLED_PROFILES 0x82 <L> #NON_VOLATILE_MEM_LIMITED_SPACE 0x83 <L> #S_VOLATILE_MEM |
| INITIATE_AUTH_DS_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDS_SIGNED1>, "serverSignature1" : <S_SMDS_SIGNATURE1>, "euiccCiPKIdTobeUsed" : <EUICC_CI_PK_ID_TO_BE_USED>, "serverCertificate" : #CERT_S_SM_DSauth_SIG, "crlList" : #CRL_LIST } |

| INITIATE_AUTH_DS_OK_1 | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDS_SIGNED_ADDR1>, "serverSignature1" : <S_SMDS_SIGNATURE1>, "euiccCiPKIdTobeUsed" : <EUICC_CI_PK_ID_TO_BE_USED>, "serverCertificate" : #CERT_S_SM_DSauth_ SIG, "crlList" : #CRL_LIST } |
| --- | --- |
| INITIATE_AUTH_DS_OK_VARIANT_A | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDS_SIGNED1>, "serverSignature1" : <S_SMDS_SIGNATURE1>, "serverCertificate" : #CERT_S_SM_DSauth_SIG, "otherCertsInChain" : #CERT_S_SM_DS_SubCA_SIG, "crlList" : #CRL_LIST } |
| INITIATE_AUTH_INV_CERT_DS | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDS_SIGNED1>, "serverSignature1" : <S_SMDS_SIGNATURE1>, "euiccCiPKIdTobeUsed" : <EUICC_CI_PK_ID_TO_BE_USED>, -- NOTE: select the CI Key ID in highest priority from the <EUICC_CI_PK_ID_LIST_FOR_SIGNING> "serverCertificate" : #CERT_S_SM_DSauth_INV_SIGN, "crlList" : #CRL_LIST } |
| INITIATE_AUTH_INV_CI_DS | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDS_SIGNED1>, "serverSignature1" : <S_SMDS_SIGNATURE1>, "euiccCiPKIdTobeUsed" : #CI_PK_ID_INV, "serverCertificate" : #CERT_S_SM_DSauth_SIG, "crlList" : #CRL_LIST } NOTE: select and choose the #CERT_S_SM_DSauth_SIG leading to the CI Key ID in highest priority from the <EUICC_CI_PK_ID_LIST_FOR_SIGNING> |
| INITIATE_AUTH_INV_SIGN_DS | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDS_SIGNED1>, "serverSignature1" : <S_SMDS_SIGNATURE_INV>, "euiccCiPKIdTobeUsed" : <EUICC_CI_PK_ID_TO_BE_USED>, "serverCertificate" : #CERT_S_SM_DSauth_SIG, "crlList" : #CRL_LIST } -- NOTE: select the CI Key ID in highest priority from the <EUICC_CI_PK_ID_LIST_FOR_SIGNING> and choose the #CERT_S_SM_DSauth_SIG leading to the same Root CI certificate |
| INITIATE_AUTH_INV_SMDS_ADDRESS | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDS_SIGNED_INV_ADDR>, "serverSignature1" : <S_SMDS_SIGNATURE1>, "euiccCiPKIdTobeUsed" : <EUICC_CI_PK_ID_TO_BE_USED>, "serverCertificate" : #CERT_S_SM_DSauth_SIG, "crlList" : #CRL_LIST } -- NOTE: select the CI Key ID in highest priority from the <EUICC_CI_PK_ID_LIST_FOR_SIGNING> and choose the #CERT_S_SM_DSauth_SIG leading to the same Root CI certificate |
| INITIATE_AUTH_INV_CERT | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDP_SIGNED1>, "serverSignature1" : <S_SMDP_SIGNATURE1>, "euiccCiPKIdTobeUsed" : <EUICC_CI_PK_ID_TO_BE_USED>,-- NOTE: select the CI Key ID in highest priority from the <EUICC_CI_PK_ID_LIST_FOR_SIGNING> "serverCertificate" : #CERT_S_SM_DPauth_INV_SIGN, "crlList" : #CRL_LIST } |
| INITIATE_AUTH_INV_CI | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDP_SIGNED1>, "serverSignature1" : <S_SMDP_SIGNATURE1>, "euiccCiPKIdTobeUsed" : #CI_PKI_ID2, "serverCertificate" : #CERT_S_SM_DPauth, "crlList" : #CRL_LIST } NOTE: select and choose the #CERT_S_SM_DPauth_SIG leading to the CI Key ID in highest priority from the <EUICC_CI_PK_ID_LIST_FOR_SIGNING> |
| INITIATE_AUTH_INV_OID | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDP_SIGNED1>, "serverSignature1" : <S_SMDP_SIGNATURE1>, "euiccCiPKIdTobeUsed" : <EUICC_CI_PK_ID_TO_BE_USED>, "serverCertificate" : #CERT_S_SM_DP2auth_SIG, "crlList" : #CRL_LIST } -- NOTE: select the CI Key ID in highest priority from the <EUICC_CI_PK_ID_LIST_FOR_SIGNING> -- NOTE: serverSignature1 SHALL be calculated correctly, using the secret key related to CERT_S_SM_DP2auth_SIG. |
| INITIATE_AUTH_INV_SIGN | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDP_SIGNED1>, "serverSignature1" : <S_SMDP_SIGNATURE_INV>, "euiccCiPKIdTobeUsed" : <EUICC_CI_PK_ID_TO_BE_USED>, "serverCertificate" : #CERT_S_SM_DPauth_SIG, "crlList" : #CRL_LIST } -- NOTE: select the CI Key ID in highest priority from the <EUICC_CI_PK_ID_LIST_FOR_SIGNING> and choose the #CERT_S_SM_DPauth_SIG leading to the same Root CI certificate |
| INITIATE_AUTH_INV_SMDP+_ADDRESS | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDP_SIGNED_INV_ADDR>, "serverSignature1" : <S_SMDP_SIGNATURE1>, "euiccCiPKIdTobeUsed" : <EUICC_CI_PK_ID_TO_BE_USED>, "serverCertificate" : #CERT_S_SM_DPauth_SIG, "crlList" : #CRL_LIST } -- NOTE: select the CI Key ID in highest priority from the <EUICC_CI_PK_ID_LIST_FOR_SIGNING> and choose the #CERT_S_SM_DPauth_SIG leading to the same Root CI certificate -- NOTE: serverSignature1 SHALL be calculated correctly, using <S_SMDP_SIGNED_INV_ADDR>. |
| INITIATE_AUTH_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDP_SIGNED1>, "serverSignature1" : <S_SMDP_SIGNATURE1>, "euiccCiPKIdTobeUsed" : <EUICC_CI_PK_ID_TO_BE_USED>, "serverCertificate" : #CERT_S_SM_DPauth_SIG, "crlList" : #CRL_LIST } |
| INITIATE_AUTH_OK_DIFF_CERT_VARIANT_A | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDP_SIGNED1>, "serverSignature1" : <S_SMDP_SIGNATURE1>, "serverCertificate" : #CERT_S_SM_DPauth_PK_CI2_SIG, "otherCertsInChain" : #CERT_S_SM_DP_SubCA_SIG, "crlList" : #CRL_LIST } |
| INITIATE_AUTH_OK_VARIANT_A | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDP_SIGNED1>, "serverSignature1" : <S_SMDP_SIGNATURE1>, "serverCertificate" : #CERT_S_SM_DPauth_SIG, "otherCertsInChain" : #CERT_S_SM_DP_SubCA_SIG, "crlList" : #CRL_LIST } |
| INITIATE_AUTH_OK_VARIANT_B | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDP_SIGNED1>, "serverSignature1" : <S_SMDP_SIGNATURE1>, "serverCertificate" : #CERT_S_SM_DPauth_SIG, “otherCertsInChain” : #CERT_CI_SubCA_SIG, “crlList” : #CRL_LIST } |
| INITIATE_AUTH_OK_VARIANT_C | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDP_SIGNED1>, "serverSignature1" : <S_SMDP_SIGNATURE1>, "serverCertificate" : #CERT_S_SM_DPauth_SIG, “otherCertsInChain” : { #CERT_S_SM_DP_SubCAList_SIG, #CERT_CI_SubCA_SIG}, “crlList” : #CRL_LIST } |
| INITIATE_AUTH_OK_VARIANT_FLEX | Depending on the certificate chain supported by the Test eUICC, either #INITIATE_AUTH_OK_VARIANT_A or #INITIATE_AUTH_OK_VARIANT_B or #INITIATE_AUTH_OK_VARIANT_C |
| MATCHING_ID_EMPTY |  |
| NON_VOLATILE_MEM_LIMITED_SPACE | '0x00 01' |
| PENDING_NOTIF_DEL1 | response PendingNotification ::= otherSignedNotification :{ tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_DEL2 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS2, iccid #ICCID_OP_PROF2 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_DEL4 | response PendingNotification ::= otherSignedNotification :{ tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS4, iccid #ICCID_OP_PROF4 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_DEL5 | response PendingNotification ::= otherSignedNotification :{ tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF5 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_DEL6 | response PendingNotification ::= otherSignedNotification :{ tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationLocalDelete }, notificationAddress #TEST_DP_ADDRESS2, iccid #ICCID_OP_PROF6 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_DIS1 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationLocalDisable }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_DIS2 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationLocalDisable }, notificationAddress #TEST_DP_ADDRESS2, iccid #ICCID_OP_PROF2 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_DIS5 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationLocalDisable }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF5 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_DIS8 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationLocalDisable }, notificationAddress #TEST_DP_ADDRESS8, iccid #ICCID_OP_PROF8 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_EN1 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationLocalEnable }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_EN2 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationLocalEnable }, notificationAddress #TEST_DP_ADDRESS2, iccid #ICCID_OP_PROF2 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_EN5 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationLocalEnable }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF5 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_EN6 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationLocalEnable }, notificationAddress #TEST_DP_ADDRESS2, iccid #ICCID_OP_PROF6 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |
| PENDING_NOTIF_IMPLICIT_DIS2 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationLocalDisable }, notificationAddress #TEST_DP_ADDRESS2, iccid #ICCID_OP_PROF2 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain <EUICC_NEXT_CERT>, otherCertsInChain <EUICC_OTHER_CERTS> } |
| PENDING_NOTIF_RPM_EN1 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationRpmEnable }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain <EUICC_NEXT_CERT>, otherCertsInChain <EUICC_OTHER_CERTS> } |
| PENDING_NOTIF_RPM_EN2 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationRpmEnable }, notificationAddress #TEST_DP_ADDRESS2, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain <EUICC_NEXT_CERT>, otherCertsInChain <EUICC_OTHER_CERTS> } |
| PP_VERSION | 0x01 00 00 |
| S_PN_PIR_OK1 | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult successResult : { aid <ISD_P_AID>, simaResponse #SIMA_RESULT_OK } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_INVALID_TRANS_ID | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <INVALID_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult successResult : { aid <ISD_P_AID>, simaResponse #SIMA_RESULT_OK } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_INCORRECT_INPUT_VALUES | response PendingNotification ::= profileInstallationResult : profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId configureISDP, errorReason incorrectInputValues } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_INVALID_SIGN | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId initialiseSecureChannel, errorReason invalidSignature } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_UNSUPPORTED_CRT | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId initialiseSecureChannel, errorReason unsupportedCrtValues } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_UNSUP_REMOTE_OP_TYPE | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId initialiseSecureChannel, errorReason unsupportedRemoteOperationType } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_UNSUP_PROFILE_CLASS | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId storeMetadata, errorReason unsupportedProfileClass } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_SCP03T_STRUCTURE_ERROR | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId storeMetadata, errorReason scp03tStructureError } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_SCP03T_SECURITY_ERROR | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId replaceSessionKeys, errorReason scp03tSecurityError } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_ICCID_ALREADY_EXISTS | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId storeMetadata, errorReason installFailedDueToIccidAlreadyExistsOnEuicc } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_INSUFFICIENT_MEMORY | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId storeMetadata, errorReason installFailedDueToInsufficientMemoryForProfile } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_INSTALL_INTERRUPTION | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId storeMetadata, errorReason installFailedDueToInterruption } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_PE_PROCESSING_ERROR | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId loadProfileElements, errorReason installFailedDueToPEProcessingError } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_DATA_MISMATCH | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId loadProfileElements, errorReason installFailedDueToDataMismatch } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_TEST_PROFILE_INVALID_NAA_KEY | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId loadProfileElements, errorReason testProfileInstallFailedDueToInvalidNaaKey } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_PPR_NOT_ALLOWED | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId storeMetadata, errorReason pprNotAllowed } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PN_PIR_UNKNOWN_ERROR | response PendingNotification ::= profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, smdpOid #IUT_SM_DP_OID, finalResult errorResult : { bppCommandId storeMetadata, errorReason installFailedDueToUnknownError } }, euiccSignPIR <EUICC_SIGN_PIR> } |
| S_PENDING_NOTIF_OTHER_INST1 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| S_PENDING_NOTIF_EN1 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationLocalEnable }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| S_PENDING_NOTIF_DIS1 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationLocalDisable }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| S_PENDING_NOTIF_DE1 | response PendingNotification ::= otherSignedNotification :{ tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationLocalDelete }, notificationAddress #IUT_SM_DP_ADDRESS, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| S_SMDP_SIGNED2 | req SmdpSigned2 ::= { transactionId <S_TRANSACTION_ID>, ccRequiredFlag FALSE } |
| S_SMDP_SIGNED2_CC | req SmdpSigned2 ::= { transactionId <S_TRANSACTION_ID>, ccRequiredFlag TRUE } |
| S_SMDP_SIGNED2_INV_TRANSACTION_ID | req SmdpSigned2 ::= { transactionId <INVALID_TRANSACTION_ID>, ccRequiredFlag FALSE } |
| S_SMDP_SIGNED2_RPM_PENDING | req SmdpSigned2 ::= { transactionId <S_TRANSACTION_ID>, ccRequiredFlag FALSE, rpmPending NULL } |
| S_SMDP_SIGNED3 | req SmdpSigned3 ::= { transactionId <S_TRANSACTION_ID>, rpmPackage #RPM_PKG_EN, rpmPending NULL } |
| S_SMDP_SIGNED3_DELETE | req SmdpSigned3 ::= { transactionId <S_TRANSACTION_ID>, rpmPackage #RPM_PKG_DELETE } |
| S_SMDP_SIGNED3_DISABLE | req SmdpSigned3 ::= { transactionId <S_TRANSACTION_ID>, rpmPackage #RPM_PKG_DISABLE } |
| S_SMDP_SIGNED3_RPM_PENDING | req SmdpSigned3 ::= { transactionId <S_TRANSACTION_ID>, rpmPackage #RPM_PKG_EN, rpmPending NULL } |
| S_SMDP_SIGNED3_UM_PPR | req SmdpSigned3 ::= { transactionId <S_TRANSACTION_ID>, rpmPackage #RPM_PKG_UM_PPR } |
| S_SMDP_SIGNED3_UM_REF_ENT_RULE | req SmdpSigned3 ::= { transactionId <S_TRANSACTION_ID>, rpmPackage #RPM_PKG_UM_REF_ENT_RULE } |
| S_VOLATILE_MEM | '0x01 00' |
| SAS_ACREDITATION_NUMBER | GSMA_SAS_123456789 |
| UICC_CAPABILITY | uiccCapability UICCCapability ::= { contactlessSupport, usimSupport, isimSupport, akaMilenage, akaTuak128, gbaAuthenUsim, eapClient, javacard, multipleUsimSupport } |
| UICC_CAPABILITY_EXT | uiccCapability UICCCapability ::= { contactlessSupport, usimSupport, isimSupport, akaMilenage, akaTuak128, gbaAuthenUsim, eapClient, javacard, multipleUsimSupport, berTlvFileSupport, dfLinkSupport, catTp, getIdentity, profile-a-x25519, profile-b-p256, suciCalculatorApi, unknownServiceSupport } Note: the definition of UICCCapability used above is equivalent to the definition in SGP.22 v2.3 (specific version of [2]) with the additional of a further field called “unknownServiceSupport” after the “suciCalculatorApi” field. |

- D.2.2	ES9+ Responses

| Name | Content |  |
| --- | --- | --- |
| AUTH_CLIENT_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "profileMetadata" : #METADATA_OP_PROF1, "smdpSigned2" : #S_SMDP_SIGNED2, "smdpSignature2" : <S_SM_DP+_SIGNATURE2>, "smdpCertificate" : #CERT_S_SM_DPpb_SIG } |  |
| AUTH_CLIENT_OK_CC | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "profileMetadata" : #METADATA_OP_PROF1, "smdpSigned2" : #S_SMDP_SIGNED2_CC, "smdpSignature2" : <S_SM_DP+_SIGNATURE2>, "smdpCertificate" : #CERT_S_SM_DPpb_SIG } |  |
| AUTH_CLIENT_OK_OLD_KEYS |  | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "profileMetadata" : #METADATA_OP_PROF1, "smdpSigned2" : #S_SMDP_SIGNED2_OLD_KEYS, "smdpSignature2" : <S_SM_DP+_SIGNATURE2>, "smdpCertificate" : #CERT_S_SM_DPpb_SIG } |
| AUTH_CLIENT_INV_PB_CERT | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "profileMetadata" : #METADATA_OP_PROF1, "smdpSigned2" : #S_SMDP_SIGNED2, "smdpSignature2" : <S_SM_DP+_SIGNATURE2>, "smdpCertificate" : #CERT_S_SM_DPpb_INV_SIGN } |  |
| AUTH_CLIENT_INV_CI | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "profileMetadata" : #METADATA_OP_PROF1, "smdpSigned2" : #S_SMDP_SIGNED2, "smdpSignature2" : <S_SM_DP+_SIGNATURE2>, "smdpCertificate" : #CERT_S_SM_DP2pb_SIG } |  |
| AUTH_CLIENT_INV_SIGN | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "profileMetadata" : #METADATA_OP_PROF1, "smdpSigned2" : #S_SMDP_SIGNED2, "smdpSignature2" : <S_SM_DP+_SIGNATURE2>, "smdpCertificate" : #CERT_S_SM_DPpb_SIG } The <S_SM_DP+_SIGNATURE2> SHALL NOT be computed using the #SK_S_SM_DPpb_SIG but SHALL have the same length as for a valid signature |  |
| AUTH_CLIENT_INV_TRANSACTION_ID | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "profileMetadata" : #METADATA_OP_PROF1, "smdpSigned2" : #S_SMDP_SIGNED2_INV_TRANSACTION_ID, "smdpSignature2" : <S_SM_DP+_SIGNATURE2>, "smdpCertificate" : #CERT_S_SM_DPpb_SIG } |  |
| AUTH_CLIENT_RPM_DELETE_OK |  | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "smdpSigned3" : #S_SMDP_SIGNED3_DELETE, "smdpSignature3" : <S_SM_DP+_SIGNATURE3> } |
| AUTH_CLIENT_RPM_DISABLE_OK |  | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "smdpSigned3" : #S_SMDP_SIGNED3_DISABLE, "smdpSignature3" : <S_SM_DP+_SIGNATURE3> } |
| AUTH_CLIENT_RPM_OK |  | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "smdpSigned3" : #S_SMDP_SIGNED3, "smdpSignature3" : <S_SM_DP+_SIGNATURE3> } |
| AUTH_CLIENT_RPM_OK_AND_RPM_PENDING |  | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "smdpSigned3" : #S_SMDP_SIGNED3_RPM_PENDING, "smdpSignature3" : <S_SM_DP+_SIGNATURE3> } |
| AUTH_CLIENT_RPM_UM_PPR_OK |  | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "smdpSigned3" : #S_SMDP_SIGNED3_UM_PPR, "smdpSignature3" : <S_SM_DP+_SIGNATURE3> } |
| AUTH_CLIENT_RPM_UM_REF_ENT_RULE_OK |  | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "smdpSigned3" : #S_SMDP_SIGNED3_UM_REF_ENT_RULE, "smdpSignature3" : <S_SM_DP+_SIGNATURE3> } |
| CS_OK_EU_LOAD_BPP_ERROR | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #S_SM_DP+_OID, reason loadBppExecutionError }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |  |
| CS_OK_EU_POSTPONED | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #S_SM_DP+_OID, reason postponed }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |  |
| CS_OK_EU_REJ | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #S_SM_DP+_OID, reason endUserRejection }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |  |
| CS_OK_PPR_NOT_ALLOWED | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #S_SM_DP+_OID, reason pprNotAllowed }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |  |
| CS_OK_RPM_DISABLED |  | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #S_SM_DP+_OID, reason rpmDisabled }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_OK_RPM_EU_REJECT |  | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #S_SM_DP+_OID, reason endUserRejection }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |
| CS_OK_TIMEOUT | resp CancelSessionResponse ::= cancelSessionResponseOk : { euiccCancelSessionSigned { transactionId <S_TRANSACTION_ID>, smdpOid #S_SM_DP+_OID, reason timeout }, euiccCancelSessionSignature <EUICC_CANCEL_SESSION_SIGNATURE> } |  |
| GET_BPP_LOAD_ERROR | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "boundProfilePackage" : BoundProfilePackage { #S_INIT_SC_PROF1, firstSequenceOf87 { #CONF_ISDP_PROF1 }, sequenceOf88 { <METADATA_OP_PROF1_SEG> … <METADATA_OP_PROF1_SEG> } } } NOTE 1: boundProfilePackage is enconded as base64 therefore the test tool SHALL decode boundProfilePackage to access the ASN.1. NOTE 2: For sequenceOf88 there will be only one or two '88' TLV segments depending on the size of StoreMetadata. |  |
| GET_BPP_LOAD_ERROR_UNKNOWN_TAG | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "boundProfilePackage" { #S_INIT_SC_PROF1, #UNKNOWN_BPP_SEGMENT, firstSequenceOf87 { #CONF_ISDP_PROF1 }, sequenceOf88 { <METADATA_OP_PROF1_SEG> … <METADATA_OP_PROF1_SEG> }, sequenceOf86 { <PPP_OP_PROF1_SEG_SK> … <PPP_OP_PROF1_SEG_SK> } } } NOTE 1: boundProfilePackage is encoded as base64 therefore the test tool shall decode boundProfilePackage to access the ASN.1. NOTE 2: For sequenceOf88 there will be only one or two '88' TLV segments depending on the size of StoreMetadata. |  |
| GET_BPP_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "boundProfilePackage" : BoundProfilePackage { #S_INIT_SC_PROF1, firstSequenceOf87 { #CONF_ISDP_PROF1 }, sequenceOf88 { <METADATA_OP_PROF1_SEG> … <METADATA_OP_PROF1_SEG> }, sequenceOf86 { <PPP_OP_PROF1_SEG_SK> … <PPP_OP_PROF1_SEG_SK> } } } NOTE 1: boundProfilePackage is enconded as base64 therefore the test tool SHALL decode boundProfilePackage to access the ASN.1. NOTE 2: For sequenceOf88 there will be only one or two '88' TLV segments depending on the size of StoreMetadata. |  |
| GET_BPP_OK_PPK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "boundProfilePackage" : BoundProfilePackage { #S_INIT_SC_PROF1, firstSequenceOf87 { 0x87 <L> #CONF_ISDP_PROF1 }, sequenceOf88 { <METADATA_OP_PROF1_SEG> … <METADATA_OP_PROF1_SEG> }, secondSequenceOf87 { 0x87 <L> #REPLACE_S_KEYS_REQ }, sequenceOf86 { <PPP_OP_PROF1_SEG_SK> … <PPP_OP_PROF1_SEG_SK> } } } |  |
| GET_BPP_INV | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "boundProfilePackage" : BoundProfilePackage { #S_INIT_SC_PROF1, firstSequenceOf87 { 0x87 <L> #CONF_ISDP_PROF1 }, sequenceOf88 { <METADATA_OP_PROF1_SEG> … <METADATA_OP_PROF1_SEG> }, sequenceOf86 { <PPP_OP_PROF1_SEG_SK_INV> … <PPP_OP_PROF1_SEG_SK_INV> } } } |  |
| NOTIF_METADATA_PROF1_DP1_RPR (NotificationMetadata) |  | { seqNumber <NOTIF_SEQ_NO_PROF1_RPR>, profileManagementOperation { loadRpmPackageResult }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 } |
| PENDING_NOTIF_INST_ADDRESS2 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS2, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |  |
| PENDING_NOTIF_INST1 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |  |
| R_AUTH_SERVER_DS_MATCH_ID_DEV_INFO | resp AuthenticateServerResponse ::authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #TEST_ROOT_DS_ADDRESS, serverChallenge <S_SMDS_CHALLENGE>, euiccInfo2 <R_EUICC_INFO2>, ctxParams1 #CTX_PARAMS1_MATCH_ID_DEV_INFO }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |  |
| R_AUTH_SERVER_DS_MATCH_ID_DEV_INFO_1 | resp AuthenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #TEST_DS_ADDRESS1, serverChallenge <S_SMDS_CHALLENGE>, euiccInfo2 <R_EUICC_INFO2>, ctxParams1 #CTX_PARAMS1_MATCH_ID_DEV_INFO }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |  |
| R_AUTH_SERVER_MATCH_ID_DEV_INFO | resp AuthenticateServerResponse ::= authenticateResponseOk { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #TEST_DP_ADDRESS1, serverChallenge <S_SMDP_CHALLENGE>, euiccInfo2 <R_EUICC_INFO2>, ctxParams1 #CTX_PARAMS1_MATCH_ID_DEV_INFO }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain #CERT_EUM_SIG } |  |
| R_AUTH_SERVER_MATCH_ID_DEV_INFO_V3 |  | resp AuthenticateServerResponse ::= authenticateResponseOk { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #TEST_DP_ADDRESS1, serverChallenge <S_SMDP_CHALLENGE>, euiccInfo2 <R_EUICC_INFO2>, ctxParams1 #CTX_PARAMS1_MATCH_ID_DEV_INFO_V3 }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_SIG, nextCertInChain <EUICC_NEXT_CERT>, otherCertsInChain <EUICC_OTHER_CERTS> } |
| R_GET_BPP_RESP_OP1_PPK (Pre-generated PPP for Profiles) | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId": <TRANSACTION_ID_GBPP>, "boundProfilePackage" : BoundProfilePackage { #INIT_SC_PROF1, firstSequenceOf87 { <CONF_ISDP_PROF1_ENC> }, sequenceOf88 { <SMDP_METADATA_SEG_MAC> … <SMDP_METADATA_SEG_MAC> }, secondSequenceOf87 { <REPLACE_S_KEYS_REQ_ENC> }, sequenceOf86 { <PPP_OP_PROF1_SEG_PPK> … <PPP_OP_PROF1_SEG_PPK> } } } NOTE 1: boundProfilePackage is enconded as base64 therefore the test tool SHALL decode boundProfilePackage to access the ASN.1. NOTE 2: For sequenceOf88 there will be only one or two '88' TLV segments depending on the size of StoreMetadata. |  |
| R_GET_BPP_RESP_OP1_SK (Dynamically-generated PPP for Profiles) | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId": <TRANSACTION_ID_GBPP>, "boundProfilePackage" : BoundProfilePackage { #INIT_SC_PROF1, firstSequenceOf87 { <CONF_ISDP_PROF1_ENC> }, sequenceOf88 { <SMDP_METADATA_SEG_MAC> … <SMDP_METADATA_SEG_MAC> }, sequenceOf86 { <PPP_OP_PROF1_SEG_SK> … <PPP_OP_PROF1_SEG_SK> } } } NOTE 1: boundProfilePackage is enconded as base64 therefore the test tool SHALL decode boundProfilePackage to access the ASN.1. NOTE 2: For sequenceOf88 there will be only one or two '88' TLV segments depending on the size of StoreMetadata. |  |
| R_HTTP_204_OK | HTTP/1.1 204 No Content X-Admin-Protocol: gsma/rsp/v<2.1.0> NOTE: If the HTTP response is being received from the server under test, then the "Content-type" header MAY be present. |  |
| R_PIR_OK | response ProfileInstallationResult ::= { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 }, smdpOid #S_SM_DP+_OID, finalResult successResult : { aid <ISD_P_AID>, ppiResponse #SIMA_RESULT_OK } }, euiccSign <EUICC_SIGN_PIR> } |  |
| R_PIR_SECU_INVALID | resp ProfileInstallationResult ::= { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, … smdpOid #S_SM_DP+_OID, finalResult errorResult : { bppCommandId loadProfileElements, errorReason incorrectInputValues OR bspStructureError OR bspSecurityError … } }, euiccSign <EUICC_SIGN_PIR> } |  |
| SMDP_METADATA_OP_PROF1 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileClass operational } |  |
| SMDP_METADATA_OP_PROF1_2_SEG | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1_2_SEG, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationLocalEnable, notificationLocalDisable, notificationLocalDelete }, notificationAddress #IUT_SM_DP_ADDRESS } }, profileOwner { mccMnc #MCC_MNC1 } } |  |
| SMDP_METADATA_OP_PROF3 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF3, serviceProviderName #SP_NAME3, profileName #NAME_OP_PROF3, profileClass operational, profileOwner { mccMnc #MCC_MNC2 }, profilePolicyRules { ppr2 } } |  |
| SMDP_SIGNED2 | smdpSigned2 SmdpSigned2 ::= { transactionId <TRANSACTION_ID_SIGNED_AC>, ccRequiredFlag FALSE } |  |
| SMDP_SIGNED2_CC | smdpSigned2 SmdpSigned2 ::= { transactionId <TRANSACTION_ID_SIGNED_AC>, ccRequiredFlag TRUE } |  |
| SMDP_SIGNED2_CC_RETRY | smdpSigned2 SmdpSigned2 ::= { transactionId <TRANSACTION_ID_SIGNED_AC>, ccRequiredFlag TRUE, bppEuiccOtpk <BPP_OTPK_EUICC_ECKA> } |  |
| SMDP_SIGNED2_RETRY | smdpSigned2 SmdpSigned2 ::= { transactionId <TRANSACTION_ID_SIGNED_AC>, ccRequiredFlag FALSE, bppEuiccOtpk <BPP_OTPK_EUICC_ECKA> } |  |

- D.3	VOID

- D.4	VOID

- D.5	VOID

- D.6	ES11 Requests And Responses

- D.6.1	ES11 Requests

| Name | Content |
| --- | --- |
| AUTH_SERVER_RESP_MATCHING_ID_EMPTY | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_MATCHING_ID_EVENT_ID | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EVENT_ID }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_MATCHING_ID_EVENT_ID_R | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EVENT_ID_R }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_MATCHING_ID_OMITTED | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_OMITTED }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_SMDS_8_1_2_6_1_EX_BC_cA | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA_INVALID_EX_BC_cA } |
| AUTH_SERVER_RESP_SMDS_8_1_2_6_1_EX_BC_PLC | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA_INVALID_EX_BC_PLC } |
| AUTH_SERVER_RESP_SMDS_8_1_2_6_1_EX_CP | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA_INVALID_EX_CP } |
| AUTH_SERVER_RESP_SMDS_8_1_2_6_1_EX_KU | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA_INVALID_EX_KU } |
| AUTH_SERVER_RESP_SMDS_8_1_2_6_1_SIG | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA_INVALID_SIG } |
| AUTH_SERVER_RESP_SMDS_8_1_2_6_3 | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA_EXPIRED } |
| AUTH_SERVER_RESP_SMDS_8_1_3_6_1_EX_CP | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA_INVALID_EX_CP, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_SMDS_8_1_3_6_1_EX_KU | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA_INVALID_EX_KU, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_SMDS_8_1_3_6_1_SIG | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA_INVALID_SIG, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_SMDS_8_1_3_6_1_SUB_ORG | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA_INVALID_SUB_ORG, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_SMDS_8_1_3_6_1_SUB_SN | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA_INVALID_SUB_SN, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_SMDS_8_1_3_6_3 | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA_EXPIRED, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_SMDS_8_1_6_1_CHA | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE_INVALID>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_SMDS_8_1_6_1_SIG | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1_INVALID>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| AUTH_SERVER_RESP_SMDS_8_10_1_3_9 | resp authenticateServerResponse ::= authenticateResponseOk : { euiccSigned1 { transactionId <INVALID_TRANSACTION_ID>, serverAddress #IUT_SM_DS_ADDRESS_ES11, serverChallenge <SMDS_CHALLENGE>, euiccInfo2 #S_EUICC_INFO2, ctxParams1 #CTX_PARAMS1_MATCHING_ID_EMPTY }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |
| CTX_PARAMS1_MATCHING_ID_EVENT_ID (CtxParams1) | ctxParamsForCommonAuthentication : { matchingId #EVENT_ID_1, deviceInfo #S_DEVICE_INFO } |
| CTX_PARAMS1_MATCHING_ID_EVENT_ID_R (CtxParams1) | ctxParamsForCommonAuthentication : { matchingId <EVENT_ID_R>, deviceInfo #S_DEVICE_INFO } |
| CTX_PARAMS1_MATCHING_ID_OMITTED (CtxParams1) | ctxParamsForCommonAuthentication : { deviceInfo #S_DEVICE_INFO } |

- D.6.2	ES11 Responses

| Name | Content |
| --- | --- |
| AUTH_CLIENT_DS_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "eventEntries" : #EVENT_ENTRY } |
| AUTH_CLIENT_DS_OK1 | { "header" :{ "functionExecutionStatus":{ "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "eventEntries" : [ #EVENT_ENTRY_1 ] } |
| AUTH_CLIENT_DS_OK2 | { "header" :{ "functionExecutionStatus":{ "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "eventEntries" : [ #EVENT_ENTRY_2 ] } |
| AUTH_CLIENT_DS_OK_DSADDR1 | { "header" :{ "functionExecutionStatus":{ "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "eventEntries" : [ #EVENT_ENTRY_DSADDR1 ] } |
| EVENT_ENTRY | { "eventId" : <EVENT_ID>, "rspServerAddress" : <RSP_SERVER_ADDRESS> } |
| EVENT_ENTRY_1 | { "eventId" : #EVENT_ID_1, "rspServerAddress" : #TEST_DP_ADDRESS1 } |
| EVENT_ENTRY_1_ALT_DS | { "eventId" : #EVENT_ID_1, "rspServerAddress" : #TEST_ALT_DS_ADDRESS } |
| EVENT_ENTRY_2 | { "eventId" : #EVENT_ID_2, "rspServerAddress" : #TEST_DP_ADDRESS1 } |
| EVENT_ENTRY_DSADDR1 | { "eventId" : #EVENT_ID_1, "rspServerAddress" : #TEST_DS_ADDRESS1 } |
| EVENT_ENTRY_MULTI | { "eventId" : #EVENT_ID_1, "rspServerAddress" : #TEST_DP_ADDRESS1 }, { "eventId" : #EVENT_ID_2, "rspServerAddress" : #TEST_DP_ADDRESS2 } |
| R_AUTH_CLIENT_DS_EVENT_ENTRY_1_ALT_DS_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "eventEntries" : [#EVENT_ENTRY_1_ALT_DS] } |
| R_AUTH_CLIENT_DS_EVENT_ENTRY_1_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "eventEntries" : [#EVENT_ENTRY_1] } |
| R_AUTH_CLIENT_DS_EVENT_ENTRY_EMPTY_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "eventEntries" : [] } |
| R_AUTH_CLIENT_DS_EVENT_ENTRY_MULTI_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "eventEntries" : [#EVENT_ENTRY_MULTI] } |

- D.7	VOID

- D.8	VOID

- D.9	Common Server Responses

For all responses with a JSON component the “subjectIdentifier” and “message” are optional and may or may not be present in the response received from the RSP server.

| Name | Content |
| --- | --- |
| R_ERROR_1_2_4_2 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "1.2", "reasonCode" : "4.2" } } } } |
| R_ERROR_8_1_1_2_2 | - { - "header" : { - "functionExecutionStatus" : { - "status" : "Failed", - "statusCodeData” : { - "subjectCode" : "8.1.1", - "reasonCode" : "2.2" - } - } - } } |
| R_ERROR_8_1_1_3_8 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.1.1", "reasonCode" : "3.8" } } } } |
| R_ERROR_8_1_1_3_10 | - { - "header" : { - "functionExecutionStatus" : { - "status" : "Failed", - "statusCodeData” : { - "subjectCode" : "8.1.1", - "reasonCode" : "3.10" - } - } - } } |
| R_ERROR_8_1_3_6_1 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.1.3", "reasonCode" : "6.1" } } } } |
| R_ERROR_8_1_3_6_3 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.1.3", "reasonCode" : "6.3" } } } } |
| R_ERROR_8_1_4_8 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.1", "reasonCode" : "4.8" } } } } |
| R_ERROR_8_1_6_1 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.1", "reasonCode" : "6.1" } } } } |
| R_ERROR_8_2_1_2 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.2", "reasonCode" : "1.2" } } } } |
| R_ERROR_8_2_1_3_3 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData" : { "subjectCode" : "8.2.1", "reasonCode" : "3.3" } } } } |
| R_ERROR_8_2_1_3_9 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData" : { "subjectCode" : "8.2.1", "reasonCode" : "3.9" } } } } |
| R_ERROR_8_2_3_7 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData" : { "subjectCode" : "8.2", "reasonCode" : "3.7" } } } } |
| R_ERROR_8_2_5_4_3 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.2.5", "reasonCode" : "4.3" } } } } |
| R_ERROR_8_2_6_3_3 | - { - "header" : { - "functionExecutionStatus" : { - "status" : "Failed", - "statusCodeData” : { - "subjectCode" : "8.2.6", - "reasonCode" : "3.3" - } - } - } } |
| R_ERROR_8_2_6_3_8 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.2.6", "reasonCode" : "3.8" } } } } |
| R_ERROR_8_2_6_3_10 | - { - "header" : { - "functionExecutionStatus" : { - "status" : "Failed", - "statusCodeData” : { - "subjectCode" : "8.2.6", - "reasonCode" : "3.10" - } - } - } } |
| R_ERROR_8_2_7_2_2 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData" : { "subjectCode" : "8.2.7", "reasonCode" : "2.2" } } } } |
| R_ERROR_8_2_7_3_8 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData" : { "subjectCode" : "8.2.7", "reasonCode" : "3.8" } } } } |
| R_ERROR_8_2_7_6_4 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.2.7", "reasonCode" : "6.4" } } } } |
| R_ERROR_8_8_1_3_8 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.8.1", "reasonCode" : "3.8" } } } } |
| R_ERROR_8_8_2_3_1 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.8.2", "reasonCode" : "3.1" } } } } |
| R_ERROR_8_8_3_3_1 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.8.3", "reasonCode" : "3.1" } } } } |
| R_ERROR_8_8_3_10 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.8", "reasonCode" : "3.10" } } } } |
| R_ERROR_8_8_4_3_7 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.8.4", "reasonCode" : "3.7" } } } } |
| R_ERROR_8_8_5_4_10 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.8.5", "reasonCode" : "4.10" } } } } |
| R_ERROR_8_8_5_6_4 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.8.5", "reasonCode" : "6.4" } } } } |
| R_ERROR_8_9_1_3_8 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.9.1", "reasonCode" : "3.8" } } } } |
| R_ERROR_8_9_2_3_1 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.9.2", "reasonCode" : "3.1" } } } } |
| R_ERROR_8_9_3_3_1 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.9.3", "reasonCode" : "3.1" } } } } |
| R_ERROR_8_9_4_2 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.9", "reasonCode" : "4.2" } } } } |
| R_ERROR_8_9_4_3_7 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.9.4", "reasonCode" : "3.7" } } } } |
| R_ERROR_8_9_5_1 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.9", "reasonCode" : "5.1" } } } } |
| R_ERROR_8_9_5_3_3 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.9.5", "reasonCode" : "3.3" } } } } |
| R_ERROR_8_9_5_3_9 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.9.5", "reasonCode" : "3.9" } } } } |
| R_ERROR_8_10_1_3_9 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.10.1", "reasonCode" : "3.9" } } } } |
| R_ERROR_8_11_1_3_9 | { "header" : { "functionExecutionStatus" : { "status" : "Failed", "statusCodeData” : { "subjectCode" : "8.11.1", "reasonCode" : "3.9" } } } } |
| R_ERROR_ANY | - { - "header" : { - "functionExecutionStatus" : { - "status" : "Failed", - "statusCodeData” : { - "subjectCode" : <SUBJECT_CODE_ANY>, - "reasonCode" : <REASON_CODE_ANY> - } - } - } } |
| R_SUCCESS | - { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } } } |

- D.10	ES2+ Requests And Responses

D.10.1 ES2+ Requests

D.10.2 ES2+ Responses

| Name | Content |
| --- | --- |
| R_SUCCESS_ICCID1 | - { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } } - "iccid" : "#ICCID_OP_PROF1" } |
| R_SUCCESS_MATCHING_ID | - { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } } "matchingId" : <MATCHING_ID> - } |
| R_SUCCESS_MATCHING_ID_EID | - { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } } "matchingId" : <MATCHING_ID> - "eid" : "#EID1" - } |

- Annex E	Profiles

| Profile | GENERIC_PROFILE_STRUCTURE |
| --- | --- |
| Description | Generic Operational Profile ASN.1 structure to be used as a basis for all Profiles used in this specification. |
| Details | headerValue ProfileElement ::= header : { major-version 2, minor-version 3, profileType "GSMA Profile Package", iccid '89019990001234567893'H, eUICC-Mandatory-services { usim NULL, milenage NULL }, eUICC-Mandatory-GFSTEList { -- see Note 1 id-MF, id-USIM } } mfValue ProfileElement ::= mf : { mf-header { mandated NULL, identification 1 }, templateID id-MF, mf { fileDescriptor : { pinStatusTemplateDO '01020A'H } }, ef-pl { fileDescriptor : { -- EF PL modified to use Access Rule 15 within EF ARR securityAttributesReferenced '0F'H } }, ef-iccid { -- swapped ICCID: 98109909002143658739 fillFileContent '98109909002143658739'H }, ef-dir { fileDescriptor { -- Shareable Linear Fixed File -- 4 records, record length: 38 bytes fileDescriptor '42210026'H, efFileSize '98'H }, -- USIM AID: A0000000871002FF33FF018900000100 fillFileContent '61184F10A0000000871002FF33FF01890000010050045553494D'H }, ef-arr { fileDescriptor : { fileDescriptor '42210025'H, lcsi '05'H, efFileSize '022B'H }, fillFileContent : '8001019000800102A406830101950108800158A40683010A950108'H, fillFileOffset : 10, fillFileContent : '800101A40683010195010880015AA40683010A950108'H, fillFileOffset : 15, fillFileContent : '80015BA40683010A950108'H, fillFileOffset : 26, fillFileContent : '800101900080015A9700'H, fillFileOffset : 27, fillFileContent : '800103A406830101950108800158A40683010A950108'H, fillFileOffset : 15, fillFileContent : '800111A40683010195010880014AA40683010A950108'H, fillFileOffset : 15, fillFileContent : '800103A406830101950108800158A40683010A950108840132A406830101950108'H, fillFileOffset : 4, fillFileContent : '800101A406830101950108800102A406830181950108800158A40683010A950108'H, fillFileOffset : 4, fillFileContent : '800101900080011AA406830101950108800140A40683010A950108'H, fillFileOffset : 10, fillFileContent : '800101900080015AA40683010A950108'H, fillFileOffset : 21, fillFileContent : '8001019000800118A40683010A9501088001429700'H, fillFileOffset : 16, fillFileContent : '800101A40683010195010880015A9700'H, fillFileOffset : 21, fillFileContent : '800113A406830101950108800148A40683010A950108'H, fillFileOffset : 15, fillFileContent : '80015EA40683010A950108'H, fillFileOffset : 26, fillFileContent '8001019000800102A010A40683010195 0108A406830102950108800158A40683 010A950108'H } } pukVal ProfileElement ::= pukCodes : { puk-Header { mandated NULL, identification 2 }, pukCodes { { keyReference pukAppl1, pukValue '3030303030303030'H, -- maxNumOfAttemps:9, retryNumLeft:9 maxNumOfAttemps-retryNumLeft 153 }, { keyReference pukAppl2, pukValue '3132333435363738'H }, { keyReference secondPUKAppl1, pukValue '3932393435363738'H, -- maxNumOfAttemps:8, retryNumLeft:8 maxNumOfAttemps-retryNumLeft 136 } } } pinVal ProfileElement ::= pinCodes : { pin-Header { mandated NULL, identification 3 }, pinCodes pinconfig : { { keyReference pinAppl1, pinValue '31323334FFFFFFFF'H, unblockingPINReference pukAppl1 }, { keyReference pinAppl2, pinValue '30303030FFFFFFFF'H, unblockingPINReference pukAppl2 }, { keyReference adm1, pinValue '35363738FFFFFFFF'H, pinAttributes 1 } } } usimValue ProfileElement ::= usim : { usim-header { mandated NULL, identification 4 }, templateID id-USIM, adf-usim { fileDescriptor : { fileID '7FF1'H, dfName 'A0000000871002FF33FF018900000100'H, pinStatusTemplateDO '01810A'H } }, ef-imsi { -- numerical format: 234101943787656 fillFileContent '082943019134876765'H }, ef-arr { fileDescriptor { linkPath '2F06'H } }, ef-ust { -- Service Dialling Numbers, Short Message Storage… fillFileContent '0A2E178CE73204000000000000'H }, ef-spn { -- ASCII format: "GSMA eUICC" fillFileContent '0247534D41206555494343FFFFFFFFFFFF'H }, ef-est { -- Services deactivated fillFileContent '00'H }, ef-acc { -- Access class 4 fillFileContent '0040'H }, ef-ecc { -- Emergency Call Code 911 fillFileContent '19F1FF01'H } } usimPin ProfileElement ::= pinCodes : { pin-Header { mandated NULL, identification 5 }, pinCodes pinconfig : { { keyReference secondPINAppl1, pinValue '39323338FFFFFFFF'H unblockingPINReference secondPUKAppl1, -- PIN is Enabled pinAttributes 1, -- maxNumOfAttemps:2, retryNumLeft:2 maxNumOfAttemps-retryNumLeft 34 } } } akaParamValue ProfileElement ::= akaParameter : { aka-header { mandated NULL, identification 6 }, algoConfiguration algoParameter : { algorithmID milenage, -- RES and MAC 64 bits, CK and IK 128 bits algorithmOptions '01'H, key '000102030405060708090A0B0C0D0E0F'H, opc '0102030405060708090A0B0C0D0E0F00'H, -- rotationConstants uses default: '4000204060'H -- xoringConstants uses default value authCounterMax '010203'H } -- sqnOptions uses default: '02'H -- sqnDelta uses default: '000010000000'H -- sqnAgeLimit uses default: '000010000000'H -- sqnInit uses default: all bytes zero } mnoSdValue ProfileElement ::= securityDomain : { sd-Header { mandated NULL, identification 7 }, instance { applicationLoadPackageAID 'A0000001515350'H, classAID 'A000000151535041'H, instanceAID 'A000000151000000'H, applicationPrivileges '82FC80'H, -- Secured lifeCycleState '0F'H, -- SCP80 supported applicationSpecificParametersC9 '810280008201F08701F0'H, -- other parameters MAY be necessary applicationParameters { -- TAR: B20100, MSL: 12 uiccToolkitApplicationSpecificParametersField '0100000100000002011203B2010000'H } }, keyList { { -- C-ENC + R-ENC keyUsageQualifier '38'H, -- ENC key keyIdentifier '01'H, keyVersionNumber '01'H, keyCompontents { { -- DES mode implicitly known (as an example) keyType '80'H, -- This value MAY be freely changed keyData '112233445566778899AABBCCDDEEFF10'H } } }, { -- C-MAC + R-MAC keyUsageQualifier '34'H, -- MAC key keyIdentifier '02'H, keyVersionNumber '01'H, keyCompontents { { -- DES mode implicitly known (as an example) keyType '80'H, -- This value MAY be freely changed keyData '112233445566778899AABBCCDDEEFF10'H } } }, { -- C-DEK + R-DEK keyUsageQualifier 'C8'H, -- data ENC key keyIdentifier '03'H, keyVersionNumber '01'H, keyCompontents { { -- DES mode implicitly known (as an example) keyType '80'H, -- This value MAY be freely changed keyData '112233445566778899AABBCCDDEEFF10'H } } }, -- AES Token Key (as an example) -- This value MAY be freely changed keyUsageQualifier '81'H, -- MAY be used by SD keyAccess '01'H, -- Key Id 01 keyIdentifier '01'H, keyVersionNumber '70'H, keyCompontents { { -- AES (16 bytes key length) -- This value MAY be freely changed keyType '88'H, -- This value MAY be freely changed keyData 'CDFE56B7B72FAE6A047341F003D7A48D'H } } }, { -- Receipt (the AES scheme SHALL be supported) keyUsageQualifier '44'H, -- MAY be used by SD keyAccess '01'H, -- Key Id 01 keyIdentifier '01'H, keyVersionNumber '71'H, keyCompontents { { -- AES (16 bytes key length) keyType '88'H, -- This value MAY be freely changed keyData '11121314212223243132333441424344'H } } } } } ssdValue ProfileElement ::= securityDomain : { sd-Header { mandated NULL, identification 8 }, instance { applicationLoadPackageAID 'A0000001515350'H, classAID 'A000000151535041'H, instanceAID 'A00000055910100102736456616C7565'H, -- by default extradited under MNO-SD -- Privileges: Security Domain + Trusted Path applicationPrivileges '808000'H, -- Personalized lifeCycleState '0F'H, -- SCP80 supported, extradiction supported applicationSpecificParametersC9 '810280008201F0'H, applicationParameters { -- TAR: 6C7565, MSL: 12 uiccToolkitApplicationSpecificParametersField '01000001000000020112036C756500'H } }, keyList { { -- C-ENC + R-ENC keyUsageQualifier '38'H, keyIdentifier '01'H, keyVersionNumber '01'H, keyCompontents { { -- DES mode implicitly known (as an example) keyType '80'H, -- This value MAY be freely changed keyData '11223344556677881122334455667788'H } } }, { -- C-MAC + R-MAC keyUsageQualifier '34'H, -- MAC key keyIdentifier '02'H, keyVersionNumber '01'H, keyCompontents { { -- DES mode implicitly known (as an example) keyType '80'H, -- This value MAY be freely changed keyData '11223344556677881122334455667788'H } } }, { -- C-DEK + R-DEK keyUsageQualifier 'C8'H, -- data ENC key keyIdentifier '03'H, keyVersionNumber '01'H, keyCompontents { { -- DES mode implicitly known (as an example) keyType '80'H, -- This value MAY be freely changed keyData '11223344556677881122334455667788'H } } } } } rfmUicc ProfileElement ::= rfm : { rfm-header { identification 11 }, -- Instance AID instanceAID ' A00000055910100001'H, tarList { 'B00000'H }, -- cryptographic checksum + counter higher minimumSecurityLevel '12'H, -- full access uiccAccessDomain '00'H, -- full access uiccAdminAccessDomain '00'H } rfmUsim ProfileElement ::= rfm : { rfm-header { identification 12 }, -- Instance AID instanceAID 'A00000055910100002'H, tarList { 'B00020'H }, -- cryptographic checksum + counter higher minimumSecurityLevel '12'H, -- full access uiccAccessDomain '00'H, -- full access uiccAdminAccessDomain '00'H, adfRFMAccess { adfAID 'A0000000871002FF33FF018900000100'H, -- UICC access condition: ADM1 adfAccessDomain '02000100'H, -- UICC access condition: ADM1 adfAdminAccessDomain '02000100'H } } endValue ProfileElement ::= end : { end-header { mandated NULL, identification 99 } } |
| Note 1: The following OIDs are used: id-MF OBJECT IDENTIFIER ::= {joint-iso-itu-t(2) international-organizations(23) simalliance(143) euicc-profile(1) template(2) mf(1)} id-USIM OBJECT IDENTIFIER ::= {joint-iso-itu-t(2) international-organizations(23) simalliance(143) euicc-profile(1) template(2) usim(4)} |  |

| Profile | PROFILE_OPERATIONAL1 |
| --- | --- |
| Description | Operational Profile This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF1, except if defined differently in the test sequence. The Unprotected Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE except that: -  the iccid field SHALL be set to #ICCID_OP_PROF1 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF1 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF1 -  the pinAttributes of pinAppl1 present in the PE_PIN SHALL be set to 6 -  the SCP80 encryption key configured in the PE-SecurityDomain that corresponds to the MNO-SD SHALL be set to #MNO_SCP80_ENC_KEY -  the SCP80 message authentication key configured in the PE-SecurityDomain that corresponds to the MNO-SD SHALL be set to #MNO_SCP80_AUTH_KEY -  the SCP80 data encryption key configured in the PE-SecurityDomain that corresponds to the MNO-SD SHALL be set to #MNO_SCP80_DATA_ENC_KEY -  the instance AID configured in the PE-SecurityDomain that corresponds to the Supplementary Security Domain PE_SSD SHALL be set to #SSD_AID -  the ef-dir present in the PE-MF SHALL be configured with the AID #USIM_AID -  the ef-ust SHALL be set in accordance to #EF_UST1 (service 17 and 18 are not available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H The PROFILE_OPERATIONAL1 UPP is named #UPP_OP_PROF1 in the scope of this document. |

| Profile | PROFILE_OPERATIONAL2 |
| --- | --- |
| Description | Operational Profile This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF2, except if defined differently in the test sequence. The Unprotected Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE except that: -  the iccid field SHALL be set to #ICCID_OP_PROF2 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF2 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF2 -  The pinAttributes of pinAppl1 present in the PE_PIN SHALL be set to 6 -  the ef-ust SHALL be set in accordance to #EF_UST1 (service 17 and 18 are not available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H The PROFILE_OPERATIONAL2 UPP is named #UPP_OP_PROF2 in the scope of this document. |

| Profile | PROFILE_OPERATIONAL3 |
| --- | --- |
| Description | Operational Profile with PPR2 but without notification This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF3, except if defined differently in the test sequence. The Unprotected Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE except that: -  the iccid field SHALL be set to #ICCID_OP_PROF3 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF3 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF3 -  the pinAttributes of pinAppl1 present in the PE_PIN SHALL be set to 6 -  the ef-ust SHALL be set in accordance to #EF_UST1 (service 17 and 18 are not available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H The PROFILE_OPERATIONAL3 UPP is named #UPP_OP_PROF3 in the scope of this document. |

| Profile | PROFILE_OPERATIONAL4 |
| --- | --- |
| Description | Operational Profile with PPR1 and notification This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF4, except if defined differently in the test sequence. The Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE] except that: -  the iccid field SHALL be set to #ICCID_OP_PROF4 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF4 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF4 -  the pinAttributes of pinAppl1 present in the PE_PIN SHALL be set to 6 -  the ef-ust SHALL be set in accordance to #EF_UST1 (service 17 and 18 are not available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H The PROFILE_OPERATIONAL4 UPP is named #UPP_OP_PROF4 in the scope of this document. |

| Profile | PROFILE_OPERATIONAL5 |
| --- | --- |
| Description | Operational Profile with pinAppl1 enabled. This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF5, except if defined differently in the test sequence. The Unprotected Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE except that: -  the iccid field SHALL be set to #ICCID_OP_PROF5 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF5 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF5 -  the pinAppl1 present in the PE_PIN SHALL be enabled and has the value #PO1_PIN1 -  the ef-ust SHALL be set in accordance to #EF_UST1 (service 17 and 18 are not available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H |

| Profile | PROFILE_OPERATIONAL6 |
| --- | --- |
| Description | Operational Profile with pinAppl1 enabled. This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF6, except if defined differently in the test sequence. The Unprotected Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE except that: -  the iccid field SHALL be set to #ICCID_OP_PROF6 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF6 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF6 -  The pinAppl1 present in the PE_PIN SHALL be enabled and has the value #PO2_PIN1 -  the ef-ust SHALL be set in accordance to #EF_UST1 (service 17 and 18 are not available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H |

| Profile | PROFILE_OPERATIONAL7 |
| --- | --- |
| Description | Operational Profile with PPR2 and notification This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF7, except if defined differently in the test sequence. The Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE except that: -  the iccid field SHALL be set to #ICCID_OP_PROF7 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF7 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF7 -  the pinAttributes of pinAppl1 present in the PE_PIN SHALL be set to 6 -  the ef-ust SHALL be set in accordance to #EF_UST1 (service 17 and 18 are not available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H |

| Profile | PROFILE_OPERATIONAL8 |
| --- | --- |
| Description | Operational Profile with PPR2, pinAppl1 enabled and notification This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF8, except if defined differently in the test sequence. The Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE except that: -  the iccid field SHALL be set to #ICCID_OP_PROF8 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF8 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF8 -  The pinAppl1 present in the PE_PIN SHALL be enabled and has the value #PO2_PIN1 -  the ef-ust SHALL be set in accordance to #EF_UST1 (service 17 and 18 are not available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H |

| Profile | PROFILE_OPERATIONAL9 |
| --- | --- |
| Description | Operational Profile with GID1 and GID2 set This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF9, except if defined differently in the test sequence. The Unprotected Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE except that: -  the iccid field SHALL be set to #ICCID_OP_PROF9 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF9 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF9 -  the pinAppl1 present in the PE_PIN SHALL be enabled and has the value #PO1_PIN1 -  the ef-ust SHALL be set to #EF_UST2 (service 17 and 18 are available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H -  the following new Profile Element PE_OPT_USIM SHALL be inserted right after PE_USIM: PE_OPT_USIM. optusimValue ProfileElement ::= opt-usim : { optusim-header { mandated NULL, identification 15 }, templateID id-OPT-USIM, ef-gid1 { fileDescriptor { efFileSize '04'H }, fillFileContent #GID1 }, ef-gid2 { fileDescriptor { efFileSize '04'H }, fillFileContent #GID2 } }. NOTE : The following OIDs are used: id-OPT-USIM OBJECT IDENTIFIER ::= {joint-iso-itu-t(2) international-organizations(23) simalliance(143) euicc-profile(1) template(2) opt-usim(5)}. The PROFILE_OPERATIONAL9 UPP is named #UPP_OP_PROF9 in the scope of this document. |

- Annex F	IUT Settings

- F.1	VOID

- F.2	VOID

- F.3	Device Settings

| Device Setting name | Description |
| --- | --- |
| IUT_CAT_CLASSES | The bit string to indicate the set of supported Card Application Toolkit letter classes as defined in TS 102 223. |
| IUT_CDMA2000_1X_REL | If cdma2000 1X is supported, this SHALL be encoded as the octet string {1, 0, 0}. |
| IUT_CDMA2000_EHRPD_REL | If cdma2000 eHRPD, is supported this SHALL be the highest 3GPP release N fully supported by the Device, encoded as the octet string {N, 0, 0}. |
| IUT_CDMA2000_HRPD_REL | If cdma2000 HRPD is supported, this SHALL be encoded as the octet string {R, 0, 0}. The value R SHALL represent the EVDO revision as follows: Rev 0 SHALL be encoded as 1 Rev A SHALL be encoded as 2 Rev B SHALL be encoded as 3 |
| IUT_EU_CONFIRMATION_TIMEOUT | Timeout in seconds for LPAd for the End User Intent confirmation starting when the LPAd displays the dialog for confirmation. |
| IUT_EUICC_FORM_FACTOR | Indicates whether the eUICC is removable (0) or non-removable (1) |
| IUT_EUTRAN_5GC_REL | If EUTRAN 5GC is supported, this SHALL be the highest 3GPP release N supported by the Device for E-UTRAN 5GC core access and encoded as the octet string {N, 0, 0}. |
| IUT_GSM_GERAN_REL | If GSM/GERAN is supported, this is the highest 3GPP release N fully supported by the Device, encoded as the octet string {N, 0, 0}. |
| IUT_IMEI | International Mobile Equipment Identity value of the Device in human readable format, including the check digit. The value is used as a reference for verification of the TAC (mandatory) and IMEI (optional) retrieved from DeviceInfo. |
| IUT_RSP_VERSION_HIGHEST | The highest version of SGP.22 supported by the LPA encoded as the value part of an ASN.1 VersionType (e.g. 0x03 01 00) |
| IUT_LPAd_Confirmation | Description of the way to perform Strong Confirmation. |
| IUT_LPAd_CI | CI subjectPublicKeyInfo of CERT.CI.SIG (used to verify CERT.DP.TLS) stored in LPAd. Based on NIST [11] in this version of specification. |
| IUT_LPAd_NOTIFICATION_TIMEOUT | Timeout in seconds for LPAd to send a Notification to the SM-DP+ on ES9+ interface assuming IP connection is available. |
| IUT_LPAd_READY_AFTER_REBOOT_TIMEOUT | Timeout in seconds for the LPAd to be ready after a reboot. The time starts from the power off at the start of the reboot and ends when the LPAd is ready after the reboot. |
| IUT_LPAd_SESSION_CLOSE_TIMEOUT | Timeout in seconds for LPAd to send a next command for Profile Download to the SM-DP+ (or SM-DS) on ES9+ (or ES11) interface assuming IP connection is available. The timeout SHALL start after sending of the previous command by the LPAd. |
| IUT_LTE_EUTRAN_REL | If LTE/E-UTRAN is supported, this SHALL be the highest 3GPP release N fully supported by the Device, encoded as the octet string {N, 0, 0}. |
| IUT_NFC_REL | If NFC is supported, this SHALL be the highest (version, revision) number of TS.26 [15], encoded as the octet string {version, revision, 0}. |
| IUT_NR_5GC_REL | If NR 5GC is supported, this SHALL be the highest 3GPP release N supported by the Device for NR 5GC core access and encoded as the octet string {N, 0, 0}. |
| IUT_NR_EPC_REL | If NR EPC is supported, this SHALL be the highest 3GPP release N supported by the Device for NR EPC core access and encoded as the octet string {N, 0, 0}. |
| IUT_TLS_VERSION | Highest TLS protocol version supported by LPAd, at least v1.2. By versions higher then TLS v1.2 backwards compatibility is assumed. |
| IUT_UMTS_UTRAN_REL | If UMTS/UTRAN is supported, this SHALL be the highest 3GPP release N fully supported by the Device, encoded as the octet string {N, 0, 0}. |

- F.4	VOID

- Annex G	Initial States

Unless it is defined differently in a particular test case, the IUTs SHALL be set in the following initial state before the test case execution.

- G.1	Device

- G.1.1 Device (default)

The Device is “powered on”.

The Device is in the normal execution mode after Device boot-up and Device initial configuration. The Device is NOT in the Test Mode.

The LPAd has access to the root CI key #CERT_CI_ECDSA (or the CI public key) for verification of the TLS certificates of SM-DP+ or SM-DS. No CRL is loaded.

The Device contains a Test eUICC pre-configured as defined below in G.1.3.

- G.1.2 Companion Device connected to a Primary Device

The Companion Device is connected to the Primary Device as defined by the Device vendor.

Companion Device and the connected Primary Device are “powered on”.

The Companion Device and Primary Device are in the normal execution mode (NOT in the boot-up mode).

The LPAd of the Companion Device has access to the root CI #CERT_CI_ECDSA (or the CI public key) for verification of the TLS certificates of SM-DP+ or SM-DS. No CRL is loaded.

The Companion Device contains a Test eUICC preconfigured as defined below in G.1.3.

- G.1.3 Test eUICC Settings

Depending on the test cases and on the supported options, the Test eUICC SHALL be configured according to the following Initial States.

- 	The Test eUICC is configured with the ISD-R AID #ISD_R_AID and the EID #EID1.

- 	The Test eUICC does not contain any Profile.

- 	The Test eUICC is configured with the default SM-DS address #TEST_ROOT_DS_ADDRESS.

- 	The Test eUICC contains #TEST_DP_ADDRESS1 as default SM-DP+ address.

The ECASD is configured with at least the following Keys and Certificates based on NIST P-256 [11] or on brainpoolP256r1 [8] for this version of the SGP.23:

- 	The Test eUICC’s Private Key #SK_EUICC_ECDSA (for creating ECDSA signatures)

- 	The Test eUICC’s Certificate #CERT_EUICC_ECDSA (for eUICC authentication) containing the eUICC’s Public Key #PK_EUICC_ECDSA

- 	The GSMA Certificate Issuer’s Public Key #PK_CI_ECDSA (for verifying off-card entities certificates)

- 	The Certificate of the EUM #CERT_EUM_ECDSA

Other Certificates and Keys MAY be present. No CRL is loaded on the Test eUICC.

The Test eUICC SHALL support all certificate chain variants as defined in SGP.22[2] for verification, variant O and at least one of the new certificate chain variant (variant A, B, or C) for signing.  Therefore both, euiccCiPKIdListForSigning and euiccCiPKIdListForSigningV3 SHALL be present.

The CI, identified as highest priority in euiccCiPKIdListForSigning or euiccCiPKIdListForSigningV3, is also selectable in the euiccCiPKIdListForVerification (i.e. all EUM and eUICC Certificates lead to a Root CI certificate linked to a #PK_CI_ECDSA contained in the eUICC).

This CI corresponds to the SubjectKeyIdentifier of one of the #CERT_CI_ECDSA defined in sections G.2.2 and G.2.3.

For devices supporting O_D_REMOVABLE_DOWNLOAD_PPR, the Test eUICC SHALL contain the RAT configuration specified in #PPRS_ALLOWED.

For devices supporting a removable eUICC but not supporting O_D_REMOVABLE_DOWNLOAD_PPR, the Test eUICC can be configured with any RAT.

For devices supporting a non-removable eUICC:

- For some combinations of device options, RAT configurations with certain constraints are required for some sequences, as specified below. These constraints can be satisfied using any valid RAT table; for example, Allowed Operators can be specified explicitly or using wildcards.

| Device option(s) supported | RAT configuration of Test eUICC |
| --- | --- |
| O_D_EMB_ALLOWS_PPR1_EUC_REQ | PPR1 is allowed and End User Consent is required for #MCC_MNC4 with gid1 and gid2 absent. |
| O_D_EMB_ALLOWS_PPR2_EUC_REQ | PPR2 is allowed and End User Consent is required for #MCC_MNC2 with gid1 and gid2 absent. |
| NOT O_D_EMB_ALLOWS_PPR1_EUC_REQ AND O_D_EMB_ALLOWS_PPR1_EUC_NOT_REQ | PPR1 is allowed and End User Consent is not required for #MCC_MNC4 with gid1 and gid2 absent. |
| NOT O_D_EMB_ALLOWS_PPR2_EUC_REQ AND O_D_EMB_ALLOWS_PPR2_EUC_NOT_REQ | PPR2 is allowed and End User Consent is not required for #MCC_MNC2 with gid1 and gid2 absent. |

- If none of the constraints above apply, the Test eUICC can be configured with any RAT.

- Note: in the current version of this document, it is possible to satisfy the relevant constraints above with a single RAT configuration. It is recommended to supply a single device for testing with the RAT configuration satisfying all of the relevant constraints above, rather than to supply multiple devices.

A separate Test eUICC needs to be provided for each additional RAT configuration (not used in this version of the test specification). In case the Test eUICC is non-removable the additional Device SHALL contain the same software and hardware except the Test eUICC configuration.

If the DUT indicates support of the O_D_MEP then the Test eUICC SHALL support at least one of the MEP modes (mode A-1/ mode A-2/ mode B).

- G.2	VOID

- G.3	VOID

- Annex H	Icons and QR Codes

The files for the eUICC Consumer Devices Icons and QR Codes are provided within in SGP.23_AnnexH_Icons.zip and SGP.23_AnnexH_QRCodes.zip packages, which accompany the present document.

- Annex I	Requirements

The requirements used in the specified test cases are provided within SGP_23_AnnexI_Requirements_v1_3.zip package, which accompanies the present document.

- Annex J	VOID

- Annex K	Document Management

- K.1	Document History

| Version | Date | CR No | Brief Description of Change | Entity | Approval Authority | Editor / Company |
| --- | --- | --- | --- | --- | --- | --- |
| v1.0 | 9th June 2017 |  | Initial version of SGP.23 v1.0 Test Specification |  | PSMC | Yolanda Sanz, GSMA |
| v1.1 | 28th Sept 2017 |  | Minor version of SGP.23 Test specifications |  | RSPLEN | Yolanda Sanz, GSMA |
| v1.2 | 3rd Jan 2018 |  | Minor version of SGP.23 Test specifications |  | RSPLEN | Yolanda Sanz, GSMA |
| SGP.23-2 v3.0 | 01 December 2023 |  | Divide SGP.23 in three different documents (SGP.23-1, 2 and 3) |  | ISAG | Guido Abate/STMicroelectronics |
|  |  | CR2004R00 | LPA_GetBPP_Response_Correction | LPA |  |  |
|  |  | CR2011R02 | LPA_DEVICE_INFO |  |  |  |
|  |  | CR2012R00 | LPA_AuthenticateClient_Corrections |  |  |  |
|  |  | CR2013R02 | LPA_GetBoundProfilePackage_Restart |  |  |  |
|  |  | CR2014R00 | LPA_R_EUICC_INFO1_svn |  |  |  |
|  |  | CR2016R00 | LPA_Avoid_EU_Rejection_Option |  |  |  |
|  |  | CR2024R00 | LPA_IUT_settings corrected |  |  |  |
|  |  | CR2025R00 | LPA_R_EUICC_INFO2_correction |  |  |  |
|  |  | CR2027R02 | LPA_Editorial_Constants_Definitions |  |  |  |
|  |  | CR2028R01 | LPA_SetNickname_Condtional |  |  |  |
|  |  | CR2029R00 | LPA_AC_err_16-18 |  |  |  |
|  |  | CR2030R00 | LPA_eUICC_Memory_Reset_Corrections_Clarifications |  |  |  |
|  |  | CR2032R01 | LPA_Initial_Condition_Notifications |  |  |  |
|  |  | CR2034R00 | LPA_IA_err_7,9 |  |  |  |
|  |  | CR2035R01 | LPA_Notifications_Before_Power_Off |  |  |  |
|  |  | CR2038R00 | LPA_Authenticated_Confirmation_Tested_Elsewhere |  |  |  |
|  |  | CR2039R01 | LPA_Allowed_Notification_on_PIR_Error |  |  |  |
|  |  | CR2033R00 | LPA_HTTPS_Static_RSA_DH_Checks |  |  |  |
|  |  | CR2031R02 | UL_LPA_eUICC_Memory_Reset_Multiple_Notification_Correction |  |  |  |
|  |  | CR2040R00 | LPA_CancelSession_Error_Clarifications |  |  |  |
|  |  | CR2041R01 | LPA_AddProfile_PPRs_Corrections |  |  |  |
|  |  | CR2045R01 | COMPRION_LPA_SM-DS_initial_conditions |  |  |  |
|  |  | CR2046R00 | LPA_Editorial_Direction_Corrected |  |  |  |
|  |  | CR2051R00 | LPA_5.4.1.2.11_Only_IC6_Conditional |  |  |  |
|  |  | CR2057R00 | Update_SIMalliance_Reference |  |  |  |
|  |  | CR2058R00 | Editorial_REQ_update |  |  |  |
|  |  | CR2063R01 | LPA_Multi_SM-XX |  |  |  |
|  |  | CR2064R00 | LPA_Invalid_Activation_Code |  |  |  |
|  |  | CR2066R00 | LPA_EditNickname_Initial_Condition |  |  |  |
|  |  | CR2068R01 | SM_DP+_LPA_ES11_Editorials |  |  |  |
|  |  | CR2069R01 | UL_LPA_ES11_S_SM-DP+_Availability |  |  |  |
|  |  | CR2065R00 | LPA_Delete_Acknowledge_Consequences |  |  |  |
|  |  | CR2075R02 | LPA_AddInfo_MTD_HTTP_REQ_MTD_HTTP_RESP |  |  |  |
|  |  | CR2077R0 | LPAd_Precondition_ProfileWithPPR1_enabled |  |  |  |
|  |  | CR2083R01 | LPAd_MatchingID_Empty_or_Missing |  |  |  |
|  |  | CR2084R03 | COMPRION_LPAd_SetEditNickname |  |  |  |
|  |  | CR2093R01 | LPAd_SM-X_Correction _of_Server_Responses |  |  |  |
|  |  | CR2094R02 | SM-XX_EUICC_LPA_Update_SGP.22_Version_information |  |  |  |
|  |  | CR2095R00 | LPA_Additonal_TLS_Verification_Optional |  |  |  |
|  |  | CR2100R02 | LPA_Missing_MatchingId_Section_4_4_28 | LPA |  |  |
|  |  | CR2107R00 | LPA_SM-XX_Expired_Status |  |  |  |
|  |  | CR2110R00 | LPA_CR2034_Correction |  |  |  |
|  |  | CR2111R00 | LPA_Remove_O_D_ENPROF1ST |  |  |  |
|  |  | CR2115R01 | LPA_SM-XX_ASN1_DEFAULT_Processing |  |  |  |
|  |  | CR2116R01 | LPA_SM-XX_Content-Type_UTF8 |  |  |  |
|  |  | CR2037R02 | LPA_Add_TC_MATCHING_ID_EMPTY |  |  |  |
|  |  | CR2043R00 | Update figure of scope for Devices |  |  |  |
|  |  | CR2096R01 | LPAd_Usage_Old_OTPKeys_NewTC |  |  |  |
|  |  | CR2118R01 | LPA_FieldsRenameFrom_notificationEnable_to_NotificationLocalEnable | LPA |  |  |
|  |  | CR1500R01 | LPA_Initiate_Power_Download_Via_SM-DS |  |  |  |
|  |  | CR1501R00 | LPA_Remove_Stored_On_Euicc |  |  |  |
|  |  | CR2131R01 | Device_URLs_replacement | LPA |  |  |
|  |  | CR2121R02 | Device_REQs_ReplacedBy_RefToSectionInSGP.22 | LPA |  |  |
|  |  | CR1600R03 | LPAd_TAC_Verification | LPA |  |  |
|  |  | CR1601R01 | LPAd_IMEI_Verification | LPA |  |  |
|  |  |  | Alignment with SGP.23 v1.10 Draft 8 | LPA |  |  |
|  |  | CR2192R03 | AuthenticateClientV3_RPM | LPA |  |  |
|  |  | CR2194R01 | AuthenticateClientV3_RPM_CancelSession | LPA |  |  |
|  |  | CR0193R02 | AuthenticateClientV3_RPM_userConsent | LPA |  |  |
|  |  | CR2195R00 | AuthenticateClientV3_correction | LPA |  |  |
|  |  | CR2196R01 | Initiate_Authentication_RPM | LPA |  |  |
|  |  | CR2197R02 | Initiate_Authentication_RPM_SM-DS_Event | LPA |  |  |
|  |  | CR2198R01 | Initiate_Authentication_rpmPending | LPA |  |  |
|  |  | CR11037R01 | SIMalliance References cleanup | LPA |  |  |
|  |  | CR11038R01 | LPA_fixes |  |  |  |
|  |  | CR11042R02 | eUICCMemoryReset_fixes |  |  |  |
|  |  | CR11103R01 | Update_AddProfile_with_an_enabled_PPR1_Profile | LPA |  |  |
|  |  | CR2199R04 | AuthenticateClientV3_RPM_CancelSession_2_3 | LPA |  |  |
|  |  | CR2200R01 | AuthenticateClientV3_RPM_userConsent_correction |  |  |  |
|  |  | CR2202R03 | RPM Command Execution – EnableProfile_Seq1 |  |  |  |
|  |  | CR2203R02 | RPM Command Execution – EnableProfile_Seq2 |  |  |  |
|  |  | CR2205R02 | RPM Command Execution – Delete_2_ListProfileInfo_1_UpdateMD_1 |  |  |  |
|  |  | CR2206R01 | RPM Notification – RPR_and_RPMEnable_Notifiation |  |  |  |
|  |  | CR2207R00 | RPM Command Execution – EnableProfile_Seq2_Correction |  |  |  |
|  |  | CR2201R02 | Authenticate Client Variant A for RPM – PPR Update | LPA |  |  |
|  |  | CR2204R02 | RPM Command Execution – Disable_Delete_Profile_Seq1 |  |  |  |
|  |  | CR2212R01 | RPM Command Execution – EnableProfile_with_rpmCommandResultDataError | LPA |  |  |
|  |  | CR2213R01 | RPM Command Execution – DisableProfile_with_rpmCommandResultDataError |  |  |  |
|  |  | CR2214R01 | RPM Command Execution – DeleteProfile_with_rpmCommandResultDataError |  |  |  |
|  |  | CR11401R00 | LPA_Ensure_Profile_content_and_Profile_Metadata_consistent | LPA |  |  |
|  |  | Editor’s review | Removed the Requirements columns in Test Cases and Procedures. Ordered alphabetically responses in section D.3.2. | LPA |  |  |
|  |  | Editor’s review | Action eSIMWG3.88_AP05 “to remove from the procedure when the following sentence is present “For Server testing (SM-DP+ or SM-DS), executive the following steps.” ” |  |  |  |
|  |  | CR2363R01 | TLS_Handling |  |  |  |
|  |  | CR2364R01 | HTTPS_TCs_v3_update |  |  |  |
|  |  | CR2210R02 | ConditionNbs_Update |  |  |  |
|  |  | CR2215R00 | RPM Command Execution – correction for MTD ListProfileInfo and Seq 1 |  |  |  |
|  |  | CR2216R00 | RPM Command Execution – ListProfileInfo_2_3 |  |  |  |
|  |  | CR2218R01 | RPM Command Execution – ListProfileInfo_4_5 |  |  |  |
|  |  | CR2219R01 | RPM Command Execution – UpdateMD_2_3 |  |  |  |
|  |  | CR2346R00 | Removing_references_SGP.22_v2.x |  |  |  |
|  |  | CR2361R01 | eUICCInfo_v3_Update |  |  |  |
|  |  | Editor’s review | Accepted removal of test cases for eUICC and Servers | LPA |  |  |
|  |  | CR2365R02 | InitiateAuthentication_v3_update | LPA |  |  |
|  |  | CR2366R01 | AuthenticateClient_v3_Update |  |  |  |
|  |  | CR2367R00 | Clean-up_Server_and_v3_related_content |  |  |  |
|  |  | CR2386R00 | GetBPP_Update_V3 |  |  |  |
|  |  | CR2387R00 | HandleNotification_Update_V3 |  |  |  |
|  |  | CR2388R00 | CancelSession_Update_v3 |  |  |  |
|  |  | CR2389R00 | Section_5_Minor_V3_Updates |  |  |  |
|  |  | CR2396R00 | Missing_Applicability |  |  |  |
|  |  | CR2395R01 | Test_eUICC_Req |  |  |  |
|  |  | CR2393R01 | New_TCs_MEP |  |  |  |
|  |  | CR2385R00 | v3.1_replacing_v3.0_in_references | LPA |  |  |
|  |  | CR2390R01 | QRCodeScanning_Update_v3 |  |  |  |
|  |  | CR2397R00 | Test_eUICC_Requirements_MEP |  |  |  |
|  |  | CR2399R00 | Fixing_version_3.0_to_3.1 |  |  |  |
|  |  | CR2427R00 | NoErrorUpdate | LPA |  |  |
|  |  | CR11505R00 | NoErrorUpdate | LPA |  |  |

- K.2	Other Information

| Type | Description |
| --- | --- |
| Document Owner | Yolanda Sanz, GSMA |
| Editor / Company | Guido Abate, STMicroelectronics |

It is our intention to provide a quality product for your use. If you find any errors or omissions, please contact us with your comments. You may notify us at prd@gsma.com.

Your comments or suggestions & questions are always welcome.