# SGP.33-2 IPA Test Specification

# Version 1.2

# 27 January 2025

Security Classification: Non-confidential

Access to and distribution of this document is restricted to the persons permitted by the security classification. This document is subject to copyright protection. This document is to be used only for the purposes for which it has been supplied and information contained in it must not be disclosed or in any other way made available, in whole or in part, to persons other than those permitted under the security classification without the prior written approval of the Association.

Copyright Notice

Copyright © 2025 GSM Association

Disclaimer

The GSM Association (“Association”) makes no representation, warranty or undertaking (express or implied) with respect to and does not accept any responsibility for, and hereby disclaims liability for the accuracy or completeness or timeliness of the information contained in this document. The information contained in this document may be subject to change without prior notice.

Compliance Notice

The information contain herein is in full compliance with the GSM Association’s antitrust compliance policy.

This Permanent Reference Document is classified by GSMA as an Industry Specification, as such it has been developed and is maintained by GSMA in accordance with the provisions set out in GSMA AA.35 - Procedures for Industry Specifications.

Table of Contents

1	Introduction	5

1.1	Overview	5

1.2	Scope	5

1.3	Definition of Terms	5

1.4	Abbreviations	6

1.5	Document Cross-references	6

1.6	Conventions	8

2	Testing Rules	8

2.1	Applicability	8

2.1.1	Format of the Optional Features Table	8

2.1.2	Format of the Applicability Table	8

2.1.3	Applicability and Notations	9

2.1.4	Optional Features Table	9

2.1.5	Applicability Table	10

2.2	General Consideration	13

2.2.1	Test Case Definition	13

2.2.2	Test Cases Format	13

2.2.3	General Rules for Device Testing	18

2.2.4	Pass Criteria	20

2.2.5	Future Study	20

2.2.6	Adaptation of ES9+ and ES11 test cases	20

3	Testing Architecture	21

3.1	Testing Scope	21

3.2	Testing Execution	22

3.2.3	Device/IPAd - Test Environment	23

4	Interface Compliance Testing	24

4.1	General Overview	24

4.2	IPAd Interfaces	25

4.2.1	ES10a (IPA -- eUICC): GetEuiccConfiguredAddresses	25

4.2.2	ES10b (IPA -- eUICC): LoadEUICCPackage	25

4.2.3	ES10b (IPA -- eUICC): GetEUICCInfo	25

4.2.4	ES10b (IPA -- eUICC): GetEUICCChallenge	25

4.2.5	ES10b (IPA -- eUICC): AddInitialEim	25

4.2.6	ES10b (IPA -- eUICC): eUICCMemoryReset	25

4.2.7	ES10b (IPA -- eUICC): AuthenticateServer	25

4.2.8	ES10b (IPA -- eUICC): PrepareDownload	25

4.2.9	ES10b (IPA -- eUICC): LoadBoundProfilePackage	25

4.2.10	ES10b (IPA -- eUICC): CancelSession	25

4.2.11	ES10b (IPA -- eUICC): GetCerts	25

4.2.12	ES10b (IPA -- eUICC): RetrieveNotificationList	25

4.2.13	ES10b (IPA -- eUICC): RetrieveNotificationFromList	25

4.2.14	ES10b (IPA -- eUICC): GetRAT	25

4.2.15	ES10b (IPA -- eUICC): GetProfileInfo	26

4.2.16	ES10b (IPA -- eUICC): EnableUsingDD	26

4.2.17	ES10b (IPA -- eUICC): ProfileRollBack	26

4.2.18	ES10b (IPA -- eUICC): ConfigureAutomaticProfileEnabling	26

4.2.19	ES10b (IPA -- eUICC): GetEimConfigurationData	26

4.2.20	ES10b (IPA -- eUICC): GetEID	26

4.2.21	ES9+ (IPA -- SM-DP+): InitiateAuthentication	26

4.2.22	ES9+ (IPA -- SM-DP+): GetBoundProfilePackage	28

4.2.23	ES9+ (IPA -- SM-DP+): AuthenticateClient	30

4.2.24	ES9+ (IPA – SM-DP+): HandleNotification	33

4.2.25	ES9+ (IPA – SM-DP+): CancelSession	35

4.2.26	ES9+ (IPA – SM-DP+): HTTPS	36

4.2.27	ES11 (IPA – SM-DS): InitiateAuthentication	38

4.2.28	ES11 (IPA – SM-DS): AuthenticateClient	39

4.2.29	ES11 (IPA -- SM-DS): HTTPS	40

4.2.29	ES11 (IPA -- SM-DS): HTTPS	42

4.2.30	ESipa (IPA -- EIM): InitiateAuthentication	42

4.2.31	ESipa (IPA -- EIM): GetBoundProfilePackage	42

4.2.32	ESipa (IPA -- EIM): AuthenticateClient	43

4.2.33	ESipa (IPA -- EIM): TransferEimPackage	43

4.2.34	ESipa (IPA -- EIM): GetEIMPackage	44

4.2.35	ESipa (IPA -- EIM): ProvideEimPackageResult	44

4.2.36	ESipa (IPA -- EIM): HandleNotification	44

4.2.37	ESipa (IPA -- EIM): CancelSession	45

4.3	TLS Interface	45

4.3.1	TLS, Server Authentication, TLS Establishment	45

5	Procedure - Behaviour Testing	46

5.1	General Overview	46

5.2 VOID	46

5.3 VOID	46

5.4	Device Procedures	46

5.4.1	Profile Download	46

5.4.2	VOID	57

5.4.3	VOID	57

5.4.4	Local Profile Management - Delete Profile	57

5.4.5	Profile State Management Operation - Enable Profile	63

5.4.6	Local Profile Management - Disable Profile	70

5.4.7	VOID	74

5.4.8	VOID	74

5.4.9	VOID	74

5.4.10	VOID	74

5.4.11	VOID	74

5.4.12	Local Profile Management – Set fallback attribute	74

5.4.13	Local Profile Management – Unset fallback attribute	82

6	End-to-End Testing	86

Annex A	Constants	86

A.1	Generic Constants	86

A.2	Test Certificates and Test Keys	89

Annex B	Dynamic Content	91

Annex C	Methods And Procedures	99

C.1	Methods	99

C.2	Procedures	103

Annex D	Commands And Responses	118

D.1	ES8+ Requests And Responses	118

D.1.1 ES8+ Requests	118

D.1.2 ES8+ Responses	122

D.2	ES9+ Requests And Responses	122

D.2.1	ES9+ Requests	122

D.2.2	ES9+ Responses	123

D.3 ES10x Requests And Responses	127

D.3.1	ES10x Requests	127

D.3.2	ES10x Responses	127

D.4	ESipa Requests And Responses	127

D.4.1	ESipa Requests	127

D.4.2	ESipa Responses	132

D.5	ES11 Requests And Responses	147

D.5.1	ES11 Requests	147

D.5.2	ES11 Responses	147

D.6	Common Server Responses	148

Annex E	Profiles	149

Annex F	IUT Settings	162

F.3	Device Settings	162

F.4	Common Settings	163

Annex G	Initial States	164

G.1	Device	164

G.1.1 Device (default)	164

G.1.2 Test eUICC Settings	164

Annex K	Document Management	167

K.1	Document History	167

K.2	Other Information	168

# Introduction

## 1.1	Overview

The main aim of the eSIM IoT specifications [2] & [3] is to provide solution for the Remote SIM Provisioning of IoT Devices.

This Test Plan provides a set of test cases to be used for testing the implementations of the provisioning system specifications documents [2] & [3]. This document offers to the involved entities an unified test strategy and ensures interoperability between different implementations.

## 1.2	Scope

This document is intended for:

Parties which develop test tools and platforms

Vendors (Device and eUICC Manufacturers, SM-DP+ and SM-DS Providers)

Operators

The Test Plan consists of a set of relevant test cases for testing the IPAd. The Implementations Under Test (IUT) are:

the IPA (IoT Profile Assistant)

NOTE: The IPAe is tested in SGP.33-1 [XX].

The testing scopes developed in this document are:

Interface compliance testing: Test cases to verify the compliance of the interfaces within the system.

System behaviour testing: Test cases to verify the functional behaviour of the system.

Each test case specified within this Test Plan refers to one or more requirements.

The Test Plan contains test cases for the following versions of SGP.22 and SGP.32:

GSMA RSP Technical Specification  [4]

GSMA IoT eSIM Technical Specification [31]

This document includes an applicability table providing an indication whether test cases are relevant for a specific entity.

## 1.3	Definition of Terms

In addition to the terms which are defined below, the terms defined in SGP.22 [2] and SGP.32 [31] also apply

| Term | Description |
| --- | --- |
| Integrated eUICC Test Interface | An external interface provided by its manufacturer for the purpose of testing eUICC functionality. |
| Test Plan | Current document describing the test cases that allow the RSP ecosystem to be tested. |

## 1.4	Abbreviations

In addition to the abbreviations which are defined below, the abbreviations defined in SGP.22 [2] and SGP.32 [31] also apply

| Abbreviation | Description |
| --- | --- |
| APDU | Application Protocol Data Unit |
| ATR | Answer To Reset |
| C-APDU | Command APDU |
| CCID | (USB) Chip Card Interface Device |
| DER TLV | Distinguished Encoding Rules - Tag Length Value |
| FCP | File Control Parameters |
| HW | Hardware |
| IPA | IoT Profile Assistant |
| IUT | Implementation Under Test |
| KVN | Key Version Number |
| LPA | Local Profile Assistant |
| OCE | Off-Card Entity |
| OS | Operating System |
| PIR | Profile Installation Result |
| POR | Proof Of Receipt |
| R-APDU | Response APDU |
| SoC | System on a Chip |
| SP | Service Provider |
| SSD | Supplemental Security Domain |
| USB | Universal Serial Bus |

## 1.5	Document Cross-references

| Ref | Document Number | Title |
| --- | --- | --- |
| [1] | SGP.02 | GSMA "Remote Provisioning of Embedded UICC Technical specification" V4.3 |
| [2] | SGP.22 | RSP Technical Specification V2.5 |
| [3] | SGP.21 | RSP Architecture V2.5 |
| [4] | eUICC Profile Package | Trusted Connectivity Alliance (formerly SIMalliance) eUICC Profile Package: Interoperable Format Technical Specification V2.1 or later |
| [5] | ETSI TS 102 221 | Smart Cards; UICC-Terminal interface |
| [6] | GPC_SPE_034 | GlobalPlatform Card Specification v.2.3 |
| [7] | ISO/IEC 7816-4:2013 | Identification cards – Integrated circuit cards - Part 4: Organization, security and commands for interchange |
| [8] | RFC 5639 | Elliptic Curve Cryptography (ECC) Brainpool Standard Curves and Curve Generation |
| [9] | ANSSI ECC FRP256V1 | Avis relatif aux paramètres de courbes elliptiques définis par l'Etat français. JORF n°0241 du 16 octobre 2011 page 17533. texte n° 30. 2011 |
| [10] | ITU E.118 | The international telecommunication charge card |
| [11] | NIST SP 800-56A | NIST Special Publication SP 800-56A: Recommendation for Pair-Wise Key Establishment Schemes Using Discrete Logarithm Cryptography (Revision 2), May 2013 |
| [12] | 3GPP TS 23.003 | Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); Numbering, addressing and identification |
| [13] | ETSI TS 102 225 | Secured packet structure for UICC based applications; Release 12 |
| [14] | ETSI TS 102 226 | Remote APDU structure for UICC based applications; Release 9 |
| [15] | TS.26 | GSMA NFC Handset Requirements V9.0 |
| [16] | ITU-T X.690 (11/2008) | ASN.1 Encoding Rules: Specification of Basic Encoding Rules (BER), Canonical Encoding Rules (CER) and Distinguished Encoding Rules (DER) including Corrigendum 1 and 2 |
| [17] | ETSI TS 102 241 | Smart cards; UICC Application Programming Interface (UICC API) for Java Card™ |
| [18] | 3GPP TS 31.102 | Characteristics of the Universal Subscriber Identity Module (USIM) application |
| [19] | GPC_SPE_095 | GlobalPlatform Card - Digital Letter of Approval - Version 1.0 |
| [20] | RFC 2119 | Key words for use in RFCs to Indicate Requirement Levels, S. Bradner http://www.ietf.org/rfc/rfc2119.txt |
| [21] | Void |  |
| [22] | 3GPP TS 23.040 | Technical realization of the Short Message Service (SMS) |
| [23] | Void |  |
| [24] | RFC 4492 | Elliptic Curve Cryptography (ECC) Cipher Suites for Transport Layer Security (TLS) |
| [25] | SGP.26 | RSP Test Certificates Definition v3.0.2 |
| [26] | 3GPP TS 29.002 | Mobile Application Part (MAP) specification |
| [27] | RFC 5246 | The Transport Layer Security (TLS) Protocol Version 1.2 |
| [28] | GSMA PRD AA.35 | Procedures for Industry Specifications Product |
| [29] | CCID Rev 1.1 | CCID Specification for Integrated Circuit(s) Cards Interface Devices |
| [30] | SGP.31 | eSIM IoT Architecture and Requirement Specification Version 1.2 |
| [31] | SGP.32 | eSIM IoT Technical Specification Version 1.2 |
| [32] | SGP.23 | SGP.23 Test Specifcation v1.15 |

## 1.6	Conventions

The key words "SHALL", "SHALL NOT", "SHOULD", "SHOULD NOT", and "MAY" in this document SHALL be interpreted as described in RFC 2119 [20].

# Testing Rules

## Applicability

### Format of the Optional Features Table

The columns in Table 4 have the following meaning:

| Column | Meaning |
| --- | --- |
| Option | The optional feature supported or not by the implementation. |
| Mnemonic | The mnemonic column contains mnemonic identifiers for each item. |

- Table 1: Format of the Optional Features Table

### Format of the Applicability Table

The applicability of every test in Table 5 is formally expressed by the use of a Boolean expression defined in the following clause.

The columns in Table 5 have the following meaning:

| Column | Meaning |
| --- | --- |
| Test case | The "Test case" column gives a reference to the test case number detailed in the present document and is required to validate the implementation of the corresponding item in the "Name" column. |
| Name | In the "Name" column, a short non-exhaustive description of the test is found. |
| Roles | SM-DP+, SM-DS, Device, LPAd, LPAe or eUICC Entities under test that take in charge the functions used in the test case. |
| Version | This column specifies which test cases are applicable for the given SGP.22 version. The column for the version declared in #IUT_RSP_VERSION shall be used. See clause 2.1.3 'Applicability and Notations'. |
| Test Env. | Test environment used for executing the test case. |

- Table 2: Format of the Applicability Table

### Applicability and Notations

The following notations are used for the Applicability column:

| Applicability code | Meaning |
| --- | --- |
| M | mandatory - the capability is required to be supported. |
| N/A | not applicable - in the given context, it is impossible to use the capability. |
| Ci | conditional - the requirement on the capability depends on the support of other items. "i" is an integer identifying an unique conditional status expression which is defined immediately following the table. For nested conditional expressions, the syntax "IF ... THEN (IF ... THEN ... ELSE...) ELSE ..." is to be used to avoid ambiguities. |

- Table 3: Applicability and Notations

### Optional Features Table

The supplier of the implementation SHALL state the support of possible options in Table 5.

| Device Options | Mnemonic |
| --- | --- |
| The Device supports IPAd | O_D_IPAD |
| The Device supports eIM Package retrieval | O_D_EIM_PACKAGE_RETRIEVAL |
| The Device supports eIM Package injection | O_D_EIM_PACKAGE_INJECTION |
| The Device supports direct profile download | O_D_DIRECT_DOWNLOAD |
| The Device supports fallback mechanism | O_D_FALLBACK |
| The Device supports indirect profile download | O_D_INDIRECT_DOWNLOAD |
| The Device supports HTTPS connection on ESipa interface | O_D_ESIPA_HTTPS |
| The Device supports a non-removable eUICC and eUICC RAT configurations in which PPR1 is allowed and End User Consent is NOT required. | O_D_EMB_ALLOWS_PPR1_EUC_NOT_REQ |
| The Device supports a non-removable eUICC and eUICC RAT configurations in which PPR2 is allowed and End User Consent is NOT required. | O_D_EMB_ALLOWS_PPR2_EUC_NOT_REQ |
| The Device sends eUICC Package Result to the eIM using ESipa.HandleNotification. | O_D_ESIPA_HANDLE_NOTIF |
| The Device sends eUICC Package Result to the eIM using ESipa.ProvideEimPackageResult. | O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |

- Table 4: Options

NOTE: this table should contain the IPA Options dedicated to IoT test cases. Those test cases that are applicable as SGP.23 [8] test cases should use the IPA Options as defined by SGP.23 [8].

### Applicability Table

Table 5 specifies the applicability of each test case. See clause 2.1.2 for the format of this table.

| Test case | Name | Role |  | V1.0 |  | Test Env. |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4.2.21.1 | TC_IPAd_InitiateAuthentication_Nominal | IPAd |  | SGP.23 |  | C3005 |  |
| 4.2.21.2.2 | TC_IPAd_InitiateAuthentication_ErrorCases | IPAd |  | SGP.23 |  | C3005 |  |
| 4.2.22.2.1 | TC_IPAd_ES9+_GetBoundProfilePackage_Nominal | IPAd |  | SGP.23 |  | C3005 |  |
| 4.2.22.2.2 | TC_IPAd_ES9+_GetBoundProfilePackage_Retry | IPAd |  | SGP.23 |  | C3005 |  |
| 4.2.22.2.3 | TC_IPAd_ES9+_GetBoundProfilePackage_Error | IPAd |  | SGP.23 |  | C3005 |  |
| 4.2.23.2.1 | TC_IPAd_ES9+_AuthenticatClient_Nominal | IPAd |  | SGP.23 |  | C3005 |  |
| 4.2.23.2.2 | TC_IPAd_ES9+_AuthenticateClient_ErrorCases | IPAd |  | SGP.23 |  | C3005 |  |
| 4.2.24.2.1 | TC_IPAd_ES9+_HandleNotification_Nominal All test sequences except the sequence #03 | IPAd |  | SGP.23 |  | C3005 |  |
| 4.2.24.2.1 | TC_IPAd_ES9+_HandleNotification_Nominal Only the test sequence #03 | IPAd |  | SGP.23 |  | C3005 |  |
| 4.2.25.2.1 | TC_IPAd_ES9+_CancelSession_Nominal | IPAd |  | SGP.23 |  | C3005 |  |
| 4.2.26.2.1 | TC_IPAd_HTTPS_Nominal | IPAd |  | C3005 |  |  |  |
| 4.2.26.2.2 | TC_IPAd_HTTPS_ErrorCases | IPAd |  | C3005 |  |  |  |
| 4.2.27.2.1 | TC_IPAd_ES11_InitiateAuthentication_Nominal | IPAd |  | C3007 |  |  |  |
| 4.2.27.2.2 | TC_IPAd_ES11_InitiateAuthentication_ErrorCases | IPAd |  | C3007 |  |  |  |
| 4.2.28.2.1 | TC_IPAd_ES11_AuthenticateClient_Nominal | IPAd |  | C3007 |  |  |  |
| 4.2.28.2.2 | TC_IPAd_ES11_AuthenticateClient_ErrorCases | IPAd |  | C3007 |  |  |  |
| 4.2.29.2.1 | TC_IPAd_HTTPS_Nominal | IPAd |  | C3007 |  |  |  |
| 4.2.29.2.2 | TC_IPAd_HTTPS_Error | IPAd |  | C3007 |  |  |  |
| 5.4.1.2.1 | TC_IPAd_DirectProfileDownload_IPA_initiated_with_Activation_Code |  | IPAd |  | C3001 |  |  |  |
| 5.4.1.2.2 | TC_IPAd_DirectProfileDownload_IPA_initiated_ActivationCode_InvalidFormat |  | IPAd |  | C3001 |  |  |  |
| 5.4.1.2.3 | TC_IPAd_DirectProfileDownload_IPA_initiated_with_ConfirmationCode_smdpSigned2 |  | IPAd |  | C3001 |  |  |  |
| 5.4.1.2.4 | TC_IPAd_DirectProfileDownload_IPA_initiated_default SM-DP+ |  | IPAd |  | C3001 |  |  |  |
| 5.4.1.2.5 | TC_IPAd_DirectProfileDownload_IPA_initiated_with_PPRs Only test sequence #1 |  | IPAd |  | C3002 |  |  |  |
| 5.4.1.2.5 | TC_IPAd_DirectProfileDownload_IPA_initiated_with_PPRs Only test sequence #2 |  | IPAd |  | C3003 |  |  |  |
| 5.4.1.2.5 | TC_IPAd_DirectProfileDownload_IPA_initiated_with_PPRs Only test sequence #3 |  | IPAd |  | C3004 |  |  |  |
| 5.4.1.2.6 | TC_IPAd_DirectProfileDownload_IPA_initiated_with_Empty MatchingID |  | IPAd |  | C3001 |  |  |  |
| 5.4.1.2.7 | TC_IPAd_DirectProfileDownload_IPA_initiated_default SM-DP+_immediate_profile_enabling |  | IPAd |  | C3001 |  |  |  |
| 5.4.4.2.1 | TC_IPAd_DeleteProfile_Disabled_without_PPR_IPA_initiated |  | IPAd |  | C3001 |  |  |  |
| 5.4.4.2.2 | TC_IPAd_DeleteProfile_Enabled_without_PPR_IPA_initiated |  | IPAd |  | C3001 |  |  |  |
| 5.4.4.2.3 | TC_IPAd_DeleteProfile_Error_with_PPR1_IPA_initiated |  | IPAd |  | C3002 |  |  |  |
| 5.4.4.2.4 | TC_IPAd_DeleteProfile_Error_Disabled_with_PPR2_IPA_initiated |  | IPAd |  | C3003 |  |  |  |
| 5.4.4.2.5 | TC_IPAd_DeleteProfile_Error_Enabled_with_PPR2_IPA_initiated |  | IPAd |  | C3003 |  |  |  |
| 5.4.5.2.1 | TC_IPAd_EnableProfile_IPA_initiated | IPAd |  | C3001 |  |  |  |
| 5.4.5.2.2 | TC_IPAd_EnableProfile_ImplicitDisable_IPA_initiated | IPAd |  | C3001 |  |  |  |
| 5.4.5.2.3 | TC_IPAd_EnableProfile_Error_ProfileAlreadyEnabled_IPA_initiated | IPAd |  | C3001 |  |  |  |
| 5.4.5.2.4 | TC_IPAd_EnableProfile_Error_PPR1Set_IPA_initiated | IPAd |  | C3002 |  |  |  |
| 5.4.5.2.5 | TC_LPAd_EnableProfile_Error_RollbackNoEnabledProfile_IPA_initiated | IPAd |  | C3001 |  |  |  |
| 5.4.6.2.1 | TC_IPAd_DisableProfile_IPA_initiated | IPAd |  | C3001 |  |  |  |
| 5.4.6.2.2 | TC_IPAd_DisableProfile_Error_ProfileAlreadyDisabled_IPA_initiated | IPAd |  | C3001 |  |  |  |
| 5.4.6.2.3 | TC_IPAd_DisableProfile_Error_PPR1Set_IPA_initiated | IPAd |  | C3002 |  |  |  |
| 5.4.12.2.1 | TC_IPAd_SetFallbackAttribute | IPAd |  | C3008 |  |  |  |
| 5.4.12.2.2 | TC_IPAd_SetFallbackAttribute_Error_Target_Profile_Not_Available | IPAd |  | C3008 |  |  |  |
| 5.4.12.2.3 | TC_IPAd_SetFallbackAttribute_Error_Not_Allowed | IPAd |  | C3008 |  |  |  |
| 5.4.12.2.4 | TC_IPAd_SetFallbackAttribute_Error_Fallback_Profile_Already_Enabled | IPAd |  | C3008 |  |  |  |
| 5.4.13.2.1 | TC_IPAd_UnsetFallbackAttribute | IPAd |  | C3008 |  |  |  |
| 5.4.13.2.2 | TC_IPAd_UnsetFallbackAttribute_Error_Fallback_Profile_Enabled | IPAd |  | C3008 |  |  |  |
| 5.4.13.2.3 | TC_IPAd_UnsetFallbackAttribute_Error_No_Fallback_Profile | IPAd |  | C3008 |  |  |  |

- Table 5: Applicability of Tests

| Conditional item | Condition |
| --- | --- |
| C3001 | IF O_D_IPAD AND O_D_DIRECT_DOWNLOAD AND O_D_EIM_PACKAGE_RETRIEVAL AND O_D_ESIPA_HTTPS THEN M ELSE N/A |
| C3002 | IF (O_D_IPAD AND O_D_DIRECT_DOWNLOAD AND O_D_EIM_PACKAGE_RETRIEVAL AND O_D_ESIPA_HTTPS AND (O_D_REMOVABLE_DOWNLOAD_PPR OR O_D_EMB_ALLOWS_PPR1_EUC_NOT_REQ)) THEN M ELSE N/A |
| C3003 | IF (O_D_IPAD AND O_D_DIRECT_DOWNLOAD AND O_D_EIM_PACKAGE_RETRIEVAL AND O_D_ESIPA_HTTPS AND (O_D_REMOVABLE_DOWNLOAD_PPR OR O_D_EMB_ALLOWS_PPR2_EUC_NOT_REQ)) THEN M ELSE N/A |
| C3004 | IF (O_D_IPAD AND O_D_DIRECT_DOWNLOAD AND O_D_EIM_PACKAGE_RETRIEVAL AND O_D_ESIPA_HTTPS AND O_D_ADDPREPPR1 AND (O_D_REMOVABLE_DOWNLOAD_PPR OR O_D_EMB_ALLOWS_PPR1_EUC_NOT_REQ)) THEN M ELSE N/A |
| C3005 | IF (O_D_IPAD AND O_D_DIRECT_DOWNLOAD) THEN M ELSE N/A |
| C3007 | IF (O_D_IPAD AND O_D_DIRECT_DOWNLOAD AND O_D_SMDS) THEN M ELSE N/A |
| C3008 | IF O_D_IPAD AND O_D_DIRECT_DOWNLOAD AND O_D_EIM_PACKAGE_RETRIEVAL AND O_D_ESIPA_HTTPS AND O_D_FALLBACK THEN M ELSE N/A |

- Table 6: Conditional Items Referenced by Table 5

## General Consideration

This section contains some general considerations about the test cases defined in this document. Note that some external test specifications are referred to in chapter 7. Consequently, the following sub sections SHALL only apply for test cases defined in sections 4 and 5 and 6.

### Test Case Definition

Test descriptions are independent.

For each test described in this document, a chapter provides a general description of the initial conditions applicable for the whole test. This description is completed by specific configurations to each individual sub-case.

It is implicitly assumed that all entities under test SHALL be compliant with the initial states described in Annex G. An initial state SHALL be considered as a pre-requisite to execute all the test cases described in this Test Plan.

After completing the test, the configuration is reset before the execution of the following test.

### Test Cases Format

Here is an explanation of the way to define the test cases in chapters 4, 5 and 6.

4.X.Y.Z Test Cases 4.X.Y.Z.1 TC_IUT_TestName1 General Initial Conditions. Entity; Description of the general initial condition. Entity1; Test case - general condition 1. Entity2; Test case - general condition 2. Test Sequence #01: Short Description Description of the aim of the test sequence N°1 Initial Conditions. Entity; Description of the initial condition. Entity1; Test sequence N°1 - initial condition 1. Entity2; Test sequence N°1 - initial condition 2. Step; Direction; Sequence / Description; Expected result; REQ. IC1; Entity1 → Entity2; Command or Message to send from Entity1 to Entity2; Expected result N°1.1. 1; Entity1 → Entity2; Command or Message to send from Entity1 to Entity2; 1- expected result N°1.2 2- expected result N°1.3; REQ1. 2; Entity2 → Entity3; Command or Message to send from Entity2 to Entity3. Test Sequence #02 Description of the aim of the test sequence N°2 Step; Direction; Sequence / Description; Expected result; REQ. 1; Entity1 → Entity2; Command or Message to send from Entity1 to Entity2. 2; Entity2 → Entity3; Command or Message to send from Entity2 to Entity3; 1- expected result N°2.1 2- expected result N°2.2; REQ2. 4.X.Y.Z.2 TC_IUT_TestName2 …

The test cases TC_IUT_TestName1 and TC_IUT_TestName2 are referenced in Table 5 that allows indicating the applicability of the tests.

In the test case TC_IUT_TestName1, the requirements REQ1 and REQ2 are respectively covered by the test sequences #01 and #02.

Note: For some test cases, requirements to be covered are not listed in the test sequences. In that case, references to sections in GSMA RSP Technical Specification [2] covered by the test sequences are indicated in the Conformance Requirements References section of the test case.

The test sequence #01 SHALL be executed if and only if these conditions are met:

Test case - general condition 1

Test case - general condition 2

Test sequence N°1 - initial condition 1

Test sequence N°1 - initial condition 2

The test sequence #02 SHALL be executed if and only if these conditions are met:

Test case - general condition 1

Test case - general condition 2

The tables defining the different initial conditions are optional.

Initial Conditions are intended to be reached dynamically using the Test Tool when possible.

Unless otherwise defined, no additional operation SHALL be done prior to the test sequence besides those indicated in the Initial Conditions (e.g. no other Profiles SHALL be present on the eUICC besides those defined in the Initial Conditions).

In the test sequence #01:

the step IC1 corresponds to an additional Initial Condition

in the step N°1, if the expected results N°1 and N°2 are validated, the requirement REQ1 (or a part of the REQ1) SHALL be considered as implemented

Note that all initial states (described in Annex G) SHALL be implemented by the entity under test whatever the test cases to execute.

In addition, following 2.2.1 sub sections present all information (e.g. Methods, Constants…) that MAY be referenced in test sequences.

After execution of each test sequence a clean-up procedure (CU) SHALL be executed to restore the IUT to the Common Initial State as defined in Annex G.

#### Methods and Procedures

A method is referenced as follow:

- 	MTD_NAME_OF_THE_METHOD(PARAM1, PARAM2…)

The key word “NO_PARAM” SHALL be set in method call if the related optional parameter is not used.

All methods and their related parameters are described in Annex C.1.

A procedure is a generic sub-sequence and is referenced as follow:

PROC_NAME_OF_THE_PROCEDURE

All procedures are described in Annex C.2.

The implementation of these methods and procedures is under the responsibility of the test tool providers.

#### Constants and Dynamic Content

A constant (e.g. text, ASN.1 structure, hexadecimal string, icon, URI, integer, EID, AID…) is referenced as follow:

- 	#NAME_OF_THE_CONSTANT

All constants are defined in Annex A.

When provided as an ASN.1 value notation, a constant SHALL be encoded in DER TLV (as specified in ITU-T X.690 [16]) by the test tool.

A dynamic content (e.g. TLV, ASN.1 structure, signature, integer, AID, one-time key pair…) is referenced as follow:

- 	<NAME_OF_THE_VARIABLE>

All dynamic contents are defined in Annex B.

A dynamic content is either generated by an IUT or by a test tool provider.

#### Requests and Responses

An ASN.1 or a JSON request is referenced as follow:

- 	#NAME_OF_THE_REQUEST

An ASN.1 or a JSON response is referenced as follows:

- 	#R_NAME_OF_THE_RESPONSE

Each ASN.1 or JSON request and response MAY refer to a constant or a dynamic content. All these structures are defined in Annex D.

When provided as an ASN.1 value notation, a request or a response SHALL be encoded in DER TLV (as specified in ITU-T X.690 [16]) by the test tool.

When an ASN.1 element definition contains three points (i.e. “…”), it means that fields MAY be present but SHALL not be checked by the test tool.

In the following example, several fields MAY be part of the ProfileInfoListResponse but only the profileNickname SHALL be verified.

resp ProfileInfoListResponse ::=

profileInfoListOk :{

{ 
       ...
       profileNickname #NICKNAME
       ...

}

}

This rule applies also for Constants definition.

Some ASN.1 SEQUENCE components have a DEFAULT value (for example, profileClass in StoreMetadataRequest). In this specification, when values are specified in ASN.1 syntax and the DEFAULT value is intended, two different formulations (both of which are valid) may be used:

the relevant component is specified with the DEFAULT value;

the relevant component is missing entirely.

These are logically equivalent and lead to the same DER encoding. In both cases, the following rules apply:

When the test tool is sending the DER value, it SHALL NOT include the component (as per DER rules).

When the test tool is checking a received DER value from the entity under test, it SHALL check that the component is NOT present.

Test tools SHALL consider two BIT STRINGs to be equivalent if the BIT STRINGs have the same DER encoding. For example, '0101'B shall be considered to be equivalent to '010100'B.

NOTE:	this is equivalent to removing any trailing zero bits from the BIT STRINGs in "bstring" notation (e.g. '010100'B à '0101'B) and then comparing the strings textually.

NOTE:	according to the DER format, the encoding of transmitted values will remove the trailing zeroes. The definition above allows for values which are specified using ASN.1 value notation and are not transmitted, such as values specified in the Annexes of the current document, including IUT settings which might be specified by a user of the current document and may contain trailing zeroes in the ASN.1 value notation.

#### APDUs

A C-APDU is referenced as follow:

- 	[NAME_OF_THE_CAPDU]

All C-APDUs are defined in Annex D.4.

An R-APDU is referenced as follow:

- 	[R_NAME_OF_THE_RAPDU]

All R-APDUs are defined in Annex D.4.

Each APDU MAY refer to a constant or a dynamic content.

The APDU TERMINAL RESPONSE SHALL be dynamically generated by the test tool according to the related proactive command. Therefore, this particular command is not referenced with brackets in this specification. If not explicitly defined in the step, the general result SHALL be set by default to “Command performed successfully” (i.e. 0x83 01 00).

#### Profiles

In order to execute the test cases described in this document, Operational, Test and Provisioning Profiles are necessary. All these Profiles are defined in Annex E with the Profile Metadata content and the corresponding Profile Package as defined in the eUICC Profile Package Specification [4].

A Profile is referenced as follow:

- 	PROFILE_OPERATIONALx with x the identifier of the Operational Profile

or

- 	PROFILE_TESTx with x the identifier of the Test Profile

or

- 	PROFILE_PROVISIONINGx with x the identifier of the Provisioning Profile

NOTE:	Test Profiles and Provisioning Profiles are out of the scope of this version of test specification.

#### IUT Settings

For the purpose of some test cases, Device and eUICC manufacturers and Platforms (i.e. SM-DP+, SM-DS) providers need to give some information related to their products to the test tools providers (e.g. supported Java Card version).

An IUT setting is referenced as follow:

- 	#IUT_NAME_OF_SETTING

All these settings are defined in Annex F.

### General Rules for Device Testing

#### Default Profile Download and LPM Process on the Device Under Test

By default, when an Operational Profile needs to be downloaded, installed (and if necessary enabled) on the (Test) eUICC resided in the Device Under Test (e.g. As mentioned in an initial condition), the following rules apply except if it is defined differently in the Test Case.

The default way to execute the Profile download SHALL be the Add Profile procedure with Activation Code #ACTIVATION_CODE_1. The way to apply the Activation Code (manual typing or QR code scanning) depends on the Device/LPAd implementation. In order to execute the Common Mutual Authentication procedure and the Sub-procedure Profile Download and Installation (End User Confirmation), the following responses SHALL be sent by the S_SM-DP+:

- 	#INITIATE_AUTH_OK

with the <EUICC_CI_PK_ID_TO_BE_USED> set to the CI for signing indicated as highest priority in euiccCiPKIdListForSigning in the #R_EUICC_INFO1

with the #CERT_S_SM_DPauth_ECDSA leading to the same CI as the one chosen for signing

with the SM-DP+ address #TEST_DP_ADDRESS1

- 	#AUTH_CLIENT_OK

with the #CERT_S_SM_DPpb_ECDSA leading to the same CI as the one chosen for signing

Metadata of the downloaded Profile instead of #METADATA_OP_PROF1

- 	#GET_BPP_OK with the content of the installed Profile (no session keys used)

Before running a test sequence, and after establishing the Initial conditions, all pending Notifications (sent on the best-effort basis as soon as connectivity is available as defined in section 3.5 of SGP.22 [2]) SHALL have been acknowledged by the simulated SM-DP+(s). S_SM-DP+(s) SHALL be run with suitable addresses in order to receive and acknowledge all pending Notifications (including install, enable, disable and delete). The addresses which are required depend on the server address used for recent profile downloads (typically #TEST_DP_ADDRESS1 to receive and acknowledge PIR), and the notificationAddress values in the Metadata of recently downloaded Profiles (for otherSignedNotification). Each S_SM_DP+ SHALL use the TLS certificate corresponding to its address (CERT_S_SM_DP_TLS, CERT_S_SM_DP2_TLS, etc).

If only O_D_ADD_ENABLE_COMBINED (or any other combined operation, like combined “disable and delete”) is supported by the DUT, the user might have to perform actions in a particular manner in order to achieve the initial conditions related to enabled/disabled state of profiles (for example: disable a profile after installing, install profiles in a particular order, re-enable an initial profile after installing a subsequent profile).

Some devices may always combine the “disable” procedure with a “delete” procedure. For such devices, further actions might be required to achieve the initial condition that a particular profile is disabled; in particular, this might be the case when the device supports only the combine “add and enable” procedure, and not the “add only” procedure. In this case, if neither O_D_DISABLE_SEPARATED nor O_D_ADD_ENABLE_SEPARATED are supported, one of the following procedures is required (where the profile which needs to be disabled is denoted as Profile A):

- Install (and enable) Profile A; install another “helper” profile; enable the “helper” profile (this should automatically disable Profile A).

- Install (and enable) another “helper” profile; install Profile A (Profile A should remain disabled).

In some cases, the “helper” profile has to be deleted before the start of the actual test sequence to achieve the required state of the initial conditions.

If the test case requires a Profile Download to be initiated via SM-DS:

The mechanism used to initiate this is device-specific.

If the device is using Power-on Profile Discovery the following applies:

when it is supported, the value of the configuration parameter for Device Power-on Profile discovery is 'Enabled'.

the Device has to be powered-off and then powered-on before each test sequence.

#### TLS Testing Recommendations

The TLS connection may be rejected either:

- by sending a TLS alert, or

- by closing of the TCP connection, though TLS handshake completed, or

- TLS handshake not completed without sending a TLS alert, or

- No further RSP communication has been initiated by LPAd on ES9+/ES11 within the #IUT_LPAd_SESSION_CLOSE_TIMEOUT

Please note that this is not an exhaustive list, and acting as guidelines for the test tools.

### Pass Criteria

A test execution is considered as successful only if the test procedure was fully carried out successfully.

A test execution is considered as failed if the tested feature provides an unexpected behaviour.

A test execution is considered as inconclusive when the pass criteria cannot be evaluated due to issues during the setup of the initial conditions (including the ICx steps) or during the execution of steps in which no requirement is referenced.

### Future Study

Some of the test cases or test sequences described in this Test Plan are FFS (For Future Study). This MAY mean that some clarifications are expected at the requirement level to conclude on a test method. As consequence, the corresponding test SHALL not be executed.

### Adaptation of ES9+ and ES11 test cases

For ES9+ test cases when the General Initial Condition, or the Initial Condition in SGP.23 says that "Add Profile operation is initiated by using #ACTIVATION_CODE_X" in this document this is equivalent with the Profile Download Trigger Request with ACTIVATION_CODE_X is received by the IPA. The Profile Download Trigger Request is sent without eimTransactionId.

The IPA is expected to send Handle Notification with profileDownloadTriggerResult on ESipa interface. It needs to be handled by the test tool, but it does not need to be validated, because it is not in the scope of ES9+ testing. However, it is validated in chapter 5.

For ES11 test cases when the General Initial Condition says "The Profile Download is initiated using SM-DS" in this document this is equivalent with the Profile Download Trigger Request with contactSmds is received by the IPA. The Profile Download Trigger Request is sent without eimTransactionId and without smdsAddress.

The IPA is expected to send Handle Notification with profileDownloadTriggerResult on ESipa interface. It needs to be handled by the test tool, but it does not need to be validated, because it is not in the scope of ES11 testing. However, it is validated in chapter 5.

# 3	Testing Architecture

## 3.1	Testing Scope

All the interfaces, intended to be tested in the scope of this document, are presented hereafter:

| Interface | Between |  | Description | SGP.33-2 |
| --- | --- | --- | --- | --- |
| ES2+ | Operator | SM-DP+ | Used by the Operator to order Profiles for specific eUICCs as well as other administrative functions as defined in SGP.31 [2]. | Out of scope |
| ES6 | Operator | eUICC | Used by the Operator for the management of Operator services via OTA services as defined in SGP.31 [2]. | Out of scope |
| ES8+ | SM-DP+ | eUICC | Provides a secure end-to-end channel between the SM-DP+ and the eUICC for the administration of the ISD-P and the associated Profile during download and installation. It provides Perfect Forward Secrecy as defined in SGP.31 [2]. | Out of scope |
| ES9+ | SM-DP+ | IPA | Used to provide a secure transport between the SM-DP+ and the IPA for the delivery of the Bound Profile Package as defined in SGP.31 [2]. | In scope |
| ES9+’ | SM-DP+ | eIM | Used to provide a secure transport between the SM-DP+ and the eIM for the delivery of the Bound Profile Package as defined in SGP.31 [2]. | Out of scope |
| ES10a | IPA | eUICC | Used between the IPA (in the IoT Device) and the eUICC to handle a Profile discovery as defined in SGP.31 [2]. | Out of scope |
| ES10b | IPA | eUICC | Used between the IPA (in the IoT Device) and the IPA Services to transfer a Bound Profile Package to the eUICC as defined in SGP.31 [2]. This interface plays no role in the decryption of Profile Packages. | Out of scope |
| ES11 | IPA | SM-DS | Used by the IPA to retrieve Event Records for the respective eUICC as defined in SGP.31 [2]. | In scope |
| ES11’ | eIM | SM-DS | Used by the eIM to retrieve Event Records for the respective eUICC as defined in SGP.31 [2]. | Out of scope |
| ES12 | SM-DP+ | SM-DS | Used by the SM-DP+ to issue or remove Event Registrations on the SM-DS as defined in SGP.31 [2]. | Out of scope |
| ESep | eIM | eUICC | Logical end-to-end interface between the eIM and the eUICC used to transfer eUICC Packages for Profile State management and eIM configuration by eIM, as defined in SGP.31 [2]. | Out of scope |
| ESipa | eIM | IPA | Logical interface between an eIM and an IPA, as defined in SGP.31 [2], used to trigger a Profile download at the IPA and to provide a secure transport for the delivery of eUICC Packages, unless the underlying transport provides necessary security. | In scope |

- Table 7: Interfaces Descriptions

## 3.2	Testing Execution

This chapter aims to describe the different testing environments and equipments to allow the test cases to be executed.

To permit the execution of the different test cases described in this Test Plan, specifics simulators SHALL be used. The simulators that have been defined are listed hereafter:

- 	S_Device: the Device Simulator used to send some commands to the eUICC under test using ISO/IEC 7816-4 [7] on the contact interface

- 	S_SM-DP+: the SM-DP+ Simulator

- 	S_SM-DS: the SM-DS Simulator

- 	S_MNO: the MNO Simulator

- 	S_IPAd: the LPAd Simulator

- 	S_LPAe: the LPAe Simulator

- S_eIM: the eIM Simulator

- 	S_CLIENT: the HTTPs client Simulator for the purpose of TLS testing. The S_CLIENT MAY be S_SM-DP+, S_SM-DS depending on the component under test.

- 	S_SERVER: the HTTPs server Simulator for the purpose of TLS testing. The S_SERVER MAY be S_SM-DP+ or S_SM-DS depending on the component under test.

- 	Implementation of these simulators remains under the responsibility of the test tool providers.

- 	The aim of all the test cases is to verify the compliance of an Actor/Component (i.e. eUICC, SM-DP+, Alternative SM-DS, Root SM-DS, LPAe, LPAd, Device).

Following notations are used:

- 	S_ComponentName for a simulated component

- 	ComponentName for the Implementation Under Test (IUT)

- 	Where ComponentName is indicated by CLIENT, SERVER

- 	Depending on the component under test, the CLIENT MAY be the SM-DP+ or the SM-DS. The Operator component is currently out of scope.

- Depending on the component under test, the SERVER MAY be the SM-DP+ or the SM-DS. The Operator component is currently out of scope.

- The use of "-- optional" in any ASN.1 elements defined within this document indicate that the test tool SHALL allow for the value either being present with that value, or being absent.

### 3.2.3	Device/IPAd - Test Environment

The following test environment is used for all IPAd Interfaces related test cases as defined in chapter 4.4 and 5.4 (unless it is specified differently in the specific test case). Following conditions apply:

- 	The Device contains an eUICC configured with Test Certificates, Test Keys and eIM Configuration

- 	The Test eUICC is either soldered or removable. In case the eUICC is removable, it SHALL NOT be removed during testing

- 	The Test eUICC is only used for IPAd testing and SHALL not be considered as an IUT

- 	The Test eUICC SHALL not support IPAe

- 	The Test eUICC SHOULD be compliant with the GSMA RSP IoT Technical Specification [31]

- 	SM-DP+ Simulator(s) SHALL be implemented by the test tools

- 	SM-DS Simulator(s) SHALL be implemented by the test tools

- 	No modification of the Device HW is required

- eIM Simulator(s) SHALL be implemented by the test tools. For the purpose of direct Profile download test cases, eIM Simulator(s) SHALL support at least HTTPS connection on ESipa and eIM Package retrieval by IPA.

- 	Test Root Certificate SHALL be configured in the Device

#### 3.2.3.1	General (Device/IPAd) Test Environment

The Test Environment consists of:

- 	IUT: IoT Device

- 	S_SM-DP+: a simulated SM-DP+ supporting a connection used by the Device to establish ES9+, (ES8+)

- 	S_SM-DS: a simulated SM-DS supporting a connection used by the Device to establish ES11

S_eIM: a simulated eIM supporting at least HTTPS connection to the Device

the interface between S_eIM and S_SM_DP+ is internal to the test tool

the interface between S_eIM and S_SM_DS is internal to the test tool

In case the Device supports a connection method different from Cellular Network it is expected that this connection method is used.

NOTE: 	Device that supports only Cellular Networks is out of scope for this specification.

#### 3.2.3.2	Device – Test Environment

If the IUT is a Device as defined in SGP.31[30]/SGP.32 [31] it SHALL provide at least one method to establish the IP connection to the S_SM-DP+, or S_SM-DS.

When executing a test case with an IUT matching this definition, default Initial States as defined in G.1.1 apply unless it is specified differently in the specific test case.

#### 3.2.3.3

# 4	Interface Compliance Testing

## 4.1	General Overview

This section focuses on the implementation of the different interfaces according to the GSMA RSP Technical Specification [2]. The aim is to verify the compliance of all interfaces within the system.

## 4.2	IPAd Interfaces

### 4.2.1	ES10a (IPA -- eUICC): GetEuiccConfiguredAddresses

This test case is defined as FFS and not applicable for this version of test specification.

### 4.2.2	ES10b (IPA -- eUICC): LoadEUICCPackage

This test case is defined as FFS and not applicable for this version of test specification.

### 4.2.3	ES10b (IPA -- eUICC): GetEUICCInfo

This test case is defined as FFS and not applicable for this version of test specification.

### 4.2.4	ES10b (IPA -- eUICC): GetEUICCChallenge

This test case is defined as FFS and not applicable for this version of test specification.

### 4.2.5	ES10b (IPA -- eUICC): AddInitialEim

This test case is defined as FFS and not applicable for this version of test specification.

### 4.2.6	ES10b (IPA -- eUICC): eUICCMemoryReset

This test case is defined as FFS and not applicable for this version of test specification.

### 4.2.7	ES10b (IPA -- eUICC): AuthenticateServer

This test case is defined as FFS and not applicable for this version of test specification.

### 4.2.8	ES10b (IPA -- eUICC): PrepareDownload

This test case is defined as FFS and not applicable for this version of test specification.

### 4.2.9	ES10b (IPA -- eUICC): LoadBoundProfilePackage

This test case is defined as FFS and not applicable for this version of test specification.

### 4.2.10	ES10b (IPA -- eUICC): CancelSession

This test case is defined as FFS and not applicable for this version of test specification.

### 4.2.11	ES10b (IPA -- eUICC): GetCerts

This test case is defined as FFS and not applicable for this version of test specification.

### 4.2.12	ES10b (IPA -- eUICC): RetrieveNotificationList

This test case is defined as FFS and not applicable for this version of test specification.

### 4.2.13	ES10b (IPA -- eUICC): RetrieveNotificationFromList

This test case is defined as FFS and not applicable for this version of test specification.

### 4.2.14	ES10b (IPA -- eUICC): GetRAT

This test case is defined as FFS and not applicable for this version of test specification.

### 4.2.15	ES10b (IPA -- eUICC): GetProfileInfo

This test case is defined as FFS and not applicable for this version of test specification.

### 4.2.16	ES10b (IPA -- eUICC): EnableUsingDD

This test case is defined as FFS and not applicable for this version of test specification.

### 4.2.17	ES10b (IPA -- eUICC): ProfileRollBack

This test case is defined as FFS and not applicable for this version of test specification.

### 4.2.18	ES10b (IPA -- eUICC): ConfigureAutomaticProfileEnabling

This test case is defined as FFS and not applicable for this version of test specification.

### 4.2.19	ES10b (IPA -- eUICC): GetEimConfigurationData

This test case is defined as FFS and not applicable for this version of test specification.

### 4.2.20	ES10b (IPA -- eUICC): GetEID

This test case is defined as FFS and not applicable for this version of test specification.

### 4.2.21	ES9+ (IPA -- SM-DP+): InitiateAuthentication

#### 4.2.21.1	Conformance Requirements

References

GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

The IPA SHALL communicate with the SM-DP+ secured by HTTPS in server authentication mode as described in SGP.22 [4] section 2.6.6.

This interface is identical to the ES9+ interface defined in section 5.6 of SGP.22 [2], where the IPA plays the role of LPA.

#### 4.2.21.2	Test Cases

##### 4.2.21.2.1	TC_IPAd_InitiateAuthentication_Nominal

Test Sequence #01 Nominal: Initiate Authentication

This test sequence is the same as SGP.23 [32] - the Test Sequence #01 Nominal: Initiate Authentication defined in section 4.4.21.2.1	TC_LPAd_InitiateAuthentication_Nominal where the LPAd play the role of IPAd.

##### 4.2.21.2.2	TC_IPAd_InitiateAuthentication_ErrorCases

Test Sequence #01 Error: Invalid SM-DP+ Address

This test sequence is the same as SGP.23 [32] - the Test Sequence #01 Error: Invalid SM-DP+ Address defined in section 4.4.21.2.2 TC_LPAd_InitiateAuthentication_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #02 Error: Unsupported Security Configuration

This test sequence is the same as SGP.23 [32] - the Test Sequence #02 Error: Unsupported Security Configuration defined in section 4.4.21.2.2 TC_LPAd_InitiateAuthentication_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #03 Error: Unsupported SVN

This test sequence is the same as SGP.23 [32] - the Test Sequence #03 Error: Unsupported SVN defined in section 4.4.21.2.2 TC_LPAd_InitiateAuthentication_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #04 Error: Unavailable SM-DP+ Certificate

This test sequence is the same as SGP.23 [32] - the Test Sequence #04 Error: Unavailable SM-DP+ Certificate defined in section 4.4.21.2.2 TC_LPAd_InitiateAuthentication_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #05 Error:  Invalid SM-DP+ Certificate

This test sequence is the same as SGP.23 [32] - the Test Sequence #05 Error:  Invalid SM-DP+ Certificate defined in section 4.4.21.2.2 TC_LPAd_InitiateAuthentication_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #06 Error:  Invalid SM-DP+ Signature

This test sequence is the same as SGP.23 [32] - the Test Sequence #06 Error:  Invalid SM-DP+ Signature defined in section 4.4.21.2.2 TC_LPAd_InitiateAuthentication_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #07 Error: Invalid SM-DP+ Address sent by the SM-DP+

This test sequence is the same as SGP.23 [32] - the Test Sequence #07 Error: Invalid SM-DP+ Address sent by the SM-DP+ defined in section 4.4.21.2.2 TC_LPAd_InitiateAuthentication_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #08 Error: Unsupported CI Key ID

This test sequence is the same as SGP.23 [32] - the Test Sequence #08 Error: Unsupported CI Key ID defined in section 4.4.21.2.2 TC_LPAd_InitiateAuthentication_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #09 Error: Invalid SM-DP+ OID

This test sequence is the same as SGP.23 [32] - the Test Sequence #09 Error: Invalid SM-DP+ OID Address defined in section 4.4.21.2.2 TC_LPAd_InitiateAuthentication_ErrorCases where the LPAd play the role of IPAd.

### 4.2.22	ES9+ (IPA -- SM-DP+): GetBoundProfilePackage

#### 4.2.22.1	Conformance Requirements

References

References

GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

The IPA SHALL communicate with the SM-DP+ secured by HTTPS in server authentication mode as described in SGP.22 [4] section 2.6.6.

This interface is identical to the ES9+ interface defined in section 5.6 of SGP.22 [2], where the IPA plays the role of LPA.

#### 4.2.22.2	Test Cases

##### 4.2.22.2.1	TC_IPAd_ES9+_GetBoundProfilePackage_Nominal

Test Sequence #01 Nominal: Get BPP using S-ENC and S-MAC without Confirmation Code

This test sequence is the same as SGP.23 [32] - the Test Sequence #01 Nominal: Get BPP using S-ENC and S-MAC without Confirmation Code defined in section 4.4.22.2.1 TC_LPAd_ES9+_GetBoundProfilePackage_Nominal  where the LPAd play the role of IPAd.

The method to provide confirmation code to IPAd is IPAd dependant.

Test Sequence #02 Nominal: Get BPP using S-ENC and S-MAC with Confirmation Code

This test sequence is the same as SGP.23 [32] - the Test Sequence #02 Nominal: Get BPP using S-ENC and S-MAC with Confirmation Code defined in section 4.4.22.2.1 TC_LPAd_ES9+_GetBoundProfilePackage_Nominal  where the LPAd play the role of IPAd.

The method to provide confirmation code to IPAd is IPAd dependant.

Test Sequence #03 Nominal: Get BPP using PPK-ENC and PPK-MAC without Confirmation Code

This test sequence is the same as SGP.23 [32] - the Test Sequence #03 Nominal: Get BPP using PPK-ENC and PPK-MAC without Confirmation Code defined in section 4.4.22.2.1 TC_LPAd_ES9+_GetBoundProfilePackage_Nominal  where the LPAd play the role of IPAd.

The method to provide confirmation code to IPAd is IPAd dependant.

Test Sequence #04 Nominal: Get BPP using PPK-ENC and PPK-MAC with Confirmation Code

This test sequence is the same as SGP.23 [32] - the Test Sequence #04 Nominal: Get BPP using PPK-ENC and PPK-MAC with Confirmation Code defined in section 4.4.22.2.1 TC_LPAd_ES9+_GetBoundProfilePackage_Nominal  where the LPAd play the role of IPAd.

The method to provide confirmation code to IPAd is IPAd dependant.

##### 4.4.22.2.2	TC_IPAd_ES9+_GetBoundProfilePackage_Retry

Test Sequence #01 Nominal: Get BPP Retry after incorrect Confirmation Code

This test sequence is the same as SGP.23 [32] - the Test Sequence #01 Nominal: Get BPP Retry after incorrect Confirmation Code defined in section 4.4.22.2.2 TC_LPAd_ES9+_GetBoundProfilePackage_Retry where the LPAd play the role of IPAd.

The method to provide confirmation code to IPAd is IPAd dependant.

##### 4.4.22.2.3	TC_IPAd_ES9+_GetBoundProfilePackage_Error

Test Sequence #01 Error: Wrong eUICC Signature

This test sequence is the same as SGP.23 [32] - the Test Sequence #01 Error: Wrong eUICC Signature defined in section 4.4.22.2.3 TC_LPAd_ES9+_GetBoundProfilePackage_Error where the LPAd play the role of IPAd.

The method to provide confirmation code to IPAd is IPAd dependant.

Test Sequence #02 Error: BPP Not Available

This test sequence is the same as SGP.23 [32] - the Test Sequence #02 Error: BPP Not Available defined in section 4.4.22.2.3 TC_LPAd_ES9+_GetBoundProfilePackage_Error where the LPAd play the role of IPAd.

The method to provide confirmation code to IPAd is IPAd dependant.

Test Sequence #03 Error: Unknown TransactionID received by SM-DP+

This test sequence is the same as SGP.23 [32] - the Test Sequence #03 Error: Unknown TransactionID received by SM-DP+ defined in section 4.4.22.2.3 TC_LPAd_ES9+_GetBoundProfilePackage_Error where the LPAd play the role of IPAd.

The method to provide confirmation code to IPAd is IPAd dependant.

Test Sequence #04 Error: Missing Confirmation Code

This test sequence is the same as SGP.23 [32] - the Test Sequence #04 Error: Missing Confirmation Code defined in section 4.4.22.2.3 TC_LPAd_ES9+_GetBoundProfilePackage_Error where the LPAd play the role of IPAd.

The method to provide confirmation code to IPAd is IPAd dependant.

Test Sequence #05 Error: Download Order Expired

This test sequence is the same as SGP.23 [32] - the Test Sequence #05 Error: Download Order Expired defined in section 4.4.22.2.3 TC_LPAd_ES9+_GetBoundProfilePackage_Error where the LPAd play the role of IPAd.

The method to provide confirmation code to IPAd is IPAd dependant.

Test Sequence #06 Error: Wrong Confirmation Code

This test sequence is the same as SGP.23 [32] - the Test Sequence #06 Error: Wrong Confirmation Code defined in section 4.4.22.2.3 TC_LPAd_ES9+_GetBoundProfilePackage_Error where the LPAd play the role of IPAd.

The method to provide confirmation code to IPAd is IPAd dependant.

Test Sequence #07 Error: Maximum number of Confirmation Code retries exceeded

This test sequence is the same as SGP.23 [32] - the Test Sequence #07 Error: Maximum number of Confirmation Code retries exceeded defined in section 4.4.22.2.3 TC_LPAd_ES9+_GetBoundProfilePackage_Error where the LPAd play the role of IPAd.

The method to provide confirmation code to IPAd is IPAd dependant.

### 4.2.23	ES9+ (IPA -- SM-DP+): AuthenticateClient

#### 4.2.23.1	Conformance Requirements

References

GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

The IPA SHALL communicate with the SM-DP+ secured by HTTPS in server authentication mode as described in SGP.22 [4] section 2.6.6.

This interface is identical to the ES9+ interface defined in section 5.6 of SGP.22 [2], where the IPA plays the role of LPA.

#### 4.2.23.2	Test Cases

##### 4.2.23.2.1	TC_IPAd_AuthenticateClient_Nominal

Test Sequence #01 Nominal: Authenticate Client without Confirmation Code

This test sequence is the same as SGP.23 [32] - the Test Sequence #01 Nominal: Authenticate Client without Confirmation Code defined in section 4.4.23.2.1 TC_LPAd_AuthenticateClient_Nominal where the LPAd play the role of IPAd.

The method to provide confirmation code to IPAd is IPAd dependant.

Test Sequence #02 Nominal: Authenticate Client with Confirmation Code

This test sequence is the same as SGP.23 [32] - the Test Sequence #02 Nominal: Authenticate Client with Confirmation Code defined in section 4.4.23.2.1 TC_LPAd_AuthenticateClient_Nominal where the LPAd play the role of IPAd.

The method to provide confirmation code to IPAd is IPAd dependant.

Test Sequence #03 Nominal: Authenticate Client with Confirmation Code Retry

This test sequence is the same as SGP.23 [32] - the Test Sequence #03 Nominal: Authenticate Client with Confirmation Code Retry defined in section 4.4.23.2.1 TC_LPAd_AuthenticateClient_Nominal where the LPAd play the role of IPAd.

The method to provide confirmation code to IPAd is IPAd dependant.

##### 4.2.23.2.2	TC_IPAd_AuthenticateClient_ErrorCases

Test Sequence #01 Error: Invalid EUM Certificate

This test sequence is the same as SGP.23 [32] - the Test Sequence #01 Error: Invalid EUM Certificate defined in section 4.4.23.2.2 TC_LPAd_AuthenticateClient_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #02 Error: Expired EUM Certificate

This test sequence is the same as SGP.23 [32] - the Test Sequence #02 Error: Expired EUM Certificate defined in section 4.4.23.2.2 TC_LPAd_AuthenticateClient_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #03 Error: Invalid eUICC Certificate

This test sequence is the same as SGP.23 [32] - the Test Sequence #03 Error: Invalid eUICC Certificate defined in section 4.4.23.2.2 TC_LPAd_AuthenticateClient_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #04 Error: Expired eUICC Certificate

This test sequence is the same as SGP.23 [32] - the Test Sequence #04 Error: Expired eUICC Certificate defined in section 4.4.23.2.2 TC_LPAd_AuthenticateClient_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #05 Error: Invalid eUICC Signature or serverChallenge

This test sequence is the same as SGP.23 [32] - the Test Sequence #05 Error: Invalid eUICC Signature or serverChallenge defined in section 4.4.23.2.2 TC_LPAd_AuthenticateClient_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #06 Error: Insufficient Memory

This test sequence is the same as SGP.23 [32] - the Test Sequence #06 Error: Insufficient Memory defined in section 4.4.23.2.2 TC_LPAd_AuthenticateClient_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #07 Error: Unknown CI Root Key

This test sequence is the same as SGP.23 [32] - the Test Sequence #07 Error: Unknown CI Root Key defined in section 4.4.23.2.2 TC_LPAd_AuthenticateClient_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #08 Error: Profile not Allowed (Not in 'released' State)

This test sequence is the same as SGP.23 [32] - the Test Sequence #08 Error: Profile not Allowed (Not in 'released' State) defined in section 4.4.23.2.2 TC_LPAd_AuthenticateClient_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #09 Error: Unknown TransactionID

This test sequence is the same as SGP.23 [32] - the Test Sequence #09 Error: Unknown TransactionID defined in section 4.4.23.2.2 TC_LPAd_AuthenticateClient_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #10 Error: Refused MatchingID

This test sequence is the same as SGP.23 [32] - the Test Sequence #10 Error: Refused MatchingID defined in section 4.4.23.2.2 TC_LPAd_AuthenticateClient_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #11 Error: Refused EID

This test sequence is the same as SGP.23 [32] - the Test Sequence #11 Error: Refused EID defined in section 4.4.23.2.2 TC_LPAd_AuthenticateClient_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #12 Error: No Eligible Profile for this eUICC/Device

This test sequence is the same as SGP.23 [32] - the Test Sequence #12 Error: No Eligible Profile for this eUICC/Device defined in section 4.4.23.2.2 TC_LPAd_AuthenticateClient_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #13 Error: Expired Download Order

This test sequence is the same as SGP.23 [32] - the Test Sequence #13 Error: Expired Download Order defined in section 4.4.23.2.2 TC_LPAd_AuthenticateClient_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #14 Error: Maximum Number of Retries Exceeded

This test sequence is the same as SGP.23 [32] - the Test Sequence #14 Error: Maximum Number of Retries Exceeded defined in section 4.4.23.2.2 TC_LPAd_AuthenticateClient_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #15 Error: Invalid SM-DP+(pb) certificate

This test sequence is the same as SGP.23 [32] - the Test Sequence #15 Error: Invalid SM-DP+(pb) certificate defined in section 4.4.23.2.2 TC_LPAd_AuthenticateClient_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #16 Error: Different OID for SM-DP+ Certificates (CERT.DPpb.ECDSA and CERT.DPauth.ECDSA not belonging to the same entity)

This test sequence is the same as SGP.23 [32] - the Test Sequence #16 Error: Different OID for SM-DP+ Certificates (CERT.DPpb.ECDSA and CERT.DPauth.ECDSA not belonging to the same entity) defined in section 4.4.23.2.2 TC_LPAd_AuthenticateClient_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #17 Error: Invalid SM-DP+ signature (smdpSignature2)

This test sequence is the same as SGP.23 [32] - the Test Sequence #17 Error: Invalid SM-DP+ signature (smdpSignature2) defined in section 4.4.23.2.2 TC_LPAd_AuthenticateClient_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #18 Error: Invalid TransactionID sent by SM-DP+

This test sequence is the same as SGP.23 [32] - the Test Sequence #18 Error: Invalid TransactionID sent by SM-DP+ defined in section 4.4.23.2.2 TC_LPAd_AuthenticateClient_ErrorCases where the LPAd play the role of IPAd.

### 4.2.24	ES9+ (IPA – SM-DP+): HandleNotification

#### 4.2.24.1	Conformance Requirements

References

GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

The IPA SHALL communicate with the SM-DP+ secured by HTTPS in server authentication mode as described in SGP.22 [4] section 2.6.6.

This interface is identical to the ES9+ interface defined in section 5.6 of SGP.22 [2], where the IPA plays the role of LPA.

#### 4.2.24.2	Test Cases

##### 4.2.24.2.1	TC_IPAd_ES9+_HandleNotification_Nominal

Throughout all the test cases the maximum number of Notifications simultaneously tested has been set as to two as there is not minimum defined in SGP.21 [3] or SGP.22 [2] or SGP.23 [31] for the number of Notifications that can be stored by the eUICC.

Test Sequence #01 Nominal: Successful PIR and Install Notifications to the Same SM-DP+ Address

This test sequence is the same as SGP.23 [32] - the Test Sequence #01 Nominal: Successful PIR and Install Notifications to the Same SM-DP+ Address defined in section 4.4.24.2.1 TC_LPAd_ES9+_HandleNotification_Nominal  where the LPAd play the role of IPAd.

Test Sequence #02 Nominal: Successful PIR and Enable Notifications to the Same SM-DP+ Address

This test sequence is the same as SGP.23 [32] - the Test Sequence #02 Nominal: Successful PIR and Enable Notifications to the Same SM-DP+ Address defined in section 4.4.24.2.1 TC_LPAd_ES9+_HandleNotification_Nominal  where the LPAd play the role of IPAd.

Test Sequence #03 Nominal: Disable and Delete Notifications to the Same SM-DP+ Address

This test sequence is the same as SGP.23 [32] - the Test Sequence #03 Nominal: Disable and Delete Notifications to the Same SM-DP+ Address defined in section 4.4.24.2.1 TC_LPAd_ES9+_HandleNotification_Nominal  where the LPAd play the role of IPAd.

Test Sequence #04 Nominal: Enable and Disable Notifications with Different SM-DP+ Addresses

This test sequence is the same as SGP.23 [32] - the Test Sequence #04 Nominal: Enable and Disable Notifications with Different SM-DP+ Addresses defined in section 4.4.24.2.1 TC_LPAd_ES9+_HandleNotification_Nominal  where the LPAd play the role of IPAd.

Test Sequence #05 Nominal: Different SM-DP+ Addresses in PIR and Install Notifications

This test sequence is the same as SGP.23 [32] - the Test Sequence #05 Nominal: Different SM-DP+ Addresses in PIR and Install Notifications defined in section 4.4.24.2.1 TC_LPAd_ES9+_HandleNotification_Nominal  where the LPAd play the role of IPAd.

Test Sequence #06 Nominal: Profile Download with PIR Failed

This test sequence is the same as SGP.23 [32] - the Test Sequence #01 Nominal: Successful PIR and Install Notifications to the Same SM-DP+ Address defined in section 4.4.24.2.1 TC_LPAd_ES9+_HandleNotification_Nominal  where the LPAd play the role of IPAd.

Test Sequence #07 Nominal: Successful PIR and Install Notifications after Connectivity Interruption

This Test Sequence is FFS.

Test Sequence #08 Nominal: No Acknowledge for Successful PIR results in No Further Notifications

The purpose of this test case is to verify that the next Notification of a group is not sent until LPA receives a successful response from the SM-DP+ for the previous Notification.

This test sequence is the same as SGP.23 [32] - the Test Sequence #08 Nominal: No Acknowledge for Successful PIR results in No Further Notifications defined in section 4.4.24.2.1 TC_LPAd_ES9+_HandleNotification_Nominal  where the LPAd play the role of IPAd.

Test Sequence #09 Nominal: Disable and Delete Notifications to the Same SM-DP+ Address using Delete Operation

This test sequence is the same as SGP.23 [32] - the Test Sequence #09 Nominal: Disable and Delete Notifications to the Same SM-DP+ Address using Delete Operation defined in section 4.4.24.2.1 TC_LPAd_ES9+_HandleNotification_Nominal  where the LPAd play the role of IPAd.

### 4.2.25	ES9+ (IPA – SM-DP+): CancelSession

#### 4.2.25.1	Conformance Requirements

References

GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

The IPA SHALL communicate with the SM-DP+ secured by HTTPS in server authentication mode as described in SGP.22 [4] section 2.6.6.

This interface is identical to the ES9+ interface defined in section 5.6 of SGP.22 [2], where the IPA plays the role of LPA.

#### 4.2.25.2	Test Cases

##### 4.2.25.2.1	TC_IPAd_ES9+_CancelSession_Nominal

Test Sequence #01 Nominal: Profile Download with PPR1 not allowed due to Operational Profile already present

This test sequence is the same as SGP.23 [32] - the Test Sequence #01 Nominal: Profile Download with PPR1 not allowed due to Operational Profile already present defined in section 4.4.25.2.1 TC_LPAd_ES9+_CancelSession_Nominal where the LPAd play the role of IPAd.

Test Sequence #02 Nominal: Load BPP Error

This test sequence is the same as SGP.23 [32] - the Test Sequence #03 Nominal: Load BPP Error defined in section 4.4.25.2.1 TC_LPAd_ES9+_CancelSession_Nominal where the LPAd play the role of IPAd.

Test Sequence #03 Nominal: Load BPP Error due to unknown TAG

This test sequence is the same as SGP.23 [32] - the Test Sequence #05 Nominal: Load BPP Error due to unknown TAG defined in section 4.4.25.2.1 TC_LPAd_ES9+_CancelSession_Nominal where the LPAd play the role of IPAd.

4

### 4.2.26	ES9+ (IPA – SM-DP+): HTTPS

#### 4.2.26.1	Conformance Requirements

References

GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

The IPA SHALL communicate with the SM-DP+ secured by HTTPS in server authentication mode as described in SGP.22 [4] section 2.6.6.

This interface is identical to the ES9+ interface defined in section 5.6 of SGP.22 [2], where the IPA plays the role of LPA.

#### 4.2.26.2	Test Cases

##### 4.2.26.2.1	TC_IPAd_HTTPS_Nominal

Test Sequence #01 Nominal: HTTPS Session Establishment

This test sequence is the same as SGP.23 [32] - the Test Sequence #01 Nominal: HTTPS Session Establishment defined in 4.4.26.2.1 TC_LPAd_HTTPS_Nominal where the LPAd play the role of IPAd.

Test Sequence #02 Nominal: non-reuse of session keys

The purpose of this test sequence is to verify that the LPAd is not reusing ephemeral keys from the previous session.

This test sequence is the same as SGP.23 [32] - the Test Sequence #02 Nominal: non-reuse of session keys defined in 4.4.26.2.1 TC_LPAd_HTTPS_Nominal where the LPAd play the role of IPAd.

##### 4.2.26.2.2	TC_IPAd_HTTPS_ErrorCases

Test Sequence #01 Error: Invalid (SM-DP+) TLS Certificate signature

This test sequence is the same as SGP.23 [32] - the Test Sequence #01 Error: Invalid (SM-DP+) TLS Certificate signature defined in 4.4.26.2.2 TC_LPAd_HTTPS_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #02 Error: Expired TLS Certificate

This test sequence is the same as SGP.23 [32] - the Test Sequence #02 Error: Expired TLS Certificate defined in 4.4.26.2.2 TC_LPAd_HTTPS_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #03 Error: VOID

Test Sequence #04 Error: VOID

Test Sequence #05 Error: VOID

Test Sequence #06 Error: VOID

Test Sequence #07 Error: Invalid TLS Certificate based on Invalid CI (Invalid Curve)

This test sequence is the same as SGP.23 [32] - the Test Sequence #07 Error: Invalid TLS Certificate based on Invalid CI (Invalid Curve) defined in 4.4.26.2.2 TC_LPAd_HTTPS_ErrorCases where the LPAd play the role of IPAd.

### 4.2.27	ES11 (IPA – SM-DS): InitiateAuthentication

#### 4.2.27.1	Conformance Requirements

References

GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

This interface is identical to the ES11 interface defined in section 5.8 of SGP.22 [2], where the IPA plays the role of LPA.

#### 4.2.27.2	Test Cases

##### 4.2.27.2.1	TC_IPAd_ES11_InitiateAuthentication_Nominal

Test Sequence #01 Nominal: Initiate Authentication

This test sequence is the same as SGP.23 [32] - the Test Sequence #01 Nominal: Initiate Authentication defined in section 4.4.27.2.1 TC_LPAd_ES11_InitiateAuthentication_Nominal where the LPAd play the role of IPAd.

##### 4.2.27.2.2	TC_IPAd_ES11_InitiateAuthentication_ErrorCases

Test Sequence #01 Error: Invalid SM-DS Address

This test sequence is the same as SGP.23 [32] - the Test Sequence #01 Error: Invalid SM-DS Address defined in section 4.4.27.2.2 TC_LPAd_ES11_InitiateAuthentication_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #02 Error: Unsupported Security Configuration

This test sequence is the same as SGP.23 [32] - the Test Sequence #02 Error: Unsupported Security Configuration defined in section 4.4.27.2.2 TC_LPAd_ES11_InitiateAuthentication_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #03 Error: Unsupported SVN

This test sequence is the same as SGP.23 [32] - the Test Sequence #03 Error: Unsupported SVN defined in section 4.4.27.2.2 TC_LPAd_ES11_InitiateAuthentication_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #04 Error: Unavailable SM-DS Certificate

This test sequence is the same as SGP.23 [32] - the Test Sequence #04 Error: Unavailable SM-DS Certificate defined in section 4.4.27.2.2 TC_LPAd_ES11_InitiateAuthentication_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #05 Error:  Invalid SM-DS Certificate

This test sequence is the same as SGP.23 [32] - the Test Sequence #05 Error:  Invalid SM-DS Certificate defined in section 4.4.27.2.2 TC_LPAd_ES11_InitiateAuthentication_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #06 Error:  Invalid SM-DS Signature

This test sequence is the same as SGP.23 [32] - the Test Sequence #06 Error:  Invalid SM-DS Signature defined in section 4.4.27.2.2 TC_LPAd_ES11_InitiateAuthentication_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #07 Error: Invalid SM-DS Address sent by the SM-DS

This test sequence is the same as SGP.23 [32] - the Test Sequence #07 Error: Invalid SM-DS Address sent by the SM-DS defined in section 4.4.27.2.2 TC_LPAd_ES11_InitiateAuthentication_ErrorCases where the LPAd play the role of IPAd.

Test Sequence #08 Error: Unsupported CI Key ID

This test sequence is the same as SGP.23 [32] - the Test Sequence #08 Error: Unsupported CI Key ID defined in section 4.4.27.2.2 TC_LPAd_ES11_InitiateAuthentication_ErrorCases where the LPAd play the role of IPAd.

### 4.2.28	ES11 (IPA – SM-DS): AuthenticateClient

#### 4.2.28.1	Conformance Requirements

References

GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

This interface is identical to the ES11 interface defined in section 5.8 of SGP.22 [2], where the IPA plays the role of LPA.

#### 4.2.28.2	Test Cases

##### 4.2.28.2.1	TC_IPAd_ES11_AuthenticateClient_Nominal

Test Sequence #01 Nominal: Authenticate Client with empty MatchingID

This test sequence is the same as SGP.23 [32] - the Test Sequence #01 Nominal: Authenticate Client with empty MatchingID defined in section 4.4.28.2.1 TC_LPAd_ES11_AuthenticateClient_Nominal where the LPAd play the role of IPAd.

Test Sequence #02 Nominal: Authenticate Client with MatchingID set to EventID

This test sequence is the same as SGP.23 [32] - the Test Sequence #02 Nominal: Authenticate Client with MatchingID set to EventID defined in section 4.4.28.2.1 TC_LPAd_ES11_AuthenticateClient_Nominal where the LPAd play the role of IPAd.

##### 4.2.28.2.2	TC_IPAd_ES11_AuthenticateClient_ErrorCases

Test Sequence #01 Error: Invalid EUM Certificate

This test sequence is the same as SGP.23 [32] - the Test Sequence #01 Error: Invalid EUM Certificate set to EventID defined in section 4.4.28.2.2 TC_LPAd_ES11_AuthenticateClient_ErrorCases  where the LPAd play the role of IPAd.

Test Sequence #02 Error: Expired EUM Certificate

This test sequence is the same as SGP.23 [32] - the Test Sequence #02 Error: Expired EUM Certificate defined in section 4.4.28.2.2 TC_LPAd_ES11_AuthenticateClient_ErrorCases  where the LPAd play the role of IPAd.

Test Sequence #03 Error: Invalid eUICC Certificate

This test sequence is the same as SGP.23 [32] - the Test Sequence #03 Error: Invalid eUICC Certificate defined in section 4.4.28.2.2 TC_LPAd_ES11_AuthenticateClient_ErrorCases  where the LPAd play the role of IPAd.

Test Sequence #04 Error: Expired eUICC Certificate

This test sequence is the same as SGP.23 [32] - the Test Sequence #04 Error: Expired eUICC Certificate defined in section 4.4.28.2.2 TC_LPAd_ES11_AuthenticateClient_ErrorCases  where the LPAd play the role of IPAd.

Test Sequence #05 Error: Invalid eUICC signature or serverChallenge

This test sequence is the same as SGP.23 [32] - the Test Sequence #05 Error: Invalid eUICC signature or serverChallenge defined in section 4.4.28.2.2 TC_LPAd_ES11_AuthenticateClient_ErrorCases  where the LPAd play the role of IPAd.

Test Sequence #06 Error: Unknown TransactionID

This test sequence is the same as SGP.23 [32] - the Test Sequence #06 Error: Unknown TransactionID defined in section 4.4.28.2.2 TC_LPAd_ES11_AuthenticateClient_ErrorCases  where the LPAd play the role of IPAd.

Test Sequence #07 Error: Unknown Event Record

This test sequence is the same as SGP.23 [32] - the Test Sequence #07 Error: Unknown Event Record defined in section 4.4.28.2.2 TC_LPAd_ES11_AuthenticateClient_ErrorCases  where the LPAd play the role of IPAd.

### 4.2.29	ES11 (IPA -- SM-DS): HTTPS

#### 4.2.29.1	Conformance Requirements

References

GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

This interface is identical to the ES11 interface defined in section 5.8 of SGP.22 [2], where the IPA plays the role of LPA.

#### 4.2.29.2	Test Cases

##### 4.2.29.2.1	TC_IPAd_ES11_HTTPS_Nominal

Test Sequence #01 Nominal: HTTPS Session Establishment

This test sequence is the same as SGP.23 [32] - the Test Sequence #01 Nominal: HTTPS Session Establishment defined in section 4.4.29.2.1	TC_LPAd_ES11_HTTPS_Nominal where the LPAd play the role of IPAd.

Test Sequence #02 Nominal: non-reuse of session keys

The purpose of this test sequence is to verify that the LPAd is not reusing ephemeral keys from the previous session.

This test sequence is the same as SGP.23 [32] - the Test Sequence #02 Nominal: non-reuse of session keys defined in section 4.4.29.2.1	TC_LPAd_ES11_HTTPS_Nominal where the LPAd play the role of IPAd.

##### 4.2.29.2.2	TC_IPAd_ES11_HTTPS_Error

Test Sequence #01 Error: Invalid (SM-DS) TLS Certificate signature

This test sequence is the same as SGP.23 [32] - the Test Sequence #01 Error: Invalid (SM-DS) TLS Certificate signature defined in section 4.4.29.2.2	TC_LPAd_ES11_HTTPS_Error where the LPAd play the role of IPAd.

Test Sequence #02 Error: Expired TLS Certificate

This test sequence is the same as SGP.23 [32] - the Test Sequence #02 Error: Expired TLS Certificate defined in section 4.4.29.2.2	TC_LPAd_ES11_HTTPS_Error where the LPAd play the role of IPAd.

Test Sequence #03 Error: VOID

Test Sequence #04 Error: VOID

Test Sequence #05 Error: VOID

Test Sequence #06 Error: VOID

Test Sequence #07 Error: Invalid TLS Certificate based on Invalid CI (Invalid Curve)

This test sequence is the same as SGP.23 [32] - the Test Sequence #07 Error: Invalid TLS Certificate based on Invalid CI (Invalid Curve) defined in section 4.4.29.2.2 TC_LPAd_ES11_HTTPS_Error where the LPAd play the role of IPAd.

### 4.2.29	ES11 (IPA -- SM-DS): HTTPS

#### 4.2.29.1	Conformance Requirements

References

GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

This interface is identical to the ES11 interface defined in section 5.8 of SGP.22 [2], where the IPA plays the role of LPA.

### 4.2.30	ESipa (IPA -- EIM): InitiateAuthentication

#### 4.2.30.1	Conformance Requirements

References

GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

This function requests the SM-DP+/SM-DS authentication via the eIM.

According to SGP.32 [31], the error codes returned by ESipa.InitiateAuthentication SHALL be the same as those of ES9+’.InitiateAuthentication / ES11’.InitiateAuthentication with the following additions:

- smdpAddressMismatch – indicates an error when matching SM-DP+/SM-DS Address sent in ES9+’.InitiateAuthentication with / ES11’.InitiateAuthentication SM-DP+/SM-DS Address received from the SM-DP+/SM-DS,

- smdpOidMismatch – indicates an error when matching SM-DP+ OID from AC with SM-DP+ OID from SM-DP+ Certificate

#### 4.2.30.2	Test Cases

##### 4.2.30.2.1	TC_IPAd_ESipa_ InitiateAuthentication_Nominal#

##### The test sequences for this section are FFS 4.2.30.2.2	TC_IPAd_ESipa_ InitiateAuthentication_ErrorCases

The test sequences for this section are FFS.

### 4.2.31	ESipa (IPA -- EIM): GetBoundProfilePackage

#### 4.2.31.1	Conformance Requirements

References

GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

This function requests the delivery and the binding of a Profile Package for the eUICC.

#### 4.2.31.2	Test Cases

##### 4.2.31.2.1	TC_IPAd_ESipa_GetBoundProfilePackage_Nominal

The test sequences for this section are FFS.

##### 4.2.31.2.2	TC_IPAd_ESipa_GetBoundProfilePackage _ErrorCases

The test sequences for this section are FFS.

### 4.2.32	ESipa (IPA -- EIM): AuthenticateClient

#### 4.2.32.1	Conformance Requirements

References

GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

This function SHALL be called by the IPA to request the authentication of the eUICC by the SM-DP+/SM-DS

#### 4.2.32.2	Test Cases

##### 4.2.32.2.1	TC_IPAd_ESipa_AuthenticateClient _Nominal

The test sequences for this section are FFS.

##### 4.2.32.2.2	TC_IPAd_ESipa_ AuthenticateClient _ErrorCases

The test sequences for this section are FFS.

### 4.2.33	ESipa (IPA -- EIM): TransferEimPackage

#### 4.2.33.1	Conformance Requirements

References

GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

This function is used by the eIM to transfer single eIM Package to the IPA

#### 4.2.33.2	Test Cases

##### 4.2.33.2.1	TC_IPAd_ESipa_TransferEimPackage_Nominal

The test sequences for this section are FFS.

##### 4.2.33.2.2	TC_IPAd_ESipa_TransferEimPackage_ErrorCases

The test sequences for this section are FFS.

### 4.2.34	ESipa (IPA -- EIM): GetEIMPackage

#### 4.2.34.1	Conformance Requirements

References

GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

This function is used by the IPA to retrieve an eIM Package.

#### 4.2.34.2	Test Cases

##### 4.2.34.2.1	TC_IPAd_ESipa_GetEIMPackage_Nominal

The test sequences for this section are FFS.

##### 4.2.34.2.2	TC_IPAd_ESipa_GetEIMPackage_ErrorCases

The test sequences for this section are FFS.

### 4.2.35	ESipa (IPA -- EIM): ProvideEimPackageResult

This function is used by the IPA to retrieve an eIM Package.

This function is used by the IPA to deliver an eIM Package Result optionally including one or more Notifications to the eIM in the same function call.

#### 4.2.35.1	Conformance Requirements

References

GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

#### 4.2.35.2	Test Cases

##### 4.2.35.2.1	TC_IPAd_ESipa_ ProvideEimPackageResult_Nominal

The test sequences for this section are FFS.

##### 4.2.35.2.2	TC_IPAd_ESipa_ ProvideEimPackageResult_ErrorCases

The test sequences for this section are FFS.

### 4.2.36	ESipa (IPA -- EIM): HandleNotification

#### 4.2.36.1	Conformance Requirements

References

GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

This function SHALL be called by the IPA to notify the eIM and/or SM-DP+ that a Profile has been successfully installed on the eUICC or that a profile has been successfully enabled, disabled, or deleted on the eUICC

#### 4.2.36.2	Test Cases

##### 4.2.36.2.1	TC_IPAd_ESipa_HandleNotification_Nominal

The test sequences for this section are FFS.

##### 4.2.36.2.2	TC_IPAd_ESipa_HandleNotification_ErrorCases

The test sequences for this section are FFS.

### 4.2.37	ESipa (IPA -- EIM): CancelSession

#### 4.2.37.1	Conformance Requirements

References

GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

This function SHALL be called by the eIM to request the cancellation of an on-going RSP session.

#### 4.2.37.2	Test Cases

##### 4.2.37.2.1	TC_IPAd_ESipa_CancelSession_Nominal

The test sequences for this section are FFS.

##### 4.2.37.2.2	TC_IPAd_ESipa_CancelSession_ErrorCases

The test sequences for this section are FFS.

## 4.3	TLS Interface

### 4.3.1	TLS, Server Authentication, TLS Establishment

#### 4.3.1.1	Conformance Requirements

GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

#### 4.3.1.2	Test Cases

##### 4.3.1.2.1	TC_Server_Authentication_for_HTTPS_EstablishmentNIST

The test sequences for this section are FFS.

##### 4.3.1.2.2	TC_Server_Authentication_for_HTTPS_EstablishmentBRP

The test sequences for this section are FFS.

# 5	Procedure - Behaviour Testing

## 5.1	General Overview

## 5.2 VOID

## 5.3 VOID

## 5.4	Device Procedures

### 5.4.1	Profile Download

#### 5.4.1.1	Conformance Requirements

References

- GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

#### 5.4.1.2	Test Cases

##### 5.4.1.2.1	TC_IPAd_DirectProfileDownload_IPA_initiated_with_Activation_Code

Test Sequence #01 Nominal: Add a new Operational Profile initiated by IPA by using Activation Code

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1). |
| S_eIM | #ACTIVATION_CODE_1 is available on S_eIM |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| 1 | PROC_ESIPA_GET_EIM_PACKAGE_PROFILE_DOWNLOAD_AC with #ACTIVATION_CODE_1 as <ACTIVATION_CODE> |  |  |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH |  |  |
| 4 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_1 as <MATCHING_ID> |  |  |
| 5 | PROC_ES9+_GET_BPP |  |  |
| 6 | PROC_ES9+_HANDLE_NOTIF See NOTE1 |  |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| 8 | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_PDTR See NOTE1 |  |  |
| 9 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 10 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_1> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 11 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_1> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE1: The Notification and eIM Package Result (steps 6 and 8) MAY be sent to S_eIM and S_SM-DP+ in any order or in parallel. NOTE2: This procedure needs to be run only if the TLS connection is not initialized on ESipa. |  |  |  |

Test Sequence #02 Nominal: Add a new Operational Profile initiated by IPA by using Activation Code with Confirmation Code

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_3 (PROFILE_OPERATIONAL1) associated with #CONFIRMATION_CODE1. |
| S_eIM | #ACTIVATION_CODE_3 is available on S_eIM |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| 1 | PROC_ESIPA_GET_EIM_PACKAGE_PROFILE_DOWNLOAD_AC with #ACTIVATION_CODE_3 as <ACTIVATION_CODE> |  |  |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH |  |  |
| 4 | PROC_ES9+_AUTH_CLIENT_CC with #MATCHING_ID_3 as <MATCHING_ID> |  |  |
| 5 | CONFIRMATION_CODE1 is provided to the IPA See NOTE1 |  |  |
| 6 | PROC_ES9+_GET_BPP_CC |  |  |
| 7 | PROC_ES9+_HANDLE_NOTIF See NOTE2 |  |  |
| 8 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE3 |  |  |
| 9 | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_PDTR See NOTE2 |  |  |
| 10 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE3 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 11 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_1> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 12 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_1> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE1: the method to provide confirmation code to IPAd is IPAd dependant NOTE2: The Notification and eIM Package Result (steps 7 and 9) MAY be sent to S_eIM and S_SM-DP+ in any order or in parallel. NOTE3: This procedure needs to be run only if the TLS connection is not initialized on ESipa. |  |  |  |

##### 5.4.1.2.2	TC_IPAd_DirectProfileDownload_IPA_initiated_ActivationCode_InvalidFormat

Test Sequence #01 Error: Add a new Operational Profile initiated by eIM by using wrongly formatted Activation Code

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_eIM | #ACTIVATION_CODE_INVALID_FORMAT is available on S_eIM |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| IC2 | IPAd is triggered to send ESipa.GetEimPackage method See NOTE |  |  |
| 1 | IPAd → S_eIM | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA ( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |
| 2 | S_eIM → IPAd | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_PROFILE_DOWNLOAD_TRIGGER_AC_OK with #ACTIVATION_CODE_INVALID_FORMAT as <ACTIVATION_CODE>) |  |
| 3 | IPAd → S_EIM+ | Send ESipa.HandleNotification method with eIMPackageResultResponseError | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_EPRRE_INVALID_PACKAGE_FORMAT)) |
| 4 | S_EIM → IPAd | #R_HTTP_204_OK | IPAd does not send TLS Client Hello to S_SM-DP+ within IUT_IPAd_TLS_INIT |
| NOTE: It is IPAd dependent, if there is a need for a separate trigger, or the trigger in PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA is triggering the eIM Package retrieval procedure. |  |  |  |

##### 5.4.1.2.3	TC_IPAd_DirectProfileDownload_IPA_initiated_with_ConfirmationCode_smdpSigned2

Test Sequence #01 Nominal: Add a new Operational Profile initiated by IPA by using Activation Code with confirmation code indicated only in smdpSigned2

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_1 (PROFILE_OPERATIONAL1) which requires confirmation code. |
| S_eIM | #ACTIVATION_CODE_1 is available on S_eIM |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| 1 | PROC_ESIPA_GET_EIM_PACKAGE_PROFILE_DOWNLOAD_AC with #ACTIVATION_CODE_1 as <ACTIVATION_CODE> |  |  |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH |  |  |
| 4 | PROC_ES9+_AUTH_CLIENT_CC with #MATCHING_ID_1 as <MATCHING_ID> |  |  |
| 5 | CONFIRMATION_CODE1 is provided to the IPA See NOTE1 |  |  |
| 6 | PROC_ES9+_GET_BPP_CC |  |  |
| 7 | PROC_ES9+_HANDLE_NOTIF See NOTE2 |  |  |
| 8 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE3 |  |  |
| 9 | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_PDTR See NOTE2 |  |  |
| 10 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE3 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 11 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_1> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 12 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_1> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE1: the method to provide confirmation code to IPAd is IPAd dependant NOTE2: The Notification and eIM Package Result (steps 7 and 9) MAY be sent to S_eIM and S_SM-DP+ in any order or in parallel. NOTE:3 This procedure needs to be run only if the TLS connection is not initialized on ESipa. |  |  |  |

##### 5.4.1.2.4	TC_IPAd_DirectProfileDownload_IPA_initiated_default SM-DP+

Test Sequence #01 Nominal: DirectProfileDownload from the default SM-DP+ address

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for PROFILE_OPERATIONAL1 linked to the EID of the eUICC. |
| S_eIM | No secure connection is established between S_eIM and IPAd. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| 1 | PROC_ESIPA_GET_EIM_PACKAGE_PROFILE_DOWNLOAD_DEFAULT_SM-DP+ |  |  |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH |  |  |
| 4 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_EMPTY as <MATCHING_ID> or missing MatchingID data object |  |  |
| 5 | PROC_ES9+_GET_BPP |  |  |
| 6 | PROC_ES9+_HANDLE_NOTIF See NOTE1 |  |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| 8 | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_PDTR See NOTE |  |  |
| 9 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 10 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_1> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 11 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_1> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE1: The Notification and eIM Package Result (steps 6 and 8) MAY be sent to S_eIM and S_SM-DP+ in any order or in parallel. NOTE2: This procedure needs to be run only if the TLS connection is not initialized on ESipa. |  |  |  |

##### 5.4.1.2.5	TC_IPAd_DirectProfileDownload_IPA_initiated_with_PPRs

Test Sequence #01 Nominal: DirectProfileDownload with PPR1

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The Test eUICC’s RAT is configured as follows: PPR1 is allowed and End User Consent is not required for #MCC_MNC4 with gid1 and gid2 absent. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_4 (associated with PROFILE_OPERATIONAL4). |
| S_eIM | #ACTIVATION_CODE_4 is available on S_eIM |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| 1 | PROC_ESIPA_GET_EIM_PACKAGE_PROFILE_DOWNLOAD_AC with #ACTIVATION_CODE_4 as <ACTIVATION_CODE> |  |  |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH |  |  |
| 4 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_4 as <MATCHING_ID> |  |  |
| 5 | PROC_ES9+_GET_BPP |  |  |
| 6 | PROC_ES9+_HANDLE_NOTIF See NOTE1 |  |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| 8 | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_PDTR See NOTE1 |  |  |
| 9 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 10 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_4> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 11 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_4> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE1: The Notification and eIM Package Result (steps 6 and 8) MAY be sent to S_eIM and S_SM-DP+ in any order or in parallel. NOTE2: This procedure needs to be run only if the TLS connection is not initialized on ESipa. |  |  |  |

Test Sequence #02 Nominal: DirectProfileDownload with PPR2

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The Test eUICC’s RAT is configured as follows: PPR2 is allowed and End User Consent is not required for #MCC_MNC2 with gid1 and gid2 absent. |
| S_SM-DP+ | There is a pending Profile download order for #MATCHING_ID_3 (associated with PROFILE_OPERATIONAL3). |
| S_eIM | #ACTIVATION_CODE_3_NO_CC is available on S_eIM |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| 1 | PROC_ESIPA_GET_EIM_PACKAGE_PROFILE_DOWNLOAD_AC with #ACTIVATION_CODE_3_NO_CC as <ACTIVATION_CODE> |  |  |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH |  |  |
| 4 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_3 as <MATCHING_ID> |  |  |
| 5 | PROC_ES9+_GET_BPP |  |  |
| 6 | PROC_ES9+_HANDLE_NOTIF See NOTE1 |  |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| 8 | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_PDTR See NOTE1 |  |  |
| 9 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 10 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_3> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 11 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_3> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE1: The Notification and eIM Package Result (steps 6 and 8) MAY be sent to S_eIM and S_SM-DP+ in any order or in parallel. NOTE2: This procedure needs to be run only if the TLS connection is not initialized on ESipa. |  |  |  |

Test Sequence #03 Nominal: DirectProfileDownload when profile with PPR1 already present

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Entity |  | Description of the initial condition |  |
| eUICC |  | The Test eUICC’s RAT is configured as follows: PPR1 is allowed and End User Consent is not required for #MCC_MNC4 with gid1 and gid2 absent. |  |
| eUICC | The PROFILE_OPERATIONAL4 with PPR1 is installed and enabled on the eUICC. |  |
| S_SM-DP+ |  | There is a pending Profile download order for #MATCHING_ID_1 (associated with PROFILE_OPERATIONAL1). |  |
| S_eIM |  | #ACTIVATION_CODE_1 is available on S_eIM |  |
| S_eIM |  | No secure connection is established between S_eIM and IPAd |  |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| 1 | PROC_ESIPA_GET_EIM_PACKAGE_PROFILE_DOWNLOAD_AC with #ACTIVATION_CODE_1 as <ACTIVATION_CODE> |  |  |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH |  |  |
| 4 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_1 as <MATCHING_ID> |  |  |
| 5 | PROC_ES9+_GET_BPP |  |  |
| 6 | PROC_ES9+_HANDLE_NOTIF See NOTE1 |  |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| 8 | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_PDTR See NOTE1 |  |  |
| 9 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 10 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_1> and <PROFILE_INFO_IOT_4> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 11 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_1> and <PROFILE_INFO_IOT_4> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE1: The Notification and eIM Package Result (steps 6 and 8) MAY be sent to S_eIM and S_SM-DP+ in any order or in parallel. NOTE2: This procedure needs to be run only if the TLS connection is not initialized on ESipa. |  |  |  |

##### 5.4.1.2.6	TC_IPAd_DirectProfileDownload_IPA_initiated_with_Empty MatchingID

Test Sequence #01 Nominal: Add a new Operational Profile initiated by IPA by using Activation Code with empty MatchingID

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for PROFILE_OPERATIONAL1 linked to the EID of the eUICC. |
| S_eIM | No secure connection is established between S_eIM and IPAd. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| 1 | PROC_ESIPA_GET_EIM_PACKAGE_PROFILE_DOWNLOAD_AC with #ACTIVATION_CODE_5 as <ACTIVATION_CODE> |  |  |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |
| 3 | PROC_ES9+_INIT_AUTH |  |  |
| 4 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_EMPTY as <MATCHING_ID> |  |  |
| 5 | PROC_ES9+_GET_BPP |  |  |
| 6 | PROC_ES9+_HANDLE_NOTIF See NOTE1 |  |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| 8 | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_PDTR See NOTE1 |  |  |
| 9 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 10 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_1> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 11 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_1> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE1: The Notification and eIM Package Result (steps 6 and 8) MAY be sent to S_eIM and S_SM-DP+ in any order or in parallel. NOTE2: This procedure needs to be run only if the TLS connection is not initialized on ESipa. |  |  |  |

##### 5.4.1.2.7	TC_IPAd_DirectProfileDownload_IPA_initiated_default SM-DP+_immediate_profile_enabling

Test Sequence #01 Nominal: DirectProfileDownload from the default SM-DP+ address with immediate profile enabling

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| S_SM-DP+ | The PROFILE_OPERATIONAL1 on the S_SM-DP+ is in “Released” state. |
| S_SM-DP+ | There is a pending Profile download order for PROFILE_OPERATIONAL1 linked to the EID of the eUICC. |
| S_eIM | No secure connection is established between S_eIM and IPAd. |

| Step | Direction | Sequence / Description |  | Expected result |  |
| --- | --- | --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |  |  |
| IC2 | PROC_ESIPA_GET_EIM_PACKAGE_CONFIGURE_IMMEDIATE_ENABLE |  |  |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |  |  |
| IC3 | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_CIER |  |  |  |  |
| ENDIF |  |  |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |  |  |
| IC4 | PROC_ESIPA_PROVIDE_EIM_PACKAGE_RESULT_CIER |  |  |  |  |
| ENDIF |  |  |  |  |  |
| IC4 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE1 |  |  |  |  |
| 1 | PROC_ESIPA_GET_EIM_PACKAGE_PROFILE_DOWNLOAD_DEFAULT_SM-DP+ |  |  |  |  |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ |  |  |  |  |
| 3 | PROC_ES9+_INIT_AUTH |  |  |  |  |
| 4 | PROC_ES9+_AUTH_CLIENT with #MATCHING_ID_EMPTY as <MATCHING_ID> or missing MatchingID data object |  |  |  |  |
| 5 | PROC_ES9+_GET_BPP |  |  |  |  |
| 6 | PROC_ES9+_HANDLE_NOTIF See NOTE2 |  |  |  |  |
| 7 | IPAd → S_SM-DP+ |  | Send the Enable Notification containing #ICCID_OP_PROF1 |  | The Enable Notification MTD_HANDLE_NOTIF(#PENDING_NOTIF_EN1) is received by the S_SM-DP+ within the timeout #IUT_IPAd_NOTIFICATION_TIMEOUT Verify the euiccNotificationSignature <TBS_EUICC_NOTIF_SIG> using the #PK_EUICC_ECDSA S_SM-DP+ SHALL return #R_HTTP_204_OK |
| 8 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE1 |  |  |  |  |
| 9 | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_PDTR See NOTE2 |  |  |  |  |
| 10 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE1 |  |  |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |  |  |
| 11 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_2> as <PROFILE_INFO> |  |  |  |  |
| ENDIF |  |  |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |  |  |
| 12 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_2> as <PROFILE_INFO> |  |  |  |  |
| ENDIF |  |  |  |  |  |
| NOTE1: This procedure needs to be run only if the TLS connection is not initialized on ESipa. NOTE2: The Notification and eIM Package Result (steps 6 and 9) MAY be sent to S_eIM and S_SM-DP+ in any order or in parallel. |  |  |  |  |  |

### 5.4.2	VOID

### 5.4.3	VOID

### 5.4.4	Local Profile Management - Delete Profile

#### 5.4.4.1	Conformance Requirements

References

- GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

#### 5.4.4.2	Test Cases

##### 5.4.4.2.1	TC_IPAd_DeleteProfile_Disabled_without_PPR_IPA_initiated

Test Sequence #01 Nominal: Deleting Disabled Profile, No PPRs

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is in Disabled state. |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| 1 | PROC_ESIPA_GET_EIM_PACKAGE_DELETE_PROFILE |  |  |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ (S_SERVER configured with #TEST_DP_ADDRESS1) |  |  |
| 3 | PROC_ES9+_HANDLE_NOTIF_DEL1 See NOTE1 |  |  |
| 4 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 5 | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_DELPR See NOTE1 |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 6 | PROC_ESIPA_PROVIDE_EIM_PACKAGE_RESULT_DELPR See NOTE1 |  |  |
| ENDIF |  |  |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 8 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with empty <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 9 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with empty <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE1: The Notifications (steps 3, 5 and 6) MAY be sent to S_eIM and S_SM-DP+ in any order or in parallel. NOTE2: This procedure needs to be run only if the TLS connection is not initialized on ESipa. |  |  |  |

##### 5.4.4.2.2	TC_IPAd_DeleteProfile_Enabled_without_PPR_IPA_initiated

Test Sequence #01 Nominal: Deleting Enabled Profile, No PPRs

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is in Enabled state. |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| 1 | PROC_ESIPA_GET_EIM_PACKAGE_DELETE_PROFILE |  |  |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ (S_SERVER configured with #TEST_DP_ADDRESS1) |  |  |
| 3 | PROC_ES9+_HANDLE_NOTIF_DIS1 See NOTE1 |  |  |
| 4 | PROC_ES9+_HANDLE_NOTIF_DEL1 See NOTE1 |  |  |
| 5 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 6 | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_DPR See NOTE1 |  |  |
| 7 | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_DELPR See NOTE1 |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 8 | PROC_ESIPA_PROVIDE_EIM_PACKAGE_RESULT_DPR See NOTE1 |  |  |
| 9 | PROC_ESIPA_PROVIDE_EIM_PACKAGE_RESULT_DELPR See NOTE1 |  |  |
| ENDIF |  |  |  |
| 10 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 11 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with empty <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 12 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with empty <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE1: The Notifications (steps 3, 4, 6, 7, 8 and 9) MAY be sent to S_eIM and S_SM-DP+ in any order or in parallel. NOTE2: This procedure needs to be run only if the TLS connection is not initialized on ESipa. |  |  |  |

##### 5.4.4.2.3	TC_IPAd_DeleteProfile_Error_with_PPR1_IPA_initiated

Test Sequence #01 Error: Deleting Enabled Profile, PPR1 set

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The Test eUICC’s RAT is configured as follows: PPR1 is allowed for #MCC_MNC4 with gid1 and gid2 absent |
| eUICC | The PROFILE_OPERATIONAL4 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL4 is in Enabled state. |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| IC2 | IPA is triggered to send ESipa.GetEimPackage method See NOTE1 |  |  |
| 1 | IPAd → S_eIM | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA ( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |
| 2 | S_eIM → IPAd | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_DELETE_PROFILE_4_TRIGGER_OK) | IPAd does not send any notification to S_SM-DP+ |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 3 | IPAd → S_EIM | Send ESipa.HandleNotification method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_DELPR_ERR_PPR)) |
| 4 | S_EIM → IPAd | #R_HTTP_204_OK | No error |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 5 | IPAd → S_EIM | Send ESipa.ProvideEimPackageResult method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_PROVIDE_EIM_PACKAGE_RESULT, MTD_PROVIDE_EIM_PACKAGE_RESULT (#R_EPR_DELPR_ERR_PPR)) |
| 6 | S_EIM → IPAd | MTD_HTTP_RESP_ESIPA (#S_EIM_ACKNOWLEDGEMENT) | No error |
| ENDIF |  |  |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 8 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_4_EN> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 9 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_4_EN> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE1: It is IPA dependent, if there is a need for a separate trigger, or the trigger in PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA is triggering the eIM Package retrieval procedure. NOTE2: This procedure needs to be run only if the TLS connection is not initialized on ESipa. |  |  |  |

##### 5.4.4.2.4	TC_IPAd_DeleteProfile_Error_Disabled_with_PPR2_IPA_initiated

Test Sequence #01 Error: Deleting Disabled Profile, PPR2 set

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The Test eUICC’s RAT is configured as follows: PPR2 is allowed for #MCC_MNC2 with gid1 and gid2 absent |
| eUICC | The PROFILE_OPERATIONAL7 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL7 is in Disabled state. |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| IC2 | IPA is triggered to send ESipa.GetEimPackage method See NOTE1 |  |  |
| 1 | IPAd → S_eIM | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA ( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |
| 2 | S_eIM → IPAd | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_DELETE_PROFILE_7_TRIGGER_OK) | IPAd does not send any notification to S_SM-DP+ |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 3 | IPAd → S_EIM | Send ESipa.HandleNotification method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_DELPR_ERR_PPR)) |
| 4 | S_EIM → IPAd | #R_HTTP_204_OK | No error |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 5 | IPAd → S_EIM | Send ESipa.ProvideEimPackageResult method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_PROVIDE_EIM_PACKAGE_RESULT, MTD_PROVIDE_EIM_PACKAGE_RESULT (#R_EPR_DELPR_ERR_PPR)) |
| 6 | S_EIM → IPAd | MTD_HTTP_RESP_ESIPA (#S_EIM_ACKNOWLEDGEMENT) | No error |
| ENDIF |  |  |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 8 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_7_DIS> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 9 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_7_DIS> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE1: It is IPA dependent, if there is a need for a separate trigger, or the trigger in PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA is triggering the eIM Package retrieval procedure. NOTE2: This procedure needs to be run only if the TLS connection is not initialized on ESipa. |  |  |  |

##### 5.4.4.2.5	TC_IPAd_DeleteProfile_Error_Enabled_with_PPR2_IPA_initiated

Test Sequence #01 Error: Deleting Enabled Profile, PPR2 set

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The Test eUICC’s RAT is configured as follows: PPR2 is allowed for #MCC_MNC2 with gid1 and gid2 absent |
| eUICC | The PROFILE_OPERATIONAL8 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL8 is in Enabled state. |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step |  | Direction | Sequence / Description | Expected result |  |
| --- | --- | --- | --- | --- | --- |
| IC1 |  | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |  |
| IC2 |  | IPA is triggered to send ESipa.GetEimPackage method See NOTE1 |  |  |  |
| 1 |  | IPAd → S_eIM | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA ( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |  |
| 2 |  | S_eIM → IPAd | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_DELETE_PROFILE_8_TRIGGER_OK) | IPAd does not send any notification to S_SM-DP+ |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |  |  |
| 3 | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_DPR See NOTE3 |  |  |  |
| 4 |  | IPAd → S_EIM | Send ESipa.HandleNotification method with eIM Package Result See NOTE3 | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_DELPR_ERR_PPR)) |  |
| 5 |  | S_EIM → IPAd | #R_HTTP_204_OK | No error |  |
| ENDIF |  |  |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |  |  |
| 6 | PROC_ESIPA_PROVIDE_EIM_PACKAGE_RESULT_DPR See NOTE3 |  |  |  |
| 7 |  | IPAd → S_EIM | Send ESipa.ProvideEimPackageResult method with eIM Package Result See NOTE3 | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_PROVIDE_EIM_PACKAGE_RESULT, MTD_PROVIDE_EIM_PACKAGE_RESULT (#R_EPR_DELPR_ERR_PPR)) |  |
| 8 |  | S_EIM → IPAd | MTD_HTTP_RESP_ESIPA (#S_EIM_ACKNOWLEDGEMENT) | No error |  |
| ENDIF |  |  |  |  |  |
| 9 |  | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |  |  |
| 10 |  | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_8_DIS> as <PROFILE_INFO> |  |  |  |
| ENDIF |  |  |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |  |  |
| 11 |  | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_8_DIS> as <PROFILE_INFO> |  |  |  |
| ENDIF |  |  |  |  |  |
| NOTE1: It is IPA dependent, if there is a need for a separate trigger, or the trigger in PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA is triggering the eIM Package retrieval procedure. NOTE2: This procedure needs to be run only if the TLS connection is not initialized on ESipa. NOTE3: The Notifications (steps 3, 4, 6, 7) MAY be sent to eIM in any order. |  |  |  |  |  |

### 5.4.5	Profile State Management Operation - Enable Profile

#### 5.4.5.1	Conformance Requirements

References

- GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

#### 5.4.5.2	Test Cases

##### 5.4.5.2.1	TC_IPAd_EnableProfile_IPA_initiated

Test Sequence #01 Nominal: Enable a formerly disabled Profile

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is in Disabled state. |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| 1 | PROC_ESIPA_GET_EIM_PACKAGE_ENABLE_PROFILE |  |  |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ (S_SERVER configured with #TEST_DP_ADDRESS1) |  |  |
| 3 | PROC_ES9+_HANDLE_NOTIF_EN1 See NOTE1 |  |  |
| 4 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 5 | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_EPRSee NOTE1 |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 6 | PROC_ESIPA_PROVIDE_EIM_PACKAGE_RESULT_EPR See NOTE1 |  |  |
| ENDIF |  |  |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 8 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_2> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 9 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_2> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE1: The Notifications (steps 3, 5 and 6) MAY be sent to S_eIM and S_SM-DP+ in any order or in parallel. NOTE2: This procedure needs to be run only if the TLS connection is not initialized on ESipa. |  |  |  |

##### 5.4.5.2.2	TC_IPAd_EnableProfile_ImplicitDisable_IPA_initiated

Test Sequence #01 Nominal: Enable a Profile with implicit disabling of the formerly enabled Profile

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 and PROFILE_OPERATIONAL2 are installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is in Disabled state. |
| eUICC | The PROFILE_OPERATIONAL2 is in Enabled state. |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| 1 | PROC_ESIPA_GET_EIM_PACKAGE_ENABLE_PROFILE |  |  |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ (S_SERVER configured with #TEST_DP_ADDRESS1) |  |  |
| 3 | PROC_ES9+_HANDLE_NOTIF_EN1 See NOTE1 |  |  |
| 4 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ (S_SERVER configured with #TEST_DP_ADDRESS2) |  |  |
| 5 | PROC_ES9+_HANDLE_NOTIF_DIS2 See NOTE1 |  |  |
| 6 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 7 | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_EPR See NOTE1 |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 8 | PROC_ESIPA_PROVIDE_EIM_PACKAGE_RESULT_EPR See NOTE1 |  |  |
| ENDIF |  |  |  |
| 9 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 10 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_5> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 11 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_5> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE: The Notifications (steps 3, 5, 7 and 8) MAY be sent to S_eIM and S_SM-DP+ in any order or in parallel. NOTE2: This procedure needs to be run only if the TLS connection is not initialized on ESipa. |  |  |  |

##### 5.4.5.2.3	TC_IPAd_EnableProfile_Error_ProfileAlreadyEnabled_IPA_initiated

Test Sequence #01 Error: Enable an already enabled Profile

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is in Enabled state. |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| IC2 | IPA is triggered to send ESipa.GetEimPackage method See NOTE |  |  |
| 1 | IPAd → S_eIM | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA ( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |
| 2 | S_eIM → IPAd | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_ENABLE_PROFILE_NO_RB_TRIGGER_OK) | IPAd does not send any notification to S_SM-DP+ |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 3 | IPAd → S_EIM | Send ESipa.HandleNotification method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_EPR_ERR_NOT_DIS)) |
| 4 | S_EIM → IPAd | #R_HTTP_204_OK | No error |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 5 | IPAd → S_EIM | Send ESipa.ProvideEimPackageResult method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_PROVIDE_EIM_PACKAGE_RESULT, MTD_PROVIDE_EIM_PACKAGE_RESULT (#R_EPR_EPR_ERR_NOT_DIS)) |
| 6 | S_EIM → IPAd | MTD_HTTP_RESP_ESIPA (#S_EIM_ACKNOWLEDGEMENT) | No error |
| ENDIF |  |  |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 8 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_2> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 9 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_2> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE: This procedure needs to be run only if the TLS connection is not initialized on ESipa. |  |  |  |

##### 5.4.5.2.4	TC_IPAd_EnableProfile_Error_PPR1Set_IPA_initiated

Test Sequence #01 Error: Enabled Profile when a formerly enabled Profile has set PPR1

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The Test eUICC’s RAT is configured as follows: PPR1 is allowed for #MCC_MNC4 with gid1 and gid2 absent |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is in Disabled state. |
| eUICC | The PROFILE_OPERATIONAL4 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL4 is in Enabled state. |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| IC2 | IPA is triggered to send ESipa.GetEimPackage method See NOTE |  |  |
| 1 | IPAd → S_eIM | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA ( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |
| 2 | S_eIM → IPAd | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_ENABLE_PROFILE_NO_RB_TRIGGER_OK) | IPAd does not send any notification to S_SM-DP+ |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 3 | IPAd → S_EIM | Send ESipa.HandleNotification method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_EPR_ERR_PPR)) |
| 4 | S_EIM → IPAd | #R_HTTP_204_OK | No error |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 5 | IPAd → S_EIM | Send ESipa.ProvideEimPackageResult method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_PROVIDE_EIM_PACKAGE_RESULT, MTD_PROVIDE_EIM_PACKAGE_RESULT (#R_EPR_EPR_ERR_PPR)) |
| 6 | S_EIM → IPAd | MTD_HTTP_RESP_ESIPA (#S_EIM_ACKNOWLEDGEMENT) | No error |
| ENDIF |  |  |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 8 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_6> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 9 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_6> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE: This procedure needs to be run only if the TLS connection is not initialized on ESipa. |  |  |  |

##### 5.4.5.2.5	 TC_LPAd_EnableProfile_Error_RollbackNoEnabledProfile_IPA_initiated

Test Sequence #01 Error: Enable a Profile with Rollback set while no other profile is Enabled

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is in Disabled state. |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| IC2 | IPA is triggered to send ESipa.GetEimPackage method See NOTE |  |  |
| 1 | IPAd → S_eIM | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA ( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |
| 2 | S_eIM → IPAd | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_ENABLE_PROFILE_RB_TRIGGER_OK) | IPAd does not send any notification to S_SM-DP+ |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 3 | IPAd → S_EIM | Send ESipa.HandleNotification method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_EPR_ERR_UNKNOWN)) |
| 4 | S_EIM → IPAd | #R_HTTP_204_OK | No error |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 5 | IPAd → S_EIM | Send ESipa.ProvideEimPackageResult method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_PROVIDE_EIM_PACKAGE_RESULT, MTD_PROVIDE_EIM_PACKAGE_RESULT (#R_EPR_EPR_ERR_UNKNOWN)) |
| 6 | S_EIM → IPAd | MTD_HTTP_RESP_ESIPA (#S_EIM_ACKNOWLEDGEMENT) | No error |
| ENDIF |  |  |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 8 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_1> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 9 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_1> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE: This procedure needs to be run only if the TLS connection is not initialized on ESipa. |  |  |  |

### 5.4.6	Local Profile Management - Disable Profile

#### 5.4.6.1	Conformance Requirements

References

- GSMA RSP Technical Specification [2] and GSMA IoT eSIM Technical Specification [31]

#### 5.4.6.2	Test Cases

##### 5.4.6.2.1	TC_IPAd_DisableProfile_IPA_Initiated

Test Sequence #01 Nominal: Disable an Enabled Profile

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is in Enabled state. |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| 1 | PROC_ESIPA_GET_EIM_PACKAGE_DISABLE_PROFILE |  |  |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH on ES9+ (S_SERVER configured with #TEST_DP_ADDRESS1) |  |  |
| 3 | PROC_ES9+_HANDLE_NOTIF_DIS1 See NOTE1 |  |  |
| 4 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 5 | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_DPRSee NOTE1 |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 6 | PROC_ESIPA_PROVIDE_EIM_PACKAGE_RESULT_DPR See NOTE1 |  |  |
| ENDIF |  |  |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 8 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_1> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 9 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_1> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE1: The Notifications (steps 3, 5 and 6) MAY be sent to S_eIM and S_SM-DP+ in any order or in parallel. NOTE2: This procedure needs to be run only if the TLS connection is not initialized on ESipa. |  |  |  |

##### 5.4.6.2.2	TC_IPAd_DisableProfile_Error_ProfileAlreadyDisabled_IPA_Initiated

Test Sequence #01 Error: Disable an already disabled Profile

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is in Disabled state. |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| IC2 | IPA is triggered to send ESipa.GetEimPackage method See NOTE1 |  |  |
| 1 | IPAd → S_eIM | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA ( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |
| 2 | S_eIM → IPAd | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_DISABLE_PROFILE_TRIGGER_OK) | IPAd does not send any notification to S_SM-DP+ |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 3 | IPAd → S_EIM | Send ESipa.HandleNotification method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_DPR_ERR_NOT_EN)) |
| 4 | S_EIM → IPAd | #R_HTTP_204_OK | No error |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 5 | IPAd → S_EIM | Send ESipa.ProvideEimPackageResult method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_PROVIDE_EIM_PACKAGE_RESULT, MTD_PROVIDE_EIM_PACKAGE_RESULT (#R_EPR_DPR_ERR_NOT_EN)) |
| 6 | S_EIM → IPAd | MTD_HTTP_RESP_ESIPA (#S_EIM_ACKNOWLEDGEMENT) | No error |
| ENDIF |  |  |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 8 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_1> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 9 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_1> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE1: It is IPA dependent, if there is a need for a separate trigger, or the trigger in PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA is triggering the eIM Package retrieval procedure. NOTE2: This procedure needs to be run only if the TLS connection is not initialized on ESipa. |  |  |  |

##### 5.4.6.2.3	TC_IPAd_DisableProfile_Error_PPR1Set_IPA_Initiated

Test Sequence #01 Error: Disable an Enabled Profile with PPR1 set

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The Test eUICC’s RAT is configured as follows: PPR1 is allowed and End User Consent is not required for #MCC_MNC4 with gid1 and gid2 absent. |
| eUICC | The PROFILE_OPERATIONAL4 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL4 is in Enabled state. |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| IC2 | IPA is triggered to send ESipa.GetEimPackage method See NOTE1 |  |  |
| 1 | IPAd → S_eIM | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA ( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |
| 2 | S_eIM → IPAd | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_DISABLE_PROFILE_4_TRIGGER_OK) | IPAd does not send any notification to S_SM-DP+ |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 3 | IPAd → S_EIM | Send ESipa.HandleNotification method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_DPR_ERR_PPR)) |
| 4 | S_EIM → IPAd | #R_HTTP_204_OK | No error |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 5 | IPAd → S_EIM | Send ESipa.ProvideEimPackageResult method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_PROVIDE_EIM_PACKAGE_RESULT, MTD_PROVIDE_EIM_PACKAGE_RESULT (#R_EPR_DPR_ERR_PPR)) |
| 6 | S_EIM → IPAd | MTD_HTTP_RESP_ESIPA (#S_EIM_ACKNOWLEDGEMENT) | No error |
| ENDIF |  |  |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 8 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_4_EN> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 9 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_4_EN> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE1: It is IPA dependent, if there is a need for a separate trigger, or the trigger in PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA is triggering the eIM Package retrieval procedure. NOTE2: This procedure needs to be run only if the TLS connection is not initialized on ESipa. |  |  |  |

### 5.4.7	VOID

### 5.4.8	VOID

### 5.4.9	VOID

### 5.4.10	VOID

### 5.4.11	VOID

### 5.4.12	Local Profile Management – Set fallback attribute

#### 5.4.12.1	Conformance Requirements

References

- GSMA IoT eSIM Technical Specification [31]

#### 5.4.12.2	Test Cases

##### 5.4.12.2.1	TC_IPAd_SetFallbackAttribute

Test Sequence #01 Nominal: Set Fallback Attribute

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_FALLBACK_ALLOWED is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_FALLBACK_ALLOWED is in Disabled state. |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| 1 | PROC_ESIPA_GET_EIM_PACKAGE_SET_FALLBACK |  |  |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE1 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 3 | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_SET_FALLBACK |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 4 | PROC_ESIPA_PROVIDE_EIM_PACKAGE_RESULT_SET_FALLBACK |  |  |
| ENDIF |  |  |  |
| 5 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE1 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 6 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_1_FALLBACK> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 7 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_1_FALLBACK> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE1: This procedure needs to be ran only if the TLS connection is not initialized on ESipa. |  |  |  |

Test Sequence #02 Nominal: Set Fallback Attribute when already set

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_FALLBACK_SET is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_FALLBACK_SET is in Disabled state. |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| 1 | PROC_ESIPA_GET_EIM_PACKAGE_SET_FALLBACK |  |  |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE1 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 3 | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_SET_FALLBACK |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 4 | PROC_ESIPA_PROVIDE_EIM_PACKAGE_RESULT_SET_FALLBACK |  |  |
| ENDIF |  |  |  |
| 5 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE1 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 6 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_1_FALLBACK> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 7 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_1_FALLBACK> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE1: This procedure needs to be ran only if the TLS connection is not initialized on ESipa. |  |  |  |

Test Sequence #03 Nominal: Set Fallback Attribute with a Disabled Fallback Profile

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL2 with METADATA_OP_PROF2_FALLBACK_SET is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL2 with METADATA_OP_PROF2_FALLBACK_SET is in disabled state. |
| eUICC | The PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_FALLBACK_ALLOWED is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_FALLBACK_ALLOWED is in disabled state. |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| 1 | PROC_ESIPA_GET_EIM_PACKAGE_SET_FALLBACK |  |  |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE1 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 3 | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_SET_FALLBACK |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 4 | PROC_ESIPA_PROVIDE_EIM_PACKAGE_RESULT_SET_FALLBACK |  |  |
| ENDIF |  |  |  |
| 5 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE1 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 6 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_7> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 7 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_7> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE1: This procedure needs to be ran only if the TLS connection is not initialized on ESipa. |  |  |  |

##### 5.4.12.2.2	TC_IPAd_SetFallbackAttribute_Error_Target_Profile_Not_Available

Test Sequence #01 Error: Target_Profile_Not_Available

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is not installed on the eUICC. |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step |  | Direction | Sequence / Description | Expected result |  |
| --- | --- | --- | --- | --- | --- |
| IC1 |  | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |  |
| IC2 |  | IPA is triggered to send ESipa.GetEimPackage method See NOTE1 |  |  |  |
| 1 |  | IPAd → S_eIM | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA ( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |  |
| 2 |  | S_eIM → IPAd | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_SET_FALLBACK_TRIGGER_OK) | No error |  |
| 3 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |  |  |
| 4 |  | IPAd → S_EIM | Send ESipa.HandleNotification method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_SET_FALLBACK_ERR_PROFILE_NOT_AVAILABLE)) |  |
| 5 |  | S_EIM → IPAd | #R_HTTP_204_OK | No error |  |
| ENDIF |  |  |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |  |  |
| 6 |  | IPAd → S_EIM | Send ESipa.ProvideEimPackageResult method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_PROVIDE_EIM_PACKAGE_RESULT, MTD_PROVIDE_EIM_PACKAGE_RESULT (#R_EPR_SET_FALLBACK_ERR_PROFILE_NOT_AVAILABLE)) |  |
| 7 |  | S_EIM → IPAd | MTD_HTTP_RESP_ESIPA (#S_EIM_ACKNOWLEDGEMENT) | No error |  |
| ENDIF |  |  |  |  |  |
| NOTE1: It is IPA dependent, if there is a need for a separate trigger, or the trigger in PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA is triggering the eIM Package retrieval procedure. NOTE2: This procedure needs to be ran only if the TLS connection is not initialized on ESipa. |  |  |  |  |  |

##### 5.4.12.2.3	TC_IPAd_SetFallbackAttribute_Error_Not_Allowed

Test Sequence #01 Error: Fallback_Not_allowed

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 is in Disabled state. |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step |  |  | Direction | Sequence / Description | Expected result |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IC1 |  |  | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |  |  |
| IC2 |  |  | IPA is triggered to send ESipa.GetEimPackage method See NOTE1 |  |  |  |  |
| 1 |  |  | IPAd → S_eIM | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA ( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |  |  |
| 2 |  |  | S_eIM → IPAd | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_SET_FALLBACK_TRIGGER_OK) | No error |  |  |
| 3 |  | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |  |  |  |  |
| 4 |  |  | IPAd → S_EIM | Send ESipa.HandleNotification method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_SET_FALLBACK_ERR_PROFILE_NOT_ALLOWED)) |  |  |
| 5 |  |  | S_EIM → IPAd | #R_HTTP_204_OK | No error |  |  |
| ENDIF |  |  |  |  |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |  |  |  |  |
| 6 |  |  | IPAd → S_EIM | Send ESipa.ProvideEimPackageResult method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_PROVIDE_EIM_PACKAGE_RESULT, MTD_PROVIDE_EIM_PACKAGE_RESULT (#R_EPR_SET_FALLBACK_ERR_PROFILE_NOT_ALLOWED)) |  |  |
| 7 |  |  | S_EIM → IPAd | MTD_HTTP_RESP_ESIPA (#S_EIM_ACKNOWLEDGEMENT) | No error |  |  |
| ENDIF |  |  |  |  |  |  |  |
| 8 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE1 |  |  |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |  |  |
| 9 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_1 > as <PROFILE_INFO> |  |  |  |  |
| ENDIF |  |  |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |  |  |
| 10 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_1 > as <PROFILE_INFO> |  |  |  |  |
| ENDIF |  |  |  |  |  |
| NOTE1: It is IPA dependent, if there is a need for a separate trigger, or the trigger in PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA is triggering the eIM Package retrieval procedure. NOTE2: This procedure needs to be ran only if the TLS connection is not initialized on ESipa. |  |  |  |  |  |  |  |

##### 5.4.12.2.4	TC_IPAd_SetFallbackAttribute_Error_Fallback_Profile_Already_Enabled

Test Sequence #01 Error: Fallback_Profile_Already_Enabled

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL2 with METADATA_OP_PROF2_FALLBACK_SET is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL2 with METADATA_OP_PROF2_FALLBACK_SET is in Enabled state. |
| eUICC | The PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_FALLBACK_ALLOWED is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_FALLBACK_ALLOWED is in disabled state. |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step |  |  | Direction | Sequence / Description | Expected result |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IC1 |  |  | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |  |  |
| IC2 |  |  | IPA is triggered to send ESipa.GetEimPackage method See NOTE1 |  |  |  |  |
| 1 |  |  | IPAd → S_eIM | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA ( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |  |  |
| 2 |  |  | S_eIM → IPAd | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_SET_FALLBACK_TRIGGER_OK) | No error |  |  |
| 3 |  | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE2 |  |  |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |  |  |  |  |
| 4 |  |  | IPAd → S_EIM | Send ESipa.HandleNotification method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_SET_FALLBACK_ERR_FALLBACK_PROFILE_ENABLED)) |  |  |
| 5 |  |  | S_EIM → IPAd | #R_HTTP_204_OK | No error |  |  |
| ENDIF |  |  |  |  |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |  |  |  |  |
| 6 |  |  | IPAd → S_EIM | Send ESipa.ProvideEimPackageResult method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_PROVIDE_EIM_PACKAGE_RESULT, MTD_PROVIDE_EIM_PACKAGE_RESULT (#R_EPR_SET_FALLBACK_ERR_FALLBACK_PROFILE_ENABLED)) |  |  |
| 7 |  |  | S_EIM → IPAd | MTD_HTTP_RESP_ESIPA (#S_EIM_ACKNOWLEDGEMENT) | No error |  |  |
| ENDIF |  |  |  |  |  |  |  |
| 8 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE1 |  |  |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |  |  |
| 9 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_8> as <PROFILE_INFO> |  |  |  |  |
| ENDIF |  |  |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |  |  |
| 10 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_8> as <PROFILE_INFO> |  |  |  |  |
| ENDIF |  |  |  |  |  |
| NOTE1: It is IPA dependent, if there is a need for a separate trigger, or the trigger in PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA is triggering the eIM Package retrieval procedure. NOTE2: This procedure needs to be ran only if the TLS connection is not initialized on ESipa. |  |  |  |  |  |  |  |

### 5.4.13	Local Profile Management – Unset fallback attribute

#### 5.4.13.1	Conformance Requirements

References

- GSMA IoT eSIM Technical Specification [31]

#### 5.4.13.2	Test Cases

##### 5.4.13.2.1	TC_IPAd_UnsetFallbackAttribute

Test Sequence #01 Nominal: Unset_Fallback_Attribute

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_FALLBACK_SET is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_FALLBACK_SET is in Disabled state. |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction | Sequence / Description | Expected result |
| --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |
| 1 | PROC_ESIPA_GET_EIM_PACKAGE_UNSET_FALLBACK |  |  |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE1 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 3 | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_UNSET_FALLBACK |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 4 | PROC_ESIPA_PROVIDE_EIM_PACKAGE_RESULT_UNSET_FALLBACK |  |  |
| ENDIF |  |  |  |
| 5 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE1 |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |
| 6 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_1_FALLBACK_ALLOWED> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |
| 7 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_1_FALLBACK_ALLOWED> as <PROFILE_INFO> |  |  |
| ENDIF |  |  |  |
| NOTE1: This procedure needs to be ran only if the TLS connection is not initialized on ESipa. |  |  |  |

##### 5.4.13.2.2	TC_IPAd_UnsetFallbackAttribute_Error_Fallback_Profile_Enabled

Test Sequence #01 Error: Unset_Fallback_Attribute_Fallback_Profile_Enabled

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_FALLBACK_SET is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_FALLBACK_SET is in Enabled state. |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction |  | Sequence / Description |  |  | Expected result |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |  |  |  |  |  |
| 1 | PROC_ESIPA_GET_EIM_PACKAGE_UNSET_FALLBACK |  |  |  |  |  |  |  |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE1 |  |  |  |  |  |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |  |  |  |  |  |
| 3 | IPAd → S_EIM |  |  | Send ESipa.HandleNotification method with eIM Package Result |  |  | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_UNSET_FALLBACK_ERR_FALLBACK_PROFILE_ENABLED)) |  |
| 4 | S_EIM → IPAd |  |  | #R_HTTP_204_OK |  |  | No error |  |
| ENDIF |  |  |  |  |  |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |  |  |  |  |  |
| 5 |  | IPAd → S_EIM |  |  | Send ESipa.ProvideEimPackageResult method with eIM Package Result |  |  | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_PROVIDE_EIM_PACKAGE_RESULT, MTD_PROVIDE_EIM_PACKAGE_RESULT (#R_EPR_UNSET_FALLBACK_ERR_FALLBACK_PROFILE_ENABLED)) |
| 6 |  | S_EIM → IPAd |  |  | MTD_HTTP_RESP_ESIPA (#S_EIM_ACKNOWLEDGEMENT) |  |  | No error |
| ENDIF |  |  |  |  |  |  |  |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE1 |  |  |  |  |  |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |  |  |  |  |  |
| 8 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_1_FALLBACK_ENABLED> as <PROFILE_INFO> |  |  |  |  |  |  |  |
| ENDIF |  |  |  |  |  |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |  |  |  |  |  |
| 9 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_1_FALLBACK_ENABLED> as <PROFILE_INFO> |  |  |  |  |  |  |  |
| ENDIF |  |  |  |  |  |  |  |  |
| NOTE1: This procedure needs to be ran only if the TLS connection is not initialized on ESipa. |  |  |  |  |  |  |  |  |

##### 5.4.13.2.3	TC_IPAd_UnsetFallbackAttribute_Error_No_Fallback_Profile

Test Sequence #01 Error: Unset_Fallback_Attribute_No_Fallback_Profile

| Initial Conditions |  |
| --- | --- |
| Entity | Description of the initial condition |
| eUICC | The PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_FALLBACK_ALLOWED is installed on the eUICC. |
| eUICC | The PROFILE_OPERATIONAL1 with METADATA_OP_PROF1_FALLBACK_Allowed is in Disabled state. |
| S_eIM | No secure connection is established between S_eIM and IPAd |

| Step | Direction |  | Sequence / Description |  |  | Expected result |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IC1 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |  |  |  |  |  |  |
| 1 | PROC_ESIPA_GET_EIM_PACKAGE_UNSET_FALLBACK |  |  |  |  |  |  |  |
| 2 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE1 |  |  |  |  |  |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |  |  |  |  |  |
| 3 | IPAd → S_EIM |  |  | Send ESipa.HandleNotification method with eIM Package Result |  |  | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_UNSET_FALLBACK_ERR_NO_FALLBACK_PROFILE)) |  |
| 4 | S_EIM → IPAd |  |  | #R_HTTP_204_OK |  |  | No error |  |
| ENDIF |  |  |  |  |  |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |  |  |  |  |  |
| 5 |  | IPAd → S_EIM |  |  | Send ESipa.ProvideEimPackageResult method with eIM Package Result |  |  | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_PROVIDE_EIM_PACKAGE_RESULT, MTD_PROVIDE_EIM_PACKAGE_RESULT (#R_EPR_UNSET_FALLBACK_ERR_NO_FALLBACK_PROFILE)) |
| 6 |  | S_EIM → IPAd |  |  | MTD_HTTP_RESP_ESIPA (#S_EIM_ACKNOWLEDGEMENT) |  |  | No error |
| ENDIF |  |  |  |  |  |  |  |  |
| 7 | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA See NOTE1 |  |  |  |  |  |  |  |
| IF O_D_ESIPA_HANDLE_NOTIF |  |  |  |  |  |  |  |  |
| 8 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF with <PROFILE_INFO_IOT_1_FALLBACK_ALLOWED> as <PROFILE_INFO> |  |  |  |  |  |  |  |
| ENDIF |  |  |  |  |  |  |  |  |
| IF O_D_ESIPA_PROVIDE_EIM_PACKAGE_RESULT |  |  |  |  |  |  |  |  |
| 9 | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT with <PROFILE_INFO_IOT_1_FALLBACK_ALLOWED> as <PROFILE_INFO> |  |  |  |  |  |  |  |
| ENDIF |  |  |  |  |  |  |  |  |
| NOTE1: This procedure needs to be ran only if the TLS connection is not initialized on ESipa. |  |  |  |  |  |  |  |  |

# 6	End-to-End Testing

This section is defined as FFS and not applicable for this version of test specification.

- Annex A	Constants

- A.1	Generic Constants

| Name | Content |  |
| --- | --- | --- |
| ACTIVATION_CODE_1 |  | 1$#TEST_DP_ADDRESS1$#MATCHING_ID_1 |
| ACTIVATION_CODE_3 |  | 1$#TEST_DP_ADDRESS1$#MATCHING_ID_3$$1 |
| ACTIVATION_CODE_3_NO_CC |  | 1$#TEST_DP_ADDRESS1$#MATCHING_ID_3 |
| ACTIVATION_CODE_4 |  | 1$#TEST_DP_ADDRESS1$#MATCHING_ID_4 |
| ACTIVATION_CODE_5 |  | 1$#TEST_DP_ADDRESS1$#MATCHING_ID_EMPTY |
| ACTIVATION_CODE_INVALID_FORMAT | 1#TEST_DP_ADDRESS1$#MATCHING_ID_1 |  |
| CONFIRMATION_CODE1 |  | 0102030405 |
| CTX_PARAMS1_MATCH_ID_DEV_INFO (CtxParams1) |  | ctxParamsForCommonAuthentication : { matchingId <MATCHING_ID>, -- OPTIONAL - see NOTE #DEVICE_INFO } NOTE: the matchingId field may be present (with value <MATCHING_ID>) or may be absent. The presence or absence of matchingId may be checked in individual test cases. |
| DEVICE_INFO |  | deviceInfo { tac ..., deviceCapabilities { ... }, imei ... -- Optional }-- Check only that the field is present and has a valid TLV asn.1 structure NOTE: The content of deviceInfo is verified in individual test cases. |
| EF_UST1 |  | 0x0A 2E 14 8C E7 32 04 00 00 00 00 00 00 -- NOTE: Service n°17 (GID1) and n°18 (GID2) not available |
| EID1 |  | 0x89 04 90 32 12 34 51 23 45 12 34 56 78 90 12 35 |
| EIM_ID |  | Identical with #TEST_EIM_ADDRESS1 |
| HOST_ID |  | 0x47 53 4D 41 20 53 4D 2D 58 58 -- NOTE: 'GSMA SM-XX' in ASCII |
| ICCID_OP_PROF1 |  | 0x98 92 09 01 21 43 65 87 09 F5 |
| ICCID_OP_PROF2 |  | 0x98 92 09 01 32 54 76 98 10 F9 |
| ICCID_OP_PROF3 |  | 0x98 92 09 01 43 65 87 09 21 F5 |
| ICCID_OP_PROF4 |  | 0x98 92 09 01 54 76 98 10 32 F9 |
| ICCID_OP_PROF7 |  | 0x98 92 09 01 87 09 21 43 65 F5 |
| ICCID_OP_PROF8 |  | 0x98 92 09 01 98 10 32 54 76 F9 |
| ICON_OP_PROF1 |  | profile_O1.png as defined in Annex H |
| ICON_OP_PROF2 |  | profile_O2.png as defined in Annex H |
| ICON_OP_PROF7 |  | profile_O7.png as defined in Annex H |
| ICON_OP_PROF8 |  | profile_O8.png as defined in Annex H |
| IMSI_OP_PROF3 |  | 0x08 29 99 28 11 32 54 76 96 |
| IMSI_OP_PROF4 |  | 0x08 29 99 48 43 65 87 09 21 |
| IMSI_OP_PROF7 |  | 0x08 29 99 28 43 65 87 09 21 |
| IMSI_OP_PROF8 |  | 0x08 29 99 28 43 65 87 09 21 |
| KEY_LENGTH |  | 0x10 |
| KEY_TYPE |  | 0x88 |
| MATCHING_ID_1 |  | 04386-AGYFT-A74Y8-3F815 |
| MATCHING_ID_3 |  | 04386-AGYFT-A74Y8-3F817 |
| MATCHING_ID_4 |  | 04386-AGYFT-A74Y8-3F818 |
| MCC_MNC1 |  | 0x92 F9 18 |
| MCC_MNC2 |  | 0x92 F9 28 |
| MCC_MNC4 |  | 0x92 F9 48 |
| NAME_OP_PROF1 |  | Operational Profile Name 1 |
| NAME_OP_PROF2 |  | Operational Profile Name 2 |
| NAME_OP_PROF3 |  | Operational Profile Name 3 |
| NAME_OP_PROF4 |  | Operational Profile Name 4 |
| NAME_OP_PROF7 |  | Operational Profile Name 7 |
| NAME_OP_PROF8 |  | Operational Profile Name 8 |
| PATH_AUTH_CLIENT |  | /gsma/rsp2/es9plus/authenticateClient |
| PATH_GET_BPP |  | /gsma/rsp2/es9plus/getBoundProfilePackage |
| PATH_GET_EIM_PACKAGE |  | /gsma/rsp2/esipa/getEimPackage |
| PATH_HANDLE_NOTIF |  | /gsma/rsp2/es9plus/handleNotification |
| PATH_HANDLE_NOTIF_IPA |  | /gsma/rsp2/esipa/handleNotification |
| PATH_INITIATE_AUTH |  | /gsma/rsp2/es9plus/initiateAuthentication |
| PATH_PROVIDE_EIM_PACKAGE_RESULT |  | /gsma/rsp2/esipa/provideEimPackageResult |
| PPRS_ALLOWED_EUC_NOT_REQ (ProfilePolicyAuthorisationRule) |  | { pprIds { ppr1, ppr2 }, allowedOperators { { mccMnc 'EEEEEE'H, gid1 ''H, gid2 ''H} }, pprFlags ''B } |
| REMOTE_OP_ID_INSTALL |  | 1 |
| SIMA_RESULT_OK |  | simaresp EUICCResponse ::= { peStatus { {status ok} } } |
| SP_NAME1 |  | SP Name 1 |
| SP_NAME2 |  | SP Name 2 |
| SP_NAME3 |  | SP Name 3 |
| SP_NAME4 |  | SP Name 4 |
| SP_NAME7 |  | SP Name 7 |
| SP_NAME8 |  | SP Name 8 |
| S_SM_DP+_OID |  | 2.999.10 |
| S_TLS_CIPHER_SUITE |  | TLS cipher suite selected as follows: o TLS_ECDHE_ECDSA_WITH_AES_128_GCM_SHA256 if present in <TLS_CIPHER_SUITES>, otherwise o TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 |
| TEST_DP_ADDRESS1 |  | testsmdpplus1.example.com |
| TEST_DP_ADDRESS2 |  | testsmdpplus2.example.com |
| TEST_DP_ADDRESS4 |  | testsmdpplus4.example.com |
| TEST_EIM_ADDRESS1 |  | testeim1.example.com |
| TLS_VERSION_1_2 |  | 1.2 The minimum TLS Version supported by the Server. |
| UPP_OP_PROF1 |  | The Unprotected Profile Package related to the PROFILE_OPERATIONAL1 (see Annex E). |
| UPP_OP_PROF3 |  | The Unprotected Profile Package related to the PROFILE_OPERATIONAL3 (see Annex E). |
| UPP_OP_PROF4 |  | The Unprotected Profile Package related to the PROFILE_OPERATIONAL4 (see Annex E). |

- A.2	Test Certificates and Test Keys

All ECC certificates and keys described below are based on either:

- 	NIST P-256 curve, defined in Digital Signature Standard [11]

NOTE: 	SGP.26 [25] contains test keys, valid test certificates and instructions for how to generate invalid certificates. Unless specified differently, the test keys and test certificates used in the present document are bundled with SGP.26 [25].

| Name | Description |  |
| --- | --- | --- |
| CERT_CI_ECDSA | Certificate of the CI for its Public ECDSA Key |  |
| CERT_EUICC_ECDSA |  | Certificate of the eUICC for its Public ECDSA key CERT.EUICC.ECDSA in the X.509 format signed by the EUM with SK.EUM.ECDSA |
| CERT_EUM_ECDSA |  | Certificate of the EUM for its Public ECDSA key CERT.EUM.ECDSA in the X.509 format signed by the requested CI with SK.CI.ECDSA. |
| CERT_S_SERVER_TLS |  | CERT.SERVER.TLS certificate of the S_SERVER, based on NIST or Brainpool for this version of the specification, where the Certificate MAY be one of the following depending on the role of the simulator: - #CERT_S_SM_DP_TLS on ES9+ - #CERT_S_SM_DS_TLS on ES11 or ES12 |
| CERT_S_SM_DPauth_ECDSA |  | Certificate of the S_SM-DP+ for its Public ECDSA key used for SM-DP+ authentication. This certificate contains the OID #S_SM_DP+_OID. |
| CERT_S_SM_DPpb_ECDSA |  | Certificate of the S_SM-DP+ for its Public ECDSA key used for Profile Package Binding. This certificate contains the OID #S_SM_DP+_OID. |
| PK_CI_ECDSA | Public Key of the CI, contained within #CERT_CI_ECDSA |  |
| PK_EUICC_ECDSA | Public Key of the eUICC, contained within #CERT_EUICC_ECDSA |  |
| SK_EUICC_ECDSA |  | Private key of the eUICC for creating signatures |
| SK_S_EIMsign_ECDSA |  | Private key of the eIM for creating signatures |
| SK_S_SM_DPpb_ECDSA |  | Private key of the S_SM-DP+ used to provide signatures for Profile binding |

- Annex B	Dynamic Content

| Variable | Description |
| --- | --- |
| ACTIVATION_CODE | An Activation Code value. |
| ANY_ADD_PP_VERSIONS | Any value of the content of the EUICCInfo2.additionalEuiccProfilePackageVersions field |
| ANY_PROFILE_VERSION | Any value of type VersionType |
| ANY_SVN | Any value of type VersionType |
| CLIENT_TLS_EPHEM_KEY | Client's ephemeral key and associated information. |
| COUNTER_EIM | Integer value coded maximum on two bytes. Incremented each time a test tool generates eUICC Package Request. |
| EIM_SIGNATURE | The eIM signature 1 (eimSignature) computed using #SK_S_EIMsign_ECDSA across the euiccPackageSigned. |
| EUICC_SIGN_EPR_EPR | The eUICC signature of the eUICC Package Result containing Enable Profile Result. The input data used to generate the <EUICC_SIGN_EPR_EPR> is the eUICCPackageResultDataSigned TLV. |
| EIM_TRANSACTION_ID | The TransactionID (Unique Transaction Identifier) generated by the (S_)EIM which is used to uniquely identify the RSP session and to correlate the multiple ESXX request messages that belong to the same RSP session. This value (binary value) can start from 0x01 and can be increased by 1 each time a Profile is downloaded in the eUICC. 1-16 bytes (ASN.1 OCTET STRING). |
| EUICC_CHALLENGE | Random eUICC challenge, coded as asn.1 OCTET STRING, 16 bytes. |
| EUICC_CI_PK_ID_LIST_FOR_SIGNING | List of CI Public Key Identifiers supported on the eUICC for signature creation, coded as ASN.1 sequence of SubjectKeyIdentifier. The CI Public Key Identifier is from the list of possible CI Public Key Identifier. This possible CI Public Key Identifiers as supported by the eUICC will be defined later on. |
| EUICC_CI_PK_ID_LIST_FOR_VERIFICATION | List of CI Public Key Identifiers supported on the eUICC for signature verification, coded as ASN.1 sequence of SubjectKeyIdentifier. The CI Public Key Identifier is from the list of possible CI Public Key Identifier. This possible CI Public Key Identifiers as supported by the eUICC will be defined later on. |
| EUICC_CI_PK_ID_TO_BE_USED | CI Public Key Identifier to be used by the eUICC for signature, coded as ASN.1 sequence of SubjectKeyIdentifier. |
| EUICC_RSP_CAPABILITY | RspCapability of the eUICC, coded as ASN.1 BIT STRING |
| EUICC_SIGNATURE1 | The eUICC signature 1 (euiccSignature1) computed using #SK_EUICC_ECDSA across the euiccSigned1 present in the AuthenticateServerResponse structure, coded as ASN.1 OCTET STRING. |
| EUICC_SIGNATURE2 | The eUICC signature 2 (euiccSignature2) computed using the #SK_EUICC_ECDSA across the following data objects: - euiccSigned2 - smdpSignature2 present in the PrepareDownloadRequest structure |
| EUICC_SIGN_EPR_CIER | The eUICC signature of the eUICC Package Result containing Configure Immediate Enable Result. The input data used to generate the <EUICC_SIGN_EPR_CIER > is the eUICCPackageResultDataSigned TLV. |
| EUICC_SIGN_EPR_DPR | The eUICC signature of the eUICC Package Result containing Disable Profile Result. The input data used to generate the <EUICC_SIGN_EPR_DPR> is the eUICCPackageResultDataSigned TLV. |
| EUICC_SIGN_EPR_DELPR | The eUICC signature of the eUICC Package Result containing Delete Profile Result. The input data used to generate the <EUICC_SIGN_EPR_DELPR> is the eUICCPackageResultDataSigned TLV. |
| EUICC_SIGN_EPR_LPIR | The eUICC signature of the eUICC Package Result containing List Profile Info Result. The input data used to generate the <EUICC_SIGN_EPR_LPIR > is the eUICCPackageResultDataSigned TLV. |
| EUICC_SIGN_EPR_SET_FALLBACK | The eUICC signature of the eUICC Package Result containing Set Fallback Attribute Result. The input data used to generate the <EUICC_SIGN_EPR_SET_FALLBACK> is the eUICCPackageResultDataSigned TLV. |
| EUICC_SIGN_EPR_UNSET_FALLBACK | The eUICC signature of the eUICC Package Result containing Unset Fallback Attribute Result. The input data used to generate the <EUICC_SIGN_EPR_UNSET_FALLBACK> is the eUICCPackageResultDataSigned TLV. |
| EUICC_SIGN_PIR | The eUICC signature of the Profile Installation Result (PIR). The input data used to generate the <EUICC_SIGN_PIR> is the profileInstallationResultData TLV. |
| EXT_CARD_RESOURCE | Extended Card Resource Information according to ETSI TS 102 226 [14], coded as ASN.1 OCTET STRING. 'Number of installed application' value field is '00'. |
| EXT_SHA256_ECDSA | TLS extension data for "supported_signature_algorithms" set as a minimum of HashAlgorithm sha256 (04) and SignatureAlgorithm ecdsa (03). |
| IOT_VERSION | The value of the iotVersion field in EUICCInfo2. |
| IPA_MODE | The value of the ipaMode field in EUICCInfo2. |
| ISD_P_AID | The ISD-P AID newly created in the eUICC. This AID value is in the range from 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 00 10 00 to 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 FF FF 00. Last byte is set to '00' as defined in SGP.02[1]. |
| ISD_P_AID1 | The ISD-P AID created in the eUICC for the PROFILE_OPERATIONAL1. This AID value belongs to the range from 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 00 10 00 to 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 FF FF 00. Last byte is set to '00' as defined in SGP.02[1]. |
| ISD_P_AID2 | The ISD-P AID created in the eUICC for the PROFILE_OPERATIONAL2. This AID value belongs to the range from 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 00 10 00 to 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 FF FF 00. Last byte is set to '00' as defined in SGP.02[1]. |
| ISD_P_AID3 | The ISD-P AID created in the eUICC for the PROFILE_OPERATIONAL3. This AID value belongs to the range from 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 00 10 00 to 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 FF FF 00. Last byte is set to '00' as defined in SGP.02[1]. |
| ISD_P_AID4 | The ISD-P AID created in the eUICC for the PROFILE_OPERATIONAL4. This AID value belongs to the range from 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 00 10 00 to 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 FF FF 00. Last byte is set to '00' as defined in SGP.02[1]. |
| ISD_P_AID7 | The ISD-P AID created in the eUICC for the PROFILE_OPERATIONAL7. This AID value belongs to the range from 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 00 10 00 to 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 FF FF 00. Last byte is set to '00' as defined in SGP.02[1]. |
| ISD_P_AID8 | The ISD-P AID created in the eUICC for the PROFILE_OPERATIONAL7. This AID value belongs to the range from 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 00 10 00 to 0xA0 00 00 05 59 10 10 FF FF FF FF 89 00 FF FF 00. Last byte is set to '00' as defined in SGP.02[1]. |
| L | Exact length of the corresponding tag or of the remaining data. |
| MATCHING_ID | Unique identifier as defined in [2]. The content can be either empty, or the value of the EventID, or the value of the Activation Code token. |
| METADATA_OP_PROF1_SEG | The #METADATA_OP_PROF1 is mac-ed with <S_MAC> and split as necessary into segments of a maximum size of 1020 bytes (including the tag, length field, and MAC), |
| OTPK_EUICC_ECKA | One-time Public Key generated by the eUICC for ECKA. Depending on the eUICC configuration, this key is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| OTPK_S_SM_DP+_ECKA | One-time Public Key generated by the S_SM-DP+ for ECKA. Depending on the eUICC configuration, this key is based on NIST P-256, brainpoolP256r1 or FRP256V1. |
| PPP_OP_PROF1_SEG_SK | An element of sequenceOf86, consisting of a <UPP_OP_PROF1_SEG> segment protected with <S_ENC> and <S_MAC> and encapsulated in a TLV with tag 0x86, length <L>, up to a maximum size of 1020 bytes including the tag and length field. |
| PPR_IDS | Forbidden Profile Policy Rules. This PPR list MAY be empty or MAY contain either PPR1 or PPR2 or both. |
| PROFILE_INFO | ProfileInfo structure(s) |
| PROFILE_INFO_IOT_1 (ProfileInfo) | { iccid #ICCID_OP_PROF1, isdpAid <ISD_P_AID1>, profileState disabled, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileClass operational } |
| PROFILE_INFO_IOT_1_FALLBACK (ProfileInfo) | { iccid #ICCID_OP_PROF1, isdpAid <ISD_P_AID1>, profileState disabled, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileClass operational, fallbackAttribute TRUE, fallbackAllowed TRUE } |
| PROFILE_INFO_IOT_1_FALLBACK_ALLOWED (ProfileInfo) | { iccid #ICCID_OP_PROF1, isdpAid <ISD_P_AID1>, profileState disabled, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileClass operational, fallbackAttribute FALSE, fallbackAllowed TRUE } |
| PROFILE_INFO_IOT_1_FALLBACK_ENABLED (ProfileInfo) | { iccid #ICCID_OP_PROF1, isdpAid <ISD_P_AID1>, profileState enabled, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileClass operational, fallbackAttribute TRUE, fallbackAllowed TRUE } |
| PROFILE_INFO_IOT_2 (ProfileInfo) | { iccid #ICCID_OP_PROF1, isdpAid <ISD_P_AID1>, profileState enabled, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileClass operational } |
| PROFILE_INFO_IOT_3 (ProfileInfo) | { iccid #ICCID_OP_PROF3, isdpAid <ISD_P_AID3>, profileState disabled, serviceProviderName #SP_NAME3, profileName #NAME_OP_PROF3, profileClass operational } |
| PROFILE_INFO_IOT_4 (ProfileInfo) | { iccid #ICCID_OP_PROF4, isdpAid <ISD_P_AID4>, profileState disabled, serviceProviderName #SP_NAME4, profileName #NAME_OP_PROF4, profileClass operational } |
| PROFILE_INFO_IOT_4_EN (ProfileInfo) | { iccid #ICCID_OP_PROF4, isdpAid <ISD_P_AID4>, profileState enabled, serviceProviderName #SP_NAME4, profileName #NAME_OP_PROF4, profileClass operational } |  |
| PROFILE_INFO_IOT_7 (ProfileInfo) | { iccid #ICCID_OP_PROF2, isdpAid <ISD_P_AID2>, profileState enabled, serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, profileClass operational, fallbackAttribute FALSE, fallbackAllowed TRUE }, { iccid #ICCID_OP_PROF1, isdpAid <ISD_P_AID1>, profileState disabled, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileClass operational, fallbackAttribute TRUE, fallbackAllowed TRUE } |  |
| PROFILE_INFO_IOT_7_DIS (ProfileInfo) | { iccid #ICCID_OP_PROF7, isdpAid <ISD_P_AID7>, profileState enabled, serviceProviderName #SP_NAME7, profileName #NAME_OP_PROF7, profileClass operational } |  |
| PROFILE_INFO_IOT_8 (ProfileInfo) | { iccid #ICCID_OP_PROF2, isdpAid <ISD_P_AID2>, profileState enabled, serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2, profileClass operational, fallbackAttribute TRUE, fallbackAllowed TRUE }, { iccid #ICCID_OP_PROF1, isdpAid <ISD_P_AID1>, profileState disabled, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileClass operational, fallbackAttribute FALSE, fallbackAllowed TRUE } |  |
| PROFILE_INFO_IOT_8_DIS (ProfileInfo) | { iccid #ICCID_OP_PROF8, isdpAid <ISD_P_AID8>, profileState enabled, serviceProviderName #SP_NAME8, profileName #NAME_OP_PROF8, profileClass operational } |  |
| R_EUICC_INFO1 | The eUICC information EUICCInfo1 coded as an ASN.1 SEQUENCE, as defined in SGP.22 v3.1[2]. |  |
| R_EUICC_INFO2 | The eUICC information EUICCInfo2 coded as an ASN.1 SEQUENCE, as defined in SGP.22 v3.1[2]. |  |
| SEQ_NUMBER | Sequence Number related to a Notification Metadata generated by the eUICC. |  |
| S_ENC | SCP03T Encryption Session key (128 bits length) resulting from the key agreement with eUICC. |  |
| S_HASHED_CC | Hashed Confirmation Code generated by the IPA. When generated by the S_IPAd, the S_IPAd SHALL use #CONFIRMATION_CODE1 in the calculation unless otherwise specified. |  |
| S_MAC | SCP03T MACing Session key (128 bits length) resulting from the key agreement with eUICC. |  |
| S_SESSION_ID_SERVER | Random value of the TLS session_id in ServerHello which is different from <SESSION_ID_CLIENT>. This value is non-empty. |  |
| S_SMDP_CHALLENGE | The SM-DP+ Challenge (serverChallenge) randomly chosen by the simulated SM-DP+ to be signed later by the eUICC for the eUICC authentication, coded as ASN.1 OCTET STRING of 16 bytes. |  |
| S_SMDP_SIGNATURE1 | The ASN.1 OCTET STRING encoded SM-DP+ signature (field serverSignature1) computed using the private key related to the server certificate (field serverCertificate) present in the AuthenticateServerRequest structure. |  |
| S_SMDP_SIGNED1 (ServerSigned1) | { transactionId <S_TRANSACTION_ID>, euiccChallenge <EUICC_CHALLENGE>, serverAddress #TEST_DP_ADDRESS1, serverChallenge <S_SMDP_CHALLENGE> } |  |
| S_SM_DP+_SIGN | The S_SM-DP+ signature (smdpSign), computed using the #SK_S_SM_DPpb_ECDSA across the following data objects: - remoteOpId - transactionId - controlRefTemplate - smdpOtpk - euiccOtpk, as provided earlier in the prepareDownloadResponse data object |  |
| S_SM_DP+_SIGNATURE2 | The ASN.1 OCTET STRING encoded SM-DP+ signature 2 (field smdpSignature2) computed using the private key related to the server certificate (field smdpCertificate) present in the PrepareDownloadRequest structure. This signature SHALL be generated across the following data objects: - smdpSignature2 - euiccSignature1 present in the AuthenticateServerResponse structure |  |
| S_TRANSACTION_ID | The TransactionID (Unique Transaction Identifier) generated by the (S_)SM-DP+, or (S_)SM-DS , or (S_)EIM which is used to uniquely identify the RSP session and to correlate the multiple ESXX request messages that belong to the same RSP session. This value (binary value) can start from 0x01 and can be increased by 1 each time a Profile is downloaded in the eUICC. 1-16 bytes (ASN.1 OCTET STRING). |  |
| SEQ_NUMBER | Sequence Number related to a Notification Metadata generated by the eUICC. |  |
| SESSION_ID_CLIENT | Random or empty value of the TLS session_id in ClientHello. |  |
| TBS_EUICC_NOTIF_SIG | The eUICC signature generated over tbsOtherNotification. NotificationMetadata, coded as ASN.1 OCTET STRING. |  |
| TLS_CIPHER_SUITES | TLS cipher suite list supported by IPAd or the Client (SM-DP+ or SM-DS) under test. |  |
| TRE_PROPERTIES | The value of the treProperties field in EUICCInfo2. |  |
| TRE_REFERENCE | The value of the treProductReference field in EUICCInfo2. |  |
| UPP_OP_PROF1_SEG | A segment of the #UPP_OP_PROF1, with a maximum size of 1007 bytes. |  |

- Annex C	Methods And Procedures

This section describes methods and procedures used in the interfaces compliance test cases. They are part of test cases and SHALL not be executed in standalone mode.

- C.1	Methods

If the method is used in the “expected result” column, all parameters SHALL be verified by the simulated entity (test tool). If the method is used in the “Sequence / Description” column, the command SHALL be generated by the simulated entity.

| Method | MTD_AUTHENTICATE_CLIENT |
| --- | --- |
| Description | Generates or verifies the JSON formatted AuthenticateClient request |
| Parameter(s) | - paramTransactionId: random 16 byte identifier encoded as String Hexadecimal. - paramAuthenticateServerResponse: server authentication response structured as ASN.1 encoded as base 64. |
| Details | JSON body { "transactionId" : paramTransactionId, "authenticateServerResponse" : paramAuthenticateServerResponse } |

| Method | MTD_GET_BPP |
| --- | --- |
| Description | Generates or verifies the JSON formatted GetBoundProfilePackage request |
| Parameter(s) |  paramTransactionId: random 16 byte identifier.  paramPrepareDownloadResponse structured as ASN.1 encoded as base 64. |
| Details | JSON body { "transactionId" : paramTransactionId, "prepareDownloadResponse" : paramPrepareDownloadResponse } |

| Method | MTD_GET_EIM_PACKAGE |
| --- | --- |
| Description | Generates or verifies the JSON formatted GetEimPackage request |
| Parameter(s) | - paramEidValue: EID as described in SGP.22 |
| Details | JSON body { "eidValue" : paramEidValue, } |

| Method | MTD_HANDLE_NOTIF |
| --- | --- |
| Description | Generates or verifies the JSON formatted HandleNotification request |
| Parameter(s) | paramPendingNotification: PendingNotification data object |
| Details | JSON body { "pendingNotification" : paramPendingNotification } |

| Method | MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT |
| --- | --- |
| Description | Generates or verifies the JSON formatted HandleNotification request with provideEimPackageResult |
| Parameter(s) | paramProvideEimPackageResult: ProvideEimPackageResult data object |
| Details | JSON body { "provideEimPackageResult" : paramProvideEimPackageResult } |

| Method | MTD_HTTP_REQ |
| --- | --- |
| Description | Sends or verifies a secured HTTP request message delivering a JSON object payload using a network to an off-card entity. |
| Parameter(s) | - paramServerAddress: Target Server address - paramFunctionPath: Function path - paramRequestMessage: JSON Request message |
| Details | HTTP POST paramFunctionPath HTTP/1.1 Host: paramServerAddress User-Agent: See NOTE 1 X-Admin-Protocol:gsma/rsp/v#RSP_SVN Content-Type:application/json OR application/json;charset=UTF-8 (see NOTE 2) Content-Length: <L> paramRequestMessage NOTE 1: If the request is sent by the IPAd, the User-Agent SHALL be gsma-rsp-ipad. The "User-Agent" field may contain additional information after a semicolon. Otherwise the value of User-Agent is not specified by the current document. The additional information shall not be checked. NOTE 2: the Content-Type checking is relaxed in this specification, in order to allow for common internet usage of “charset=UTF-8” and for compatibility with SGP.22 v3.0. If the request is sent by the entity under test, both values are acceptable (where linear white space as specified in RFC 2616 is allowed after the semi-colon). Further, all parts of these allowed Content-Type value SHALL be checked in a case-insensitive manner, as per RFC 2616. If the request is sent by a simulator, application/json shall be used.The HTTP POST request may contain additional header fields. These shall not be checked. |

| Method | MTD_HTTP_REQ_ESIPA |
| --- | --- |
| Description | Sends or verifies a secured HTTP request message delivering a JSON object payload using a network to eIM. |
| Parameter(s) | - paramServerAddress: Target Server address - paramFunctionPath: Function path - paramRequestMessage: JSON Request message |
| Details | HTTP POST paramFunctionPath HTTP/1.1 Host: paramServerAddress User-Agent: See NOTE 1 X-Admin-Protocol:gsma/rsp/v2.1.0 Content-Type: application/json;charset=UTF-8 Content-Length: <L> paramRequestMessage NOTE 1: The value of User-Agent is not specified by [31]. It shall not be checked. |

| Method | MTD_HTTP_RESP |
| --- | --- |
| Description | Sends or verifies a secured HTTP response message delivering a JSON object payload using a network to an off-card entity. |
| Parameter(s) |  paramResponseMessage: JSON Response message |
| Details | HTTP/1.1 200 (OK) X-Admin-Protocol: gsma/rsp/v#RSP_SVN Content-Type: application/json OR application/json;charset=UTF-8 (see NOTE) Content-Length: <L> paramResponseMessage NOTE: the Content-Type checking is relaxed in this specification, in order to allow for common internet usage of “charset=UTF-8” and for compatibility with SGP.22 v3.0 If the response is sent by the entity under test, both values are acceptable (where linear white space as specified in RFC 2616 is allowed after the semi-colon). Further, all parts of these allowed Content-Type value SHALL be checked in a case-insensitive manner, as per RFC 2616. If the response is sent by a simulator, application/json shall be used. The HTTP response may contain additional header fields. These shall not be checked. |

| Method | MTD_HTTP_RESP_ESIPA |
| --- | --- |
| Description | Sends or verifies a secured HTTP response message delivering a JSON object payload using a network to an off-card entity. |
| Parameter(s) |  paramResponseMessage: JSON Response message |
| Details | HTTP/1.1 200 (OK) X-Admin-Protocol: gsma/rsp/v2.1.0 Content-Type: application/json;charset=UTF-8 Content-Length: <L> paramResponseMessage The HTTP response may contain additional header fields. These shall not be checked. |

| Method | MTD_INITIATE_AUTHENTICATION |
| --- | --- |
| Description | Generates or verifies the JSON formatted Initiate Authentication request on ES9+ or ES11 as applicable. |
| Parameter(s) | - paramEUICCChallenge: random 16 byte challenge coded as base 64 - paramEUICCInfo1: eUICC information structured coded as base 64 - paramServerAddress: FQDN of the Server. |
| Details | JSON body { "euiccChallenge" : paramEUICCChallenge, "euiccInfo1" : paramEUICCInfo1, "smdpAddress" : paramServerAddress } |

| Method | MTD_PROVIDE_EIM_PACKAGE_RESULT |
| --- | --- |
| Description | Generates or verifies the JSON formatted ProvideEimPackageResult request with eimPackageResult |
| Parameter(s) | - paramEIDValue: EID value of the targeted eUICC - paramEimPackageResult: eimPackageResult data object |
| Details | JSON body { "eidValue" : paramEIDValue, "eEimPackageResult" : paramEimPackageResult } |

| Method | MTD_TLS_CLIENT_KEY_EXCH_ETC |
| --- | --- |
| Description | Finalizes the Transport Layer Security (TLS) handshake in Server authentication mode on ES9+, or ES11 (Client side). |
| Parameter(s) | -  paramClientKeyExchange: ClientKeyExchange message |
| Details | Sends the session key information in TLS ClientKeyExchange message, ChangeCipherSpec and Finished message. |

| Method | MTD_TLS_CLIENT_HELLO |
| --- | --- |
| Description | Sends or checks the Client Hello message used to initiate the Transport Layer Security (TLS) handshake in Server authentication or Mutual authentication mode on ES9+, ES11, ES12 or ES15. |
| Parameter(s) | -  paramTLSversion: TLS protocol version -  paramAlgs: cipher suite types supported -  paramSessionID: Session ID -  paramExts: Extensions data for “supported_signature_algorithms”, “trusted_ca_keys” or other (optional) |
| Details | Sends or receives a TLS ClientHello message according to the parameters defined above. In addition the following parameters will be set: -  The list of compression algorithms supported by the client is not explicitly defined, but by default it will be set to NULL. -  The random of 4 bytes representing time since epoch on client host and 28 random bytes is not explicitly defined but it SHALL be generated by the test tool TLS implementation NOTE: The Supported Elliptic Curves Extension and the Supported Point Formats Extension extensions MAY be sent by the Client. |

| Method | MTD_TLS_SERVER_HELLO_ETC |
| --- | --- |
| Description | Send or Receives to the Client Hello in the Transport Layer Security (TLS) handshake in Server authentication mode on ES9+, or ES11. |
| Parameter(s) | -  paramTLSversion: TLS protocol version -  paramAlgs: cipher suite selected -  paramSessionID: Session ID -  paramCertificate: TLS server certificate for authentication -  paramServerTLSEphemeralKey: TLS Server ephemeral key. |
| Details | Sends or Receives a TLS ServerHello, Server Certificate, ServerKeyExchange and ServerHelloDone message in this order according to the parameters defined above. NOTE 1: The random of 4 bytes representing time since epoch on client host and 28 random bytes is not explicitly defined in the Server Hello message but it SHALL be generated by the Server under test. NOTE 2: If no parameter mentioned paramServerTLSEphemeralKey, the value SHALL be set as defined in [24] for ServerKeyExchange. No verification required. |

- C.2	Procedures

|  | Procedure | PROC_ES9+_AUTH_CLIENT |  |
| --- | --- | --- | --- |
|  | Description | Authenticate Server procedure without Confirmation Code. #R_AUTH_SERVER_MATCH_ID_DEV_INFO is used with the correct MatchingID contained in the profile download trigger (Activation Code content or Empty MatchingID). |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | IPAd → S_SM-DP+ | Send ES9+.AuthenticateClient method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DP+ → IPAd | MTD_HTTP_RESP (#AUTH_CLIENT_OK) | No error |

|  | Procedure | PROC_ES9+_AUTH_CLIENT_CC |  |  |
| --- | --- | --- | --- | --- |
|  | Description | Authenticate Server procedure (via Activation Code) with Confirmation Code. #R_AUTH_SERVER_MATCH_ID_DEV_INFO and #AUTH_SERVER_RESP_ACT_CODE_UC_OK are used with the correct MatchingID defined by the profile download trigger (Activation Code content or Empty MatchingID). |  |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | IPAd → S_SM-DP+ | Send ES9+.AuthenticateClient method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_AUTH_CLIENT, MTD_AUTHENTICATE_CLIENT(<S_TRANSACTION_ID>, #R_AUTH_SERVER_MATCH_ID_DEV_INFO)) |
| 2 | S_SM-DP+ → IPAd | MTD_HTTP_RESP (#AUTH_CLIENT_OK_CC) | No error |

|  | Procedure | PROC_ES9+_GET_BPP |  |
| --- | --- | --- | --- |
|  | Description | Get BPP procedure without Confirmation Code. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | IPAd → S_SM-DP+ | Send ES9+.GetBoundProfilePackage method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_GET_BPP, MTD_GET_BPP(<S_TRANSACTION_ID>, #R_PREP_DOWNLOAD_NO_CC)) |
| 2 | S_SM-DP+ → IPAd | MTD_HTTP_RESP(#GET_BPP_OK) | No error |

|  |  | Procedure | PROC_ES9+_GET_BPP_CC |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Description | Get BPP procedure with Confirmation Code. |  |  |
| Step |  | Direction | Sequence / Description | Expected result |  |
| 1 | IPAd → S_SM-DP+ | Send ES9+.GetBoundProfilePackage method |  | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_GET_BPP, MTD_GET_BPP(<S_TRANSACTION_ID>, #R_PREP_DOWNLOAD_WITH_CC)) |
| 2 | S_SM-DP+ → IPAd | MTD_HTTP_RESP(#GET_BPP_OK) |  | No error |

|  | Procedure | PROC_ES9+_HANDLE_NOTIF |  |
| --- | --- | --- | --- |
|  | Description | Handle Notification procedure. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | IPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF(#R_PIR_OK)) See NOTE 2 |
| 2 | S_SM-DP+ → IPAd | #R_HTTP_204_OK | No error |
| NOTE 1: Other Notifications MAY be sent within the same HTTPS session. NOTE 2: The values of notificationAddress, iccid and smdpOid used in #R_PIR_OK MAY vary depending on the context (ICCID of the downloaded profile, used SM-DP+ address and certificate). |  |  |  |

|  | Procedure | PROC_ES9+_HANDLE_NOTIF_DEL1 |  |
| --- | --- | --- | --- |
|  | Description | Handle Notification procedure. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF(#PENDING_NOTIF_DEL1)) |
| 2 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |
| NOTE 1: Other Notifications MAY be sent within the same HTTPS session. |  |  |  |

|  | Procedure | PROC_ES9+_HANDLE_NOTIF_DIS1 |  |
| --- | --- | --- | --- |
|  | Description | Handle Notification procedure. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF(#PENDING_NOTIF_DIS1)) |
| 2 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |
| NOTE 1: Other Notifications MAY be sent within the same HTTPS session. |  |  |  |

|  | Procedure | PROC_ES9+_INIT_AUTH |  |
| --- | --- | --- | --- |
|  | Description | Initiate Authentication procedure. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | IPAd → S_SM-DP+ | Send ES9+.InitiateAuthentication method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_INITIATE_AUTH, MTD_INITIATE_AUTHENTICATION( <EUICC_CHALLENGE>, <R_EUICC_INFO1>, #TEST_DP_ADDRESS1)) |
| 2 | S_SM-DP+ → IPAd | MTD_HTTP_RESP( #INITIATE_AUTH_OK) | No error |

|  | Procedure | PROC_TLS_INITIALIZATION_SERVER_AUTH |  |  |
| --- | --- | --- | --- | --- |
|  | Description | Establishes the Transport Layer Security (TLS) v1.2 connection between the Client IPAd and (S_)SERVER using Server authentication mode on ES9+ or ES11. |  |  |
| Step | Direction | Sequence / Description | Expected result | REQ |
| 1 | IPAd → S_SERVER | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_TLS_VERSION, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) |  |
| 2 | S_SERVER → IPAd | MTD_TLS_SERVER_HELLO_ETC(#TLS_VERSION_1_2, #S_TLS_CIPHER_SUITE, <S_SESSION_ID_SERVER>, #CERT_S_SERVER_TLS) | MTD_TLS_CLIENT_KEY_EXCH_ETC(<CLIENT_TLS_EPHEM_KEY>) |  |
| 3 | S_SERVER → IPAd | Finalize TLS Handshake (send Server ChangeCipherSpec and Finished messages) | HTTPS connection established |  |

|  | Procedure | PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA |  |
| --- | --- | --- | --- |
|  | Description | Establishes the Transport Layer Security (TLS) v1.2 connection between the Client IPAd and Server (S_EIM) using Server authentication mode on ES ipa with Variant O certificate. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | IPAd is triggered to establish a secure TLS connection with eIM if the TLS connection is not established yet See NOTE |  |  |
| 2 | IPAd → S_EIM | Send TLS Client Hello | MTD_TLS_CLIENT_HELLO( #IUT_TLS_VERSION, <TLS_CIPHER_SUITES>, <SESSION_ID_CLIENT>, <EXT_SHA256_ECDSA>) |
| 3 | S_EIM → IPAd | MTD_TLS_SERVER_HELLO_ETC(#TLS_VERSION_1_2, #S_TLS_CIPHER_SUITE, <S_SESSION_ID_SERVER>, #CERT_S_SERVER_TLS, NO_PARAM) | MTD_TLS_CLIENT_KEY_EXCH_ETC(<CLIENT_TLS_EPHEM_KEY>) |
| 4 | S_EIM → IPAd | Finalize TLS Handshake (send Server ChangeCipherSpec and Finished messages) | HTTPS connection established |
| NOTE: the method to trigger the IPAd is IPAd dependent. |  |  |  |

|  | Procedure | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_CIER |  |
| --- | --- | --- | --- |
|  | Description | Handle Notification procedure between IPAd and S_EIM for eIM Package Result containing Configure Immediate Enable Result. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | IPAd → S_EIM+ | Send ESipa.HandleNotification method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_CIER_OK)) |
| 2 | S_EIM → IPAd | #R_HTTP_204_OK | No error |

|  | Procedure | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_DPR |  |
| --- | --- | --- | --- |
|  | Description | Handle Notification procedure between IPA and S_EIM for eIM Package Result containing Disable Profile Result. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | IPAd → S_EIM | Send ESipa.HandleNotification method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_DPR_OK)) |
| 2 | S_EIM → IPAd | #R_HTTP_204_OK | No error |

|  | Procedure | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_DELPR |  |
| --- | --- | --- | --- |
|  | Description | Handle Notification procedure between IPA and S_EIM for eIM Package Result containing Delete Profile Result. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | IPAd → S_EIM | Send ESipa.HandleNotification method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_DELPR_OK)) |
| 2 | S_EIM → IPAd | #R_HTTP_204_OK | No error |

|  | Procedure | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_PDTR |  |
| --- | --- | --- | --- |
|  | Description | Handle Notification procedure between IPAd and S_EIM for eIM Package Result containing Profile Download Trigger Result. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | IPAd → S_EIM+ | Send ESipa.HandleNotification method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_PDTR_OK)) See NOTE |
| 2 | S_EIM → IPAd | #R_HTTP_204_OK | No error |
| NOTE: The values of notificationAddress, iccid and smdpOid used in #R_EPR_PDTR_OK MAY vary depending on the context (ICCID of the downloaded profile, used SM-DP+ address and certificate). |  |  |  |

|  | Procedure | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_SET_FALLBACK |  |
| --- | --- | --- | --- |
|  | Description | Handle Notification procedure between IPA and S_EIM for eIM Package Result containing Set Fallback Attribute Result. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | IPAd → S_EIM | Send ESipa.HandleNotification method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_SET_FALLBACK_OK)) |
| 2 | S_EIM → IPAd | #R_HTTP_204_OK | No error |

|  | Procedure | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_UNSET_FALLBACK |  |
| --- | --- | --- | --- |
|  | Description | Handle Notification procedure between IPA and S_EIM for eIM Package Result containing Unset Fallback Attribute Result. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | IPAd → S_EIM | Send ESipa.HandleNotification method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_UNSET_FALLBACK_OK)) |
| 2 | S_EIM → IPAd | #R_HTTP_204_OK | No error |

|  |  |  | Procedure | PROC_ESIPA_GET_EIM_PACKAGE_CONFIGURE_IMMEDIATE_ENABLE |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Description | Retrieval of the configureImmediateEnable PSMO on ESipa interface using eIM Package retrieval. |  |  |  |
| Step |  |  | Direction | Sequence / Description |  | Expected result |  |
| 1 | IPAd is triggered to send ESipa.GetEimPackage method See NOTE |  |  |  |  |  |
| 2 | IPAd → S_eIM |  |  | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA ( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |
| 3 | S_eIM → IPAd |  |  | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_EPR_CONF_IMMEDIATE_ENABLE_OK) | No error |
| NOTE: It is IPAd dependent, if there is a need for a separate trigger, or the trigger in PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA is triggering the eIM Package retrieval procedure. |  |  |  |  |  |

|  |  |  | Procedure | PROC_ESIPA_GET_EIM_PACKAGE_DELETE_PROFILE |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Description | Delete Profile Trigger on ESipa interface using eIM Package retrieval. |  |  |  |
| Step |  |  | Direction | Sequence / Description |  | Expected result |  |
| 1 | IPA is triggered to send ESipa.GetEimPackage method See NOTE |  |  |  |  |  |
| 2 | IPAd → S_eIM |  |  | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA ( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |
| 3 | S_eIM → IPAd |  |  | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_DELETE_PROFILE_TRIGGER_OK) | No error |
| NOTE: It is IPA dependent, if there is a need for a separate trigger, or the trigger in PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA is triggering the eIM Package retrieval procedure. |  |  |  |  |  |

|  |  |  | Procedure | PROC_ESIPA_GET_EIM_PACKAGE_DISABLE_PROFILE |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Description | Disable Profile Trigger on ESipa interface using eIM Package Retrieval. |  |  |  |
| Step |  |  | Direction | Sequence / Description |  | Expected result |  |
| 1 | IPA is triggered to send ESipa.GetEimPackage method See NOTE |  |  |  |  |  |
| 2 | IPAd → S_eIM |  |  | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA ( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |
| 3 | S_eIM → IPAd |  |  | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_DISABLE_PROFILE_TRIGGER_OK) | No error |
| NOTE: It is IPA dependent, if there is a need for a separate trigger, or the trigger in PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA is triggering the eIM Package Retrieval procedure. |  |  |  |  |  |

|  |  |  | Procedure | PROC_ESIPA_GET_EIM_PACKAGE_PROFILE_DOWNLOAD_AC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Description | Profile Download Trigger with Activation Code on ESipa interface using eIM Package retrieval. |  |  |  |
| Step |  |  | Direction | Sequence / Description |  | Expected result |  |
| 1 | IPAd is triggered to send ESipa.GetEimPackage method See NOTE |  |  |  |  |  |
| 2 | IPAd → S_eIM |  |  | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA ( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |
| 3 | S_eIM → IPAd |  |  | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_PROFILE_DOWNLOAD_TRIGGER_AC_OK) | No error |
| NOTE: It is IPAd dependent, if there is a need for a separate trigger, or the trigger in PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA is triggering the eIM Package retrieval procedure. |  |  |  |  |  |

|  |  |  | Procedure | PROC_ESIPA_GET_EIM_PACKAGE_PROFILE_DOWNLOAD_DEFAULT_SM-DP+ |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Description | Profile Download Trigger with default SM-DP+ on ESipa interface using eIM Package retrieval. |  |  |  |
| Step |  |  | Direction | Sequence / Description |  | Expected result |  |
| 1 | IPAd is triggered to send ESipa.GetEimPackage method See NOTE |  |  |  |  |  |
| 2 | IPAd → S_eIM |  |  | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA ( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |
| 3 | S_eIM → IPAd |  |  | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_PROFILE_DOWNLOAD_TRIGGER_DEFAULT_SM-DP+_OK) | No error |
| NOTE: It is IPAd dependent, if there is a need for a separate trigger, or the trigger in PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA is triggering the eIM Package retrieval procedure. |  |  |  |  |  |

|  |  |  | Procedure | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_HANDLE_NOTIF |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Description | List Profile Info on ESipa interface using eIM Package retrieval. It includes handle notification procedure between IPAd and S_EIM for eIM Package Result containing List Profile Info result. |  |  |  |
| Step |  |  | Direction | Sequence / Description |  | Expected result |  |
| 1 | IPAd is triggered to send ESipa.GetEimPackage method See NOTE |  |  |  |  |  |
| 2 | IPAd → S_eIM |  |  | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |  |
| 3 | S_eIM → IPAd |  |  | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_EPR_LIST_PROFILE_INFO_OK) | No error |  |
| 4 |  |  | IPAd → S_EIM+ | Send ESipa.HandleNotification method with eIM Package Result |  | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_EPR_LPIR_OK)) |  |
| 5 |  |  | S_EIM → IPAd | #R_HTTP_204_OK |  | No error |  |
| NOTE: It is IPAd dependent, if there is a need for a separate trigger, or the trigger in PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA is triggering the eIM Package retrieval procedure. |  |  |  |  |  |

|  |  |  | Procedure | PROC_ESIPA_GET_EIM_PACKAGE_LIST_PROFILE_EIM_PACKAGE_RESULT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Description | List Profile Info on ESipa interface using eIM Package retrieval. It includes provide eIM package result procedure between IPAd and S_EIM for eIM Package Result containing List Profile Info result. |  |  |  |
| Step |  |  | Direction | Sequence / Description |  | Expected result |  |
| 1 | IPAd is triggered to send ESipa.GetEimPackage method See NOTE |  |  |  |  |  |
| 2 | IPAd → S_eIM |  |  | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |  |
| 3 | S_eIM → IPAd |  |  | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_EPR_LIST_PROFILE_INFO_OK) | No error |  |
| 4 |  |  | IPAd → S_EIM+ | Send ESipa.ProvideEimPackageResult method with List Profile Info Result |  | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_PROVIDE_EIM_PACKAGE_RESULT, MTD_PROVIDE_EIM_PACKAGE_RESULT (#R_EPR_EPR_LPIR_OK)) |  |
| 5 |  |  | S_EIM → IPAd | MTD_HTTP_RESP_ESIPA (#S_EIM_ACKNOWLEDGEMENT) |  | No error |  |
| NOTE: It is IPAd dependent, if there is a need for a separate trigger, or the trigger in PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA is triggering the eIM Package retrieval procedure. |  |  |  |  |  |

|  |  |  | Procedure | PROC_ESIPA_GET_EIM_PACKAGE_SET_FALLBACK |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Description | Set Fallback attribute trigger on ESipa interface using eIM Package retrieval. |  |  |  |
| Step |  |  | Direction | Sequence / Description |  | Expected result |  |
| 1 | IPA is triggered to send ESipa.GetEimPackage method See NOTE |  |  |  |  |  |
| 2 | IPAd → S_eIM |  |  | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA ( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |
| 3 | S_eIM → IPAd |  |  | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_SET_FALLBACK_TRIGGER_OK) | No error |
| NOTE: It is IPA dependent, if there is a need for a separate trigger, or the trigger in PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA is triggering the eIM Package retrieval procedure. |  |  |  |  |  |

|  |  |  | Procedure | PROC_ESIPA_GET_EIM_PACKAGE_UNSET_FALLBACK |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Description | Unset Fallback attribute trigger on ESipa interface using eIM Package retrieval. |  |  |  |
| Step |  |  | Direction | Sequence / Description |  | Expected result |  |
| 1 | IPA is triggered to send ESipa.GetEimPackage method See NOTE |  |  |  |  |  |
| 2 | IPAd → S_eIM |  |  | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA ( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |
| 3 | S_eIM → IPAd |  |  | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_UNSET_FALLBACK_TRIGGER_OK) | No error |
| NOTE: It is IPA dependent, if there is a need for a separate trigger, or the trigger in PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA is triggering the eIM Package retrieval procedure. |  |  |  |  |  |

|  | Procedure | PROC_ESIPA_PROVIDE_EIM_PACKAGE_RESULT_CIER |  |
| --- | --- | --- | --- |
|  | Description | Provide Eim Package Result procedure between IPA and S_EIM with eUICC Package Result containing Configure Immediate Enable Result. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | IPAd → S_EIM+ | Send ESipa.ProvideEimPackageResult method with Configure Immediate Enable Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_PROVIDE_EIM_PACKAGE_RESULT, MTD_PROVIDE_EIM_PACKAGE_RESULT (#R_EPR_CIER_OK)) |
| 2 | S_EIM → IPAd | MTD_HTTP_RESP_ESIPA (#S_EIM_ACKNOWLEDGEMENT) | No error |

|  | Procedure | PROC_ESIPA_PROVIDE_EIM_PACKAGE_RESULT_DPR |  |
| --- | --- | --- | --- |
|  | Description | Provide eIM Package Result procedure between IPA and S_EIM with eUICC Package Result containing Disable Profile Result. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | IPAd → S_EIM | Send ESipa.ProvideEimPackageResult method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_PROVIDE_EIM_PACKAGE_RESULT, MTD_PROVIDE_EIM_PACKAGE_RESULT (#R_EPR_DPR_OK)) |
| 2 | S_EIM → IPAd | MTD_HTTP_RESP_ESIPA (#S_EIM_ACKNOWLEDGEMENT) | No error |

|  | Procedure | PROC_ESIPA_PROVIDE_EIM_PACKAGE_RESULT_DELPR |  |
| --- | --- | --- | --- |
|  | Description | Provide Eim Package Result procedure between IPA and S_EIM with eUICC Package Result containing Delete Profile Result. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | IPAd → S_EIM | Send ESipa.ProvideEimPackageResult method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_PROVIDE_EIM_PACKAGE_RESULT, MTD_PROVIDE_EIM_PACKAGE_RESULT (#R_EPR_DELPR_OK)) |
| 2 | S_EIM → IPAd | MTD_HTTP_RESP_ESIPA (#S_EIM_ACKNOWLEDGEMENT) | No error |

|  | Procedure | PROC_ESIPA_PROVIDE_EIM_PACKAGE_RESULT_SET_FALLBACK |  |
| --- | --- | --- | --- |
|  | Description | Provide Eim Package Result procedure between IPA and S_EIM with eUICC Package Result containing Set Fallback Attribute Result. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | IPAd → S_EIM | Send ESipa.ProvideEimPackageResult method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_PROVIDE_EIM_PACKAGE_RESULT, MTD_PROVIDE_EIM_PACKAGE_RESULT (#R_EPR_SET_FALLBACK_OK)) |
| 2 | S_EIM → IPAd | MTD_HTTP_RESP_ESIPA (#S_EIM_ACKNOWLEDGEMENT) | No error |

|  | Procedure | PROC_ESIPA_PROVIDE_EIM_PACKAGE_RESULT_UNSET_FALLBACK |  |
| --- | --- | --- | --- |
|  | Description | Provide Eim Package Result procedure between IPA and S_EIM with eUICC Package Result containing Unset Fallback Attribute Result. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | IPAd → S_EIM | Send ESipa.ProvideEimPackageResult method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_PROVIDE_EIM_PACKAGE_RESULT, MTD_PROVIDE_EIM_PACKAGE_RESULT (#R_EPR_UNSET_FALLBACK_OK)) |
| 2 | S_EIM → IPAd | MTD_HTTP_RESP_ESIPA (#S_EIM_ACKNOWLEDGEMENT) | No error |

|  | Procedure | PROC_ES9+_HANDLE_NOTIF_EN1 |  |
| --- | --- | --- | --- |
|  | Description | Handle Notification procedure. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF(#PENDING_NOTIF_EN1)) |
| 2 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |
| NOTE 1: Other Notifications MAY be sent within the same HTTPS session. |  |  |  |

|  | Procedure | PROC_ES9+_HANDLE_NOTIF_DIS2 |  |
| --- | --- | --- | --- |
|  | Description | Handle Notification procedure. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | LPAd → S_SM-DP+ | Send ES9+.HandleNotification method | MTD_HTTP_REQ( #TEST_DP_ADDRESS1, #PATH_HANDLE_NOTIF, MTD_HANDLE_NOTIF(#PENDING_NOTIF_DIS2)) |
| 2 | S_SM-DP+ → LPAd | #R_HTTP_204_OK | No error |
| NOTE 1: Other Notifications MAY be sent within the same HTTPS session. |  |  |  |

|  | Procedure | PROC_ESIPA_HANDLE_NOTIF_EIM_PACKAGE_RESULT_EPR |  |
| --- | --- | --- | --- |
|  | Description | Handle Notification procedure between IPA and S_EIM for eIM Package Result containing Enable Profile Result. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | IPAd → S_EIM | Send ESipa.HandleNotification method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_HANDLE_NOTIF_IPA, MTD_HANDLE_NOTIF_EIM_PACKAGE_RESULT (#R_EPR_EPR_OK)) |
| 2 | S_EIM → IPAd | #R_HTTP_204_OK | No error |

|  | Procedure | PROC_ESIPA_PROVIDE_EIM_PACKAGE_RESULT_EPR |  |
| --- | --- | --- | --- |
|  | Description | Provide Eim Package Result procedure between IPA and S_EIM with eUICC Package Result containing Enable Profile Result. |  |
| Step | Direction | Sequence / Description | Expected result |
| 1 | IPAd → S_EIM | Send ESipa.ProvideEimPackageResult method with eIM Package Result | MTD_HTTP_REQ_ESIPA( #TEST_EIM_ADDRESS1, #PATH_PROVIDE_EIM_PACKAGE_RESULT, MTD_PROVIDE_EIM_PACKAGE_RESULT (#R_EPR_EPR_OK)) |
| 2 | S_EIM → IPAd | MTD_HTTP_RESP_ESIPA (#S_EIM_ACKNOWLEDGEMENT) | No error |

|  |  |  | Procedure | PROC_ESIPA_GET_EIM_PACKAGE_ENABLE_PROFILE |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Description | Enable Profile Trigger on ESipa interface using eIM Package retrieval. |  |  |  |
| Step |  |  | Direction | Sequence / Description |  | Expected result |  |
| 1 | IPA is triggered to send ESipa.GetEimPackage method See NOTE |  |  |  |  |  |
| 2 | IPAd → S_eIM |  |  | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA ( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |
| 3 | S_eIM → IPAd |  |  | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_ENABLE_PROFILE_NO_RB_TRIGGER_OK) | No error |
| NOTE: It is IPA dependent, if there is a need for a separate trigger, or the trigger in PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA is triggering the eIM Package retrieval procedure. |  |  |  |  |  |

|  |  |  | Procedure | PROC_ESIPA_GET_EIM_PACKAGE_ENABLE_PROFILE_RB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Description | Enable Profile Trigger with Rollback on ESipa interface using eIM Package retrieval. |  |  |  |
| Step |  |  | Direction | Sequence / Description |  | Expected result |  |
| 1 | IPA is triggered to send ESipa.GetEimPackage method See NOTE |  |  |  |  |  |
| 2 | IPAd → S_eIM |  |  | Send ESipa.GetEimPackage method | MTD_HTTP_REQ_ESIPA ( #TEST_EIM_ADDRESS1, #PATH_GET_EIM_PACKAGE, MTD_GET_EIM_PACKAGE ( #EID1)) |
| 3 | S_eIM → IPAd |  |  | MTD_HTTP_RESP_ESIPA( #GET_EIM_PACKAGE_ENABLE_PROFILE_RB_TRIGGER_OK) | No error |
| NOTE: It is IPA dependent, if there is a need for a separate trigger, or the trigger in PROC_TLS_INITIALIZATION_SERVER_AUTH_ESIPA is triggering the eIM Package retrieval procedure. |  |  |  |  |  |

- Annex D	Commands And Responses

- D.1	ES8+ Requests And Responses

- D.1.1 ES8+ Requests

| Name | Content |
| --- | --- |
| CONF_ISDP_PROF1 | req ConfigureISDPRequest ::={ dpProprietaryData { dpOid #S_SM_DP+_OID } } |
| METADATA_OP_PROF1 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS1 } }, profileOwner { mccMnc #MCC_MNC1 } } |
| METADATA_OP_PROF1_FALLBACK_ALLOWED | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS1 } }, profileOwner { mccMnc #MCC_MNC1 }, fallbackAllowed TRUE } |
| METADATA_OP_PROF1_FALLBACK_SET | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF1, serviceProviderName #SP_NAME1, profileName #NAME_OP_PROF1, iconType png, icon #ICON_OP_PROF1, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS1 } }, profileOwner { mccMnc #MCC_MNC1 }, fallbackAttribute TRUE, fallbackAllowed TRUE } |
| METADATA_OP_PROF2_FALLBACK_SET | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF2, serviceProviderName #SP_NAME2, profileName #NAME_OP_PROF2 iconType png, icon #ICON_OP_PROF2, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS2 } }, profileOwner { mccMnc #MCC_MNC2 }, fallbackAttribute TRUE, fallbackAllowed TRUE } |
| METADATA_OP_PROF3 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF3, serviceProviderName #SP_NAME3, profileName #NAME_OP_PROF3, profileClass operational, profileOwner { mccMnc #MCC_MNC2 }, profilePolicyRules { ppr2 } } |
| METADATA_OP_PROF4 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF4, serviceProviderName #SP_NAME4, profileName #NAME_OP_PROF4, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS4 } }, profileOwner { mccMnc #MCC_MNC4 }, profilePolicyRules { ppr1 } } |
| METADATA_OP_PROF7 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF7, serviceProviderName #SP_NAME7, profileName #NAME_OP_PROF7, iconType png, icon #ICON_OP_PROF7, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS8 } }, profileOwner { mccMnc #MCC_MNC2 }, profilePolicyRules { ppr2 } } |
| METADATA_OP_PROF8 | metadataReq StoreMetadataRequest ::= { iccid #ICCID_OP_PROF8, serviceProviderName #SP_NAME8, profileName #NAME_OP_PROF8, iconType png, icon #ICON_OP_PROF8, profileClass operational, notificationConfigurationInfo { { profileManagementOperation { notificationInstall, notificationEnable, notificationDisable, notificationDelete }, notificationAddress #TEST_DP_ADDRESS8 } }, profileOwner { mccMnc #MCC_MNC2 }, profilePolicyRules { ppr2 } } |
| S_INIT_SC_PROF1 | req InitialiseSecureChannelRequest ::={ remoteOpId #REMOTE_OP_ID_INSTALL, transactionId <S_TRANSACTION_ID>, controlRefTemplate { keyType #KEY_TYPE, keyLen #KEY_LENGTH, hostId #HOST_ID }, smdpOtpk <OTPK_S_SM_DP+_ECKA>, smdpSign <S_SM_DP+_SIGN> } |

- D.1.2 ES8+ Responses

| Name | Content |  |
| --- | --- | --- |

- D.2	ES9+ Requests And Responses

- D.2.1	ES9+ Requests

| Name | Content |  |
| --- | --- | --- |
| INITIATE_AUTH_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "serverSigned1" : <S_SMDP_SIGNED1>, "serverSignature1" : <S_SMDP_SIGNATURE1>, "euiccCiPKIdTobeUsed" : <EUICC_CI_PK_ID_TO_BE_USED>, "serverCertificate" : #CERT_S_SM_DPauth_ECDSA } -- NOTE: select the CI as defined in the note in the chapter 2.1.4 of SGP.23 |
| MATCHING_ID_EMPTY |  |
| PENDING_NOTIF_EN1 | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationEnable }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |  |
| S_SMDP_SIGNED2 | req SmdpSigned2 ::= { transactionId <S_TRANSACTION_ID>, ccRequiredFlag FALSE } |
| S_SMDP_SIGNED2_CC | req SmdpSigned2 ::= { transactionId <S_TRANSACTION_ID>, ccRequiredFlag TRUE } |

- D.2.2	ES9+ Responses

| Name |  | Content |  |  |  |
| --- | --- | --- | --- | --- | --- |
| AUTH_CLIENT_OK |  |  | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "profileMetadata" : #METADATA_OP_PROF1, "smdpSigned2" : #S_SMDP_SIGNED2, "smdpSignature2" : <S_SM_DP+_SIGNATURE2>, "smdpCertificate" : #CERT_S_SM_DPpb_ECDSA } |  |
| AUTH_CLIENT_OK_CC |  |  | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "profileMetadata" : #METADATA_OP_PROF1, "smdpSigned2" : #S_SMDP_SIGNED2_CC, "smdpSignature2" : <S_SM_DP+_SIGNATURE2>, "smdpCertificate" : #CERT_S_SM_DPpb_ECDSA } |  |
| GET_BPP_OK |  |  | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "transactionId" : <S_TRANSACTION_ID>, "boundProfilePackage" : BoundProfilePackage { #S_INIT_SC_PROF1, firstSequenceOf87 { #CONF_ISDP_PROF1 }, sequenceOf88 { <METADATA_OP_PROF1_SEG> … <METADATA_OP_PROF1_SEG> }, sequenceOf86 { <PPP_OP_PROF1_SEG_SK> … <PPP_OP_PROF1_SEG_SK> } } } NOTE 1: boundProfilePackage is enconded as base64 therefore the test tool SHALL decode boundProfilePackage to access the ASN.1. NOTE 2: For sequenceOf88 there will be only one or two '88' TLV segments depending on the size of StoreMetadata. |  |
| PENDING_NOTIF_EN1 |  |  | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationEnable }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |  |
| PENDING_NOTIF_DEL1 |  |  | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationDelete }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |  |
| PENDING_NOTIF_DIS1 |  |  | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationDisable }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |  |
| PENDING_NOTIF_DIS2 |  |  | response PendingNotification ::= otherSignedNotification : { tbsOtherNotification { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationDisable }, notificationAddress #TEST_DP_ADDRESS2, iccid #ICCID_OP_PROF2 }, euiccNotificationSignature <TBS_EUICC_NOTIF_SIG>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |  |
| R_AUTH_SERVER_MATCH_ID_DEV_INFO |  |  | resp AuthenticateServerResponse ::= authenticateResponseOk { euiccSigned1 { transactionId <S_TRANSACTION_ID>, serverAddress #TEST_DP_ADDRESS1, serverChallenge <S_SMDP_CHALLENGE>, euiccInfo2 <R_EUICC_INFO2>, -- check only that the field is present but not the values ctxParams1 #CTX_PARAMS1_MATCH_ID_DEV_INFO }, euiccSignature1 <EUICC_SIGNATURE1>, euiccCertificate #CERT_EUICC_ECDSA, eumCertificate #CERT_EUM_ECDSA } |  |
| R_HTTP_204_OK |  | HTTP/1.1 204 No Content X-Admin-Protocol: gsma/rsp/v<2.1.0> NOTE: If the HTTP response is being received from the server under test, then the "Content-type" header MAY be present. |  |
| R_PIR_OK |  | response ProfileInstallationResult ::= { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation { notificationInstall }, notificationAddress #TEST_DP_ADDRESS1, iccid #ICCID_OP_PROF1 }, smdpOid #S_SM_DP+_OID, finalResult successResult : { aid <ISD_P_AID>, ppiResponse #SIMA_RESULT_OK } }, euiccSign <EUICC_SIGN_PIR> } |  |

- D.3 ES10x Requests And Responses

- D.3.1	ES10x Requests

There are no ES10x requests for this version of the specification.

- D.3.2	ES10x Responses

| Name |  | Content |  |
| --- | --- | --- | --- |
| R_PREP_DOWNLOAD_NO_CC |  | resp PrepareDownloadResponse ::= downloadResponseOk : { euiccSigned2 { transactionId <S_TRANSACTION_ID>, euiccOtpk <OTPK_EUICC_ECKA> }, euiccSignature2 <EUICC_SIGNATURE2> } |  |
| R_PREP_DOWNLOAD_WITH_CC | resp PrepareDownloadResponse ::= downloadResponseOk : { euiccSigned2 { transactionId <S_TRANSACTION_ID>, euiccOtpk <OTPK_EUICC_ECKA>, hashCc <S_HASHED_CC> }, euiccSignature2 <EUICC_SIGNATURE2> } |  |

- D.4	ESipa Requests And Responses

- D.4.1	ESipa Requests

| Name | Content |
| --- | --- |
| DELETE_PROFILE_TRIGGER | value1 EuiccPackageRequest ::= { euiccPackageSigned { eimId #EIM_ID1, eidValue #EID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, euiccPackage psmoList : { delete{ iccid #ICCID_OP_PROF1 } } }, eimSignature <EIM_SIGNATURE> } |
| DELETE_PROFILE_4_TRIGGER | value1 EuiccPackageRequest ::= { euiccPackageSigned { eimId #EIM_ID1, eidValue #EID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, euiccPackage psmoList : { delete { iccid #ICCID_OP_PROF4 } } }, eimSignature <EIM_SIGNATURE> } |
| DELETE_PROFILE_7_TRIGGER | value1 EuiccPackageRequest ::= { euiccPackageSigned { eimId #EIM_ID1, eidValue #EID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, euiccPackage psmoList : { delete { iccid #ICCID_OP_PROF7 } } }, eimSignature <EIM_SIGNATURE> } |
| DELETE_PROFILE_8_TRIGGER | value1 EuiccPackageRequest ::= { euiccPackageSigned { eimId #EIM_ID1, eidValue #EID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, euiccPackage psmoList : { delete { iccid #ICCID_OP_PROF8 } } }, eimSignature <EIM_SIGNATURE> } |
| DISABLE_PROFILE_TRIGGER | value1 EuiccPackageRequest ::= { euiccPackageSigned { eimId #EIM_ID1, eidValue #EID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, euiccPackage psmoList : { disable { iccid #ICCID_OP_PROF1 } } }, eimSignature <EIM_SIGNATURE> } |
| DISABLE_PROFILE_4_TRIGGER | value1 EuiccPackageRequest ::= { euiccPackageSigned { eimId #EIM_ID1, eidValue #EID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, euiccPackage psmoList : { disable { iccid #ICCID_OP_PROF4 } } }, eimSignature <EIM_SIGNATURE> } |
| ENABLE_PROFILE_TRIGGER_NO_RB | value1 EuiccPackageRequest ::= { euiccPackageSigned { eimId #EIM_ID1, eidValue #EID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, euiccPackage psmoList : { enable { iccid #ICCID_OP_PROF1 } } }, eimSignature <EIM_SIGNATURE> } |
| ENABLE_PROFILE_TRIGGER_RB | value1 EuiccPackageRequest ::= { euiccPackageSigned { eimId #EIM_ID1, eidValue #EID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, euiccPackage psmoList : { enable { iccid #ICCID_OP_PROF1, rollbackFlag } } }, eimSignature <EIM_SIGNATURE> } |
| PROFILE_DOWNLOAD_TRIGGER_AC | response ProfileDownloadTriggerRequest ::= { profileDownloadData activationCode : # <ACTIVATION_CODE>, eimTransactionId <EIM_TRANSACTION_ID> } |
| PROFILE_DOWNLOAD_TRIGGER_DEFAULT_SM-DP+ | response ProfileDownloadTriggerRequest ::= { profileDownloadData contactDefaultSmdp : NULL, eimTransactionId <EIM_TRANSACTION_ID> } |
| SET_FALLBACK_TRIGGER | value1 EuiccPackageRequest ::= { euiccPackageSigned { eimId #EIM_ID1, eidValue #EID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, euiccPackage psmoList : { setFallbackAttribute { iccid #ICCID_OP_PROF1 } } }, eimSignature <EIM_SIGNATURE> } |
| UNSET_FALLBACK_TRIGGER | value1 EuiccPackageRequest ::= { euiccPackageSigned { eimId #EIM_ID1, eidValue #EID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, euiccPackage psmoList : { unsetFallbackAttribute { iccid #ICCID_OP_PROF1 } } }, eimSignature <EIM_SIGNATURE> } |

- D.4.2	ESipa Responses

| Name | Content |
| --- | --- |
| EUICC_PACKAGE_REQUEST_CONF_IMMEDIATE_ENABLE | value1 EuiccPackageRequest ::= { euiccPackageSigned { eimId #EIM_ID1, eidValue #EID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, euiccPackage psmoList : { configureImmediateEnable : { immediateEnableFlag NULL } } }, eimSignature <EIM_SIGNATURE> } |
| EUICC_PACKAGE_REQUEST_LIST_PROFILE_INFO | value1 EuiccPackageRequest ::= { euiccPackageSigned { eimId #EIM_ID1, eidValue #EID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, euiccPackage psmoList : { listProfileInfo : { } } }, eimSignature <EIM_SIGNATURE> } |
| GET_EIM_PACKAGE_DELETE_PROFILE_TRIGGER_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "euiccPackageRequest": #DELETE_PROFILE_TRIGGER } |
| GET_EIM_PACKAGE_DELETE_PROFILE_4_TRIGGER_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "euiccPackageRequest": #DELETE_PROFILE_4_TRIGGER } |
| GET_EIM_PACKAGE_DELETE_PROFILE_7_TRIGGER_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "euiccPackageRequest": #DELETE_PROFILE_7_TRIGGER } |
| GET_EIM_PACKAGE_DELETE_PROFILE_8_TRIGGER_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "euiccPackageRequest": #DELETE_PROFILE_8_TRIGGER } |
| GET_EIM_PACKAGE_DISABLE_PROFILE_TRIGGER_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "euiccPackageRequest": #DISABLE_PROFILE_TRIGGER } |
| GET_EIM_PACKAGE_DISABLE_PROFILE_4_TRIGGER_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "euiccPackageRequest": #DISABLE_PROFILE_4_TRIGGER } |
| GET_EIM_PACKAGE_ENABLE_PROFILE_NO_RB_TRIGGER_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "euiccPackageRequest": #ENABLE_PROFILE_TRIGGER_NO_RB } |
| GET_EIM_PACKAGE_ENABLE_PROFILE_RB_TRIGGER_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "euiccPackageRequest": #ENABLE_PROFILE_TRIGGER_RB } |
| GET_EIM_PACKAGE_EPR_CONF_IMMEDIATE_ENABLE_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "euiccPackageRequest": #EUICC_PACKAGE_REQUEST_CONF_IMMEDIATE_ENABLE, } |
| GET_EIM_PACKAGE_EPR_LIST_PROFILE_INFO_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "euiccPackageRequest": #EUICC_PACKAGE_REQUEST_LIST_PROFILE_INFO, } |
| GET_EIM_PACKAGE_PROFILE_DOWNLOAD_TRIGGER_AC_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "profileDownloadTriggerRequest": #PROFILE_DOWNLOAD_TRIGGER_AC } |
| GET_EIM_PACKAGE_PROFILE_DOWNLOAD_TRIGGER_DEFAULT_SM-DP+_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "profileDownloadTriggerRequest": #PROFILE_DOWNLOAD_TRIGGER_DEFAULT_SM-DP+ } |
| GET_EIM_PACKAGE_SET_FALLBACK_TRIGGER_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "euiccPackageRequest": #SET_FALLBACK_TRIGGER } |
| GET_EIM_PACKAGE_UNSET_FALLBACK_TRIGGER_OK | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "euiccPackageRequest": #UNSET_FALLBACK_TRIGGER } |
| R_EPR_DELPR_OK | Response ProvideEimPackageResult ::= { eidValue #EID1, eimPackageResult euiccPackageResult : euiccPackageResultSigned : { euiccPackageResultDataSigned { eimId #EIM_ID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, seqNumber <SEQ_NUMBER>, euiccResult { DeleteProfileResult : ok } }, euiccSignEPR <EUICC_SIGN_EPR_DELPR> } } |
| R_EPR_DPR_OK | Response ProvideEimPackageResult ::= { eidValue #EID1, eimPackageResult euiccPackageResult : euiccPackageResultSigned : { euiccPackageResultDataSigned { eimId #EIM_ID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, seqNumber <SEQ_NUMBER>, euiccResult { DisableProfileResult : ok } }, euiccSignEPR <EUICC_SIGN_EPR_DPR> } } |
| R_EPR_DELPR_ERR_PPR | Response ProvideEimPackageResult ::= { eidValue #EID1, eimPackageResult euiccPackageResult : euiccPackageResultSigned : { euiccPackageResultDataSigned { eimId #EIM_ID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, seqNumber <SEQ_NUMBER>, euiccResult { DisableProfileResult : disallowedByPolicy} }, euiccSignEPR <EUICC_SIGN_EPR_DELPR> } } |
| R_EPR_DPR_ERR_NOT_EN | Response ProvideEimPackageResult ::= { eidValue #EID1, eimPackageResult euiccPackageResult : euiccPackageResultSigned : { euiccPackageResultDataSigned { eimId #EIM_ID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, seqNumber <SEQ_NUMBER>, euiccResult { DisableProfileResult : profileNotInEnabledState } }, euiccSignEPR <EUICC_SIGN_EPR_DPR> } } |
| R_EPR_DPR_ERR_PPR | Response ProvideEimPackageResult ::= { eidValue #EID1, eimPackageResult euiccPackageResult : euiccPackageResultSigned : { euiccPackageResultDataSigned { eimId #EIM_ID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, seqNumber <SEQ_NUMBER>, euiccResult { DisableProfileResult : disallowedByPolicy} }, euiccSignEPR <EUICC_SIGN_EPR_DPR> } } |
| R_EPR_EPR_OK | Response ProvideEimPackageResult ::= { eidValue #EID1, eimPackageResult euiccPackageResult : euiccPackageResultSigned : { euiccPackageResultDataSigned { eimId #EIM_ID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, seqNumber <SEQ_NUMBER>, euiccResult { EnableProfileResult : ok } }, euiccSignEPR <EUICC_SIGN_EPR_EPR> } } |
| R_EPR_EPR_ERR_NOT_DIS | Response ProvideEimPackageResult ::= { eidValue #EID1, eimPackageResult euiccPackageResult : euiccPackageResultSigned : { euiccPackageResultDataSigned { eimId #EIM_ID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, seqNumber <SEQ_NUMBER>, euiccResult { EnableProfileResult : profileNotInDisabledState } }, euiccSignEPR <EUICC_SIGN_EPR_EPR> } } |
| R_EPR_EPR_ERR_PPR | Response ProvideEimPackageResult ::= { eidValue #EID1, eimPackageResult euiccPackageResult : euiccPackageResultSigned : { euiccPackageResultDataSigned { eimId #EIM_ID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, seqNumber <SEQ_NUMBER>, euiccResult { EnableProfileResult : disallowedByPolicy } }, euiccSignEPR <EUICC_SIGN_EPR_EPR> } } |
| R_EPR_EPR_ERR_UNKNOWN | Response ProvideEimPackageResult ::= { eidValue #EID1, eimPackageResult euiccPackageResult : euiccPackageResultSigned : { euiccPackageResultDataSigned { eimId #EIM_ID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, seqNumber <SEQ_NUMBER>, euiccResult { EnableProfileResult : undefinedError } }, euiccSignEPR <EUICC_SIGN_EPR_EPR> } } |
| R_EPR_PDTR_OK | response ProvideEimPackageResult ::= { eidValue #EID1, eimPackageResult profileDownloadTriggerResult : { eimTransactionId <EIM_TRANSACTION_ID>, profileDownloadTriggerResultData profileInstallationResult : { profileInstallationResultData { transactionId <S_TRANSACTION_ID>, notificationMetadata { seqNumber <SEQ_NUMBER>, profileManagementOperation notificationInstall, notificationAddress #TEST_EIM_ADDRESS1, iccid #ICCID_OP_PROF1 }, smdpOid #S_SM_DP+_OID, finalResult successResult : { aid <ISD_P_AID>, simaResponse #SIMA_RESULT_OK } }, euiccSignPIR <EUICC_SIGN_PIR> } } } |
| R_EPR_EPR_CIER_OK | value1 ProvideEimPackageResult ::= { eidValue #EID1, eimPackageResult euiccPackageResult : euiccPackageResultSigned : { euiccPackageResultDataSigned { eimId #EIM_ID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, seqNumber <SEQ_NUMBER>, euiccResult { configureImmediateEnableResult : ok } }, euiccSignEPR <EUICC_SIGN_EPR_CIER> } } |
| R_EPR_EPRRE_INVALID_PACKAGE_FORMAT | value1 ProvideEimPackageResult ::= { eidValue #EID1, eimPackageResult eimPackageResultResponseError : { eimPackageResultErrorCode invalidPackageFormat } } |
| R_EPR_EPR_LPIR_OK | value1 ProvideEimPackageResult ::= { eidValue #EID1, eimPackageResult euiccPackageResult : euiccPackageResultSigned : { euiccPackageResultDataSigned { eimId #EIM_ID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, seqNumber <SEQ_NUMBER>, euiccResult { listProfileInfoResult : profileInfoListOk : { { <PROFILE_INFO> } } } }, euiccSignEPR <EUICC_SIGN_EPR_LPIR> } } |
| R_EPR_SET_FALLBACK_OK | Response ProvideEimPackageResult ::= { eidValue #EID1, eimPackageResult euiccPackageResult : euiccPackageResultSigned : { euiccPackageResultDataSigned { eimId #EIM_ID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, seqNumber <SEQ_NUMBER>, euiccResult { SetFallbackAttributeResult: ok } }, euiccSignEPR <EUICC_SIGN_EPR_SET_FALLBACK> } } |
| R_EPR_SET_FALLBACK_ERR_PROFILE_NOT_AVAILABLE | Response ProvideEimPackageResult ::= { eidValue #EID1, eimPackageResult euiccPackageResult : euiccPackageResultSigned : { euiccPackageResultDataSigned { eimId #EIM_ID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, seqNumber <SEQ_NUMBER>, euiccResult { setFallbackAttributeResult: iccidOrAidNotFound } }, euiccSignEPR <EUICC_SIGN_EPR_SET_FALLBACK> } } |
| R_EPR_SET_FALLBACK_ERR_FALLBACK_PROFILE_ENABLED | Response ProvideEimPackageResult ::= { eidValue #EID1, eimPackageResult euiccPackageResult : euiccPackageResultSigned : { euiccPackageResultDataSigned { eimId #EIM_ID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, seqNumber <SEQ_NUMBER>, euiccResult { setFallbackAttributeResult: fallbackProfileEnabled } }, euiccSignEPR <EUICC_SIGN_EPR_SET_FALLBACK> } } |
| R_EPR_SET_FALLBACK_ERR_PROFILE_NOT_ALLOWED | Response ProvideEimPackageResult ::= { eidValue #EID1, eimPackageResult euiccPackageResult : euiccPackageResultSigned : { euiccPackageResultDataSigned { eimId #EIM_ID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, seqNumber <SEQ_NUMBER>, euiccResult { setFallbackAttributeResult: fallbackNotAllowed } }, euiccSignEPR <EUICC_SIGN_EPR_SET_FALLBACK> } } |
| R_EPR_UNSET_FALLBACK_OK | Response ProvideEimPackageResult ::= { eidValue #EID1, eimPackageResult euiccPackageResult : euiccPackageResultSigned : { euiccPackageResultDataSigned { eimId #EIM_ID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, seqNumber <SEQ_NUMBER>, euiccResult { UnsetFallbackAttributeResult: ok } }, euiccSignEPR <EUICC_SIGN_EPR_UNSET_FALLBACK> } } |
| R_EPR_UNSET_FALLBACK_ERR_FALLBACK_PROFILE_ENABLED | Response ProvideEimPackageResult ::= { eidValue #EID1, eimPackageResult euiccPackageResult : euiccPackageResultSigned : { euiccPackageResultDataSigned { eimId #EIM_ID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, seqNumber <SEQ_NUMBER>, euiccResult { unsetFallbackAttributeResult: fallbackProfileEnabled } }, euiccSignEPR <EUICC_SIGN_EPR_UNSET_FALLBACK> } } |
| R_EPR_UNSET_FALLBACK_ERR_NO_FALLBACK_PROFILE | Response ProvideEimPackageResult ::= { eidValue #EID1, eimPackageResult euiccPackageResult : euiccPackageResultSigned : { euiccPackageResultDataSigned { eimId #EIM_ID1, counterValue <COUNTER_EIM>, eimTransactionId <EIM_TRANSACTION_ID>, seqNumber <SEQ_NUMBER>, euiccResult { unsetFallbackAttributeResult: noFallbackAttribute } }, euiccSignEPR <EUICC_SIGN_EPR_UNSET_FALLBACK> } } |
| S_EIM_ACKNOWLEDGEMENT | { "header" : { "functionExecutionStatus" : { "status" : "Executed-Success" } }, "eimAcknowledgements": #EIM_ACKNOWLEDGEMENTS, } |
| EIM_ACKNOWLEDGEMENTS | value1 EimAcknowledgements ::= { <SEQ_NUMBER> } |

- D.5	ES11 Requests And Responses

- D.5.1	ES11 Requests

There are no ES11 requests for this version of the specification.

- D.5.2	ES11 Responses

There are no ES11 responses for this version of the specification.

- D.6	Common Server Responses

For all responses with a JSON component the “subjectIdentifier” and “message” are optional and may or may not be present in the response received from the RSP server.

- Annex E	Profiles

| Profile | GENERIC_PROFILE_STRUCTURE |
| --- | --- |
| Description | Generic Operational Profile ASN.1 structure to be used as a basis for all Profiles used in this specification. |
| Details | headerValue ProfileElement ::= header : { major-version 2, minor-version 3, profileType "GSMA Profile Package", iccid '89019990001234567893'H, eUICC-Mandatory-services { usim NULL, milenage NULL }, eUICC-Mandatory-GFSTEList { -- see Note 1 id-MF, id-USIM } } mfValue ProfileElement ::= mf : { mf-header { mandated NULL, identification 1 }, templateID id-MF, mf { fileDescriptor : { pinStatusTemplateDO '01020A'H } }, ef-pl { fileDescriptor : { -- EF PL modified to use Access Rule 15 within EF ARR securityAttributesReferenced '0F'H } }, ef-iccid { -- swapped ICCID: 98109909002143658739 fillFileContent '98109909002143658739'H }, ef-dir { fileDescriptor { -- Shareable Linear Fixed File -- 4 records, record length: 38 bytes fileDescriptor '42210026'H, efFileSize '98'H }, -- USIM AID: A0000000871002FF33FF018900000100 fillFileContent '61184F10A0000000871002FF33FF01890000010050045553494D'H }, ef-arr { fileDescriptor : { fileDescriptor '42210025'H, lcsi '05'H, efFileSize '022B'H }, fillFileContent : '8001019000800102A406830101950108800158A40683010A950108'H, fillFileOffset : 10, fillFileContent : '800101A40683010195010880015AA40683010A950108'H, fillFileOffset : 15, fillFileContent : '80015BA40683010A950108'H, fillFileOffset : 26, fillFileContent : '800101900080015A9700'H, fillFileOffset : 27, fillFileContent : '800103A406830101950108800158A40683010A950108'H, fillFileOffset : 15, fillFileContent : '800111A40683010195010880014AA40683010A950108'H, fillFileOffset : 15, fillFileContent : '800103A406830101950108800158A40683010A950108840132A406830101950108'H, fillFileOffset : 4, fillFileContent : '800101A406830101950108800102A406830181950108800158A40683010A950108'H, fillFileOffset : 4, fillFileContent : '800101900080011AA406830101950108800140A40683010A950108'H, fillFileOffset : 10, fillFileContent : '800101900080015AA40683010A950108'H, fillFileOffset : 21, fillFileContent : '8001019000800118A40683010A9501088001429700'H, fillFileOffset : 16, fillFileContent : '800101A40683010195010880015A9700'H, fillFileOffset : 21, fillFileContent : '800113A406830101950108800148A40683010A950108'H, fillFileOffset : 15, fillFileContent : '80015EA40683010A950108'H, fillFileOffset : 26, fillFileContent '8001019000800102A010A40683010195 0108A406830102950108800158A40683 010A950108'H } } pukVal ProfileElement ::= pukCodes : { puk-Header { mandated NULL, identification 2 }, pukCodes { { keyReference pukAppl1, pukValue '3030303030303030'H, -- maxNumOfAttemps:9, retryNumLeft:9 maxNumOfAttemps-retryNumLeft 153 }, { keyReference pukAppl2, pukValue '3132333435363738'H }, { keyReference secondPUKAppl1, pukValue '3932393435363738'H, -- maxNumOfAttemps:8, retryNumLeft:8 maxNumOfAttemps-retryNumLeft 136 } } } pinVal ProfileElement ::= pinCodes : { pin-Header { mandated NULL, identification 3 }, pinCodes pinconfig : { { keyReference pinAppl1, pinValue '31323334FFFFFFFF'H, unblockingPINReference pukAppl1 }, { keyReference pinAppl2, pinValue '30303030FFFFFFFF'H, unblockingPINReference pukAppl2 }, { keyReference adm1, pinValue '35363738FFFFFFFF'H, pinAttributes 1 } } } usimValue ProfileElement ::= usim : { usim-header { mandated NULL, identification 4 }, templateID id-USIM, adf-usim { fileDescriptor : { fileID '7FF1'H, dfName 'A0000000871002FF33FF018900000100'H, pinStatusTemplateDO '01810A'H } }, ef-imsi { -- numerical format: 234101943787656 fillFileContent '082943019134876765'H }, ef-arr { fileDescriptor { linkPath '2F06'H } }, ef-ust { -- Service Dialling Numbers, Short Message Storage… fillFileContent '0A2E178CE73204000000000000'H }, ef-spn { -- ASCII format: "GSMA eUICC" fillFileContent '0247534D41206555494343FFFFFFFFFFFF'H }, ef-est { -- Services deactivated fillFileContent '00'H }, ef-acc { -- Access class 4 fillFileContent '0040'H }, ef-ecc { -- Emergency Call Code 911 fillFileContent '19F1FF01'H } } usimPin ProfileElement ::= pinCodes : { pin-Header { mandated NULL, identification 5 }, pinCodes pinconfig : { { keyReference secondPINAppl1, pinValue '39323338FFFFFFFF'H unblockingPINReference secondPUKAppl1, -- PIN is Enabled pinAttributes 1, -- maxNumOfAttemps:2, retryNumLeft:2 maxNumOfAttemps-retryNumLeft 34 } } } akaParamValue ProfileElement ::= akaParameter : { aka-header { mandated NULL, identification 6 }, algoConfiguration algoParameter : { algorithmID milenage, -- RES and MAC 64 bits, CK and IK 128 bits algorithmOptions '01'H, key '000102030405060708090A0B0C0D0E0F'H, opc '0102030405060708090A0B0C0D0E0F00'H, -- rotationConstants uses default: '4000204060'H -- xoringConstants uses default value authCounterMax '010203'H } -- sqnOptions uses default: '02'H -- sqnDelta uses default: '000010000000'H -- sqnAgeLimit uses default: '000010000000'H -- sqnInit uses default: all bytes zero } mnoSdValue ProfileElement ::= securityDomain : { sd-Header { mandated NULL, identification 7 }, instance { applicationLoadPackageAID 'A0000001515350'H, classAID 'A000000151535041'H, instanceAID 'A000000151000000'H, applicationPrivileges '82FC80'H, -- Secured lifeCycleState '0F'H, -- SCP80 supported applicationSpecificParametersC9 '810280008201F08701F0'H, -- other parameters MAY be necessary applicationParameters { -- TAR: B20100, MSL: 12 uiccToolkitApplicationSpecificParametersField '0100000100000002011203B2010000'H } }, keyList { { -- C-ENC + R-ENC keyUsageQualifier '38'H, -- ENC key keyIdentifier '01'H, keyVersionNumber '01'H, keyCompontents { { -- DES mode implicitly known (as an example) keyType '80'H, -- This value MAY be freely changed keyData '112233445566778899AABBCCDDEEFF10'H } } }, { -- C-MAC + R-MAC keyUsageQualifier '34'H, -- MAC key keyIdentifier '02'H, keyVersionNumber '01'H, keyCompontents { { -- DES mode implicitly known (as an example) keyType '80'H, -- This value MAY be freely changed keyData '112233445566778899AABBCCDDEEFF10'H } } }, { -- C-DEK + R-DEK keyUsageQualifier 'C8'H, -- data ENC key keyIdentifier '03'H, keyVersionNumber '01'H, keyCompontents { { -- DES mode implicitly known (as an example) keyType '80'H, -- This value MAY be freely changed keyData '112233445566778899AABBCCDDEEFF10'H } } }, -- AES Token Key (as an example) -- This value MAY be freely changed keyUsageQualifier '81'H, -- MAY be used by SD keyAccess '01'H, -- Key Id 01 keyIdentifier '01'H, keyVersionNumber '70'H, keyCompontents { { -- AES (16 bytes key length) -- This value MAY be freely changed keyType '88'H, -- This value MAY be freely changed keyData 'CDFE56B7B72FAE6A047341F003D7A48D'H } } }, { -- Receipt (the AES scheme SHALL be supported) keyUsageQualifier '44'H, -- MAY be used by SD keyAccess '01'H, -- Key Id 01 keyIdentifier '01'H, keyVersionNumber '71'H, keyCompontents { { -- AES (16 bytes key length) keyType '88'H, -- This value MAY be freely changed keyData '11121314212223243132333441424344'H } } } } } ssdValue ProfileElement ::= securityDomain : { sd-Header { mandated NULL, identification 8 }, instance { applicationLoadPackageAID 'A0000001515350'H, classAID 'A000000151535041'H, instanceAID 'A00000055910100102736456616C7565'H, -- by default extradited under MNO-SD -- Privileges: Security Domain + Trusted Path applicationPrivileges '808000'H, -- Personalized lifeCycleState '0F'H, -- SCP80 supported, extradiction supported applicationSpecificParametersC9 '810280008201F0'H, applicationParameters { -- TAR: 6C7565, MSL: 12 uiccToolkitApplicationSpecificParametersField '01000001000000020112036C756500'H } }, keyList { { -- C-ENC + R-ENC keyUsageQualifier '38'H, keyIdentifier '01'H, keyVersionNumber '01'H, keyCompontents { { -- DES mode implicitly known (as an example) keyType '80'H, -- This value MAY be freely changed keyData '11223344556677881122334455667788'H } } }, { -- C-MAC + R-MAC keyUsageQualifier '34'H, -- MAC key keyIdentifier '02'H, keyVersionNumber '01'H, keyCompontents { { -- DES mode implicitly known (as an example) keyType '80'H, -- This value MAY be freely changed keyData '11223344556677881122334455667788'H } } }, { -- C-DEK + R-DEK keyUsageQualifier 'C8'H, -- data ENC key keyIdentifier '03'H, keyVersionNumber '01'H, keyCompontents { { -- DES mode implicitly known (as an example) keyType '80'H, -- This value MAY be freely changed keyData '11223344556677881122334455667788'H } } } } } rfmUicc ProfileElement ::= rfm : { rfm-header { identification 11 }, -- Instance AID instanceAID ' A00000055910100001'H, tarList { 'B00000'H }, -- cryptographic checksum + counter higher minimumSecurityLevel '12'H, -- full access uiccAccessDomain '00'H, -- full access uiccAdminAccessDomain '00'H } rfmUsim ProfileElement ::= rfm : { rfm-header { identification 12 }, -- Instance AID instanceAID 'A00000055910100002'H, tarList { 'B00020'H }, -- cryptographic checksum + counter higher minimumSecurityLevel '12'H, -- full access uiccAccessDomain '00'H, -- full access uiccAdminAccessDomain '00'H, adfRFMAccess { adfAID 'A0000000871002FF33FF018900000100'H, -- UICC access condition: ADM1 adfAccessDomain '02000100'H, -- UICC access condition: ADM1 adfAdminAccessDomain '02000100'H } } endValue ProfileElement ::= end : { end-header { mandated NULL, identification 99 } } |
| Note 1: The following OIDs are used: id-MF OBJECT IDENTIFIER ::= {joint-iso-itu-t(2) international-organizations(23) simalliance(143) euicc-profile(1) template(2) mf(1)} id-USIM OBJECT IDENTIFIER ::= {joint-iso-itu-t(2) international-organizations(23) simalliance(143) euicc-profile(1) template(2) usim(4)} |  |

| Profile | PROFILE_OPERATIONAL1 |
| --- | --- |
| Description | Operational Profile This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF1, except if defined differently in the test sequence. The Unprotected Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE except that: -  the iccid field SHALL be set to #ICCID_OP_PROF1 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF1 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF1 -  the pinAttributes of pinAppl1 present in the PE_PIN SHALL be set to 6 -  the SCP80 encryption key configured in the PE-SecurityDomain that corresponds to the MNO-SD SHALL be set to #MNO_SCP80_ENC_KEY -  the SCP80 message authentication key configured in the PE-SecurityDomain that corresponds to the MNO-SD SHALL be set to #MNO_SCP80_AUTH_KEY -  the SCP80 data encryption key configured in the PE-SecurityDomain that corresponds to the MNO-SD SHALL be set to #MNO_SCP80_DATA_ENC_KEY -  the instance AID configured in the PE-SecurityDomain that corresponds to the Supplementary Security Domain PE_SSD SHALL be set to #SSD_AID -  the ef-dir present in the PE-MF SHALL be configured with the AID #USIM_AID -  the ef-ust SHALL be set in accordance to #EF_UST1 (service 17 and 18 are not available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H The PROFILE_OPERATIONAL1 UPP is named #UPP_OP_PROF1 in the scope of this document. |

| Profile | PROFILE_OPERATIONAL3 |
| --- | --- |
| Description | Operational Profile with PPR2 but without notification This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF3, except if defined differently in the test sequence. The Unprotected Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE except that: -  the iccid field SHALL be set to #ICCID_OP_PROF3 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF3 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF3 -  the pinAttributes of pinAppl1 present in the PE_PIN SHALL be set to 6 -  the ef-ust SHALL be set in accordance to #EF_UST1 (service 17 and 18 are not available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H The PROFILE_OPERATIONAL3 UPP is named #UPP_OP_PROF3 in the scope of this document. |

| Profile | PROFILE_OPERATIONAL4 |
| --- | --- |
| Description | Operational Profile with PPR1 and notification This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF4, except if defined differently in the test sequence. The Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE] except that: -  the iccid field SHALL be set to #ICCID_OP_PROF4 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF4 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF4 -  the pinAttributes of pinAppl1 present in the PE_PIN SHALL be set to 6 -  the ef-ust SHALL be set in accordance to #EF_UST1 (service 17 and 18 are not available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H The PROFILE_OPERATIONAL4 UPP is named #UPP_OP_PROF4 in the scope of this document. |

| Profile | PROFILE_OPERATIONAL7 |
| --- | --- |
| Description | Operational Profile with PPR2 and notification This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF7, except if defined differently in the test sequence. The Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE except that: -  the iccid field SHALL be set to #ICCID_OP_PROF7 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF7 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF7 -  the pinAttributes of pinAppl1 present in the PE_PIN SHALL be set to 6 -  the ef-ust SHALL be set in accordance to #EF_UST1 (service 17 and 18 are not available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H |

| Profile | PROFILE_OPERATIONAL8 |
| --- | --- |
| Description | Operational Profile with PPR2, pinAppl1 enabled and notification This Profile acts as an Operational Profile in the scope of this specification. NOTE: Milenage algorithm is used in this Profile |
| Details | The Profile Metadata SHALL be set to #METADATA_OP_PROF8, except if defined differently in the test sequence. The Profile Package content SHALL follow the ASN.1 structure specified above for GENERIC_PROFILE_STRUCTURE except that: -  the iccid field SHALL be set to #ICCID_OP_PROF8 in the ProfileHeader element, in non-swapped format -  the ef-iccid present in the PE-MF SHALL be set to #ICCID_OP_PROF8 -  the ef-imsi present in the PE-USIM SHALL be set to #IMSI_OP_PROF8 -  The pinAppl1 present in the PE_PIN SHALL be enabled and has the value #PO2_PIN1 -  the ef-ust SHALL be set in accordance to #EF_UST1 (service 17 and 18 are not available) -  the applicationPrivileges in PE-MNO-SD SHALL be set to '82DC00'H - the Token Verification and the Receipt Generation keys SHALL not be set in the PE-MNO-SD -  the applicationSpecificParametersC9 in PE-MNO-SD SHALL be set to '810280008201F08701F0'H |

- Annex F	IUT Settings

- F.3	Device Settings

| Device Setting name | Description |
| --- | --- |
| IUT_TLS_VERSION | Highest TLS protocol version supported by IPAd, at least v1.2. By versions higher then TLS v1.2 backwards compatibility is assumed. |
| IUT_IPAd_NOTIFICATION_TIMEOUT | Timeout in seconds for IPAd to send a Notification to the SM-DP+ on ES9+ interface assuming IP connection is available. |
| IUT_IPAd_TLS_INIT | Timeout in seconds for IPAd to send TLS Client Hello to the SM-DP+ (or SM-DS) on ES9+ (or ES11) interface assuming IP connection is available. The timeout SHALL start after the S-EIM sends the eIM Package with Profile Download Trigger. |
| IUT_CDMA2000_1X_REL | If cdma2000 1X is supported, this SHALL be encoded as the octet string {1, 0, 0}. |
| IUT_CDMA2000_EHRPD_REL | If cdma2000 eHRPD, is supported this SHALL be the highest 3GPP release N fully supported by the Device, encoded as the octet string {N, 0, 0}. |
| IUT_CDMA2000_HRPD_REL | If cdma2000 HRPD is supported, this SHALL be encoded as the octet string {R, 0, 0}. The value R SHALL represent the EVDO revision as follows: Rev 0 SHALL be encoded as 1 Rev A SHALL be encoded as 2 Rev B SHALL be encoded as 3 |
| IUT_EU_CONFIRMATION_TIMEOUT | Timeout in seconds for IPAd for the End User Intent confirmation starting when the LPAd displays the dialog for confirmation. |
| IUT_GSM_GERAN_REL | If GSM/GERAN is supported, this is the highest 3GPP release N fully supported by the Device, encoded as the octet string {N, 0, 0}. |
| IUT_IMEI | International Mobile Equipment Identity value of the Device in human readable format, including the check digit. The value is used as a reference for verification of the TAC (mandatory) and IMEI (optional) retrieved from DeviceInfo. |
| IUT_LPAd_Confirmation | Description of the way to perform Authenticated Confirmation (devices supporting SGP.22 v2.2.2 or earlier) or Strong Confirmation (devices supporting SGP.22 v2.3 or later). |
| IUT_LPAd_CI | CI subjectPublicKeyInfo of CERT.CI.ECDSA (used to verify CERT.DP.TLS) stored in LPAd. Based on NIST [11] in this version of specification. |
| IUT_LPAd_READY_AFTER_REBOOT_TIMEOUT | Timeout in seconds for the LPAd to be ready after a reboot. The time starts from the power off at the start of the reboot and ends when the LPAd is ready after the reboot. |
| IUT_IPAd_Triggering | Description of the way how to perform manually by the tester the triggering of the establishment of the secure connection on ESipa and the triggering of eIM Package Retrieval. |
| IUT_LPAd_SESSION_CLOSE_TIMEOUT | Timeout in seconds for LPAd to send a next command for Profile Download to the SM-DP+ (or SM-DS) on ES9+ (or ES11) interface assuming IP connection is available. The timeout SHALL start after sending of the previous command by the LPAd. |
| IUT_LTE_EUTRAN_REL | If LTE/E-UTRAN is supported, this SHALL be the highest 3GPP release N fully supported by the Device, encoded as the octet string {N, 0, 0}. |
| IUT_NFC_REL | If NFC is supported, this SHALL be the highest (version, revision) number of TS.26 [15], encoded as the octet string {version, revision, 0}. |
| IUT_UMTS_UTRAN_REL | If UMTS/UTRAN is supported, this SHALL be the highest 3GPP release N fully supported by the Device, encoded as the octet string {N, 0, 0}. |

- F.4	Common Settings

In order to execute the test cases defined in this document, the IUT provider SHALL deliver following settings:

| IUT Setting name | Description |
| --- | --- |
| IUT_RSP_VERSION | Version of SGP.22 supported by the IUT encoded as a string of three integers separated with dots (for example: 2.1.0). In the scope of this specification, this value SHALL indicate one of the versions of SGP.22 for which this specification contains test cases, as specified in section 1.2. |

- Annex G	Initial States

Unless it is defined differently in a particular test case, the IUTs SHALL be set in the following initial state before the test case execution.

- G.1	Device

- G.1.1 Device (default)

The Device is “powered on”.

The Device is in the normal execution mode after Device boot-up and Device initial configuration. The Device is NOT in the Test Mode.

The IPAd has access to the root CI key #CERT_CI_ECDSA (or the CI public key) for verification of the TLS certificates of SM-DP+ or SM-DS. No CRL is loaded.

The IPAd can be triggered to establish the secured communication with the eIM.

If the IPAd supports O_IPA_EIM_PACKAGE_RETRIEVAL the IPAd can be triggered to send ESipa.GetEimPackage method to the eIM.

If the IPAd supports O_IPA_DIRECT the IPAd shall provide a way for the tester to input the confirmation code during profile download.

The IPAd has the address of the associated S_eIM.

The Device contains a Test eUICC pre-configured as defined below in G.1.3.

- G.1.2 Test eUICC Settings

Depending on the test cases and on the supported options, the Test eUICC SHALL be configured according to the following Initial States.

- 	The Test eUICC is configured with the ISD-R AID #ISD_R_AID and the EID #EID1.

- 	The Test eUICC does not contain any Profile.

- 	The Test eUICC is configured with the default SM-DS address #TEST_ROOT_DS_ADDRESS.

- 	The Test eUICC contains #TEST_DP_ADDRESS1 as default SM-DP+ address.

- 	The Test eUICC supports the fallback mechanism

The ECASD is configured with at least the following Keys and Certificates based on NIST P-256 [11] for this version of the SGP.33:

- 	The Test eUICC’s Private Key #SK_EUICC_ECDSA (for creating ECDSA signatures)

- 	The Test eUICC’s Certificate #CERT_EUICC_ECDSA (for eUICC authentication) containing the eUICC’s Public Key #PK_EUICC_ECDSA

- 	The GSMA Certificate Issuer’s Public Key #PK_CI_ECDSA (for verifying off-card entities certificates)

- 	The Certificate of the EUM #CERT_EUM_ECDSA

Other Certificates and Keys MAY be present. No CRL is loaded on the Test eUICC.

The CI, identified as highest priority in euiccCiPKIdListForSigning, is also selectable in the euiccCiPKIdListForVerification (i.e. all EUM and eUICC Certificates lead to a Root CI certificate linked to a #PK_CI_ECDSA contained in the eUICC).

This CI corresponds to the SubjectKeyIdentifier of one of the #CERT_CI_ECDSA defined in sections G.2.2 and G.2.3.

For devices supporting O_D_REMOVABLE_DOWNLOAD_PPR, the Test eUICC SHALL contain the RAT configuration specified in #PPRS_ALLOWED_EUC_NOT_REQ

For devices supporting a removable eUICC but not supporting O_D_REMOVABLE_DOWNLOAD_PPR, the Test eUICC can be configured with any RAT.

For devices supporting a non-removable eUICC:

- For some combinations of device options, RAT configurations with certain constraints are required for some sequences, as specified below. These constraints can be satisfied using any valid RAT table; for example, Allowed Operators can be specified explicitly or using wildcards.

| Device option(s) supported | RAT configuration of Test eUICC |
| --- | --- |
| NOT O_D_EMB_ALLOWS_PPR1_EUC_REQ AND O_D_EMB_ALLOWS_PPR1_EUC_NOT_REQ | PPR1 is allowed and End User Consent is not required for #MCC_MNC4 with gid1 and gid2 absent. |
| NOT O_D_EMB_ALLOWS_PPR2_EUC_REQ AND O_D_EMB_ALLOWS_PPR2_EUC_NOT_REQ | PPR2 is allowed and End User Consent is not required for #MCC_MNC2 with gid1 and gid2 absent. |

- If none of the constraints above apply, the Test eUICC can be configured with any RAT.

- Note: in the current version of this document, it is possible to satisfy the relevant constraints above with a single RAT configuration. It is recommended to supply a single device for testing with the RAT configuration satisfying all of the relevant constraints above, rather than to supply multiple devices.

A separate Test eUICC needs to be provided for each additional RAT configuration (not used in this version of the test specification). In case the Test eUICC is non-removable the additional Device SHALL contain the same software and hardware except the Test eUICC configuration.

The Test eUICC is associated to the S_eIM and has the S_eIM public key and S_eIM ID (#EIM_ID1).

The COUNTER_EIM is set to “0”.

Immediate profile enabling is not activated.

- Annex K	Document Management

- K.1	Document History

| Version | Date | Brief Description of Change | Approval Authority | Editor / Company |
| --- | --- | --- | --- | --- |
| SGP.33-2 V1.2 | 22 January | Initial version of SGP.33-2 coming from SGP.23 v1.14 | ISAG | Yolanda Sanz, GSMA |
|  | 05 April 2024 | CR00001R01 Adaption of ES9+ and ES11 test cases CR00002R00 LUI settings deletion CR00003R01 IPAd Test Environment CR00004R02 DirectProfileDownload CR00005R06 Procedure Enable Profile IPA Initiated CR00006R02 ES9+ Test Cases CR00007R00 IUT Setting for IPAd triggering |  | Yolanda Sanz, GSMA |
|  | 30 May 2024 | CR00008R00 Fix_TC5.4.1.2.2 |  |  |
|  | 12 December 2024 | Revised during the eSIMWG3.117 |  | Yolanda Sanz, GSMA |
|  | 13 December 2024 | Revised during the eSIMWG3.117 |  | Yolanda Sanz, GSMA |
|  | 17 December 2024 | Following eSIMWG116_2 discussion version updated to v1.2 (note v1.0 and v1.1 were never published, updated to 1.2 to align numbering with Core Specifications.) CR00009R01 ES11 Test Cases CR00010R02 Procedure Disable Profile_IPA_Initiated CR00011R03 Procedure Delete Profile_IPA_Initiated CR00012R02 Procedure Set Unset FallbackAttribute CR00013R00 Procedure Enable Profileupdate |  | Stephen Packer, GSMA |

- K.2	Other Information

| Type | Description |
| --- | --- |
| Document Owner | eSIMG |
| Editor / Company | Stephen Packer, GSMA |

It is our intention to provide a quality product for your use. If you find any errors or omissions, please contact us with your comments. You may notify us at prd@gsma.com.

Your comments or suggestions & questions are always welcome.