Application Note Addressing Issue within SGP.22 v2.7 ASN.1

19th May 2026

Overview:

The purpose of this Application Note is to provide guidance for the industry on the recently released SGP.22 v2.7, specifically on the ASN.1 file extracted and also the ASN.1 contained within SGP.22 v2.7 Annex H.

Details of Issue:

There were two instances within SGP.22 v2.7 where an em dash “—”  rather than a hyphen “-”  was inserted into the ASN.1.

Within the specification, in section 5.7.8 Function (ES10b): GetEUICCInfo, on page 205 within the Response Data there is this section:

Between ‘OPTIONAL’ and ‘value’ there should be a space, two hyphens followed by another space. There is an error where the first hyphen is an em dash.

A few lines later, on the same page, there is this section:

Between ‘(28)’ and ‘support’ there should be a space, two hyphens followed by another space.  There is an error where both hyphens are replaced by a single em dash.

The same errors are repeated in Annex H where all of the ASN.1 Definitions are presented; both errors are on page 277.

This error also existed within the ASN.1 file that was placed on GSMA.com on the 27th of April. The file was updated on the 4th of May.

Recommendations:

Any ASN.1 files formed from either the text within SGP.22 v2.7, or from the associated ASN.1 files published on GSMA.com should be checked for the em dash issue in the two sections within Function (ES10b): GetEUICCInfo mentioned above.

The SGP.22 v2.7 ASN.1 file currently on GSMA.com has had this issue addressed.