# eUICC Security Assurance Principles

# Version 2.4

# 22 October 2026

Security Classification: Non-Confidential

Access to and distribution of this document is restricted to the persons permitted by the security classification. This document is subject to copyright protection. This document is to be used only for the purposes for which it has been supplied and information contained in it must not be disclosed or in any other way made available, in whole or in part, to persons other than those permitted under the security classification without the prior written approval of the Association.

Copyright Notice

Copyright © 2026 GSM Association

Disclaimer

The GSMA makes no representation, warranty or undertaking (express or implied) with respect to and does not accept any responsibility for, and hereby disclaims liability for the accuracy or completeness or timeliness of the information contained in this document. The information contained in this document may be subject to change without prior notice.

Compliance Notice

The information contain herein is in full compliance with the GSMA Antitrust Compliance Policy.

This Permanent Reference Document is classified by GSMA as an Industry Specification, as such it has been developed and is maintained by GSMA in accordance with the provisions set out GSMA AA.35 - Procedures for Industry Specifications.

Contents

1	Introduction	3

1.1	Overview	3

1.2	Scope	3

1.3	Definitions	3

1.4	Abbreviations	4

1.5	References	4

1.6	Conventions	5

2	TOE Overview and Scheme Contact Details	6

2.1	TOE-type overview	6

2.2	Contact details	7

3	GSMA Certification Body (GSMA CB)	7

3.1	ISO17065 accreditation:	7

3.2	Laboratory Licensing:	7

3.3	Certification Consistency:	7

4	Laboratory Licensing	8

5	Process	8

5.1	Submission Phase	9

5.2	Evaluation Phase	9

5.3	Certification Phase	9

5.4	Certification Procedures and Assurance Continuity	9

5.4.1	New Certification	10

5.4.2	Maintenance with Minor Changes	10

5.4.3	Maintenance with Major Changes	10

5.4.4	Re-Certification	10

6	Scheme Oversight	10

7	Appeals	11

8	Scheme Monitoring	12

Annex A	GSMA eUICC Security Assurance Application Form	13

A.1	General Product Information	13

A.2	Details of Submitter (Developer)	13

A.3	Details of Evaluator	14

A.4	Final Evaluation Meeting Schedule Date	14

Annex B	Document Management	14

B.1	Document History	14

B.2	Other Information	15

# Introduction

## Overview

The GSMA eUICC Security Assurance Scheme is an independent security evaluation scheme for evaluating eUICCs against the provisions of Protection Profiles for eUICCs. The scheme aims to establish trust for Service Providers and other risk-owners that their assets, including profiles for eUICC remote provisioning, are secure against state-of-the-art attackers. The scheme is based on the Common Criteria methodology ISO15408 [6], optimised for GSMA compliant eUICCs.

The scheme owner is the GSMA. The scheme is operated in accordance with the provisions and expectations of ISO17065 [7]. It includes a certification function with the Certification Bodies (CB), appointed by GSMA.

## Scope

This document provides the GSMA eUICC Security Assurance principles for the GSMA eUICC Scheme including key details, contacts, and links.

## Definitions

| Term | Description |
| --- | --- |
| Certifier | Person acting on behalf of the GSMA Certification Body |
| Developer | Person acting on behalf of the EUM |
| Evaluation Technical Report | Report that documents the overall verdict and its justification, produced by the evaluator and submitted to an evaluation authority. |
| ETR for composition | Evaluation Technical Report for composite evaluation. |
| eSA Appeals Board | Group in charge of ruling on appealed Evaluation and Certification Results, deciding to uphold the Evaluation and Certification Results or decide on the interpretation of the eSA scheme. |
| eUICC | A removable or non-removable UICC which enables the remote and/or local management of Profiles in a secure way. NOTE: The term originates from "embedded UICC". |
| Evaluator | Person acting on behalf of the Licensed Laboratories |
| GSMA Certification Body | Certification Body role, appointed by GSMA |
| Licensed Laboratory | A security evaluation laboratory licensed by a GSMA CB to perform eUICC security evaluations for the GSMA eUICC Scheme |
| SOG-IS Authorising Scheme | As defined by https://www.sogis.eu/uk/status_participant_en.html and https://www.sogis.eu/uk/tech_domain_en.html (technical domain: smart cards and similar devices) |
| Security Target | Implementation-dependent statement of security needs for a specific identified TOE |
| Target of Evaluation | Set of software, firmware and/or hardware possibly accompanied by guidance. |

## Abbreviations

| Term | Description |
| --- | --- |
| CB | Certification Body |
| CC | Common Criteria |
| CCRA | Common Criteria Recognition Agreement |
| eSA | eUICC Security Assurance |
| ETR | Evaluation Technical Report |
| ETRfc | ETR for composition |
| ETRfi | ETR for integration |
| EUM | eUICC Manufacturer |
| GSMA CB | GSMA Certification Body |
| IC | Integrated Circuit |
| ST | Security Target |
| TOE | Target of Evaluation |

## References

| Ref | Doc Number | Title |
| --- | --- | --- |
|  | [SGP.05] | Embedded UICC Protection Profile, version - 1.0, GSM Association, September 2014 - 1.1, GSM Association, August 2015 also published by BSI as BSI-CC-PP-0089-2015 - 4.0, GSM Association, October 2022 - 4.1, GSM Association, March 2023. |
|  | [SGP.25] | eUICC for Consumer and IoT Device Protection Profile, version: - 1.0, GSM Association, June 2018 also published by BSI as BSI-CC-PP-0100-2018 2.0 GSM Association, December 2023 2.1 GSM Assocciation, February 2025 also published by BSI as BSI-CC-PP-0100-v2 |
|  | [ SGP.07] | GSMA eUICC Security Assurance Methodology |
|  | [AA.35] | GSMA Procedures for Industry Specification |
|  | [RFC2119] | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner http://www.ietf.org/rfc/rfc2119.txt |
|  | [ISO15408] | The Common Criteria for Information technology — Security techniques — Evaluation criteria for IT security |
|  | [ISO17065] | Conformity assessment — Requirements for bodies certifying products, processes, and services |
|  | [CCRA] | Assurance Continuity: CCRA Guidelines |
|  | Void | Void |
|  | [RFC8174] | Ambiguity of Uppercase vs Lowercase in RFC 2119 Key Words https://www.rfc-editor.org/info/rfc8174 |
|  | [AMS] | JIL Attack Methods for Smartcards and Similar Devices |
|  | [AAPS] | JIL Application of Attack Potential to Smartcards EUCC - Application of attack potential to smartcards and similar devices |
|  | [PP-0084] | Security IC Platform Protection Profile with Augmentation Packages version 1.0, February 2014, BSI-CC-PP-0084-2014. |
|  | [PP-0117] | Secure Sub-System in System-on-Chip (3S in SoC) Protection Profile, Version: - v1.5 - BSI-CC-PP-0117-2022 - v1.8 – BSI-CC-PP-0117-v2-2023 |
|  | [COMP] | EUCC - Composite product evaluation for Smart Cards and similar devices for CC 3.1 EUCC - Composite product evaluation and certification for CC: 2022 |
|  | [VA-3S] | JIL Guidance for Vulnerability Analysis and Penetration Testing of a Secure Sub-System within a System-on-Chip |
|  | [SGP.27] | PP Module for Application Isolation in the eUICC |
|  | [SGP.17-1] | - Security Target template for eUICC Consumer Devices |
|  | [SGP.17-2] | Security Target Template for Consumer eUICC with LPAe |
|  | [SGP.17-3] | Security Target Template for IoT eUICC |

## Conventions

The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” in this document are to be interpreted as described in RFC2119 [5] and clarified by RFC8174 [10], when, and only when, they appear in all capitals, as shown here.

“FFS” or “For Further Study” means that it will be covered in the next version of SGP.06.

# TOE Overview and Scheme Contact Details

## TOE-type overview

The scope for GSMA eUICC Security Assurance (eSA) evaluation is the combination of the hardware and software components implementing an eUICC (IC, JavaCard, etc.) holding profiles for remote provisioning, excluding the specific profiles themselves.

The following evaluations are possible with eSA scheme:

- (A) Consumer and/or IoT eUICC on top of certified IC /TRE: in this case, the TOE SHALL comply with PP-0100 (SGP.25 [2]) + SGP.27 [17]. An ETR is provided at the end of evaluation.

- (B) M2M eUICC on top of certified IC /TRE: in this case, the TOE SHALL comply with PP-0089 (SGP.05[1]). An ETR is provided at the end of evaluation.

- (C) IC /TRE and Consumer and/or IoT eUICC in a single evaluation project and two certifications: The IC/TRE TOE SHALL comply with PP-0084/PP-0117. The eUICC TOE SHALL comply with PP-0100 (SGP.25[2]) + SGP.27 [17] and it requires composition [15] with the IC/TRE. Two STs (as per SGP.25) SHALL be provided: one for the IC/TRE and another for the Consumer and/or IoT eUICC. At the end of evaluation ETR, ETRfc and ETRfi (if required) will be provided for the IC/TRE and ETR will be provided for the eUICC.

- (D) IC /TRE and M2M eUICC in a single evaluation project and two certifications: The IC/TRE TOE SHALL comply with PP-0084/PP-0117. The eUICC TOE SHALL comply with PP-0089 (SGP.05) and it requires composition [15] with the IC/TRE. Two STs SHALL be provided: one for the IC/TRE and another for the M2M eUICC. At the end of evaluation ETR, ETRfc and ETRfi (if required) will be provided for the IC/TRE and ETR will be provided for the eUICC.

Note: In the case of IC /TRE and eUICC evaluation in a single project (C and D), the ETRfc of the IC/TRE could be only reused by another eSA eUICC evaluation using the same IC /TRE. The certificate is not published for the IC/TRE.

- (E) IC/TRE evaluation: the TOE SHALL comply with PP-0084. An ETR, ETRfc and ETRfi(if required) is provided at the end of evaluation, to be only used for an eUICC composite certification.

- (F) Integrated TRE evaluation: the TOE SHALL comply with PP-0117. An ETR, ETRfc and ETRfi (if required) is provided at the end of evaluation, to be only used for an eSA eUICC composite certification [15].

Note: In the case of IC/TRE evaluation (E and F), the certificate is not published.

The details of the security certification scopes and requirements are provided in SGP.05 [1], SGP.25 [2] , SGP.27 [17], PP-0084[13] and PP-0117[14].

## Contact details

The primary source of contact information on the scheme SHALL be published on the GSMA website. This includes details of the scheme Certification Body and Licensed Laboratories as well as links to scheme procedures and documents and the scheme owner contact address.

# GSMA Certification Body (GSMA CB)

For a Certification Body to become a GSMA appointed Certification Body and stay appointed under the eSA Scheme, they need to:

- Respond to the GSMA eSA CB Request for Proposal.

- Satisfy the requirements stated in the GSMA eSA CB Request for Proposal.

- Sign the GSMA eSA Scheme Standards Terms and Conditions.

- Sign the Letter of Agreement on GSMA eSA Scheme.

A GSMA appointed Certification Body SHALL be responsible for: ISO17065 accreditation, Laboratory Licensing, and Certification Consistency.

## ISO17065 accreditation:

- Gaining and maintaining ISO17065 [7] accreditation for the scheme, including identifying necessary updates to the GSMA scheme documentation to align with ISO17065 [7] expectations.

- Including the GSMA eSA scheme within its scope of ISO17065 [7] accredited certification activities.

## Laboratory Licensing:

- Licensing their laboratories for the scheme (according to the scheme defined criteria).

- Verifying that the eSA Licensed Laboratory has the capacity to perform SOG-IS/EUCC security evaluation for the IC/TRE using PP-0084[13] or PP-0117 [14].

- Alignment of Licensed Laboratories to ensure a common approach to evaluations performed under the scheme methodology. In case of the Vulnerability Analysis, the alignment SHALL be based on the latest JIL guidance documentation [11] and [12].

## Certification Consistency:

In order to assist the consistent application of the criteria and methods between Evaluation and Certification processes,  each GSMA Certification Bodies SHALL ensure and work towards a uniform interpretation of the currently applicable eSA Scheme Principles and Methodology [3] and in case of Vulnerability Analysis, it will base its final judgment on current JIL/EUCC guidance [11],  [12] and [16].

For the IC/TRE certification and Laboratory licensing, they SHALL have proven expertise as a CB by certifying products through security evaluation for the IC/TRE using PP-0084[13]  and/or PP-0117[14].

# Laboratory Licensing

For a laboratory to become and stay licensed under the eSA Scheme, they need to:

- Show and continue to show quality in performing evaluation considering state-of-the-art attackers.

- Have and maintain a valid ISO-17025 [8] accreditation with Common Criteria (including performing at least EAL4+ALC_DVS.2+AVA_VAN.5 assurance requirements) in the technical domain of smart cards and similar devices.

- Have and maintain a valid accreditation and authorisation under a SOG-IS Authorising Scheme or EUCC Scheme for the technical domain of smart cards and similar devices at AVA_VAN.5. For the IC/TRE evaluation they SHALL have proven expertise by certifying products through security evaluation for the IC/TRE using PP-0084[13]  and/or PP-0117[14]  under the same SOG-IS (EUCC) CB.

- Agree to the standard NDA with the GSMA CB.

- Pay the relevant GSMA administration fee, this fee waived for laboratories who are GSMA members.

- Sign the GSMA eSA Standards Terms and Conditions.

A GSMA Licensed Laboratory SHALL be responsible for:

- Reporting its analysis, describing the evaluation activities according to the evaluation methodology and provide a clear conclusion whether the product meets the requirements or not.

- Performing the vulnerability analysis and testing, needed to ensure that the product is protected against state-of-the-art attacks as they are defined in the applicable documents, including the current JIL guidance [11] and [12].

Laboratories wishing to be licensed SHALL contact a GSMA CB and GSMA and be prepared to provide evidence of fulfilling the above requirements. The result of a successful submission is listing by GSMA as an eSA Licensed Laboratory.

The GSMA CB SHALL verify that the eSA Licensed Laboratory has the capacity to perform SOG-IS CC security evaluation for the chip/IC using PP-0084[13] or PP-0117 [14].

# Process

The scheme consists of the following phases:

- Submission

Evaluation

Certification.

Prior to submission, the Developer SHALL have contracted with a Licensed Laboratory.to fill the application form together

The expected response time for all parties involved in each phase (submission, evaluation and certification) is 10 working days.

## Submission Phase

A signed copy of the application form , together with a draft Security Target (ST), SHALL be sent to the GSMA CB.

The (draft) ST SHALL be in accordance with the scheme scope; compliance claims to the associated protection profiles: PP-0084[13], PP-0117 [14], PP-0089  SGP.05 [1] or PP-0100 (SGP.25 [2]) including SGP.27[17].

The ST templates defined in SGP.17-1 [18], or SGP.17-2 [19], MAY be used for Consumer eUICC or Consumer eUICC with LPAe, respectively. The ST templates defined in SGP.17-3 [19], MAY be used for IoT eUICC.

The GSMA CB SHALL respond with a quotation for certification for acceptance by the EUM. Upon agreement of quotation and schedule the evaluation phase SHALL commence.

A fixed price list SHALL be published by the GSMA CBs, and updated annually for all types of certifications.

## Evaluation Phase

By default, the three-stage evaluation phases defined by the GSMA eUICC Security Assurance Methodology [3] will be applied for eUICC evaluations under the scheme, following the GSMA process. The first two evaluation stages MAY be combined resulting in a single meeting with the approval of the GSMA CB, if this is desired by the Developer, with a potentially increased project risk for the Evaluator and Developer.

Reporting for GSMA eUICC evaluations SHALL follow the GSMA eUICC Security Assurance Methodology [3].

## Certification Phase

Certificates are published by the GSMA even after expiry.

The certificate validity period for GSMA eUICC certificates SHALL be five (5) years from last or more recent of the issuance date. If required, this period can be repeatedly extended through recertification.

Where certification relies on underlying composite product certificates these certificates SHALL have at least 12 months remaining validity at the time of certification issuance.

## Certification Procedures and Assurance Continuity

The certification process supports new product certification and changed TOE certification based on the CCRA terminology supporting document “Assurance Continuity: CCRA Guidelines.”[8] Specifically, the GSMA eUICC scheme provides the following certification procedures.

### New Certification

This procedure SHALL be used for new product evaluations.

### Maintenance with Minor Changes

This procedure SHALL be used for changes of the TOE without security impact. Maintenance MAY not require involving a Licensed Laboratory. If it is not clear whether a security function has changed, the Developer SHALL assist the Certifier by providing an analysis from a Licensed Laboratory.

The maintained certificate will have the same validity as the original certificate.

### Maintenance with Major Changes

This procedure SHALL be used for security relevant changes, where only the changed functionality will be assessed. It always requires the involvement of a Licensed Laboratory and the vulnerability analysis and testing are limited to the changes. The certificate validity from the original certificate will be kept.

Notwithstanding the above, a maintenance with major changes MAY introduce functionality within the full scope of the PP Module [17]. In such cases, the modified functionality and its associated security requirements SHALL be assessed against the applicable PP Module [17].

### Re-Certification

This procedure SHALL be used to extend the certificate validity with or without changing the TOE.

If major changes are present, then the evaluation focusses on the changes of the TOE using the current state-of-the-art attack techniques.

If the TOE is not changed, then the vulnerability analysis and testing is updated considering the current state-of-the-art attack techniques.

Notwithstanding the above, a re-certification MAY introduce or extend functionality that falls within the scope of the applicable PP Module [17]. In such cases, the changed or newly introduced functionality and its associated security properties SHALL be assessed against the PP Module [17] requirements, in addition to the re-certification activities defined above.

# Scheme Oversight

Scheme oversight SHALL be performed by committee comprised of GSMA staff and GSMA members representatives of all participants in the scheme.

It is responsible for maintenance of the eSA Scheme documentation.

The roles of this panel SHALL include:

The periodic review of the eSA scheme documentation SHOULD focus on:

Review feedback on the scheme operation and ensure the scheme remains fit for purpose.

Propose scheme improvements to ensure it remains optimised for GSMA eUICC scheme evaluations.

Contributes to the development of Certification Body and Licensed Laboratory selection criteria.

The eSIMWG5 Subgroup is also responsible for risk management of the eSA Scheme such as:

Agree on potential waivers to the security requirements, as identified by the GSMA CB and other inputs

Analyse potential security flaws not covered by the scheme and manage the evolution of the scheme, the GSMA Protection Profile [1] and [2], and the corresponding list of attacks

Analyse and mitigate security issues reported to the eSIMWG5 Subgroup by the GSMA CB

- The eSIMWG5 Subgroup is also responsible for ensuring alignment between performance and assurance of the GSMA eSA Certification Bodies.

- Updates will normally arise from a regular review meeting of the eSIMWG5 Subgroup.

- Where acute issues are identified ad hoc meetings MAY be convened to discuss updates to the eSA Scheme documentation.

# Appeals

Appeals SHOULD first be brought to the GSMA CB that SHALL apply their appeal process as described in ISO17065[7] and that SHALL take precedent to the appeal process by the eSA Appeal Board.

In the event that the GSMA CB appeal process as described in ISO17065[7]  does not resolve the dispute, the Developer MAY lodge an appeal with the GSMA within twenty (20) business days of completion of the Evaluation/Certification. The GSMA will refer the appeal to the eSA Appeals Board.

In case of an appeal, the evaluation report will also be provided to the eSA Appeals Board.

The eSA Appeals Board is comprised of one Licensed Laboratory and one GSMA CB (if it exist) separate from the Licensed Laboratory and GSMA CB companies that performed the Evaluation/Certification that is the subject of the appeal.

The eSA Appeals Board will consider and rule on appealed Evaluation and Certification Results, deciding to uphold the Evaluation and Certification Results or decide on the interpretation of the eSA scheme. The process to be followed by the eSA Appeals Board will include:

 Review of the Evaluation Report, focussing on the appealed assessment(s)

 Discussion with the Licensed Laboratory and GSMA CB that perfomed the   Evaluation/Certification

The Developer MAY request the members of the eSA Appeals Board to sign an NDA prior to receiving a copy of the Evaluation Report and other information about the product in question.

The eSA Appeals Board will seek to rule on appeals within twenty (20) business days of lodgement of the appeal, subject to the availability of the Licensed Laboratory, GSMA CB and the Developer and the prompt provision of any information requested from either party.

The Developer and the Licensed Laboratory and GSMA CB companies that performed the Evaluation/Certification that is the subject of the appeal, agree to accept the decision of the eSA Appeals Board as final.

# Scheme Monitoring

Scheme monitoring SHALL be performed by GSMA. This monitoring SHALL include the following indicators:

- Time monitoring:

Time between the receipt of the application form including the draft Security Target from the Developer and the GSMA CB sending the appropriate quotation.

Time between receipt of evaluation reporting from Licensed Laboratory and the approval of the ETR by the GSMA CB.

Time between the approval of the ETR by the GSMA CB and the delivery of the final certificate.

- Number of certified products per year to evaluate how the scheme is used.

These indicators SHALL be shared by GSMA on regular basis.

GSMA eUICC Security Assurance Application Form

This application form requests an eUICC Security Assessment under the GSMA scheme operated by GSMA CB.

This document SHALL be completed and submitted to the appropriate GSMA recipient. The signed form (in pdf format) SHALL be included along with the (draft) Security Target.

| Certificate type | ☐ New Certification ☐ Maintenance with Minor Changes: [Reference of the original certificate] [Latest issue date] ☐ Maintenance with Major Changes: [Reference of the original certificate] [ Latest issue date] ☐ Re-Certification: [Reference of the original certificate] [Latest issue date] |
| --- | --- |
| Publish certificate in the GSMA website | ☐ Yes ☐ No |

General Product Information

| TOE reference | [Name] [Version] |
| --- | --- |
| ST reference | [Name] [Version] [Date] |
| Protection Profile | [Depending on the ‘Product type’ above, indicate the PP document and version. Please mark only one] (A.1)☐ PP-0100 (SGP.25): eUICC on top of certified IC (TRE)(B)☐ PP-0089 (SGP.05) : eUICC on top of certified IC (TRE) (A.2) ☐ PP-0100 (SGP.25) + SGP.27 eUICC on top of certified IC (TRE) (C.1) ☐ PP-0084/PP0117 and PP-0100 (SGP.25): IC (TRE) and eUICC (C.2) ☐ PP-0084/PP0117 and PP-0100 (SGP.25) + SGP.27: IC (TRE) and eUICC (D)☐ PP-0084/PP0117 and PP-0089 (SGP.05): IC (TRE) and eUICC (E)☐ PP-0084: IC (TRE) (F)☐ PP-0117: IC (iTRE) |
| Version | [add the PP version here] |

Details of Submitter (Developer)

| Company Name | [Company name as registered for GSMA] |
| --- | --- |
| Contact Name | [Name of the person] |
| Position | [Position of the person] |
| Address | [Address of the person] |
| Telephone | + |
| Email Address | [Email of the person] |

Details of Evaluator

| Company Name | [Company name as registered for GSMA CB] |
| --- | --- |
| Contact Name | [Name of the person] |
| Position | [Position of the person] |
| Address | [Address of the person] |
| Telephone | + |
| Email Address | [Email of the person] |

Final Evaluation Meeting Schedule Date

| Proposed Final Evaluation Meeting date |  |
| --- | --- |

Document Management

Document History

| Version | Date | Brief Description of Change | Approval Authority | Editor / Company |
| --- | --- | --- | --- | --- |
| 1.0 | 07 July 2020 | New Document for | ISAG | Gloria Trujillo, GSMA |
| 1.1 | 05 July 2023 | CR0030R01 - Introduce multiple CBs CR0031R04 – CR Advisory Panel Changes CR0032R01 - SGP.06 Annex A modifications CR0033R03 - SGP.06 Annex A further modifications | ISAG | Gloria Trujillo, GSMA |
| 2.0 | 19 December 2023 | CR0035R02 - Add latest versions of SGP.05 and SGP.25 to the scope | ISAG | Gloria Trujillo, GSMA |
| 2.1 | 22 March 2024 | CR0036R01 – Add JIL document into reference section and add correspondent sentences | ISAG | Gloria Trujillo, GSMA |
| 2.2 | 13 March 2025 | CR0039R00_Add SGP.25 V2.1 | eSIM G | Gloria Trujillo, GSMA |
| 2.3 | 27 May 2025 | CR0037R06 - Add of TRE to SGP.06 CR0040R01 - Review SGP.06 regarding TRE addition and correct references | ISAG | Gloria Trujillo, GSMA |
| 2.4 | 22 October 2026 | CR0041R01_Add SGP.17-X references CR0042R02 Add PP Module CR0043R01 PP Module in Product Maintenance and Re-certification | ISAG | Gloria Trujillo, GSMA |

Other Information

| Type | Description |
| --- | --- |
| Document Owner | eSIMG |
| Editor / Company | Gloria Trujillo, GSMA |

It is our intention to provide a quality product for your use. If you find any errors or omissions, please contact us with your comments. You may notify us at prd@gsma.com

Your comments or suggestions & questions are always welcome.

Under the SOG-IS rules at the time of issuance of this document, point 3 implies 1 and 2 are addressed.